3GPP TSG RAN WG1 #106bis-e		                                                               R1-2109637

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.13.2

Source: 	Intel Corporation

Title:	On efficient activation/de-activation for SCells

Document for: 	Discussion/Decision

# Introduction

At RAN plenary meeting #88-e, a revised work item (WI) on Further Multi-RAT Dual-Connectivity enhancements was approved for Release 17 [1], which includes efficient SCell activation/deactivation mechanism from RAN1 perspective:

- Support efficient activation/de-activation mechanism for one SCG and SCells

- Support for one SCG applies to (NG)EN-DC, and NR-DC [RAN2, RAN3, RAN4]

- Support for SCells applies to NR CA, based on RAN1 leading mechanisms [RAN1, RAN2, RAN4]

- This objective applies to FR1 and FR2

In last RAN1 meetings, the following agreements were made,

Working Assumption At least for the case of known cell, temporary RS is supported to expedite the activation process during the SCell activation procedure for efficient SCell activation for both FR1 and FR2: · The temporary RS should provide at least the functionalities of AGC settling and time/frequency tracking during SCell activation procedure. · FFS potential functionalities of CSI measurement/acquisition and cell search Agreements: TRS is selected as temporary RS for Scell activation · If more functionalities are confirmed to be supported by temporary RS, other RS candidates, e.g. aperiodic CSI-RS, P/SP-CSI RS, SRS and RS based on SSS/PSS, are not precluded. · The TRS should be triggered by DCI or MAC-CE. FFS which exact triggering command. Agreements: UEs measure the triggered temporary RS during Scell activation procedure no earlier than a slot m: · FFS timeline values m which may need coordination with RAN4. · FFS if the triggered temporary RS can be associated with a BWP, then the measurement above is independent of the activation state of the BWP. Agreements: Companies are encouraged to provide design details of temporary RS next meeting, at least including: - TRS structure, e.g. whether to fully reuse existing Rel-15/16 TRS structure and configuration restriction (refer to S5.1.6.1.1 of TS 38.214), or any modification - QCL information, if any - Triggering command: DCI format/fields or MAC-CE fields - Triggering timeline/scheduling offset Agreements: As working assumption, with respect to efficient SCell activation, reuse existing Rel-15/16 TRS structure for temporary RS - FFS: how many burst/symbols are required for both AGC settling and Time/Frequency tracking for different cases, e.g. FR1 and FR2, known and unknown SCell - A burst of temporary RS is notated as in S5.1.6.1.1 of TS 38.214 - “2-slot with four CSI-RSs resources (4 samples)” for FR1 - either “1-slot with two CSI-RSs resources (2 samples)” or “2-slot with four CSI-RSs resources (4 samples)” for FR2 - The working assumption can be confirmed after RAN4 check. (A LS for such request is planned). Agreements: For efficient SCell activation, discuss and agree from the following alternatives at RAN1#104-e - Alt 1: the trigger of temporary RS is integrated into a single triggering signaling with the trigger of SCell activation transmitted on an activated cell. - FFS detailed design of this integrated triggering signaling. - Potential examples of single triggering signaling for further discussions - A PDSCH TB, e.g. containing two respective MAC-CEs for both triggers, one MAC-CE for both triggers - A DCI for both triggers - A PDSCH TB and its scheduling DL grant, e.g. MAC-CE for activation and DL grant for temporary RS - A DL grant and a UL grant received in the same slot/OFDM symbols of PDCCH where the DL grant is scheduling a MAC-CE for SCell activation and the UL grant is triggering the RS. - Rel-15/16 SCell activation MAC-CE and a specific configuration of temporary RS being implicitly triggered as well - Alt2: Triggering of temporary RS separately from SCell activation command is not precluded and both ‘separate’ triggers (examples below) and ‘integrated’ triggers (examples in Alt 1) are considered for SCell activation - FFS detailed design of separate triggering signaling. - Potential examples of separate triggering signaling for further discussions - Rel-15/16 SCell activation MAC-CE and Rel 15/16 DCI triggering - Rel-15/16 SCell activation MAC-CE and new DCI triggering for temporary RS - Note: temporary RS should be triggered by DCI or MAC-CE. - Note: the final mechanism of trigger signaling targets at applicability to one or more SCell activation. - FFS handling of SCell activation by existing Rel15/16 CA activation command when temporary RS is configured and triggered/not triggered Draft LS (R1-2009786) to RAN4 is approved. Final LS in R1-2009798. Agreement For efficient activation of SCells, down select at least one option from below: - Option 1a: MAC CE(s) contained in a single PDSCH to trigger both SCell activation and corresponding temporary RS(s) - Details FFS including timeline design for receiving temporary RS - Option 1b: A single DCI to trigger both SCell activation and corresponding temporary RS(s) - Details FFS including potential impact on SCell activation related procedures and, e.g. timeline design for SCell activation and for receiving temporary RS - FFS: The same DCI for SCell deactivation - Option 2: A Rel-15/16 SCell activation MAC-CE to trigger SCell activation and a Rel-15/16 DCI to trigger corresponding temporary RS(s) with enhancement of timeline - Details FFS including timeline design for receiving a DCI trigger of temporary RS, and for receiving temporary RS - Note: Companies are encouraged to provide complete solutions for fast SCell activation. - Note: the previous agreement on the definitions of Alt 1 and Alt 2 is still effective Agreement For efficient activation of SCells - Option 1a: MAC CE(s) contained in a single PDSCH to trigger both SCell activation and corresponding temporary RS(s) - Details FFS including timeline design for receiving temporary RS Note: Separate from the support of Option 1a, it is up to RAN4 whether or not to consider an activation time enhancement for Option 2 without requiring further RAN1 work - Option 2: A Rel-15/16 SCell activation MAC-CE to trigger SCell activation and a Rel-15/16 DCI to trigger corresponding Rel-15/16 A-TRS(s) Send an LS to RAN4. The LS is endorsed in R1-2104110. Agreement For efficient activation of Scells, the triggered temporary RS is aperiodic. Agreement For efficient activation of a Scell (in known Scell case), at least the number of temporary RS bursts is indicated by a field in new MAC-CE - The number of temporary RS bursts is RRC configurable. - FFS: which field in MAC-CE is used and how this field is associated with the number of bursts - For the purpose of designing temporary RS Scell activation, there is no RAN1 specification impact for the case where the number of indicated temporary RS bursts is smaller than what is expected by the UE Agreement To trigger temporary RS for efficient activation of SCells, the contents of the triggering MAC-CE(s) in a single PDSCH provide at least the following information (explicitly or implicitly): - Whether or not temporary RS is triggered - FFS detailed Information of temporary RS, e.g.: - Resources used for triggered Temporary RS - Triggering time offset of triggered Temporary RS - QCL source for triggered Temporary RS - FFS: Detailed signaling structure of the triggering MAC-CE(s) including the down-selection between the following example options and whether the decision should be made in RAN1 or RAN2 - Opt. 1.1: One new MAC CE for both SCell activation triggering and corresponding temporary RS triggering - Opt. 1.2: One R15/16 SCell activation MAC CE for SCell activation triggering and one new MAC CE (in the same PDSCH) for corresponding temporary RS triggering Agreement For efficient activation of a Scell (in known Scell case), the triggering offset of temporary RS is indicated by a field in new MAC-CE - The candidate value(s) of triggering offset(s) is RRC configurable - FFS: which field in MAC-CE is used and how this field is associated with the value of triggering offset Agreement For the reference slot for triggering offset of temporary RS - Option 2: the last DL slot of the to-be-activated Scell overlapping with slot n+k as defined in 38.213 sub-clause 4.3 - FFS: the earliest slot no earlier than the reference slot for a UE to receive a triggered temporary RS Agreement If a UE measures a temporary RS triggered by a MAC-CE during SCell activation procedure, the measurement is performed within the BWP bandwidth of BWP indicated by firstActiveDownlinkBWP-Id Agreement For efficient SCell activation, the earliest slot for a UE to receive a triggered temporary RS is the reference slot (i.e., the last DL slot of the to-be-activated Scell overlapping with slot n+k as defined in 38.213 sub-clause 4.3). Conclusion For the purpose of designing temporary RS for Scell activation, RAN1 will not discuss for the case where a gNB may assume the to-be-activated SCell with assistance of temporary RS is a known SCell for a UE but it is actually unknown SCell from the UE side during the SCell activation duration. Agreement For to-be-activated SCell, if any BWP ID is configured as part of temporary RS(s) configuration, the value of the BWP ID is expected to be equal to firstActiveDownlinkBWP-Id; Agreement To trigger temporary RS, - MAC-CE at least provides the following information: - temporary RSs are to be triggered on X out of Y (Y≥X) to-be-activated SCells, respectively, while no temporary RS is to be triggered on the other to-be-activated SCells. - The following information can be provided by RRC for temporary RS for each SCell - The number of RS bursts and the gap length between the RS bursts (Opt 2.3.3) - Triggering offset of temporary RS (Opt 2.3.4) - Triggering offset can be provided, e.g., by reusing existing CSI-RS framework - QCL information (Opt 2.3.5) - Triggering QCL information can be provided, e.g., by reusing existing CSI-RS framework - A unique temporary RS configuration index - FFS: the maximum number of temporary RS per cell/per UE Note: Reusing A-TRS triggering framework is not precluded. - Information for 0, 1, or more temporary RS can be provided for each configured SCell Agreement - For triggering temporary RS, down-select based on the following alternatives, or let RAN2 be aware the status of this discussion - Alt 1: Bitmap approach in MAC-CE similar to SCell activation - Every Z-bit block in the bitmap corresponds to a SCell, Z>=0 - A Z-bit block indicates the temporary RS [configuration index], and a value zero indicated by the bit block means no RS resource transmitted. - The to-be-activated SCell is indicated via the C values in the legacy SCell activation/de-activation MAC CE or in the new MAC-CE - Alt 2: Reuse A-TRS triggering framework - A trigger state is indicated by the MAC-CE explicitly - The association between a trigger state and aperiodic temporary RS for one or multiple SCells is configured by RRC according Rel-16 A-TRS triggering framework - SCell ID is configured as a part of the temporary RS configuration. Some SCell IDs derived from the trigger state triggered by the new MAC-CE may not refer to to-be-activated SCells that are indicated by the new MAC-CE or the legacy SCell activation/de-activation MAC-CE - FFS: The value zero of the MAC-CE indication means no temporary RS is triggered by the MAC-CE for all to-be-activated SCells - Note: The down-selection targets at a RAN1 consensus on MAC-CE functionality and the list of RRC parameters for this feature. Any MAC-CE signaling design above are reference concept, its final MAC-CE signaling design is up to RAN2.

In this contribution, we provide our views on the remaining issues of temporary RS design for fast SCell activation.

# Discussions

The SCell activation delay generically defined as follows: for an activation command received in slot n, the UE shall have completed the activation at latest by slot n + THARQ + Tactivation_time + TCSI_Reporting. The main contributor on delay of the SCell activation delay comes from the Tactivation_time for both FR1 and FR2, which includes AGC settling and fine synchronization time. In case the SCell is unknown, it also includes the cell detection time. In case of FR2, it may further include the L1-RSRP measurement with beam management and reporting time. Tactivation_time = TSMTC_SCell + 5msec. TSMTC_SCell is the SMTC periodicity of SCell being activated and the value can be configured from 5ms to 160ms or more. Meanwhile, a typical value for TSMTC_SCell is 20 msec which is the default SS/PBCH periodicity used in initial access. THARQ is the timing between DL data transmission and acknowledgement, i.e., k1 as defined in RAN1. TCSI_reporting is the delay including uncertainty of the timing of CSI-RS transmission, UE processing time for CSI reporting and uncertainty of UL resource for CSI feedback.

The temporary RS is agreed to be aperiodic tracking RS (TRS). According to RAN4 inputs, the required number of temporary RS burst can be either one or two depending on whether the SCell to be activated is known or not, the SCell measurement cycle and FR1/FR2 differentiation. The following parameters will be configured for temporary RS:

  - The number of RS bursts and the gap length between the RS bursts

  - Triggering offset of temporary RS

  - QCL information

Two alternatives to trigger temporary RS transmission were identified. Alt 1 is a bitmap-based scheme. The MAC CE triggering temporary RS transmission can respectively indicate the configuration of temporary RS of each SCell to be activated. Therefore, the total overhead in the MAC CE is b∙N, where b is the number of bits per SCell, e.g., 2 bits, N is the total number of SCells to be activated. N can be up to 15. On the other hand, Alt 2 is to reuse the existing scheme for aperiodic TRS. In general, multiple trigger states of temporary RS for the SCells to be activated are configured by RRC signalling. Then, one trigger state can be indicated by the MAC CE triggering fast SCell activation and temporary RS transmission. Alt 2 is preferred since it maximizes reusing existing procedure and saves overhead of MAC CE

Proposal 1: to trigger temporary RS transmission for the to be active SCell(s), Alt 2 is preferred, i.e., to reuse A-TRS triggering framework.

Temporary RS is agreed to be based on TRS. One remaining issue is whether CSI measurement is enhanced in the fast SCell activation procedure. Two solutions can be considered. The temporary RS may be enhanced to support CSI measurement. Alternatively, an A-CSI-RS transmission and A-CSI report may be triggered earlier than the existing NR timeline. Since CSI feedback is critical for efficient DL transmission, an applicable CSI report should be available during the procedure of fast SCell activation. The first option may reuse the triggering scheme of temporary RS, but it changes the construction of temporary RS. On the other hand, the second option doesn’t impact temporary RS design, but it needs a clarification whether/how to trigger A-CSI during the SCell activation procedure.

Proposal 2: CSI measurement should be supported in the fast SCell activation procedure

# Conclusion

In this contribution, we provide our views on the remaining issues of temporary RS for efficient SCell activation. we make the following proposals.

Proposal 1: to trigger temporary RS transmission for the to be active SCell(s), Alt 2 is preferred, i.e., to reuse A-TRS triggering framework.

Proposal 2: CSI measurement should be supported in the fast SCell activation procedure

# References

[1] RP-201040, Revised WID on Further Multi-RAT Dual-Connectivity enhancements, Huawei