3GPP TSG RAN WG1 Meeting #106bis-e			               R1- 2109648

e-Meeting, October 11th – October 19th, 2021

Source:	Intel Corporation

Title:	Discussion on UE features for UE power saving

Agenda item:	8.17.7

Document for:	Discussion and Decision

Introduction

In this contribution, we express our views on UE features regarding paging enhancements, TRS/CSI-RS occasions in idle/inactive mode, and PDCCH monitoring adaptation in active time.

# Discussion on UE features for UE power saving

Here, we provide our views based on the preliminary version of RAN1 UE feature list [1].

Paging enhancements

For the FG 29-1, we have the following suggestions:

- If UE sub-grouping information is only carried via PEI, then it is fine to group support of PEI and UE subgrouping indication under a common FG.

  - If paging DCI is also supported for UE subgrouping information then, a separate FG would be needed

- Capture that support of PEI is based on a DCI format

- We do not agree the description in the column “Consequence if the feature is not supported by the UE”. It is sufficient to say “UE does not support PEI and UE sub grouping indication” as consequence.

- “Note” column should capture the following

  - maximum number of sub-groups per PO can be eight

  - Behv-A for PEI detection

Observation 1: If UE sub-grouping information is only carried via PEI, then support of PEI and UE subgrouping indication can be grouped under a common FG.

Observation 2: If paging DCI is also supported for UE subgrouping information then, a separate FG would be needed.

TRS/CSI-RS occasions

For the FG 29-2, we have the following suggestions

- Update component description as “Support receiving L1 indication for TRS availability via paging DCI”. It is expected that paging DCI based indication would be default.

  - If PEI based availability indication is agreed/supported, a separate FG can be created such as FG 29-2A where FG 29-2 can be prerequisite.

Observation 3: If PEI based availability indication is agreed/supported, a separate FG can be created such as FG 29-2A where FG 29-2 can be prerequisite.

PDCCH monitoring adaptation in active time

For PDCCH monitoring adaptation FG 29-3, we suggest revising component description as follows:

- Support PDCCH monitoring adaptation by scheduling DCI formats, where up to Y bits can be configured in a field

  - In Note column, we could mention  Y can be 1 or 2

  - Another note can be added as PDCCH based monitoring adaptation is applied to USS and type-3 CSS 2.

- Support of PDCCH monitoring adaptation behaviour [1], 1A, 2, 2A, [2B]

- Support X number of SSSGs

  - Note column could capture that X can be 1, 2, or 3

- Support Indication of Beh 1A when SSSG(s) are not configured.

- [Support Indication of Beh 1A for current SSSG when two SSSG(s) are configured]

# Conclusions

In summary, we have following observations and suggestions to update the feature lists table for FGs 29-1/2/3.

Observation 1: If UE sub grouping information is only carried via PEI, then support of PEI and UE subgrouping indication can be grouped under a common FG.

Observation 2: If paging DCI is also supported for UE subgrouping information then, a separate FG would be needed

Observation 3: If PEI based availability indication is agreed/supported, a separate FG can be created such as FG 29-2A where FG 29-2 can be prerequisite.

Proposal: Update the feature descriptions for Rel-17 UE Power Saving as follows

- FG 29-1

  - Capture that support of PEI is based on a DCI format

  - It is sufficient to say “UE does not support PEI and UE sub grouping indication” as consequence of not supporting the feature.

  - In Note column, add

    - maximum number of sub-groups per PO can be eight

    - Behv-A for PEI detection

- FG 29-2

  - Update component description as “Support receiving L1 indication for TRS availability via paging DCI” based on the assumption that paging DCI based indication would be default.

- FG 29-3

  - Support PDCCH monitoring adaptation by scheduling DCI formats, where up to Y bits can be configured in a field

    - In Note column, add

      - Y can be 1 or 2

      - PDCCH based monitoring adaptation is applied to USS and type-3 CSS 2.

  - Support of PDCCH monitoring adaptation behaviour [1], 1A, 2, 2A, [2B]

  - Support X number of SSSGs

    - In Note column, add that X can be 1, 2, or 3

  - Support Indication of Beh 1A when SSSG(s) are not configured.

  - [Support Indication of Beh 1A for current SSSG when two SSSG(s) are configured]

References

[1]    R1-2108679, “Preliminary RAN1 UE features list for Rel-17 NR”, Moderators (AT&T, NTT DOCOMO, INC.)