## NR Multicast and Broadcast Services

Please refer to RP-201038 for detailed scope of the WI.

Please also refer to section 5.1 of RP-212559 for additional guidance for this e-meeting.

[106bis-e-R17-RRC-MBS] Email discussion on Rel-17 RRC parameters for NR MBS – Fei (CMCC)

- 1st check point: October 14

- Final check point: October 19

### Mechanisms to support group scheduling for RRC_CONNECTED UEs

[106bis-e-NR-MBS-01] Email discussion/approval on mechanisms to support group scheduling for RRC_CONNECTED UEs with checkpoints for agreements on October 14 and 19 – Fei (CMCC)

R1-2108723	Resource configuration and group scheduling for RRC_CONNECTED UEs	Huawei, HiSilicon, CBN

R1-2108804	Group Scheduling Aspects for Connected UEs	FUTUREWEI

R1-2108851	Discussion on Mechanisms to Support Group Scheduling for RRC_CONNECTED UEs	ZTE

R1-2108926	Discussion on MBS group scheduling for RRC_CONNETED UEs	Spreadtrum Communications

R1-2109001	Discussion on mechanisms to support group scheduling for RRC_CONNECTED Ues	vivo

R1-2109067	Discussion on group Scheduling mechanism for RRC_CONNECTED UEs	OPPO

R1-2109135	Discussion on Group Scheduling Mechanisms for RRC_CONNECTED Ues	NEC

R1-2109194	Discussion on group scheduling mechanism for RRC_CONNECTED UEs in MBS	CATT

R1-2109303	Discussion on group scheduling mechanisms	CMCC

R1-2109316	Group Scheduling Mechanisms to Support 5G Multicast / Broadcast Services for RRC_CONNECTED Ues	Nokia, Nokia Shanghai Bell

R1-2109387	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UE	Xiaomi

R1-2109515	Support of group scheduling for RRC_CONNECTED UEs	Samsung

R1-2109538	On group scheduling mechanism for RRC_CONNECTED UEs	Lenovo, Motorola Mobility

R1-2109567	Discussion on NR MBS group scheduling for RRC_CONNECTED UEs	MediaTek Inc.

R1-2109633	NR-MBS Group Scheduling for RRC_CONNECTED UEs	Intel Corporation

R1-2109701	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	NTT DOCOMO, INC.

R1-2109767	Group scheduling related questions for RRC_CONNECTED UEs	TD Tech

R1-2109823	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	FGI, Asia Pacific Telecom

R1-2109983	Support of group scheduling for RRC_CONNECTED UEs	LG Electronics

R1-2110056	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Apple

R1-2110074	Discussion on common frequency resource configuration for multicast of RRC_CONNECTED UEs	ETRI

R1-2110118	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Convida Wireless

R1-2110210	Views on group scheduling for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2110249	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Google Inc.

R1-2110255	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	ASUSTeK

R1-2110355	Mechanisms to support MBS group scheduling for RRC_CONNECTED Ues	Ericsson

R1-2109302	Summary#1 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

R1-2110465	Summary#2 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

R1-2110466	Summary#3 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

R1-2110467	Summary#4 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

R1-2110543	Summary#5 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

R1-2110544	Summary#6 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

R1-2110637	Final summary on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

Agreement:

The starting PRB and the length of PRBs of CFR are jointly indicated reusing the RIV indication mechanism in the same way as locationAndBandwidth of a BWP.

Agreement:

RBG and PRG for multicast GC-PDSCH in CFR are defined using the same procedure as for unicast PDSCH in DL BWP.

      - For RBG, the size is defined based on the starting PRB of the CFR, size of the CFR and the higher layer parameter rbg-Size configured by PDSCH-Config for multicast in the CFR.

      - For PRG, the size is defined based on the starting PRB of the CFR, size of the CFR and precoding granularity for multicast which can be equal to one of the values among {2, 4, wideband}.

      - Note: Whether the RBG and PRG size for multicast (configured directly or indirectly) is the same as for unicast can be discussed separately.

Agreement:

The number of CFRs for multicast is no more than one per dedicated unicast BWP in Rel-17.

Agreement:

For LBRM and TBS determination for GC-PDSCH, the default value of the maximum number of layers is 1 if maxMIMO-Layers in PDSCH-Config for MBS in CFR is not configured.

Agreement:

For determination of maximum modulation order for LBRM and TBS determination for GC-PDSCH,

- if mcs-Table in PDSCH-Config for MBS is not configured in CFR, Table 5.1.3.1-1 in TS38.214 is used (similar as the default value in R16).

Agreement:

For multicast of RRC_CONNECTED UEs, the G-RNTI(s) is/are configured

- Opt.2: per serving cell.

- FFS G-CS-RNTI(s)

Agreement:

The ‘TPC command for scheduled PUCCH’ field is not needed for the first DCI format for multicast.

- FFS: Whether the field should be reserved or should be removed.

Agreement:

The ‘TPC command for scheduled PUCCH’ field is not needed for the second DCI format for multicast.

- FFS: Whether the field should be reserved or should be removed.

Agreement:

The first and second DCI formats for multicast can be configured in the same or different search space sets belonging to type-x CSS.

Agreement:

For FDRA determination of the first DCI format for GC-PDCCH, Option 2 is supported.

- Option 2:

  - NRBDL,BWP is given by

    - the size of CORESET 0 if CORESET 0 is configured for the cell; and

    - the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

  - For resource indication value (RIV) of downlink resource allocation type 1, the similar scheme as for the case that the DCI size for DCI format 1_0 in USS is derived from the size of DCI format 1_0 in CSS but applied to an active BWP is used.

    - If the size of CFR (i.e. ) is larger than the size of CORESET0/initial DL bandwidth part, the resource indication value (RIV) is defined as in section 5.1.2.2.2 in TS38.214, where K is the maximum value from set {1, 2, 4, 6, 8, 10, 12} which satisfies ;otherwise,

Agreement:

For GC-PDSCH scheduled with the first DCI format for multicast, RB numbering starts from the lowest RB of the CFR.

Agreement:

For initializing scrambling sequence generator for GC-PDCCH with the second DCI format for RRC_CONNECTED UEs, =0.

Agreement:

For initializing scrambling sequence generator for GC-PDSCH scheduled by the first DCI format for multicast received in Type-x CSS for RRC_CONNECTED UEs,

- equals the higher layer parameter dataScramblingIdentityPDSCH if it is configured in PDSCH-Config in a CFR used for GC-PDSCH and the RNTI equals the G-RNTI or G-CS-RNTI;  otherwise.

- corresponds to the RNTI associated with the GC-PDSCH transmission (i.e., the G-RNTI used by the scheduling GC-PDCCH, or the G-CS-RNTI used by the SPS GC-PDSCH activation PDCCH)

Agreement:

For initializing sequence generator for DMRS of GC-PDSCH,

- and are given by the higher-layer parameters scramblingID0 and scramblingID1, respectively, in the DMRS-DownlinkConfig IE if provided in PDSCH-Config in a CFR used for GC-PDSCH and the GC-PDSCH is scheduled by GC-PDCCH using the second DCI format

- is given by the higher-layer parameter scramblingID0 if provided in PDSCH-Config in a CFR used for GC-PDSCH and the GC-PDSCH is scheduled by GC-PDCCH using the first DCI format;

- otherwise;

- FFS:  is given by the DM-RS sequence initialization field, if present, in the DCI associated with the GC-PDSCH transmission if second DCI format is used, otherwise .

Agreement:

The association between a G-CS-RNTI and a SPS-Config-Multicast is indicated by the activation GC-PDCCH for SPS GC-PDSCH, i.e., a value of the HARQ process number field in a DCI format indicates an activation for a SPS GC-PDSCH configuration for multicast with a same value as provided by sps-ConfigIndex in a SPS-Config-Multicast.

Agreement:

For initializing scrambling sequence generator for GC-PDCCH with the first DCI format for RRC_CONNECTED UEs,

- equals the higher layer parameter pdcch-DMRS-ScramblingID if it is configured in the CORESET configured within CFR-Config-Multicast for the GC-PDCCH;  otherwise.

- = 0.

Agreement:

For initializing sequence generator for DMRS of GC-PDCCH with the first DCI format received in Type-x CSS for RRC_CONNECTED UEs,

- equals the higher layer parameter pdcch-DMRS-ScramblingID if it is configured in the CORESET configured within CFR-Config-Multicast for the GC-PDCCH;  otherwise.

Agreement:

Study the following options for the LBRM/TBS determination for PTP retransmission of multicast.

- Option 1: based on the LBRM/TBS determination of the PTM initial transmission using same HPID and NDI.

- Option 2: based on the LBRM/TBS determination of the legacy unicast PDSCH transmission.

### Mechanisms to improve reliability for RRC_CONNECTED UEs

[106bis-e-NR-MBS-02] Email discussion/approval on mechanisms to improve reliability for RRC_CONNECTED UEs with checkpoints for agreements on October 14 and 19 – Jinhuan (Huawei)

R1-2108724	Mechanisms to improve reliability for RRC_CONNECTED UEs	Huawei, HiSilicon, CBN

R1-2108805	Further Discussions on Reliability for RRC_CONNECTED UEs	FUTUREWEI

R1-2108852	Discussion on Mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

R1-2108927	Mechanisms to improve MBS reliability for RRC_CONNECTED UEs	Spreadtrum Communications

R1-2109002	Discussion on mechanisms to improve reliability for RRC_CONNECTED Ues	vivo

R1-2109068	UL feedback for RRC-CONNECTED UEs in MBS	OPPO

R1-2109136	Discussion on Reliability Improvements for RRC_CONNECTED UEs	NEC

R1-2109195	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT

R1-2109304	Discussion on reliability improvement	CMCC

R1-2109317	Reliability Improvements for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

R1-2109516	On mechanisms to improve reliability for RRC_CONNECTED UEs	Samsung

R1-2109539	On reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

R1-2109568	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	MediaTek Inc.

R1-2109634	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

R1-2109702	Discussion on mechanisms to improve reliability for multicast for RRC_CONNECTED UEs	NTT DOCOMO, INC.

R1-2109768	PUCCH formats for NACK-ONLY based HARQ-ACK feedback	TD Tech

R1-2109984	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

R1-2110057	Discussion on MBS reliability improvement for RRC_CONNECTED UEs	Apple

R1-2110075	Discussion on HARQ-ACK feedback mechanism for multicast of RRC_CONNECTED UEs	ETRI

R1-2110119	Discussion on reliability enhancement for RRC_CONNECTED UEs	Convida Wireless

R1-2110211	Views on UE feedback for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2110250	Discussion on MBS reliability for RRC_CONNECTED UEs	Google Inc.

R1-2110356	Mechanisms to improve reliability for RRC_CONNECTED Ues	Ericsson

R1-2110408	FL summary#1 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

R1-2110493	FL summary#2 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

R1-2110526	FL summary#3 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

R1-2110545	FL summary#4 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

R1-2110583	FL summary#5 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

R1-2110646	FL summary#6 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

Agreement:

The group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling.

Agreement:

If the group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is not configured, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling.

Agreement:

When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for HARQ-ACK feedback/CSI for unicast for the same priority or PUSCH transmission for the same priority, support UE multiplexing the NACK-only based feedback with the HARQ-ACK feedback/CSI on PUCCH or on to PUSCH by transforming NACK-only into the ACK/NACK HARQ bit.

- This applies to at least the case of the feedback addressing one TB. NACK-only based feedback for more than one TBs is to be handled separately.

- Note: When the TB is correctly decoded, the ACK will be transmitted and multiplexed with others.

- FFS: The case of PUCCH for SR.

Agreement:

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, further decide based on the following subset of alternatives (from previous agreement) with potential further down-selection:

- Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

- Alt2: Support sub-slot based PUCCH for this case.

- Alt3: Support UE transmitting more than one slot-based PUCCHs in the same PUCCH slot.

- Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

- Alt5: NACK-only bundling

Agreement:

Confirm the WA made in RAN1#106-e meeting regarding enabling/disabling HARQ-ACK feedback.

Agreement:

For group-common DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled, down-select from the following alternatives:

- Alt1: Reuse one existing field in the group-common DCI.

- Alt2: Introduce a new field in the group-common DCI.

Agreement:

For multicast SPS PDSCH without PDCCH scheduling, HARQ-ACK feedback option is configured by UE RRC signalling.

- FFS: Whether the configuration is per SPS configuration index or per G-CS-RNTI.

- Note: Whether there is a UE capability for support of NACK-only based HARQ-ACK or not will be discussed as part of UE features discussion.

Agreement:

- If configured, the pdsch-AggregationFactor for multicast dynamic scheduling is configured per G-RNTI.

- If configured, the pdsch-AggregationFactor for multicast SPS is configured per SPS-Config-Multicast.

Agreement:

For slot-level repetition for SPS GC-PDSCH for multicast RRC_CONNECTED UEs.

- Config A or Config B can be configured to UE:

  - (Config A) UE can be optionally configured with pdsch-AggregationFactor per SPS-Config-Multicast.

  - (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Multicast. If UE is configured with Config B, UE does not expect to be configured with Config A for the same SPS group-common PDSCH.

- For Config A, if pdsch-AggregationFactor in SPS-Config-Multicast is not configured, default value is

  - Alt1: equal to 1.

Agreement:

For UE supporting both ACK/NACK based and NACK-only based feedback for multicast, for the same G-RNTI, support the following

- UE can be configured with either ACK/NACK based or NACK-only feedback for a single G-RNTI.

  - Note: Case1-1: if configured with ACK/NACK based feedback, UE can be optionally configured a separate PUCCH-Config/PUCCH-ConfigurationList for multicast. Otherwise, PUCCH-Config/PUCCH-ConfigurationList for unicast applies (This has been agreed.)

  - Case 1-2: if configured with NACK-only based feedback, when separate PUCCH-Config/PUCCH-ConfigurationList for NACK-only is not configured, PUCCH-Config/PUCCH-ConfigurationList for unicast applies.

Agreement:

- For the priority index for the first DCI format for GC-PDCCH, support the following Alt2 from the previous agreement:

  - Alt2: Always low priority, i.e., the priority index is not included in the DCI format.

Agreement:

For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions can be configured between the following alternatives from the previous agreement:

- Alt 1:

  - for slot timing values  in the intersection of  set for unicast (termed set A) and  set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values  in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values  in set B but not in set A, based on PDSCH TDRA set for multicast.

- Alt 2: for slot timing values  in the union of  set for unicast and  set for multicast, based on the union of the PDSCH TDRA sets.

- Support of Alt. 1 is a UE capability

Agreement:

For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to the following Alt1 from the previous agreement:

- Alt.1: The last DCI for unicast

- FFS: Any details when last DCI is missed by the UE if it is necessary to make them different from current specifications for this case.

### Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs

[106bis-e-NR-MBS-03] Email discussion/approval on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs with checkpoints for agreements on October 14 and 19 – David (BBC)

R1-2108725	Discussion on UE receiving broadcast in RRC IDLE/INACTIVE state	Huawei, HiSilicon, CBN

R1-2108806	MBS Support for RRC IDLE/INACTIVE UEs	FUTUREWEI

R1-2108853	Discussion on basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs	ZTE

R1-2108928	Basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs	Spreadtrum Communications

R1-2109003	Remaining issues on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE Ues	vivo

R1-2109069	Discussion on basic functions for RRC_IDLE/RRC_INACTIVE UEs	OPPO

R1-2109196	Discussion on basic functions for broadcast multicast for RRC_IDLE RRC_INACTIVE UEs	CATT

R1-2109305	Discussion on NR MBS in RRC_IDLE/ RRC_INACTIVE states	CMCC

R1-2109318	Basic Functions for Broadcast / Multicast for  RRC_IDLE / RRC_INACTIVE Ues	Nokia, Nokia Shanghai Bell

R1-2109517	On basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	Samsung

R1-2109540	Basic functions for broadcast/multicast in idle/inactive states	Lenovo, Motorola Mobility

R1-2109569	Discussion on MBS for RRC_IDLE/INACTIVE UEs	MediaTek Inc.

R1-2109635	NR-MBS for RRC_IDLE/INACTIVE UEs	Intel Corporation

R1-2109703	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	NTT DOCOMO, INC.

R1-2109769	Further discussion on basic functions for RRC_IDLE/RRC_INACTIVE UEs	TD Tech, Chengdu TD Tech

R1-2109802	Considerations on MBS functions for RRC_IDLE/INACTIVE UEs	Sony

R1-2109985	Basic function for broadcast/multicast	LG Electronics

R1-2110058	Discussion on MBS for RRC_IDLE and RRC_INACTIVE UEs	Apple

R1-2110120	Discussion on MBS for RRC_IDLE/RRC_INACTIVE UEs	Convida Wireless

R1-2110212	Views on group scheduling for Broadcast RRC_IDLE/INACTIVE UEs	Qualcomm Incorporated

R1-2110251	Discussion on MBS for RRC_IDLE/RRC_INACTIVE UEs	Google Inc.

R1-2110258	Discussion on basic functions for broadcast or multicast for RRC_IDLE and RRC_INACTIVE UEs	ASUSTeK

R1-2110357	Support for NR multicast reception in RRC Inactive/Idle	Ericsson

R1-2109388	Discussion on basic functions for broadcastmulticast for RRC_IDLERRC_INACTIVE UEs	Xiaomi

R1-2110409	Feature Lead summary #1 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/RRC_INACTIVE states	Moderator (BBC)

R1-2110410	Feature Lead summary #2 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/RRC_INACTIVE states	Moderator (BBC)

R1-2110411	Feature Lead summary #3 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/RRC_INACTIVE states	Moderator (BBC)

R1-2110412	Feature Lead summary #4 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/RRC_INACTIVE states	Moderator (BBC)

R1-2110595	Feature Lead summary #5 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/RRC_INACTIVE states	Moderator (BBC)

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, both searchSpace#0 and common search space other than searchSpace#0 can be configured for GC-PDCCH scheduling MTCH.

Agreement:

The PDCCH/PDSCH parameters for broadcast reception with GC-PDCCH/PDSCH, which are not configured, use as default the value of the PDCCH/PDSCH parameters for the configuration of the Rel-15/Rel-16 initial BWP for RRC_IDLE/RRC_INACTIVE UEs.

Agreement:

For initializing scrambling sequence generator for GC-PDCCH for MCCH/MTCH for broadcast,

- equals the higher layer parameter pdcch-DMRS-ScramblingID if it is configured in a CFR used for the GC-PDCCH for MCCH/MTCH;  otherwise.

Agreement:

For broadcast reception with UEs in RRC_IDLE/INACTIVE states, support slot-level repetition for MTCH.

Agreement:

For initializing scrambling sequence generator for GC-PDSCH for MCCH/MTCH for broadcast,

- equals the higher layer parameter dataScramblingIdentityPDSCH if it is configured in a CFR used for GC-PDSCH for MCCH/MTCH and the RNTI equals the G-RNTI or MCCH-RNTI;  otherwise.

- corresponds to the RNTI associated with the GC-PDSCH transmission.

Agreement:

For initializing sequence generator for DMRS of GC-PDCCH for MCCH/MTCH for broadcast,

- equals the higher layer parameter pdcch-DMRS-ScramblingID if it is configured in a CFR used for the GC-PDCCH for MCCH/MTCH;  otherwise.

Agreement:

For initializing sequence generator for DMRS of GC-PDSCH for MCCH/MTCH for broadcast,

- equals the higher-layer parameters scramblingID0 if it is configured in the DMRS-DownlinkConfig IE in a CFR used for GC-PDSCH for MCCH/MTCH;  otherwise.

Working assumption:

Alt 2 (from previous agreement) is supported for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs for the notification of MCCH configuration changes.

- Send an LS to RAN2 with the mechanism agreed in RAN1

Proposal:

For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state.

- Support Case D

- Working assumption: Support Case E

- Note: Case D and E are defined in previous agreements

Objection sustained by: Lenovo, CMCC, Xiaomi, Spreadtrum, Oppo

Proposal:

For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state.

- Support Case D

- FFS: Support Case E

- Note: Case D and E are defined in previous agreements

Objection sustained by: NSB, ZTE, vivo, LG, Qualcomm

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs for broadcast reception, MTCH scheduling is associated with a window defined by the MTCH monitoring periodicity and the starting of the periodicity

- FFS: the window is associated to one or multiple or all G-RNTI.

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs for broadcast reception, at least support that within the MTCH scheduling window, the association between the PDCCH monitoring occasions and SSB is defined as:

- the [x×N+K]th PDCCH monitoring occasion(s) for MTCH in the scheduling window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in MTCH transmission window/N).

- For the purpose of associating PDCCH monitoring occasion for MTCH and SSB, the UE assumes that, in the MTCH scheduling window, PDCCH for an MTCH scrambled by G-RNTI is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB.

### Others

R1-2108854	Consideration on potential further enhancement for MBS	ZTE

R1-2109004	Other issues for Rel-17 MBS	vivo

R1-2109197	Discussion on other issues in Rel-17 MBS	CATT

R1-2109319	Other Aspects for Rel-17 MBS	Nokia, Nokia Shanghai Bell

R1-2109389	Discussion on remaining issues for idle and inactive UE	Xiaomi

R1-2109742	Impact from MCCH and MTCH on broadcast reception	Huawei, HiSilicon

R1-2109770	Discussion on typical configuration for NR MBS	CHENGDU TD TECH LTD.

R1-2109986	Other aspects for MBS	LG Electronics

R1-2110358	Assumptions for performance evaluations of NR-MBS	Ericsson