3GPP TSG RAN WG1 #106bis-e		R1-2108889

e-Meeting, October 11th – 19th, 2021

Title: 	Performance Evaluation Results for XR

Source: 	ZTE, Sanechips

Agenda item:	8.14.1

Document for:	Discussion and decision

Introduction

In 3GPP TSG RAN1#106-e meeting, initial conclusions for XR evaluation results are given in [1].

In this paper, based on [1], for capacity performance analysis, baseline results with some supplements have been provided firstly. Moreover, the impacts due to data rate, PER, PDB and multiple streams have been discussed. Last but not least, evaluation results for potential enhancement scheme, e.g. finer preemption indication for XR coexistence are further provided. For power performance, evaluation results of eCDRX for both joint UL&DL and DL only are provided.

Capacity

Baseline capacity performance

In RAN1#106-e meeting, we provided evaluation results for baseline traffic model, e.g. including {AR/VR, 30Mbps} and {CG, 30Mbps}. In order to enrich the simulation results, we have supplied evaluation results for optional traffic model, e.g. {AR/VR, 45Mbps}. The simulation parameters for FR1 DL are listed in Table A.1, simulation parameters for FR1 UL are listed in Table A.2, while simulation parameters for FR2 DL are listed in Table A.3.

FR1 InH DL

| - Capacity | - RU |
| --- | --- |

Figure 1 Capacity simulation results in indoor Hotspots scenario ([10.5, 150, 50]%).

| (a)Capacity | (b)RU |
| --- | --- |

Figure 2 Capacity simulation results in indoor Hotspots scenario ([3, 109, 91]%).

According to Figure 1, with MU-MIMO and the traffic model for [10.5, 150, 50]% relationship, the capacity performance for {AR/VR, 30Mbps}, {CG, 30Mbps} and {AR/VR, 45Mbps} in Indoor Hotspot scenario are 11.4, 12.9 and 7.2, respectively.

According to Figure 2, with MU-MIMO and the traffic model for [3, 109, 91]% relationship, the capacity performance for {AR/VR, 30Mbps}, {CG, 30Mbps} and {AR/VR, 45Mbps} in Indoor Hotspot scenario are 11.8, 13.3 and 7.3, respectively.

FR1 DU DL

| - Capacity | - RU |
| --- | --- |

Figure 3 Capacity simulation results in Dense Urban scenario ([10.5, 150, 50]%).

| (a)Capacity | (b) RU |
| --- | --- |

Figure 4 Capacity simulation results in Dense Urban scenario ([3, 109, 91]%).

According to Figure 3, with MU-MIMO and the traffic model for [10.5, 150, 50]% relationship, the capacity performance for {AR/VR, 30Mbps}, {CG, 30Mbps} and {AR/VR, 45Mbps} in Dense Urban scenario are 12.5, 14.7 and 7.8, respectively.

According to Figure 4, with MU-MIMO and the traffic model for [3, 109, 91]% relationship, the capacity performance for {AR/VR, 30Mbps}, {CG, 30Mbps} and {AR/VR, 45Mbps} in Dense Urban scenario are 13.2, 14.8 and 7.9, respectively.

FR1 Uma DL

| - Capacity | - RU |
| --- | --- |

Figure 5 Capacity simulation results in Urban Marco scenario ([10.5, 150, 50]%).

According to Figure 5, with MU-MIMO and the traffic model for [10.5, 150, 50]% relationship, the capacity performance for {AR/VR, 30Mbps}, {CG, 30Mbps} and {AR/VR, 45Mbps} in Urban Macro scenario are 10.0, 11.6 and 6.0, respectively.

FR1 InH UL

Figure 6 Capacity simulation results of pose/control traffic model in UL Indoor Hotspot scenario

According to Figure 6, with MU-MIMO, the capacity performance for {pose/control, fixed 100Bytes} in Indoor Hotspot scenario is larger than 40.

FR1 DU UL

Figure 7 Capacity simulation results of {AR, 10Mbps} in UL Dense Urban scenario

According to Figure 7, with MU-MIMO, the capacity performance for {AR, 10Mbps} in Dense Urban scenario is 10.9.

FR2 InH DL

Figure 8 Capacity simulation results in indoor Hotspots scenario

According to Figure 8, with SU-MIMO, the capacity performance for {AR/VR, 30Mbps} and {CG, 30Mbps} in Indoor Hotspot scenario are 7.8 and 9.9, respectively.

Summary

In summary, the baseline FR1 DL capacity performances for our evaluated traffic model and simulation scenario are listed as follows.

Table 1 Baseline capacity performance for FR1 DL

| Scenario Traffic | AR/VR, 30Mbps |  | CG, 30Mbps |  | AR/VR, 45Mbps |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | [10.5,150,50] | [3,109,91] | [10.5,150,50] | [3,109,91] | [10.5,150,50] | [3,109,91] |
| Indoor Hotspot | 11.4 | 11.7 | 12.9 | 13.3 | 7.2 | 7.3 |
| Dense Urban | 12.5 | 13.2 | 14.7 | 14.8 | 7.8 | 7.9 |
| Urban Marco | 10.0 | - | 11.6 | - | 6.0 | - |

For FR1 DL transmission with 99% reliability requirement, the capacity results can be summarized as follows.

| Scenario Traffic | AR/VR, 30Mbps |  | CG, 30Mbps |  | AR/VR, 45Mbps |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | [10.5,150,50] | [3,109,91] | [10.5,150,50] | [3,109,91] | [10.5,150,50] | [3,109,91] |
| Indoor Hotspot | 11.4 | 11.7 | 12.9 | 13.3 | 7.2 | 7.3 |
| Dense Urban | 12.5 | 13.2 | 14.7 | 14.8 | 7.8 | 7.9 |
| Urban Marco | 10.0 | - | 11.6 | - | 6.0 | - |

Some performance difference can be observed under different traffic model ratios.

The baseline FR1 UL capacity performances for our evaluated traffic models and simulation scenarios are listed as follows.

Table 2 Baseline capacity performance for FR1 UL

| Scenario Traffic | Pose/Control Fixed 100 Byte | AR, 10Mbps |
| --- | --- | --- |
| Indoor Hotspot | >40 | - |
| Dense Urban | - | 10.9 |

For FR1 UL transmission with 99% reliability, the capacity results can be summarized as follows.

| Scenario Traffic | Pose/Control Fixed 100 Byte | AR, 10Mbps |
| --- | --- | --- |
| Indoor Hotspot | >40 | - |
| Dense Urban | - | 10.9 |

The baseline FR2 DL capacity performances for our evaluated traffic models and simulation scenarios are listed as follows.

Table 3 Baseline capacity performance for FR2 DL

| Scenario Traffic | AR/VR, 30Mbps | CG, 30Mbps |
| --- | --- | --- |
| Indoor Hotspot | 7.8 | 9.9 |

For FR2 DL transmission with 99% reliability, the capacity results can be summarized as follows.

| Scenario Traffic | AR/VR, 30Mbps | CG, 30Mbps |
| --- | --- | --- |
| Indoor Hotspot | 7.8 | 9.9 |

Impacts on capacity performance

Impact of data rate

The impact of data rate on capacity performance is presented in Figure 9. The simulation is performed for 8Mbps, 30Mbps and 45Mbps traffic models, with the same periodicity and packet delay budget. As expected, increasing the data rate may reduce system capacity owing to the increased packet/frame size but limited bandwidth. For instance, the capacity for {8Mbps, 10ms} traffic model is larger than 15 UEs, while the capacity for {30Mbps, 10ms} traffic model is reduced to 12.4 UEs. When data rate increases to 45Mbps, the capacity is further reduced to 7.8 UEs.

Figure 9 Capacity performance comparison among traffic models with different data rates

Reducing the data rate requirement can increase system capacity.

Impact of packet delay budget

The impact of PDB is presented in Figure 10. The simulation is performed for {AR/VR, 30Mbps} and {CG, 30Mbps} traffic models. It can be observed that the relaxed PDB is capable of increasing system capacity.

| (a)Capacity | (b)RU |
| --- | --- |

Figure 10 Capacity performance & RU comparison among traffic models with different PDB

Relaxing the PDB can increase system capacity

Impact of packet error rate

The impact of PER is presented in Figure 11. The simulation is performed for {AR/VR, 30Mbps}, with different reliability requirement. According to Figure 11, stringent reliability requirement results in low system capacity, while relaxed reliability requirement can obtain impressive system capacity gain. For example, system capacity for 99.9% reliability requirement is 10.1 UEs, when the reliability requirement is relaxed to 95%, the system capacity can increase to 12.9 UEs.

As a result, network coding can be considered. In detail, some redundancy packets can be added via network coding to increase the reliability in high layer, which is capable of relaxing the PER in PHY.

Figure 11 Capacity performance comparison among 30Mbps traffic model with different PERs.

Relaxing the PER can increase system capacity.

Capacity performance under I/P frame multiple stream model

In RAN1#106-e meeting, the agreement of multiple traffic models for I/P-stream is shown as follows [1].

Agreement For evaluation of separate streams of I-frame and P-frame that is an optional evaluation scenario, - Alpha value: 2.0 and 1.5, Other values, e.g., 3.0 can be optionally evaluated - This alpha value assumption applies to both Option 1A (slice-based) and Option 1B (GOP-based) evaluations Agreement For evaluation of separate streams of I-frame and P-frame that is an optional evaluation scenario, - RAN1 agree upon the below reference case, while leaving other study cases up to companies. - Reference case - For DL - [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1 %, 10ms, 10ms] for AR/VR - [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1 %, 15ms, 15ms] for CG - For UL AR video streams - [PER_I, PER_P, PDB_I, PDB_P] = [1 %, 1 %, 30ms, 30ms]

In our simulations, we utilize alpha = 2.0 for both Option 1A (slice-based) and Option 1B (GOP-based). And we firstly present the evaluation results for reference case and then present some evaluation results for some study cases.

Group of Picture-based traffic model

For GOP-based traffic model, the combinations of different PDB and PER values are listed as follows.

Table 4 The combinations of different PDB and PER values for GOP-based traffic model

| Reference Case | [PER_I, PER_P, PDB_I, PDB_P] = [1%, 1%, 10 ms, 10 ms] |
| --- | --- |
| Case 1 | [PER_I, PER_P, PDB_I, PDB_P] = [10%, 1%, 10 ms, 10 ms] |
| Case 2 | [PER_I, PER_P, PDB_I, PDB_P] = [1%, 10%, 10 ms, 10 ms] |
| Case 3 | [PER_I, PER_P, PDB_I, PDB_P] = [1%, 5%, 10 ms, 10 ms] |

Figure 12 Capacity results of the multiple stream model Option 1B (GOP-based) with different (PER, PDB)

According to Figure 12, the capacity performance for Option1B (GOP-based) traffic model of each case is summarized as follows.

Table 5 Summary of capacity performance (GOP-based) for different cases

| [PER_I, PER_P, PDB_I, PDB_P] Combinations | Satisfied UE number per cell |
| --- | --- |
| Reference Case [1%, 1%, 10ms, 10ms] | 10.8 |
| Case 1 [10%, 1%, 10ms, 10ms] | 12.2 |
| Case 2 [1%, 10%, 10ms, 10ms] | 10.9 |
| Case 3 [1%, 5%, 10ms, 10ms] | 10.9 |

By making comparison between reference case and Case 1, the capacity increases obviously from 10.8 to 12.2, which implies that relaxing the reliability of I-frame will dramatically increase the system capacity for GOP-based traffic model. While when we make comparison among reference case, Case 2 and Case 3, the capacity slightly increases from 10.8 to 10.9. Because for GOP-based traffic model, I-frames transmission is the bottleneck of system capacity. If some improvements for I-frames, such as, network coding, in high layer can be done to ensure the reliability in high layer, the PER in PHY can be relaxed and the capacity can increase consequently.

For GOP-based traffic model, relaxing the reliability of I-frame is capable of significantly improving system capacity while relaxing the reliability of P-frame slightly improves system capacity.

Slice-based traffic model

For slice-based traffic model, the combinations of different PDB and PER values are listed as follows.

Table 6 The combinations of different PDB and PER values for slice-based traffic model

| Reference Case | [PER_I, PER_P, PDB_I, PDB_P] = [1%, 1%, 10 ms, 10 ms] |
| --- | --- |
| Case 1 | [PER_I, PER_P, PDB_I, PDB_P] = [1%, 5%, 10 ms, 10 ms] |

In our simulation, 7 P-slices are generated and scheduled as a large P packet. And the PER and PDB are defined for the large P packet.

Figure 13 Capacity results of the multiple stream model Option 1A (Slice-based) with different (PER, PDB)

According to Figure 13, the capacity performance for Option1A (Slice-based) traffic model of each case is summarized as follows.

Table 7 Summary of capacity performance (Slice-based) for different cases

| [PER_I, PER_P, PDB_I, PDB_P] Combinations | Satisfied UE number per cell |
| --- | --- |
| Reference Case [1%, 1%, 10ms, 10ms] | 12.7 |
| Case 1 [1%, 5%, 10ms, 10ms] | 14.6 |

By making comparison between reference case and Case 1, the capacity increases obviously from 12.7 to 14.6, which implies that relaxing the reliability of P-slice will dramatically increase the system capacity for slice-based traffic model.

For Slice-based traffic model, relaxing the reliability of P slice is capable of significantly improving system capacity.

Potential enhancement on capacity performance

Considering the case when the existing network starts to carry the XR service, the network is likely to encounter severe challenges. One of the unavoidable problems is XR coexistence with existing service, including, e.g. uRLLC and eMBB.

Taking coexistence between uRLLC service and XR service as an example, the available approach is to utilize preemption scheme to achieve uRLLC and XR multiplexing. The uRLLC transmission may preempt an ongoing XR transmission, i.e. puncturing the resource elements already scheduled for XR. To this end, the preemption indication is needed for indicating the preempted area to help XR UE transmission/re-transmission. For uRLLC and eMBB multiplexing, current Rel-15 preemption indication mechanism is introduced for uRLLC enhancement by indicating the preemption area of eMBB at symbol level. This mechanism may not cause heavy performance loss of eMBB, because of relatively loose latency requirement for eMBB, which allows more re-transmissions to reach the reliability constraint. However, when it comes to XR, things are different. Due to its eMBB-like large data amount and uRLLC-like stringent latency, excessive radio resources of XR are wasted by current Rel-15 preemption indication mechanism, resulting in frequent re-transmissions and large amount of XR packets dropped due to the failure to meeting the PDB.

In a word, XR is more sensitive in PDB/reliability compared with eMBB. Therefore, finer granularity preemption indication should be considered to solve the problem of Rel-15 preemption indication in uRLLC and XR multiplexing.

Finer granularity preemption indication should be considered for XR coexistence.

In our simulation, uRLLC traffic and XR traffic are considered as the two types of traffic to be transmitted in the system, where uRLLC traffic has higher priority (HP) while XR traffic has a relatively low priority (LP). According to [3-6], the parameters of two traffic models are shown as follows.

Table 8 Parameters of uRLLC and eMBB

| Traffic description | Packet size | Periodicity | PER | PDB | Priority |
| --- | --- | --- | --- | --- | --- |
| Rel-15 enabled use case (e.g. AR/VR) | Fixed 4096 byte | 2 ms | 0.1% | 7ms | 1 |
| AR/VR | 30Mbps, truncate Gaussian distribution | 16.67 ms | 1% | 10ms | 0 |

In our simulation, there are around 20% uRLLC UEs and 80% XR UEs in system. Some of uRLLC UEs are scheduled orthogonal to XR UEs by PF scheduler, while others are activated in concurrence with XR UEs.

According to traffic models and assumptions mentioned above, we consider the following cases of preemption patterns for uRLLC UEs and XR UEs multiplexing:

- Enhanced preemption indication: Precise preemption area indication

- Rel-15 preemption indication: Bitmap for indicating preempted symbols, e.g. 4 out of 14 symbols

- No preemption: Directly transmit the multiplexing TB size with inter-UE interference.

Figure 14 Illustration for different preemption pattern

The simulation parameters are listed in Table A.1. Then, the simulation results are shown as follows.

| (a)Capacity | (b)RU |
| --- | --- |

Figure 15 Capacity performance & RU comparison among Enhanced preemption indication, Rel-15 preemption indication and No preemption indication.

According to Figure 15, with MU-MIMO, the system capacity of no preemption indication is 8.5 UEs, the system capacity of Rel-15 preemption indication is 11.8 UEs, which the system capacity of enhanced preemption indication is 16.6 UEs.

The system capacity of enhanced preemption indication scheme can be improved by finer granularity indication, and the resource utilization of enhanced preemption indication scheme can be reduced by finer granularity indication.

Power consumption

As discussed in [7], power consumption is one important KPI of XR. In this section, power consumption evaluation results are provided. In line with the previous meetings’ agreement, the following assumptions are adopted in performance evaluation of power consumption.

- The power consumption results for both all UEs and only satisfied UEs are counted to comprehensively evaluate XR power consumption performance.

- Both linear interpolation method and step function with only two transmission power values are used to count UL power consumption.

  - Linear interpolation method: the power of UL slot is calculated according to the following function:

Tx_power is the Tx power values in one slot.

  - Step function: power number is given as A for X= [0, M]dBm and B for X =[M, 23]dBm, where A and B (defined in 38.840) correspond to power consumption numbers for a given uplink slot for 0dBm and 23dBm respectively, M=20dBm

- Using the power model of 0 dBm for UE with transmission power less than 0 dBm.

- The same number of UE per cell is used in baseline and power saving schemes and the results for baseline capacity are reported.

- The power consumption of CSI reporting and SRS transmission is not considered. The power models are shown in Table B.1 and B.2 in Appendix B.

- The power of “PDCCH+PDSCH+PUSCH”, “PDSCH+PUSCH” and “PDCCH+PUSCH” is the sum of corresponding DL power consumption and PUSCH power consumption. For example, the power value of “PDCCH+PUSCH” is equal to the sum of power of “PDCCH-only” and power of “PUSCH”.

In this section, the following two schemes are evaluated:

- Baseline (Always On): No power saving technique is applied, that is, UE is always in Active Time.

- Enhanced DRX mechanism (eCDRX): The enhanced CDRX is a scheme which drx-startoffset changes dynamically and UE can wake up at an additional active window during DRX off state. The details of eCDRX can be found in [7].

  - The dynamic change of drx-startoffset can be triggered periodically, i.e., drx-startoffset changes every 100ms

  - The additional active window can be triggered if UE does not receive any data during the DRX ON period, or UE receives a trigger indication.

For the evaluation of power saving gain of eCDRX, the results of both joint UL&DL and DL only are provided.

In the following power consumption evaluation, the DRX settings are captured in Table 9.

Table 9 DRX settings

|  | DRX cycle | Drx-onDurationTimer | Drx-inactivityTimer | Drx-RetransmissionTimer |
| --- | --- | --- | --- | --- |
| DRX setting 1(ms) | 16 | 6 | 3 | 3 |
| DRX setting 2(ms) | 16 | 6 | 4 | 4 |

The traffic models used for power consumption evaluation are presented in Table 10.

Table 10 Traffic models for power consumption evaluation

| Traffic Model No. | Mean (Byte) | STD (Byte) | Maximum (Byte) | Minimum (Byte) | PDB (ms) |
| --- | --- | --- | --- | --- | --- |
| TM1-1 | 62500 | 6563 | 93750 | 31250 | 10 |
| TM1-2 | 62500 | 1875 | 68125 | 56875 | 10 |
| TM2 | 62500 | 6563 | 93750 | 31250 | 15 |
| TM3 | 93750 | 9844 | 140625 | 46875 | 10 |

DL+UL simulation results

In this section, a method of obtaining a joint power consumption by independently evaluating DL and UL power consumption is used. The method includes:

- Evaluating DL and UL power consumption independently;

- Collecting DL and UL slot states respectively;

- Recombining these slot states in a single timeline;

- Calculating overall power consumption according to the recombined timeline.

Single stream of video with jitter is used in DL and pose/control traffic model is used for UL evaluation. UE can transmit UL data in UL slots even if UE is in “DRX off” state.

FR1 InH DL+UL

The power consumption results of VR traffic with 10ms PDB and UL pose traffic are shown in Table 11-13. The power consumption results of CG traffic with 15ms PDB and UL pose traffic are shown in Table 14.

Table 11 FR1 power consumption results in Indoor Hotspot scenario (TM1-1, UL pose traffic)

| Power Saving Scheme | #UEs per cell | C1=floor(Capacity) | Percentage of satisfied UE | Type of counted UEs | Mean power |  | Mean PS gain compared to Case 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Linear interpolation | Step function | Linear interpolation | Step function |
| Case 1 Always On | 11 | 11 | 93.18% | All UE | 150 | 150.2 | / | / |
|  |  |  |  | Satisfied UE | 149 | 149.2 | / | / |
|  | 10 | 11 | 93% | All UE | 151.4 | 151.6 | / | / |
|  |  |  |  | Satisfied UE | 150.3 | 150.5 | / | / |
| eCDRX: DRX setting 1 | 11 | 11 | 83% | All UE | 117.2 | 117.4 | 22.6% | 22.6% |
|  |  |  |  | Satisfied UE | 113.9 | 114 | 24.2% | 24.3% |
|  | 10 | 11 | 85.83% | All UE | 118.9 | 119.1 | 21.5% | 21.4% |
|  |  |  |  | Satisfied UE | 114.9 | 115 | 23.6% | 23.6% |
| eCDRX: DRX setting 2 | 11 | 11 | 87.12% | All UE | 117.5 | 117.7 | 21.7% | 21.6% |
|  |  |  |  | Satisfied UE | 114.8 | 115 | 23.6% | 23.6% |

Table 12 FR1 power consumption results in Indoor Hotspot scenario (TM1-2, UL pose traffic)

| Power Saving Scheme | #UEs per cell | C1=floor(Capacity) | Percentage of satisfied UE | Type of counted UEs | Mean power |  | Mean PS gain compared to Case 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Linear interpolation | Step function | Linear interpolation | Step function |
| Case 1 Always On | 11 | 11 | 93.2% | All UE | 150.5 | 150.6 | / | / |
|  |  |  |  | Satisfied UE | 149.2 | 149.3 | / | / |
|  | 10 | 11 | 93.3% | All UE | 148.9 | 149 | / | / |
|  |  |  |  | Satisfied UE | 147.9 | 148 | / | / |
| eCDRX: DRX setting 1 | 11 | 11 | 85.6% | All UE | 114.9 | 115.1 | 23.6% | 23.6% |
|  |  |  |  | Satisfied UE | 117.6 | 117.8 | 21.2% | 21.1% |
|  | 10 | 11 | 90.3% | All UE | 115.5 | 115.7 | 22.4% | 22.4% |
|  |  |  |  | Satisfied UE | 113.7 | 113.9 | 23.1% | 23% |

Table 13 FR1 power consumption results in Indoor Hotspot scenario (TM3, UL pose traffic)

| Power Saving Scheme | #UEs per cell | C1=floor(Capacity) | Percentage of satisfied UE | Type of counted UEs | Mean power |  | Mean PS gain compared to Case 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Linear interpolation | Step function | Linear interpolation | Step function |
| Case 1 Always On | 7 | 7 | 91% | All UE | 140 | 140 | / | / |
|  |  |  |  | Satisfied UE | 138.6 | 138.5 | / | / |
| eCDRX: DRX setting 2 | 7 | 7 | 86.3% | All UE | 100 | 100.1 | 28.6% | 28.5% |
|  |  |  |  | Satisfied UE | 103.8 | 103.9 | 25.1% | 25% |

According to the above simulation results, 21%-28% power saving gain can be provided by eCDRX for VR traffic. The percentages of satisfied UE decrease by 3%-10% compared with baseline capacity. And the percentages of satisfied UE decrease little if the number of UE per cell is configured less than the baseline capacity.

Comparing to UE always on, following is observed for DL video-stream (30Mbps, 10ms PDB) + UL pose/control-stream (0.2Mbps, 10ms PDB) in Indoor scenario:

- for eCDRX power saving scheme, the power saving gain is in the range of {21.1%,24.3%} for high load with{3%-10.18%} of satisfied UE loss.

Comparing to UE always on, following is observed for DL video-stream (45Mbps, 10ms PDB) + UL pose/control-stream (0.2Mbps, 10ms PDB) in Indoor scenario:

- for eCDRX power saving scheme, the power saving gain is in the range of {25%,28.6%} for high load with 4.7% of satisfied UE loss.

The difference between power consumption results for all UEs and the counterpart for only satisfied UEs is small.

Table 14 FR1 power consumption results in Indoor Hotspot scenario for CG (TM2, UL pose traffic)

| Power Saving Scheme | #UEs per cell | C1=floor(Capacity) | Percentage of satisfied UE | Type of counted UEs | Mean power |  | Mean PS gain compared to Case 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Linear interpolation | Step function | Linear interpolation | Step function |
| Case 1 Always On | 12 | 12 | 96.53% | All UE | 151.2 | 151.4 | / | / |
|  |  |  |  | Satisfied UE | 150.5 | 150.6 | / | / |
| eCDRX DRX setting 1 | 12 | 12 | 88.19% | All UE | 118.9 | 119.1 | 21.4% | 21.3% |
|  |  |  |  | Satisfied UE | 116.2 | 116.4 | 22.8% | 22.7% |

According to the simulation results for CG in Table 14, the following can be observed:

Comparing to UE always on, following is observed for DL video-stream (30Mbps, 15ms PDB) + UL pose/control-stream (0.2Mbps, 10ms PDB) in Indoor scenario:

- for eCDRX power saving scheme, the power saving gain is in the range of {21.3%,22.7%} for high load with 8.34% of satisfied UE loss.

DL simulation results

The results considering only DL states in the evaluation are provided in this section. In the DL only evaluation, the state of UL slots are all set to “sleep state”.

FR1 InH DL

The power consumption results of only DL VR traffic with 10ms PDB are shown in Table 15-17. The power consumption results of only DL CG traffic with 15ms PDB are shown in Table 18.

Table 15 FR1 power consumption results in Indoor Hotspot scenario (TM1-1 only)

| Power Saving Scheme | #UEs per cell | C1=floor(capacity) | Percentage of satisfied UE | Type of counted UEs | Mean power | Mean PS gain compared to Case 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 Always On | 11 | 11 | 93.18% | All UE | 119.2 | / |
|  |  |  |  | Satisfied UE | 118.2 | / |
|  | 10 | 11 | 93% | All UE | 120.6 | / |
|  |  |  |  | Satisfied UE | 119.5 | / |
| eCDRX: DRX setting 1 | 11 | 11 | 83% | All UE | 79.8 | 33.1% |
|  |  |  |  | Satisfied UE | 77 | 34.8% |
|  | 10 | 11 | 85.83% | All UE | 81.6 | 32.3% |
|  |  |  |  | Satisfied UE | 78 | 34.7% |
| eCDRX: DRX setting 2 | 11 | 11 | 87.12% | All UE | 84.7 | 29% |
|  |  |  |  | Satisfied UE | 82.9 | 30% |

Table 16 FR1 power consumption results in Indoor Hotspot scenario (TM1-2 only)

| Power Saving Scheme | #UEs per cell | C1=floor(capacity) | Percentage of satisfied UE | Type of counted UEs | Mean power | Mean PS gain compared to Case 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 Always On | 11 | 11 | 93.2% | All UE | 119.6 | / |
|  |  |  |  | Satisfied UE | 118.4 | / |
|  | 10 | 11 | 93.3% | All UE | 118 | / |
|  |  |  |  | Satisfied UE | 117 | / |
| eCDRX: DRX setting 1 | 11 | 11 | 85.6% | All UE | 80.2 | 32.9% |
|  |  |  |  | Satisfied UE | 77.2 | 34.8% |
|  | 10 | 11 | 90.3% | All UE | 77.8 | 34.1% |
|  |  |  |  | Satisfied UE | 76.7 | 34.4% |

Table 17 FR1 power consumption results in Indoor Hotspot scenario (TM3 only)

| Power Saving Scheme | #UEs per cell | C1=floor(capacity) | Percentage of satisfied UE | Type of counted UEs | Mean power | Mean PS gain compared to Case 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 Always On | 7 | 7 | 91% | All UE | 113.4 | / |
|  |  |  |  | Satisfied UE | 112.2 | / |
| eCDRX: DRX setting 2 | 7 | 7 | 86.3% | All UE | 79.7 | 29.7% |
|  |  |  |  | Satisfied UE | 76 | 32.3% |

Table 18 FR1 power consumption results in Indoor Hotspot scenario for CG (TM2 only)

| Power Saving Scheme | #UEs per cell | C1=floor(capacity) | Percentage of satisfied UE | Type of counted UEs | Mean power | Mean PS gain compared to Case 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 Always On | 12 | 12 | 96.53% | All UE | 120.4 | / |
|  |  |  |  | Satisfied UE | 119.64 | / |
| eCDRX: DRX setting 1 | 12 | 12 | 88.19% | All UE | 81.4 | 32.4% |
|  |  |  |  | Satisfied UE | 78.4 | 34.5% |

According to the power evaluation results in Table 15-18, eCDRX can provide 29%-34% power saving gain with 3%-10% capacity loss. As the UL slots are all set to “sleep states” in DL only evaluation, the power saving gain for DL only evaluation is greater than the gain obtained in DL+UL evaluation.

Comparing to UE always on, following is observed for VR/AR, 30Mbps, 10ms PDB, 60 FPS in Indoor scenario:

- for eCDRX power saving scheme, the power saving gain is in the range of {29%, 34.8%} for high load with {3%, 10.18%} of satisfied UE loss.

Comparing to UE always on, following is observed for VR/AR, 45Mbps, 10ms PDB, 60 FPS in Indoor scenario:

- for eCDRX power saving scheme, the power saving gain is in the range of {29.7%, 32.3%} for high load with 4.7% of satisfied UE loss.

Comparing to UE always on, following is observed for CG, 30Mbps, 15ms PDB, 60 FPS in Indoor scenario:

- for eCDRX power saving scheme, the power saving gain is in the range of {32.4%, 34.5%} for high load with 8.34% of satisfied UE loss.

FR1 DU DL

Table 19 FR1 power consumption results in Dense urban scenario (TM3 only)

| Power Saving Scheme | #UEs per cell | C1=floor(capacity) | Percentage of satisfied UE | Type of counted UEs | Mean power | Mean PS gain compared to Case 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 Always On | 7 | 7 | 96.6% | All UE | 109.8 | / |
|  |  |  |  | Satisfied UE | 109.2 | / |
| eCDRX: DRX setting 2 | 7 | 7 | 90% | All UE | 77 | 29.9% |
|  |  |  |  | Satisfied UE | 75.2 | 31.2% |

Comparing to UE always on, following is observed for VR/AR, 45Mbps, 10ms PDB, 60 FPS in dense urban scenario:

- for eCDRX power saving scheme, the power saving gain is in the range of {29.9%, 31.23%} for high load with 6.6% of satisfied UE loss.

Conclusion

In this contribution, capacity results for single stream, multiple stream, power consumption results and preemption scheme for XR enhancement are presented, and we have the following observations:

Observation 1: For FR1 DL transmission with 99% reliability requirement, the capacity results can be summarized as follows.

| Scenario Traffic | AR/VR, 30Mbps |  | CG, 30Mbps |  | AR/VR, 45Mbps |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | [10.5,150,50] | [3,109,91] | [10.5,150,50] | [3,109,91] | [10.5,150,50] | [3,109,91] |
| Indoor Hotspot | 11.4 | 11.7 | 12.9 | 13.3 | 7.2 | 7.3 |
| Dense Urban | 12.5 | 13.2 | 14.7 | 14.8 | 7.8 | 7.9 |
| Urban Marco | 10.0 | - | 11.6 | - | 6.0 | - |

Observation 2: Some performance difference can be observed under different traffic model ratios.

Observation 3: For FR1 UL transmission with 99% reliability, the capacity results can be summarized as follows.

| Scenario Traffic | Pose/Control Fixed 100 Byte | AR, 10Mbps |
| --- | --- | --- |
| Indoor Hotspot | >40 | - |
| Dense Urban | - | 10.9 |

Observation 4: For FR2 DL transmission with 99% reliability, the capacity results can be summarized as follows.

| Scenario Traffic | AR/VR, 30Mbps | CG, 30Mbps |
| --- | --- | --- |
| Indoor Hotspot | 7.8 | 9.9 |

Observation 5: Reducing the data rate requirement can increase system capacity.

Observation 6: Relaxing the PDB can increase system capacity

Observation 7: Relaxing the PER can increase system capacity.

Observation 8: For GOP-based traffic model, relaxing the reliability of I-frame is capable of significantly improving system capacity while relaxing the reliability of P-frame slightly improves system capacity.

Observation 9: For Slice-based traffic model, relaxing the reliability of P slice is capable of significantly improving system capacity.

Observation 10: Finer granularity preemption indication should be considered for XR coexistence.

Observation 11: The system capacity of enhanced preemption indication scheme can be improved by finer granularity indication, and the resource utilization of enhanced preemption indication scheme can be reduced by finer granularity indication.

Observation 12: Comparing to UE always on, following is observed for DL video-stream (30Mbps, 10ms PDB) + UL pose/control-stream (0.2Mbps, 10ms PDB) in Indoor scenario:

- for eCDRX power saving scheme, the power saving gain is in the range of {21.1%,24.3%} for high load with{3%-10.18%} of satisfied UE loss.

Observation 13: Comparing to UE always on, following is observed for DL video-stream (45Mbps, 10ms PDB) + UL pose/control-stream (0.2Mbps, 10ms PDB) in Indoor scenario:

- for eCDRX power saving scheme, the power saving gain is in the range of {25%,28.6%} for high load with 4.7% of satisfied UE loss.

Observation 14: The difference between power consumption results for all UEs and the counterpart for only satisfied UEs is small.

Observation 15: Comparing to UE always on, following is observed for DL video-stream (30Mbps, 15ms PDB) + UL pose/control-stream (0.2Mbps, 10ms PDB) in Indoor scenario:

- for eCDRX power saving scheme, the power saving gain is in the range of {21.3%,22.7%} for high load with 8.34% of satisfied UE loss.

Observation 16: Comparing to UE always on, following is observed for VR/AR, 30Mbps, 10ms PDB, 60 FPS in Indoor scenario:

- for eCDRX power saving scheme, the power saving gain is in the range of {29%, 34.8%} for high load with {3%, 10.18%} of satisfied UE loss.

Observation 17: Comparing to UE always on, following is observed for VR/AR, 45Mbps, 10ms PDB, 60 FPS in Indoor scenario:

- for eCDRX power saving scheme, the power saving gain is in the range of {29.7%, 32.3%} for high load with 4.7% of satisfied UE loss.

Observation 18: Comparing to UE always on, following is observed for CG, 30Mbps, 15ms PDB, 60 FPS in Indoor scenario:

- for eCDRX power saving scheme, the power saving gain is in the range of {32.4%, 34.5%} for high load with 8.34% of satisfied UE loss.

Observation 19: Comparing to UE always on, following is observed for VR/AR, 45Mbps, 10ms PDB, 60 FPS in dense urban scenario:

- for eCDRX power saving scheme, the power saving gain is in the range of {29.9%, 31.23%} for high load with 6.6% of satisfied UE loss.

References

“RAN1 Chair’s Notes”, 3GPP TSG RAN WG1 #106-e, e-Meeting, August 16th - 27th, 2021.

R1-2108890, “Remaining Issues on XR Traffic and Evaluation Methodology”, ZTE, Sanechips, 3GPP TSG RAN WG1 #106bis-e, e-Meeting, October 11th - 19th, 2021.

3GPP TR38.824 V16.0.0, “Study on physical layer enhancements for NR ultra-reliable and low-latency case (URLLC) (Release 16)”

R1-1901250, “Baseline performance achievable with Rel-15 URLLC for R15 enabled cases”, Huawei, HiSilicon, 3GPP TSG RAN WG1 Ad-Hoc Meeting 1901, Taipei, 21st - 25th January, 2019.

R1-1900079, “Performance evaluation for Rel-15 enabled use cases”, ZTE, Sanechips, 3GPP TSG RAN WG1 Ad-Hoc Meeting 1901, Taipei, 21st - 25th January, 2019.

R1-1900976, “Views and evaluations for URLLC scenarios”, NTT DOCOMO, INC. 3GPP TSG RAN WG1 Ad-Hoc Meeting 1901, Taipei, 21st - 25th January, 2019.

R1-2108891, “On Capacity and Power Working Areas for XR”, ZTE, Sanechips, 3GPP TSG RAN WG1 #106bis-e, e-Meeting, October 11th - 19th, 2021.

- Appendix

- A Simulation Parameters

Table A.1 simulation parameters for FR1 DL

| Parameters | Value |  |  |
| --- | --- | --- | --- |
| Scenario | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m | Scenario-2: Dense Urban hexagonal layout with 7, 3 Sectors | Scenario-3: Urban Macro hexagonal layout with 7, 3 Sectors |
| Inter-BS distance | 20m | 200m | 500m |
| Carrier frequency | 4 GHz |  |  |
| Duplex Mode / Simulation bandwidth | 100 MHz |  |  |
| SCS | 30KHz |  |  |
| TDD pattern | DDDSU |  |  |
| BS Antenna Configuration | 32 Tx antenna ports, and (dH, dV) = (0.5λ, 0.5λ); 32TX:(M, N, P, Mg, Ng; Mp, Np) = (4,4, 2, 1, 1; 4, 4); The antenna tilt is 90 degrees. | 64 Tx antenna ports, and (dH, dV) = (0.5λ, 0.5λ); 64TX:(M, N, P, Mg, Ng; Mp, Np) = (8,8, 2, 1, 1; 4, 8); The antenna tilt is 12 degrees. |  |
| UE Antenna Configuration | 4 Rx antenna ports, and Panel model 1: dH = 0.5λ 4 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1; 1,2); | 4 Rx antenna ports, and Panel model 1: dH = 0.5λ 4 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1; 1,2); |  |
| Transmit Power | 24 dBm per 20M at TRP, and 23 dBm at UE | 44 dBm per 20M at TRP, and 23 dBm at UE |  |
| BS Height | 3 m | 25 m |  |
| UE Height | 1.5m | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |  |
| Antenna Element Gain | 5dBi for BS and 0 dBi for UE | 8dBi for BS and 0 dBi for UE |  |
| Receiver Noise Figure | 5 dB for BS and 9 dB for UE |  |  |
| UE distribution | 100% of users are indoor Use 3km/h for modeling fading channel | 80% of users are indoor,20%of users are outdoor; Use 3km/h for modeling fading channel |  |
| Number of UEs per cell | up to 15 |  |  |
| Scheduling Algorithm | MU-MIMO+ PF |  |  |
| HARQ/repetition | HARQ retransmission |  |  |
| Channel estimation | Realist |  |  |
| Target BLER | 10% for first transmission |  |  |
| PHY processing delay | UE PUSCH processing Capability #1,N1=10 |  |  |
| PDCCH overhead | 1/7(2symbols per 14symbol) |  |  |
| DMRS overhead | 1 symbol per 14symbol |  |  |
| BS receiver | MMSE-IRC |  |  |

Table A.2 Simulation assumption for FR1 UL

| Parameter | value |  |
| --- | --- | --- |
| Scenarios | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m | Scenario-2 : Dense Urban |
| Carrier frequency | 4GHz |  |
| Bandwidth | 100 MHz |  |
| Subcarrier spacing | 30 KHz |  |
| Frame structure | DDDSU (S: 10D:2G:2U) for UL interaction/pose information delivering |  |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32R: (4,4,2,1,1;4,4) (dH,dV) = (0.5, 0.5)λ | For 64R: (8,8, 2, 1, 1; 4, 8) (dH,dV) = (0.5λ, 0.5λ); |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | For 2T: (1,1,2,1,1;1,1) (dH, dV)=( 0.5, N/A)λ |  |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi | Ceiling-mount pattern, 8 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi | Omnidirectional, 0 dBi |
| UE max Power | 23 dBm |  |
| Power control | P0 = -80 dBm, alpha = 0.8 |  |
| Noise Figure | 5dB for BS and 9dB for UE |  |
| Scheduler | MU-MIMO+PF |  |
| Device deployment | 100% indoor | 80% of users are indoor,20%of users are outdoor; Use 3km/h for modeling fading channel |
| Down-tilt | 90 degrees | 12 degrees |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic |  |
| Target BLER | 10% for first transmission |  |
| UE speed | 3 km/h |  |
| HARQ/repetition | HARQ retransmission,Maximum number of retransmissions is 3 |  |
| PHY processing delay | UE PUSCH processing Capability #1,N2=12 |  |
| DMRE overhead | 4RE |  |

Table A.3 simulation parameters for FR2 DL

| Parameters | Value |
| --- | --- |
| Scenario | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m |
| Inter-BS distance | 20m |
| Carrier frequency | 30 GHz |
| Duplex Mode / Simulation bandwidth | 100 MHz |
| SCS | 120KHz |
| TDD pattern | DDDSU |
| BS Antenna Configuration | For 2T: (16,8,2,1,1;1,1) (dH, dV)=(0.5, 0.5)λ |
| UE Antenna Configuration | For 4R/panel: (2,4,2,1,2;1,2) (dH,dV) = (0.5, 0.5)λ (dg,V,dg,H) = (0, 0)λ |
| Transmit Power | 23 dBm per 80M at TRP, EIRP should not exceed 58 dBm |
| Antenna Height | 3 m for BS and 1.5 m for UE |
| Antenna Element Gain | 5dBi for BS and 5 dBi for UE |
| Receiver Noise Figure | 7 dB for BS and 13dB for UE |
| UE distribution | 100% of users are indoor Use 3km/h for modeling fading channel |
| Number of UEs per cell | up to 10 |
| Scheduling Algorithm | SU-MIMO+ PF |
| HARQ/repetition | HARQ retransmission，Maximum number of retransmissions is 3 |
| Channel estimation | Realist |
| Target BLER | 10% for first transmission |
| PHY processing delay | UE PUSCH processing Capability #1,N1=20 |
| PDCCH overhead | 1/7(2symbols per 14symbol) |
| DMRS overhead | 1 symbol per 14symbol |
| BS receiver | MMSE-IRC |

- B Power model

Table B.1 Power model for DL

| Power state | Relative Power(1 slot) |
| --- | --- |
| PDCCH-only | 100 |
| PDCCH+PDSCH | 300 |
| Micro sleep | 45 |
| Light sleep | 20 |
| Deep sleep | 1 |

Table B.2 Power model for UL

| Power state | Relative Power(1 slot) |  |  |
| --- | --- | --- | --- |
|  | 0dBm | 23dBm | Other Tx power |
| UL(long PUCCH or PUSCH) | 250 | 700 | Obtained by linear function or step function |
| PDCCH+PUSCH | Sum(PDCCH-only, PUSCH) |  |  |
| PDCCH+PDSCH+PUSCH | Sum(PDCCH+PDSCH, PUSCH) |  |  |

Note: Power model of “PDCCH+PUSCH” is the sum of “PDCCH-only” power consumption and “PUSCH” power consumption. Power model of “PDCCH+PDSCH+PUSCH” is the sum of “PDCCH+PDSCH” power consumption and “PUSCH” power consumption.