3GPP TSG RAN WG1 #106bis-e 						      				   R1- 2109003

e-Meeting, October 11th – October 19th, 2021

Source:	vivo

Title:	Remaining issues on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs

Agenda Item:	8.12.3

Document for:  Discussion and Decision

Introduction

According to the WID [1] of the NR MBS, the MBS service of NR is to be provided to both the RRC IDLE/INACTIVE UEs and the RRC CONNECTED UEs. In this contribution, we provide our considerations on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs, including MBS common frequency resource, intra-DU SFN, PDCCH, HARQ feedback, and semi-persistent scheduling.

Discussion

MBS common frequency resource

In RAN 1 #106e meeting, one proposal for MBS common frequency resource (CFR) in terms of supporting case C, D, and E has been extensively discussed and the proposal was endorsed in RAN P #93 e meeting as shown below [2]:

For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with

UEs in RRC IDLE/INACTIVE state:

- Support Case-C

- Support at least one of Case D and Case E. Down-selection to be made at RAN1#106b-e

Note: Case C, D and E are defined in previous agreements.

Case E is beneficial as it provides flexibility for the network to configure CFR independent of SIB-1 configured initial downlink BWP, so that proper size of CFR can be freely configured to facilitate MBS services well. It is not wise to consider case E as an optimized solution over case C, on the contrary, case E is complementary to case C and is essential to be supported in Rel-17. SIB-1 configured initial downlink BWP is only valid for UEs after entering RRC-connected mode, and it may be also served as first active DL BWP and default DL BWP. For CFR defined for UEs in RRC IDLE/INACTIVE state, it targets for broadcast services, and thus, case C only works well when the bandwidth required by CFR in RRC IDLE/INACTIVE state match that of SIB-1 configured initial DL BWP, otherwise, a solution derived by taking union set in case C will cause impacts on the side with less bandwidth requirement. To get rid of such adverse impacts, case E could be used to provide appropriate CFR configuration without adopting any constraint on SIB-1 configured initial DL BWP.

During RAN1#106e meeting and RAN P #93 e meeting, companies raised some issues for case E, though they have already been answered via online and offline discussion, we list them and provide some elaboration in appendix for convenient access.

On the other hand, case D can be also supported to provide additional flexibility for the network, especially for cases that the bandwidth required by broadcast services is much smaller than that of SIB-1 configured initial DL BWP.

Proposal 1: For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state:

- Support Case-C

- Support Case D and Case E.

Note: Case C, D and E are defined in previous agreements.

Among multiple MBS services, some RRC IDLE/INACTIVE UEs may be interested in only a subset of services while some other UEs are interested in another subset of services, thus, transmitting all MBS services in one CFR for RRC IDLE/INACTIVE UEs is not friendly to power saving purpose. If several CFRs are defined and only a subset of MBS services are transmitted in one CFR, UE can select one unique CFR depending on its interested services. As shown in figure 1, suppose there are four services: service 1-4. When only one CFR is defined, UE must perform RF tuning to the bandwidth of the CFR, no matter what the interested service is. While multiple CFRs are defined with each fully containing CORESET0 in frequency domain, UE can perform RF tuning to only the bandwidth of certain CFR, and thus, it is beneficial to define multiple CFRs for UEs in RRC IDLE/INACTIVE.

Figure 1: multiple MBS CFRs

Proposal 2: For UEs in RRC_IDLE/RRC_INACTIVE, more than one common frequency resource can be defined/configured.

Intra-DU SFN

In RAN P #93e meeting, one proposal targeting to resolve intra-DU SFN issues was endorsed as shown below[2]:

The following aspects can be considered to be within the scope of the Rel-17 MBS WID and can be further

discussed in the WGs with the aim of minimizing specification impacts:

- Configurable scrambling sequence initialization for PDCCH/PDSCH and DMRS sequence generator initialization

for PDCCH/PDSCH for broadcast transmission (as supported for RRC_CONNECTED UE).

- Configuring TRS as QCL sources for broadcast transmission (as supported for RRC_CONNECTED UE).

Note: For broadcast transmission, the presence of TRS would be optional from a network perspective.

Note: Any SFN operation is transparent to the UE

As noted above and stated in WID, any SFN operation is transparent to the UE. And thus, explicit/implicit SFN notification to Rel-17 MBS UE is not expected. To cope with the issues brought by supporting SFN operation, it is desired to find non SFN-specific solution and minimize specification impact.

Regarding the scrambling sequence initialization for PDCCH/PDSCH and DMRS sequence generator initialization for PDCCH/PDSCH for broadcast transmission, the following aspects can be considered.

- scrambling sequence initialization for GC-PDCCH

According to previous agreements, for RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, both searchSpace#0 and common search space other than searchSpace#0 can be configured, and thus, to provide flexibility, it is better to support that nID in scrambling sequence initialization for GC-PDCCH can be given by higher-layer parameter e.g., PDCCH configuration defined in CFR, when GC-PDCCH using DCI format 1_0 with the CRC scrambled by G-RNTI, MCCH-RNTI and any other RNTIs used by broadcast. And then, it is up to network implementation to configure the same value of nID for multiple cells to support intra-DU SFN.

Regarding to , as discussed for 2nd DCI format, it can take choices from one or more of the following:

- Alt1: G-RNTI used for the GC-PDCCH

- Alt2: 0

- Alt3: Other fixed values

- scrambling sequence initialization for GC-PDSCH

Similarly, as discussed for GC-PDCCH, to provide flexibility, it is better to support that in scrambling sequence initialization for GC-PDSCH can be given by higher-layer parameter e.g., PDSCH configuration defined in CFR, when GC-PDSCH scheduled by GC-PDCCH using DCI format 1_0 with the CRC scrambled by G-RNTI, MCCH-RNTI, and any other RNTIs used by broadcast. And then, it is up to network implementation to configure the same value of nID for multiple cells to support intra-DU SFN.

And  can take the RNTI associated with the PDSCH transmission, e.g., G-RNTI, MCCH-RNTI, and any other RNTIs used by broadcast.

- DMRS sequence generator initialization for GC-PDCCH

Following the existed rule defined for DMRS for PDCCH in 38.211, NID can be given by higher-layer parameter, e.g., PDCCH configuration defined in CFR, when GC-PDCCH using DCI format 1_0 with the CRC scrambled by G-RNTI, MCCH-RNTI and any other RNTI used by broadcast. And then, it is up to network implementation to configure the same value of NID for multiple cells to support intra-DU SFN.

- DMRS sequence generator initialization for GC-PDSCH

Following the existed rule as defined for the PDSCH scheduled by PDCCH using DCI format 1_0 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI in 38.211, can be given by higher-layer parameter, e.g., PDSCH configuration defined in CFR, when GC-PDSCH scheduled by GC-PDCCH using DCI format 1_0 with the CRC scrambled by G-RNTI, MCCH-RNTI, and any other RNTIs used by broadcast. And then, it is up to network implementation to configure the same value of for multiple cells to support intra-DU SFN.

Proposal 3: For scrambling sequence initialization for GC-PDCCH using DCI format 1_0 with the CRC scrambled by G-RNTI, MCCH-RNTI, and any other RNTIs further agreed for broadcast,

- can be configured by high-layer parameters, i.e., GC-pdcch-DMRS-ScramblingID-broadcast, nID=NIDcell if not configured.

- can be selected as one or more of the following

- Alt1: G-RNTI used for the GC-PDCCH

- Alt2: 0

- Alt3: Other fixed values

Proposal 4: For scrambling sequence initialization for GC-PDSCH scheduled by GC-PDCCH using DCI format 1_0 with the CRC scrambled by G-RNTI, MCCH-RNTI, and any other RNTIs further agreed for broadcast,

- can be configured by high-layer parameters, i.e., DataScramblingIdentityGC-PDSCH-broadcast, nID=NIDcell if not configured.

- corresponds to the RNTI associated with the GC-PDSCH transmission.

Proposal 5: For DMRS sequence generator initialization for GC-PDCCH using DCI format 1_0 with the CRC scrambled by G-RNTI, MCCH-RNTI, and any other RNTIs further agreed for broadcast,

- can be configured by high-layer parameters, i.e., GC-pdcch-DMRS-ScramblingID-broadcast,if not configured.

Proposal 6: For DMRS sequence generator initialization for GC-PDSCH scheduled by GC-PDCCH using DCI format 1_0 with the CRC scrambled by G-RNTI, MCCH-RNTI, and any other RNTIs further agreed for broadcast,

- can be configured by high-layer parameters, i.e., GC-pdsch-DMRS-ScramblingID-broadcast,if not configured.

Regarding TRS as QCL source for broadcast transmission, there exist some issues for discussion:

First, the QCL information of TRS, whether it is associated with one SSB or not.

According to the agreements achieved for TRS for idle/inactive UEs in power saving session, the QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as an SSB index in range of 0 to 63[3]. If the same indication method is used, TRS is QCLed with SSB in the serving cell, and thus, it may not be necessary to use TRS as QCL source for broadcast transmission, as SSB can be used directly. If the QCL information of TRS is not indicated by SSB, then how to indicate it should be discussed. Furthermore, when TRS is considered as a QCL source independent of SSB, then, which one to be selected as the QCL source between TRS and SSB should also be discussed.

Second, whether TRS is transmitted in beam sweeping manner.

When TRS is considered as a QCL source independent of SSB, UE has to determine the corresponding beam to receive TRS, especially for FR2. And if TRS is transmitted in beam sweeping manner, the related procedure should be defined and it may increase network load and UE complexity.

Third, the timing acquisition if TRS is used as a QCL source.

It is difficult to acquire timing by detecting TRS only, and thus, it should be further discussed how to acquire timing for transmission and reception when TRS is used as the QCL source.

Proposal 7: Study the following aspects to determine whether to support TRS as QCL source for broadcast transmission.

- Indication method for QCL information of TRS, i.e., whether associated with SSB

- Transmission manner of TRS, e.g., whether beam sweeping is supported in FR2

- Timing acquisition, e.g., how to acquire cell timing

PDCCH

As agreed in previous meeting, common search space can be configured for RRC idle/inactive UEs besides searchSpace#0. Meanwhile, for the type of common search space configured for MBS services, a parallel discussion is ongoing for RRC connected UEs and specification impact of having a new Type-x CSS is under discussion. As no additional requirement is observed for CSS for RRC idle/inactive UEs over that for RRC connected UEs in MBS, the same type of CSS, i.e., type-x CSS can be used.

Proposal 8: The same type of CSS supported for multicast in RRC_CONNECTED can be reused for broadcast in RRC_IDLE/RRC_INACTIVE for GC-PDCCH scheduling MCCH and MTCH.

Regarding to the DCI field for broadcast in RRC_IDLE/RRC_INACTIVE, besides FDRA, TDRA, MCS, and RV, HARQ process number and new data indicator should also be included if PDSCH repetition is supported. For UE in RRC_IDLE/RRC_INACTIVE, as no unicast and groupcast transmission is expected, HARQ processes defined for unicast and groupcast can be used for combing broadcast PDSCH repetition. And it is possible to leave it to RRC_IDLE/RRC_INACTIVE UE implementation to select one HPN without any indication in DCI. However, broadcast PDSCH with repetition can be also received by RRC_CONNECTED UE, if HPN and NDI is not indicated in DCI and RRC_CONNECTED UE randomly chooses a free HPN for combination, then it will cause chaos for further unicast and multicast reception. This is because network has no information about the HPN selected by UE for broadcast PDSCH combination and may indicate the same one for the subsequent unicast and multicast transmission.

Proposal 9: If PDSCH repetition for broadcast is supported, HARQ process number and new data indicator should be included in DCI 1-0 with the CRC scrambled by G-RNTI, MCCH-RNTI, and any other RNTIs further agreed for broadcast.

HARQ feedback

Regarding the HARQ feedback for the MBS reception, we consider that the RRC IDLE/INACTIVE UE should not be required to provide the HARQ feedback at least for broadcast reception, as the received UEs are not known by gNB. Furthermore, they may even not be uplink synchronized with the network. And the reliability can be improved by PDSCH repetition if there is a need. Therefore, at least for broadcast reception, HARQ feedback is not supported.

Proposal 10: For RRC_IDLE/RRC_INACTIVE UEs, at least for broadcast reception, HARQ feedback is not supported.

Semi-persistent scheduling

Semi-persistent scheduling (SPS) is beneficial for periodic transmissions. However, in broadcast, the received UEs are not known by gNB and HARQ feedback is not expected to be supported. If UE fails to detect the activation DCI for SPS MBS PDSCH, it will miss all the subsequent transmissions. Therefore, for RRC_IDLE/RRC_INACTIVE UEs, SPS PDSCH with DCI activation/deactivation is not supported at least for broadcast reception. On the other hand, SPS PDSCH without dynamic activation/deactivation which is similar to uplink configured grant type 1 can be considered instead.

Proposal 11: For RRC_IDLE/RRC_INACTIVE UEs, at least for broadcast reception, SPS PDSCH with DCI activation/deactivation is not supported.

- FFS: SPS PDSCH without DCI activation/deactivation.

Related RRC parameters

Based on the discussion in previous sections, the following RRC parameter could be introduced.

| Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CFR-Id-broadcast | new |  | An identifier for the CFR in broadcast | FFS | FFS | Per CFR | Cell-specific | Proposal 2: For UEs in RRC_IDLE/RRC_INACTIVE, more than one common frequency resource can be defined/configured. |
| DataScramblingIdentityGC-PDSCH-broadcast | new |  | Identifier used to initialize data scrambling for GC-PDSCH in broadcast | (0..1023) | FFS | Per CFR | Cell-specific | Proposal 4: For scrambling sequence initialization for GC-PDSCH scheduled by GC-PDCCH using DCI format 1_0 with the CRC scrambled by G-RNTI, MCCH-RNTI, and any other RNTIs further agreed for broadcast, - can be configured by high-layer parameters, nID=NIDcell if not configured. - corresponds to the RNTI associated with the GC-PDSCH transmission. |
| GC-pdsch-DMRS-ScramblingID-broadcast | new |  | Identifier used to DMRS sequence generator initialization for GC-PDSCH in broadcast | (0..65535) | FFS | Per CFR | Cell-specific | Proposal 6: For DMRS sequence generator initialization for GC-PDSCH scheduled by GC-PDCCH using DCI format 1_0 with the CRC scrambled by G-RNTI, MCCH-RNTI, and any other RNTIs further agreed for broadcast, - can be configured by high-layer parameters, if not configured. |
| GC-pdcch-DMRS-ScramblingID-broadcast | new |  | GC-PDCCH DMRS scrambling initialization | (0..65535) | FFS | Per CFR | Cell-specific | Proposal 5: For DMRS sequence generator initialization for GC-PDCCH using DCI format 1_0 with the CRC scrambled by G-RNTI, MCCH-RNTI, and any other RNTIs further agreed for broadcast, - can be configured by high-layer parameters, i.e., GC-pdcch-DMRS-ScramblingID-broadcast,if not configured. |

Conclusions

For RRC IDLE/INACTIVE UEs, this contribution provides our consideration on MBS common frequency resource, intra-DU SFN, PDCCH, HARQ feedback, and semi-persistent scheduling and beam sweeping, and the following proposals are proposed:

Proposal 1: For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state:

- Support Case-C

- Support Case D and Case E.

Note: Case C, D and E are defined in previous agreements.

Proposal 2: For UEs in RRC_IDLE/RRC_INACTIVE, more than one common frequency resource can be defined/configured.

Proposal 3: For scrambling sequence initialization for GC-PDCCH using DCI format 1_0 with the CRC scrambled by G-RNTI, MCCH-RNTI, and any other RNTIs further agreed for broadcast,

- can be configured by high-layer parameters, i.e., GC-pdcch-DMRS-ScramblingID-broadcast, nID=NIDcell if not configured.

- can be selected as one or more of the following

- Alt1: G-RNTI used for the GC-PDCCH

- Alt2: 0

- Alt3: Other fixed values

Proposal 4: For scrambling sequence initialization for GC-PDSCH scheduled by GC-PDCCH using DCI format 1_0 with the CRC scrambled by G-RNTI, MCCH-RNTI, and any other RNTIs further agreed for broadcast,

- can be configured by high-layer parameters, i.e., DataScramblingIdentityGC-PDSCH-broadcast, nID=NIDcell if not configured.

- corresponds to the RNTI associated with the GC-PDSCH transmission.

Proposal 5: For DMRS sequence generator initialization for GC-PDCCH using DCI format 1_0 with the CRC scrambled by G-RNTI, MCCH-RNTI, and any other RNTIs further agreed for broadcast,

- can be configured by high-layer parameters, i.e., GC-pdcch-DMRS-ScramblingID-broadcast,if not configured.

Proposal 6: For DMRS sequence generator initialization for GC-PDSCH scheduled by GC-PDCCH using DCI format 1_0 with the CRC scrambled by G-RNTI, MCCH-RNTI, and any other RNTIs further agreed for broadcast,

- can be configured by high-layer parameters, i.e., GC-pdsch-DMRS-ScramblingID-broadcast,if not configured.

Proposal 7: Study the following aspects to determine whether to support TRS as QCL source for broadcast transmission.

- Indication method for QCL information of TRS, i.e., whether associated with SSB

- Transmission manner of TRS, e.g., whether beam sweeping is supported in FR2

- Timing acquisition, e.g., how to acquire cell timing

Proposal 8: The same type of CSS supported for multicast in RRC_CONNECTED can be reused for broadcast in RRC_IDLE/RRC_INACTIVE for GC-PDCCH scheduling MCCH and MTCH.

Proposal 9: If PDSCH repetition for broadcast is supported, HARQ process number and new data indicator should be included in DCI 1-0 with the CRC scrambled by G-RNTI, MCCH-RNTI, and any other RNTIs further agreed for broadcast.

Proposal 10: For RRC_IDLE/RRC_INACTIVE UEs, at least for broadcast reception, HARQ feedback is not supported.

Proposal 11: For RRC_IDLE/RRC_INACTIVE UEs, at least for broadcast reception, SPS PDSCH with DCI activation/deactivation is not supported.

- FFS: SPS PDSCH without DCI activation/deactivation.

Reference

[1] RP-201038, Huawei, “WID revision: NR Multicast and Broadcast Services”.

[2] RAN WG #93 e-meeting meeting report

[3] RAN 1 #105 e-meeting chairman notes

Appendix

List of typical issues of case E

| issues raised for case E | elaboration |
| --- | --- |
| - How high the data rate of MBS service is expected to be, so that a very large size CFR is required to be configured in case E? | The merit of case E is not to configure a very large size CFR, instead, it provides network flexibility by decoupling CFR configuration from SIB-1 configured initial DL BWP, and thus, proper bandwidth can be configured for both regarding respective requirement, without causing any adverse impact to each other. The following two cases can be supported by case E. SIB-1 configured initial DL BWP; Configured CFR. Case 1; Small/medium/large bandwidth; Slightly larger than initial DL BWP. Case 2; Small/medium/large bandwidth; Much larger than initial DL BWP. |
| - For UEs with small bandwidth capability, whether they can get MBS service in case E. | Sure, as analysed above, Case E does not have to be always configuring a very large bandwidth, instead, CFR can be configured to be any size according to practical requirements. If UEs with small bandwidth capability are expected to receive MBS services, network can take this into account and configure proper CFR size. When the size of SIB-1 configured initial DL BWP turns out to be smaller than that of the CFR, it is case E. When it is larger, it is case D, and for equal size, it is case C. |
| - For Rel-17 MBS UE, whether to keep the legacy framework of using SIB1-configured initial DL BWP as the first active DL BWP (if not configured) when UE enters connected mode in case E? | It is up to network to configure firstactiveBWP, and in case E, network can configure one BWP containing CFR as the firstactiveBWP, so that MBS services can be received continuously when UE enters connected mode, and there is no BWP switching issue. And if network doesn’t configure, then SIB-1 configured BWP is the firstactiveBWP by default. |
| - For Rel-17 MBS UE, how to ensure a connected UE does not miss the broadcast reception when the UE fallbacks to default BWP if default BWP is not configured and CFR is larger than SIB-1 configured initial DL BWP? | If network does not configure default BWP containing CFR in case E, UE may miss the broadcast reception, as it will fallback to SIB-1 configured initial DL BWP. However, we believe that network do configure default BWP containing CFR if continuous MBS service is expected, and it is totally up to network implementation without spec effect. |
| - By supporting case E, whether two initial DL BWPs are defined for R17 MBS UE | Same as legacy UE, only one initial DL BWP is defined for R17 MBS UE, CORESET 0 and SIB-1 configured BWP is the initial DL BWP for UEs in RRC idle/inactive state and RRC connected state, respectively. Regarding the configured BWP based on which CFR for UEs in RRC idle/inactive state is configured, it targets for configuring CFR. |