3GPP TSG RAN WG1 #106b-e	                                              R1-2109008

e-Meeting, October 11th – 19th, 2021

Source:	vivo

Title:	Performance evaluation results on XR

Agenda Item:	8.14.1

Document for:	Discussion and Decision

- Introduction

In the previous RAN1 meeting, the evaluation methodologies/assumptions and the characteristics of XR traffic model were discussed, the corresponding agreements can be found in [1][2][3][4]. Based on these, we provided our updated simulation results of capacity, power consumption, coverage under various scenarios and traffic attributes for Cloud Gaming and XR (i.e. VR/AR). The overall simulation assumptions can be found in Appendix A. The detailed simulation parameters and results can also be found in the accompanying template excel table.

- Capacity Evaluation

In this section, the results of downlink and uplink capacity for both Cloud Gaming and XR are presented and summarized in the form of tables separately. Besides, based on the agreements of evaluation methodologies/assumptions, the following assumptions are applied in the performance evaluation of system capacity.

- A packet is considered as lost when it has exceeded the PDB, such that it will be added to the PER and discarded at the transmitter (including the non-transmitted part).

- The equal number of served UEs by each cell is assumed.

- In the following tables, the value of #UEs /cell represents an integer "A", where at least 90% of UEs are satisfied UEs with "A" UEs per cell, but less than 90% of UEs are satisfied with "A+1" UEs per cell.

- The system capacity is defined as the maximum number of UEs per cell where at least 90% of UEs are satisfied UEs.

- A UE will be declared as the satisfied UE if more than X% of packets are successfully delivered within a given air interface PDB, and X=99 is assumed by default.

- UE% denotes the specific percentage of satisfied UEs per cell.

The traffic model and scenario combinations in Table 1 are evaluated for system capacity in this section.

Table 1. Traffic model and scenario combinations used for system capacity evaluation

| DL FR1 - Indoor Hotspot/Dense Urban/Urban Macro scenario: - CG: 30Mbps, 60FPS, PDB=15ms - VR/AR: 30/45Mbps, 60/120FPS, PDB=10ms - Dense Urban scenario: - Multi-stream with VR/AR 30Mbps, 60FPS, PDB=10ms - GOP based traffic model: α=1.5, 2, 3 - Slice based traffic model: α=1.5, 2, 3 | DL FR2 - Indoor Hotspot/Dense Urban scenario: - CG: 30Mbps, 60FPS, PDB=15ms - VR/AR: 30/45Mbps, 60/120FPS, PDB=10ms - Indoor Hotspot scenario: - Multi-stream with VR/AR 30Mbps, 60FPS, PDB=10ms - GOP based traffic model: α=1.5, 2, 3 - Slice based traffic model: α=1.5, 2, 3 |
| --- | --- |
| UL FR1 - Indoor Hotspot/ Dense Urban scenario: - Single stream: Pose/control stream or video stream - Two streams: Pose/control stream + video stream - Urban Macro scenario: - Single stream: Pose/control stream or video stream | UL FR2 - Indoor Hotspot/Dense Urban scenario: - Pose/control stream - Video stream: 10Mbps, 60FPS, PDB=30ms |

  - Traffic model

    - Single-stream

      - Downlink

Based on the agreements achieved in RAN1#104b-e, Table 2 and Table 3 provide the downlink traffic models for different applications (i.e., CG and VR/AR) with single-stream. The packet arrival interval and packet size are determined by the corresponding data rate and frame rate. The video frame rate is assumed to be 60 FPS when dual-eye buffers are bundled to be transmitted by the application server. In contrast, the video frame rate is assumed to be 120 FPS for separate packet arrivals in time for dual-eye buffers, which means that dual-eye buffers are sent with staggering in time.

For simplicity, one video frame is modeled as one packet and the packet size distribution follows the truncated Gaussian distribution. Besides, the jitter with respect to the packet arrival time at RAN is also considered to model the random delay caused by different encoding delays and core network transmission delays. For F FPS (e.g., F = 60 or 120), the arrival time of packet k is (k / F + J), where the jitter size J is a random variable drawn from a truncated Gaussian distribution.

Table 2. DL traffic models of Cloud Gaming (CG) and XR with 60 FPS

| Traffic model | CG | VR/AR | VR/AR |
| --- | --- | --- | --- |
| Data rate (Mbps) | 30 | 30 | 45 |
| Packet size distribution | Truncated Gaussian distribution |  |  |
| Mean packet size (Bytes) | 62500 | 62500 | 93750 |
| STD of packet sizes (Bytes) | 6562 | 6562 | 9844 |
| Maximum packet size (Bytes) | 93750 | 93750 | 140625 |
| Minimum packet size (Bytes) | 31250 | 31250 | 46875 |
| Packet arrival interval (ms) | 16.67 |  |  |
| Packet delay budget (ms) | 15 | 10 |  |
| Jitter distribution | Truncated Gaussian distribution |  |  |
| Jitter Mean (ms) | 0 |  |  |
| Jitter STD (ms) | 2 |  |  |
| Jitter Range (ms) | [-4, 4] |  |  |

Table 3. DL traffic models of Cloud Gaming (CG) and XR with 120 FPS

| Traffic model | CG | VR/AR | VR/AR |
| --- | --- | --- | --- |
| Data rate (Mbps) | 30 | 30 | 45 |
| Packet size distribution | Truncated Gaussian distribution |  |  |
| Mean packet size (Bytes) | 31250 | 31250 | 46875 |
| STD of packet sizes (Bytes) | 3281 | 3281 | 4922 |
| Maximum packet size (Bytes) | 46875 | 46875 | 70313 |
| Minimum packet size (Bytes) | 15625 | 15625 | 23438 |
| Packet arrival interval (ms) | 8.33 |  |  |
| Packet delay budget (ms) | 15 | 10 |  |
| Jitter distribution | Truncated Gaussian distribution |  |  |
| Jitter Mean (ms) | 0 |  |  |
| Jitter STD (ms) | 2 |  |  |
| Jitter Range (ms) | [-4, 4] |  |  |

      - Uplink

For the UL evaluations, the packet size distribution of the aggregating streams (scene/video/data/audio) is assumed to follow truncated Gaussian distribution and the data rate is assumed as 10 Mbps. Pose/control stream is periodically sampled and delivered with a fixed packet size. No jitter is assumed for UL evaluations. The traffic characteristics and requirements for the pose/control stream and scene/video/data/audio stream are presented in Table 4.

Table 4. UL traffic models of Cloud Gaming (CG) and XR

| Traffic model | Mean packet size (Bytes) | STD of packet sizes (Bytes) | Min packet size (Bytes) | Max packet size (Bytes) | Packet arrival interval (ms) | PDB (ms) | Jitter |
| --- | --- | --- | --- | --- | --- | --- | --- |
| pose/control stream | 100 | 0 | 100 | 100 | 4 | 10 | No |
| scene/video/data/audio stream (10Mbps) | 20833 | 2187 | 10416 | 31249 | 16.67 | 30 | No |

    - Multi-stream

      - Downlink I/P-Frame multi-stream model

In the video and image coding field, the video streams are usually encoded into I-frames and P-frames (and/or B-frames), where an I-frame is an intra-frame coding frame without any reference to other frame(s), and a P-frame (or B-frame) is an inter-frame coding frame with reference to other frame(s), e.g. the latest I-frame. According to the agreements achieved in RAN1#105-e meeting [3], the I-frames and P-frames could be modeled as two streams. Furthermore, there are two options for I/P-frame multi-stream traffic modeling, i.e., Group-Of-Picture (GOP) based traffic model and slice-based traffic model.

For GOP-based traffic model, the encoder encodes images and generates frames in the unit of GOP (Group of Pictures). GOP is a group of continuous pictures, consisting of one I-frame and several B/P-frames. The interval between two I-frame is the length of GOP, and the FPS refers to the duration between two adjacent P-frames. In this contribution, GOP length = 8 is assumed, and different alpha values each representing the average ratio between the packet size of an I-frame and that of a P-frame are evaluated. The corresponding traffic models are provided in Table 5.

For slice-based traffic model, each encoded video frame contains multiple slices, each of which is from an I-frame or a P-frame corresponding to the video frame, including one I-slice and N-1 P-slices, where N=8 is assumed. A slice refers to a specific area of a video frame and different areas are encoded into different slices separately. In this contribution, N-1 P-slices corresponding to the same video frame are modeled as one packet as a whole in the simulations, i.e. there is one packet generated in simulations to contain the N-1 P-slices at a time corresponding to a video frame. The size of the generated packet is equal to the sum of the sizes of the N-1 P-slices. The PER and PDB requirements are defined for packets each containing multiple P-slices. The detailed description and analysis can refer to our companion contribution [5]. The corresponding traffic models can be found in Table 6.

Table 5.  GOP-based traffic model with 30Mbps

| Alpha value | 1.5 |  | 2 |  | 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic model | I-stream | P-stream | I-stream | P-stream | I-stream | P-stream |  |
| Date rate (Mbps) | 30*1.58.5 | 30*78.5 | 30*29 | 30*79 | 30*310 | 30*710 | GOP length = 8 |
| Packet size distribution | Truncated Gaussian distribution |  |  |  |  |  |  |
| Mean packet size (Bytes) | 88235 | 58824 | 111112 | 55556 | 150000 | 50000 | The average size ratio between I-frame and P-frame is around α :1 |
| STD of packet sizes (Bytes) | 9265 | 6176 | 11667 | 5833 | 15750 | 5250 | 10.5% * Mean packet size |
| Maximum packet size (Bytes) | 132353 | 88236 | 166668 | 83334 | 225000 | 75000 | 150% * Mean packet size |
| Minimum packet size (Bytes) | 44118 | 29412 | 55556 | 27778 | 75000 | 25000 | 50% * Mean packet size |
| Packet arrival interval (ms) | 800060 | 100060 | 800060 | 100060 | 800060 | 100060 | 1 I-frame and 7 P-frames in one GOP |

Table 6.  Slice-based traffic model with 30Mbps

| Alpha value | 1.5 |  | 2 |  | 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic model | I-stream | P-stream | I-stream | P-stream | I-stream | P-stream |  |
| Date rate (Mbps) | 30*1.58.5 | 30*78.5 | 30*29 | 30*79 | 30*310 | 30*710 |  |
| Packet size distribution | Truncated Gaussian distribution |  |  |  |  |  |  |
| Mean packet size (Bytes) | 11029 | 51470 | 13888 | 48608 | 18750 | 43750 | The average size ratio between I-slice and P-slice is around α :1, one P-frame contains 7 P-slices at a time |
| STD of packet sizes (Bytes) | 1158 | 5404 | 1458 | 5104 | 1969 | 4594 | 10.5% * Mean packet size |
| Maximum packet size (Bytes) | 16544 | 77205 | 20832 | 72912 | 28125 | 65625 | 150% * Mean packet size |
| Minimum packet size (Bytes) | 5515 | 25735 | 6944 | 24304 | 9375 | 21875 | 50% * Mean packet size |
| Packet arrival interval (ms) | 100060 | 100060 | 100060 | 100060 | 100060 | 100060 | each encoded video frame contains 1 I-slice and 7 P-slices |

      - Uplink multi-stream traffic model

For UL AR application, both pose/control stream and scene/video/data/audio stream need to be transmitted. So it is necessary to evaluate the two-stream model, which is closer to the actual operation of UEs. For two-stream model, one stream is pose/control stream with 0.2Mbps_250FPS_10ms and the other is aggregating streams (scene/video/data/audio) with 10Mbps_60FPS_30ms. No jitter is assumed for UL evaluations. The traffic characteristics and requirements for the pose/control stream and scene/video/data/audio stream are presented in Table 7, which is the same as that for UL single-stream in Table 4.

Table 7. UL traffic models for two-stream of AR

| Traffic model | Mean packet size (Bytes) | STD of packet sizes (Bytes) | Min packet size (Bytes) | Max packet size (Bytes) | Packet arrival interval (ms) | PDB (ms) | Jitter |
| --- | --- | --- | --- | --- | --- | --- | --- |
| pose/control stream | 100 | 0 | 100 | 100 | 4 | 10 | No |
| scene/video/data/audio stream (10Mbps) | 20833 | 2187 | 10416 | 31249 | 16.67 | 30 | No |

  - Capacity Evaluation Result of Single-stream

    - FR1 Downlink

The evaluation results of DL capacity for FR1 in Indoor Hotspot, Dense Urban and Urban Macro deployment scenarios are presented in Table 8 to Table 10. Based on the notes given at the beginning of section 2, a UE will be declared as a satisfied UE if X% or more of the packets are successfully delivered within PDB and X=99 is assumed. In the evaluations, following settings are adopted.

- MIMO scheme: SU-MIMO and MU-MIMO are both evaluated. MU-MIMO scheduler is evaluated in FR1 downlink transmission which can schedule more UEs by utilizing different layers on the same frequency resource.

- Scheduler: Traditional proportional-fair (PF) scheduler is adopted by default. To meet the stringent PDB requirement with high data rate, delay-aware (DA) scheduler taking time factor into account is adopted in the evaluations. The DA scheduler prioritizes UEs having packet(s) approaching the deadline by adding time factor to the weight on top of the PF metric.

- Frame rate: The frame rate is assumed to be 60 FPS by default.  The frame rate 120 FPS is also evaluated for separate packet arrivals in time for dual-eye buffers. The specific details of the traffic models can be found in section 2.1.1.1.

- InH, FR1, DL, 100MHz bandwidth, DDDSU TDD format

Table 8. System capacity in FR1 DL Indoor Hotspot scenario

| Traffic Model | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| CG (30Mbps) | 10.14 | 10 | 91.67% | 16.2 | 16 | 91.15% |  |
|  | 11.43 | 11 | 96.06% | 16.67 | 16 | 92.01% | Note 1 |
| VR/AR(30Mbps) | 8.27 | 8 | 92.71% | 10.8 | 10 | 92.50% |  |
|  | 10.77 | 10 | 95.20% | 12.4 | 12 | 93.06% | Note 1 |
|  | - | - | - | 16.53 | 16 | 92.71% | Note 2 |
| VR/AR(45Mbps) | - | - | - | 5.91 | 5 | 96.67% |  |
|  | - | - | - | 9.22 | 9 | 91.36% | Note 2 |
| Note 1: adopting delay-aware (DA) scheduling Note 2: separate packet arrivals in time for dual-eye buffer with 120FPS |  |  |  |  |  |  |  |

- DU, FR1, DL, 100MHz bandwidth, DDDSU TDD format

Table 9. System capacity in FR1 DL Dense Urban scenario

| Traffic Model | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| CG (30Mbps) | 11.68 | 11 | 94.81% | 19.65 | 19 | 92.56% |  |
|  | 13.58 | 13 | 94.90% | 19.75 | 19 | 92.86% | Note 1 |
| VR/AR(30Mbps) | 9.49 | 9 | 94.18% | 13.59 | 13 | 92.43% |  |
|  | 12.67 | 12 | 95.12% | 14.4 | 14 | 91.84% | Note 1 |
|  | - | - | - | 20.78 | 20 | 92.54% | Note 2 |
| VR/AR(45Mbps) | - | - | - | 6.91 | 6 | 95.63% |  |
|  | - | - | - | 11.42 | 11 | 91.77% | Note 2 |
| Note 1: adopting delay-aware (DA) scheduling Note 2: separate packet arrivals in time for dual-eye buffer with 120FPS |  |  |  |  |  |  |  |

- Uma, FR1, DL, 100MHz bandwidth, DDDSU TDD format

Table 10. System capacity in FR1 DL Urban Macro scenario

| Traffic Model | SU-MIMO |  |  | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| CG (30Mbps) | 10.33 | 10 | 91.90% | 14.33 | 14 | 91.33% |  |
|  | 11.94 | 11 | 93.78% | 14.45 | 14 | 91.73% | Note 1 |
| VR/AR(30Mbps) | 7.24 | 7 | 92.48% | 8.82 | 8 | 93.75% |  |
|  | 8.56 | 8 | 92.64% | 9.55 | 9 | 92.30% | Note 1 |
|  | - | - | - | 14.59 | 14 | 92.06% | Note 2 |
| VR/AR(45Mbps) | - | - | - | 4.68 | 4 | 94.05% |  |
|  | - | - | - | 8.12 | 8 | 90.87% | Note 2 |
| Note 1: adopting delay-aware (DA) scheduling Note 2: separate packet arrivals in time for dual-eye buffer with 120FPS |  |  |  |  |  |  |  |

As shown from the results, the DA scheduler can improve system capacity, especially when the #UE per cell is small corresponding to relatively sufficient system resources. In addition, compared to SU-MIMO scheduler, MU-MIMO scheduler also can improve system capacity performance for FR1 DL.

From the above tables, it also can be observed that the system capacity is degraded with the increase of data rate due to the large packet size. For an instance, for VR/AR traffic, the system capacity could decrease by 4.14 to 9.36 with the data rate increase from 30Mbps to 45Mbps in FR1 DL. Compared with VR/AR applications, the capacity of Cloud Gaming shows significant improvement because of the relatively relaxed PDB requirement.

Moreover, separate packet arrivals in time for dual-eye buffer is an effective way to improve the system capacity. As observed, the system capacity improvement by 3.31 to 7.19 could be obtained compared to bundling transmission of dual-eye buffers. For the model of separate packet arrivals, the PDB of each frame transmission is independent, implying that the total PDB for dual-eye buffer transmission is longer.

Observation 1: In FR1 in Indoor Hotspot with the capacity requirement X=99,

- For DL CG with 30Mbps_60FPS_15ms, the capacity performance is 10.14 with SU-MIMO and 16.2 with MU-MIMO.

- For DL VR/AR with 30Mbps_60FPS_10ms, the capacity performance is 8.27 with SU-MIMO and 10.8 with MU-MIMO.

- For DL VR/AR with 45Mbps_60FPS_10ms, the capacity performance is 5.91 with MU-MIMO.

Observation 2: In FR1 in Dense Urban with the capacity requirement X=99,

- For DL CG with 30Mbps_60FPS_15ms, the capacity performance is 11.68 with SU-MIMO and 19.65 with MU-MIMO.

- For DL VR/AR with 30Mbps_60FPS_10ms, the capacity performance is 9.49 with SU-MIMO and 13.59 with MU-MIMO.

- For DL VR/AR with 45Mbps_60FPS_10ms, the capacity performance is 6.91 with MU-MIMO.

Observation 3: In FR1 in Urban Macro with the capacity requirement X=99,

- For DL CG with 30Mbps_60FPS_15ms, the capacity performance is 10.33 with SU-MIMO and 14.33 with MU-MIMO.

- For DL VR/AR with 30Mbps_60FPS_10ms, the capacity performance is 7.24 with SU-MIMO and 8.82 with MU-MIMO.

- For DL VR/AR with 45Mbps_60FPS_10ms, the capacity performance is 4.68 with MU-MIMO.

Observation 4: Compared to SU-MIMO, MU-MIMO scheduler can increase the system capacity in FR1 DL,

- In Indoor Hotspot, the capacity performances are increased by 59.76% from 10.14 to 16.2 for CG with 30Mbps_60FPS_15ms and by 30.59% from 8.27 to 10.8 for VR/AR with 30Mbps_60FPS_10ms.

- In Dense Uban, the capacity performances are increased by 68.24% from 11.68 to 19.65 for CG with 30Mbps_60FPS_15ms and by 43.20%  from 9.49 to 13.59 VR/AR with 30Mbps_60FPS_10ms.

- In Urban Macro, the capacity performances are increased by 38.72% from 10.33 to 14.33 for CG with 30Mbps_60FPS_15ms and by 21.82% from 7.24 to 8.82 VR/AR with 30Mbps_60FPS_10ms.

Observation 5：The capacity performance is degraded with the increase of data rate.

Observation 6：The capacity of Cloud Gaming outnumbers VR/AR applications due to the relatively relaxed PDB requirement.

Observation 7: Compared to bundling transmission of dual-eye buffers, separate packet arrivals in time for dual-eye buffers can increase the system capacity.

Observation 8: Delay-aware scheduler can increase the system capacity compared to typical PF scheduler.

    - FR1 Uplink

In this section, CG/VR and AR applications with single-stream in FR1 UL transmission are evaluated, and the results are presented in Table 11 to Table 13, and the detailed traffic models are provided in section 2.1.1.2. Only SU-MIMO scheduler is evaluated for UL transmission.

Specially, for pose/control traffic, the data rate is 0.2 Mbps and the packet size is only 100 bytes. It means that only a few resources are needs to complete the transmission in one UL slot, which results in a high system capacity. However, the PDCCH resource is not negligible for dynamic scheduling. Taking FR1 PUSCH transmission as an example, with the TDD structure of “DDDSU”, the PDCCH resources in the three D-slots and one S-slot can be used to schedule subsequent UL data transmissions. According to the simulation parameters, there are a total of NRB = 273 PRBs in frequency domain in FR1. Assuming that Nsymbol = 2 symbols per slot are allocated for PDCCH, then the number of available CCEs per slot is NCCE = 91, which can be calculated by the following formula.

NCCE=NRB*Nsymbol6

It is assumed that the UE is always granted with the maximum aggregation level (AL) of 16 to guarantee the high reliability of the PDCCH transmission. Based on previous assumption of the number of available CCEs, the maximum number of UEs per cell that the system can accommodate in each slot is NUE=20 that can be calculated by the following formula, where Nslot=4 donates the number of the slots with PDCCH resources. That is, with the aggregation level of 16, if the number of UEs per cell of 20 where at least 90% of the UEs are satisfied UEs, then the number of UEs per cell of 20 could be considered to be the upper limit of the system capacity of pose/control traffic.  Therefore, in this contribution, 20 UEs per cell are simulated for UL pose/control traffic.

NUE=NCCEAL*Nslot

- InH, FR1, UL, 100MHz bandwidth, DDDSU TDD format

Table 11. System capacity in FR1 UL Indoor Hotspot scenario

| Traffic Model | SU-MIMO |  |  |
| --- | --- | --- | --- |
|  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |
| pose/control-stream (0.2Mbps) | 20 | 20 | 100% |
| scene/video/data/voice-stream (10Mbps) | 13.95 | 13 | 93.59% |

- DU, FR1, UL, 100MHz bandwidth, DDDSU TDD format

Table 12. System capacity in FR1 UL Dense Urban scenario

| Traffic Model | SU-MIMO |  |  |
| --- | --- | --- | --- |
|  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |
| pose/control-stream (0.2Mbps) | 20 | 20 | 99.99% |
| scene/video/data/voice-stream (10Mbps) | 9.49 | 9 | 92.95% |

- Uma, FR1, UL, 100MHz bandwidth, DDDSU TDD format

Table 13. System capacity in FR1 UL Urban Macro scenario

| Traffic Model | SU-MIMO |  |  |
| --- | --- | --- | --- |
|  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |
| pose/control-stream (0.2Mbps) | 20 | 20 | 97.70% |
| scene/video/data/voice-stream (10Mbps) | <1 | 0 | 74.60% |

For pose/control stream of CG and VR applications in UL transmission, considering limited PDCCH resource, 20 UEs per cell can be supported in all deployment scenarios. Compared to the results of DL transmission in section 2.2.1, UL transmission of CG and VR can satisfy more UEs and is not the bottleneck that limits system capacity.

For UL video stream of AR application, in Indoor Hotspot scenario, compared with downlink transmission with 10ms PDB, more UEs can be accommodated in FR1 UL due to relatively smaller packet size and relaxed PDB (30ms). However, in Dense Urban scenario, fewer UEs can be supported in UL than that in DL transmission, because with the increase of coupling loss, the limited transmission power will limit the system capacity. As for UL video stream of AR application in Urban Macro scenario, the percentage of satisfied UEs is less than 90% even with only 1 UE per cell. This is because cell-edge UEs in Urban Macro suffer large coupling loss and limited transmission power, such that only a small amount of data can be transmitted on each transmission occasion, resulting in a large number of packets exceeding the PDB to the point of raising the PER.

Observation 9: In FR1 in Indoor Hotspot with the capacity requirement X=99,

- For UL pose/control-stream with 0.2Mbps_250FPS_10ms, the capacity performance is 20 with SU-MIMO considering limited PDCCH resource.

- For UL scene/video/data/voice-stream with 10Mbps_60FPS_30ms, the capacity performance is 13.95 with SU-MIMO.

Observation 10: In FR1 in Dense Urban with the capacity requirement X=99,

- For UL pose/control-stream with 0.2Mbps_250FPS_10ms, the capacity performance is 20 with SU-MIMO considering limited PDCCH resource.

- For UL scene/video/data/voice-stream with 10Mbps_60FPS_30ms, the capacity performance is 9.49 with SU-MIMO.

Observation 11: In FR1 in Urban Macro with the capacity requirement X=99,

- For UL pose/control-stream with 0.2Mbps_250FPS_10ms, the capacity performance is 20 with SU-MIMO considering limited PDCCH resource.

- For UL scene/video/data/voice-stream with 10Mbps_60FPS_30ms, the capacity performance is <1 with SU-MIMO.

Observation 12: For CG and VR applications in FR1, DL is the bottleneck of system capacity because of the high data rate.

    - FR2 Downlink

In this section, the system capacity of CG and VR/AR traffic is evaluated in FR2. The results in Indoor Hotspot and Dense Urban deployment scenarios are provided in Table 14 and Table 15, where SU-MIMO scheduler is assumed. Besides, the DA scheduler and dual-eye buffer were also evaluated.

- InH, FR2, DL, 100MHz bandwidth, DDDSU TDD format

Table 14. System capacity in FR2 DL Indoor Hotspot scenario

| Traffic Model | SU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- |
|  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| CG (30Mbps) | 9.91 | 9 | 95.37% |  |
|  | 10.23 | 10 | 91.11% | Note 1 |
| VR/AR (30Mbps) | 8.72 | 8 | 92.01% |  |
|  | 8.83 | 8 | 92.36% | Note 1 |
|  | 10.23 | 10 | 91.94% | Note 2 |
| VR/AR (45Mbps) | 4.67 | 4 | 94.44% |  |
|  | 6.03 | 6 | 90.28% | Note 2 |
| Note 1: adopting delay-aware (DA) scheduling Note 2: separate packet arrivals in time for dual-eye buffer with 120FPS |  |  |  |  |

- DU, FR2, DL, 100MHz bandwidth, DDDSU TDD format

Table 15. System capacity in FR2 DL Dense Urban scenario

| Traffic Model | SU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- |
|  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| CG (30Mbps) | 16.16 | 16 | 92.36% |  |
|  | 16.82 | 16 | 96.73% | Note 1 |
| VR/AR (30Mbps) | 13.44 | 13 | 95.24% |  |
|  | 14.16 | 14 | 91.27% | Note 1 |
|  | 16.28 | 16 | 93.55% | Note 2 |
| VR/AR (45Mbps) | 8.2 | 8 | 93.25% |  |
|  | 10.32 | 10 | 93.97% | Note 2 |
|  | 43.89 | 43 | 91.92% | Note 3 |
| Note 1: adopting delay-aware (DA) scheduling Note 2: separate packet arrivals in time for dual-eye buffer with 120FPS Note 3: 400MHz bandwidth |  |  |  |  |

Observation 13: In FR2 in Indoor Hotspot with the capacity requirement X=99 and 100MHz bandwidth,

- For DL CG with 30Mbps_60FPS_15ms, the capacity performance is 9.91 with SU-MIMO.

- For DL VR/AR with 30Mbps_60FPS_10ms, the capacity performance is 8.72 with SU-MIMO.

- For DL VR/AR with 45Mbps_60FPS_10ms, the capacity performance is 4.67 with SU-MIMO.

Observation 14: In FR2 in Dense Urban with the capacity requirement X=99 and 100MHz bandwidth,

- For DL CG with 30Mbps_60FPS_15ms, the capacity performance is 16.16 with SU-MIMO.

- For DL VR/AR with 30Mbps_60FPS_10ms, the capacity performance is 13.44 with SU-MIMO.

- For DL VR/AR with 45Mbps_60FPS_10ms, the capacity performance is 8.20 with SU-MIMO.

Observation 15: In FR2 in Dense Urban with the capacity requirement X=99 and 400MHz bandwidth,

- For DL VR/AR with 45Mbps_60FPS_10ms, the capacity performance is 43.89 with SU-MIMO.

    - FR2 Uplink

The simulation results of FR2 UL with 100MHz bandwidth are provided in Table 16 and Table 17. Only single-stream is assumed for XR evaluation in FR2 uplink.

For pose/control traffic, the calculation in Section 2.2.2 can be reused to consider the effect of PDCCH resource for dynamic scheduling in FR2. For FR2 with 100MHz bandwidth, there are 2 symbols each slot for PDCCH and 66 PRBs per slot, so the number of available CCEs per slot is 22. With the TDD structure of “DDDSU” and maximum AL of 16, the maximum number of UEs per cell that the system can accommodate in each slot is 4. However, for FR2 with 120KHz subcarrier spacing, the length of each TDD structure of “DDDSU” is 0.625ms, while the period of pose/control traffic is 4ms, much longer than the length of the TDD structure. Therefore, UE will not perform UL transmission at each UL slot and gNB can schedule more UEs within limited PDCCH resource. So similar to FR1 UL evaluation, only 20 UEs per cell are simulated for UL pose/control traffic.

- InH, FR2, UL,100MHz bandwidth, DDDSU TDD format

Table 16. System capacity in FR2 UL Indoor Hotspot scenario

| Traffic Model | SU-MIMO |  |  |
| --- | --- | --- | --- |
|  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |
| pose/control-stream (0.2Mbps) | 20 | 20 | 97.69% |
| scene/video/data/voice-stream (10Mbps) | 8.59 | 8 | 95.14% |

- DU, FR2, UL, 100MHz bandwidth, DDDSU TDD format

Table 17. System capacity in FR2 UL Dense Urban scenario

| Traffic Model | SU-MIMO |  |  |
| --- | --- | --- | --- |
|  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |
| pose/control-stream (0.2Mbps) | 20 | 20 | 96.51% |
| scene/video/data/voice-stream (10Mbps) | 8.30 | 8 | 92.66% |

Observation 16: In FR2 in Indoor Hotspot with the capacity requirement X=99 and 100MHz bandwidth,

- For UL pose/control-stream with 0.2Mbps_250FPS_10ms, the capacity performance is 20 with SU-MIMO considering limited PDCCH resource.

- For UL scene/video/data/voice-stream with 10Mbps_60FPS_30ms, the capacity performance is 8.59 with SU-MIMO.

Observation 17: In FR2 in Dense Urban with the capacity requirement X=99 and 100MHz bandwidth,

- For UL pose/control-stream with 0.2Mbps_250FPS_10ms, the capacity performance is 20 with SU-MIMO considering limited PDCCH resource.

- For UL scene/video/data/voice-stream with 10Mbps_60FPS_30ms, the capacity performance is 8.30 with SU-MIMO.

  - Capacity Evaluation Result of Multi-stream

    - Downlink I/P-Stream

In this section, the capacity results of downlink multi-stream with I-frame and P-frame are provided, both for slice-based traffic model and GOP-based traffic model. The detailed traffic model parameters of multi-stream I/P frame with 30Mbps can be found Table 5 and Table 6 in section 2.1.2.1. Moreover, the scenarios to be evaluated in this section include Dense Urban for FR1 and InH for FR2. And α representing the ratio of the average frame size of I-frame/slice and P-frame/slice is assumed as 1.5, 2 and 3 for GOP-based model and slice-based model. The GOP length and the number of slices per frame both equal 8. In addition, different QoS requirements between I-stream and P-stream are assumed and described in the notes of the tables.

In this contribution, frame-level model is adopted for slice-based model. The size of each P-frame is equal to the sum of the sizes of the N-1 P-slices. The PER and PDB are defined based on the P-frame containing multiple P-slices. The detailed description and analysis can refer to our company's companion’s contribution [5].

Considering that I-frame is generated by intra-frame coding and has higher importance, which will affect the decoding of the associated P-frames, priority scheduling of I-frames in the buffer is given to each UE. Besides, scheduling algorithm that prioritizes the scheduling of UEs with I-frame in the whole cell is evaluated. Unless otherwise specified, the traditional PF algorithm and MU-MIMO scheduler is applied.

- DU, FR1, DL, I/P-frame multi-stream model, 100MHz bandwidth, DDDSU TDD format

Table 18. System capacity in FR1 DL Dense Urban scenario for I/P-frame traffic model with α = 1.5

| Traffic Model | MU-MIMO |  |  | Traffic Model | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| Option 1A VR/AR slice-based (30Mbps) | 13.78 | 13 | 92.38% | Option 1B VR/AR GOP-based (30Mbps) | 6.74 | 6 | 93.12% | Note 1, Note 4 |
|  | 16.74 | 16 | 91.52% |  | 6.74 | 6 | 93.12% | Note 1, Note 5 |
|  | 16.74 | 16 | 91.52% |  | 6.39 | 6 | 91.67% | Note 1, Note 6 |
|  | 13.93 | 13 | 92.87% |  | 12.58 | 12 | 92.20% | Note 2, Note 4 |
|  | 16.79 | 16 | 91.72% |  | 12.8 | 12 | 92.86% | Note 2, Note 5 |
|  | 16.77 | 16 | 91.62% |  | 12.25 | 12 | 91.14% | Note 2, Note 6 |
|  | 13.27 | 13 | 90.86% |  | 12.39 | 12 | 91.53% | Note 3, Note 4 |
|  | 16.37 | 16 | 90.92% |  | 12.53 | 12 | 92.06% | Note 3, Note 5 |
|  | 16.33 | 16 | 90.82% |  | 12.20 | 12 | 90.87% | Note 3, Note 6 |
| Note 1: [PDB_I Frame, PDB_P Frame] = [10ms, 10ms] Note 2: [PDB_I Frame, PDB_P Frame] = [15ms, 10ms] Note 3: [PDB_I Frame, PDB_P Frame] = [15ms, 9ms] Note 4: [PER_I Frame, PER_P Frame] = [1%, 1%] Note 5: [PER_I Frame, PER_P Frame] = [1%, 5%] Note 6: [PER_I Frame, PER_P Frame] = [0.5%, 5%] |  |  |  |  |  |  |  |  |

Table 19. System capacity in FR1 DL Dense Urban scenario for I/P-frame traffic model with α = 2

| Traffic Model | MU-MIMO |  |  | Traffic Model | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| Option 1A VR/AR slice-based (30Mbps) | 13.69 | 13 | 92.25% | Option 1B VR/AR GOP-based (30Mbps) | 5.20 | 5 | 91.14% | Note 1, Note 4 |
|  | 16.84 | 16 | 91.77% |  | 5.20 | 5 | 91.14% | Note 1, Note 5 |
|  | 16.59 | 16 | 91.27% |  | 4.74 | 4 | 94.84% | Note 1, Note 6 |
|  | 13.54 | 13 | 91.72% |  | 5.35 | 5 | 91.47% | Note 1, Note 4, Note7 |
|  | 16.23 | 16 | 90.77% |  | 5.35 | 5 | 91.47% | Note 1, Note 5, Note7 |
|  | 16.17 | 16 | 90.57% |  | 4.97 | 4 | 90.87% | Note 1, Note 6, Note7 |
|  | 13.73 | 13 | 92.44% |  | 10.06 | 10 | 90.32% | Note 2, Note 4 |
|  | 16.95 | 16 | 91.96% |  | 10.06 | 10 | 90.32% | Note 2, Note 5 |
|  | 16.80 | 16 | 91.67% |  | 9.12 | 9 | 90.40% | Note 2, Note 6 |
|  | 13.36 | 13 | 91.21% |  | 9.19 | 9 | 92.70% | Note 3, Note 4 |
|  | 16.74 | 16 | 91.46% |  | 9.97 | 9 | 92.83% | Note 3, Note 5 |
|  | 16.66 | 16 | 91.36% |  | 8.99 | 8 | 93.55% | Note 3, Note 6 |
| Note 1: [PDB_I Frame, PDB_P Frame] = [10ms, 10ms] Note 2: [PDB_I Frame, PDB_P Frame] = [15ms, 10ms] Note 3: [PDB_I Frame, PDB_P Frame] = [15ms, 9ms] Note 4: [PER_I Frame, PER_P Frame] = [1%, 1%] Note 5: [PER_I Frame, PER_P Frame] = [1%, 5%] Note 6: [PER_I Frame, PER_P Frame] = [0.5%, 5%] Note 7: Priority the scheduling of UEs with I-frame in the whole cell |  |  |  |  |  |  |  |  |

Table 20. System capacity in FR1 DL Dense Urban scenario for I/P-frame traffic model with α = 3

| Traffic Model | MU-MIMO |  |  | Traffic Model | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| Option 1A VR/AR slice-based (30Mbps) | 13.77 | 13 | 92.46% | Option 1B VR/AR GOP-based (30Mbps) | 2.21 | 2 | 92.86% | Note 1, Note 4 |
|  | 16.89 | 16 | 91.67% |  | 2.21 | 2 | 92.86% | Note 1, Note 5 |
|  | 16.89 | 16 | 91.67% |  | 2.09 | 2 | 91.27% | Note 1, Note 6 |
|  | 13.84 | 13 | 92.63% |  | 5.73 | 5 | 93.58% | Note 2, Note 4 |
|  | 16.98 | 16 | 92.06% |  | 5.73 | 5 | 93.75% | Note 2, Note 5 |
|  | 16.89 | 16 | 91.85% |  | 4.91 | 4 | 94.44% | Note 2, Note 6 |
|  | 13.46 | 13 | 91.43% |  | 5.69 | 5 | 93.17% | Note 3, Note 4 |
|  | 16.75 | 16 | 91.54% |  | 5.69 | 5 | 93.17% | Note 3, Note 5 |
|  | 16.72 | 16 | 91.48% |  | 4.84 | 4 | 93.58% | Note 3, Note 6 |
| Note 1: [PDB_I Frame, PDB_P Frame] = [10ms, 10ms] Note 2: [PDB_I Frame, PDB_P Frame] = [15ms, 10ms] Note 3: [PDB_I Frame, PDB_P Frame] = [15ms, 9ms] Note 4: [PER_I Frame, PER_P Frame] = [1%, 1%] Note 5: [PER_I Frame, PER_P Frame] = [1%, 5%] Note 6: [PER_I Frame, PER_P Frame] = [0.5%, 5%] |  |  |  |  |  |  |  |  |

The capacity results of downlink multi-stream with I-frame and P-frame in the FR1 Dense Urban scenario are shown in the tables above.

For slice-based traffic model, the impact of different values of α on capacity can be almost negligible. Comparing note 1 and note 2, it can be observed that increasing the PDB of I-frame from 10ms to 15ms has only a slight increase in capacity, while comparing note 2 and note 3 it can be observed that reducing the PDB of P-frame from 10ms to 9ms has a little decrease in capacity. However, since the change in capacity is minimal for both two cases, they have the same C1 value. In addition, when the PER of the P-frame is increased from 1% to 5%, the capacity performance is significantly improved and the C1 value is increased from 13 to 16 comparing the note4 and note5. Comparing note 5 and note 6, it can be observed that reducing the PER of I-frame from 1% to 0.5% has only a slight decrease in capacity, due to the fact that I-frame in buffer for each UE is always transmitted first.

For GOP-based traffic model, different values of α have a significant impact on capacity, simply, capacity performance decreases as the value of α increases. Furthermore, with increasing the PDB of I-frame from 10ms to 15ms, the capacity performance is significantly improved. While reducing the PDB of P-frame from 10ms to 9ms, some reduction in capacity can be observed. And by analyzing the results of the different PER values it can be found that increasing the PER of P-frame from 1% to 5% results in almost no change in capacity performance. Whereas decreasing the PER of the I-frame from 1% to 0.5% shows a certain reduction in capacity.

Observation 18: In FR1 in Dense Urban, for DL VR/AR with 30Mbps of slice-based multi-stream model,

- the capacity performance is in the range of {13.27~16.79} with MU-MIMO for alpha = 1.5.

- the capacity performance is in the range of {13.54~16.95} with MU-MIMO for alpha = 2.

- the capacity performance is in the range of {13.46~16.98} with MU-MIMO for alpha = 3.

Observation 19: In FR1 in Dense Urban, for DL VR/AR with 30Mbps of GOP-based multi-stream model,

- the capacity performance is in the range of {6.39~12.58} with MU-MIMO for alpha = 1.5.

- the capacity performance is in the range of {4.74~10.06} with MU-MIMO for alpha = 2.

- the capacity performance is in the range of {2.09~5.73} with MU-MIMO for alpha = 3.

- InH, FR2, DL, I/P-frame multi-stream model, 100MHz bandwidth, DDDSU TDD format

Table 21. System capacity in FR2 DL Indoor Hotspot scenario for I/P-frame traffic model with α = 1.5

| Traffic Model | MU-MIMO |  |  | Traffic Model | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| Option 1A VR/AR slice-based (30Mbps) | 8.23 | 8 | 92.53% | Option 1B VR/AR GOP-based (30Mbps) | 5.37 | 5 | 91.20% | Note 1, Note 4 |
|  | 10.61 | 10 | 92.08% |  | 5.43 | 5 | 91.55% | Note 1, Note 5 |
|  | 10.46 | 10 | 91.67% |  | 4.98 | 4 | 93.75% | Note 1, Note 6 |
|  | 8.24 | 8 | 92.71% |  | 7.07 | 7 | 90.34% | Note 2, Note 4 |
|  | 10.77 | 10 | 92.50% |  | 7.43 | 7 | 91.61% | Note 2, Note 5 |
|  | 10.55 | 10 | 91.94% |  | 6.80 | 6 | 93.06% | Note 2, Note 6 |
|  | 8.14 | 8 | 91.67% |  | 6.91 | 6 | 93.98% | Note 3, Note 4 |
|  | 10.51 | 10 | 91.48% |  | 7.11 | 7 | 90.56% | Note 3, Note 5 |
|  | 10.43 | 10 | 91.39% |  | 6.93 | 6 | 94.44% | Note 3, Note 6 |
| Note 1: [PDB_I Frame, PDB_P Frame] = [10ms, 10ms] Note 2: [PDB_I Frame, PDB_P Frame] = [15ms, 10ms] Note 3: [PDB_I Frame, PDB_P Frame] = [15ms, 9ms] Note 4: [PER_I Frame, PER_P Frame] = [1%, 1%] Note 5: [PER_I Frame, PER_P Frame] = [1%, 5%] Note 6: [PER_I Frame, PER_P Frame] = [0.5%, 5%] |  |  |  |  |  |  |  |  |

Table 22. System capacity in FR2 DL Indoor Hotspot scenario for I/P-frame traffic model with α = 2

| Traffic Model | MU-MIMO |  |  | Traffic Model | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| Option 1A VR/AR slice-based (30Mbps) | 8.24 | 8 | 92.71% | Option 1B VR/AR GOP-based (30Mbps) | 3.53 | 3 | 92.01% | Note 1, Note 4 |
|  | 10.73 | 10 | 92.50% |  | 3.87 | 3 | 92.71% | Note 1, Note 5 |
|  | 10.46 | 10 | 91.67% |  | 2.73 | 2 | 93.06% | Note 1, Note 6 |
|  | 8.24 | 8 | 92.71% |  | 5.23 | 5 | 91.15% | Note 2, Note 4 |
|  | 10.72 | 10 | 92.50% |  | 5.52 | 5 | 92.71% | Note 2, Note 5 |
|  | 10.66 | 10 | 92.22% |  | 4.91 | 4 | 94.94% | Note 2, Note 6 |
|  | 8.18 | 8 | 92.01% |  | 4.99 | 4 | 94.68% | Note 3, Note 4 |
|  | 10.38 | 10 | 91.39% |  | 5.33 | 5 | 91.67% | Note 3, Note 5 |
|  | 10.45 | 10 | 91.53% |  | 4.78 | 4 | 94.14% | Note 3, Note 6 |
| Note 1: [PDB_I Frame, PDB_P Frame] = [10ms, 10ms] Note 2: [PDB_I Frame, PDB_P Frame] = [15ms, 10ms] Note 3: [PDB_I Frame, PDB_P Frame] = [15ms, 9ms] Note 4: [PER_I Frame, PER_P Frame] = [1%, 1%] Note 5: [PER_I Frame, PER_P Frame] = [1%, 5%] Note 6: [PER_I Frame, PER_P Frame] = [0.5%, 5%] |  |  |  |  |  |  |  |  |

Table 23. System capacity in FR2 DL Indoor Hotspot scenario for I/P-frame traffic model with α = 3

| Traffic Model | MU-MIMO |  |  | Traffic Model | MU-MIMO |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| Option 1A VR/AR slice-based (30Mbps) | 8.23 | 8 | 92.53% | Option 1B VR/AR GOP-based (30Mbps) | 2.29 | 2 | 93.06% | Note 1, Note 4 |
|  | 10.61 | 10 | 92.08% |  | 2.29 | 2 | 93.06% | Note 1, Note 5 |
|  | 10.38 | 10 | 91.39% |  | 2.03 | 2 | 90.28% | Note 1, Note 6 |
|  | 8.28 | 8 | 93.06% |  | 3.29 | 3 | 91.32% | Note 2, Note 4 |
|  | 10.63 | 10 | 92.22% |  | 3.29 | 3 | 91.32% | Note 2, Note 5 |
|  | 10.55 | 10 | 91.94% |  | 2.68 | 2 | 93.06% | Note 2, Note 6 |
|  | 8.22 | 8 | 92.36% |  | 3.29 | 3 | 90.97% | Note 3, Note 4 |
|  | 10.46 | 10 | 91.49% |  | 3.29 | 3 | 90.97% | Note 3, Note 5 |
|  | 10.48 | 10 | 91.67 |  | 2.68 | 2 | 93.06% | Note 3, Note 6 |
| Note 1: [PDB_I Frame, PDB_P Frame] = [10ms, 10ms] Note 2: [PDB_I Frame, PDB_P Frame] = [15ms, 10ms] Note 3: [PDB_I Frame, PDB_P Frame] = [15ms, 9ms] Note 4: [PER_I Frame, PER_P Frame] = [1%, 1%] Note 5: [PER_I Frame, PER_P Frame] = [1%, 5%] Note 6: [PER_I Frame, PER_P Frame] = [0.5%, 5%] |  |  |  |  |  |  |  |  |

For RF2 Indoor Hotspot scenario, the similar conclusions can be drawn as for FR1 Dense Urban scenario. The major difference is the overall worse capacity performance compared to Dense Urban scenario.

Observation 20: In FR2 in Indoor Hotspot, for DL VR/AR with 30Mbps of slice-based multi-stream model,

- the capacity performance is in the range of {8.14~10.77} with MU-MIMO for alpha = 1.5.

- the capacity performance is in the range of {8.18~10.73} with MU-MIMO for alpha = 2.

- the capacity performance is in the range of {8.22~10.63} with MU-MIMO for alpha = 3.

Observation 21: In FR2 in Indoor Hotspot, for DL VR/AR with 30Mbps of GOP-based multi-stream model,

- the capacity performance is in the range of {4.98~7.43} with MU-MIMO for alpha = 1.5.

- the capacity performance is in the range of {2.73~5.33} with MU-MIMO for alpha = 2.

- the capacity performance is in the range of {2.03~3.29} with MU-MIMO for alpha = 3.

    - Uplink Two-stream

In this section, two-stream (pose/control stream and scene/video/data/audio stream) of UL AR application is simulated in FR1 for capacity evaluation. Only SU-MIMO scheduler is evaluated for UL transmission.

- InH, FR1, UL, 100MHz bandwidth, DDDSU TDD format

Table 24. System capacity in FR1 UL Indoor Hotspot scenario

| Traffic Model | SU-MIMO |  |  |
| --- | --- | --- | --- |
|  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |
| pose/control-stream + scene/video/data/voice-stream (0.2Mbps +10Mbps) | 12.71 | 12 | 93.29% |

- DU, FR1, UL, 100MHz bandwidth, DDDSU TDD format

Table 25. System capacity in FR1 UL Dense Urban scenario

| Traffic Model | SU-MIMO |  |  |
| --- | --- | --- | --- |
|  | Capacity | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 |
| pose/control-stream + scene/video/data/voice-stream (0.2Mbps +10Mbps) | 7.43 | 7 | 92.29% |

For AR application with two streams, a UE is considered as a satisfied UE only when the two streams of data can both meet the reliability requirements. Compared to the results of single-stream in section2.2.2, due to the limited UL resources on the TDD structure of “DDDSU”, dense pose/control stream scheduling will occupy part of system resources and decrease the system capacity.

Observation 22: In FR1 in Indoor Hotspot with the capacity requirement X=99, For UL two-stream with pose/control-stream and scene/video/data/voice -stream(10Mbps), the capacity performance is 12.71 with SU-MIMO.

Observation 23: In FR1 in Dense Urban with the capacity requirement X=99, For UL two-stream with pose/control-stream and scene/video/data/voice-stream(10Mbps), the capacity performance is 7.43 with SU-MIMO.

  - Impact on capacity

    - Impact of data rate

For XR capacity evaluation in 5G NR system, the system capacity is defined as the maximum number of UEs per cell where at least 90% UEs are satisfied UEs. And a UE is declared as the satisfied UE if more than X% of packets are successfully delivered within a given air interface PDB. The packet size is determined by the corresponding data rate and frame rate. For a given frame rate, e.g., 60 FPS, the average packet size increase as the data rate increases. With limited time-frequency resources and multiple UEs competing for resources, the increased packet size makes it more difficult to be successfully delivered within a limited latency budget, e.g., 10ms. From the simulation results in section 2.2.1 and section 2.2.3, it can be observed that the capacity performance is degraded as the data rate increase from 30Mbps to 45Mbps both in FR1 and FR2 downlink transmission.

Table 26. System capacity with different data rates

| Case | Capacity with Data rate = 30 Mbps | Capacity with Data rate = 45 Mbps | Reduction from 30 Mbps to 45 Mbps (%) |
| --- | --- | --- | --- |
| FR1 DL in Indoor Hotspot (MU-MIMO) | 10.80 | 5.91 | 45.28% |
| FR1 DL in Dense Urban (MU-MIMO) | 13.59 | 6.9 | 49.23% |
| FR1 DL in Urban Macro (MU-MIMO) | 8.82 | 4.68 | 46.94% |
| FR2 DL in Indoor Hotspot (SU-MIMO) | 8.72 | 4.67 | 46.44% |
| FR2 DL in Dense Urban (SU-MIMO) | 13.44 | 8.20 | 38.99% |

Observation 24: For VR/AR, 10ms PDB, 60 FPS, with data rate increase from 30Mbps to 45Mbps,

- In FR1 DL in Indoor Hotspot, the capacity performances are decreased by 45.28% from 10.80 to 5.91 with MU-MIMO.

- In FR1 DL in Dense Urban, the capacity performances are decreased by 49.23% from 13.59 to 6.9 with MU-MIMO.

- In FR1 DL in Urban Macro, the capacity performances are decreased by 46.94% from 8.82 to 4.68 with MU-MIMO.

- In FR2 DL in Indoor Hotspot, the capacity performances are decreased by 46.44% from 8.72 to 4.67 with SU-MIMO.

- In FR2 DL in Dense Urban, the capacity performances are decreased by 38.99% from 13.44 to 8.20 with SU-MIMO.

    - Impact of relaxed PDB

Figure 1 shows the system capacity results of CG and VR/AR applications with 30Mbps and 60FPS for FR1 DL in Dense Urban scenario. The PDB is 15ms and 10ms for CG and VR/AR respectively, and MU-MIMO scheduler is assumed. By relaxing PDB from 10ms to 15ms, the system capacity can increase by 6.06 UEs per cell in CG application compared to VR/AR application under the same data rate and frame rate. Similar observation can also be made based on simulation results for other scenarios in section 2.2.1 and section 2.2.3, where the system capacity can be increased by relaxing PDB.

| - UE% | - RU |
| --- | --- |

Figure 1. Impact of relaxed PDB in FR1 DL in Dense Urban scenario

Observation 25: For DL transmission with 30Mbps data rate, by relaxing the PDB from 10ms to 15ms,

- In FR1 DL in Indoor Hotspot, the capacity performances are increased by 22.61% from 8.27 to 10.14 with SU-MIMO and by 50.00% from 10.80 to 16.20 with MU-MIMO.

- In FR1 DL in Dense Urban, the capacity performances are increased by 23.08% from 9.49 to 11.68 with MU-MIMO and by 44.59% from 13.59 to 19.65 with MU-MIMO.

- In FR1 DL in Urban Macro, the capacity performances are increased by 42.68% from 7.24 to 10.33 with MU-MIMO and by 62.47% from 8.82 to 14.33 with MU-MIMO.

- In FR2 DL in Indoor Hotspot, the capacity performances are increased by 19.83% from 8.72 to 9.91 with SU-MIMO.

- In FR2 DL in Dense Urban, the capacity performances are increased by 20.24% from 13.44 to 16.16 with SU-MIMO.

    - Impact of frame rates

For XR application, the left and right eye buffers can be encoded at the same time or at different time. Different traffic resource types correspond to different frame rates, and Figure 2 gives an example of generating dual-eye buffers simultaneously or alternately. As illustrated in Figure 2, for the same data rate, compared with type1 that bundling dual-eye buffers, traffic source type2 is equivalent to changing the FPS of video stream to twice that of type 1, and the packet size of left or right eye is the size of a packet in simulation.

Figure 2. An example of traffic source type with different frame rates

In the simulation, the video frame rate is assumed to be 60 FPS when dual-eye buffers are bundled transmitted by the application server. And the video frame rate is assumed to be 120 FPS for separate packet arrivals in time for dual-eye buffer, which means that dual-eye buffer is sent staggered in time. Based on the evaluation results in section 2.2.1 and section 2.2.3, it can be observed that increasing the frame rate is an effective way to improve the system capacity. Because for the model of separate packet arrivals with 120 FPS, the PDB of each frame transmission is independent, implying that the total PDB for dual-eye buffer transmission is longer.

Table 27. System capacity with different frame rates

| Case | Capacity with 60FPS | Capacity with 120FPS | Increase from 60FPS to 120FPS (%) |
| --- | --- | --- | --- |
| FR1 DL in Indoor Hotspot (MU-MIMO) @30Mbps | 10.80 | 16.53 | 63.99% |
| FR1 DL in Indoor Hotspot (MU-MIMO) @45Mbps | 5.91 | 9.22 | 56.01% |
| FR1 DL in Dense Urban (MU-MIMO) @30Mbps | 13.59 | 20.78 | 52.91% |
| FR1 DL in Dense Urban (MU-MIMO) @45Mbps | 6.91 | 11.42 | 65.27% |
| FR1 DL in Urban Macro (MU-MIMO) @30Mbps | 8.82 | 14.59 | 65.42% |
| FR1 DL in Urban Macro (MU-MIMO) @45Mbps | 4.68 | 8.12 | 73.50% |
| FR2 DL in Indoor Hotspot (SU-MIMO) @30Mbps | 8.72 | 10.23 | 17.32% |
| FR2 DL in Indoor Hotspot (SU-MIMO) @45Mbps | 4.67 | 6.03 | 29.12% |
| FR2 DL in Dense Urban (SU-MIMO) @30Mbps | 13.44 | 16.28 | 21.13% |
| FR2 DL in Dense Urban (SU-MIMO) @45Mbps | 8.20 | 10.32 | 25.85% |

Observation 26: For DL VR/AR transmission, by increasing frame rate from 60FPS to 120FPS,

- In FR1 DL in Indoor Hotspot, the capacity performances are increased by 63.99% from 10.80 to 16.53 for 30Mbps and by 56.01% from 5.91 to 9.22 for 45Mbps with MU-MIMO.

- In FR1 DL in Dense Urban, the capacity performances are increased by 52.91% from 13.59 to 20.78 for 30Mbps and by 65.27% from 6.91 to 11.42 for 45Mbps with MU-MIMO.

- In FR1 DL in Urban Macro, the capacity performances are increased by 65.42% from 8.82 to 14.59 for 30Mbps and by 73.50% from 4.68 to 8.12 for 45Mbps with MU-MIMO.

- In FR2 DL in Indoor Hotspot, the capacity performances are increased by 17.32% from 8.72 to 10.23 for 30Mbps and by 29.12% from 4.67 to 6.03 for 45Mbps with SU-MIMO.

- In FR2 DL in Dense Urban, the capacity performances are increased by 21.13% from 13.44 to 16.28 for 30Mbps and by 25.85% from 8.20 to 10.32 for 45Mbps with SU-MIMO.

    - Impact of scheduling algorithm

Considering stringent PDB requirement relative to high data rate, delay-aware (DA) scheduler taking time factor into account is proposed. Based on the traditional PF scheduler, the DA scheduler prioritizes UEs approaching the deadline by adding time factor to the weight, at the cost of using more system resources. As presented in section 2.2.1 and section 2.2.3, the capacity performance is simulated in FR1 DL and FR2 DL to evaluate different types of scheduling algorithm, where CG application with 30Mbps_60FPS_15ms and VR/AR application with 30Mbps_60FPS_10ms are evaluated. As shown from the results, the DA scheduler can support a larger percentage of satisfied UEs and improve system capacity, especially when the #UE per cell is small and the system resources are sufficient.

Table 28. System capacity with different scheduling algorithm @ 30Mbps data rate

| Case | Traffic Model | PF scheduler | DA scheduler | Increase from PF to DA scheduler (%) |
| --- | --- | --- | --- | --- |
| FR1 DL in Indoor Hotspot (MU-MIMO) | CG | 16.20 | 16.67 | 2.90% |
|  | VR/AR | 10.80 | 12.40 | 14.81% |
| FR1 DL in Dense Urban (MU-MIMO) | CG | 19.65 | 19.75 | 0.51% |
|  | VR/AR | 13.59 | 14.40 | 5.96% |
| FR1 DL in Urban Macro (MU-MIMO) | CG | 14.33 | 14.45 | 0.84% |
|  | VR/AR | 8.82 | 9.55 | 8.28% |
| FR1 DL in Indoor Hotspot (SU-MIMO) | CG | 10.14 | 11.43 | 12.72% |
|  | VR/AR | 8.27 | 10.77 | 30.23% |
| FR1 DL in Dense Urban (SU-MIMO) | CG | 11.68 | 13.58 | 16.27% |
|  | VR/AR | 9.49 | 12.67 | 33.51% |
| FR1 DL in Urban Macro (SU-MIMO) | CG | 10.33 | 11.94 | 15.59% |
|  | VR/AR | 7.24 | 8.56 | 18.23% |
| FR2 DL in Indoor Hotspot (SU-MIMO) | CG | 9.91 | 10.23 | 3.23% |
|  | VR/AR | 8.72 | 8.83 | 1.26% |
| FR2 DL in Dense Urban (SU-MIMO) | CG | 16.16 | 16.82 | 4.08% |
|  | VR/AR | 13.44 | 14.16 | 5.36% |

Observation 27: Compared to typical PF scheduler, delay-aware scheduler can increase the system capacity,

- In FR1 DL in Indoor Hotspot, the capacity performances are increased by 2.90% from 16.20 to 16.67 for CG 30Mbps and by 14.81% from 10.80 to 12.40 for VR/AR 30Mbps with MU-MIMO.

- In FR1 DL in Dense Urban, the capacity performances are increased by 0.51% from 19.65 to 19.75 for CG 30Mbps and by 5.96% from 13.59 to 14.40 for VR/AR 30Mbps with MU-MIMO.

- In FR1 DL in Urban Macro, the capacity performances are increased by 0.84% from 14.33 to 14.45 for CG 30Mbps and by 8.28% from 8.82 to 9.55 for VR/AR 30Mbps with MU-MIMO.

- In FR1 DL in Indoor Hotspot, the capacity performances are increased by 12.72% from 10.14 to 11.43 for CG 30Mbps and by 30.23% from 8.27 to 10.77 for VR/AR 30Mbps with SU-MIMO.

- In FR1 DL in Dense Urban, the capacity performances are increased by16.27% from 11.68 to 13.58 for CG 30Mbps and by 33.51% from 9.49 to 12.67 for VR/AR 30Mbps with SU-MIMO.

- In FR1 DL in Urban Macro, the capacity performances are increased by 15.59% from 10.33 to 11.94 for CG 30Mbps and by 18.23% from 7.24 to 8.56 for VR/AR 30Mbps with SU-MIMO.

- In FR2 DL in Indoor Hotspot, the capacity performances are increased by 3.23% from 9.91 to 10.23 for CG 30Mbps and by 1.26% from 8.72 to 8.83 for VR/AR 30Mbps with SU-MIMO.

- In FR2 DL in Dense Urban, the capacity performances are increased by 4.08% from 16.16 to 16.82 for CG 30Mbps and by 5.36% from 13.44 to 14.16 for VR/AR 30Mbps with SU-MIMO.

    - Impact of 400MHz and 100MHz bandwidth in FR2

The impact of bandwidth increasing from 100MHz to 400MHz is presented in Figure 3. The simulation is performed for VR/AR application with 45Mbps_60FPS_10ms for FR2 DL in Dense Urban scenario. As expected, increasing bandwidth can make a significant improvement in system capacity because of increased resources for transmission. For instance, with 99% reliability requirement, the capacity is 8 for 100MHz and 43.89 for 400MHz bandwidth, which can be observed from Figure 3.

| - UE% | - RU |
| --- | --- |

Figure 3. Impact of bandwidth in FR2 DL in Dense Urban scenario

Observation 28: In FR2 in Dense Urban, for DL VR/AR with 45Mbps_60FPS_10ms, by increasing bandwidth from 100MHz to 400Mhz, the capacity performance increases by 435.24% from 8.20 to 43.89.

    - Impact of size ratio and QoS requirements between I frame and P frame

Based on the simulation results in section 2.3.1, the impact of size ratio and QoS requirements between I-frame/slice and P-frame/slice is further analyzed, both for slice-based traffic model and GOP-based traffic model. The single DL video stream with 10ms PDB and 99% reliability requirements is also provided for reference. For single-stream model, I-frame and P-frame are not modeled, and the traffic characteristics correspond to the 30Mbps data rate of VR/AR. The detailed traffic model parameters of multi-stream I/P frame with 30Mbps can be found Table 5 and Table 6 in section 2.1.2.1. The combinations of different PDB and PER values for slice-based and GOP-based traffic model are shown in Table 29.

Table 29. The combinations of different PDB and PER values for I/P-Frame multi-stream model

| Case | PER and PDB value |
| --- | --- |
| Single_stream | [PER] = [1%], [PDB] = [10ms] |
| Case 1 | [PER_I, PER_P] = [1%, 1%], [PDB_I, PDB_P] = [10ms, 10ms] |
| Case 2 | [PER_I, PER_P] = [1%, 5%], [PDB_I, PDB_P] = [10ms, 10ms] |
| Case 3 | [PER_I, PER_P] = [0.5%, 5%], [PDB_I, PDB_P] = [10ms, 10ms] |
| Case 4 | [PER_I, PER_P] = [1%, 1%], [PDB_I, PDB_P] = [15ms, 10ms] |
| Case 5 | [PER_I, PER_P] = [1%, 5%], [PDB_I, PDB_P] = [15ms, 10ms] |
| Case 6 | [PER_I, PER_P] = [0.5%, 5%], [PDB_I, PDB_P] = [15ms, 10ms] |
| Case 7 | [PER_I, PER_P] = [1%, 1%], [PDB_I, PDB_P] = [15ms, 9ms] |
| Case 8 | [PER_I, PER_P] = [1%, 5%], [PDB_I, PDB_P] = [15ms, 9ms] |
| Case 9 | [PER_I, PER_P] = [0.5%, 5%], [PDB_I, PDB_P] = [15ms, 10ms] |

      - GOP based

The system evaluation results of capacity for GOP-based I/P-frame traffic model are presented as follows, where MU-MIMO scheduler is applied and GOP size K = 8. The performance of single stream traffic model, which can be deemed as multiple stream traffic model with α=1, is also provided for reference.

Figure 4. Impact of size ratio for GOP-based I/P-frame traffic model in FR1 Dense Urban

This cluster includes:

[15ms, 10ms], [1%, 1%]

[15ms, 10ms], [1%, 5%]

[15ms, 9ms], [1%, 1%]

[15ms, 9ms], [1%, 5%]

Figure 5. Impact of QoS requirements for GOP-based I/P-frame traffic model in FR1 Dense Urban

For GOP-based I/P-frame traffic model, I-frame and P-frame are not generated at the same time, and the average packet size of I-frame is larger. With limited resources, the packet size is a key factor that limits system capacity.

By comparing the results for the different α values in Figure 4, it can be seen that the capacity performances significantly decrease for the cases with α equal to 3, compared to α=1.5 or 2, which is caused by the relatively larger data size of I-frame.

The impact of QoS requirements for I/P-frame on capacity performance is evaluated as in Figure 5. It can be observed that relaxing the PDB of I-frames from 10ms to 15ms can improve the system capacity, with the PDB=10ms for P-frame and the same PER requirements for I/P-frames. With the same PDB requirements for I/P-frames, there is almost the same capacity performance by relaxing the PER of P-frames from 1% to 5%, while minor performance degradation on the system capacity can be observed for I-frames from 0.5% to 1%.

From the above analysis, it is proved that for the GOP-based model, the main factors dominating the system performance are the average packet size of I-frame and the KPI of I-frame.

Figure 6. Impact of scheduling algorithm for GOP-based I/P-frame traffic model in FR1 Dense Urban

Then Figure 6 evaluates the impact of different scheduling rule for I-frame. For PF scheduling, all the packets of I-frame and P-frame are scheduled based on the PF metric with the same priority. For prioritizing priority scheduling of I-frame, UEs with packets of I-frame in the whole cell will be prioritized for scheduling. From the Figure 6, it can be seen that with α = 2 and [PDB_I, PDB_P] = [10ms, 10ms], priority scheduling for I-frame can improve system capacity by 2.88% from 5.20 to 5.35 for [PER_I, PER_P] =[1%, 5%], and by 4.85% from 4.74 to 4.97 for [PER_I, PER_P] = [0.5%, 5%], respectively.

Observation 29: Compared the value of α equal to 3, reducing the value of α can lead to smaller average packet size of I-frame such that improve the system capacity for GOP-based I/P-frame model.

Observation 30: Compared the PDB = 10 ms, relaxing PDB of I-frame to 15 ms can improve the system capacity for GOP-based I/P-frame model.

Observation 31: Prioritizing scheduling of I-frame can improve the system capacity for GOP-based I/P-frame model, compared to the typical PF scheduling.

      - Slice based

The system evaluation results of capacity for slice-based I/P-frame traffic model are presented in Figure 7 and Figure 8, where MU-MIMO is applied and the number of slices per frame equals 8.

Figure 7. Impact of size ratio for slice-based I/P-frame traffic model in FR1 Dense Urban

Figure 8. Impact of QoS requirements for slice-based I/P-frame traffic model in FR1 Dense Urban

For slice-based I/P-frame traffic model, the packets of I-frame or P-frame for a video frame are generated at the same time.

As seen from Figure 7, slice-based I/P-frame traffic model with different α values can achieve almost the same capacity performances. Moreover, Figure 8 compares the impact of different Qos requirements on system capacity when α=2. It can be observed that increasing the PDB of I-frame from 10ms to 15ms has almost no impact on capacity, while reducing the PDB of P-frame from 10ms to 9ms has a little decrease in capacity. In addition, when the PER of the P-frame is increased from 1% to 5%, the capacity performance is significantly improved and the C1 value of system capacity is increased from 13 to 16. Moreover, it can be observed that reducing the PER of I-frame from 1% to 0.5% has only a slight decrease in capacity. Therefore, the PER/PDB requirements of P-frames for slice-based I/P-frame traffic model are dominant on the capacity performance.

Observation 32: When I-frame and P-frame adopt the same PER and PDB values, the impact of different α values on system capacity can be almost negligible for slice-based I/P-frame model.

Observation 33: Relaxing PER of P-frame can improve capacity performance for slice-based I/P-frame model.

- UE Power Consumption Evaluation

As analyzed in our companion’s contribution [6], power consumption is the crucial issue for XR traffic and it is another key metric for XR evaluation, since the specific characteristics of XR traffic i.e., dense traffic period, high traffic data rates and low latency requirement, etc. which is different from eMBB and URLLC. For power consumption evaluation, UE capacity performance should be jointly taken into consideration. Besides, in line with the agreement in RAN1#104b-e and RAN1#105-e, the following assumptions are adopted in performance evaluation of power consumption.

- The power consumption results for both all UEs and only satisfied UEs are counted separately to help comprehensively evaluate XR power performance.

- For XR/CG power consumption evaluation, for DL and UL, both Option 1(i.e., DL and UL performances are evaluated independently…) and Option 3 (i.e., DL and UL performances are evaluated together…) are evaluated.

- The linear interpolation method is adopted in a linear scale for UE Tx power values other than 0 dBm and 23 dBm.

- For FR2 UL, the fixed value 350 listed in TR38.840 [7] is assumed as the relative power of the UL state (long PUCCH or PUSCH).

- The same number of UE per cell are used in baseline and power saving schemes where the max UEs/cell at which UE can meet the capacity KPI is reported as high load case, and approximately 50% of max UEs/cell is evaluated as low load case.

In this contribution, AlwaysOn (i.e., without adopting any power saving mechanism) is the baseline scheme. And CDRX configuration adopted in R15/R16 is evaluated as a power saving scheme. In addition, the potential enhancement of CDRX and the ongoing power saving schemes in R17 power saving WI, such as PDCCH skipping, search space set group switching mechanisms are also evaluated to contrastive analysis of the potential impact on power consumption and corresponding system capacity. In this section, power consumption of different schemes is evaluated by using downlink and uplink traffic models with 60 FPS. Table 30 gives the adopted traffic models and corresponding scenarios for power consumption evaluation.

Table 30. Traffic model and scenario combinations used for power consumption evaluation

| Option 1: DL and UL performances are evaluated independently and DL and UL power consumption results are collected separately. |  |
| --- | --- |
| DL FR1 - Indoor Hotspot/Dense Urban/Urban Macro scenario: - VR/AR video stream: 30/45Mbps, 60FPS, PDB=10ms | DL FR2 - Indoor Hotspot/Dense Urban scenario: - VR/AR video stream: 30/45Mbps, 60FPS, PDB=10ms |
| UL FR1 - Indoor Hotspot/Dense Urban/Urban Macro scenario: - Single stream: - Pose/control stream - Indoor Hotspot/Dense Urban scenario: - Single stream: - Video stream - Two streams: - Pose/control stream + video stream | UL FR2 - Indoor Hotspot/Dense Urban scenario: - Single stream: - Pose/control stream - Video stream |
| Option 3: DL and UL performances are evaluated together. DL and UL power consumption are counted to obtain the total power consumption |  |
| DL and UL transmission together in FR1 - Indoor Hotspot/Dense Urban scenario: - VR: DL video (30Mbps_60FPS_10ms), UL pose/control stream (0.2Mbps) - AR with UL single-stream: DL video (30Mbps_60FPS_10ms), UL video stream (10Mbps_60FPS_30ms) - AR with UL two-stream: DL video (30Mbps), UL pose/control stream (0.2Mbps) + video stream (10Mbps) |  |

  - FR1

    - Downlink

For downlink VR/AR application, in addition to no DRX configuration (i.e., AlwaysOn), R15/16 CDRX configurations (i.e., R15/16 CDRX) with different sets of parameters are evaluated, which are captured in Table 31. Note that 10ms is the minimum long DRX cycle configuration supported by the current specification.

Due to non-integer XR traffic periodicity, the existing integer DRX cycle cannot align with the traffic arrival and it will increase data transmission delay. To guarantee system capacity performance, the longer drx-onDurationTimer is preferable to reduce waiting time, which will inevitably damage the power saving gain (PSG). To solve this issue, a straightforward method is to introduce some non-integer values of DRX cycle to align with the XR traffic periodicity. But it is not flexible because of the various traffic period and the jitter effect of DL burst arrival.

To solve the impact of jitter and align DRX cycle with traffic arrival, the adjustment to the start offset of CDRX ON Duration could be a potential solution, which is an enhancement of the CDRX scheme (i.e., e-CDRX) and is equivalent to aligning DRX cycle with the period of XR traffic. Noted that different positions of the start offset alignment can result in different trade-offs between system capacity and power saving gain. Three different alignment positions and corresponding CDRX configurations are evaluated, which are presented in Table 31. In the scheme of alignment to negative jitter boundary and the quasi-period arrival time point, jitter is both unaware. In the scheme of alignment to the actual packet arrival time, jitter is assumed aware by jitter handling to evaluate the maximum power saving gain.

Additionally, PDCCH monitoring adaptation mechanism can also make a better trade-off between power saving and capacity. Due to the existence of jitter, the packets may arrive at any time within the range of [-4ms, 4ms] around the quasi-periodic time. Since jitter is unpredictable, the most conservative solution to ensure system capacity is to indicate UE to wake up for PDCCH monitoring at the negative jitter boundary of packet arrival time, which is the earliest possible arrival time of UE packets during one packet period. Once the packet is transmitted and no more packets are waiting to be transmitted, gNB can be enabled to send the PDCCH skipping command to indicate UE to skip PDCCH monitoring until negative jitter boundary of the next packet.

Compared to the XR packet generation period, e.g., 16.67ms, the jitter range of 8ms is still relatively wide. More power can be saved if jitter handling is added to PDCCH monitoring adaptation mechanism, for example, by using the WUS signal to indicate whether the UE wakes up or not. For simplicity, we assume that jitter is aware after jitter handling and the UE can skip PDCCH monitoring until the actual packet arrival time, where the maximum power saving gain and no damage on system capacity can be observed by utilizing PDCCH monitoring adaptation with jitter handling.

Based on the assumptions and configurations mentioned above, the UE capacity and power consumption results in DL are shown in the following tables, where the PSG is calculated compared to the baseline scheme (without adopting any power saving mechanism). The triplets of numbers (CDRX cycle- On duration timer- Inactivity timer) are used to note various CDRX parameters. And the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB. The mean power consumption shown in the following tables is the average power consumption within each slot.

- AlwaysOn (Baseline): Without adopting any power saving mechanism.

- R15/16 CDRX: adopting R15/R16 DRX configuration.

- e-CDRX with adapting to the lower bound of jitter range: aligning the start time of DRX ON Duration with the negative jitter boundary.

- e-CDRX with adapting to quasi-period position: aligning the start time of DRX ON Duration with quasi-period arrival time point.

- e-CDRX with jitter handling: aligning the start time of DRX ON Duration with the position of the actual packet arrival time.

- R17 PDCCH monitoring adaptation without jitter handling: UE can skip PDCCH monitoring to the negative jitter boundary.

- PDCCH monitoring adaptation with jitter handling: UE can skip PDCCH monitoring to the actual packet arrival time.

Table 31. The configuration for R15/16 CDRX and e-CDRX

| Cases | DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer(ms) |
| --- | --- | --- | --- |
| R15/16 CDRX | 10 | 8 | 4 |
| R15/16 CDRX | 16 | 14 | 4 |
| e-CDRX with adapting to the lower bound of jitter range | 16 | 10 | 4 |
| e-CDRX with adapting to quasi-period position | 16 | 6 | 4 |
| e-CDRX with jitter handling | 16 | 3 | 3 |

- InH, FR1, DL, 100MHz bandwidth, DDDSU TDD format

Table 32 Power consumption results in FR1 DL Indoor Hotspot scenario

| Traffic Model | Power Saving scheme | avg # UEs/ cell = N1 | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell = N1 | Average PS gain (%) of all UEs | Average PS gain (%) of satisfied UEs | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VR/AR (30Mbps) | AlwaysOn - baseline | 5 | 10 | 100% | - | - |  |
|  | R15/16CDRX (10-8-4) | 5 | 10 | 100% | 5.72% | 5.72% |  |
|  | R15/16CDRX (16-14-4) | 5 | 10 | 100% | 3.67% | 3.67% |  |
|  | eCDRX (16-10-4) | 5 | 10 | 100% | 13.05% | 13.05% | Note 1 |
|  | eCDRX (16-6-4) | 5 | 10 | 100% | 28.38% | 28.38% | Note 2 |
|  | eCDRX (16-3-3) | 5 | 10 | 100% | 35.35% | 35.13% | Note 3 |
|  | PDCCH monitoring adaptation | 5 | 10 | 100% | 23.33% | 23.33% | Note 4 |
|  | PDCCH monitoring adaptation | 5 | 10 | 100% | 48.88% | 48.88% | Note 5 |
|  | AlwaysOn - baseline | 10 | 10 | 92.50% | - | - |  |
|  | R15/16CDRX (10-8-4) | 10 | 10 | 91.25% | 4.88% | 5.10% |  |
|  | R15/16CDRX (16-14-4) | 10 | 10 | 91.81% | 3.24% | 3.46% |  |
|  | eCDRX (16-10-4) | 10 | 10 | 91.94% | 9.36% | 10.08% | Note 1 |
|  | eCDRX (16-6-4) | 10 | 10 | 91.25% | 23.84% | 24.11% | Note 2 |
|  | eCDRX (16-3-3) | 10 | 10 | 91.67% | 29.06% | 30.56% | Note 3 |
|  | PDCCH monitoring adaptation | 10 | 10 | 92.17% | 20.73% | 21.35% | Note 4 |
|  | PDCCH monitoring adaptation | 10 | 10 | 92.50% | 41.03% | 42.26% | Note 5 |
| VR/AR (45Mbps) | AlwaysOn - baseline | 3 | 5 | 100% | - | - |  |
|  | R15/16CDRX (10-8-4) | 3 | 5 | 100% | 5.32% | 5.32% |  |
|  | R15/16CDRX (16-14-4) | 3 | 5 | 100% | 3.46% | 3.46% |  |
|  | eCDRX (16-10-4) | 3 | 5 | 100% | 11.96% | 11.96% | Note 1 |
|  | eCDRX (16-6-4) | 3 | 5 | 100% | 26.74% | 26.74% | Note 2 |
|  | eCDRX (16-3-3) | 3 | 5 | 100% | 35.09% | 34.84% | Note 3 |
|  | PDCCH monitoring adaptation | 3 | 5 | 100% | 22.16% | 22.16% | Note 4 |
|  | PDCCH monitoring adaptation | 3 | 5 | 100% | 45.32% | 45.32% | Note 5 |
|  | AlwaysOn - baseline | 5 | 5 | 96.67% | - | - |  |
|  | R15/16CDRX (10-8-4) | 5 | 5 | 92.78% | 4.68% | 5.31% |  |
|  | R15/16CDRX (16-14-4) | 5 | 5 | 94.44% | 2.83% | 3.14% |  |
|  | eCDRX (16-10-4) | 5 | 5 | 96.67% | 9.42% | 9.93% | Note 1 |
|  | eCDRX (16-6-4) | 5 | 5 | 93.89% | 22.61% | 23.62% | Note 2 |
|  | eCDRX (16-3-3) | 5 | 5 | 94.44% | 29.12% | 29.96% | Note 3 |
|  | PDCCH monitoring adaptation | 5 | 5 | 96.67% | 19.89% | 20.26% | Note 4 |
|  | PDCCH monitoring adaptation | 5 | 5 | 96.67% | 38.76% | 39.07% | Note 5 |
| Note 1: e-CDRX with adapting to the lower bound of jitter range Note 2: e-CDRX with adapting to quasi-period position Note 3: e-CDRX with jitter handling Note 4: R17 PDCCH monitoring adaptation without jitter handling Note 5: PDCCH monitoring adaptation with jitter handling |  |  |  |  |  |  |  |

Observation 34: In FR1 DL in Indoor Hotspot, for R15/16 CDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {3.67%~5.72%} with no satisfied UE loss for low load, and {3.24%~4.88%} with {0.69~1.25%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {3.46%~5.32%} with no satisfied UE loss for low load, and {3.14%~5.31%} with {2.23%~3.89%} of satisfied UE loss for high load.

Observation 35: In FR1 DL in Indoor Hotspot, for eCDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {13.05%~35.35%} with no satisfied UE loss for low load, and {9.36%~29.06%} with {0.56%~1.25%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {11.96%~35.09%} with no satisfied UE loss for low load, and {9.42%~29.12%} with {0%~2.78%} of satisfied UE loss for high load.

Observation 36: In FR1 DL in Indoor Hotspot, for PDCCH monitoring adaptation scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {23.33%~48.88%} with no satisfied UE loss for low load, and {20.73%~41.03} with {0%~0.33%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {22.16%~45.32%} with no satisfied UE loss for low load, and {19.89%~38.76%} with no satisfied UE loss for high load.

- DU, FR1, DL, 100MHz bandwidth, DDDSU TDD format

Table 33 Power consumption results in FR1 DL Dense Urban scenario

| Traffic Model | Power Saving scheme | avg # UEs/ cell = N1 | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell = N1 | Average PS gain (%) of all UEs | Average PS gain (%) of satisfied UEs | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VR/AR (30Mbps) | AlwaysOn - baseline | 7 | 13 | 100% | - | - |  |
|  | R15/16CDRX (10-8-4) | 7 | 13 | 100% | 5.57% | 5.57% |  |
|  | R15/16CDRX (16-14-4) | 7 | 13 | 100% | 3.65% | 3.65% |  |
|  | eCDRX (16-10-4) | 7 | 13 | 100% | 12.49% | 12.50% | Note 1 |
|  | eCDRX (16-6-4) | 7 | 13 | 100% | 27.49% | 27.49% | Note 2 |
|  | PDCCH monitoring adaptation | 7 | 13 | 100% | 22.65% | 22.65% | Note 3 |
|  | PDCCH monitoring adaptation | 7 | 13 | 100% | 47.38% | 47.38% | Note 4 |
|  | AlwaysOn - baseline | 13 | 13 | 92.43% | - | - |  |
|  | R15/16CDRX (10-8-4) | 13 | 13 | 90.11% | 4.70% | 5.48% |  |
|  | R15/16CDRX (16-14-4) | 13 | 13 | 91.58% | 3.03% | 3.46% |  |
|  | eCDRX (16-10-4) | 13 | 13 | 91.70% | 8.67% | 10.06% | Note 1 |
|  | eCDRX (16-6-4) | 13 | 13 | 91.21% | 21.72% | 23.59% | Note 2 |
|  | PDCCH monitoring adaptation | 13 | 13 | 92.43% | 18.86% | 19.79% | Note 3 |
|  | PDCCH monitoring adaptation | 13 | 13 | 92.43% | 37.83% | 40.05% | Note 4 |
| VR/AR (45Mbps) | AlwaysOn - baseline | 3 | 6 | 100% | - | - |  |
|  | R15/16CDRX (10-8-4) | 3 | 6 | 100% | 5.56% | 5.56% |  |
|  | R15/16CDRX (16-14-4) | 3 | 6 | 100% | 3.53% | 3.53% |  |
|  | eCDRX (16-10-4) | 3 | 6 | 98.94% | 12.61% | 12.61% | Note 1 |
|  | eCDRX (16-6-4) | 3 | 6 | 99.47% | 27.26% | 27.45% | Note 2 |
|  | PDCCH monitoring adaptation | 3 | 6 | 100% | 21.95% | 21.95% | Note 3 |
|  | PDCCH monitoring adaptation | 3 | 6 | 100% | 45.19% | 45.19% | Note 4 |
|  | AlwaysOn - baseline | 6 | 6 | 95.63% | - | - |  |
|  | R15/16CDRX (10-8-4) | 6 | 6 | 93.12% | 4.69% | 5.11% |  |
|  | R15/16CDRX (16-14-4) | 6 | 6 | 94.18% | 3.10% | 3.37% |  |
|  | eCDRX (16-10-4) | 6 | 6 | 95.63% | 9.72% | 10.28% | Note 1 |
|  | eCDRX (16-6-4) | 6 | 6 | 94.18% | 22.95% | 23.95% | Note 2 |
|  | PDCCH monitoring adaptation | 6 | 6 | 94.44% | 18.73% | 19.15% | Note 3 |
|  | PDCCH monitoring adaptation | 6 | 6 | 95.63% | 37.26% | 38.53% | Note 4 |
| Note 1: e-CDRX with adapting to the lower bound of jitter range Note 2: e-CDRX with adapting to quasi-period position Note 3: R17 PDCCH monitoring adaptation without jitter handling Note 4: PDCCH monitoring adaptation with jitter handling |  |  |  |  |  |  |  |

Observation 37: In FR1 DL in Dense Urban, for R15/16CDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {3.65%~5.57%} with no satisfied UE loss for low load, and {3.03%~4.70%} with {0.85%~2.32%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {3.53%~5.56%} with no satisfied UE loss for low load, and {3.10%~4.69%} with {1.45%~2.51%} of satisfied UE loss for high load.

Observation 38: In FR1 DL in Dense Urban, for eCDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {12.49%~27.49%} with no satisfied UE loss for low load, and {8.67%~21.72%} with {0.73%~1.22%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {12.61%~27.26%} with {0.53%~1.06%} of satisfied UE loss for low load, and {9.72%~22.95%} with {0%~1.45%} of satisfied UE loss for high load.

Observation 39: In FR1 DL in Dense Urban, for PDCCH monitoring adaptation scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {22.65%~47.38%} with no satisfied UE loss for low load, and {18.86%~37.83%} with no satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {21.95%~45.19%} with no satisfied UE loss for low load, and {18.73%~37.26%} with {0%~1.19%} of satisfied UE loss for high load.

- Uma, FR1, DL, 100MHz bandwidth, DDDSU TDD format

Table 34 Power consumption results in FR1 DL Urban Macro scenario

| Traffic Model | Power Saving scheme | avg # UEs/ cell = N1 | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell = N1 | Average PS gain (%) of all UEs | Average PS gain (%) of satisfied UEs | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VR/AR (30Mbps) | AlwaysOn - baseline | 4 | 8 | 98.81% | - | - |  |
|  | R15/16CDRX (10-8-4) | 4 | 8 | 98.41% | 6.26% | 6.56% |  |
|  | R15/16CDRX (16-14-4) | 4 | 8 | 98.81% | 4.05% | 4.08% |  |
|  | eCDRX (16-10-4) | 4 | 8 | 98.81% | 13.09% | 13.19% | Note 1 |
|  | eCDRX (16-6-4) | 4 | 8 | 97.22% | 29.06% | 29.48% | Note 2 |
|  | PDCCH monitoring adaptation | 4 | 8 | 98.81% | 25.15% | 25.26% | Note 3 |
|  | PDCCH monitoring adaptation | 4 | 8 | 98.81% | 47.13% | 47.32% | Note 4 |
|  | AlwaysOn - baseline | 8 | 8 | 93.75% | - | - |  |
|  | R15/16CDRX (10-8-4) | 8 | 8 | 91.47% | 5.02% | 5.57% |  |
|  | R15/16CDRX (16-14-4) | 8 | 8 | 92.85% | 3.23% | 3.59% |  |
|  | eCDRX (16-10-4) | 8 | 8 | 93.35% | 10.50% | 11.12% | Note 1 |
|  | eCDRX (16-6-4) | 8 | 8 | 91.87% | 23.33% | 25.23% | Note 2 |
|  | PDCCH monitoring adaptation | 8 | 8 | 93.75% | 20.54% | 21.23% | Note 3 |
|  | PDCCH monitoring adaptation | 8 | 8 | 93.75% | 40.59% | 41.61% | Note 4 |
| VR/AR (45Mbps) | AlwaysOn - baseline | 2 | 4 | 96.83% | - | - |  |
|  | R15/16CDRX (10-8-4) | 2 | 4 | 96.83% | 5.81% | 5.95% |  |
|  | R15/16CDRX (16-14-4) | 2 | 4 | 96.83% | 3.97% | 4.04% |  |
|  | eCDRX (16-10-4) | 2 | 4 | 96.83% | 12.09% | 12.39% | Note 1 |
|  | eCDRX (16-6-4) | 2 | 4 | 96.83% | 27.33% | 28.07% | Note 2 |
|  | PDCCH monitoring adaptation | 2 | 4 | 96.83% | 23.25% | 23.89% | Note 3 |
|  | PDCCH monitoring adaptation | 2 | 4 | 96.83% | 43.26% | 43.71% | Note 4 |
|  | AlwaysOn - baseline | 4 | 4 | 94.05% | - | - |  |
|  | R15/16CDRX (10-8-4) | 4 | 4 | 92.46% | 4.92% | 5.39% |  |
|  | R15/16CDRX (16-14-4) | 4 | 4 | 93.25% | 3.13% | 3.32% |  |
|  | eCDRX (16-10-4) | 4 | 4 | 94.05% | 9.86% | 10.46% | Note 1 |
|  | eCDRX (16-6-4) | 4 | 4 | 91.67% | 23.59% | 25.27% | Note 2 |
|  | PDCCH monitoring adaptation | 4 | 4 | 93.33% | 20.17% | 20.52% | Note 3 |
|  | PDCCH monitoring adaptation | 4 | 4 | 94.05% | 37.18% | 37.95% | Note 4 |
| Note 1: e-CDRX with adapting to the lower bound of jitter range Note 2: e-CDRX with adapting to quasi-period position Note 3: R17 PDCCH monitoring adaptation without jitter handling Note 4: PDCCH monitoring adaptation with jitter handling |  |  |  |  |  |  |  |

Observation 40: In FR1 DL in Urban Macro, for R15/16CDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {4.05%~6.26%} with {0%~0.40%} of satisfied UE loss for low load, and {3.23~5.02%} with {0.90%~2.28%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {3.97%~5.81%} with no satisfied UE loss for low load, and {3.13%~4.92%} with {0.80%~1.59%} of satisfied UE loss for high load.

Observation 41: In FR1 DL in Urban Macro, for eCDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {13.09%~29.06%} with {0%~1.59%} of satisfied UE loss for low load, and {10.50%~23.33%} with {0.40%~1.88%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {12.09%~27.33%} with no satisfied UE loss for low load, and {9.86%~23.59%} with {0%~2.38%} of satisfied UE loss for high load.

Observation 42: In FR1 DL in Urban Macro, for PDCCH monitoring adaptation scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {25.15%~47.13%} with no satisfied UE loss for low load, and {20.54%~40.59%} with no satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {23.25%~43.26%} with no satisfied UE loss for low load, and {20.17%~37.18%} with {0%~0.72%} of satisfied UE loss for high load.

Based on the simulation results given above, by using R15/16 CDRX, to ensure achieving the requirement of capacity, the values of drx-onDurationTimer and drx-InactivityTimer should be larger enough to provide sufficient scheduling, transmission opportunities and cover the jitter range (i.e. [-4, 4]ms). Thus, only about 4.68%~6.26% PSG can be obtained by CDRX with 10ms DRX cycle. Moreover, setting a 16 ms DRX cycle that is close to traffic period (e.g.,16.67ms) couldn’t solve the mismatch between the non-integer traffic period and the DRX cycle. A longer drx-onDurationTimer also needs to be set to guarantee the system capacity and only 2.83%~4.05% PSG can be obtained.

Compared to the R15/R16 CDRX configuration, both enhanced CDRX and PDCCH monitoring adaptation scheme can achieve a significant mean PSG. For e-CDRX scheme, aligning start offset with negative jitter boundary has no capacity loss, but the conservative on_duration timer configuration results in limited power saving gain of the system, where 8.67% ~13.09% PSG can be observed. By aligning start offset with quasi-period, packets that arrive earlier than the quasi-period time point have to wait for scheduling, which can be a little detrimental to system performance but also results in higher power savings gains. And aligning start offset with the position of the actual packet arrival time can get the maximum power savings gain. The adjustment of the start offset position make a trade-off between system performance and power savings gain.

Similar to e-CDRX scheme, aligning start offset with negative jitter boundary, PDCCH monitoring adaptation scheme can only achieve 18.73% ~25.15% PSG due to the conservative configuration. By performing jitter handling, UE can wake up until the arrival of packet to avoid unnecessary PDCCH monitoring. As shown in Table 32 to Table 34, compared to aligning with or skipping to the negative jitter boundary, e-CDRX with jitter handling and PDCCH monitoring adaptation with jitter handling scheme can achieve another 19.70% ~23.13% and 17.01% ~ 25.55% PSG respectively.

Observation 43: Compared to the R15/R16 CDRX configuration, both enhanced CDRX and PDCCH monitoring adaptation scheme can achieve a significant mean PSG.

Observation 44: Compared to aligning with or skipping to the negative jitter boundary, with jitter handling, eCDRX and PDCCH monitoring adaptation schemes can achieve another 19.70% ~23.13% and 17.01% ~ 25.55% power saving gain respectively without system capacity loss.

    - Uplink

According to the agreements in RAN1 #104b-e, the period of UL pose/control stream is assumed as 4ms without jitter. So, an integer DRX cycle can be configured to match the UL traffic arrival. Note that the packet arrival interval of pose/control stream is far below the corresponding PDB requirement (i.e., 10ms), and the packet size is extremely small which can be finished by using only one TTI. Due to these reasons, to further avoid the transition between wake up and go to sleep states, several packets can be bundled together. In our evaluation, considering the potential impact on the system capacity, we assume that two UL pose/control packets can be bundled transmitted. Figure 9 illustrates the principle of packets bundled transmission, and the period taken by the corresponding DRX configuration. As analyzed in Figure 9, the PDB of bundled packet should take the minimum PDB of the two single packets, i.e. the PDB of bundled packet is 6ms. And the corresponding DRX configuration parameters for without packet bundling and two packets bundling can be found in Table 35.

- AlwaysOn (Baseline): Without adopting any power saving mechanism.

- R15/16 CDRX: R15/16 CDRX configuration without packet bundling.

- CDRX for packets bundling: packets bundled transmission (i.e., two packets are bundled together for transmission at a time)

Table 35. The DRX configuration for UL pose/control stream

| Cases | DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer(ms) |
| --- | --- | --- | --- |
| R15/16 CDRX | 4 | 2 | 1 |
| CDRX for packets bundling | 8 | 3 | 1 |

Figure 9. Packets bundled transmission with corresponding DRX configurations

Similar to the characteristics of XR downlink traffic, AR uplink traffic such as scene/video information also has the non-integer period (e.g., 16.67ms). Differently, there is no need to model jitter for uplink traffic, since it can be left to UE implementation. Therefore, the four power saving schemes without jitter handling are evaluated, and evaluation method of DL XR traffic can be reused for uplink AR traffic model with single stream. Due to the absence of jitter effects, the start offset of e-CDRX scheme can align with the arrival of XR. And by deploying PDCCH monitoring adaptation scheme, UE can skip PDCCH monitoring until the arrival time of next packet when the current packet transmission is completed.

The CDRX parameters of R15/16 CDRX scheme is the same with DL transmission, which can be found in Table 31.  For e-CDRX scheme, the parameters combination 16ms-6ms-4ms is configured as CDRX cycle-On duration timer- Inactivity timer. The power consumption characteristics under high and low loads are evaluated separately for AR traffic model with single scene/video/data/audio stream (10Mbps_60FPS_30ms).The power consumption results of FR1 UL AR traffic with two streams are also provided, that one stream is pose/control stream and the other is aggregating streams (scene/video/data/audio) with 10Mbps_60FPS_30ms. Since the aggregating streams with relatively large packet size mainly limit the system capacity, the power saving schemes used for single video stream evaluation of AR are multiplexed.

- AlwaysOn (Baseline): without adopting any power saving mechanism.

- R15/16 CDRX: adopting R15/R16 CDRX configuration.

- e-CDRX: aligning the start time of DRX ON Duration with the arrival of XR traffic.

- PDCCH monitoring adaptation: UE can skip PDCCH monitoring until the actual packet arrival time.

- InH, FR1, UL, 100MHz bandwidth, DDDSU TDD format

Table 36 Power consumption results in FR1 UL Indoor Hotspot scenario

| Traffic Model | Power Saving scheme | avg # UEs/ cell = N1 | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell = N1 | Average PS gain (%) of all UEs | Average PS gain (%) of satisfied UEs |
| --- | --- | --- | --- | --- | --- | --- |
| pose/control-stream (0.2Mbps) | AlwaysOn - baseline | 20 | 20 | 100% | - | - |
|  | R15/16CDRX (4_2_1) | 20 | 20 | 94.31% | 26.33% | 26.33% |
|  | R15/16CDRX (8_3_1) | 20 | 20 | 93.33% | 36.83% | 36.83% |
| scene/video/data/voice-stream (10Mbps) | AlwaysOn - baseline | 7 | 13 | 100% | - | - |
|  | R15/16CDRX (10-8-4) | 7 | 13 | 100% | 8.17% | 8.17% |
|  | R15/16CDRX (16-14-4) | 7 | 13 | 100% | 5.03% | 5.03% |
|  | eCDRX (16-6-4) | 7 | 13 | 100% | 35.24% | 35.24% |
|  | PDCCH monitoring adaptation | 7 | 13 | 100% | 41.99% | 41.99% |
|  | AlwaysOn - baseline | 13 | 13 | 93.59% | - | - |
|  | R15/16CDRX (10-8-4) | 13 | 13 | 92.22% | 7.71% | 7.71% |
|  | R15/16CDRX (16-14-4) | 13 | 13 | 92.86% | 4.80% | 4.81% |
|  | eCDRX (16-6-4) | 13 | 13 | 92.38% | 33.64% | 34.14% |
|  | PDCCH monitoring adaptation | 13 | 13 | 92.65% | 39.21% | 39.30% |
| pose/control-stream + scene/video/data/voice-stream (0.2Mbps +10Mbps) | AlwaysOn - baseline | 6 | 12 | 100% | - | - |
|  | R15/16CDRX (10-8-4) | 6 | 12 | 100% | 4.82% | 4.82% |
|  | R15/16CDRX (16-14-4) | 6 | 12 | 100% | 2.38% | 2.38% |
|  | eCDRX (16-6-4) | 6 | 12 | 100% | 23.66% | 23.66% |
|  | PDCCH monitoring adaptation | 6 | 12 | 100% | 28.15% | 28.15% |
|  | AlwaysOn - baseline | 12 | 12 | 93.29% | - | - |
|  | R15/16CDRX (10-8-4) | 12 | 12 | 92.13% | 3.97% | 4.03% |
|  | R15/16CDRX (16-14-4) | 12 | 12 | 92.59% | 2.34% | 2.34% |
|  | eCDRX (16-6-4) | 12 | 12 | 91.90% | 22.17% | 22.35% |
|  | PDCCH monitoring adaptation | 12 | 12 | 92.36% | 25.63% | 25.98% |

Observation 45: In FR1 UL in Indoor Hotspot, for R15/16CDRX power saving scheme,

- For CG/VR pose/control-stream, the power saving gain is {26.33%~36.83%} with {5.69%~6.67%} of satisfied UE loss for high load.

- For AR, 10Mbps, 30ms PDB, the power saving gain is {5.03%~8.17%} with no satisfied UE loss for low load, and {4.80%~7.71%} with {0.73%~1.37} of satisfied UE loss for high load.

- For AR, two-stream, the power saving gain is {2.38%~4.82%} with no satisfied UE loss for low load, and {2.34%~3.97%} with {0.70%~1.16%} of satisfied UE loss for high load.

Observation 46: In FR1 UL in Indoor Hotspot, for eCDRX power saving scheme,

- For AR, 10Mbps, 30ms PDB, the power saving gain is {35.24%} with no satisfied UE loss for low load, and {33.64%} with {1.21%} of satisfied UE loss for high load.

- For AR, two-stream, the power saving gain is {23.66%} with no satisfied UE loss for low load, and {22.17%} with {1.39%} of satisfied UE loss for high load.

Observation 47: In FR1 UL in Indoor Hotspot, for PDCCH monitoring adaptation scheme,

- For AR, 10Mbps, 30ms PDB, the power saving gain is {41.99%} with no satisfied UE loss for low load, and {39.21%} with {0.94%} of satisfied UE loss for high load.

- For AR, two-stream, the power saving gain is {28.15%} with no satisfied UE loss for low load, and {25.63%} with {0.93%} of satisfied UE loss for high load.

- DU, FR1, UL, 100MHz bandwidth, DDDSU TDD format

Table 37 Power consumption results in FR1 UL Dense Urban scenario

| Traffic Model | Power Saving scheme | avg # UEs/ cell = N1 | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell = N1 | Average PS gain (%) of all UEs | Average PS gain (%) of satisfied UEs |
| --- | --- | --- | --- | --- | --- | --- |
| pose/control-stream (0.2Mbps) | AlwaysOn - baseline | 20 | 20 | 99.99% | - | - |
|  | R15/16CDRX (4_2_1) | 20 | 20 | 99.84% | 26.62% | 26.65% |
|  | R15/16CDRX (8_3_1) | 20 | 20 | 93.81% | 37.27% | 37.31% |
| scene/video/data/voice-stream (10Mbps) | AlwaysOn - baseline | 5 | 9 | 97.14% | - | - |
|  | R15/16CDRX (10-8-4) | 5 | 9 | 97.14% | 6.97% | 7.13% |
|  | R15/16CDRX (16-14-4) | 5 | 9 | 97.14% | 4.26% | 4.55% |
|  | eCDRX (16-6-4) | 5 | 9 | 95.56% | 32.02% | 33.83% |
|  | PDCCH monitoring adaptation | 5 | 9 | 96.51% | 35.84% | 36.85% |
|  | AlwaysOn - baseline | 9 | 9 | 92.95% | - | - |
|  | R15/16CDRX (10-8-4) | 9 | 9 | 91.53% | 6.73% | 7.12% |
|  | R15/16CDRX (16-14-4) | 9 | 9 | 91.71% | 4.25% | 4.89% |
|  | eCDRX (16-6-4) | 9 | 9 | 91.60% | 28.99% | 29.67% |
|  | PDCCH monitoring adaptation | 9 | 9 | 91.77% | 34.15% | 36.04% |
| pose/control-stream + scene/video/data/voice-stream (0.2Mbps +10Mbps) | AlwaysOn - baseline | 4 | 7 | 100% | - | - |
|  | R15/16CDRX (10-8-4) | 4 | 7 | 100% | 3.23% | 3.23% |
|  | R15/16CDRX (16-14-4) | 4 | 7 | 100% | 1.79% | 1.79% |
|  | eCDRX (16-6-4) | 4 | 7 | 100% | 21.35% | 21.35% |
|  | PDCCH monitoring adaptation | 4 | 7 | 100% | 24.16% | 24.16% |
|  | AlwaysOn - baseline | 7 | 7 | 92.29% | - | - |
|  | R15/16CDRX (10-8-4) | 7 | 7 | 90.70% | 3.43% | 3.63% |
|  | R15/16CDRX (16-14-4) | 7 | 7 | 92.06% | 1.99% | 2.26% |
|  | eCDRX (16-6-4) | 7 | 7 | 90.48% | 19.89% | 19.89% |
|  | PDCCH monitoring adaptation | 7 | 7 | 91.16% | 23.02% | 23.02% |

Observation 48: In FR1 UL in Dense Urban, for R15/16CDRX power saving scheme,

- For CG/VR pose/control-stream, the power saving gain is {26.62%~37.27%} with {0.15%~6.18%} of satisfied UE loss for high load.

- For AR, 10Mbps, 30ms PDB, the power saving gain is {4.26%~6.97%} with no satisfied UE loss for low load, and {4.25%~6.73%} with {1.24%~1.42%} of satisfied UE loss for high load.

- For AR, two-stream, the power saving gain is {1.79%~3.23%} with no satisfied UE loss for low load, and {1.99%~3.43%} with {0.23%~1.59%} of satisfied UE loss for high load.

Observation 49: In FR1 UL in Dense Urban, for eCDRX power saving scheme,

- For AR, 10Mbps, 30ms PDB, the power saving gain is {32.02%} with {1.58%} of satisfied UE loss for low load, and {28.99%} with {1.35%} of satisfied UE loss for high load.

- For AR, two-stream, the power saving gain is {21.35%} with no satisfied UE loss for low load, and {19.89%} with {1.81%} of satisfied UE loss for high load.

Observation 50: In FR1 UL in Dense Urban, for PDCCH monitoring adaptation scheme,

- For AR, 10Mbps, 30ms PDB, the power saving gain is {35.84%} with {0.63%} of satisfied UE loss for low load, and {34.15%} with {1.18%} of satisfied UE loss for high load.

- For AR, two-stream, the power saving gain is {24.16%} with no satisfied UE loss for low load, and {23.02%} with {1.13%} of satisfied UE loss for high load.

- Uma, FR1, UL, 100MHz bandwidth, DDDSU TDD format

Table 38 Power consumption results in FR1 UL Urban Macro scenario

| Traffic Model | Power Saving scheme | avg # UEs/ cell = N1 | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell = N1 | Average PS gain (%) of all UEs | Average PS gain (%) of satisfied UEs |
| --- | --- | --- | --- | --- | --- | --- |
| pose/control-stream (0.2Mbps) | AlwaysOn - baseline | 20 | 20 | 97.70% | - | - |
|  | R15/16CDRX (4_2_1) | 20 | 20 | 94.37% | 28.10% | 28.73% |
|  | R15/16CDRX (8_3_1) | 20 | 20 | 92.94% | 38.93% | 39.66% |

As shown in Table 36 to Table 38, for FR1 UL pose/control-stream, compared with the DRX configuration without packets bundling, the packets bundled transmission scheme can obtain additional about 10% PSG.

For FR1 UL AR with single stream, similar to the results and analyzes for downlink traffic, only 1.79%~8.17% mean PSG can be achieved by using the R15/R16 CDRX configuration. And 19.89%~35.24% mean PSG can be obtained with negligible loss of the system capacity for e-CDRX scheme. To ensure the performance of cell edge UEs, especially for high load cases, e-CDRX has to configure a longer drx-onDurationTimer and drx-InactivityTimer, which increases unnecessary power consumption for high SINR UEs. In fact, gNB can configure per UE DRX configuration to achieve a larger mean PSG for enhanced CDRX scheme. By using PDCCH monitoring adaptation, 23.02%~41.99% mean PSG can be obtained with negligible loss of the system capacity under different system loads conditions. Compared to single stream model, the PSG of two streams by deploying the same PS schemes is smaller, which is closer to the actual operation of UEs. For UL AR traffic with two streams, since scheduling of intensive pose/control stream, UEs have fewer opportunities to go to a light/deep sleep state and more power will be consumed compared to the single-stream model.

Observation 51: In FR1 UL in Urban Macro, for R15/16CDRX power saving scheme,

- For CG/VR pose/control-stream, the power saving gain is {28.10%~38.93%} with {5.63%~7.06%} of satisfied UE loss for high load.

Observation 52:  Compared with the R15/16 CDRX configuration without packet bundling, the packets bundling transmission scheme can obtain additional 10.50%~10.93% PSG for UL pose/control stream in FR1.

    - DL and UL Evaluation Together

For XR/CG applications, DL and UL transmission occur simultaneously and both contribute to power consumption. Also, XR/CG has dense UL traffic, such as pose/control stream with 4ms packets period, which may consume extra power when evaluating some power saving schemes. For example, compared to DL independent evaluation, dense UL traffic may activate the Inactivity timer constantly by using R15/16 CDRX when DL and UL performances are evaluated together. In this section, DL and UL performances are evaluated together for different power saving schemes to obtain the total power consumption for VR and AR application in FR1. A single video stream (30Mbps_60FPS_10ms) is assumed for DL transmission. For UL transmission, pose/control stream and video stream (10Mbps_60FPS_30ms) are assumed.

Due to high data rate, low latency requirement and jitter of DL transmission, the power saving schemes used for DL XR traffic in section 3.1.1 are reused for the evaluation of DL and UL together transmission. And only e-CDRX with adapting to quasi-period position scheme is evaluated for e-CDRX mechanism. For e-CDRX scheme and PDCCH monitoring adaptation, the alignment location of start offset and the location of UE wake up only refer to the DL traffic related time points. Corresponding configurations for R15/16 CDRX and e-CDRX can be found in Table 31.

- InH, FR1 DL and UL evaluation together, 100MHz bandwidth, DDDSU TDD format

Table 39 Power consumption results of DL and UL evaluation together in FR1 Indoor Hotspot scenario

| Traffic Model | Power Saving scheme | avg # UEs/ cell = N1 | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell = N1 | Average PS gain (%) of all UEs | Average PS gain (%) of satisfied UEs | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VR DL video-stream (30Mbps)+UL pose/control (0.2Mbps) | AlwaysOn - baseline | 5 | 10 | 100% | - | - |  |
|  | R15/16CDRX (10-8-4) | 5 | 10 | 100% | 3.71% | 3.71% |  |
|  | R15/16CDRX (16-14-4) | 5 | 10 | 100% | 2.64% | 2.64% |  |
|  | eCDRX (16-6-4) | 5 | 10 | 100% | 25.12% | 25.12% | Note 1 |
|  | PDCCH monitoring adaptation | 5 | 10 | 100% | 22.35% | 22.35% | Note 2 |
|  | PDCCH monitoring adaptation | 5 | 10 | 100% | 41.62% | 41.62% | Note 3 |
|  | AlwaysOn - baseline | 10 | 10 | 92.50% | - | - |  |
|  | R15/16CDRX (10-8-4) | 10 | 10 | 91.25% | 3.45% | 3.57% |  |
|  | R15/16CDRX (16-14-4) | 10 | 10 | 91.81% | 2.33% | 2.39% |  |
|  | eCDRX (16-6-4) | 10 | 10 | 90.70% | 23.56% | 23.75% | Note 1 |
|  | PDCCH monitoring adaptation | 10 | 10 | 91.81% | 21.78% | 22.14% | Note 2 |
|  | PDCCH monitoring adaptation | 10 | 10 | 91.11% | 39.86% | 40.23% | Note 3 |
| AR DL video-stream (30Mbps)+UL video-stream (10Mbps) | AlwaysOn - baseline | 5 | 10 | 100% | - | - |  |
|  | R15/16CDRX (10-8-4) | 5 | 10 | 100% | 4.20% | 4.20% |  |
|  | R15/16CDRX (16-14-4) | 5 | 10 | 100% | 2.59% | 2.59% |  |
|  | eCDRX (16-6-4) | 5 | 10 | 100% | 23.61% | 23.61% | Note 1 |
|  | PDCCH monitoring adaptation | 5 | 10 | 100% | 21.17% | 21.17% | Note 2 |
|  | PDCCH monitoring adaptation | 5 | 10 | 100% | 39.29% | 39.29% | Note 3 |
|  | AlwaysOn - baseline | 10 | 10 | 92.50% | - | - |  |
|  | R15/16CDRX (10-8-4) | 10 | 10 | 91.67% | 2.62% | 2.86% |  |
|  | R15/16CDRX (16-14-4) | 10 | 10 | 91.94% | 1.69% | 1.86% |  |
|  | eCDRX (16-6-4) | 10 | 10 | 90.83% | 14.77% | 15.35% | Note 1 |
|  | PDCCH monitoring adaptation | 10 | 10 | 91.94% | 13.28% | 13.69% | Note 2 |
|  | PDCCH monitoring adaptation | 10 | 10 | 91.67% | 34.46% | 35.23% | Note 3 |
| AR DL video-stream (30Mbps)+UL two-stream (0.2Mbps pose/control+10Mbps video) | AlwaysOn - baseline | 5 | 10 | 100% | - | - |  |
|  | R15/16CDRX (10-8-4) | 5 | 10 | 100% | 1.81% | 1.81% |  |
|  | R15/16CDRX (16-14-4) | 5 | 10 | 100% | 1.02% | 1.02% |  |
|  | eCDRX (16-6-4) | 5 | 10 | 100% | 16.65% | 16.65% | Note 1 |
|  | PDCCH monitoring adaptation | 5 | 10 | 100% | 14.47% | 14.47% | Note 2 |
|  | PDCCH monitoring adaptation | 5 | 10 | 100% | 31.97% | 31.97% | Note 3 |
|  | AlwaysOn - baseline | 10 | 10 | 92.22% | - | - |  |
|  | R15/16CDRX (10-8-4) | 10 | 10 | 90.83% | 1.59% | 1.62% |  |
|  | R15/16CDRX (16-14-4) | 10 | 10 | 91.67% | 0.83% | 0.87% |  |
|  | eCDRX (16-6-4) | 10 | 10 | 90.56% | 13.96% | 14.27% | Note 1 |
|  | PDCCH monitoring adaptation | 10 | 10 | 91.67% | 12.51% | 13.12% | Note 2 |
|  | PDCCH monitoring adaptation | 10 | 10 | 91.11% | 30.45% | 31.68% | Note 3 |
| Note 1: e-CDRX with adapting to quasi-period position Note 2: R17 PDCCH monitoring adaptation without jitter handling Note 3: PDCCH monitoring adaptation with jitter handling |  |  |  |  |  |  |  |

Observation 53: In FR1 DL and UL evaluation together in Indoor Hotspot, for R15/16CDRX power saving scheme,

- For VR, DL 30Mbps_10ms PDB and UL pose/control stream, the power saving gain is {2.64%~3.71%} with no satisfied UE loss for low load, and {2.33%~3.45%} with {0.69%~1.25%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL video stream, the power saving gain is {2.59%~4.20%} with no satisfied UE loss for low load, and {1.69%~2.62%} with {0.56%~0.83%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL two-stream, the power saving gain is {1.02%~1.81%} with no satisfied UE loss for low load, and {0.87%~1.59%} with {0.55%~1.39%} of satisfied UE loss for high load.

Observation 54: In FR1 DL and UL evaluation together in Indoor Hotspot, for eCDRX power saving scheme,

- For VR, DL 30Mbps_10ms PDB and UL pose/control stream, the power saving gain is {25.12%} with no satisfied UE loss for low load, and {23.56%} with {1.80%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL video stream, the power saving gain is {23.61%} with no satisfied UE loss for low load, and {14.77%} with {1.67%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL two-stream, the power saving gain is {16.65%} with no satisfied UE loss for low load, and {13.96%} with {1.66%} of satisfied UE loss for high load.

Observation 55: In FR1 DL and UL evaluation together in Indoor Hotspot, for PDCCH monitoring adaptation scheme,

- For VR, DL 30Mbps_10ms PDB and UL pose/control stream, the power saving gain is {23.35%~41.62%} with no satisfied UE loss for low load, and {21.78%~39.86%} with {0.69%~1.39%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL video stream, the power saving gain is {21.17%~39.29%} with no satisfied UE loss for low load, and {13.28%~34.46%} with {0.56%~0.83%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL two-stream, the power saving gain is {14.47%~31.97%} with no satisfied UE loss for low load, and {12.51%~30.45%} with {0.55%~1.11%} of satisfied UE loss for high load.

- DU, FR1 DL and UL evaluation together, 100MHz bandwidth, DDDSU TDD format

Table 40 Power consumption results of DL and UL evaluation together in FR1 Dense Urban scenario

| Traffic Model | Power Saving scheme | avg # UEs/ cell = N1 | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell = N1 | Average PS gain (%) of all UEs | Average PS gain (%) of satisfied UEs | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VR DL video-stream (30Mbps)+UL pose/control (0.2Mbps) | AlwaysOn - baseline | 7 | 13 | 100% | - | - |  |
|  | R15/16CDRX (10-8-4) | 7 | 13 | 100% | 3.56% | 3.56% |  |
|  | R15/16CDRX (16-14-4) | 7 | 13 | 100% | 2.44% | 2.44% |  |
|  | eCDRX (16-6-4) | 7 | 13 | 100% | 23.49% | 23.49% | Note 1 |
|  | PDCCH monitoring adaptation | 7 | 13 | 100% | 21.06% | 21.06% | Note 2 |
|  | PDCCH monitoring adaptation | 7 | 13 | 100% | 43.63% | 43.63% | Note 3 |
|  | AlwaysOn - baseline | 13 | 13 | 92.43% | - | - |  |
|  | R15/16CDRX (10-8-4) | 13 | 13 | 90.11% | 3.31% | 3.45% |  |
|  | R15/16CDRX (16-14-4) | 13 | 13 | 91.58% | 2.24% | 2.36% |  |
|  | eCDRX (16-6-4) | 13 | 13 | 91.21% | 21.93% | 23.09% | Note 1 |
|  | PDCCH monitoring adaptation | 13 | 13 | 92.19% | 19.98% | 20.21% | Note 2 |
|  | PDCCH monitoring adaptation | 13 | 13 | 91.94% | 37.65% | 38.34% | Note 3 |
| AR DL video-stream (30Mbps)+UL video-stream (10Mbps) | AlwaysOn - baseline | 5 | 9 | 96.51% | - | - |  |
|  | R15/16CDRX (10-8-4) | 5 | 9 | 96.19% | 3.79% | 3.80% |  |
|  | R15/16CDRX (16-14-4) | 5 | 9 | 96.51% | 2.39% | 2.39% |  |
|  | eCDRX (16-6-4) | 5 | 9 | 95.87% | 20.77% | 20.83% | Note 1 |
|  | PDCCH monitoring adaptation | 5 | 9 | 96.19% | 18.26% | 19.32% | Note 2 |
|  | PDCCH monitoring adaptation | 5 | 9 | 95.87% | 40.21% | 41.23% | Note 3 |
|  | AlwaysOn - baseline | 9 | 9 | 92.59% | - | - |  |
|  | R15/16CDRX (10-8-4) | 9 | 9 | 91.89% | 2.58% | 2.81% |  |
|  | R15/16CDRX (16-14-4) | 9 | 9 | 92.06% | 1.62% | 1.63% |  |
|  | eCDRX (16-6-4) | 9 | 9 | 90.83% | 14.04% | 14.64% | Note 1 |
|  | PDCCH monitoring adaptation | 9 | 9 | 92.24% | 12.25% | 13.14% | Note 2 |
|  | PDCCH monitoring adaptation | 9 | 9 | 91.89% | 33.36% | 34.98% | Note 3 |
| AR DL video-stream (30Mbps)+UL two-stream (0.2Mbps pose/control+10Mbps video) | AlwaysOn - baseline | 4 | 7 | 100% | - | - |  |
|  | R15/16CDRX (10-8-4) | 4 | 7 | 100% | 1.63% | 1.63% |  |
|  | R15/16CDRX (16-14-4) | 4 | 7 | 100% | 0.91% | 0.91% |  |
|  | eCDRX (16-6-4) | 4 | 7 | 100% | 14.34% | 14.34% | Note 1 |
|  | PDCCH monitoring adaptation | 4 | 7 | 100% | 12.12% | 12.12% | Note 2 |
|  | PDCCH monitoring adaptation | 4 | 7 | 100% | 32.25% | 32.25% | Note 3 |
|  | AlwaysOn - baseline | 7 | 7 | 92.06% | - | - |  |
|  | R15/16CDRX (10-8-4) | 7 | 7 | 91.16% | 1.51% | 1.56% |  |
|  | R15/16CDRX (16-14-4) | 7 | 7 | 91.61% | 0.79% | 0.83% |  |
|  | eCDRX (16-6-4) | 7 | 7 | 90.48% | 13.19% | 13.52% | Note 1 |
|  | PDCCH monitoring adaptation | 7 | 7 | 91.61% | 11.25% | 11.68% | Note 2 |
|  | PDCCH monitoring adaptation | 7 | 7 | 91.38% | 30.63% | 31.37% | Note 3 |
| Note 1: e-CDRX with adapting to quasi-period position Note 2: R17 PDCCH monitoring adaptation without jitter handling Note 3: PDCCH monitoring adaptation with jitter handling |  |  |  |  |  |  |  |

Compared with only downlink transmission, DL and UL together simulation consumes more energy for each UE due to the existence of the PUSCH state. For R15/16 CDRX scheme, due to the mismatch between the DRX cycle and the packet arrival cycle, the values of drx-onDurationTimer and drx-InactivityTimer have to be set larger enough to provide sufficient time for gNB scheduling. Since DL and UL traffic may not be generated and transmitted at the same time, the drx-InactivityTimer will be restarted by DL or UL scheduling, the UE has less opportunity to go into sleep and can only obtain 0.79% ~ 4.20% PSG.

Compared to the R15/R16 CDRX configuration, both enhanced CDRX and PDCCH monitoring adaptation scheme can achieve a significant mean PSG. Similar to e-CDRX scheme, aligning start offset with negative jitter boundary, PDCCH monitoring adaptation scheme can only achieve 11.25% ~23.35% PSG due to the conservative configuration. By performing jitter handling for PDCCH monitoring adaptation scheme, UE can wake up until the arrival of packet to avoid unnecessary PDCCH monitoring and achieve another 17.50% ~22.57% PSG compared to skipping to the negative jitter boundary.

Observation 56: In FR1 DL and UL evaluation together in Dense Urban, for R15/16CDRX power saving scheme,

- For VR, DL 30Mbps_10ms PDB and UL pose/control stream, the power saving gain is {2.44%~3.56%} with no satisfied UE loss for low load, and {2.24%~3.31%} with {0.85%~2.32%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL video stream, the power saving gain is {2.39%~3.79%} with {0%~0.32%} of satisfied UE loss for low load, and {1.62%~2.58} with {0.53%~0.70%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL two-stream, the power saving gain is {0.91%~1.63%} with no satisfied UE loss for low load, and {0.79%~1.51%} with {0.45%~0.90%} of satisfied UE loss for high load.

Observation 57: In FR1 DL and UL evaluation together in Dense Urban, for eCDRX power saving scheme,

- For VR, DL 30Mbps_10ms PDB and UL pose/control stream, the power saving gain is {23.49%} with no satisfied UE loss for low load, and {21.93%} with {1.22%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL video stream, the power saving gain is {20.77%} with {0.64%} of satisfied UE loss for low load, and {14.04%} with {1.76%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL two-stream, the power saving gain is {14.34%} with no satisfied UE loss for low load, and {13.19%} with {1.58%} of satisfied UE loss for high load.

Observation 58: In FR1 DL and UL evaluation together in Dense Urban, for PDCCH monitoring adaptation scheme,

- For VR, DL 30Mbps_10ms PDB and UL pose/control stream, the power saving gain is {21.06%~43.63%} with no satisfied UE loss for low load, and {19.98%~37.65%} with {0.24%~0.49%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL video stream, the power saving gain is {18.26%~40.21%} with {0.32%~0.64%} of satisfied UE loss for low load, and {12.25%~33.36} with {0.35%~0.70%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL two-stream, the power saving gain is {12.12%~32.25%} with no satisfied UE loss for low load, and {11.25%~30.63%} with {0.45%~0.68%} of satisfied UE loss for high load.

  - FR2

    - Downlink

In this section, the evaluation results of different power schemes in FR2 DL are provided, where the power schemes are the same as FR1 DL in section 3.1.1 and only e-CDRX with adapting to quasi-period position scheme is evaluated for FR2 DL.

In FR2 transmission, multi-beam is configured and only one beam can be scheduled per slot. Therefore, especially for e-CDRX with adapting to quasi-period position scheme in FR2, a longer drx-onDurationTimer need to be configured to avoid UEs who are not scheduled but with packets waiting for transmission to go into sleep state. For e-CDRX with adapting to quasi-period position scheme, 16ms-8ms-4ms is configured as CDRX cycle-On duration timer- Inactivity timer. The same parameters as FR1 DL are configured for R15/16 CDRX, which can be found in Table 31.

According to TR38.840, the relative power of PDCCH state in FR1 is 100 while that in FR2 is 175. By reducing PDCCH monitoring, more power can be saved in FR2. As shown from the simulation results, compared to FR1, the larger PSG can be achieved by deploying power saving schemes in FR2.

- InH, FR2, DL, 100MHz bandwidth, DDDSU TDD format

Table 41 Power consumption results in FR2 DL Indoor Hotspot scenario

| Traffic Model | Power Saving scheme | avg # UEs/ cell = N1 | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell = N1 | Average PS gain (%) of all UEs | Average PS gain (%) of satisfied UEs | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VR/AR (30Mbps) | AlwaysOn - baseline | 4 | 8 | 100% | - | - |  |
|  | R15/16CDRX (10-8-4) | 4 | 8 | 99.31% | 10.06% | 10.06% |  |
|  | R15/16CDRX (16-14-4) | 4 | 8 | 99.31% | 6.28% | 6.28% |  |
|  | eCDRX (16-8-4) | 4 | 8 | 98.61% | 34.89% | 35.06% | Note 1 |
|  | PDCCH monitoring adaptation | 4 | 8 | 100% | 33.80% | 33.80% | Note 2 |
|  | PDCCH monitoring adaptation | 4 | 8 | 100% | 61.85% | 61.85% | Note 3 |
|  | AlwaysOn - baseline | 8 | 8 | 92.01% | - | - |  |
|  | R15/16CDRX (10-8-4) | 8 | 8 | 90.63% | 9.53% | 9.79% |  |
|  | R15/16CDRX (16-14-4) | 8 | 8 | 91.67% | 5.81% | 5.91% |  |
|  | eCDRX (16-8-4) | 8 | 8 | 90.97% | 33.68% | 33.71% | Note 1 |
|  | PDCCH monitoring adaptation | 8 | 8 | 92.01% | 32.69% | 33.40% | Note 2 |
|  | PDCCH monitoring adaptation | 8 | 8 | 92.01% | 57.53% | 58.89% | Note 3 |
| VR/AR (45Mbps) | AlwaysOn - baseline | 2 | 4 | 100% | - | - |  |
|  | R15/16CDRX (10-8-4) | 2 | 4 | 98.61% | 9.52% | 9.54% |  |
|  | R15/16CDRX (16-14-4) | 2 | 4 | 98.61% | 5.98% | 6.01% |  |
|  | eCDRX (16-8-4) | 2 | 4 | 100% | 29.25% | 29.25% | Note 1 |
|  | PDCCH monitoring adaptation | 2 | 4 | 98.61% | 28.87% | 28.93% | Note 2 |
|  | PDCCH monitoring adaptation | 2 | 4 | 100% | 54.50% | 54.50% | Note 3 |
|  | AlwaysOn - baseline | 4 | 4 | 94.44% | - | - |  |
|  | R15/16CDRX (10-8-4) | 4 | 4 | 91.67% | 9.15% | 9.23% |  |
|  | R15/16CDRX (16-14-4) | 4 | 4 | 93.75% | 5.73% | 5.77% |  |
|  | eCDRX (16-8-4) | 4 | 4 | 91.67% | 28.37% | 28.99% | Note 1 |
|  | PDCCH monitoring adaptation | 4 | 4 | 93.75% | 27.36% | 27.55% | Note 2 |
|  | PDCCH monitoring adaptation | 4 | 4 | 94.44% | 52.14% | 52.71% | Note 3 |
| Note 1: e-CDRX with adapting to quasi-period position Note 2: R17 PDCCH monitoring adaptation without jitter handling Note 3: PDCCH monitoring adaptation with jitter handling |  |  |  |  |  |  |  |

Observation 59: In FR2 DL in Indoor Hotspot, for R15/16CDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {6.28%~10.06} with {0.69%} of satisfied UE loss for low load, and {5.81~9.53%} with {0.34%~1.38%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {5.98%~9.52%} with {1.39%} of satisfied UE loss for low load, and {5.73%~9.15%} with {0.69%~2.77%} of satisfied UE loss for high load.

Observation 60: In FR2 DL in Indoor Hotspot, for eCDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {34.89%} with {1.39%} of satisfied UE loss for low load, and {33.68%} with {1.04%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {29.25%} with no satisfied UE loss for low load, and {28.37%} with {2.77%} of satisfied UE loss for high load.

Observation 61: In FR2 DL in Indoor Hotspot, for PDCCH monitoring adaptation scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {33.80%~61.85%} with no satisfied UE loss for low load, and {32.69%~57.53%} with no satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {28.87%~54.50%} with {0%~1.39%} of satisfied UE loss for low load, and {27.36%~52.14%} with {0%~0.69%} of satisfied UE loss for high load.

- DU, FR2, DL, 100MHz bandwidth, DDDSU TDD format

Table 42 Power consumption results in FR2 DL Dense Urban scenario

| Traffic Model | Power Saving scheme | avg # UEs/ cell = N1 | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell = N1 | Average PS gain (%) of all UEs | Average PS gain (%) of satisfied UEs |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VR/AR (30Mbps) | AlwaysOn - baseline | 7 | 13 | 99.55% | - | - |  |
|  | R15/16CDRX (10-8-4) | 7 | 13 | 98.64% | 10.15% | 10.16% |  |
|  | R15/16CDRX (16-14-4) | 7 | 13 | 99.32% | 6.40% | 6.41% |  |
|  | eCDRX (16-8-4) | 7 | 13 | 99.09% | 32.63% | 33.24% | Note 1 |
|  | PDCCH monitoring adaptation | 7 | 13 | 99.32% | 31.74% | 32.73% | Note 2 |
|  | PDCCH monitoring adaptation | 7 | 13 | 99.55% | 59.65% | 60.28% | Note 3 |
|  | AlwaysOn - baseline | 13 | 13 | 95.24% | - | - |  |
|  | R15/16CDRX (10-8-4) | 13 | 13 | 91.82% | 9.50 | 9.62% |  |
|  | R15/16CDRX (16-14-4) | 13 | 13 | 93.53% | 5.96 | 6.01% |  |
|  | eCDRX (16-8-4) | 13 | 13 | 91.94% | 31.30 | 32.23% | Note 1 |
|  | PDCCH monitoring adaptation | 13 | 13 | 95.00% | 31.24% | 31.88% | Note 2 |
|  | PDCCH monitoring adaptation | 13 | 13 | 95.24% | 55.51% | 56.22% | Note 3 |
| VR/AR (45Mbps) | AlwaysOn - baseline | 4 | 8 | 100% | - | - |  |
|  | R15/16CDRX (10-8-4) | 4 | 8 | 100% | 9.20% | 9.20% |  |
|  | R15/16CDRX (16-14-4) | 4 | 8 | 100% | 6.06% | 6.06% |  |
|  | eCDRX (16-8-4) | 4 | 8 | 100% | 28.57% | 28.57% | Note 1 |
|  | PDCCH monitoring adaptation | 4 | 8 | 100% | 28.25% | 28.25% | Note 2 |
|  | PDCCH monitoring adaptation | 4 | 8 | 100% | 53.59% | 53.98% | Note 3 |
|  | AlwaysOn - baseline | 8 | 8 | 93.25% | - | - |  |
|  | R15/16CDRX (10-8-4) | 8 | 8 | 91.67% | 8.29% | 8.61% |  |
|  | R15/16CDRX (16-14-4) | 8 | 8 | 92.26% | 4.98% | 5.57% |  |
|  | eCDRX (16-8-4) | 8 | 8 | 91.47% | 27.16% | 27.30% | Note 1 |
|  | PDCCH monitoring adaptation | 8 | 8 | 93.25% | 26.33% | 27.12% | Note 2 |
|  | PDCCH monitoring adaptation | 8 | 8 | 93.25% | 50.46% | 51.24% | Note 3 |
| Note 1: e-CDRX with adapting to quasi-period position Note 2: R17 PDCCH monitoring adaptation without jitter handling Note 3: PDCCH monitoring adaptation with jitter handling |  |  |  |  |  |  |  |

Observation 62: In FR2 DL in Dense Urban, for R15/16CDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {6.40%~10.15%} with {0.23%~0.91%} of satisfied UE loss for low load, and {5.96%~9.5%} with {1.71%~3.42%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {6.06%~9.20%} with no satisfied UE loss for low load, and {4.98%~8.29%} with {0.99%~1.58%} of satisfied UE loss for high load.

Observation 63: In FR2 DL in Dense Urban, for eCDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {32.63%} with {0.46%} of  satisfied UE loss for low load, and {31.3%} with {3.3%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {28.57%} with no satisfied UE loss for low load, and {27.16%} with {1.78%} of satisfied UE loss for high load.

Observation 64: In FR2 DL in Dense Urban, for PDCCH monitoring adaptation scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {31.74%~59.65%} with {0%~0.23%} of satisfied UE loss for low load, and {31.24%~55.51%} with {0%~0.24%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {28.25%~53.59%} with no satisfied UE loss for low load, and {26.33%~50.46%} with no satisfied UE loss for high load.

    - Uplink

In this section, the evaluation results of different power schemes in FR2 UL are provided, and only single-stream of pose/control-stream or scene/video/data/voice-stream are evaluated. Similar to FR1 UL, there is no need to model jitter for uplink traffic, since it can be left to UE implementation. The FR2 UL uses the same power schemes as the FR1 UL, with the slight difference that the 16ms-8ms-4ms parameter is assumed as CDRX cycle-On duration timer- Inactivity timer for the eCDRX scheme.

- InH, FR2, UL, 100MHz bandwidth, DDDSU TDD format

Table 43 Power consumption results in FR2 UL Indoor Hotspot scenario

| Traffic Model | Power Saving scheme | avg # UEs/ cell = N1 | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell = N1 | Average PS gain (%) of all UEs | Average PS gain (%) of satisfied UEs |
| --- | --- | --- | --- | --- | --- | --- |
| pose/control-stream (0.2Mbps) | AlwaysOn - baseline | 20 | 20 | 97.69% | - | - |
|  | R15/16CDRX (4_2_1) | 20 | 20 | 95.90% | 35.99% | 36.14% |
|  | R15/16CDRX (8_3_1) | 20 | 20 | 92.82% | 45.07% | 45.15% |
| scene/video/data/voice-stream (10Mbps) | AlwaysOn - baseline | 4 | 8 | 100% | - | - |
|  | R15/16CDRX (10-8-4) | 4 | 8 | 100% | 10.24% | 10.24% |
|  | R15/16CDRX (16-14-4) | 4 | 8 | 100% | 6.96% | 6.96% |
|  | eCDRX (16-8-4) | 4 | 8 | 100% | 38.35% | 38.35% |
|  | PDCCH monitoring adaptation | 4 | 8 | 100% | 52.35% | 52.35% |
|  | AlwaysOn - baseline | 8 | 8 | 95.14% | - | - |
|  | R15/16CDRX (10-8-4) | 8 | 8 | 92.71% | 9.74% | 9.75% |
|  | R15/16CDRX (16-14-4) | 8 | 8 | 94.10% | 6.58% | 6.59% |
|  | eCDRX (16-8-4) | 8 | 8 | 92.36% | 36.79% | 37.05% |
|  | PDCCH monitoring adaptation | 8 | 8 | 93.06% | 51.32% | 51.34% |

Observation 65: In FR2 UL in Indoor Hotspot, for R15/16CDRX power saving scheme,

- For CG/VR pose/control-stream, the power saving gain is {35.99%~45.07%} with {2.10%~5.18%} of satisfied UE loss for high load.

- For AR, 10Mbps, 30ms PDB, the power saving gain is {6.96%~10.24%} with no satisfied UE loss for low load, and {6.58%~9.74%} with {1.04%~2.43%} of satisfied UE loss for high load.

Observation 66: In FR2 UL in Indoor Hotspot, for eCDRX power saving scheme,

- For AR, 10Mbps, 30ms PDB, the power saving gain is {38.35%} with no satisfied UE loss for low load, and {36.79%} with {2.78%} of satisfied UE loss for high load.

Observation 67: In FR2 UL in Indoor Hotspot, for PDCCH monitoring adaptation scheme,

- For AR, 10Mbps, 30ms PDB, the power saving gain is {52.35%} with no satisfied UE loss for low load, and {51.32%} with {2.08%} of satisfied UE loss for high load.

- DU, FR2, UL, 100MHz bandwidth, DDDSU TDD format

Table 44 Power consumption results in FR2 UL Dense Urban scenario

| Traffic Model | Power Saving scheme | avg # UEs/ cell = N1 | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell = N1 | Average PS gain (%) of all UEs | Average PS gain (%) of satisfied UEs |
| --- | --- | --- | --- | --- | --- | --- |
| pose/control-stream (0.2Mbps) | AlwaysOn - baseline | 20 | 20 | 96.51% | - | - |
|  | R15/16CDRX (4_2_1) | 20 | 20 | 94.13% | 35.29% | 36.36% |
|  | R15/16CDRX (8_3_1) | 20 | 20 | 92.30% | 42.51% | 43.87% |
| scene/video/data/voice-stream (10Mbps) | AlwaysOn - baseline | 4 | 8 | 100% | - | - |
|  | R15/16CDRX (10-8-4) | 4 | 8 | 99.60% | 9.36% | 9.41% |
|  | R15/16CDRX (16-14-4) | 4 | 8 | 100% | 6.41% | 6.41% |
|  | eCDRX (16-8-4) | 4 | 8 | 99.60% | 32.97% | 33.16% |
|  | PDCCH monitoring adaptation | 4 | 8 | 100% | 51.43% | 51.43% |
|  | AlwaysOn - baseline | 8 | 8 | 92.66% | - | - |
|  | R15/16CDRX (10-8-4) | 8 | 8 | 91.07% | 9.18% | 9.34% |
|  | R15/16CDRX (16-14-4) | 8 | 8 | 91.67% | 6.18% | 6.40% |
|  | eCDRX (16-8-4) | 8 | 8 | 90.67% | 31.72% | 32.31% |
|  | PDCCH monitoring adaptation | 8 | 8 | 91.27% | 46.21% | 47.27% |

Observation 68: In FR2 UL in Dense Urban, for R15/16CDRX power saving scheme,

- For CG/VR pose/control-stream, the power saving gain is {35.29~42.51%} with {2.87%~4.70} of satisfied UE loss for high load.

- For AR, 10Mbps, 30ms PDB, the power saving gain is {6.41%~9.36%} with no satisfied UE loss for low load, and {6.18%~9.18%} with {0.99%~1.59%} of satisfied UE loss for high load.

Observation 69: In FR2 UL in Dense Urban, for eCDRX power saving scheme,

- For AR, 10Mbps, 30ms PDB, the power saving gain is {32.97%} with no satisfied UE loss for low load, and {31.72%} with {1.99%} of satisfied UE loss for high load.

Observation 70: In FR2 UL in Dense Urban, for PDCCH monitoring adaptation scheme,

- For AR, 10Mbps, 30ms PDB, the power saving gain is {51.43%} with no satisfied UE loss for low load, and {46.21%} with {1.39%} of satisfied UE loss for high load.

- Coverage Evaluation

Based on the agreements approved in RAN1 #106-e meeting [4], two methodologies of system-level evaluation can be used to evaluate the performance of XR coverage, and the evaluation results are presented in this subsection. For both two methodologies, the coupling gain of satisfied UEs is used as the metric for the XR coverage evaluation, and most of the simulation assumptions for capacity evaluation can be reused.

  - Evaluation results of methodology 1

With methodology 1, for XR/CG in DL or UL, coverage is defined to be the A-percentile point in CDF of coupling gain for the “satisfied” UEs, with #UEs per cell = B, for a given XR application in a given deployment scenario. In this contribution, the CDF curves of coupling gain for the “satisfied” UEs with B=1 and B=capacity and the values at A=5 percentile point are reported. Based on the capacity evaluation results in section 2, system capacity equals the minimal value of the DL and UL system capacity results. Noted that UE satisfaction depends on, not only coverage, but also the network scheduling algorithm, neighboring cell interference, etc. So the evaluation of coupling gain using methodology 1 will be impacted by e.g., interference and scheduler mechanism, etc.

    - FR1

In system-level coverage evaluation with methodology 1, AR traffic is evaluated, where the DL video stream @30Mbps and a UL single stream (scene/video/data/audio) @10Mbps are simulated independently. The coupling gain results of AR traffic in FR1 Dense Urban scenario and Urban Macro scenario are provided from Figure 10 to Figure 12 by utilizing the PF scheduler. For Urban Macro scenario, the capacity in FR1 UL transmission is less than 1, so only the results of B=1 are provided. It can be observed that for both Dense Urban and Urban Macro scenarios in FR1, PUSCH is the bottleneck channel for AR traffic.

- 4GHz, DDDSU, 100MHz, Dense Urban

Figure 10. Coupling gain for 4GHz, DDDSU, 100MHz, Dense Urban with 1 UE/cell

Figure 11. Coupling gain for 4GHz, DDDSU, 100MHz, Dense Urban with 9 UEs/cell

- 4GHz, DDDSU, 100MHz, Urban Macro

Figure 12. Coupling gain for 4GHz, DDDSU, 100MHz, Urban Macro with 1 UE/cell (UL capacity <1)

Observation 71: For both Dense Urban and Urban Macro scenarios in FR1, for AR traffic (DL @30Mbps and UL @10Mbps) with 100MHz bandwidth and frame structure DDDSU, PUSCH is the bottleneck channel by using SLS evaluation methodology 1.

    - FR2

Following carrier frequency and frame structures with 100MHz bandwidth are evaluated in FR2 by utilizing the PF scheduler. The coupling gain results of AR traffic in FR2 Dense Urban scenario and Indoor Hotspot scenario are presented from Figure 13 to Figure 16. In system-level coverage evaluation with methodology 1, AR traffic is evaluated, where the DL video stream @30Mbps and a UL single stream (scene/video/data/audio) @10Mbps are simulated independently. For Dense Urban scenarios in FR2, PUSCH is the bottleneck channel for AR traffic. For Indoor Hotspot scenarios in FR2, PDSCH is the bottleneck channel for AR traffic.

- 30GHz, DDDSU, 100MHz, Dense Urban

Figure 13. Coupling gain for 30GHz, DDDSU, 100MHz, Dense Urban with 1 UE/cell

Figure 14. Coupling gain for 30GHz, DDDSU, 100MHz, Dense Urban with 8 UEs/cell

- 30GHz, DDDSU, 100MHz, Indoor Hotspot

Figure 15. Coupling gain for 30GHz, DDDSU, 100MHz, Indoor Hotspot with 1 UE/cell

Figure 16. Coupling gain for 30GHz, DDDSU, 100MHz, Indoor Hotspot with 8 UEs/cell

Observation 72: For Dense Urban scenarios in FR2, for AR traffic (DL @30Mbps and UL @10Mbps) with 100MHz bandwidth and frame structure DDDSU, PUSCH is the bottleneck channel by using SLS evaluation methodology 1.

Observation 73: For Indoor Hotspot scenarios in FR2, for AR traffic (DL @30Mbps and UL @10Mbps) with 100MHz bandwidth and frame structure DDDSU, PDSCH is the bottleneck channel by using SLS evaluation methodology 1.

Observation 74: The XR coverage metric with 5 %-tile point in the CDF curve of coupling gain for all the satisfied UEs by methodology 1 for AR traffic (DL @30Mbps and UL @10Mbps) are summarized as follows:

| Scenario | #UE/cell | Coupling gain [dB] |  |
| --- | --- | --- | --- |
|  |  | DL | UL |
| FR1 Dense Urban | 1 | -118.7 | -118.7 |
|  | 9 | -121.9 | -117.0 |
| FR1 Urban Macro | 1 | -140.9 | -124.2 |
| FR2 Dense Urban | 1 | -106.9 | -106.9 |
|  | 8 | -104.5 | -101.9 |
| FR2 Indoor Hotspot | 1 | -85.0 | -85.0 |
|  | 8 | -82.9 | -85.0 |

  - Evaluation results of methodology 2

With methodology 2, only one UE is randomly dropped in the entire network that is associated with one of the 3 center cells, and SLS is performed according to capacity evaluation methodology to determine whether the UE is satisfied or not. In this contribution, 5%-tile point in the CDF curve of coupling gain for all the satisfied UEs is defined as the XR coverage metric. Besides, AR traffic is evaluated, where the DL video stream @30Mbps and a UL single stream (scene/video/data/audio) @10Mbps are simulated independently. Different from methodology 1, the ISD values differ from those defined in TR 38.901 used in capacity evaluation. A sufficiently large ISD value is used to ensure that edge UEs are unlikely to become satisfied UEs. The ISD value is 1000m for Dense Urban scenario and Urban Macro scenario. And for Indoor Hotspot scenario, UE is randomly dropped in an indoor area of 120m in length and 50m in width.

    - FR1

The coupling gain results of AR traffic in FR1 Dense Urban scenario and Urban Macro scenario are provided from Figure 17 and Figure 18 by utilizing the PF scheduler. It can be observed that the 5-percentile point in CDF of DL and UL coupling gain for all satisfied UEs respectively are -144.58 and -126.84 for Dense Urban scenarios, and the 5-percentile point in CDF of DL and UL coupling gain for all satisfied UEs respectively are -150.07 and -126.39 for Urban Macro scenarios. To sum up, for both Dense Urban and Urban Macro scenarios in FR1, PUSCH is the bottleneck channel for AR traffic, which is the same as the conclusion of methodology 1.

- 4GHz, DDDSU, 100MHz, Dense Urban

Figure 17. Coupling gain for 4GHz, DDDSU, 100MHz, Dense Urban

- 4GHz, DDDSU, 100MHz, Urban Macro

Figure 18. Coupling gain for 4GHz, DDDSU, 100MHz, Urban Macro

Observation 75: For both Dense Urban and Urban Macro scenarios in FR1, for AR traffic (DL @30Mbps and UL @10Mbps) with 100MHz bandwidth and frame structure DDDSU, PUSCH is the bottleneck channel by using SLS evaluation methodology 2.

    - FR2

Following carrier frequency with 100MHz bandwidth and frame structures with DDDSU are evaluated in FR2 by utilizing the PF scheduler. The coupling gain results of AR traffic in FR2 Dense Urban scenario and Indoor Hotspot scenario are presented from Figure 19 and Figure 20. It can be observed that the 5-percentile point in CDF of DL and UL coupling gain for all satisfied UEs respectively are -127.66 and -120.17 for Dense Urban scenarios, and the 5-percentile point in CDF of DL and UL coupling gain for all satisfied UEs respectively are -102.67 and -108.17 for Indoor Hotspot scenarios. In conclusion, for Dense Urban scenarios in FR2, PUSCH is the bottleneck channel for AR traffic, and for Indoor Hotspot scenarios in FR2, PDSCH is the bottleneck channel for AR traffic, which is the same as the conclusion of methodology 1.

- 30GHz, DDDSU, 100MHz, Dense Urban

Figure 19. Coupling gain for 30GHz, DDDSU, 100MHz, Dense Urban

- 30GHz, DDDSU, 100MHz, Indoor Hotspot

Figure 20. Coupling gain for 30GHz, DDDSU, 100MHz, Indoor Hotspot

Observation 76: For Dense Urban scenarios in FR2, for AR traffic (DL @30Mbps and UL @10Mbps) with 100MHz bandwidth and frame structure DDDSU, PUSCH is the bottleneck channel by using SLS evaluation methodology 2.

Observation 77: For Indoor Hotspot scenarios in FR2, for AR traffic (DL @30Mbps and UL @10Mbps) with 100MHz bandwidth and frame structure DDDSU, PDSCH is the bottleneck channel by using SLS evaluation methodology 2.

Observation 78: The XR coverage metric with 5 %-tile point in the CDF curve of coupling gain for all the satisfied UEs by methodology 2 for AR traffic (DL @30Mbps and UL @10Mbps) are summarized as follows:

| Scenario | Coupling gain [dB] |  |
| --- | --- | --- |
|  | DL | UL |
| FR1 Dense Urban | -144.58 | -126.84 |
| FR1 Urban Macro | -150.07 | -126.39 |
| FR2 Dense Urban | -127.66 | -120.17 |
| FR2 Indoor Hotspot | 102.67 | -108.17 |

- Conclusion

In this contribution, we provide our updated simulation results on XR capacity, power consumption and coverage with the following observations:

Observation 1: In FR1 in Indoor Hotspot with the capacity requirement X=99,

- For DL CG with 30Mbps_60FPS_15ms, the capacity performance is 10.14 with SU-MIMO and 16.2 with MU-MIMO.

- For DL VR/AR with 30Mbps_60FPS_10ms, the capacity performance is 8.27 with SU-MIMO and 10.8 with MU-MIMO.

- For DL VR/AR with 45Mbps_60FPS_10ms, the capacity performance is 5.91 with MU-MIMO.

Observation 2: In FR1 in Dense Urban with the capacity requirement X=99,

- For DL CG with 30Mbps_60FPS_15ms, the capacity performance is 11.68 with SU-MIMO and 19.65 with MU-MIMO.

- For DL VR/AR with 30Mbps_60FPS_10ms, the capacity performance is 9.49 with SU-MIMO and 13.59 with MU-MIMO.

- For DL VR/AR with 45Mbps_60FPS_10ms, the capacity performance is 6.91 with MU-MIMO.

Observation 3: In FR1 in Urban Macro with the capacity requirement X=99,

- For DL CG with 30Mbps_60FPS_15ms, the capacity performance is 10.33 with SU-MIMO and 14.33 with MU-MIMO.

- For DL VR/AR with 30Mbps_60FPS_10ms, the capacity performance is 7.24 with SU-MIMO and 8.82 with MU-MIMO.

- For DL VR/AR with 45Mbps_60FPS_10ms, the capacity performance is 4.68 with MU-MIMO.

Observation 4: Compared to SU-MIMO, MU-MIMO scheduler can increase the system capacity in FR1 DL,

- In Indoor Hotspot, the capacity performances are increased by 59.76% from 10.14 to 16.2 for CG with 30Mbps_60FPS_15ms and by 30.59% from 8.27 to 10.8 for VR/AR with 30Mbps_60FPS_10ms.

- In Dense Uban, the capacity performances are increased by 68.24% from 11.68 to 19.65 for CG with 30Mbps_60FPS_15ms and by 43.20%  from 9.49 to 13.59 VR/AR with 30Mbps_60FPS_10ms.

- In Urban Macro, the capacity performances are increased by 38.72% from 10.33 to 14.33 for CG with 30Mbps_60FPS_15ms and by 21.82% from 7.24 to 8.82 VR/AR with 30Mbps_60FPS_10ms.

Observation 5：The capacity performance is degraded with the increase of data rate.

Observation 6：The capacity of Cloud Gaming outnumbers VR/AR applications due to the relatively relaxed PDB requirement.

Observation 7: Compared to bundling transmission of dual-eye buffers, separate packet arrivals in time for dual-eye buffers can increase the system capacity.

Observation 8: Delay-aware scheduler can increase the system capacity compared to typical PF scheduler.

Observation 9: In FR1 in Indoor Hotspot with the capacity requirement X=99,

- For UL pose/control-stream with 0.2Mbps_250FPS_10ms, the capacity performance is 20 with SU-MIMO considering limited PDCCH resource.

- For UL scene/video/data/voice-stream with 10Mbps_60FPS_30ms, the capacity performance is 13.95 with SU-MIMO.

Observation 10: In FR1 in Dense Urban with the capacity requirement X=99,

- For UL pose/control-stream with 0.2Mbps_250FPS_10ms, the capacity performance is 20 with SU-MIMO considering limited PDCCH resource.

- For UL scene/video/data/voice-stream with 10Mbps_60FPS_30ms, the capacity performance is 9.49 with SU-MIMO.

Observation 11: In FR1 in Urban Macro with the capacity requirement X=99,

- For UL pose/control-stream with 0.2Mbps_250FPS_10ms, the capacity performance is 20 with SU-MIMO considering limited PDCCH resource.

- For UL scene/video/data/voice-stream with 10Mbps_60FPS_30ms, the capacity performance is <1 with SU-MIMO.

Observation 12: For CG and VR applications in FR1, DL is the bottleneck of system capacity because of the high data rate.

Observation 13: In FR2 in Indoor Hotspot with the capacity requirement X=99 and 100MHz bandwidth,

- For DL CG with 30Mbps_60FPS_15ms, the capacity performance is 9.91 with SU-MIMO.

- For DL VR/AR with 30Mbps_60FPS_10ms, the capacity performance is 8.72 with SU-MIMO.

- For DL VR/AR with 45Mbps_60FPS_10ms, the capacity performance is 4.67 with SU-MIMO.

Observation 14: In FR2 in Dense Urban with the capacity requirement X=99 and 100MHz bandwidth,

- For DL CG with 30Mbps_60FPS_15ms, the capacity performance is 16.16 with SU-MIMO.

- For DL VR/AR with 30Mbps_60FPS_10ms, the capacity performance is 13.44 with SU-MIMO.

- For DL VR/AR with 45Mbps_60FPS_10ms, the capacity performance is 8.20 with SU-MIMO.

Observation 15: In FR2 in Dense Urban with the capacity requirement X=99 and 400MHz bandwidth,

- For DL VR/AR with 45Mbps_60FPS_10ms, the capacity performance is 43.89 with SU-MIMO.

Observation 16: In FR2 in Indoor Hotspot with the capacity requirement X=99 and 100MHz bandwidth,

- For UL pose/control-stream with 0.2Mbps_250FPS_10ms, the capacity performance is 20 with SU-MIMO considering limited PDCCH resource.

- For UL scene/video/data/voice-stream with 10Mbps_60FPS_30ms, the capacity performance is 8.59 with SU-MIMO.

Observation 17: In FR2 in Dense Urban with the capacity requirement X=99 and 100MHz bandwidth,

- For UL pose/control-stream with 0.2Mbps_250FPS_10ms, the capacity performance is 20 with SU-MIMO considering limited PDCCH resource.

- For UL scene/video/data/voice-stream with 10Mbps_60FPS_30ms, the capacity performance is 8.30 with SU-MIMO.

Observation 18: In FR1 in Dense Urban, for DL VR/AR with 30Mbps of slice-based multi-stream model,

- the capacity performance is in the range of {13.27~16.79} with MU-MIMO for alpha = 1.5.

- the capacity performance is in the range of {13.54~16.95} with MU-MIMO for alpha = 2.

- the capacity performance is in the range of {13.46~16.98} with MU-MIMO for alpha = 3.

Observation 19: In FR1 in Dense Urban, for DL VR/AR with 30Mbps of GOP-based multi-stream model,

- the capacity performance is in the range of {6.39~12.58} with MU-MIMO for alpha = 1.5.

- the capacity performance is in the range of {4.74~10.06} with MU-MIMO for alpha = 2.

- the capacity performance is in the range of {2.09~5.73} with MU-MIMO for alpha = 3.

Observation 20: In FR2 in Indoor Hotspot, for DL VR/AR with 30Mbps of slice-based multi-stream model,

- the capacity performance is in the range of {8.14~10.77} with MU-MIMO for alpha = 1.5.

- the capacity performance is in the range of {8.18~10.73} with MU-MIMO for alpha = 2.

- the capacity performance is in the range of {8.22~10.63} with MU-MIMO for alpha = 3.

Observation 21: In FR2 in Indoor Hotspot, for DL VR/AR with 30Mbps of GOP-based multi-stream model,

- the capacity performance is in the range of {4.98~7.43} with MU-MIMO for alpha = 1.5.

- the capacity performance is in the range of {2.73~5.33} with MU-MIMO for alpha = 2.

- the capacity performance is in the range of {2.03~3.29} with MU-MIMO for alpha = 3.

Observation 22: In FR1 in Indoor Hotspot with the capacity requirement X=99, For UL two-stream with pose/control-stream and scene/video/data/voice -stream(10Mbps), the capacity performance is 12.71 with SU-MIMO.

Observation 23: In FR1 in Dense Urban with the capacity requirement X=99, For UL two-stream with pose/control-stream and scene/video/data/voice-stream(10Mbps), the capacity performance is 7.43 with SU-MIMO.

Observation 24: For VR/AR, 10ms PDB, 60 FPS, with data rate increase from 30Mbps to 45Mbps,

- In FR1 DL in Indoor Hotspot, the capacity performances are decreased by 45.28% from 10.80 to 5.91 with MU-MIMO.

- In FR1 DL in Dense Urban, the capacity performances are decreased by 49.23% from 13.59 to 6.9 with MU-MIMO.

- In FR1 DL in Urban Macro, the capacity performances are decreased by 46.94% from 8.82 to 4.68 with MU-MIMO.

- In FR2 DL in Indoor Hotspot, the capacity performances are decreased by 46.44% from 8.72 to 4.67 with SU-MIMO.

- In FR2 DL in Dense Urban, the capacity performances are decreased by 38.99% from 13.44 to 8.20 with SU-MIMO.

Observation 25: For DL transmission with 30Mbps data rate, by relaxing the PDB from 10ms to 15ms,

- In FR1 DL in Indoor Hotspot, the capacity performances are increased by 22.61% from 8.27 to 10.14 with SU-MIMO and by 50.00% from 10.80 to 16.20 with MU-MIMO.

- In FR1 DL in Dense Urban, the capacity performances are increased by 23.08% from 9.49 to 11.68 with MU-MIMO and by 44.59% from 13.59 to 19.65 with MU-MIMO.

- In FR1 DL in Urban Macro, the capacity performances are increased by 42.68% from 7.24 to 10.33 with MU-MIMO and by 62.47% from 8.82 to 14.33 with MU-MIMO.

- In FR2 DL in Indoor Hotspot, the capacity performances are increased by 19.83% from 8.72 to 9.91 with SU-MIMO.

- In FR2 DL in Dense Urban, the capacity performances are increased by 20.24% from 13.44 to 16.16 with SU-MIMO.

Observation 26: For DL VR/AR transmission, by increasing frame rate from 60FPS to 120FPS,

- In FR1 DL in Indoor Hotspot, the capacity performances are increased by 63.99% from 10.80 to 16.53 for 30Mbps and by 56.01% from 5.91 to 9.22 for 45Mbps with MU-MIMO.

- In FR1 DL in Dense Urban, the capacity performances are increased by 52.91% from 13.59 to 20.78 for 30Mbps and by 65.27% from 6.91 to 11.42 for 45Mbps with MU-MIMO.

- In FR1 DL in Urban Macro, the capacity performances are increased by 65.42% from 8.82 to 14.59 for 30Mbps and by 73.50% from 4.68 to 8.12 for 45Mbps with MU-MIMO.

- In FR2 DL in Indoor Hotspot, the capacity performances are increased by 17.32% from 8.72 to 10.23 for 30Mbps and by 29.12% from 4.67 to 6.03 for 45Mbps with SU-MIMO.

- In FR2 DL in Dense Urban, the capacity performances are increased by 21.13% from 13.44 to 16.28 for 30Mbps and by 25.85% from 8.20 to 10.32 for 45Mbps with SU-MIMO.

Observation 27: Compared to typical PF scheduler, delay-aware scheduler can increase the system capacity,

- In FR1 DL in Indoor Hotspot, the capacity performances are increased by 2.90% from 16.20 to 16.67 for CG 30Mbps and by 14.81% from 10.80 to 12.40 for VR/AR 30Mbps with MU-MIMO.

- In FR1 DL in Dense Urban, the capacity performances are increased by 0.51% from 19.65 to 19.75 for CG 30Mbps and by 5.96% from 13.59 to 14.40 for VR/AR 30Mbps with MU-MIMO.

- In FR1 DL in Urban Macro, the capacity performances are increased by 0.84% from 14.33 to 14.45 for CG 30Mbps and by 8.28% from 8.82 to 9.55 for VR/AR 30Mbps with MU-MIMO.

- In FR1 DL in Indoor Hotspot, the capacity performances are increased by 12.72% from 10.14 to 11.43 for CG 30Mbps and by 30.23% from 8.27 to 10.77 for VR/AR 30Mbps with SU-MIMO.

- In FR1 DL in Dense Urban, the capacity performances are increased by16.27% from 11.68 to 13.58 for CG 30Mbps and by 33.51% from 9.49 to 12.67 for VR/AR 30Mbps with SU-MIMO.

- In FR1 DL in Urban Macro, the capacity performances are increased by 15.59% from 10.33 to 11.94 for CG 30Mbps and by 18.23% from 7.24 to 8.56 for VR/AR 30Mbps with SU-MIMO.

- In FR2 DL in Indoor Hotspot, the capacity performances are increased by 3.23% from 9.91 to 10.23 for CG 30Mbps and by 1.26% from 8.72 to 8.83 for VR/AR 30Mbps with SU-MIMO.

- In FR2 DL in Dense Urban, the capacity performances are increased by 4.08% from 16.16 to 16.82 for CG 30Mbps and by 5.36% from 13.44 to 14.16 for VR/AR 30Mbps with SU-MIMO.

Observation 28: In FR2 in Dense Urban, for DL VR/AR with 45Mbps_60FPS_10ms, by increasing bandwidth from 100MHz to 400Mhz, the capacity performance increases by 435.24% from 8.20 to 43.89.

Observation 29: Compared the value of α equal to 3, reducing the value of α can lead to smaller average packet size of I-frame such that improve the system capacity for GOP-based I/P-frame model.

Observation 30: Compared the PDB = 10 ms, relaxing PDB of I-frame to 15 ms can improve the system capacity for GOP-based I/P-frame model.

Observation 31: Prioritizing scheduling of I-frame can improve the system capacity for GOP-based I/P-frame model, compared to the typical PF scheduling.

Observation 32: When I-frame and P-frame adopt the same PER and PDB values, the impact of different α values on system capacity can be almost negligible for slice-based I/P-frame model.

Observation 33: Relaxing PER of P-frame can improve capacity performance for slice-based I/P-frame model.

Observation 34: In FR1 DL in Indoor Hotspot, for R15/16 CDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {3.67%~5.72%} with no satisfied UE loss for low load, and {3.24%~4.88%} with {0.69~1.25%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {3.46%~5.32%} with no satisfied UE loss for low load, and {3.14%~5.31%} with {2.23%~3.89%} of satisfied UE loss for high load.

Observation 35: In FR1 DL in Indoor Hotspot, for eCDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {13.05%~35.35%} with no satisfied UE loss for low load, and {9.36%~29.06%} with {0.56%~1.25%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {11.96%~35.09%} with no satisfied UE loss for low load, and {9.42%~29.12%} with {0%~2.78%} of satisfied UE loss for high load.

Observation 36: In FR1 DL in Indoor Hotspot, for PDCCH monitoring adaptation scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {23.33%~48.88%} with no satisfied UE loss for low load, and {20.73%~41.03} with {0%~0.33%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {22.16%~45.32%} with no satisfied UE loss for low load, and {19.89%~38.76%} with no satisfied UE loss for high load.

Observation 37: In FR1 DL in Dense Urban, for R15/16CDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {3.65%~5.57%} with no satisfied UE loss for low load, and {3.03%~4.70%} with {0.85%~2.32%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {3.53%~5.56%} with no satisfied UE loss for low load, and {3.10%~4.69%} with {1.45%~2.51%} of satisfied UE loss for high load.

Observation 38: In FR1 DL in Dense Urban, for eCDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {12.49%~27.49%} with no satisfied UE loss for low load, and {8.67%~21.72%} with {0.73%~1.22%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {12.61%~27.26%} with {0.53%~1.06%} of satisfied UE loss for low load, and {9.72%~22.95%} with {0%~1.45%} of satisfied UE loss for high load.

Observation 39: In FR1 DL in Dense Urban, for PDCCH monitoring adaptation scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {22.65%~47.38%} with no satisfied UE loss for low load, and {18.86%~37.83%} with no satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {21.95%~45.19%} with no satisfied UE loss for low load, and {18.73%~37.26%} with {0%~1.19%} of satisfied UE loss for high load.

Observation 40: In FR1 DL in Urban Macro, for R15/16CDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {4.05%~6.26%} with {0%~0.40%} of satisfied UE loss for low load, and {3.23~5.02%} with {0.90%~2.28%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {3.97%~5.81%} with no satisfied UE loss for low load, and {3.13%~4.92%} with {0.80%~1.59%} of satisfied UE loss for high load.

Observation 41: In FR1 DL in Urban Macro, for eCDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {13.09%~29.06%} with {0%~1.59%} of satisfied UE loss for low load, and {10.50%~23.33%} with {0.40%~1.88%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {12.09%~27.33%} with no satisfied UE loss for low load, and {9.86%~23.59%} with {0%~2.38%} of satisfied UE loss for high load.

Observation 42: In FR1 DL in Urban Macro, for PDCCH monitoring adaptation scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {25.15%~47.13%} with no satisfied UE loss for low load, and {20.54%~40.59%} with no satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {23.25%~43.26%} with no satisfied UE loss for low load, and {20.17%~37.18%} with {0%~0.72%} of satisfied UE loss for high load.

Observation 43: Compared to the R15/R16 CDRX configuration, both enhanced CDRX and PDCCH monitoring adaptation scheme can achieve a significant mean PSG.

Observation 44: Compared to aligning with or skipping to the negative jitter boundary, with jitter handling, eCDRX and PDCCH monitoring adaptation schemes can achieve another 19.70% ~23.13% and 17.01% ~ 25.55% power saving gain respectively without system capacity loss.

Observation 45: In FR1 UL in Indoor Hotspot, for R15/16CDRX power saving scheme,

- For CG/VR pose/control-stream, the power saving gain is {26.33%~36.83%} with {5.69%~6.67%} of satisfied UE loss for high load.

- For AR, 10Mbps, 30ms PDB, the power saving gain is {5.03%~8.17%} with no satisfied UE loss for low load, and {4.80%~7.71%} with {0.73%~1.37} of satisfied UE loss for high load.

- For AR, two-stream, the power saving gain is {2.38%~4.82%} with no satisfied UE loss for low load, and {2.34%~3.97%} with {0.70%~1.16%} of satisfied UE loss for high load.

Observation 46: In FR1 UL in Indoor Hotspot, for eCDRX power saving scheme,

- For AR, 10Mbps, 30ms PDB, the power saving gain is {35.24%} with no satisfied UE loss for low load, and {33.64%} with {1.21%} of satisfied UE loss for high load.

- For AR, two-stream, the power saving gain is {23.66%} with no satisfied UE loss for low load, and {22.17%} with {1.39%} of satisfied UE loss for high load.

Observation 47: In FR1 UL in Indoor Hotspot, for PDCCH monitoring adaptation scheme,

- For AR, 10Mbps, 30ms PDB, the power saving gain is {41.99%} with no satisfied UE loss for low load, and {39.21%} with {0.94%} of satisfied UE loss for high load.

- For AR, two-stream, the power saving gain is {28.15%} with no satisfied UE loss for low load, and {25.63%} with {0.93%} of satisfied UE loss for high load.

Observation 48: In FR1 UL in Dense Urban, for R15/16CDRX power saving scheme,

- For CG/VR pose/control-stream, the power saving gain is {26.62%~37.27%} with {0.15%~6.18%} of satisfied UE loss for high load.

- For AR, 10Mbps, 30ms PDB, the power saving gain is {4.26%~6.97%} with no satisfied UE loss for low load, and {4.25%~6.73%} with {1.24%~1.42%} of satisfied UE loss for high load.

- For AR, two-stream, the power saving gain is {1.79%~3.23%} with no satisfied UE loss for low load, and {1.99%~3.43%} with {0.23%~1.59%} of satisfied UE loss for high load.

Observation 49: In FR1 UL in Dense Urban, for eCDRX power saving scheme,

- For AR, 10Mbps, 30ms PDB, the power saving gain is {32.02%} with {1.58%} of satisfied UE loss for low load, and {28.99%} with {1.35%} of satisfied UE loss for high load.

- For AR, two-stream, the power saving gain is {21.35%} with no satisfied UE loss for low load, and {19.89%} with {1.81%} of satisfied UE loss for high load.

Observation 50: In FR1 UL in Dense Urban, for PDCCH monitoring adaptation scheme,

- For AR, 10Mbps, 30ms PDB, the power saving gain is {35.84%} with {0.63%} of satisfied UE loss for low load, and {34.15%} with {1.18%} of satisfied UE loss for high load.

- For AR, two-stream, the power saving gain is {24.16%} with no satisfied UE loss for low load, and {23.02%} with {1.13%} of satisfied UE loss for high load.

Observation 51: In FR1 UL in Urban Macro, for R15/16CDRX power saving scheme,

- For CG/VR pose/control-stream, the power saving gain is {28.10%~38.93%} with {5.63%~7.06%} of satisfied UE loss for high load.

Observation 52:  Compared with the R15/16 CDRX configuration without packet bundling, the packets bundling transmission scheme can obtain additional 10.50%~10.93% PSG for UL pose/control stream in FR1.

Observation 53: In FR1 DL and UL evaluation together in Indoor Hotspot, for R15/16CDRX power saving scheme,

- For VR, DL 30Mbps_10ms PDB and UL pose/control stream, the power saving gain is {2.64%~3.71%} with no satisfied UE loss for low load, and {2.33%~3.45%} with {0.69%~1.25%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL video stream, the power saving gain is {2.59%~4.20%} with no satisfied UE loss for low load, and {1.69%~2.62%} with {0.56%~0.83%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL two-stream, the power saving gain is {1.02%~1.81%} with no satisfied UE loss for low load, and {0.87%~1.59%} with {0.55%~1.39%} of satisfied UE loss for high load.

Observation 54: In FR1 DL and UL evaluation together in Indoor Hotspot, for eCDRX power saving scheme,

- For VR, DL 30Mbps_10ms PDB and UL pose/control stream, the power saving gain is {25.12%} with no satisfied UE loss for low load, and {23.56%} with {1.80%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL video stream, the power saving gain is {23.61%} with no satisfied UE loss for low load, and {14.77%} with {1.67%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL two-stream, the power saving gain is {16.65%} with no satisfied UE loss for low load, and {13.96%} with {1.66%} of satisfied UE loss for high load.

Observation 55: In FR1 DL and UL evaluation together in Indoor Hotspot, for PDCCH monitoring adaptation scheme,

- For VR, DL 30Mbps_10ms PDB and UL pose/control stream, the power saving gain is {23.35%~41.62%} with no satisfied UE loss for low load, and {21.78%~39.86%} with {0.69%~1.39%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL video stream, the power saving gain is {21.17%~39.29%} with no satisfied UE loss for low load, and {13.28%~34.46%} with {0.56%~0.83%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL two-stream, the power saving gain is {14.47%~31.97%} with no satisfied UE loss for low load, and {12.51%~30.45%} with {0.55%~1.11%} of satisfied UE loss for high load.

Observation 56: In FR1 DL and UL evaluation together in Dense Urban, for R15/16CDRX power saving scheme,

- For VR, DL 30Mbps_10ms PDB and UL pose/control stream, the power saving gain is {2.44%~3.56%} with no satisfied UE loss for low load, and {2.24%~3.31%} with {0.85%~2.32%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL video stream, the power saving gain is {2.39%~3.79%} with {0%~0.32%} of satisfied UE loss for low load, and {1.62%~2.58} with {0.53%~0.70%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL two-stream, the power saving gain is {0.91%~1.63%} with no satisfied UE loss for low load, and {0.79%~1.51%} with {0.45%~0.90%} of satisfied UE loss for high load.

Observation 57: In FR1 DL and UL evaluation together in Dense Urban, for eCDRX power saving scheme,

- For VR, DL 30Mbps_10ms PDB and UL pose/control stream, the power saving gain is {23.49%} with no satisfied UE loss for low load, and {21.93%} with {1.22%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL video stream, the power saving gain is {20.77%} with {0.64%} of satisfied UE loss for low load, and {14.04%} with {1.76%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL two-stream, the power saving gain is {14.34%} with no satisfied UE loss for low load, and {13.19%} with {1.58%} of satisfied UE loss for high load.

Observation 58: In FR1 DL and UL evaluation together in Dense Urban, for PDCCH monitoring adaptation scheme,

- For VR, DL 30Mbps_10ms PDB and UL pose/control stream, the power saving gain is {21.06%~43.63%} with no satisfied UE loss for low load, and {19.98%~37.65%} with {0.24%~0.49%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL video stream, the power saving gain is {18.26%~40.21%} with {0.32%~0.64%} of satisfied UE loss for low load, and {12.25%~33.36} with {0.35%~0.70%} of satisfied UE loss for high load.

- For AR, DL 30Mbps_10ms PDB and UL two-stream, the power saving gain is {12.12%~32.25%} with no satisfied UE loss for low load, and {11.25%~30.63%} with {0.45%~0.68%} of satisfied UE loss for high load.

Observation 59: In FR2 DL in Indoor Hotspot, for R15/16CDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {6.28%~10.06} with {0.69%} of satisfied UE loss for low load, and {5.81~9.53%} with {0.34%~1.38%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {5.98%~9.52%} with {1.39%} of satisfied UE loss for low load, and {5.73%~9.15%} with {0.69%~2.77%} of satisfied UE loss for high load.

Observation 60: In FR2 DL in Indoor Hotspot, for eCDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {34.89%} with {1.39%} of satisfied UE loss for low load, and {33.68%} with {1.04%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {29.25%} with no satisfied UE loss for low load, and {28.37%} with {2.77%} of satisfied UE loss for high load.

Observation 61: In FR2 DL in Indoor Hotspot, for PDCCH monitoring adaptation scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {33.80%~61.85%} with no satisfied UE loss for low load, and {32.69%~57.53%} with no satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {28.87%~54.50%} with {0%~1.39%} of satisfied UE loss for low load, and {27.36%~52.14%} with {0%~0.69%} of satisfied UE loss for high load.

Observation 62: In FR2 DL in Dense Urban, for R15/16CDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {6.40%~10.15%} with {0.23%~0.91%} of satisfied UE loss for low load, and {5.96%~9.5%} with {1.71%~3.42%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {6.06%~9.20%} with no satisfied UE loss for low load, and {4.98%~8.29%} with {0.99%~1.58%} of satisfied UE loss for high load.

Observation 63: In FR2 DL in Dense Urban, for eCDRX power saving scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {32.63%} with {0.46%} of  satisfied UE loss for low load, and {31.3%} with {3.3%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {28.57%} with no satisfied UE loss for low load, and {27.16%} with {1.78%} of satisfied UE loss for high load.

Observation 64: In FR2 DL in Dense Urban, for PDCCH monitoring adaptation scheme,

- For VR/AR, 30Mbps, 10ms PDB the power saving gain is {31.74%~59.65%} with {0%~0.23%} of satisfied UE loss for low load, and {31.24%~55.51%} with {0%~0.24%} of satisfied UE loss for high load.

- For VR/AR, 45Mbps, 10ms PDB the power saving gain is {28.25%~53.59%} with no satisfied UE loss for low load, and {26.33%~50.46%} with no satisfied UE loss for high load.

Observation 65: In FR2 UL in Indoor Hotspot, for R15/16CDRX power saving scheme,

- For CG/VR pose/control-stream, the power saving gain is {35.99%~45.07%} with {2.10%~5.18%} of satisfied UE loss for high load.

- For AR, 10Mbps, 30ms PDB, the power saving gain is {6.96%~10.24%} with no satisfied UE loss for low load, and {6.58%~9.74%} with {1.04%~2.43%} of satisfied UE loss for high load.

Observation 66: In FR2 UL in Indoor Hotspot, for eCDRX power saving scheme,

- For AR, 10Mbps, 30ms PDB, the power saving gain is {38.35%} with no satisfied UE loss for low load, and {36.79%} with {2.78%} of satisfied UE loss for high load.

Observation 67: In FR2 UL in Indoor Hotspot, for PDCCH monitoring adaptation scheme,

- For AR, 10Mbps, 30ms PDB, the power saving gain is {52.35%} with no satisfied UE loss for low load, and {51.32%} with {2.08%} of satisfied UE loss for high load.

Observation 68: In FR2 UL in Dense Urban, for R15/16CDRX power saving scheme,

- For CG/VR pose/control-stream, the power saving gain is {35.29~42.51%} with {2.87%~4.70} of satisfied UE loss for high load.

- For AR, 10Mbps, 30ms PDB, the power saving gain is {6.41%~9.36%} with no satisfied UE loss for low load, and {6.18%~9.18%} with {0.99%~1.59%} of satisfied UE loss for high load.

Observation 69: In FR2 UL in Dense Urban, for eCDRX power saving scheme,

- For AR, 10Mbps, 30ms PDB, the power saving gain is {32.97%} with no satisfied UE loss for low load, and {31.72%} with {1.99%} of satisfied UE loss for high load.

Observation 70: In FR2 UL in Dense Urban, for PDCCH monitoring adaptation scheme,

- For AR, 10Mbps, 30ms PDB, the power saving gain is {51.43%} with no satisfied UE loss for low load, and {46.21%} with {1.39%} of satisfied UE loss for high load.

Observation 71: For both Dense Urban and Urban Macro scenarios in FR1, for AR traffic (DL @30Mbps and UL @10Mbps) with 100MHz bandwidth and frame structure DDDSU, PUSCH is the bottleneck channel by using SLS evaluation methodology 1.

Observation 72: For Dense Urban scenarios in FR2, for AR traffic (DL @30Mbps and UL @10Mbps) with 100MHz bandwidth and frame structure DDDSU, PUSCH is the bottleneck channel by using SLS evaluation methodology 1.

Observation 73: For Indoor Hotspot scenarios in FR2, for AR traffic (DL @30Mbps and UL @10Mbps) with 100MHz bandwidth and frame structure DDDSU, PDSCH is the bottleneck channel by using SLS evaluation methodology 1.

Observation 74: The XR coverage metric with 5 %-tile point in the CDF curve of coupling gain for all the satisfied UEs by methodology 1 for AR traffic (DL @30Mbps and UL @10Mbps) are summarized as follows:

| Scenario | #UE/cell | Coupling gain [dB] |  |
| --- | --- | --- | --- |
|  |  | DL | UL |
| FR1 Dense Urban | 1 | -118.7 | -118.7 |
|  | 9 | -121.9 | -117.0 |
| FR1 Urban Macro | 1 | -140.9 | -124.2 |
| FR2 Dense Urban | 1 | -106.9 | -106.9 |
|  | 8 | -104.5 | -101.9 |
| FR2 Indoor Hotspot | 1 | -85.0 | -85.0 |
|  | 8 | -82.9 | -85.0 |

Observation 75: For both Dense Urban and Urban Macro scenarios in FR1, for AR traffic (DL @30Mbps and UL @10Mbps) with 100MHz bandwidth and frame structure DDDSU, PUSCH is the bottleneck channel by using SLS evaluation methodology 2.

Observation 76: For Dense Urban scenarios in FR2, for AR traffic (DL @30Mbps and UL @10Mbps) with 100MHz bandwidth and frame structure DDDSU, PUSCH is the bottleneck channel by using SLS evaluation methodology 2.

Observation 77: For Indoor Hotspot scenarios in FR2, for AR traffic (DL @30Mbps and UL @10Mbps) with 100MHz bandwidth and frame structure DDDSU, PDSCH is the bottleneck channel by using SLS evaluation methodology 2.

Observation 78: The XR coverage metric with 5 %-tile point in the CDF curve of coupling gain for all the satisfied UEs by methodology 2 for AR traffic (DL @30Mbps and UL @10Mbps) are summarized as follows:

| Scenario | Coupling gain [dB] |  |
| --- | --- | --- |
|  | DL | UL |
| FR1 Dense Urban | -144.58 | -126.84 |
| FR1 Urban Macro | -150.07 | -126.39 |
| FR2 Dense Urban | -127.66 | -120.17 |
| FR2 Indoor Hotspot | 102.67 | -108.17 |

# References

- Chairman’s Notes of RAN1 #104-e final (Online meeting, January 25th – February 5th, 2021).

- Chairman’s Notes of RAN1 #104b-e final (Online meeting, April 12th – April 20th, 2021).

- Chairman’s Notes of RAN1 #105-e final (Online meeting, May 10th – May 27th, 2021).

- Chairman’s Notes of RAN1 #106-e final (Online meeting, October 11th – October 19th, 2021).

- R1-2109009, Remaining issues on traffic model and EVM for XR, vivo.

- R1-2109010, Challenges and potential enhancements of XR, vivo.

- TR 38.840, “Study on User Equipment (UE) power saving in NR (Release 16)” V16.0.0 (2019-06).

# Appendix A – Simulation assumptions

Table I. Simulation assumption for FR1

| Parameter | value |  |  |
| --- | --- | --- | --- |
| Scenarios | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m | Scenario-2: Dense Urban hexagonal layout with 7, 3 Sectors | Scenario-3: Urban Macro hexagonal layout with 7, 3 Sectors |
| Channel model | InH | Uma | Uma |
| Carrier frequency | 4GHz |  |  |
| Bandwidth | 100 MHz, 1.72% Guard Band |  |  |
| Subcarrier spacing | 30 KHz |  |  |
| Frame structure | DDDSU (S: 10D:2G:2U) |  |  |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32T: (4,4,2,1,1;4,4) (dH,dV) = (0.5, 0.5)λ | For 64T: (8,8,2,1,1;4,8) (dH, dV)=(0.5, 0.5)λ |  |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | DL: For 4R: (1,2,2,1,1;1,2), (dH, dV)=(0.5, N/A)λ UL: For 2T: (1,1,2,1,1;1,1), (dH, dV)=( 0.5, N/A)λ |  |  |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi | 3-TRxP pattern, 8 dBi | 3-TRxP pattern, 8 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |  |  |
| BS Power | 24 dBm per 20MHz | 44 dBm per 20MHz | 49 dBm/20 MHz |
| UE max Power | 23 dBm |  |  |
| UE Power | Max Tx power: 23 dBm, (P0 = -80, alpha = 0.8) | Max Tx power: 23 dBm, (P0 = -74, alpha = 0.6) | Max Tx power: 23 dBm, (P0 = -74, alpha = 0.6) |
| ISD | 20 m | 200 m | 500m |
| BS height | 3 m | 25 m | 25m |
| UE height | 1.5 m | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl~uniform(1,Nfl) where Nfl~uniform(4,8) |  |
| Noise Figure | BS:5 dB, UE:9 dB |  |  |
| Max MCS | 256QAM |  |  |
| Device deployment | 100% indoor | 20% outdoor, 80% indoor |  |
| Down-tilt | 90 degrees | 12 degrees | 6 degrees |
| BS receiver | MMSE-IRC |  |  |
| UE receiver | MMSE-IRC |  |  |
| Channel estimation | Realistic |  |  |
| Target BLER | 10% |  |  |
| UE speed | 3 km/h |  |  |

Table II. Simulation assumption for FR2

| Parameter | value |  |
| --- | --- | --- |
| Scenarios | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m | Scenario-2: Dense Urban hexagonal layout with 7, 3 Sectors |
| Carrier frequency | 30 GHz |  |
| Bandwidth | 100 MHz, 4.96% Guard Band 400 MHz, 1.72% Guard Band |  |
| Subcarrier spacing | 120 KHz |  |
| Frame structure | DDDSU (S: 10D:2G:2U) |  |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 2T: (16,8,2,1,1;1,1) (dH, dV)=(0.5, 0.5)λ | For 2T/panel: (4,8,2,2,2;1,1) (dH, dV)=(0.5, 0.5)λ (dg,H,dg,V) = (4.0, 2.0) λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | For 4R/panel: (1,4,2,1,3;1,2) (dH,dV) = (0.5, 0.5)λ (dg,V,dg,H) = (0, 0)λ |  |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi | 3-TRxP pattern, 8dBi |
| UE antenna pattern | Directional antenna panel, 5 dBi |  |
| Min. BE-UT distance(2D) | 0 m | 10 m |
| BS height | 3 m | 25 m |
| UE height | 1.5 m | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl~uniform(1,Nfl) where Nfl~uniform(4,8) |
| BS Power | 23dBm per 80MHz, EIRP should not exceed 58 dBm | 40dBm per 80MHz, EIRP should not exceed 73 dBm |
| UE Power | Max Tx power: 23 dBm, maximum EIRP 43 dBm, (P0 = -60, alpha = 0.6) | Max Tx power: 23 dBm, maximum EIRP 43 dBm, (P0 = -74, alpha = 0.6) |
| Noise Figure | BS:7 dB, UE:13 dB |  |
| Scheduler | SU-MIMO Proportional Fair |  |
| Max MCS | 256QAM |  |
| Device deployment | 100% indoor | 100% outdoor |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic |  |
| Target BLER | 10% |  |
| UE speed | 3 km/h |  |