3GPP TSG RAN WG1 #106bis-e		R1-2109068

e-Meeting, October 11th – 19th, 2021

Source:	OPPO

Title:	UL feedback for RRC-CONNECTED UEs in MBS

Agenda Item:	8.12.2

Document for:	Discussion and Decision

Introduction

In RAN1#106-e meeting, how to improve reliability of RRC-Connected UE in MBS was discussed and the following agreements and working assumption were achieved [1]:

Agreement:

For UE supporting both unicast and multicast, the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList can be separately configured for multicast from that for unicast.

Agreement:

When UE is configured Type-1 codebooks for unicast and multicast with different priorities, respectively, the UE separately generates each of the Type-1 codebooks.

- FFS: How UE is configured one codebook for unicast and one codebook for multicast and the two codebooks are of different priorities.

Agreement:

For a UE configured with Type-1 HARQ-ACK codebook,

- If UE is not configured to receive FDM-ed unicast and multicast, Type-1 HARQ codebook is generated as the agreement for TDM-ed unicast and multicast.

- If UE is configured to receive FDM-ed unicast and multicast, Type-1 HARQ codebook is generated as the agreement for FDM-ed unicast and multicast.

Agreement:

For UEs supporting ACK/NACK-based HARQ-ACK feedback for multicast and unicast, the following values are unchanged compared to unicast in Rel-16:

  - The maximum number of PUCCH resources sets in each PUCCH-Config,

  - The maximum number of PUCCH resources in a PUCCH resource set in each PUCCH-Config,

  - The maximum number of UCI information bits for the first PUCCH resource set.

  - The total number of PUCCH resources from all PUCCH-Config/PUCCH-ConfigurationList.

  - Note:

    - This applies to both cases of whether or not UE is configured optionally with a separate PUCCH-Config or PUCCH-ConfigurationList for multicast.

    - The case of NACK-only based is discussed separately.

Agreement:

When UE is configured with the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList for ACK/NACK based feedback for multicast, it is applied to all G-RNTIs configured to UE.

Agreement:

For the separate PUCCH-ConfigurationList that is optionally configured to UE for NACK-only based HARQ-ACK feedback for multicast,

  - The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority feedback and high priority feedback, respectively.

  - FFS: how to handle the case when separate PUCCH-ConfigurationList is not configured to UE for NACK-only based HARQ-ACK feedback for multicast.

Agreement:

The priority index is,

- for the second DCI format for GC-PDCCH, optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

- for the first DCI format for GC-PDCCH, down-select from:

    - Alt1: Optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

    - Alt2: Always low priority, i.e., the priority index is not included in the DCI format.

Agreement:

The priority of multicast for NACK-only based feedback is the same as the priority of unicast for the same priority index of HARQ-ACK.

Agreement:

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, down-select from the following alternatives:

  - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

  - Alt2: Support sub-slot based PUCCH for this case.

  - Alt3: Support UE transmitting more than one slot-based PUCCHs in the same PUCCH slot.

  - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

  - Alt5: NACK-only bundling

Agreement:

When UE supports and is configured with more than one G-RNTI,

  - for Type-2 codebook construction, DAI is separately counted per G-RNTI.

  - Type-2 codebook is constructed by concatenating Type-2 sub-codebook of each RNTI following the ascending order of the G-RNTI value.

Agreement:

Update the WA made in RAN1#105-e meeting regarding enabling/disabling HARQ-ACK feedback as follows:

Working assumption:

For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH:

- RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback.

  - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled

  - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling.

  - FFS details on RRC signaling and group-common DCI indicating.

- FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases.

- FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction.

- FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission.

- FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback.

- FFS whether/how to apply it to SPS group-common PDSCH.

- UE capability for enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is introduced and FFS details.

- Note: It is up to network implementation to avoid any potential HARQ ACK mismatch between different UEs in the same multicast group

Agreement

For UE supports both ACK/NACK-based and NACK-only based HARQ-ACK feedback for multicast SPS PDSCH without PDCCH scheduling, select one or more of the following alternatives:

  - Alt1: HARQ-ACK feedback option is configured per SPS configuration index.

  - Alt2: HARQ-ACK feedback option is indicated in the SPS activation DCI.

  - Note: enabling/disabling HARQ-ACK feedback for multicast SPS can be discussed separately.

In this contribution, we continue discussing the mechanism to improve reliability of Broadcast/Multicast service for RRC-CONNECTED UEs.

Discussion

  - ACK/NACK based HARQ-ACK feedback

In RAN1#105-e meeting [2] it was agreed that separate codebooks for unicast and multicast could be configured at least for different priorities. As also agreed in RAN1#104 and RAN1#104bis respectively, a separate PUCCH-Configuration/PUCCH-ConfigurationList can be optionally configured for multicast, hence for unicast and multicast with same priority index, separate codebooks for unicast and multicast can also be configured if PUCCH resources configured for unicast and multicast are not overlapping.

- For the case of same priority for multicast and uncast, separate codebook can be configured for multicast and unicast.

In Rel-16 unicast UE can be configured with 1 or 2 PUCCH-config, if 2 PUCCH-config are provided, the 2 configurations are used for priority 0 and priority 1 respectively, and HARQ-ACK of different priorities are not multiplexed in one PUCCH. If separate PUCCH-config/PUCCH-ConfigurationList for MBS is not configured by gNB HARQ-ACK for MBS and unicast are supposed to be multiplexed into a single HARQ-ACK codebook and to be transmitted in the same PUCCH resource. As HARQ-ACK feedback for MBS may also have 2 priorities, and the priority of multicast is the same as the priority of unicast for the same priority index, UE should also be configured to generate up to 2 HARQ-ACK codebooks for MBS. In general, following 3 configurations may happen (assuming that UE should not be configured with up to 2 HARQ-ACK codebooks if only 1 PUCCH-config is configured):

- 1 PUCCH-config for unicast and 1 HARQ-ACK codebook for MBS (i.e., only Priority #0) is configured. In this case, HARQ-ACK bits for MBS should be multiplexed with that for unicast into one HARQ-ACK codebook.

- 2 PUCCH-config for unicast and 1 HARQ-ACK codebooks are configured for MBS (i.e., only Priority #0). In this case, HARQ-ACK bits for MBS should be multiplexed with that for unicast priority 0, and transmitted in the PUCCH of priority 0.

- 2 PUCCH-config for unicast and 2 HARQ-ACK codebooks are configured for MBS (i.e., Priority #0 and Priority #1). In this case, HARQ-ACK bits for MBS of priority 0/1 should be multiplexed with that of unicast priority 0/1, and transmitted in PUCCH of priority 0/1 respectively.

- If PUCCH-config/PUCCH-ConfigurationList for unicast is also applied for MBS, HARQ-ACK bits of unicast and MBS with same priority are multiplexed and transmitted in the PUCCH of the same priority level.

If HARQ-ACK feedback for MBS and that for unicast are multiplexed into a single HARQ-ACK codebook and Type 2 HARQ-ACK codebook is configured, HARQ-ACK sub-codebook for MBS is appended to the end of HARQ-ACK sub-codebook for unicast. In the last meeting, it was further agreed that DAI is separately counted per G-RNTI. In Rel-17 it is preferable that at most one Type 2 MBS HARQ-ACK sub-codebook is concatenated with unicast Type 2 HARQ-ACK sub-codebook, otherwise, more blind detection is needed at gNB side due the ambiguity of each Type 2 MBS HARQ-ACK sub-codebook size.

- At most one Type 2 MBS HARQ-ACK sub-codebook is concatenated with type 2 unicast HARQ-ACK sub-codebook.

If separate PUCCH-config/PUCCH-ConfigurationList for MBS is configured by gNB, HARQ-ACK for MBS and unicast are supposed to be transmitted in separate PUCCH resources. Following unicast design, the number of PUCCH-config configured for MBS should also be same as the number of HARQ-ACK priority levels for MBS. In this case, HARQ-ACK codebook for MBS and unicast should be constructed separately and transmitted in PUCCH resource dedicated for MBS or unicast.

- If separate PUCCH-config/PUCCH-ConfigurationList is configured for MBS, HARQ-ACK codebook for MBS and unicast are constructed separately and transmitted in PUCCH resource dedicated for MBS or unicast.

There were 2 alternatives on Type-1 HARQ-ACK codebook construction for TDM-ed unicast and multicast agreed in RAN1#105-e meeting for down selection, where the union of the PDSCH TDRA sets for all values of K1 of unicast and multicast are used for the codebook construction in Alt 2, and Alt 1 is a kind of optimization on top of Alt 2. Considering that Alt 2 has already been agreed in RAN1#104-e (as reproduced below), and payload size reduction is not a design target of Type-1 HARQ-ACK codebook, there is no strong motivation to support Alt 1.

Agreement: For ACK/NACK based feedback if supported for multicast, construction of Type-1 HARQ-ACK codebook based on the union of the PDSCH TDRA sets of the unicast service and the multicast service (if they are separately configured), at least of the same priority, is supported - FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast. - FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed multicast and multicast if supported. FFS: whether/how to optimize the Type-1 codebook construction to reduce the HARQ-ACK feedback payload

- For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, PDSCH reception candidate occasions are based on Alt 2.

  - NACK-only based HARQ-ACK feedback mechanism

In RAN1#105-e meeting it was agreed that both PUCCH format 0 and PUCCH format 1 are supported for NACK-only based HARQ-ACK feedback. For a UE which PUCCH format is used for NACK-only based HARQ-ACK feedback should be semi-statically configured by RRC. As UE may have multiple G-RNTI [2], for each G-RNTI using NACK-only based HARQ-ACK feedback, the corresponding PUCCH resources should be separately configured.

- The PUCCH format used for NACK-only based HARQ-ACK feedback is semi-statically configured by RRC.

- The PUCCH resource for NACK-only based HARQ-ACK feedback is configured per G-RNTI.

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, 5 alternatives were agreed in last meeting for down selection. For simplicity only one HARQ-ACK bit should be reported in NACK-only based HARQ feedback scheme. If 2 TB transmissions within one PDSCH is supported, or HARQ feedback for more than one MBS PDSCH are transmitted in one UL slot, “and” operation should be performed for the HARQ-ACK bits to generate only 1 bit to transmit.

- When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, NACK-only bundling is performed to generate only 1 HARQ-ACK bit (i.e., Alt 5).

  - Enabling/disabling HARQ feedback

How to enable/disable HARQ-ACK feedback was discussed in the last meeting with one working assumption achieved. As UE may have multiple G-RNTI to receive multiple MBS services, and gNB may also expect different HARQ-ACK feedback manner for different MBS services, hence the RRC signaling for enabling/disabling function of group-common DCI or enabling/disabling HARQ-ACK feedback should be configured per G-RNTI. Furthermore, as both ACK/NACK based HARQ-ACK feedback and NACK-only based HARQ-ACK feedback are supported, for a G-RNTI with enabling/ disabling function of group-common DCI configured by RRC, 2 bits in DCI can be used to indicate whether/which HARQ feedback is enabled, as shown in the Table 2.

Table 2 HARQ feedback mode indicator

| Value of HARQ feedback indicator | HARQ feedback mode |
| --- | --- |
| 00 | HARQ feedback disabled |
| 01 | NACK-only based HARQ feedback |
| 10 | ACK/NACK based HARQ-feedback |
| 11 | reserved |

- For HARQ feedback enabling/disabling of dynamic group-common PDSCH:

  - RRC signalling for enabling/disabling the function of group-common DCI or enabling/disabling HARQ-ACK feedback is configured per G-RNTI.

  - 2 bits in group-common DCI are used to indicate whether/which HARQ feedback is enabled;

For SPS group-common PDSCH, as there is no associated group-common DCI, whether HARQ-ACK is enabled or disabled should be configured by RRC signaling, i.e. RRC signaling for enabling/disabling function of group-common DCI is not applied. However, for the reliability of the DCI activating/deactivating SPS configuration, the same mechanism as that for dynamic group-common PDSCH should be used.

- HARQ-ACK feedback enabling/disabling of SPS group-common PDSCH is configured by RRC signaling.

- HARQ-ACK feedback enabling/disabling mechanism for dynamic group-common PDSCH is reused for the DCI activating/deactivating SPS configuration.

  - PUCCH power control

In case of group-common PDSCH is transmitted with PTM scheme 1, as there is no UE specific PDCCH, gNB cannot adjust the PUCCH transmission power per UE, this may impact the reception of HARQ feedback for the group-common PDSCH at gNB. To address the problem, a TPC-PUCCH-RNTI should be configured for a UE within the MBS group such that gNB can adjust the PUCCH transmission power of the UE. Considering that the TRP for MBS and that for unicast may be different, and even though the TPR for unicast and MBS are same, gNB may expect different transmission power for PUCCH carrying NACK-only HARQ-ACK bit (if configured), hence it is preferable that a separate TPC-PUCCH-RNTI is configured for MBS.

- For a UE receiving group-common PDSCH transmitted with PTM scheme 1 a TPC-PUCCH-RNTI different from that for unicast should be configured.

  - Shared or separate PUCCH resource configuration for multicast

For ACK/NACK based HARQ feedback, if the number of UEs within the group is limited, it is beneficial to allocate separate PUCCH resource for each UE within the group so that the gNB can differentiate which UE feeds back ACK or NACK. And accordingly, the gNB can re-transmit to the UE that does not receive the multicast service correctly, such as based on PTP transmission with suitable MCS level. While if the number of UEs within the group is large, to reduce PUCCH resource overhead, a shared PUCCH resource among all UEs within the group is also preferable.

- For ACK/NACK based HARQ-ACK feedback for multicast, both shared and separate PUCCH resources among UEs within the group are supported.

Furthermore, a subset of UEs within the group can share the same PUCCH resource, different subsets of UEs can be configured with different PUCCH resources. For example, the receiver of MBS can be divided into several sets based on RSRP range. The UEs within the same set or the same RSRP range share the same PUCCH resource. Different set of UEs can be configured with different PUCCH resource so that gNB can differentiate whether and which set of UEs have received the data correctly or not.

Figure 1 Illustration of RSRP based PUCCH resource allocation

One example is given in Figure 1, where 3 RSRP thresholds and 4 set of PUCCH resources are configured, each set of PUCCH resources corresponds to different RSRP range. Within each PUCCH resource set, there are two PUCCH resources configured, one for ACK and another for NACK. The UEs within the same RSRP range will share the same PUCCH resource for ACK or NACK. Based on the UL feedback corresponds to different RSRP ranges, gNB can know the HARQ status of UEs per RSRP ranges.

The above PUCCH resource configuration method can also be applied to NACK-only feedback mechanism. For example, for each RSRP range, only NACK resource is configured and the UEs within the RSRP range share the same PUCCH resource for NACK reporting. Different PUCCH resources can be configured for UEs located within different RSRP range.

- For ACK/NACK based feedback and NACK only based feedback for multicast, RSRP based PUCCH resource configuration is supported.

Whether shared or separate PUCCH resources are used can be up to gNB configuration or scheduling. No necessary to specify such behaviour.

- Whether shared or separate PUCCH resources are used can be up to gNB configuration or scheduling.

Conclusion

In this contribution, we discuss the mechanisms to improve reliability of MBS. Based on the discussion, the following proposals are given:

- For the case of same priority for multicast and uncast, separate codebook can be configured for multicast and unicast.

- If PUCCH-config/PUCCH-ConfigurationList for unicast is also applied for MBS, HARQ-ACK bits of unicast and MBS with same priority are multiplexed and transmitted in the PUCCH of the same priority level.

- At most one Type 2 MBS HARQ-ACK sub-codebook is concatenated with type 2 unicast HARQ-ACK sub-codebook.

- If separate PUCCH-config/PUCCH-ConfigurationList is configured for MBS, HARQ-ACK codebook for MBS and unicast are constructed separately and transmitted in PUCCH resource dedicated for MBS or unicast.

- For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, PDSCH reception candidate occasions are based on Alt 2.

- The PUCCH format used for NACK-only based HARQ-ACK feedback is semi-statically configured by RRC.

- The PUCCH resource for NACK-only based HARQ-ACK feedback is configured per G-RNTI.

- When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, NACK-only bundling is performed to generate only 1 HARQ-ACK bit (i.e., Alt 5).

- For HARQ feedback enabling/disabling of dynamic group-common PDSCH:

  - RRC signalling for enabling/disabling the function of group-common DCI or enabling/disabling HARQ-ACK feedback is configured per G-RNTI.

  - 2 bits in group-common DCI are used to indicate whether/which HARQ feedback is enabled;

- HARQ-ACK feedback enabling/disabling of SPS group-common PDSCH is configured by RRC signaling.

- HARQ-ACK feedback enabling/disabling mechanism for dynamic group-common PDSCH is reused for the DCI activating/deactivating SPS configuration.

- For a UE receiving group-common PDSCH transmitted with PTM scheme 1 a TPC-PUCCH-RNTI different from that for unicast should be configured.

- For ACK/NACK based HARQ-ACK feedback for multicast, both shared and separate PUCCH resources among UEs within the group are supported.

- For ACK/NACK based feedback and NACK only based feedback for multicast, RSRP based PUCCH resource configuration is supported.

- Whether shared or separate PUCCH resources are used can be up to gNB configuration or scheduling.

Reference

[1] Chairman note’s, RAN1#106-e, August 2021.

[2] Chairman note’s, RAN1#105-e, May 2021.