3GPP TSG RAN WG1 #106bis-e		R1-2109278

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.3.1.2

Title:	Discussion on CSI feedback enhancements for URLLC

Source:	CMCC

Document for:	Discussion and Decision

- Introduction

In RAN1#106-e meeting, agreements were reached on the granularity of CQI report and delta-MCS, as follows:

Agreement For subband CQI reporting with more than 2 bits per subband - Support 4-bits CQI only Agreement For subband CQI reporting in Rel-17, RRC can configure use of legacy 2-bits D-CQI or 4-bits CQI for each CSI report configuration. - This feature is subject to UE capability - FFS: Whether wideband CQI report can be omitted Conclusion There is no consensus in RAN1 on the support of delta-MCS in Rel-17.

In this contribution, we’d like to state our view for a remaining issue.

- Increasing the number of bits for sub-band CQI report

Increasing granularity of sub-band CQI improves the accuracy of the sub-band report and bring more information on the whole picture of the channel such as the signal quality distribution. It was agreed that 4-bits sub-band CQI is supported so now the finest granularity is achieved on sub-band level.

There is a remaining issue as whether the 4-bit wideband CQI is still needed.

The benefit of omitting wideband CQI is a 4-bits overhead reduction. However, the cost is:

- CSI reporting framework is changed

- Extra RRC parameter is required to switch on/off the wideband CQI

- Extra implementation effort is introduced on UE side to check condition and decide whether to calculate/report the wideband CQI

- Extra implementation effort is introduced on gNB side to check condition and decide whether to simply read wideband CQI or to calculate one.

- In industries automation scenario, it is usually more important that the performance requirements (latency, reliability) are met or even over-reached than optimizing overhead, because the site and spectrum are dedicated to the customer in many cases.

Considering the above pros/cons, and the fact that 4 bits extra overhead is marginal when all sub-band CQIs are reported, we prefer to report wideband CQI as usual so the current CQI reporting framework is kept.

Proposal 1: When 4-bits sub-band CQI is applied to UE, wideband CQI is still reported.

- Conclusions

. Proposal 1: When 4-bits sub-band CQI is applied to UE, wideband CQI is still reported.

- References

- R1-2108450, Feature lead summary #4 on CSI feedback enhancements for enhanced URLLC/IIoT after RAN1#106-e