3GPP TSG-RAN WG1#106bis-e	Tdoc R1-2109725

e-Meeting, October 11-19, 2021

Agenda Item:	8.17.3

Source:	Ericsson

Title:	Rel-17 UE Features for IIoT/URLLC

Document for:	Discussion, Decision

# Introduction

In this contribution, Rel-17 UE Features for IIoT/URLLC are discussed. For each enhancement aspect, the candidate UE features are proposed in each section below.

# Discussion

## UE Feedback Enhancements for HARQ-ACK

### PUCCH repetition enhancements

For FG 25-3a, the description “using dynamic repetition indication” could be understood as there is an explicit repetition indication in DCI for dynamic repetition while the dynamic repetition is implicitly achieved by configuring repetition factor in PUCCH resource when a PUCCH resource is dynamically indicated as in legacy. To avoid misinterpretation and improve consistency, we proposed the following changes in red for proposed FG 25-2, FG25-3, FG25-3a.

Adopt the proposed changes in red in Table 1 for the proposed FG 25-2, 25-3 and 25-3a in R1-2108679.

Table 1: Proposed FG for PUCCH repetition in R1-2108679

| Index | Feature group | Components | Prerequisite feature groups | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 25-2 | Semi-static repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 0 and 2 over multiple slots with RRC configured repetition factor K = 2, 4, 8 | 4-23 |  | Optional with capability signaling |
| 25-3 | Semi-static repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 | 4-23 11-3 |  | Optional with capability signaling |
| 25-3a | Dynamic repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition factor RRC configured in a PUCCH resource indication. | 25-3 30-X |  | Optional with capability signaling |

### Enhanced Type-3 HARQ-ACK codebook

With respect to enhanced Type-3 codebook features and related RRC parameters, the relevant agreements are listed below and numbered for convenience.

Agreement#1 Support Rel-16 Type 3 HARQ-ACK CB triggering using DCI format 1_2 in Rel-17 for a UE supporting DCI format 1_2. The support is subject to a Rel-17 UE capability and a UE supporting this capability can be configured with DCI format 1_2 triggering of the Rel-16 Type 3 HARQ-ACK CB. Agreement#2 Support PHY priority handling for a PUCCH carrying the Rel-16 Type 3 HARQ-ACK CB in Rel-17. Agreement#3 Support Rel-17 enhanced Type 3 HARQ-ACK CB of smaller size triggering using DCI format 1_2 for a UE supporting DCI format 1_2. - The triggering support for DCI format 1_2 is independently (from triggering using DCI format 1_1) RRC configured to the UE. Agreement#4 Support PHY priority handling for a PUCCH carrying the Rel-17 enhanced Type 3 HARQ-ACK CB of smaller size. Agreement#5 For enh. Type 3 HARQ-ACK CB(s), support dynamic selection based on indication in the triggering DCI of one of at least one enh. Type 3 HARQ-ACK CB(s). - Each of the at least one enh. Type 3 HARQ-ACK CBs is at least defined by RRC configuration This includes the option to configure all DL HARQ process of all configured CCs as one enh. Type 3 HARQ-ACK CB (resulting in same structure and size as the Rel-16 Type 3 HARQ-ACK CB) - This includes UE capability signaling (value range {1…X}) on the maximum number of supported simultaneously configured enh. Type 3 HARQ-ACK CBs that can be dynamically indicated - Details including the value of X are FFS

First, we provide an overview of the enhancements in Rel-17 for Type-3 HARQ-ACK CB as compared to Rel-16. We would like to emphasize that in discussion as well as description of the agreements, only size reduction is referred to an enhancement. However, it should be well understood that any improved functionality for Type-3 codebook as compared to Rel-16 is an enhancement in Rel-17.  With that understanding, the Rel-17 enhancements can fit into three categories as the following:

- Type-3 HARQ-ACK codebook enhancements in Rel-17:

  - Category 1: Triggered by only DCI 1_1 in Rel-16 => Triggered by DCI 1_1 or DCI 1_2 in Rel-17

  - Category 2: Associated to PUCCH with low priority in Rel-16 => Associated to PUCCH with low or high priority in Rel-17

  - Category 3: With only full size in Rel-16 => With reduced sizes in Rel-17

Table 2: Overview of enhanced Type-3 codebook features in Rel-17 as compared to Rel-16

| Rel- 16 oneShotHARQ-feedback | Rel-17 Enh. Type-3 HARQ-ACK CB |  |  |
| --- | --- | --- | --- |
| FG: 10-16 | FG :25-A1 (25-4) |  |  |
| CB: Full size Trigger: DCI 1_1 Priority: low | CB: Full size (as in Rel-16) Trigger: DCI 1-1/1-2 Priority: low Pre-requisite FG: 10-16,11-1 |  | Category 1 |
|  |  | FG :25-B1 (25-5) |  |
|  |  | CB: Full size (as in Rel-16) Trigger: DCI 1_1/1-2 Priority: low/high Pre-requisite FG: 10-16, 11-4 | Category 2 |
|  | FG :25-A2 (25-4a, X=1) | FG :25-B2 (25-5a, X=1) |  |
|  | CB: One reduced size Trigger: DCI 1-1/1-2 Priority: low Pre-requisite FG: 10-16,11-1 | CB: One reduced size CB Trigger: DCI 1_1/1-2 Priority: low/high Pre-requisite FG: 10-16, 11-4 | Category 3 |
|  | FG :25-A3 (25-6a, X=2) | FG :25-B3 (25-6, X=2) | Category 3 ext. |
|  | CB: Multiple reduced size Trigger: DCI 1-1/1-2 Priority: low Dynamic selection between X CB sizes. Pre-requisite FG: 10-16,11-1 | CB: Multiple reduced size Trigger: DCI 1_1/1-2 Priority: low/high Dynamic selection between X CB sizes. Pre-requisite FG: 10-16, 11-4 |  |

Next, we provide an overview in Table 2 in order to have a clear understanding on the additional functionality that a feature supports as compared to Rel-16 when incorporating prerequisite features. Several aspects are clarified in the following.

- Category 1 - FG 25-A1:

  - This feature addresses the enhancement supported by Agreement#1 where the Type-3 HARQ-ACK CB can be triggered by DCI 1_2 with low priority/without priority indication. Note that excluding triggering by DCI 1-1 from this feature is problematic and not well motivated for the following reasons:

    - The prerequisite feature FG 10-16 supports triggering by DCI 1-1. It is conflicting to exclude DCI 1-1 and use FG 10-16 as prerequisite. If FG 10-16 is not used as prerequisite, a similar feature as 10-16 but for DCI 1_2 should be developed. This brings additional complexity for signalling.

    - From testing of related functionalities, triggering by DCI 1_2 is no different from DCI 1_1.

    - Agreement#2 requires FG 11-1 to be prerequisite. However, it does not state the functionality in this case is only supported for DCI 1_2.

  - Therefore, FG 25-A1 corresponds to FG 25-4 with following modification and should be supported.

| Index | Feature group | Components | Prerequisite feature groups | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 25-4 | One-shot HARQ ACK feedback Type 3 HARQ-ACK codebook triggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook with full size with low priority, triggered by a DCI format 1_1 or 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook with full size with low priority, triggered by a DCI format 1_1 or 1_2 without scheduling a PDSCH using a reserved FDRA value | 10-16 11-1 | A type 3 HARQ-ACK codebook with full size is a type 3 HARQ-ACK codebook constructed as in Rel-16. | Optional with capability signaling |

- Category 2 - FG 25-B1:

  - This feature addresses the enhancement supported by Agreement#2 where two-level priority is supported for Type-3 HARQ-ACK CB in full size as in Rel-16. Note that two-level priority is enabled by FG 11-4 which supports both DCI formats 1-1/1-2.

  - Therefore, FG 25-B1 corresponds to FG 25-5 with following modification and should be supported.

| Index | Feature group | Components | Prerequisite feature groups | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 25-5 | PHY priority handling for one-shot HARQ ACK feedback Type 3 HARQ-ACK codebook | Support transmission of type 3 HARQ-ACK codebook with full size using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI. | 10-16 11-4 | A type 3 HARQ-ACK codebook with full size is a type 3 HARQ-ACK codebook constructed as in Rel-16. | Optional with capability signaling |

- Category 3:

  - The features corresponding to this category are extension of previous features with support of reduced size Type-3 HARQ-ACK codebook. There are few aspects that need to be considered:

    - A feature that supports reduced size Type-3 HARQ-ACK codebook, has FG 10-16 as prerequisite with full size CB. However, the feature overrides the full size to a reduced size by RRC configuration as agreed.

      - FG 25-A2 and FG 25-B2 represents enhancements by Agreement#3 and Agreement#4, respectively.

    - Due to Agreement#5, X individual capabilities are needed if UE supports simultaneously maximum X reduced sizes of Type3 HARQ-ACK codebook. Table 2 illustrates the case for X=1 and X=2 corresponding to FG25-A2/B2 and FG 25-A3/B3, respectively.

    - Due to Agreement#5, for X>1, the UE should support dynamic indication for selection between X reduced sized Type-3 HARQ-ACK codebook.

      - Note that from our point of view, only X=1 should be supported as we explained in R1-2108829.

  - Therefore, the following features cover the related discussions and agreements and are described as the following:

    - FG 25-A2 corresponds to FG 25-4a as reduced CB size version of FG 25-4 with X=1

    - FG 25-B2 corresponds to FG 25-5a as reduced CB size version of FG 25-5 with X=1

    - FG 25-A3 corresponds to FG 25-6a as enhanced version of FG 25-4a with X=2 and dynamic selection

    - FG 25-B3 corresponds to FG 25-6 as enhanced version of FG 25-5a with X=2 and dynamic selection

  - Note that from our point of view FG 25-A3 and FG 25-B3 are not supported.

| Index | Feature group | Components | Prerequisite feature groups | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 25-4a | Reduced size Type 3 HARQ-ACK codebook triggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook with reduced size with low priority, triggered by a DCI format 1_1 or 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook with reduced size with low priority, triggered by a DCI format 1_1 or 1_2 without scheduling a PDSCH using a reserved FDRA value | 10-16, 11-1 | The size of Type 3 HARQ-ACK codebook can be reduced by configuration as compared to the size of the type 3 HARQ-ACK codebook constructed as in Rel-16. | Optional with capability signaling |
| 25-5a | PHY priority handling for reduced size Type 3 HARQ-ACK codebook | Support transmission of type 3 HARQ-ACK codebook with reduced size using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI. | 10-16, 11-4 | The size of Type 3 HARQ-ACK codebook can be reduced by configuration as compared to the size of the type 3 HARQ-ACK codebook constructed as in Rel-16. | Optional with capability signaling |
| FFS: 25-6 | Enhanced Reduced type 3 HARQ-ACK codebook feedback with dynamic codebook size selection | 1. Support feedback of enhanced type 3 HARQ-ACK codebook with reduced size, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to X=2 enhanced reduced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced size for type 3 HARQ-ACK codebook (FFS: based on triggering information in DCI 1_1 and DCI 1_2) (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced reduced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) | 10-16, 11-4 | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1,...,X} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured | Optional with capability signaling |
| FFS: 25-6b | Reduced size Type 3 HARQ-ACK codebook triggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook with reduced size with low priority, triggered by a DCI format 1_1 or 1_2 2. Support configuration of up to X=2 reduced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected size for type 3 HARQ-ACK codebook (FFS: based on triggering information in DCI 1_1 and DCI | 10-16, 11-1 | The size of Type 3 HARQ-ACK codebook can be reduced by configuration as compared to the size of the type 3 HARQ-ACK codebook constructed as in Rel-16. | Optional with capability signaling |

Based on the above discussion, we propose the following:

Adopt the proposed changes in red in Table 1 for the proposed FG 25-2, 25-3 and 25-3a in R1-2108679

Adopt the proposed changes in red in Table 3 for the proposed FG 25-2, 25-3 and 25-3a in R1-2108679

Table 3: Proposed FG for Enhanced Type-3 HARQ-ACK codebook in R1-2108679. Proposed FG 25-4, FG 25-5, FG 25-4a, FG 25-5a are supported. Proposed FG 25-6 and FG 25-6a are not supported.

| Index | Feature group | Components | Prerequisite feature groups | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 25-4 | One-shot HARQ ACK feedback Type 3 HARQ-ACK codebook triggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook with full size with low priority, triggered by a DCI format 1_1 or 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook with full size with low priority, triggered by a DCI format 1_1 or 1_2 without scheduling a PDSCH using a reserved FDRA value | 10-16 11-1 | A type 3 HARQ-ACK codebook with full size is a type 3 HARQ-ACK codebook constructed as in Rel-16. | Optional with capability signaling |
| 25-5 | PHY priority handling for one-shot HARQ ACK feedback Type 3 HARQ-ACK codebook | Support transmission of type 3 HARQ-ACK codebook with full size using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI. | 10-16 11-4 | A type 3 HARQ-ACK codebook with full size is a type 3 HARQ-ACK codebook constructed as in Rel-16. | Optional with capability signaling |
| 25-4a | Reduced size Type 3 HARQ-ACK codebook triggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook with reduced size with low priority, triggered by a DCI format 1_1 or 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook with reduced size with low priority, triggered by a DCI format 1_1 or 1_2 without scheduling a PDSCH using a reserved FDRA value | 10-16, 11-1 | The size of Type 3 HARQ-ACK codebook can be reduced by configuration as compared to the size of the type 3 HARQ-ACK codebook constructed as in Rel-16. | Optional with capability signaling |
| 25-5a | PHY priority handling for reduced size Type 3 HARQ-ACK codebook | Support transmission of type 3 HARQ-ACK codebook with reduced size using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI. | 10-16, 11-4 | The size of Type 3 HARQ-ACK codebook can be reduced by configuration as compared to the size of the type 3 HARQ-ACK codebook constructed as in Rel-16. | Optional with capability signaling |
| FFS: 25-6 | Enhanced Reduced type 3 HARQ-ACK codebook feedback with dynamic codebook size selection | 1. Support feedback of enhanced type 3 HARQ-ACK codebook with reduced size, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to X=2 enhanced reduced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced size for type 3 HARQ-ACK codebook (FFS: based on triggering information in DCI 1_1 and DCI 1_2) (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced reduced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) | 10-16, 11-4 | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1,...,X} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured | Optional with capability signaling |
| FFS: 25-6b | Reduced size Type 3 HARQ-ACK codebook triggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook with reduced size with low priority, triggered by a DCI format 1_1 or 1_2 2. Support configuration of up to X=2 reduced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected size for type 3 HARQ-ACK codebook (FFS: based on triggering information in DCI 1_1 and DCI | 10-16, 11-1 | The size of Type 3 HARQ-ACK codebook can be reduced by configuration as compared to the size of the type 3 HARQ-ACK codebook constructed as in Rel-16. | Optional with capability signaling |

## CSI Feedback Enhancements

For FG 25-11 in the preliminary list [1], no prerequisite FG is stated. However, the 4-bit subband CQI reporting should be an enhancement of basic CSI feedback. Hence FG 2-32 “Basic CSI feedback” should be added as a prerequisite as shown below.

Table 4: Proposed FG for enhanced CSI feedback.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-11 | 4-bits subband CQI | Subband CQI reporting with 4 bits per subband | 2-32 |

Adopt the proposed changes in red in Table 4 for the proposed FG 25-11 in R1-2108679.

## Enhancements for Unlicensed Band URLLC/IIoT

Void

## Intra-UE Multiplexing/Prioritization

For the topic of simultaneous PUCCH/PUSCH for CA, currently it has been agreed to support it for inter-band CA. For intra-band CA, it’s still FFS. Based on companies’ input, the main concern of supporting simultaneous PUCCH/PUSCH for intra-band CA is phase discontinuity if the PUCCH and PUSCH do not start and end at the same times. Phase discontinuity has been discussed since Rel-15. FG 6-23 was introduced for UE to indicate the incapability of PA phase discontinuity. As shown below, FG 6-23 can cover overlapping PUSCH-PUSCH as well as overlapping PUCCH-PUSCH. With this understanding, simultaneous PUCCH/PUSCH for intra-band CA should be supported for UEs that do not indicate the incapability FG 6-23. Thus we recommend to introduce the FG 25-18a, as shown in Table 5 below, with the note clarifying that FG 25-18a is applicable to UEs capable of handling PA phase discontinuity.

| 6-23 | Incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band EN-DC, intra-band CA, and FDM based ULSUP | Incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band EN-DC, intra-band CA, and FDM based ULSUP |  | pa-PhaseDiscontinuityImpacts |
| --- | --- | --- | --- | --- |

Table 5: Proposed FG for simultaneous PUCCH/PUSCH for intra-band CA.

| Features | Index | Feature group | Components | Prerequisite feature groups | Note |
| --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-18a | Parallel PUCCH and PUSCH transmission across CCs in intra-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA. | 6-5 | Applicable to UEs not indicating FG 6-23 |

For FG 25-14 and 25-15 in R1-2108679, the description should be aligned to avoid any confusion that there are any differences other than the LP vs HP of DG-PUSCH vs CG-PUSCH. Thus we recommend the editorial changes in Table 4.

For FG 25-18, FG 6-5 “Basic DL NR-NR CA operation” should be added as a prerequisite FG. Also “[at least]” can be deleted, since FG 25-18 describes inter-band CA only, i.e., intra-band CA is not a concern here.

Table 6: Proposed FG for Intra-UE Multiplexing/Prioritization.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-14 | PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH | Support PHY prioritization of overlapping for the case where low-priority DG-PUSCH and collides with high-priority CG-PUSCH on a BWP of a serving cell | 12-1 |
| 25. NR_IIOT_URLLC_enh | 25-15 | PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH | Support PHY prioritization of overlapping high-priority dynamic grant DG-PUSCH and low-priority configured grant CG-PUSCH on a BWP of a serving cell | 12-1 |
| 25. NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. | 6-5 |

FG 25-18a is added for simultaneous PUCCH/PUSCH in intra-band CA.

Adopt the proposed changes in red in Table 6 for the proposed FG 25-14, 25-15, 25-18 in R1-2108679.

## Propagation Delay Compensation

For the topic of propagation delay compensation, progress has been made in RAN1 discussion that enhancements are needed for UEs to provide time synchronization for IIoT use cases. Thus an FG needs to be introduced for such UEs. An example is shown in Table 7 below. The details of the FG can be updated once more design details are developed in RAN1 discussion.

A feature group is introduced for supporting propagation delay compensation in Rel-17.

Table 7: Exemplary FG for propagation delay compensation.

| Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Propagation delay compensation | Support propagation delay compensation for time synchronization of the Uu interface | N/A | Yes | Per UE | No | No | N/A | Optional with capability signalling |

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt the proposed changes in red in Table 1 for the proposed FG 25-2, 25-3 and 25-3a in R1-2108679.

Proposal 2	Adopt the proposed changes in red in Table 3 for the proposed FG 25-2, 25-3 and 25-3a in R1-2108679

Proposal 3	Adopt the proposed changes in red in Table 4 for the proposed FG 25-11 in R1-2108679.

Proposal 4	FG 25-18a is added for simultaneous PUCCH/PUSCH in intra-band CA.

Proposal 5	Adopt the proposed changes in red in Table 6 for the proposed FG 25-14, 25-15, 25-18 in R1-2108679.

Proposal 6	A feature group is introduced for supporting propagation delay compensation in Rel-17.

# References

R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#106e, August 2021.