3GPP TSG RAN WG1 #106bis-e		R1-2109783

e-Meeting, October 11th – 19th, 2021

Agenda Item: 	8.3.1.2

Source: 	Sony

Title: 	Remaining issues in 4 bits sub-band CQI reporting

Document for: 	Discussion / decision

# Introduction

In RAN1#106e, we agreed and concluded the following:

Agreement

For subband CQI reporting with more than 2 bits per subband

- Support 4-bits CQI only

Agreement

For subband CQI reporting in Rel-17, RRC can configure use of legacy 2-bits D-CQI or 4-bits CQI for each CSI report configuration.

- This feature is subject to UE capability

- FFS: Whether wideband CQI report can be omitted

Conclusion

There is no consensus in RAN1 on the support of delta-MCS in Rel-17.

This contribution discusses some remaining issues on 4 bits sub-band CQI reporting.

# Discussions

Two issues were raised on the 4-bits sub-band CQI reporting feature [1]:

- Report of Wideband CQI together with 4-bits sub-band CQI

- Enhanced mapping of “Out-of-Range” CQI

In the Rel-15 2-bits sub-band CQI reports, the sub-band CQI is reported as a differential value relative to a wideband CQI.  It is argued in [2] that since the 4-bits sub-band CQI reports the absolute value there is no need for a wideband CQI to be used as a reference and that this would reduce the CSI report overhead.  The UE can be configured to report a subset of sub-bands and therefore for such configuration, the wideband CQI would not be available at the gNB as it cannot be derived from a subset of sub-bands.  For the case where all the subbands are reported, then an additional 4 bits for the wideband CQI would not significantly reduce any overhead.  Hence, we prefer that the wideband CQI is reported with the 4-bits sub-band CQI reporting.

Proposal 1: Wideband CQI is reported together with the 4-bits sub-band CQI reporting.

In Rel-15, the UE reports CQI=0 indicating that the SNR is “Out-of-Range”, i.e. very low.  It is suggested in [3], that for 4-bits sub-band CQI, CQI=0 is mapped to an SNR instead of “Out-of-Range” as CQI=0 can be occur quite often.  We do not see a strong motivation to enhance the mapping for “Out-of-Range” CQI as a CQI less than 1 may not offer reliable DL transmission.

Proposal 2: Do not support enhanced mapping of “Out-of-Range” CQI value.

# Conclusion

In this contribution, we discuss some remaining issues on 4-bits sub-band CQI reporting and propose the following:

Proposal 1: Wideband CQI is reported together with the 4-bits sub-band CQI reporting.

Proposal 2: Do not support enhanced mapping of “Out-of-Range” CQI value.

# References

[1] R1-2108450, “Feature lead summary #4 on CSI feedback enhancements for enhanced URLLC/IIoT,” Moderator (InterDigital, Inc.), RAN1#106e

[2] R1-2106491, “CSI feedback enhancements,” Huawei, HiSilicon, RAN1#106e

[3] R1-2107584, “On enhanced SB CQI reporting granularity and delta-MCS reporting,” Intel, RAN1#106e