3GPP TSG RAN WG1 #106bis-e			      R1-2110181

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.3.3

Source: 	Qualcomm Incorporated

Title: 	Intra-UE multiplexing and prioritization for IOT and URLLC

Document for:	Discussion/Decision

# Introduction

In RAN plenary #86, the work item on Enhanced Industrial Internet of Things (IIOT) and URLLC Support was agreed [1]. One of the main objectives of the work item is to study

“Intra-UE multiplexing and prioritization of traffic with different priority based on work done in Rel.16 [RAN1]:

- Specify multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH. “

In this section, the enhancement for intra-UE multiplexing and prioritization is discussed.

# Simultaneous x-CC PUCCH/PUSCH transmissions

With this new feature of simultaneous PUCCH/PUSCH, PHR needs to support a new scenario, which is PUCCH transmission on PCC in parallel with PUSCH transmission on SCC. UE needs to report a “new” type of PHR for PUCCH on PCC, and a type 1 PHR for PUSCH on SCC. For the “new” type of PHR for PUCCH on PCC, we can either reuse LTE type 2 PHR with a minor modification/clarification or define a new type, i.e., type 4, PHR for this purpose. If reusing LTE type 2 PHR, one should notice that LTE type 2 PHR is for simultaneous PUCCH/PUSCH transmission on the same CC, which does not support in NR yet. One way to walk around this is utilize the virtual PHR for PUSCH in type 2, assuming a virtual/reference transmission of PUSCH on PCC when UE report type 2 PHR for PCC. If RAN1 does not want to define the virtual PHR for this purpose, a new type 4 PHR can be defined for PUCCH transmission on PCC.

Proposal 1: Support the PHR for simultaneous PUCCH/PUSCH for inter-band CA with either of the following two options.

- Option 1: reuse LTE type 2 PHR for PUCCH transmission on PCC with a virtual/reference PUSCH

- Option 2: define a type 4 PHR for PUCCH transmission on a component carrier

In the past RAN1 meetings, there was a debate whether simultaneous PUCCH/PUSCH transmission applies to only PUCCH/PUSCH with different priorities or applies to PUCCH/PUSCH transmission with same or different priorities. In our view, this debate is settled with the following agreement made in RAN1 #102e. First of all, the agreement is a generic agreement covers same and different priority, as the agreement did to limit the scope to only different priority.

Agreements:

Support simultaneous PUCCH/PUSCH transmissions on different cells at least for inter-band CA.

- FFS how to trigger this function.

- FFS for intra-band CA.

Secondly, from specification and UE implementation point of view, there is no much difference to support simultaneous transmission of PUCCH/PUSCH with same or different priorities.

Thirdly, the benefit of this feature will be heavily diminished if RAN1 limit the utilization of this feature to only PUCCH/PUSCH transmission with different priorities. One should notice that, forcing PUCCH always multiplexing on an overlapping PUSCH on another CC may create performance degradation for PUCCH. As shown in the following Fig 1, a long PUCCH is scheduled on Pcell for a cell edge UE, i.e., gNB schedule the PUCCH very long duration and or Pcell to guarantee the coverage of the PUCCH transmission. However, on Scell, which could be a FR2 cell, a PUSCH is scheduled due to UL small packet data transmission and this PUSCH is with a short duration because of the TB is small. It is assumed that the PUCCH and PUSCH are with the same priority (both are LP or both are HP). If simultaneous transmission does not apply to PUCCH/PUSCH with same priority, due to overlapping, the “long” PUCCH has to be multiplexed on the “short” PUSCH, which will significantly degrade the coverage of the PUCCH. One could argue that gNB scheduler can avoid overlapping of the PUCCH/PUSCH. However, if both PUCCH/PUSCH are with HP, gNB may not able to avoid the overlapping by delay one of two channels. If both channels are with LP, in theory, gNB could intentionally delay one channel in scheduling. However, it imposes unnecessary scheduling restriction and complexity to gNB scheduler implementation.

Fig 1: Example of a “long” PUCCH overlap with a “short” PUSCH on different CC

With the above analysis, we make the following proposal.

Proposal 2: Clarify the agreement made in RAN1 102e “Support simultaneous PUCCH/PUSCH transmissions on different cells at least for inter-band CA” applies to PUCCH/PUSCH with same or different priorities.

# Interactions between simultaneous x-CC PUCCH and PUSCH transmission and intra-UE multiplexing

In previous sections, we have discussed our view on the design of intra-UE multiplexing (between uplink transmission of different priorities) and on simultaneous x-CC PUCCH and PUSCH transmission. However, if the UE is capable of both features, the interaction between these two features needs to be addressed. In this section, we discuss a general framework to solve uplink collisions for UEs that are capable of both intra-UE multiplexing and simultaneous PUCCH and PUSCH transmission.

One the one hand, we notice that the following working assumption was agreed in the previous meeting.

Working Assumption

For handling overlapping PUCCHs/PUSCHs with different priorities in R17

- Step 1: Resolve overlapping PUCCHs and/or PUSCHs with the same priority

- Step 2: Resolve overlapping PUCCHs and/or PUSCHs with different priorities

Note: Avoid recursive pseudo-code to implement this procedure

Note: It is expected that Rel-15 intra-UE UCI multiplexing timeline will be applicable

Therefore, the design shall be compatible with the two-step principle in the working assumption.

On the other hand, within each step of step 1 and step 2 in the working assumption, the UE may need to determine whether to go with simultaneous PUCCH/PUSCH transmission or to go with intra-UE multiplexing.  One naïve approach will be to let the UE perform such determinations within each step of Step 1 and Step 2. However, this may be complicated, since if the UE decides to go with simultaneous Tx for PUCCH and PUSCH of the same priority in step 1, it may still need to solve overlapping between the PUCCH, PUSCH and other uplink transmissions of different priorities. And simultaneous transmission may need to be checked again.

In order to simplify the procedure, one may let the UE select simultaneous PUCCH/PUSCH transmission for channels of a given priority, only if there’re no other transmissions of different priorities that collide with the PUCCH/PUSCH of the given priority resulting from Step 1 in the working assumption. If there’re both high priority and low priority channels after resolving collisions between channels of the same priority, then the UE shall always multiplex together channels with the same priority in Step 1.

The discussion above suggests the following flow diagram.

Fig 2: Flow diagram of interaction between simultaneous PUCCH/PUSCH transmission and UCI multiplexing

We describe the above procedure in the following proposal.

Proposal 3: when a UE is configured with simultaneous PUCCH/PUSCH transmission and Rel-17 intra-UE multiplexing, take the following steps to resolve collision between overlapping of two or more uplink channels:

- Step 1: Resolve overlapping PUCCH(s) and/or PUSCH(s) with the same priority

  - Step 1.1: Overlapping PUCCHs of same priority are first resolved to obtain one final PUCCH for a given priority

  - Step 1.2: Resolve overlapping between PUCCH and PUSCH(s) of the same priority:

    - If all overlapping channels are of the same priority

      - If the remaining PUCCH and PUSCH can be transmitted simultaneously, then

        - Step 1.2.1: transmit the PUCCH and PUSCH(s) simultaneously

      - Otherwise

        - Step 1.2.2: multiplex the UCI on a PUSCH of the same priority

    - Otherwise (i.e., if the remaining overlapping channels are with different priorities),

      - Step 1.2.2: multiplex the UCI on a PUSCH of the same priority

- Step 2: Resolve overlapping PUCCH(s) and/or PUSCH(s) with different priorities

  - Step 2.1: resolving overlapping between HP PUCCH and LP PUCCH

  - Step 2.2: resolving overlapping between PUCCH and PUSCH(s) of different priorities

    - If the remaining PUCCH and PUSCH(s) can be transmitted simultaneously

      - Step 2.2.1: transmit PUCCH and PUSCH(s) simultaneously

    - Otherwise

      - Step 2.2.2: multiplex the UCI on a PUSCH with different priority

# UCI multiplexing on PUCCH

We discuss the specification impact of HP and LP HARQ-ACK multiplexing with more than two bits in Section 4.1, respectively. In section 4.2, HARQ-ACK and SR multiplexing with different priorities are discussed.

## Spec impact to support HP/LP HARQ-ACK separate encoding

### Drop UCI to avoid exceeding 2 UCI encoders

In RAN1 104bis-e, the following agreement was made.

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, support separate coding for the two HARQ-ACKs.

- FFS for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s).

- (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N.

  - FFS Strive to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1.

  - FFS Strive to let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2.

The spirit of the above agreement is to make sure separate encoding of HP and LP HARQ-ACK does not require UE to increase number of encoders used in Rel-15 UCI multiplying on PUCCH. Because CSI on PUCCH is only LP CSI (there is no HP CSI on PUCCH), it is then very naturally to drop CSI if CSI would multiplex with HP HARQ-ACK.

In RAN1 #106e, part of the WA above was agreed at least for PUCCH format 3 and PUCCH format 4, as shown in the following agreement.

Agreement

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17,

- HP A/N reuses rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1.

- LP A/N reuses rate matching equation, and RE mapping rules in Rel-15 for CSI-2.

Above applies at least for PUCCH format 3 and 4.

There’re two main remaining issues to be addressed.

The first issue how to treat CSI in case they collide with the LP and HP HARQ-ACK. We propose to confirm the working assumption in RAN1 #104bis-e.

Proposal 4: Confirm the working assumption made in RAN1 #104bis-e

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17,

- Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N.

The second issue is how to how to encode the LP and HP HARQ-ACK on a PUCCH, when the LP or HP HARQ-ACK has less than or equal to 2 bits.

There can be two design Option 1:

- Option 1: follow the encoding approach of UCI on PUSCH in Rel-15, i.e., use repetition encoding (for 1 bit) or simplex encoding (for 2 bits)

- Option 2: follow the same encoding approach as CSI part 2 on PUCCH as in Rel-15, i.e., zero-pad the HP or LP HARQ-ACK into 3 bits, and use Reed-Muller code to encode the HARQ-ACK.

For the 1 bit case, the two options yield the same performance. Indeed, the design in Option 2 reduces to a repetition code when there is only 1 bit payload.

However, for the 2 bit case, Option 1 provides a better performance than Option 2. To see how much performance difference between the two options for the 2 bit case, we present in the figure below the minimum distance of the coding schemes in Option 1 and Option 2 for varying number of coded bits.

Fig 3: Minimum distance comparison between simplex code and zero-padded Reed Muller code for encoding HARQ-ACK with 2 bits information

As we can see from Fig 3, the simplex code (Option 1) has much better minimum distance compared to the zero-padded Reed Muller code (Option 2). Since the encoding complexity between the two Options are exactly the same, we propose the following.

Proposal 5: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into PUCCH format 3 or format 4, when the total number of LP and HP HARQ-ACK bits is more than 2, and when the number of HP or LP HARQ-ACK has less than or equal to 2 bits

- The HP or LP  HARQ-ACK uses repetition encoding if the payload size is 1 bit, and uses the simplex encoding if the payload size is 2 bits

### PUCCH coding rate signalling

In RAN1 106e, the following agreement was made for PUCCH coding rate signalling.

Agreement

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, an additional maxCodeRate for LP HARQ-ACK can be configured in the second PUCCH-Config per PUCCH format.

One remaining issue related to the coding rate signaling is that,  the HARQ-ACK payload size of a given priority can have wide range from 1 bit to several tens of bits. In this case, it may not be sufficient to configure only one coding rate for a given priority, as different coding rates are needed to achieve the same reliability for different UCI payload size. To this end, it may be beneficial for the gNB to configure different coding rates for a given HARQ-ACK priority based on the HARQ-ACK payload size. For example, up to three coding rates can be configured for HARQ-ACK of a given priority depending on the range of the HARQ-ACK payload size (which is the same approach as the configuration of beta offsets for UCI piggyback on PUSCH):

- 1-2 bits

- 3-11 bits

- More than 11 bits

Note that, in Rel-15 and Rel-16, the gNB doesn’t need to have separate signaling for coding rate per UCI payload range, because, by configuration, a PUCCH format may be configured only support one range of UCI payload sizes. However, in Rel-17 with separate encoding between HP and LP UCIs on a PUCCH, such a design will not be sufficient.

Based on the discussion above, we make the following proposal.

Proposal 6: In NR Rel-17, for multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into PUCCH, when the total number of low priority (LP) and high priority (HP) HARQ-ACK bits is more than 2

- For a given priority, support gNB to configure multiple coding rates for HARQ-ACK based on the payload size.

### PUCCH resource determination

For transmission of LP and HP HARQ-ACK on a PUCCH format 3, the UE may need to select a number of RBs based on the payload sizes and coding rates, as agreed in the following agreement in RAN #106e.

Agreement

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17,

- PUCCH resource set determination is based on: UCI payload size = the number of HP UCI bits + the number of LP UCI bits.

- FFS PRB number determination for HP A/N and LP A/N, e.g. based on their coding rates.

- FFS the impact to the number of LP UCI bits due to missed DCI and potential solutions

- Note: the number of LP UCI bits in the above agreement does may not necessarily mean the actual number of LP UCI bits until the second FFS is resolved

Given PUCCH coding rate rHP and rLP for HP HARQ-ACK and LP HARQ-ACK respectively, the number of RBs used to transmit HP+LP HARQ-ACK can be calculated based on the following equation,

L=KHP/rHP+KLP/rLP/2/S/D

where KHP and KLP is the payload size for HP and LP HARQ-ACK respectively. S is number of OFDM symbols in the PUCCH resource. D is the number of available tones for UCI in one RB.

One should notice that the total number of RBs L depends on the LP HARQ-ACK payload size KLP. If LP HARQ-ACK is with type 2 codebook, in case of missing LP DCI occurs, the size misalignment of LP HARQ-ACK could create RB misalignment at gNB, which will even impact HP HARQ-ACK decoding. As shown in Fig 4, due to LP size misalignment, several junk RBs will be included in HP A/N decoding. More severely, a few valid REs could be missed in HP A/N decoding. Because of the HP A/N has 10^-5 reliability requirement, the 10^-2 LP DCI miss detection rate is a serious issue which could lead to HP A/N fail to meet reliability requirement. Therefore, a solution is needed to solve this issue.

Fig 4: LP A/N size misalignment impact HP A/N decoding

To solve this issue, we could define (a few) reference LP HARQ-ACK size. The actual LP HARQ-ACK size is rounded up to the nearest reference size. The reference size is then used to calculate number of RBs. This could mitigate the occurrence of size misalignment for LP HARQ-ACK between UE and gNB.

The reason that using quantized reference LP HARQ-ACK size can mitigate occurrence of size misalignment for LP HARQ-ACK is as illustrated in Fig 5. Given a few quantization points (marked in green in Fig 5) for LP A/N size (where the quantization granularity can be configured by gNB or hardcoded to 4, following DAI granularity), the LP A/N is rounded up to nearest quantization point. Suppose gNB schedules UE to transmit M bits LP A/N. While due to missing LP DCI, UE only have N bits LP A/N to transmit, where N<M. As long as N and M do not fall into two different quantization sectors, by rounding up N and M to the same value N’, the LP HARQ-ACK size is aligned between gNB and UE to determine the number of RBs.

Fig 5: Round up LP HARQ-ACK size to avoid size misalignment between UE and gNB

The same issue of missing LP DCI also applies to the problem of PUCCH resource set selection. Namely, due to missing LP DCIs, UE and gNB may end up selecting different PUCCH resource sets, therefore causing communication errors. The same solution as discussed above for RB selection could be applied to solve this problem.

Based on the discussion above, we propose the following.

Proposal 7: For HP UCI and LP HARQ-ACK (in type 2 codebook) multiplexing on a PUCCH, round up LP HARQ-ACK size to a nearest reference size, in the calculation of total number of RBs for HP and LP UCI and in the PUCCH resource set determination.

### RE mapping for PUCCH format 2

For PUCCH format 2, in Rel-15, because CSI part 2 can not be transmitted on PUCCH format 2, only a single encoder is used to jointly encode HARQ-ACK and CSI part 1. The encoded bits are then mapped to REs in PUCCH format 2.

Within Rel-17, due to separate encoding of HP and LP HARQ-ACK, the two separately encoded bit streams need to be mapped to REs separately. To guarantee the desired coding rate of HP HARQ-ACK, it should be mapped first. The REs for HP HARQ-ACK should be distributed in frequency domain to span cross all available RBs in the PUCCH resource, in order to explore the frequency diversity to guarantee the high reliability of HP HARQ-ACK.

Fig 6: RE mapping for HP UCI and LP UCI on PUCCH format 2

Proposal 8: For HP UCI and LP UCI multiplexing on PUCCH format 2, support mapping encoded HP UCI bits first with a distributed RE mapping in frequency domain, followed by mapping encoded LP UCI bits onto remaining REs.

The remaining open issue is how to determine the distance d to map the HP UCI bits in distributed fashion. Following the principle to distribute the HP UCI as much as possible in frequency (and time) domain to explore the frequency and time diversity, the distance d can be calculated based on the following equation

d=S∙8∙L∙2∙rHPKHP

where

- KHP is the payload size for HP UCI, rHP is the coding rate for HP UCI.

- S is number of OFDM symbols in the PUCCH resource.

- L is the total number of RBs determined for multiplexed HP UCI and LP UCI transmission

- 2 in the above equation corresponding to 2 bits per QPSK modulated symbol. 8 in the above equation corresponding to 8 UCI tones per RB for PUCCH format 2.

Based on the above analysis, we have the following proposal.

Proposal 9: the distance d for HP UCI distributed RE mapping is determined as d=S∙8∙L∙2∙rHPKHP, where

- KHP is the payload size for HP UCI, rHP is the coding rate for HP UCI.

- S is number of OFDM symbols in the PUCCH resource.

- L is the total number of RBs determined for multiplexed HP UCI and LP UCI transmission

### Power control

Another issue to solve is how to determine the power for PUCCH with multiplexed HP and LP HARQ-ACK. In NR Rel-15, the power for PUCCH format 2/3/4 is determined based on the following equation

PPUCCH=min⁡{PC,max,P0+PL+10log102μMRB+ΔTF+ΔF+g}                   (1)

where P0 denotes the open loop power control parameter, and ΔTF is a PUCCH transmission power adjustment component, which depends on the bit per RE (i.e., the BPRE) determined via the coding rate.

Note that, for PUCCH power control, the parameter that controls the reliability of the transmission is the P0 parameter. A smaller P0 value generally implies a larger BLER for the transmission. Reducing/increasing the BPRE of the UCI doesn’t directly affect the reliability of the UCI on PUCCH, since the transmit power is scaled proportional to the BPRE via ΔTF.

When the PUCCH contains both HP and LP HARQ-ACK, one could consider the following power control scheme:

PPUCCH=max{PLP, PHP}

where PLP and PHP is separately computed from (1) using the corresponding coding rates and P0's. Note also that, when computing PLP and PHP, the total number of RBs for both LP and HP UCI are used.

The motivation to calculate the power of HP and LP UCI separately is that, the required (per-RE) power for LP and HP UCI depends on the corresponding coding rate, payload size and reliability (which is controlled by P0). And the required per-RE power for HP UCI may not always be larger than the required power for LP UCI to achieve the corresponding reliability. Furthermore, it may not be easy to control the reliability only based on the coding rate. Therefore, it is more convenient to compute the required power of HP and LP UCI separately, and take the maximum of the two to guarantee that both HP and LP can get sufficient power to deliver the desired reliability.

In order to see the issue raised above, let’s look at the BLER curve of a 11 bit UCI, with a rate of R=11/32 shown in the figure below.

Fig 7: BLER for 11 bit HARQ-ACK with

For the sake of example, let’s assume the BLER requirement for LP and HP HARQ-ACK are 10^-2 and 10^-4, respectively.  And assuming that both LP and HP UCI is 11 bits.  For a same reference coding rate 11/32, the difference in P0 required to achieve the respective BLERs are 2 dB, i.e., P0LP=-2dB and P0HP=0dB. On the other hand, when the coding rate for the HP UCI is half of that of the LP UCI, the ΔTF term calculated based on the LP and HP UCI coding rate differ by 3 dB. Therefore, we have

PLP-PHP=-2+10log106*1132-10log106*1164=1 dB

In this case, we see that the per-RE power required to convey the LP HARQ-ACK is 1 dB larger than the per-RE power required to transmit the HP HARQ-ACK, although LP HARQ-ACK has a higher required BLER target than the HP HARQ-ACK. The final PUCCH (per-RE) power shall be taken as the PLP in order to guarantee the reliability of the LP HARQ-ACK and the HP HARQ-ACK.

Based on the discussion above, we propose the following.

Proposal 10: For HP UCI and LP UCI multiplexing on PUCCH format 2/3/4, support the following

- Two open-loop power control P0 values are configured for multiplexing LP and HP UCI

- Two separate powers are computed for LP UCI and HP UCI (following TS 38.213 Section 7.2.1) based on the corresponding P0 and BPRE for LP and HP UCI respectively, and based on the total number of RBs used to HP and LP UCI

- The final PUCCH power is determined based on the max power of the HP and LP powers

## HARQ-ACK and SR multiplexing with different priorities

For the case in which 1 or 2 bits HARQ-ACK collide with a 1-bit SR, there’re 8 possible cases depending on the priority levels and PUCCH formats of the HARQ-ACK and SR. Before discussing the detailed solutions for resolving collisions in each of the 8 cases, we’d like to discuss the general principles for multiplexing HARQ-ACK and SR of different priorities. On the one hand, we should try not to drop the low priority transmissions if possible; on the other hand, we should protect the high priority transmissions from both reliability and latency perspective as much as possible. More specifically, the following enhancement from the Rel-15 design can be considered. When the HARQ-ACK and SR are multiplexed, they shall be multiplexed on the high priority channel since the power control associated with the high priority channel may lead to higher reliability. In some cases, it may not be feasible to always multiplex HARQ-ACK and SR on the high priority channel, e.g., in case of RB selection. However, in such cases, it may be desirable to use the power associated with the high priority channel to transmit the UCI payload.

We summarize principles discussed above in the following observation.

Observation 1: Multiplexing HARQ-ACK and SR with different priorities shall take into account the following design principles:

- Reuse the Rel-15 rule to multiplex the HARQ-ACK and SR when appropriate

- High priority channels should be better protected to guarantee its reliability and latency via i) putting the multiplexed payload on the high priority PUCCH resources if possible ii) use the power control parameters related to the high priority channel to transmit the multiplexed payload.

Next, we share our view on the collision resolutions rules for each of the overlapping cases below.

- Case 1: low priority (LP) HARQ-ACK on PF 0 collide with high priority (HP) SR on PF 0: in this case, we may reuse the Rel-15 solution to multiplex the HARQ-ACK and SR. However, different from Rel-15, in Rel-16 and beyond, the high priority SR and low priority HARQ-ACK may be scheduled with different power control parameters (including both open-loop and closed-loop power).  To ensure reliable delivery of the high priority transmission, one possible enhancement in Rel-17 is to use the SR PUCCH resource to transmit the multiplexed LP HARQ-ACK and the HP SR. In addition, since the power control for PUCCH format 0 is independent on the payload size of the UCI multiplexed on the SR, one may apply an additional power boost to the multiplexed UCI transmission.

- Case 2: low priority (LP) HARQ-ACK on PF 0 collide with high priority (HP) SR on PF 1: in Rel-15, a HARQ-ACK on PF0 that collides with SR on PF 1 will be multiplexed on the HARQ-ACK resource. However, this may affects the reliability of the SR. Therefore, we would like to enhance the design in Rel-17 by performing an RB selection. More specifically, if the SR is negative, then HARQ-ACK is transmitted on the HARQ-ACK resource. However, if the SR is positive, the HARQ-ACK is transmitted on the SR resource to indicate the positive SR. This way, we protect the reliability of SR whenever SR is positive.

- Case 3: low priority (LP) HARQ-ACK on PF 1 collide with high priority (HP) SR on PF 0: in NR Rel-15, if a HARQ-ACK on PF1 collides with an SR on PF0, UE will drop the SR and transmit HARQ-ACK. However, when SR is of higher priority than the HARQ-ACK, dropping SR may not be appropriate. In NR Rel-17, we may enhance the design by using RB selection. More specifically, if the SR is negative, then HARQ-ACK is transmitted on the HARQ-ACK resource. However, if the SR is positive, the HARQ-ACK is transmitted on the SR resource to indicate the positive SR. This way, we will not drop the SR or the HARQ-ACK, but we also guarantee that SR is transmitted with low latency whenever it is positive.

- Case 4: low priority (LP) HARQ-ACK on PF 1 collide with high priority (HP) SR on PF 1: Same rule as in Rel-15 (i.e., RB selection) can be applied.

- Case 5: HP HARQ-ACK on PF 0 with LP SR on PF 0: As explained earlier, in NR Rel-15, an HARQ-ACK on PF0 that collide with SR on PF0 may be multiplexed on the HARQ-ACK resource. The same rule may be applied in NR Rel-17 to handle colliding HARQ-ACK and SR of different priorities. In addition, to guarantee the reliability of the HP HARQ-ACK, an additional power boost may be applied to the multiplexed payload.

- Case 6: HP HARQ-ACK on PF 0 with LP SR on PF 1: Similar to the Case 5 above, we may reuse the Rel-15 rule to multiplex the HARQ-ACK and SR on the HARQ-ACK resource. In addition, to guarantee the reliability of the HP HARQ-ACK, an additional power boost may be applied to the multiplexed payload.

- Case 7: HP HARQ-ACK on PF 1 with LP SR on PF 0: In this case, we shall use the same rule as in NR Rel-15 and Rel-16 and drop SR.

- Case 8: HP HARQ-ACK on PF 1 with LP SR on PF 1: In this case, we may reuse the Rel-15 rule to indicate the value of SR using RB selection. Furthermore, since the SR and HARQ-ACK are of different priorities, which implies that the power determined on the SR resource may be different from the power derived from the HARQ-ACK resource. To guarantee the reliability of the HP HARQ-ACK, the UE may always use the power determined form the HARQ-ACK resource to transmit the HARQ-ACK (regardless of whether the HARQ-ACK is transmitted on the HARQ-ACK resource or the SR resource).

The design options above are summarized in the following proposal.

Proposal 11: In NR Rel-17, if a HARQ-ACK (with single priority) transmission on PUCCH format 0 or PUCCH format 1 collide with one SR, the UE performs the actions in Table 1 to resolve the collision.

- FFS: collision resolution for 1-bit HP HARQ-ACK and 1-bit LP HARQ-ACK overlapping with 1-bit HP or LP SR

Table 1. Collision resolution for overlapping HARQ-ACK and SR in NR Rel-17

|  | Ack: PF0, LP | Ack: PF1, LP | Ack: PF0, HP | Ack: PF1, HP |
| --- | --- | --- | --- | --- |
| SR: PF 0, LP | Same as Rel-15 (i.e., multiplex on HARQ-ACK resource). | Same as Rel-15 (i.e., drop SR) | Multiplex the HARQ-ACK and SR on the HARQ-ACK resource (as in Rel-15), with a power boost to the multiplexed transmission. | Same as Rel-15 (drop SR). |
| SR: PF1, LP | Same as rel-15 (i.e., multiplex on HARQ-ACK resource) | Same as Rel-15 (RB selection) | Multiplex the HARQ-ACK and SR on the HARQ-ACK resource (as in Rel-15), with a power boost to the multiplexed transmission. | RB selection (as in Rel-15) but with the enhancement that, if SR is positive, the power of the PUCCH transmission follows the power of the HARQ-ACK resource. |
| SR: PF0, HP | Use the SR resource to transmit multiplexed SR and HARQ-ACK, with a power boost to the multiplexed transmission. | Perform RB selection (i.e., if SR is negative, then transmit HARQ-ACK on the HARQ-ACK resource. Otherwise, transmit HARQ-ACK on the SR resource.) | Same as Rel-15 | Same as Rel-15 |
| SR: PF1, HP | Perform RB selection (i.e., if SR is negative, then transmit HARQ-ACK on the HARQ-ACK resource. If SR is positive, transmit HARQ-ACK on the SR resource.) | Same as Rel-15 (i.e., RB selection). | Same as Rel-15 | Same as Rel-15 |

Next, for the case of when a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, the more important open issue is that how to transmit the multiplexed payload, i.e., choosing which 4 or 8 CS indices out of the 12 available CS indices to transmit the 2 or 3 multiplexed bits?

In Rel-15, to transmit 1-bit SR with 1 or 2 bits HARQ-ACK, the following 4 or 8 CS indices are used, as shown in Fig 8. With same priority between HARQ-ACK and SR, the following design is reasonable, because the distance between different hypothesis is maximized, i.e., distance =3 for 1 bit SR and 1 bit A/N case, and distance =1 for 1 bit SR and 2 bits A/N case. However, with different priorities for SR and HARQ-ACK, the following design is problematic, because it can not provide different reliability between HP and LP bit. For example, in case the 1 bit SR is HP and 2 bits A/N is LP, the distance between the negative and positive SR is only 1 CS index, while the distance between different HARQ-ACK hypothesis is 3 CS indices, which will make the HP SR performance much worse than LP HARQ-ACK.

Fig 8: Rel-15 1 bit SR and up to 2 bits HARQ-ACK multiplexing in PUCCH format 0

To improve from Rel-15 design, for the case of 1 bit SR and up to 2 bits HARQ-ACK with different priorities in PUCCH format 0, we should seek for new CS indices mapping to protect high priority bit with larger distance and sacrifice the low priority bit with smaller distance. The new mapping rule should keep larger distance between hypotheses for HP payload, while keep smaller distance between hypothesis of LP payload. For example, for 1 bit SR and 1 bit HARQ-ACK with different priorities, the following CS mapping can be used, as shown in Fig 9. With the following mapping, in the case of 1 bit HP SR and 1 bit LP A/N, the distance between the negative SR and positive SR is 5 (while in Rel-15 mapping, the distance is 3), which will boost the high priority SR performance. Similarly, in the case of 1 bit LP SR and 1 bit HP A/N, the distance between the ACK and NACK is 5 (while in Rel-15 mapping, the distance is 3), which will boost the high priority HARQ-ACK performance. For the case of 1 bit HP SR and 2 bits LP HARQ-ACK, with the mapping as shown in Fig 10, the distance between positive SR and negative SR is 3 (while in Rel-15 mapping, the distance is only 1). For the case of 1 bit LP SR and 2 bits HP HARQ-ACK, the release 17 new mapping happens to be the same as Rel-15 mapping.

The main design principle of the new CS mapping in Rel-17 for 1 bit SR and up to 2 bits HARQ-ACK with different priorities can be summarized as following:

- Use a plurality of subsets of CS indices, which are separated with larger gap/distance among adjacent subsets, to transmit the high priority bit(s).

- Use different CS in a subset, which are separated with smaller gap/distance (e.g., set the distance equals to 1), to transmit the low priority bit(s).

The impact of Rel-17 new mapping to both UE and gNB implementation is very small. On UE side, for a multiplexed payload, the only difference is mapping the payload to a different CS index, before applying the CS to the sequence and transmit the sequence in PUCCH format 0. At the receiver side, with Rel-15 mapping, the gNB correlate the received signal with base sequence S with CS set of {0,1,3,4,6,7,9,10} for 3 bits payload for example, which are 8 sequence correlations. Now, with Rel-17 new mapping, gNB still correlate the received signal with base sequence S with 8 CS indices, which are still 8 sequence correlations. The only difference is that the CS set now is {0,1,2,3,6,7,8,9}, just to list as an example. The change to gNB implementation to support this looks minor.

Fig 9: Rel-17 proposal of 1 bit SR and 1 bit HARQ-ACK multiplexing in PUCCH format 0

Fig 10: Rel-17 proposal of 1 bit SR and 2 bits HARQ-ACK multiplexing in PUCCH format 0

Finally, the performance of Rel-17 mapping and Rel-15 mapping is compared. The simulated case is 1 bit HP SR multiplexing with 2 bits LP HARQ-ACK in PUCCH format 0.  As shown in Fig 11, with Rel-17 new CS mapping, 3dB gain can be observed over Rel-15 baseline mapping, with zero frequency error and timing offset/error of 3% of an OFDM symbol.

Fig 11: Performance comparison between Rel-15 and Rel-17 for the case of 1 bit HP SR and 2 bits LP HARQ-ACK multiplexing in PUCCH format 0

With the above study, we have the following proposal.

Proposal 12: In NR Rel-17, for the case of multiplexing 1 bit SR and up to 2 bits HARQ-ACK with different priorities in a PUCCH format 0, adopt the multiplexed payload to CS indices mapping as shown in Fig 9 and Fig 10.

Next, we consider the scenario in which the HARQ-ACK are transmitted using PUCCH format 2, 3, or 4. In this case, if the HARQ-ACK transmission collide with K SRs, including K1 HP SRs and K2 LP SRs, the UE may multiplex the HARQ-ACK with the K=K1+K2 SR using the Rel-15 rule. Furthermore, HP SR should be prioritized when both HP SR and LP SR are positive (similar to LRR and SR prioritization rule in NR Rel-16). That is, if any of the HP SR is positive, then the log2(1+K) bit used to convey the HP and LP SR shall indicate the positive HP SR among the HP and LP SRs (regardless of whether LP SR is positive or not). And a positive LP SR can only be indicated when all other HP SRs colliding with it are negative.

Proposal 13: In NR Rel-17, if a HARQ-ACK transmission on PUCCH format 2/3/4 collide with K SR transmissions including K1 HP SRs and K2 LP SRs, the UE append log2(1+K) bits to the HARQ-ACK payload.  Furthermore, if any of the K1 HP SR is positive, the log2(1+K) bits shall indicate a positive HP SR.

# UCI multiplexing on PUSCH

## Spec impact to support HP/LP HARQ-ACK separate encoding

### UCI dropping to avoid exceeding 3 UCI encoders

In RAN1 104bis-e, the following agreement was made.

Agreement:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, support separate coding for the two HARQ-ACKs.

- It is understood that it is intended that the number of encoding chains for all UCI multiplexing combinations in Rel-17 should not exceed that in Rel-15/16.

The spirit of above agreement is to support separate encoding of HP and LP HARQ-ACK without requiring more UCI encoders than Rel-15, which utilizes 3 UCI encoders for HARQ-ACK, CSI part 1, and CSI part 2.

Following this spirit, as illustrated by Fig 12 and Fig 13, when HP A/N, LP A/N, and LP CSI are multiplexed on PUSCH, the LP CSI part 2 should be dropped to be confined with 3 UCI encoders. when HP A/N, LP A/N, and HP CSI multiplexing on PUSCH, the LP A/N should be dropped to be confined with 3 UCI encoders.

Fig 12: HP A/N + LP A/N + LP CSI part 1/2 multiplexing on PUSCH

Fig 13: HP A/N + LP A/N + HP CSI part 1/2 multiplexing on PUSCH

Proposal 14: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if CSI would multiplex on the same PUSCH,

- Drop CSI part 2, if CSI is a low priority CSI.

  - HP A/N reuse encoder and rate matching equation for Rel-15 A/N

  - LP A/N reuse encoder and rate matching equation for Rel-15 CSI part 1

  - LP CSI part 1 reuse encoder and rate matching equation for Rel-15 CSI part 2

- Drop LP HARQ-ACK, if CSI is a high priority CSI.

  - HP A/N reuse encoder and rate matching equation for Rel-15 A/N

  - HP CSI part 1 reuse encoder and rate matching equation for Rel-15 CSI part 1

  - HP CSI part 2 reuse encoder and rate matching equation for Rel-15 CSI part 2

- FFS: RE mapping rules.

### Alpha factors indication

In NR Rel-15, when a HARQ-ACK is multiplexed on the PUSCH, the gNB may indicate a beta-offset value to the UE, which can be used by the UE to determine the number of resources on the PUSCH that are allocated to UCI. In particular, the beta offset value may be either dynamically indicated via 2 bits in the UL DCI or semi-statically via RRC.

In NR Rel-17, HARQ-ACK of different priorities can be multiplexed on a PUSCH. In this case, using a same beta offset value or a same set of beta offset values may not be sufficient. Instead, it may be beneficial to allow the gNB to indicate different beta offset values (or different sets of beta offset values) to the UE based on the priorities of the HARQ-ACK and the PUSCH. Indeed, the following agreement was made in RAN1 #106e to address the above issue.

Agreement

In NR Rel-17, [at least] 2 new set of beta offset values can be configured to the UE to indicate separate beta_offset values for the following cases:

- Multiplexing LP HARQ-ACK on HP PUSCH

- Multiplexing HP HARQ-ACK on LP PUSCH

Another open issue related to the parameters for multiplexing is the configuration of the scaling factor “alpha”. In NR Rel-15 and Rel-16, the alpha factor limits the total number of resource elements assigned to UCI on PUSCH, relative to the total number of resources on the PUSCH. The question is whether it is sufficient to configure one set of alpha values regardless of the priority of the UCI and PUSCH, and rely solely on the beta factors to control the reliabilities of the UCI and PUSCH.

In our view, it is be beneficial to configure separate alpha values for different priority combinations. For example, consider a case where the PUSCH is a high priority. Then, naturally it is desirable to allocate less #REs to the UCI when the UCI is of low priority compared to the case when the UCI is of high priority.  As we shall see shortly, to achieve such a goal, it is not sufficient to only control the beta factors.

To see why this is the case, let’s consider the following example. Assuming that the PUSCH has N=100 REs, and the PUSCH has a spectral efficiency of 1 bit/RE.  And the goal is to allocate at most 10% of the REs to LP UCI, and at most 20% of REs to the HP UCI. This goal can be achieved as long as  K⋅β/R≤10 for the LP UCI, where K denote the LP UCI payload size. However, when K is large, a very small β is needed to meet the constraint, and such beta could result in a channel coding rate of the UCI that is greater than 1. In this case, no information can be transmitted correctly about the LP UCI. One could argue that when the resulting coding rate >1, compression/dropping of LP UCI should be applied. However, UE can not determine dropping or compress how many bits, because that should be controlled by α. Apparently, different α is needed with different priority.

On the other hand, when the alpha can be separately configured based on the priority of the UCI and the PUSCH, then gNB can configure α=0.1 for LP UCI piggyback on HP PUSCH, and α=0.2 for HP UCI piggyback on HP PUSCH. The number of REs allocated to the LP UCI is equal to min{KβR, αN}. If the payload size is large, the UE can partially drop or compress the LP UCI such that KβR≤αN.  In other words, by configuring both α and β, β is used to determine the UCI coding rate while α is used to determine the compression/dropping of LP HARQ-ACK. But only configuring β based on priority is not sufficient to achieve such purpose.

Based on the discussion above, we make the following proposal.

Proposal 15: In NR Rel-17, up to four sets of scaling factors alpha can be configured to the UE to indicate separate alpha values for the following cases:

- Multiplexing LP HARQ-ACK/UCI on LP PUSCH

- Multiplexing LP HARQ-ACK/UCI on HP PUSCH

- Multiplexing HP HARQ-ACK/UCI on LP PUSCH

- Multiplexing HP HARQ-ACK/UCI on HP PUSCH

### RE mapping

In Rel-15, when HARQ-ACK and CSI were multiplexed on PUSCH, the following RE mapping rules are specified. The motivation to introduce A/N puncture CSI part 2 and PUSCH is to avoid CSI part 2 and PUSCH decoding failure due to A/N size misalignment (because of missing DL DCI), in case the PUSCH is scheduled by DCI format 0_0, which does not have uplink tDAI to indicate A/N payload size. The motivation to introduce tone reservation is to avoid <=2 bits A/N puncture CSI part 1.

- If A/N <=2 bits, the following RE mapping steps apply.

  - Reserve REs for A/N assuming A/N=2 bits

  - CSI-1 mapping to REs (rate match around reserved REs)

  - CSI-2 mapping to REs (ignore reserved REs, rate match around CSI-1 REs)

  - PUSCH mapping to REs (ignore reserved REs, rate match around CS1-1 and CSI-2 REs)

  - A/N mapping to reserved tones, puncturing CSI-2 and PUSCH.

- If A/N>2 bits, the following RE mapping steps apply.

  - A/N mapping to REs first

  - CSI-1 mapping to REs, rate match around A/N REs

  - CSI-2 mapping to REs, rate match around A/N, CSI-1 REs

  - PUSCH mapping to REs, rate match around A/N, CSI-1, and CSI-2 REs.

For HP A/N, assuming it is scheduled by HP DL DCI whose reliability can be guarantee, there is no missing HP DL DCI issue and there is no issue due to misalignment of HP A/N payload size, we could let HP A/N always been rate matching around by other UCIs and UL-SCH, without introduce RE reservation and puncturing other UCIs and PUSCH to complicate the specification and UE/gNB implementation.

For LP A/N, there is missing LP DCI issue and there is issue of misalignment of LP A/P payload size, to solve this issue, we could introduce RE reservation and puncturing for <=2 bits LP A/N payload size, and let other UCIs/PUSCH to rate match around LP A/N of >2 bits LP A/N payload size. However, it is not compliant with the spirit to let LP A/N reuse Rel-15 CSI part 1 encoder/rate match/RE mapping rule. To following Rel-15 RE mapping for CSI part 1, which is rate matching around other UCI and PUSCH, we can simply pad <2 bits LP A/N to 2 bits (the purpose of padding is to avoid LP A/N size misalignment in case of PUSCH is scheduled by DCI 0_0 without uplink tTAI) then treat the padded LP A/N as Rel-15 CSI part 1.

With the above analysis, we have the following proposal for RE mapping for HP A/N and LP A/N multiplexing on PUSCH.

Proposal 16: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, the following RE mapping rules are supported.

- HP HARQ-ACK is rate match around by other UCIs (if exist) and UL-SCH (if exist)

- LP HARQ-ACK is padded to 2 bits if it is 0 or 1 bit. The padded LP HARQ-ACK is rate matched around by other UCIs (if exist) and UL-SCH (if exist)

With the above proposal, there is no tone reservation and puncturing needed in RE mapping. A unified rate matching behaviour is applied to all UCI types and UL-SCH, which simplifies specification and both UE and gNB implementation.

In details, if HP A/N + LP A/N + LP CSI are multiplexed on PUSCH. The following RE mapping rules apply, assuming LP CSI part 2 is dropped.

- HP A/N map to REs first (without RE reservation as in Rel-15 even if HP A/N <=2 bits)

- LP A/N map to REs by rate-matching around HP A/N REs

- LP CSI part 1 map to REs by rate-matching around HP A/N, LP A/N REs

- UL-SCH map to REs by rate-matching around HP A/N, LP A/N, LP CSI part 1 REs

If HP A/N + LP A/N + HP CSI are multiplexed on PUSCH. The following RE mapping rules apply, assuming LP A/N is dropped.

- HP A/N map to REs first (without RE reservation as in Rel-15 even if HP A/N <=2 bits)

- HP CSI part 1 map to REs by rate-matching around HP A/N REs

- HP CSI part 2 map to REs by rate-matching around HP A/N, HP CSI part 1 REs

- UL-SCH map to REs by rate-matching around HP A/N, HP CSI part 1, HP CSI part 2 REs

### Power control

In the sections above, we have discussed approaches to multiplex HP and LP HARQ-ACKs on a PUSCH, in which the HP and LP are separately encoded. One remaining question is how to determine the transmit power of the PUSCH including the HP and LP HARQ-ACK. To this end, we observe that, NR Rel-16 introduces an enhanced open-loop power control mechanism to dynamically control the open loop power parameters of a PUSCH transmission. Although the original purpose of this scheme is for inter-UE multiplexing, this mechanism can be reused without any change to support HP and LP HARQ-ACK multiplexing on PUSCH. In particular, the gNB could indicate different P0 values already based on the priority of the HARQ-ACK as well as the priority of the PUSCH. This method works for both PUSCH with UL-SCH data and PUSCH without UL-SCH data. Based on this discussion, we make the following proposal.

Proposal 17: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, reuse the same power control formula as in Rel-15.

# LP HARQ-ACK compression

In Rel-17 UCI multiplexing on PUCCH or PUSCH with different priorities, it is necessity and beneficial to support LP HARQ-ACK compression.

The most useful use case for LP HARQ-ACK compression is in case of Tx power limited scenario. For example, in FR2, if several Tx antennas are handblocked, UE effectively lose at least a few dB of Tx power. Therefore, UE cannot transmit HP and LP HARQ-ACK together to the NW. In this case, LP HARQ-ACK compression/partial dropping is needed to make sure at least the HP HARQ-ACK can get through.

Another scenario which motivates LP HARQ-ACK compression is HP HARQ-ACK and LP HARQ-ACK multiplexing on a PUCCH with less number of RBs to accommodate all HARQ-ACK bits. This scenario is not fully avoidable by gNB scheduling because the PUCCH resources are preconfigured, the urgent (later scheduled) URLLC traffic could lead to total number of HARQ-ACK bits exceeding the max number of RBs in a PUCCH resource (set). Also, in case of SPS A/N, this scenario is not available neither. When max number of RBs in the PUCCH resource is not enough to accommodate HP and LP HARQ-ACK, the naturally solution is dropping or compress LP HARQ-ACK bits. Consider dropping as a naïve way of compression, LP HARQ-ACK compression should be supported in Rel-17 UCI multiplexing.

Fig 14: Compress LP A/N in case total # RBs in PUCCH resource is not sufficient

Another scenario that we see necessity of LP HARQ-ACK compression is eMBB HARQ-ACK overlap with URLLC PUCCH/PUSCH. When LP A/N payload size is large, the impact of max large size A/N to high priority URLLC PUSCH performance degradation is not negligible. To recover the PDSCH performance loss, one solution, as shown in Fig 15, could be transmitting a compressed version of the eMBB HARQ-ACK codebook. For example, UE can bundle the bits in HARQ-ACK codebook into less number of bits (say X bits) and multiplex the compressed X bits with URLLC PUSCH. In such way, part of the eMBB HARQ-ACK information get though, and the impact of eMBB HARQ-ACK to URLLC service can be minimized.

Fig 15: Compress LP A/N before multiplexing LP A/N on high priority PUSCH

Proposal 18: In Rel-17 UCI multiplexing, support low priority HARQ-ACK compression.

- FFS conditions to trigger low priority HARQ-ACK compression

- FFS details of compression scheme.

# PUSCH/PUSCH collision with different priorities

This scenario of collision between CG-PUSCH and DG-PUSCH was extensively discussed in Rel-16 and RAN1 decided to not handle that scenario in Rel-16. The same issue was added back in Rel-17 WID.

In our view, there are two cases in this scenario to consider

- Case 1: high-priority DG-PUSCH collide with low-priority CG-PUSCH

- Case 2: low-priority DG-PUSCH collide with high-priority CG-PUSCH

For case 1, it is not reasonable to reuse the timeline defined for Rel-16. The reason is because this Rel-17 scenario involves PUSCH vs PUSCH collision, which involves a different set of blocks at the UE as compared with PUCCH vs PUCCH or PUCCH vs PUSCH cancellation discussed in Rel-16. For example, UE need to cancel a LDPC encoder for the low priority PUSCH and start a new LDPC encoder for the high priority PUSCH, which take more time than the PUCCH vs PUCCH or PUCCH vs PUSCH cancellation discussed in Rel-16. The additional time on top of Rel-16 in terms of number of OFDM symbols depends on subcarrier spacing, as listed in Table 2.

Based on the above reasoning, we make the following proposal.

Proposal 19: On top of Rel-16 cancellation time (N2+d1) for PUCCH/PUCCH or PUCCH/PUSCH collision, additional time d2 is needed (which results N2+d1+d2 in total cancellation time) for LP CG-PUSCH and HP DG-PUSCH collision resolution. The additional number of OFDM symbols (d2) needed is listed in following table

Table 2. d2 for LP CG-PUSCH and HP DG-PUSCH collision resolution

|  | d2 [symbols] |
| --- | --- |
| 0 | 1 |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |

Besides the additional d2 as discussed above, for the value of d1 as specified in Rel-16 for PUCCH vs PUCCH or PUCCH vs PUSCH cancellation, there is a caveat in the definition of d1. In Rel-16, it is specified that base on UE capability, d1 can be 0, 1, or 2 OFDM symbols. However, how does d1 scales with subcarrier spacing is not specified in Rel-16. Clearly, if a UE needs d1=2 for 30Khz in FR1, it will need d1=8 for 120Khz in FR2. This bug should be fixed in spec. Therefore, we have the following proposal.

Proposal 20: For d1 defined for PUCCH vs PUCCH or PUCCH vs PUSCH cancellation with different priorities, support subcarrier spacing dependent d1 values. FFS exact d1 values for each subcarrier spacing.

# Signalling to trigger Rel-17 intra-UE mux functionality

In this section, the condition to trigger Rel-17 intra-UE multiplexing functionality, on top of Rel-16 prioritization, among UL transmissions with different priorities is discussed.

In principle, there are two approach to trigger this Rel-17 functionality.

- One approach is allowing indication to enable this feature on per UL transmission basis. If the UL transmission is dynamically scheduled, the indicator can be included in the DCI scheduling the UL transmission. If the UL transmission in semi-statically scheduled, the indicator can be included in the RRC configure the UL transmission.

- Another approach is keeping the trigger semi-statically on per UE basis via RRC signalling. According to UE capability signalling, gNB can use RRC configuration to either enable or disable this feature.

Among the two approaches, the second approach is preferred, because it is more robust to DCI miss direction and simpler to specify in standard. Furthermore, from UE implementation point of view, the second approach is much simpler.

Proposal 21: The Rel-17 intra-UE multiplexing feature is enabled/disabled via RRC configuration on per UE basis.

# RRC parameters for intra-UE multiplexing

In the description of RRC parameter “simultaneousPUCCH-PUSCH”, there is a “Note: Still FFS whether the feature is supported for same priority”, which is captured without consensus to captured it. At least Qualcomm clearly expressed disagreement to capture the note in the 3rd round email discussion post RAN1 106e. The comments from us in 3rd round of email discussion is copied in the following for reference.

“We don’t support the first note with V003. The note should be removed.

Please refer to the following agreement in RAN1# 102e. The agreement is a generic agreement covers same and different priority. The first note overturns an existing agreement.

Agreements:

Support simultaneous PUCCH/PUSCH transmissions on different cells at least for inter-band CA.

- FFS how to trigger this function.

- FFS for intra-band CA.

If companies want to add a note to reflect current situation, I think a fair note should be: ‘Note: Still FFS whether the same or a separate RRC parameter is used to configure simultaneous PUCCH/PUSCH transmissions with a same priority’. ”

Based on the same rationale as we express in the email discussion, we propose to modify the description of RRC parameter “simultaneousPUCCH-PUSCH” as in the following proposal.

Proposal 22: Update the description of the RRC parameter of “simultaneousPUCCH-PUSCH” as the following.

- Parameter indicates whether simultaneous PUCCH and PUSCH transmissions with different priorities is configured.

- Note: Still FFS whether the feature is supported for same priority.

- Note: Still FFS whether the same or a separate RRC parameter is used to configure simultaneous PUCCH/PUSCH transmissions with a same priority

- Note: Still FFS whether the feature is supported for intra-band CA.

# Conclusions

In summary, we have the following proposals for intra-UE multiplexing and prioritization for Rel-17 IOT and URLLC.

Proposal 1: Support the PHR for simultaneous PUCCH/PUSCH for inter-band CA with either of the following two options.

- Option 1: reuse LTE type 2 PHR for PUCCH transmission on PCC with a virtual/reference PUSCH

- Option 2: define a type 4 PHR for PUCCH transmission on a component carrier

Proposal 2: Clarify the agreement made in RAN1 102e “Support simultaneous PUCCH/PUSCH transmissions on different cells at least for inter-band CA” applies to PUCCH/PUSCH with same or different priorities.

Proposal 3: when a UE is configured with simultaneous PUCCH/PUSCH transmission and Rel-17 intra-UE multiplexing, take the following steps to resolve collision between overlapping of two or more uplink channels:

- Step 1: Resolve overlapping PUCCH(s) and/or PUSCH(s) with the same priority

  - Step 1.1: Overlapping PUCCHs of same priority are first resolved to obtain one final PUCCH for a given priority

  - Step 1.2: Resolve overlapping between PUCCH and PUSCH(s) of the same priority:

    - If all overlapping channels are of the same priority

      - If the remaining PUCCH and PUSCH can be transmitted simultaneously, then

        - Step 1.2.1: transmit the PUCCH and PUSCH(s) simultaneously

      - Otherwise

        - Step 1.2.2: multiplex the UCI on a PUSCH of the same priority

    - Otherwise (i.e., if the remaining overlapping channels are with different priorities),

      - Step 1.2.2: multiplex the UCI on a PUSCH of the same priority

- Step 2: Resolve overlapping PUCCH(s) and/or PUSCH(s) with different priorities

  - Step 2.1: resolving overlapping between HP PUCCH and LP PUCCH

  - Step 2.2: resolving overlapping between PUCCH and PUSCH(s) of different priorities

    - If the remaining PUCCH and PUSCH(s) can be transmitted simultaneously

      - Step 2.2.1: transmit PUCCH and PUSCH(s) simultaneously

    - Otherwise

      - Step 2.2.2: multiplex the UCI on a PUSCH with different priority

Proposal 4: Confirm the working assumption made in RAN1 #104bis-e

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17,

- Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N.

Proposal 5: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into PUCCH format 3 or format 4, when the total number of LP and HP HARQ-ACK bits is more than 2, and when the number of HP or LP HARQ-ACK has less than or equal to 2 bits

- The HP or LP  HARQ-ACK uses repetition encoding if the payload size is 1 bit, and uses the simplex encoding if the payload size is 2 bits

Proposal 6: In NR Rel-17, for multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into PUCCH, when the total number of low priority (LP) and high priority (HP) HARQ-ACK bits is more than 2

- For a given priority, support gNB to configure multiple coding rates for HARQ-ACK based on the payload size.

Proposal 7: For HP UCI and LP HARQ-ACK (in type 2 codebook) multiplexing on a PUCCH, round up LP HARQ-ACK size to a nearest reference size, in the calculation of total number of RBs for HP and LP UCI and in the PUCCH resource set determination.

Proposal 8: For HP UCI and LP UCI multiplexing on PUCCH format 2, support mapping encoded HP UCI bits first with a distributed RE mapping in frequency domain, followed by mapping encoded LP UCI bits onto remaining REs.

Proposal 9: the distance d for HP UCI distributed RE mapping is determined as d=S∙8∙L∙2∙rHPKHP, where

- KHP is the payload size for HP UCI, rHP is the coding rate for HP UCI.

- S is number of OFDM symbols in the PUCCH resource.

- L is the total number of RBs determined for multiplexed HP UCI and LP UCI transmission

Proposal 10: For HP UCI and LP UCI multiplexing on PUCCH format 2/3/4, support the following

- Two open-loop power control P0 values are configured for multiplexing LP and HP UCI

- Two separate powers are computed for LP UCI and HP UCI (following TS 38.213 Section 7.2.1) based on the corresponding P0 and BPRE for LP and HP UCI respectively, and based on the total number of RBs used to HP and LP UCI

- The final PUCCH power is determined based on the max power of the HP and LP powers

Proposal 11: In NR Rel-17, if a HARQ-ACK (with single priority) transmission on PUCCH format 0 or PUCCH format 1 collide with one SR, the UE performs the actions in Table 1 to resolve the collision.

- FFS: collision resolution for 1-bit HP HARQ-ACK and 1-bit LP HARQ-ACK overlapping with 1-bit HP or LP SR

Proposal 12: In NR Rel-17, for the case of multiplexing 1 bit SR and up to 2 bits HARQ-ACK with different priorities in a PUCCH format 0, adopt the multiplexed payload to CS indices mapping as shown in Fig 9 and Fig 10.

Proposal 13: In NR Rel-17, if a HARQ-ACK transmission on PUCCH format 2/3/4 collide with K SR transmissions including K1 HP SRs and K2 LP SRs, the UE append log2(1+K) bits to the HARQ-ACK payload.  Furthermore, if any of the K1 HP SR is positive, the log2(1+K) bits shall indicate a positive HP SR.

Proposal 14: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if CSI would multiplex on the same PUSCH,

- Drop CSI part 2, if CSI is a low priority CSI.

  - HP A/N reuse encoder and rate matching equation for Rel-15 A/N

  - LP A/N reuse encoder and rate matching equation for Rel-15 CSI part 1

  - LP CSI part 1 reuse encoder and rate matching equation for Rel-15 CSI part 2

- Drop LP HARQ-ACK, if CSI is a high priority CSI.

  - HP A/N reuse encoder and rate matching equation for Rel-15 A/N

  - HP CSI part 1 reuse encoder and rate matching equation for Rel-15 CSI part 1

  - HP CSI part 2 reuse encoder and rate matching equation for Rel-15 CSI part 2

- FFS: RE mapping rules.

Proposal 15: In NR Rel-17, up to four sets of scaling factors alpha can be configured to the UE to indicate separate alpha values for the following cases:

- Multiplexing LP HARQ-ACK/UCI on LP PUSCH

- Multiplexing LP HARQ-ACK/UCI on HP PUSCH

- Multiplexing HP HARQ-ACK/UCI on LP PUSCH

- Multiplexing HP HARQ-ACK/UCI on HP PUSCH

Proposal 16: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, the following RE mapping rules are supported.

- HP HARQ-ACK is rate match around by other UCIs (if exist) and UL-SCH (if exist)

- LP HARQ-ACK is padded to 2 bits if it is 0 or 1 bit. The padded LP HARQ-ACK is rate matched around by other UCIs (if exist) and UL-SCH (if exist)

Proposal 17: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, reuse the same power control formula as in Rel-15.

Proposal 18: In Rel-17 UCI multiplexing, support low priority HARQ-ACK compression.

- FFS conditions to trigger low priority HARQ-ACK compression

- FFS details of compression scheme.

Proposal 19: On top of Rel-16 cancellation time (N2+d1) for PUCCH/PUCCH or PUCCH/PUSCH collision, additional time d2 is needed (which results N2+d1+d2 in total cancellation time) for LP CG-PUSCH and HP DG-PUSCH collision resolution. The additional number of OFDM symbols (d2) needed is listed in following table

Table 3. d2 for LP CG-PUSCH and HP DG-PUSCH collision resolution

|  | d2 [symbols] |
| --- | --- |
| 0 | 1 |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |

Proposal 20: For d1 defined for PUCCH vs PUCCH or PUCCH vs PUSCH cancellation with different priorities, support subcarrier spacing dependent d1 values. FFS exact d1 values for each subcarrier spacing.

Proposal 21: The Rel-17 intra-UE multiplexing feature is enabled/disabled via RRC configuration on per UE basis.

Proposal 22: Update the description of the RRC parameter of “simultaneousPUCCH-PUSCH” as the following.

- Parameter indicates whether simultaneous PUCCH and PUSCH transmissions with different priorities is configured.

- Note: Still FFS whether the feature is supported for same priority.

- Note: Still FFS whether the same or a separate RRC parameter is used to configure simultaneous PUCCH/PUSCH transmissions with a same priority

- Note: Still FFS whether the feature is supported for intra-band CA.

# References

- 3GPP TSG RAN Meeting #86, RP-193233, “3GPP Work Item Description; Enhanced Industrial Internet of Things (IIOT) and URLLC Support”, Nokia, Nokia Shanghai Bell, Sitges, Spain, December 9-12, 2019.

- 3GPP TSG SA, 3GPP TR 22.804, V16.3.0, “Technical Specification Group Services and Systems Aspects; Study on Communication for Automation in Vertical Domains (Release 16)”, July 2020.

- 3GPP TSA RAN1 Meeting #104, R1-2101459, “HARQ-ACK enhancement for IOT and URLLC,” e-Meeting, Jan. 25-Feb. 5, 2021