3GPP TSG RAN WG1 #106bis-e		R1- 2110225

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.17.7

Source: 	Qualcomm Incorporated

Title: 	UE features for UE power saving enhancement

Document for:	Discussion/Decision

- Introduction

In this contribution, we share our views on UE feature group drafted for NR Rel-17 UE Power Saving.

- Discussion on Power Saving UE Features for NR Rel-17

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  | N |  | High idle/inactive mode UE power consumption if NR SA networks | Per UE | N | N | N |  | Optional without capability signalling |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receving L1 indication for TRS availability |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode | Per UE | N | N | N |  | Optional without capability signalling |
| 29. NR_UE_pow_sav_enh | 29-3 | PDCCH monitoring adaptation within an active BWP | - Support of PDCCH monitoring adaptation behaviour 1/1A - Support of PDCCH monitoring adaptation behaviour 2/2A/[2B] |  | Y |  | PDCCH monitoring adaptation within an active BWP is not supported | Per UE | N | N | N | FFS: Support of PDCCH monitoring adaptation behaviour 1/1A/2/2A/2B | Optional |

  - Licensed and unlicensed differentiation

For FG 29, the differentiation between licensed and unlicensed bands is necessary. It is not because there are significant implementation challenges specifically in the unlicensed band or vice versa, but because it is unlikely that deployment schedules of NR in licensed and unlicensed bands are the same. Note that NR has already been deployed worldwide in licensed bands, while the deployment for unlicensed bands has not been started. If the feature is based on per-UE signaling, in order to introduce the UE power saving feature for either licensed or unlicensed band first, it requires IODT for both licensed and unlicensed bands, and thus the introduction of the feature would be delayed. However, if the feature is differentiated between licensed and unlicensed, the feature can be introduced for licensed band after IODT in the licensed band first and without IODT in the unlicensed band, and vice versa. The same argument also applies to the NTN band.

Among the types of signaling, “Per Band” should be used to support the licensed-unlicensed differentiation. Otherwise, if a signaling type other than “Per Band” is used, the differentiation between licensed and unlicensed should separately be enabled with the feature.

Proposal 1: Unless otherwise stated, the type for the UE power saving feature should be at least per band (or preferreably a type with finer granularity), given the potential UE testing differentiation among licensed, unlicensed, and NTN band.

- Conclusion

Proposal 1: Unless otherwise stated, the type for the UE power saving feature should be at least per band (or preferreably a type with finer granularity), given the potential UE testing differentiation among licensed, unlicensed, and NTN band.