3GPP TSG-RAN WG1 Meeting #106bis-e	Tdoc R1- 2110424

E-meeting, October 11th – 19th, 2021

Agenda Item:	8.3.2

Source:	Moderator (Ericsson)

Title:	Summary#2 - Enhancements for IIOT/URLLC on Unlicensed Band

Document for:	Discussion, Decision

# 1	Introduction

This document summarizes the discussions in input contributions and during RAN1#106bis-e under the following email thread assigned by RAN1 Chair:

[106bis-e-NR-R17-IIoT-URLLC-03] Email discussion on unlicensed band URLLC/IIoT – Sorour (Ericsson)

- 1st check point: October 14

- Final check point: October 19

This document is revision of R1-2110423.

## GTW and Email approvals

### Issue#1: Section 2.1

Moderator’s recommendation: Adopt Alt-3.

Proposal 1-1 (updated):

In semi-static channel access mode, for PUSCH repetition Type B select one of the following:

- Alt 1: If a nominal repetition overlaps with an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT or associated to UE’s FFP in case UE assumes UE-initiated COT, the nominal repetition is dropped (i.e. no segmentation around the idle period).

- Alt 2: If a nominal repetition overlaps with a set of symbols in an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT for the nomial repetition or associated to UE’s FFP in case UE assumes UE-initiated COT for the nomial repetition, all the symbols in the idle period should be considered as invalid symbols which are not considered for an actual repetition as in Rel-16.

  - Segmentation before and/or after the idle period is applied when applicable.

- Alt 3: If a nominal repetition overlaps with a set of symbols in an idle period associated to gNB’s FFP or UE’s FFP, all the symbols in the idle period should be considered as invalid symbols which are not considered for an actual repetition as in Rel-16.

  - Segmentation before and/or after the idle period is applied when applicable

Summary of views on Proposal 1-1:

- Alt 1:

  - Supported by: Intel, LG, HW/HiSi, Nokia/NSB, Apple, Ericsson, FW, Sharp

  - OK to compromise to Alt-2: Ericsson

- Alt 2:

  - Supported by: ZTE, ETRI, Sony, IDC, Pana, Spreadtrum, Samsung, CATT, LG, DCM, vivo, Len/MOT, ZTE (also Alt-3), Qualcomm

  - OK to compromise to Alt-1: DCM, CATT, Pana

### Issue#2: Section 2.2

Moderator’s recommendation: Adopt Option 2 (with A)

Proposal 2-1:

- In semi-static channel access mode, for PUSCH repetition Type B select one of the following:

  - Option 1: Orphan symbol(s) are dropped as in Rel-16

    - Supported by: HW/HiSi, Pana, ETRI, Sony, Ericsson, Samsung, FW, DCM (if DFT-s-OFDM), Sharp

  - Option 2: Orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted.

    - Supported by: QC, LG, ZTE, Nokia/NSB, IDC, Spreadtrum, Apple, Intel, DCM (if OFDM)

      - Support of the following alternatives to construct the orphan symbols:

        - A) Perform CP extention (QC)

        - B) Repeat the previous or following symbol (Nokia/NSB, Spreadtrum)

        - C) Use DMRS symbol (ZTE)

Summary of views on Proposal 2-1:

- Option 1:

    - Supported by: HW/HiSi, Pana, ETRI, Sony, Ericsson, Samsung, FW, DCM (if DFT-s-OFDM), Sharp, CATT

    - OK to compromise to Option 2: Pana, Ericsson

  - Option 2:

    - Supported by: QC, LG, ZTE, Nokia/NSB, IDC, Spreadtrum, Apple, Intel, DCM (if OFDM), Len/MOT, vivo

    - OK to compromise to Option 1: DCM

### Issue#3: Section 2.3

Moderator’s recommendation: Adopt Alt-B

Proposal 3-1 (updated):

Select one of the following alternatives:

- Alt-A: In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include only scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

- Alt-B: In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

  - A DL transmission to any other UE in the cell than the COT initiating UE and/or a broadcast transmission can be additionally included in the DL transmission burst if the gNB fulfills the follwong condition:

    - It is gNB responsibility to ensure that reception of “the DL transmission or the broadcast transmission” does not affect any channel access related assumptions at UE for any UL transmission or DL transmission reception.

      - That means that the reception of “the DL transmission or the broadcast transmission” does not affect any channel access related procedures (i.e. COT-ownership assumption, initiating a COT or sharing or using an initiated COT, sensing, skipping transmission/reception due to idle period of an initiated COT, etc.) for any UL transmission or a DL reception scheduled or configured to UE.

Summary of views on Proposa 3-1:

- Alt-A:

  - Supported by: Intel, HW/HiSi (1st), Ericsson, CATT, Apple, Len/MOT (1st prio), Pana, Spreadtrum, ETRI (2nd)

- Alt-B:

  - Supported by: Ericsson, Sony, FW, DCM, HW/HiSI (2nd), Apple, CATT, Sharp, vivo, Pana, Spreadtrum, Nokia/NSB, Len/Mot, ETRI

### Issue#4: Section 2.5

Moderator’s recommendation: Adopt following proposals

Proposal 5A-1:

Support configuration of harq-ProcID-Offset2 for operation in unlicensed spectrum when the cg-RetransmissionTimer-r16 is not configured.

Proposal 5B-1 (updated):

The following RRC parameters is NOT needed when UE is confiured to operate with semi-static channel access mode.

- channelAccessPriority

Proposal 5B-3:

The following RRC parameters are NOT needed when cg-RetransmissionTimer is configured for CG operation with shared spectrum channel access.

- pusch-RepTypeIndicator

- startingFromRV0

Proposal 5B-4:

The RRC parameter of phy-PriorityIndex should be applicable for CG operation in unlicensed band.

Proposal 5E-1:

Introduce new RRC parameters ul-AccessConfigListDCI-0-2 and ul-AccessConfigListDCI-1-2 to support indication of CP extension, LBT type, and CAPC with DCI 0_2 and 1_2 with dynamic channel access.

# Discussion topics

## 2.1	Segmentation of Type-B PUSCH repetition

One of the open issues from last meeting was that when a normail repetiton overlaps with an idle period that UE is not allowed to transmit within, whether to drop the repetition or do the segmetation around the idle periods. Companies’ views are different with respect to this issue.

Agreements

- For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether PUSCH segmentation should take into account the idle period of an FFP.

  - FFS on details

Summary of views:

- Drop the Nominal repetiton

- Intel, LG, HW/HiSi, Nokia/NSB, Apple, Ericsson, FW

- Summary of arguments: Simplicity, prevent ambiguity issue causing mis-detection and potentially collision, respecting the design principle in Rel-16 (i.e. the segmentation itself is done based on semi-static configuration/signaling only, while the dynamic signaling is used only to decide whether to transmit or drop an actual repetition.), no need to scheduling restriction to avoid problems

- Segment the Nominal repetition

- ZTE, ETRI, Sony, IDC, Pana, Spreadtrum, Samsung, CATT

- Summary of arguments: ambiguty issue is general and not critical (low probability in controlled enviorement) to compensate the PUSCH performance by dropping the repetitons, issues can be avoided by prper scheduling and configuration.

## 2.1.1	Discussion – 1st round

Proposal 1-1:

In semi-static channel access mode, for PUSCH repetition Type B select one of the following:

- Alt 1: If a nominal repetition overlaps with an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT or associated to UE’s FFP in case UE assumes UE-initiated COT, the nominal repetition is dropped (i.e. no segmentation around the idle period).

  - Supported by: Intel, LG, HW/HiSi, Nokia/NSB, Apple, Ericsson, FW, Sharp

- Alt 2: If a nominal repetition overlaps with a set of symbols in an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT for the nomial repetition or associated to UE’s FFP in case UE assumes UE-initiated COT for the nomial repetition, all the symbols in the idle period should be considered as invalid symbols which are not considered for an actual repetition as in Rel-16.

  - Segmentation before and/or after the idle period is applied when applicable.

  - Supported by: ZTE, ETRI, Sony, IDC, Pana, Spreadtrum, Samsung, CATT, LG

| Questions: - Q1: Companies are kindly requested to provide any update/correction on the discussion and their positions with respect to Proposal 1-1. - Q2: Proponents of Alt-1 kindly clarify how critical is mis-aligment issue as compared to performance loss due to dropping PUSCH. Proponents of Alt-2 kindly clarify why violating the design principle is not an issue in Rel-17 as compared to Rel-16. - Q3: Please indicate if you are flexible to consider your non-preferred alternative as it appears to be a dead-lock. - Q4: Please share any other comment if any. |  |
| --- | --- |
| Company | Comment |
| Sony | Idle Periods are semi-statically configured and unlike flexible symbols, it does not depend on any dynamic indication such as SFI to change its status to DL or UL. Hence, we do not think there is significant deviation from Rel-16 principle to segment the nominal PUSCH repetition around Idle Period. |
| Intel | We support Alt.1. As pointed out at the end of the last meeting due to ambiguity between gNB and UE on who may be the initiating device, there would be consequent ambiguity between the gNB and UE on how segmention may be applied. For this reason, we prefer Alt.1. |
| Futurewei | We support Alt-1, we have same concern as Intel. |
| DOCOMO | We prefer Alt 2 not to drop the nominal repletition from URLLC perspective, but can live with Alt 1 if majority companies prefer it. |
| Huawei, HiSilicon | We support Alt 1 and we are not open to further eahncements to mitigate the issues associated with Alt 2. If segmentation is supported around the g-idle period while the nominal repetition is sharing gNB COT), the second actual repetition would be transmitted after g-idle leading to potential collision with gNB’s transmission if gNB initiates the next g-FFP. Avoiding configuring a nominal repetition overlapping with any g-idle imposes restriction on the CG resources to be used as well in UE initiated COT. Furthermore, in contrast to segmentation in Rel-16, gNB may not be aware whether or not UE shares the gNB COT based on the UE’s rules for configured UL. Hence, gNB may expect the UE to segement the overlapping nominal repetition around its g-idle but UE determines that it is initiating its own COT and does not segment that nomial repetion. The UE would instead segement another nominal repetition if it is overlapping with the applicable u-idle which was also not expected by the gNB. This in fact complicates the issue of misalignment between the UE and gNB for configured UL. Similar issues can be observed in the other case of UE segementing a nominal repetition around its u-idle. Attempting to avoid both cases by ‘proper’ configuration would be too restrictive especially when the u-FFP and g-FFP are of different periods. |
| Apple | We support Alt-1. To respond to Sony’s comment, even though idle periods are semi-statically configured, there can be ambiguity in terms of whether gNB or UE is the COT initiator (which is dynamically determined by the UE). Therefore Alt-1 is preferred. |
| CATT | We prefer Alt-2 as in Rel-16 but we are also fine with Alt-1 considering the potential misalignment on the COT initiator between gNB and UE. |
| vivo | Q1: For Proposal 1-1, our position is Alt 2. Q2: As pointed out by compnies, there may be ambiguity between gNB and UE regarding the COT initiator when using current mechanisms. However, ambiguity is a general issue which will not only impact segmentation, but also impact other transmissions during the idle period. Even by Alt.1, the ambiguity on who is the COT initiator still exists, since the dropping is done only around the idle period that associated to gNB’s FFP in case UE shares gNB-initiated COT or associated to UE’s FFP in case UE assumes UE-initiated COT. In addition, Alt.1 violates the design principle for PUSCH repetition Type B, not Alt.2. Alt.2 keeps the design principle and if gNB configures the CG transmission occasion overlaps/cross gNB’s idle period, it means gNB should receive the potential CG transmissions. The configuration is under gNB’s control, if there is ‘blocking’ issue for gNB to initate the COT in the next FFP, then gNB should not configure the CG resource cross its FFP’s idle period. |
| Sharp | We prefer Alt 1 (have added our company name under Alt 1) |
| Panasonic | We prefer Alt.2, but Alt.1 is acceptable for simplicity. |
| LG | It seems our position is wrongly captured in above, so I corrected. We prefer Alt-2 and also think there is no reason not to apply same rule as in Rel-16. |
| Nokia, NSB | We support Alt 1 |
| Lenovo, Motorola Mobility | We prefer Alt2 to achieve more PUSCH transmission. The misalignment issue between UE & gNB regarding the COT initiator assumption is a general issue for any PUSCH transmission. |
| ZTE | First we support Alt 2. More specifically, we think the nominal repetition should be segmented by the idle period (including the idle period associated to the gNB’s FFP or UE’s FFP) in any case and the idle period are not for actual transmission. It can be seen as the combination of Alt. 1 and Alt. 2. As shown in the figure below, the nominal 2 is always segmented by UE-idle period and nominal 3 is always segmented by gNB-idle period. In case the UE shares the gNB-initiated FFP, PUSCH 2 is transmitted instead of nominal 2 and there is no transmission in the UE-idle period even though the transmission is allowed. In case the UE assumes UE-initiated FFP, PUSCH 2 is transmitted since the UE is not allowed to transmit in the UE-idle period. It can be seen that there is no ambiguity between the gNB and the UE. Compared with Alt. 1 to drop the whole nominal transmission, it can also ensure the transmission as many as possible to improve the reliability. Therefore, our proposal is that both the gNB idle period and UE idle period are considered as invalid symbol for nominal PUSCH segmentation. |
| ETRI | Our preference is Alt-2. In URLLC, basically many DL/UL burst transmissions should sufficiently be reliable, and the probability of DL miss-detection can be controlled to be very small by network. Thus, we think that the ambiguity mentioned in the discussion is not a critical issue. |
| Samsung | Q1: We support Alt-2. Q2: We share similar view as vivo. Segementation to semi-configuration is the design principle for type B repetition, to ensure the transmission within a time window to increase the reliability and reduce the latecy. Otherwise, it loses the spirit of Type B repetition. Regarding on the issue pointed out by Huawei, we think the transmission across g-idle is general issue, which is not only for segementation. With Alt 1(or alt 2), for the case when UE initial a COT, it will not segement or drop the transmission overlap with on g-idle period. This can be discussed seperataely. For the ambiguity case pointed out by Apple, we think it is a corner case. It only happens when UE fail to detect gNB COT (1%?), UE can initialize UE COT, the repetition across the idle period. And the performance different may be, wrongly added one repetition, which may cause 0.xdB performace loss. Moreover, for a smart gNB, it the gNb is not sure about whether UE may fail to detect gNB COT, the gNB can not combine such segementation, assuming it is dropped(e.g., for a UE in poor coverage). The performance is the same as dropping. There is no loss compared to Alt 2. However, in most of the situation, gNB can assume there is no mis-understanding, e.g., gNB knows the channel status of this UE, which only has 0.01% of chance the UE fail to detect gNB COT. However, if we drop all the nominal repetitions for all the case, it will lose 1/2 ~1/32 transmissions for sure, even for most of other cases (99%) there is no issue. In short, the loss of alt 1 is big and for sure, which is not a good design for URLLC. The potential loss for alt 1 small, and with proper (e.g., not combine) gNB implemnation, the performace is no worse than alt 1 but for most of the case the performance is definitely better than alt 1. |
| Qualcomm | We support Alt-2 with similar view as Vivo and Samsung. |
| Moderator | Upt0 v017 (Samsung) Summary of views on Proposal 1-1: - Alt 1: - Supported by: Intel, LG, HW/HiSi, Nokia/NSB, Apple, Ericsson, FW, Sharp - OK to compromise to Alt-2: Ericsson - Alt 2: - Supported by: ZTE, ETRI, Sony, IDC, Pana, Spreadtrum, Samsung, CATT, LG, DCM, vivo, Len/MOT, ZTE (also Alt-3), Qualcomm - OK to compromise to Alt-1: DCM, CATT, Pana @Sony: As Apple explained, true that idle periods are configured semi-statically. But whether they would be an active idle period, happens in a dynamic fashion, depending if the corresponding initiated COT is used for UL transmission. I think that is what Apple refers to in my understanding -> semi-statically configured but becoming idle, dynamically. @LG: Apologies and thanks for correction. @All: With respect to design principle on Rel-16, we have two opposing views: - Apple: Semi-static resources are considered invalid (Alt-2 violates the principle). - Vivo/Samsung: Alt.1 violates the design principle for PUSCH repetition Type B, not Alt.2. Alt.2 keeps the design principle and if gNB configures the CG transmission occasion overlaps/cross gNB’s idle period, it means gNB should receive the potential CG transmissions. The configuration is under gNB’s control, if there is ‘blocking’ issue for gNB to initate the COT in the next FFP, then gNB should not configure the CG resource cross its FFP’s idle period. @All: Propoents of Alt-2 have the view that ambiguity is general issue. Samsung further analyses that having design based on Alt-1 can have severe performance degregation as Alt-2 (basically, design based on error cases) @All: With respect to ambiguity issue, ZTE proposes that both the gNB idle period and UE idle period are considered as invalid symbol for nominal PUSCH segmentation. It is added for discussion as Alt-3. @Moderator recommendation: From Moderator perspective ZTE proposal (captured as Alt-3) below addresses all the concern. It may not be as optimum as Alt-2 if idle period of g-FFP and u-FFP do not fully overlap. But it also doesn’t have the performance degration due to dropping nominal PUSCH repetiton as in Alt-1. It also respect both design principles @All: Is Alt-3 acceptable as the best compromise that we can achieve? Proposal 1-1 (updated): In semi-static channel access mode, for PUSCH repetition Type B select one of the following: - Alt 1: If a nominal repetition overlaps with an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT or associated to UE’s FFP in case UE assumes UE-initiated COT, the nominal repetition is dropped (i.e. no segmentation around the idle period). - Alt 2: If a nominal repetition overlaps with a set of symbols in an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT for the nomial repetition or associated to UE’s FFP in case UE assumes UE-initiated COT for the nomial repetition, all the symbols in the idle period should be considered as invalid symbols which are not considered for an actual repetition as in Rel-16. - Segmentation before and/or after the idle period is applied when applicable. - Alt 3: If a nominal repetition overlaps with a set of symbols in an idle period associated to gNB’s FFP or UE’s FFP, all the symbols in the idle period should be considered as invalid symbols which are not considered for an actual repetition as in Rel-16. - Segmentation before and/or after the idle period is applied when applicable |

## 2.2	Orphan symbol of Type-B PUSCH repetition

Another open issue with respect to PUSCH repetition Type-B is that whether to drop Orphan symbols or not for operation with semi-static chanel access mode. In Rel-16, the orphan symbol(s) or repetition segments of one symbol duration are dropped. Companies’ views are different with respect to this issue.

Agreements

- For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted. FFS on details

Summary of views:

- Drop the Orphan symbol

  - HW/HiSi, Pana, ETRI, Sony, Ericsson, Samsung, FW, DCM (if DFT-s-OFDM)

  - Arguments: Simpicity and resue as Rel-16, Extra LBT is not a burden, and maintain channel access is not an issue

- Transmit the Orphan symbol

  - QC, LG, ZTE, Nokia/NSB, IDC, Spreadtrum, Apple, Intel, DCM (if OFDM)

  - Arguments: Avoid extra LBT and maintain channel access

  - How to construct orphan symbols: With no UCI multiplexing, By repetition of a symbol, CP extension, or DMRS, No UCI multiplexing on Orphan symbol

## 2.2.1	Discussion – 1st round

Proposal 2-1:

- In semi-static channel access mode, for PUSCH repetition Type B select one of the following:

  - Option 1: Orphan symbol(s) are dropped as in Rel-16

    - Supported by: HW/HiSi, Pana, ETRI, Sony, Ericsson, Samsung, FW, DCM (if DFT-s-OFDM), Sharp

  - Option 2: Orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted.

    - Supported by: QC, LG, ZTE, Nokia/NSB, IDC, Spreadtrum, Apple, Intel, DCM (if OFDM)

      - Support of the following alternatives to construct the orphan symbols:

        - A) Perform CP extention (QC)

        - B) Repeat the previous or following symbol (Nokia/NSB, Spreadtrum)

        - C) Use DMRS symbol (ZTE)

| Questions: - Q1: Companies are kindly requested to provide any update/correction on the discussion and their positions with respect to Proposal 2-1. - Q2: Proponents of Option-1 kindly clarify why simple orphan os problemtic. Proponents of Option-2 kindly clarify why extra LBT is burden. - Q3: Please indicate if you are flexible to consider your non-preferred alternative as it appears to be a dead-lock. - Q4: Please share any other comment if any. |  |
| --- | --- |
| Company | Comment |
| Sony | Since we do not have enhancement on Type B CG-PUSCH repetition, the repetitions can occur in semi-statically configured Transmission Occasions (cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16), which can be configured to avoid semi-static DL symbols and slot boundaries. Hence, the likely case for segmentation is around Idle Period (if we decide to segment) and here, the UE would need to perform LBT if there is an actual repetition after that Idle Period. |
| Intel | We prefer Option 2 (no strong view on A/B or C), and the main motivation is due to gap that the orphan may create across consecutive PUSCH transmissions that in principle could be transmitted with no LBT. However, when the condition that lead to an orphan symbol occurs and this is not transmitted, a gap between PUSCH transmissions may be created and in this case a UE may be mandated to perform LBT, which with no doubt may be highly detrimental if the LBT may not succeed since this would have high impact in terms of latency no only to the PUSCH following the orphan symbol, but to all other PUSCHs that fall within that specific FFP. |
| Futurewei | We prefer Option 1, we do not see LBT failure as detrimental to justify sending a “busy” signal during the orphan symbols. Sending busy signals for coexistence is potential controversial in the unlicensed community (including previous strong opposition from the IEEE), and not encouraged by the regulators. We prefer having the same approach as in Rel-16. |
| DOCOMO | We prefer Option 2 not to perform extra LBT, which has risk of LBT failure, but can live with Option 1 it this issue cannot be concluded easily. It may not be critical issue in controlled environment. No consensus for this issue means Option 1. |
| Huawei, HiSilicon | We support Option 1. Given that the operation is in unlicensed controlled environments, gaps due to orphan symbols between two actual repetitions do not pose a threat to lose the initiated COT. In our view, there is not enough motivation to change the Rel-16 behavior. We do not prefer any of the alternatives of Option 2 |
| Apple | We prefer Option 2 to avoid unnecessary LBT and possible loss of channel access. In terms of what is transmitted for the orphan symbol, we have slight preference not to over-complicate the situation. Treating it as a non-dropped repetition (either DMRS or DMRS+data in case of CP-OFDM) can be a simple solution. However, it should be noted that such a repetition should not be considered for UCI multiplexing. |
| CATT | We prefer Option 1 for simplicity. |
| vivo | We slightly prefer Option 2. The transmission in unlicensed band is different from that in licensed band. In unlicensed band, we tried very hard to improve the channel access probability and avoid non-necessary LBTs. Therefore, it will be better to tranmist the orphan symbol to avoid additional LBT. Even in controlled scenario, the LBT may still fail, transmission in orphan symbol will cause no harm but can avoid potential LBT failure. We are open for the signals transmitted in the orphan symbol, e.g., it can be alt. A/B/C as in the proposal, it can also be the original segmentation. |
| Sharp | We prefer Option 1 (have added our company name under Option 1) |
| Panasonic | We prefer Option 1 for less specification impact, but we are open to consider Option 2 if simple solution (such as A, B, or C) is agreeable. |
| LG | We prefer Option 2 (and prefer Alt A using CP and also OK with Alt B/C), and share the same view with Intel. Even if Option 1 is applied, the orphan symbol, anyhow, cannot be utilized by the gNB for DL transmission in order to ensure LBT gap for the UE. Given that, the UE would need to perform LBT unnecessarily, and would have to drop later PUSCHs (after the orphan symbol) unnecessarily in case of LBT failure. This is quite different (performance loss) compared to Rel-16 licensed band case. |
| Nokia, NSB | We prefer (slightly) Option 2. For the options there, A and B are probably equivalent, since CP extionsion with a duration of one symbols results in repetition of the following symbol. |
| Lenovo, Motorola Mobility | We prefer option 2 to transmit in orphan symbol(s) between the actual repetitions. Furthermore, we would prefer to transmit DMRS on the orphan symbol. This DMRS (orphan) symbol could be potentially used for joint channel estimation (as already agreed under coverage enhancements WI). |
| ZTE | We support Option 2 since any scheme that can keep the transmission continuity and avoid LBT failure in unlicensed spectrum should be supported and this is very important for NR-U and URLLC service. |
| ETRI | We prefer Option 1. We do not see a critical problem with the legacy operation for FBE. |
| Samsung | We support Option 1. First of all, orphan symbol is a corner case. For normal case, gNB can easily avoid to create orphan symbol, since all the segementation is based on semi-static configuration. This is also the main reason to decide to drop in Rel-16 Type B PUSCH transmission. Secondly, we share similar view as other companies that we don’t think it will increase LBT failure rate in controlled environments. |
| Qualcomm | We support option 2. |
| Moderator | Summary of views on Proposal 2-1: - Option 1: - Supported by: HW/HiSi, Pana, ETRI, Sony, Ericsson, Samsung, FW, DCM (if DFT-s-OFDM), Sharp, CATT - OK to compromise to Option 2: Pana, Ericsson - Option 2: - Supported by: QC, LG, ZTE, Nokia/NSB, IDC, Spreadtrum, Apple, Intel, DCM (if OFDM), Len/MOT, vivo - OK to compromise to Option 1: DCM No LBT argument @All: In addition to previous arguments, few points are raised that need your attention. - FW raised a valid point that we should not specifiy solutions for sake of occupying channels. Totally agree. Moderator’s understanding is that is not intention/consequence of Option 2. The solutions are based on CP extension, or repletion (beside DMRS) that can be useful at NW for improving detection. And the better detection, the less need to occupy channel for retransmission and live it for others to use (Recalling discussions at ETSI BRAN in good old days ) - DCM mentioned that no consencus means Option 1. That is no agreement to enhancements in Rel-17. @ Moderator’s understanding of situation: - (For Option 1)I think we should be consistent with our arguments across designing different features. If the design shouldn’t be based on mis-detection and hence ambiguity, that means channel is fairly reliable and design shouldn’t be based on compensating for LBT failure. Ambiguity is corner case for controlled environment (for Option 1). - (For Option 2) It seems to me the argument for Option 2 is simplifying UE behavior (not need to do another LBT). Basically, UE knows the reptiton and can continue transmit, without adding pause to go to RF front-end, during LBT, and then comeback. Kind of unnessarily operation. - Also, it seems in lic, the design of orphan was base don generating a new PUSCH. But here, repetition based solution supported in unliced (CP extension), are not complicated as the design was in mind in Rel-16 for lic. - These make the cases and motivaitons different from licensed operation. @Moderator’s recommnedation: - Based on Moderator understanding of the arguments, it seems avoiding the need for additional LBT when possbile in a simple way, does benefit UE’s implementation and makes sense. - Could we consider Option 2 (with A)? |

## 2.3	DL transmissions in UE-to-gNB COT sharing

In this section, the discussion aims to resolve the open issues related to content of DL transmission bursts in UE-to-gNB COT sharing.

Based on the agreements below, there is an open issue whether DL transmission burst based on sharing a COT initiated by a UE can include transmissions intended to other UEs than the COT-initiating. Views were expressed during that last meeting that EDT adjustments could be simplified for UE initiated-COT. However there were also views expressing that the simplification may depend on the decison on the content of DL in UE-to-gNB COT sharing. This issue is discussed in next section.

Agreement

In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include at least scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

- FFS whether/how the DL transmission burst can include transmission to any other UE in the cell than the COT initiating UE and/or broadcast transmission while ensuring that the COT initiated by the UE is not shared by any other UE in the cell for any UL transmission

Summary of views:

With respect to whether DL transmisison burst includes transmisisons to other UEs than the one who initiated the COT, views are divided as the following:

- Alt-1: In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include only scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

  - Supported by: Intel, HW/HiSi, Ericsson, CATT, Apple (If Alt-2a is not supported), Len/MOT (1st prio)

- Alt-2: In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, in addiiton to scheduled DL transmission or a DCI intended for the UE that initiated that FFP, can include DL transmission to any other UE in the cell than the COT initiating UE and/or broadcast transmission while ensuring that the COT initiated by the UE is not shared by any other UE in the cell for any UL transmission.

  - Supported by: Apple (If Alt-2a is supported), Nokia/NSB, FW, WILUS, Sharp, DCM, LG, Samsung, vivo, MTK, Len/MOT (2nd prio)

Supprotive of Alt-1 claim that the added complexity to convery the information to the UE on COT-ownership is not worth it. Some companies also have the view that this flexibility may complicate EDT adjustment (see next discussion).

Supportive of Alt-2 claim that not allowing this functionality is an unnessariry restriction that is not even motivated by regulations.

On how to ensure UE-to-UE COT sharing is not occurred, views are listed below as variants of Alt-2.

- Alt-2a: It is gNB’s responsbility to ensure that COT initiated by the UE is not shared by any other UE in the cell for any UL transmission.

  - Supported by: Apple, Nokia/NSB, FW, WILUS, Sharp, DCM

- Alt-2b: There is no UL resource allocated for any UE after the DL reception or there is only UL resource allocated for the COT-initiating UE after the DL reception.

  - Supported by: LG

- Alt-2c: Using one of the following options:

  - Option 1: Explicit gNB-to-UE COT sharing indication in DCI.

    - gNB uses 1 bit to explicitly indicate to UE whether it has initiated a COT. The 1 bit can be carried in the GC-PDCCH or UE-specific DCI, etc.

  - Option 2: DL signal detection from dedicated positions.

    - Any DL signal detected at the specific position(s) indicates that gNB has initiated a COT. If signals are detected in these dedicated positions, UE assumes that gNB has initiated a COT, otherwise, UE assumes that gNB shares the UE-initiated COT

  - Supported by: vivo

- Alt-2d: Using one of the following options:

  - Option 1: A UE assumes gNB has initiated a COT if the UE receives explicit indication in DCI 2_0.

  - Option 2: A UE assumes gNB has initiated a COT if the UE detects DL transmission at the beginning of gNB FFP.

  - Supported by: Samsung

- Alt-2e: A group-common DCI at the beginning of a gNB-FFP can indicate whether the COT is a gNB-COT. UE assumes that the gNB has not acquired the COT if it does not detect the group-common DCI.

  - Supported by: Len/MOT (2nd prio)

- Alt-2f: For a DL transmission based on sharing a UE intiated COT, other UEs can recognize that the gNB is sharing the UE COT through the gNB use of a different DMRS encoding. FFS details.

  - Supported by: MTK

Moderator analysis:

In the following, Moderator provides an analysis on the proposed alternatives. Please note that in this analysis, the EDT aspects are not considered (see next section).

- Alt-1 and Alt-2a: If seems supporter of both Alt-1 and Alt-2a prefer simplicity and not introdcuing new solutions. However, Alt-2a beleives that there are conditions that DL transmissions could be included such that would not result in any UL transmission or action at on transmisison or reception at UE. Therefore, one should not dismiss such a possiblity as Alt-1 and benefit from it when possible.

- Alt-2b: This alternative is seems to be one of the conditions in Alt-2a.

  - The question is should the operation be limited only to this case or could Alt-2b be merged with Alt-2a since it qualifies as one of the conditions?

- Alt-2c Option 1: 1-bit in group common DCI enables explcit COT determination (gNB initiated or not). This approach reduces complexity at gNB, and it is claimed not to add complexity at UE.

  - The question is whehter this option is intended for UE-specific signslling because it is not clear how 1-bit is incorporated for UE-specifc signalling in case Alt-2c, consdiering previous agreements. Also, what about boradcast signals?

  - It seems to Moderator that if this option is supported, the DL signal to other UEs could only be group-common signallign with 1-bit explcit COT determination in DCI. Is this a correct understanding?

- Alt-2d/Option 1 & Alt-2e: These two cases are similar. It is correct that DCI 2_0 can inidcate gNB initiated COT. However, in Rel-17 there could be a DL transmisison based on gNB-initiated COT and another DL transmisison witihn g-FFP that gNB is initiated its CO that is based on sharing UE-COT. Because the associating is determined per transmisison. Due to this issue, this solution while intends to provide flexibility, imposes restrcition and also creating depedency on DCI 2_0.

- Alt-2c/Alt-2d Option 2: These two cases are similar where Alt-2d is a special case of Alt-2c. These solution have the same issue as described above. In addiiton, it adds another restrcition that the DL transmisison at the beginning of e.g. g-FPP shoudl be always assumed as gNB-initiated COT. This eliminiates the possiblity of perfoming DL at the beginning of gNB-FPP based on sharign UE-iniitated COT where in some cases could be b2b and hence without LBT.

- Alt-2f: Details of the solution are not provided. It seems that specification impact could be rather considerable since one has toinvestigate how different DMRs encodig is done and would be same or different for different DL signals.

Moderator recommendation:

Based on the above analysis, it seems that Alt-1 and Alt-2a are the simplest approaches. However, Alt-2a needs to be formulated more clearly. The reason is that as it is formulated now, gNB should ensure UE-to-UE COT sharing is not occurred. However, this transmission should not be interpreted as a DL that shares gNB COT (whether gNB initiated the COT or not). Which means that because of this DL transmission, the UE for example would not skip reception of DL during following gNB idle period (may skip due to other reasons). Basically, what is important is that no channel access related actions are triggered because of reception of this DL.

Therefore, Alt-2a is modified as Alt-B below the following which covers both Alt-2a and Alt-2b in more general manner. Alt-A is the same as Alt-1.

Moderator’s recommendation is to focus on Alt-A and Alt-B.

- Alt-A: In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include only scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

- Alt-B: In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

  - A DL transmission to any other UE in the cell than the COT initiating UE and/or a broadcast transmission can be additionally included in the DL transmission burst if the gNB fulfills the follwong condition:

    - It is gNB responsibility to ensure that reception of the DL transmission or the broadcast transmission does not affect any channel access related assumption at UE for any UL or DL transmission.

## 2.3.1	Discussion – 1st round

Proposal 3-1:

Select one of the following alternatives:

- Alt-A: In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include only scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

  - Supported by: Intel, HW/HiSi, Ericsson, CATT, Apple (If Alt-2a(Alt-B?) is not supported), Len/MOT (1st prio)

- Alt-B: In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

  - A DL transmission to any other UE in the cell than the COT initiating UE and/or a broadcast transmission can be additionally included in the DL transmission burst if the gNB fulfills the follwong condition:

    - It is gNB responsibility to ensure that reception of the DL transmission or the broadcast transmission does not affect any channel access related assumption at UE for any UL or DL transmission.

  - Supported by: Ericsson, [Apple?, Nokia/NSB?, FW?, WILUS?, Sharp?, DCM?, LG?]

- Alt-2(a/b/c/d/e/f): In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, in addiiton to scheduled DL transmission or a DCI intended for the UE that initiated that FFP, can include DL transmission to any other UE in the cell than the COT initiating UE and/or broadcast transmission while ensuring that the COT initiated by the UE is not shared by any other UE in the cell for any UL transmission.

  - Alt-2a: It is gNB’s responsbility to ensure that COT initiated by the UE is not shared by any other UE in the cell for any UL transmission.

    - Supported by: Apple, Nokia/NSB, FW, WILUS, Sharp, DCM

  - Alt-2b: There is no UL resource allocated for any UE after the DL reception or there is only UL resource allocated for the COT-initiating UE after the DL reception.

    - Supported by: LG

  - Alt-2c: Using one of the following options:

    - Option 1: Explicit gNB-to-UE COT sharing indication in DCI.

      - gNB uses 1 bit to explicitly indicate to UE whether it has initiated a COT. The 1 bit can be carried in the GC-PDCCH or UE-specific DCI, etc.

    - Option 2: DL signal detection from dedicated positions.

      - Any DL signal detected at the specific position(s) indicates that gNB has initiated a COT. If signals are detected in these dedicated positions, UE assumes that gNB has initiated a COT, otherwise, UE assumes that gNB shares the UE-initiated COT

    - Supported by: vivo

  - Alt-2d: Using one of the following options:

    - Option 1: A UE assumes gNB has initiated a COT if the UE receives explicit indication in DCI 2_0.

    - Option 2: A UE assumes gNB has initiated a COT if the UE detects DL transmission at the beginning of gNB FFP.

    - Supported by: Samsung

  - Alt-2e: A group-common DCI at the beginning of a gNB-FFP can indicate whether the COT is a gNB-COT. UE assumes that the gNB has not acquired the COT if it does not detect the group-common DCI.

    - Supported by: Len/MOT (2nd prio)

  - Alt-2f: For a DL transmission based on sharing a UE intiated COT, other UEs can recognize that the gNB is sharing the UE COT through the gNB use of a different DMRS encoding. FFS details.

    - Supported by: MTK

| Questions: - Q1: Companies are kindly requested to provide any update/correction on the discussion and their positions with respect to all the alternatives (A/B/2a/2b/2c/2d/2e/2f) in Proposal 3-1 above. - Q2: Please review Moderator analysis and share your view (including agree/disagree or additional comments). - Q3: Please review Moderator recommendation. Do you agree that Alt-B covers Alt-2a and Alt-2b (specially proponets of these two alternatives)? Do you agree to focus on Alt-A and Alt-B or not? - Q4: Please share any other comments if any. |  |
| --- | --- |
| Company | Comment |
| Sony | We support Alt-B. Alt 2b limits the possible DL scheduling for other UE. The gNB sharing a COT from a 1st UE (that initiated the COT) can schedule a PDSCH to a 2nd UE where the PUCCH for the HARQ-ACK is scheduled to be transmitted after the end of the FFP of the 1st UE. In this case the gNB can still expect an UL from the 2nd UE but that UL transmission would not occur within the FFP of the 1st UE (that initiated the COT). |
| Intel | Between Alt. A and B, we prefer Alt.A. The main reason is that Alt.B: - may impose several restrictions in terms of scheduling capability to the gNB, while Alt.B could be rarely used under those restrictions. - may lead to additional considerations when discussing the ED threshould, which may complicate even further the design. As for other solutions, we would rather prefer at this stage not to make substantial changes to the framework that we have built so far and aim for simplicity. |
| Futurewei | We prefer Alt 2a or Alt B. Alt-A is a more restrictive option of Alt-2a, which we prefer. The same of Alt A vs Alt B, which is a reformulation of Alt-2a. Given that these alternatives in th essence have the same purpose (to make sure that do not mess up the UE COT ownership) , we could also compromise fo Alt A if the majority wants it. |
| DOCOMO | We originally support Alt-2a and also fine with Alt-B. As long as the reguration allows, no additional restriction is necessary. |
| Huawei, HiSilicon | Thanks for Moderator’s thorough analysis. Our 1st preference is Alt A. Based on our understanding, Alt-2b would be restrictive if not infeasible for configured UL. We also wonder how Alt B or Alt-2a would ensure that COT initiated by the UE is not shared by any other UE in the cell if a Rel-16 UE operates with semi-static channel access on the same channel and detects the DL burst within the gNB FFP. It seems that for Alt B/Alt-2a, gNB would in practice implement Alt-2b, or avoid configuring Rel-16 UEs to operate on a same channel/cell with Rel-17 UEs. If the latter understanding is correct, we are also open to Alt B. Similarly, for alternative Alt-2c through Alt-2f, we wonder how introducing a new indication, rule or procedure in Rel-17 would prevent UE-to-UE COT sharing through gNB if a Rel-16 UE operates with semi-static channel access on the same channel and detects the DL burst within the gNB FFP. |
| Apple | We are fine with either Alt A or B. But for Alt B, we are not sure why “DL transmission” is included in the following: “It is gNB responsibility to ensure that reception of the DL transmission or the broadcast transmission does not affect any channel access related assumption at UE for any UL or DL transmission.” |
| CATT | We are fine with both Alt A and Alt B. |
| vivo | Thanks moderator’s analysis for Alt-2c. About Alt-2c Option 1, the intention is the same as Alt-2d Option 1. UE can share gNB-initiated COT only when it receives the indication in the GC-DCI which indicates that gNB has initiated a COT. For all the other DL transmissions, they do not imply anything about the COT initiator, UE simply receives these transmissions. There is no restriction on the DL transmission. For progress, we can accept Alt-B. |
| Sharp | We are fine with Alt B. |
| Panasonic | Were are fine with both Alt.A and Alt.B. |
| Spreadtrum | We are fine with both Alt A and Alt B. |
| LG | Alt-B and Alt-2a seems too abstract or conceptual to be captured for gNB behavior in the spec as has been captured so far for unlicensed band case. Alt-2c/2d/2e/2f would require consensus and potentially have problem since legacy Rel-16 UE couldn’t understand the proposed signaling and rule. Alt-A could be reliable but seems too restrictive, so for this reason, we prefer Alt-2b to relax the restriction by keeping the operation reliability. |
| Nokia, NSB | We agree that there should be a single alternative (Alt-B) to cover both Alt-2a and Alt-2b. Alt-2c and Alt-2d are also essentially the same. Alt-2e is very similar to Alt-2c/Alt-2d Option 1. Alt-B could be a feasible option. Alt-2c/Alt-2d Option 1 and Alt-2e cannot work for legacy/idle UEs. Alt-2c/Alt-2d Option 2 can work with legacy/idle UEs but would still require a clarification of the legacy (Rel-16) UE behavior. Alt-2f may be feasible but increases Rel-17 UE complexity due to the need to perform blind detection of multiple DMRS sequences. “ Finally, it needs to be clarified how Alt-A can avoid UE-to-UE COT sharing as TS 37.213 specifies that “a UE may transmit UL transmission burst(s) after detection of a DL transmission burst(s) within the channel occupancy time”, i.e. any existing signal can be used for detection of the gNB COT. The most reliable option would be to have a Rel-16 CRs to change the assumption on the COT initiator as described in R1-2109138. However, we understand that this would be a rather late change. |
| Lenovo, Motorola Mobility | We are ok to focus on the FL’s recommendation. Wondering how Alt-B can be specified. |
| ZTE | Our preference is Alt 2d, among the 2 options, Option 1, i.e. explicit indication in DCI 2_0 would be better. We think Alt A is too restrictive. As for Alt B, although it seems simple, it’s not clear to us how could gNB ensure that reception of the DL transmission or the broadcast transmission does not affect any channel access related assumption at UE for any UL or DL transmission. We understand that gNB should also ensure that Rel-16 UEs can be prevented from receiving messages from gNB through a Rel-17 UE-initiated COT, this can be additional restriction to Alt 2d Option 1. |
| ETRI | From the regulation point of view Alt-A seems sufficient. However, in practice Alt-2 may help gNB to utilize the shared UE COT for DL transmissions. Thus, we slightly prefer Alt-2. For the sub-alternatives, we feel that it is difficult to prevent UE-to-UE COT sharing by purely relying on gNB implementation and some indication mechanism (either implicit or explicit) may be required. Thus, we prefer the approach in Alt-2c/2d/2e, and are also OK with Alt-B and Alt-A (as 2nd priority) if companies think it is feasible. |
| Samsung | For Q2, we think all alternatives have some level of restriction/limitations. However, especially for alt. 2d, we don’t think as much restriction as moderator analysis since there is no use case for gNB to indicate DCI format 2_0 when gNB uses COT that is initiated by a UE. For Q3, we would like to hear more views on what/how specification impact for Alt. B from proponents of Alt. B. Actually, coexistence issue raised by Huawei, we think that Alt-2d can applied to Rel-16 UE transparently since Rel-16 doesn’t support UE initiated COT. |
| Qualcomm | We prefer Alt-2c. |
| Moderator | Summary of views on Proposa 3-1: - Alt-A: - Supported by: Intel, HW/HiSi (1st), Ericsson, CATT, Apple, Len/MOT (1st prio), Pana, Spreadtrum, ETRI (2nd) - Alt-B: - Supported by: Ericsson, Sony, FW, DCM, HW/HiSI (2nd), Apple, CATT, Sharp, vivo, Pana, Spreadtrum, Nokia/NSB, Len/Mot, ETRI - Alt-2(a/b/c/d/e/f): - Alt-2a:. - Supported by: Apple, Nokia/NSB, FW, WILUS, Sharp, DCM - Alt-2b: - Supported by: LG - Alt-2c: - Supported by: vivo, ETRI, QC - Alt-2d: - Supported by: Samsung, ETRI - Alt-2e: - Supported by: Len/MOT (2nd prio), ETRI - Alt-2f: - Supported by: MTK @Intel: Myself, originall thought about restrictions, but based on input, I myself realized (based on companies’ inputs on variants of Alt-2) there are many cases that DL for other UEs could be transmitted that doesn’t trigger any action at Ue with respect to channel access. Since it is allowed by regulation, it is restrictive if we don’t allow. For example, if there is no UL configured or scheduled to other UEs in that gNB FFP or UE’s FFP, why cant we send broadcast or CG-PDCCH? So, Alt-B let us do that. In that case, it is not restrictions as gNB, quite contrary. I hope this clarification addresses your concern. @Íntel/HW/HiSi: On fair coexistence comment, I have been thinking a lot. The way I see the story is that this transmission is a piggyback without changing any channel access related principles. We had the same situation in LAA when I recall (adopted also for NR-U). We discussed also the case in ETSI BRAN and understood that is valid (hence covered by our specifications). When access the channel using an priority class (let’s say high priority) and get right for an MCOT, if your high priority data needs e.g. 1ms, one could also other types of traffic as long as doesn texceed from 1ms. I see kind of same analogy. For FBE, gNB get’s the right to use the channel. It cant fill it out with data only to Ue initiated. Can piggy-back other data and it is not going to give promotion to any other UEs to access the channel in one way or another. Based on the piggy-back data. So, it is legitimate. And in that regard, I started to think to special adjustment is also needed with respect to EDT (I will comment in next section). I hope you find the explanation reasonable. @Nokia/NSB: We should discuss the issues you raised in your contributon. I will capture that in next roud if it is OK. @All: There has been comments by (FW, DCM; LG, Nokia/NSB, Len/Mot, ZTE, Samsung) on clarity of Alt-B from regulatory aspects and specification aspects. Below some clairfcations, and update from Mdoerator to address the concern. - With respect to channel access, as you know we specify gNB behaviour due to regulation. Since LAA, there are cases that are covered in spec with statement such as “it is eNB (now gNB) responsibility to do bla bla.. Fro example in LBE, the choice of priority class for LBE is based on traffic (whehre t is best effort, etc). We have in RAN2 spec, how eNB is recommended to behave. That is as much 3gpp specifications can do. Then, there is aspect of testing, as well as regions that have explicit regulations. A manufacture either declare compliance or perform tesintg, etc. With that background, hopefully it is clear how much we need to clairfy in the proposal. Please note that the situation for UE behavour is different since 3gpp specs defines Ue behavior, so we have to be clear. - I have updated Alt-B. I hope it is clearer. Let’s use an example: We specify the behavior for CG. Alt-B means that when Ue wants to determine COT-ownership for a CG (if gNb initiated the Cot or UE, etc as we agreed), the DL transmission is not going to be involved in that procedure. That is the guarantee. Proposal 3-1 (updated): Select one of the following alternatives: - Alt-A: In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include only scheduled DL transmission or a DCI intended for the UE that initiated that FFP. - Alt-B: In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include scheduled DL transmission or a DCI intended for the UE that initiated that FFP. - A DL transmission to any other UE in the cell than the COT initiating UE and/or a broadcast transmission can be additionally included in the DL transmission burst if the gNB fulfills the follwong condition: - It is gNB responsibility to ensure that reception of “the DL transmission or the broadcast transmission” does not affect any channel access related assumptions at UE for any UL transmission or DL transmission reception. - That means that the reception of “the DL transmission or the broadcast transmission” does not affect any channel access related procedures (i.e. COT-ownership assumption, initiating a COT or sharing or using an initiated COT, sensing, skipping transmission/reception due to idle period of an initiated COT, etc.) for any UL transmission or a DL reception scheduled or configured to UE. @All: Based on the discussions and clarifcations above, is there any resistance to adopt Alt-B (specially companies who showed concern)? |

## 2.4	Energy detection threshold (EDT) adjustment

Last meeting, few companies motivated relaxation in EDT for UL in Rel-17 for semi-static channel access mode operation mode as compared to Rel-16.

In Rel-16, using gNB transmit power for adjusting the sensing threshold at UE when UE intends to share its channel occupancy with gNB was motivated by the power imbalance between gNB and UE, which may be harmful to other systems given that the gNB would transmit at a much higher transmit power than the UE.

During discussion, although companies largely shared the same view, there were some consideration for the case of gNB sharing the UE’s COT with transmission to other UEs as reflected in the Note below.

Agreement

- When a UE operates as an initiating device, and the gNB shares a UE’s FFP for DL transmission, regardless of the gap between any UL and DL bursts, no restriction is imposed on the maximum duration of each of the DL bursts such that each can continue until the UE FFP idle period starts.

    - Note: The applicability of the EDT calculation based on the UE’s transmit power to the UE COT initiation in accordance to the UL-DL gap duration and/or the content of the DL burst is separately discussed

Summary of views:

- Intel, DCM and Ericsson express that for semi-static channel access mode in a controlled environment, there is no issue with harming other systems and the UE should use its transmit power for sensing irrespective of sharing its channel occupancy or not. HW/HiSi also considers the sensing based on UE transmit power, however Moderator’s understanding is that such consideration is conditioned on support of Alt-A in previous section.

  - For semi-static channel access mode, UE transmit power is applied for EDT adjustment at UE.

- ZTE even suggestes that adjustment is not needed for ETD assuming long term absence of other thechnologies and ED threshold calculation is not relevant with gNB’s or UE’s transmit power.

  - For semi-static channel access mode, EDT adjustment is not considered.

- NEC further proposes that for high priority transmissions a higher energy detection threshold level that may be configured or even no sensing may be allowed to transmit immediately.

  - For semi-static channel access mode, EDT calculation based on transmission priority is supported by configuration.

Moderator analysis:

Discussion is needed whether the EDT adjustment based on UE tx power can be supported or not.

On ZTE proposal, although the proposal is reasonable and would be perhaps the typical operational mode, it is not clear the necessaity to restrict the specification to this case rather than keep it feature proof and its capability to address more use cases.

On NEC proposal, it is not clear why different EDT are needed while improving the high priority transmission can be controlled by increasing power satisfying regulations.

## 2.4.1	Discussion – 1st round

Proposal 4-1:

Select one of the following alternatives:

- Alt-1: In semi-static channel access mode, for adjustment of ED threshold to perform sensing at UE, UE transmit power is applied.

  - Supported by: Intel, DCM, Ericsson

- Alt-2: For semi-static channel access mode, EDT adjustment is not considered.

  - Supported by: ZTE

Proposal 4-2:

- For semi-static channel access mode, EDT calculation based on transmission priority is supported by configuration.

  - Supported by: NEC

| Questions: - Q1: Companies are kindly requested to provide any update/correction on the discussion and their positions with respect to Proposals 4-1 and 4-2. - Q2: Please review Moderator analysis and share your view (including agree/disagree or additional comments). - Q3: Please share any other comment if any. |  |
| --- | --- |
| Company | Comment |
| Sony | We support Alt-1 of Proposal 4-1. |
| Intel | We prefer Alt.1, and as explained in the past we consider this as a very important issue. Alt-1 would offer a relaxation of a constrain that was imposed in Rel.16 only as a compromised solution with the Wi-Fi’s vendors to enable the UL-to-DL COT sharing procedure, which otherwise would not have been agreed. We would like to highlight to ZTE that with this constrain the UE is limited to perform a CCA with a much lower noise threshould when acquiring a FFP or attempting to transmit within a g-FFP, while in reality this contrain is not even mandated by the ETSI BRAN. As for proposal 4-2, we do not support it, since we do not understand the technical motivation behind it. |
| Futurewei | We prefer Alt 1, of the Proposal 4-1; we think we should follow the regulator constraints EN 301 893 We do not see the reasons for supporting Proposal 4-2. |
| DOCOMO | We support Alt-1 for Proposal 4-1. We don’t see the necessity to support Proposal 4-2 |
| Huawei, HiSilicon | Moderator’s understanding of our position is correct. Our support of Alt-1 of Proposal 4-1 is conditioned upon supporting Alt-A in the previous section. Otherwise, there would be a concern that the gNB would often rely on sharing a UE-initiated CO (due to higher EDT) especially without LBT, to transmit user plane data to other UEs. In fact, resolving the EDT adjustment issue is another motivation for Alt-A in the previous section. |
| Apple | We support Alt-1 of P4-1, subject to the following clarification. In R16, the EDT determination based on gNB’s transmit power only applies when absenceOfAnyOtherTechnology-r16 is not configured and ul-toDL-COT-SharingED-Threshold-r16 is configured. In all other cases, the UE calculates EDT based on its own transmit power. Does P4-1 mean that the UE calculates EDT based on its own transmit power in all cases? If so, can we formulate the proposal in this way? It is a bit unclear what “adjustment” refers to in Alt-1/Alt-2. Alt-2 is not very clear to us what “EDT adjustment is not considered” means. How does the UE calculates EDT with Alt-2? We do not see the need for P4-2. |
| CATT | We support Alt-1 for Proposal 4-1. We do not see the need to support Proposal 4-2 |
| vivo | We support Alt.1 of Proposal 4-1; Proposal 4-2 is not necessary for controlled environment. |
| Sharp | Proposal 4-1: We prefer Alt 1. Proposal 4-2: The nessisity is not clear. |
| Spreadtrum | We support Alt.1 of Proposal 4-1. |
| LG | Before discussing and choosing the alternatives, we have a question for clarification on Proposal 4-1. Are Alt-1 and Alt-2 proposing different things? (or) Are those proposing the same? |
| Nokia, NSB | In proposal 4-1, it is not clear what Alt-2 means, and what is the difference between Alt-1 and Alt-2. Nokia is ok to support Alt-1, although the use of semi-static channel access mode does not guarantee absence of other systems operating on the same unlicensed channel. Semi-static channel access mode is optimized for scenarios with no inter-system interference, but co-existence with other systems still needs to be guaranteed. We are not supportive of Proposal 4-2. |
| Lenovo, Motorola Mobility | Ok with Alt-1 in a controlled environment. We are also fine to have EDT adjusted only if the RRC parameter absenceOfAnyOtherTechnology is not configured. |
| ZTE | We support Alt 2 of the Proposal 4-1. But before we try make a decision, we think it’s better to first align our understanding of current spec. In TS 37.213, section 4.3, it describes the intended environments of FBE mode, i.e. for environments where the absence of other technologies is guaranteed. Then in section 4.1.5 and 4.3.2, the ED threshold calculation has 2 branches and it will go into the first branch when the absence of any other technology can be guaranteed, which in our understanding is the same environment as described for FBE mode in section 4.3. In this branch, the ED threshold calculation is not related to either UE’s or gNB’s transmit power, it only depends on the regulation requirements and channel bandwidth, that’s why we think there is no need to consider EDT adjustment for FBE. It would be much appreciated if other companies could indicate which part we have misunderstanding, thanks. 4.3 Channel access procedures for semi-static channel occupancy Channel assess procedures based on semi-static channel occupancy as described in this Clause, are intended for environments where the absence of other technologies is guaranteed e.g., by level of regulations, private premises policies, etc. 4.1.5 Energy detection threshold adaptation procedures An eNB/gNB accessing a channel on which transmission(s) are performed, shall set the energy detection threshold () to be less than or equal to the maximum energy detection threshold . is determined as follows: - If the absence of any other technology sharing the channel can be guaranteed on a long-term basis (e.g. by level of regulation) then: - - is maximum energy detection threshold defined by regulatory requirements in dBm when such requirements are defined, otherwise ; - otherwise, - where: - =5dB for transmissions including discovery burst(s) as described in clause 4.1.2, and otherwise; - ; - is the set maximum eNB/gNB output power in dBm for the channel; - eNB/gNB uses the set maximum transmission power over a single channel irrespective of whether single channel or multi-channel transmission is employed - ; - is the single channel bandwidth in MHz. #### 4.2.3.1 Default maximum energy detection threshold computation procedure If the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided - where - is Maximum energy detection threshold defined by regulatory requirements in dBm when such requirements are defined, otherwise otherwise - where - ; - ; - is the set to the value of PCMAX_H,c as defined in [3]; - ; - is the single channel bandwidth in MHz. |
| ETRI | We support Alt-1 for Proposal 4-1. |
| Samsung | We are fine with Alt-1 for P4-1. For the question by Apple, our understanding is that the proposal is applied when 1) absenceOfAnyOtherTechnology-r16 is configured and 2) ul-toDL-COT-SharingED-Threshold-r16 is configured. However, we are open to have further discussion on possible specification impact. |
| Qualcomm | We prefer Alt-1 for Proposal 4-1. |

| NEC | We support Proposal 4-2. UE power saving is an important aspect in 5G applications. Furthermore, the issue cannot be solved by increasing the power of high priority transmission alone, since it would have been blocked by low priority UL configured grant transmissions. Proposal 4-2 is also needed for controlled environment. For example, in a factory with automation, the communication with goods delivery vehicle may be of high priority while the packaging arms on goods conveyor may be transmitting via low priority configured grant. |
| --- | --- |
| Moderator | Summary of views on Proposal 4-1: - Alt-1: - Supported by: Intel, DCM, Ericsson, Sony, FW, Apple, CATT, vivo, Sharp,Nokia/NSB, Len/Mot, ETRI, Samsung - Note: HW/HiSi concern - Alt-2: - Supported by: ZTE Summary of view on Proposal 4-2: - For semi-static channel access mode, EDT calculation based on transmission priority is supported by configuration. - Supported by: NEC - Not support: Intel, FW, DCM, Apple, CATT, vivo, Sharp, Nokia/NSB @NEC: Based on inputs, Moderator’s recommendation is that not to consider Proposal 4-2 for the rest of discussion. However, considering your comment, the question is whether to ensure URLLC application, is there a need to configure different threshold for low priority and high priority? @HW/HiSi: Considering the discussion/comments in previous section, and considering that in Typical FBE scenarios, coexistence is not an issue, do you think it is really needed to still carry the constraint for FBE, similarly to LBE? I understand your concern, but I am wondering if it is a critical issue that should really be reflected in design. @ZTE: Your understanding is correct in identifying the tracks if absenceOfAnyOtherTechnology is configured. However, mandating only this mode of behaviour, as it appears to be simplied, put 3GPP technologies at disadvatange as Intel explained since there are forced unnessarily to operate as a threahold, without being able to be adjust it. Also, for FBE, although the recommended deployment scenario is for controlled enviroements, but as Nokia commented in previous section, it is good to not restrict the design. In other words, there is no need to get rid of the flexibility that is already supported and can be useful Also, considering the comments, I hope it is OK with you if we consider Alt-1 for the rest of discussions. @All: There has been questions what is meant by adjustment of EDT. Moderator’s understanding is that the raised by companies issue is due to the behaviour that is defined in Rel-16. Please check the following in 37.213 and 38.331. In short, as Samsung explained , for a set of conditions (configurations), when gNB shares UE COT (please note that in Rel-16 this behaviour is only defined for LBE) the gNB has to consider its own power than UE power to determine a threshold, to configure the UE with. The proposal, if I understood correctly, relaxes this situation for FBE in Rel-17 when gNB shares UE COT. I copy below the relevant text in the spec that you can trace the behaviour. Please see updated proposal to reflect the concern. ### 4.2.3 Energy detection threshold adaptation procedure .... If the higher layer parameter absenceOfAnyOtherTechnology-r16 is not configured to a UE, and the higher layer parameter ul-toDL-COT-SharingED-Threshold-r16 is configured to the UE, the gNB should use the gNB's transmit power in determining the resulting energy detection threshold ul-toDL-COT-SharingED-Threshold-r16. ….. @All: Some thoughts and reflections. Please share your view: Based on your comments, I am wondering maybe the right question to ask is the following: - COT sharing in FBE is different from LBE. - For FBE, we don’t have the constraint on duration (except by FFP), SCS, possibly content of transmission (pending discussion on section 2.3) as in LBE. Also, the main point of Alt-1 is that the EDT can also be relaxed. - So maybe we need to ask the question that do we need ul-toDL-COT-SharingED-Threshold, or maxEnergyDetectionThreshold is enough? - Please note that configuration of ul-toDL-COT-SharingED-Threshold instead of maxEnergyDetectionThreshold is when COT sharing is applicable in LBE, and some constraint are considered due to coexistence with WiFi. - In the update of spec for Rel-17, I plan to have separate section for FBE, also for COT sharing. - In other words, we have a lot of baggage from LBE that they don’t make sense for FBE. Isnt better to simplify the operation nd configuration? - The same case when CG-UCI indicates to gNB how long gNB can share the COT, etc (X, D, O). Those parameters are not applicable to FBE. Let’s discuss that in section 2.5. @Moderator’s recommendation: - Could we not consider Proposal 4-2? - Please reflect and share your view on “Some thoughts and reflections” above. - Can we focus on Focus the discussion on Alt-1a in updated Proposal 4-1 and skip Alt-1 and Alt-2? - If yes, can you please share your view if Alt-1a is accurate enough to address the intention? Proposal 4-1 (updated): Select one of the following alternatives: - Alt-1: In semi-static channel access mode, for adjustment of ED threshold to perform sensing at UE, UE transmit power is applied. - Alt- 1a: In semi-static channel access mode, the configuration of energy detection threshold to perform sensing at UE is based on maxEnergyDetectionThreshold. - That means that in semi-static channel access mode, configuration of ul-toDL-COT-SharingED-Threshold is not applicable. - Alt-2: For semi-static channel access mode, EDT adjustment is not considered. |

## 2.5	Enhancements impacting RRC

In the following, list of proposals that have RRC impact are considered. The proposals are categorized but the detailed descriptions and motivations can be found in the respective contributions of the supportive companies. Please share your view for the proposals in each category A, B, C, D, E.

## 2.5A Configuration aspects of CG Harmonization

Proposal 5A-1:

Support configuration of harq-ProcID-Offset2 for operation in unlicensed spectrum when the cg-RetransmissionTimer-r16 is not configured.

- Supported by: HW/HiSi

Proposal 5A-2:

When the cg-RetransmissionTimer is enabled, DCI 0_2 should be enhanced to carry the DFI information based on configuration.

- Supported by: Intel

Proposal 5A-3:

A same CG type (e.g., Rel-16 NR-U CG type or Rel-16 URLLC CG type) is configured per cell.

Independently on whether cg-RetransmissionTimer is enabled or disabled, multi-TB transmission should be supported to fully utilize the MCOT available.

- Supported by: Intel

Proposal 5A-4:

If useInterlacePUCCH-PUSCH is configured, the cancellation indication carried in DCI format 2_4 is applied to an entire interlace, if at least one PRB of that interlace belongs to the indicated timeFrequencyRegion for that cancelation indication.

- Supported by: Intel

## 2.5A.1	Discussion – 1st round

| Question: Please share your view on Proposals 5A-1, 5A-2, 5A-3, 5A-4. |  |
| --- | --- |
| Company | Comment |
| Sony | We support Proposal 5A-1. |
| Intel | For Proposal 5A-1, this seems reasonable to us. For Proposal 5A-2, as proponent of this proposal, we believe that as discussed during prior meeting, it would be highly beneficial to enable unlicensed features using the compact DCIs. In particular, we believe that it could be beneficial to carry the DFI information (in a configurable manner, such that this field is not always carried) within the compact DCI 0_2 so that to use this neawly defined DCI which has been introduced to meet URLLC reliability requirements to carry HARQ information related to CG-UEs. For Proposal 5A-3, as proponent of this proposal, we believe that this has not been well captured by the FL, and we have replaced the FL’s proposal with our original text. Our view is that we should enable multi-TB transmission even when the cg-retransmassionTimer is disabled. The main reason is that through this enhancement, which was been supported in all unlicensed designs so far, a UE is allowed to fully utilize a FFP by perfoming longer consecutive transmission bursts, which would greatly minimize the LBT overhead. As for proposal 5A-4, we would like to highlight that the cancellation indication procedure introduced in Rel.16 has been designed having in mind the resources allocation type 1 (contiguous PRB allocation), and if the gNB may configure useInterlacePUCCH-PUSCH, for which an interlaced-based resource allocarion may be used, the cancellation of an transmission occurring over an interlace may have the following drawbacks: - a gNB may need to provide multiple cancellation indications indicating each a group of contiguous PRBs (which in some cases may require a number of bits larger than what DCI 2_4 can support) leading to an excessive overhead; - a gNB could potentially use a single cancellation indication by referring to the entire system BW over which the interlace spans. However, this would lead to cancelling all frequency domain resources in between, which means all other interlaces which may belong to other UEs, where those are performing specific transmissions. As understandable, this would lead to a very inefficient spectrum utilization. In order to mitigate the issues mentioned above, if the interlace-based resource allocation is used, a cancellation indication carried in DCI format 2_4 could be applied to an entire interlace, if at least one PRB of that interlace belongs to the indicated timeFrequencyRegion for that cancelation indication, without the need to indicate individually all the PRBs that belong to an interlace. We believe that this is quite a strainghtforward way to generalize the cancellation indication introduced in Rel.16 to the case interlace-based resource allocation is used with very little specification impact. |
| Futurewei | Suppport the Proposal 5A-1 |
| DOCOMO | We support proposal 5A-1 |
| Huawei, HiSilicon | We support Proposal 5A-1 Proposal 5A-2, we have a concern that including the DFI flag for CG operation in unlicensed would result in a reserved bit in the compact DCI 0_2 scrambled with the other RNTIs, i.e., similar to the following spec for DCI 0_1 “For a DCI format 0_1 with CRC scrambled by C-RNTI/SP-CSI-RNTI/MCS-C-RNTI and for operation in a cell with shared spectrum channel access, the bit is reserved.” For Proposal 5A-3, we think that the benefit of transmitting multiple TBs in a period can be offset by configuring multiple CGs. |
| Apple | We support P5A-1. We do not see the need/motivation for P5A-2, which seems unrelated to any of the enhancements we have discussed so far. For Intel’s P5A-3, our understanding of the previous agreement (though not formulated this way) is that if cg-RetransmissionTimer is not configured, we fall back to CG operations on licensed spectrum without any further enhancements. Even though we previously support multi-TB enhancements, we prefer not to reopen the discussion. For P5A-4, we had a previous proposal to enhance the UL CI to be based on interlace instead of PRB. If we want to enhance the UL CI operation, we prefer to go with a more complete solution, which is not complicated either. |
| CATT | We support Proposal 5A-1. |
| vivo | We support Proposal 5A-1; We support Proposal 5A-2; For Proposal 5A-3, we do not support this proposal since it was already agreed that no enhancement for PUSCH repetition Type B; Multi-TB transmission is supported when cg-RetransmissionTimer is enabled so that the CG-UCI carrying the NDI, HPN can inform gNB about the TBs. Without CG-UCI, i.e., cg-RetransmissionTimer is not configured, gNB has no idea about how many different TBs are transmitted by UE. For Proposal 5A-4, it seems not necessary. From our understanding, for UL CI, current spec defines that even if only part(s) resource of the UL transmission belongs to the indicated timeFrequencyRegion cancelation, the whole PUSCH transmission is dropped. The consequence is the same as what Proposal 5A-4 wants to achieve. |
| Panasonic | We suoport Proposal 5A-1. |
| Spreadtrum | We support Proposal 5A-1; We are open to disucss Proposal 5A-2. One question is whether there could be some cases that size of DCI 0_2 cannot be enough for DFI? We do not support Proposal 5A-3. It has concluded that when cg-RetransmissionTimer is enabled, URLLC type CG-PUSCH is used. So only one TB for a CG configuration in a period. We are open to disucss Proposal 5A-4, maybe UL CI enhancement for interlaced structure can be agreed first. |
| LG | Regarding Proposal 5A-4, we think it needs more considerations including other approach, for example, use of PRB interlace-level bitmap rather than legacy RB-level bitmap, in order to well fit with UL resource allocation. |
| Nokia, NSB | Proposal 5A-1: Support. Proposal 5A-2: Support. Proposal 5A-3: Do not support. We prefer to avoid unnecessary mixing of unlicensed and licensed features. Proposal 5A-4: Do not support. We see this as an unnecessary optimization, and the existing functionality seems good enough. |
| Lenovo, Motorola Mobility | 5A-2: ok 5A-4: agree, wondering if any specification change needed? |
| ZTE | For proposal 5A-1, we think it is reasonable since the URLLC feature including the configuration of HARQ process ID offset 2 is applied when the CGretransmissiontimer-r16 is not configured. For proposal 5A-2 and 5A-4, we don’t think they are in the scope of this WID. For proposal 5A-3, to follow the previous agreement, multi-TB transmission can be supported only when the cg-RetransmissionTimer is enabled. |
| Qualcomm | Support proposal 5A-1. |
| Moderator | Summary of views: Proposal 5A-1: - Supported by: HW/HiSi, Sony, Intel, FW, DCM, Apple, CATT, vivo, Pana, Spreadtrum, Nokia/NSB, ZTE, QC Proposal 5A-2: - Supported by: Intel, vivo, Nokia/NSB, Len/MOT - No need: HW/HiSi, Apple, ZTE - FFS: Spreadtrum Proposal 5A-3: - Supported by: Intel - No need: HW/HiSi, Apple, vivo, Spreadtrum, ZTE Proposal 5A-4: - Supported by: Intel - No need: Apple (or more complete solution), vivo, ZTE - FFS: Spreadtrum, LG? @Intel: Thanks for correction @All: Moderator makes the following observation based on the inputs. - On PA-1: It seems Proposal 5A-1 is supported by all. - On P5A-2, the views are split for this proposal. Moderator’s understanding is that to extend the field for DFI in DCI 0_1 to DCI 0_2. Is that correct understanding? - On PA-3: Lacks support. - On PA-4: Lacks support with two companies showed interest. However it seems it is not needed as vivo explained. ========================================= @Moderator recommendation: - Support Proposal 5A-1. Suggest to Chair for approval in GTW/Email. - Continue disucsison on P5A-2. - Drop 5A-3 & P5A-4. - Needless to say, propoenets are free to continue discussion to promote the proposal. However, Moderator rather not to drive the discussion as these proposals seem to be preferred by majority to be down-priorotized. - If proponets are interested to continue discussion, please let the moderator know to arrange a place holder in the summary document. Proposal 5A-1: Support configuration of harq-ProcID-Offset2 for operation in unlicensed spectrum when the cg-RetransmissionTimer-r16 is not configured. Proposal 5A-2: When the cg-RetransmissionTimer is enabled, DCI 0_2 should be enhanced to carry the DFI information based on configuration. |

## 2.5B	CG configuration and cg-RetransmissionTimer

Proposal 5B-1:

- When the cg-RetransmissionTimer-r16 is enabled and a UE operates as an initiating device, the COT sharing information field which is included within the CG-UCI indicates only the length of the shared resources and an offset, which indicates the exact starting symbol from when the gNB may be able to use those resources regardless of the slot boundary.

- When the cg-RetransmissionTimer-r16 is disabled and a CG UE operates as an initiating device, the same procedure established for DG UEs in Rel.16 is reused.

- Supported by: Intel

Proposal 5B-2:

- The COT initiator information should be included in the CG UL transmission.

- Supported by: vivo, Intel

Proposal 5B-3:

The following RRC parameters are NOT needed when cg-RetransmissionTimer is configured for CG operation with shared spectrum channel access.

- pusch-RepTypeIndicator

- startingFromRV0

- The RRC parameter of phy-PriorityIndex should be applicable for CG operation in unlicensed band.

- Supported by: vivo

## 2.5B.1	Discussion – 1st round

| Question: Please share your view on Proposals 5B-1, 5B-2, 5B-3. |  |
| --- | --- |
| Company | Comment |
| Intel | For proposal 5B-1, we support this proposal, as proponent company. The general idea is to: - Allow the UE to provide the COT sharing information to the gNB when cg-RetransmissionTimer-r16 is enabled, and the cg-UCI is carried. In this case, it is important to recall that cg-COT-SharingList-r16 includes an entry indicating the CAPC which is no longer needed for FBE: cg-COT-SharingList-r16 SEQUENCE (SIZE (1..1709)) OF CG-COT-Sharing-r16 CG-COT-Sharing-r16 ::= CHOICE { noCOT-Sharing-r16 NULL, cot-Sharing-r16 SEQUENCE { duration-r16 INTEGER (1.. 39), offset-r16 INTEGER (1.. 39), channelAccessPriority-r16 INTEGER (1..4) } } Also, the offset information was defined so that it would always link to the first symbol of the first slot that could be used for DL transmission irrespective of when the UL transmission ends. This would always contrain the network from operating with no LBT, since it would require an additional LBT if the UL burst does not end at the slot boundary. For this reason, we would like to propose the following RRC parameter to carry the COT sharing information cg-COT-SharingList-r17 SEQUENCE (SIZE (1..Z)) OF CG-COT-Sharing-r17 CG-COT-Sharing-r17 ::= CHOICE { noCOT-Sharing-r17 NULL, cot-Sharing-r17 SEQUENCE { duration-r17 INTEGER (1.. X), offset-r17 INTEGER (1.. X), } } Where the value of X and Z could be defined based on the definition of offset that RAN1 would decide to pursue. - Given that when cg-RetransmissionTimer-r16 is disabled, the cg-UCI is not transmitted, the straightforward way to still support UL-to-DL COT sharing is to reuse the Rel.16 DG behavior. In other terms, for the case the cg-UCI is not carried, it will be left up to gNB to utilize the unused resources within a u-FFP, which will be only allowed to transmit within the time-domain resources that are not configured. As for proposal 5B-2, we would suggest to rephrased it as follows: Proposal 5B-2: - When cg-RetransmissionTimer-r16 is enabled, Tthe COT initiator information should be included within the cg-UCI which is piggybacked in each of the CG UL transmissions. For Proposal 5B-3, we are OK in principle with the proposal, even though the text could be further refined. Perhaps, the last bullet could be left out of proposal 5B-3, and could be a separate one: Proposal 5B-3: The following RRC parameters are NOT needed when cg-RetransmissionTimer is configured for CG operation with shared spectrum channel access. - pusch-RepTypeIndicator - startingFromRV0 - The RRC parameter of phy-PriorityIndex should be applicable for CG operation in unlicensed band. Proposal 5B-4: The RRC parameter of phy-PriorityIndex should be applicable for CG operation in unlicensed band. |
| Futurewei | We support 5B-2. |
| DOCOMO | We don’t agree with Proposal 5B-1. cg-RetransmissionTimer-r16 can be enabled/disabled not only for FBE mode but also for LBE mode, where CAPC should be indicated to gNB. |
| Huawei, HiSilicon | Proposal 5B-1 seems to be an enhancement to NR-U CG in unlicensed rather than a harmonization of URLLC CG and NR-U CG features. Proposal 5B-2 does not resolve the misalignement issue between the UE and gNB in general case of configured UL with semi-static channel access since it is only specific to CG and only when CG-ReTxTimer is configured. We are OK with Proposals 5B-3 and 5B-4 |
| Apple | On P5B-2, it is not exact clear to us whether there is a need to indicate the COT initiator. If the gNB had initiated the COT, it can always use its own COT for DL transmission; otherwise it is clear that the UE had initiated its own COT. We wonder if there is any undesirable consequence if the COT initiator is not included in CG-UCI. On P5B-3, the first two sub-bullets look reasonable. On phy-PriorityIndex, it is probably a more generic question. If we think the two PHY priorities in R16 also apply on unlicensed spectrum, there is no need to separately mention this parameter for CG. |
| vivo | For Proposal 5B-1, we are fine with deleting the IE channelAccessPriority-r16 since it is not necessary for FBE. Optimizations for other IEs like duration and offset can be discussed if time allows. For Proposal 5B-2, we support it. For Proposal 5B-3, we support it and we are fine with Intel’s modification. |
| Nokia, NSB | Proposal 5B-1: ok to remove channelAccessPriority-r16 for semi-static channel access, although this is not critical. Other changes are less needed. Proposal 5B-2: agree with Apple: we are unsure why this information would be needed in the CG-UCI. Proposal 5B-3: agree with the updated proposal by Intel |
| Lenovo, Motorola Mobility | 5B-1: - good to clarify what is meant by “the same procedure established for DG UEs in Rel.16 is reused.” - Suggest keeping “which indicates the exact starting symbol from when the gNB may be able to use those resources regardless of the slot boundary.” FFS 5B-2: if the motivation is to avoid misalignment between gNB and UE on COT initiator assumption, we suggest to first conclude the discussion point 2.3. Besides, considering configured UL transmission can include e.g. SRS, wondering if the proposal is applicable to all configured UL transmissions. |
| ZTE | For proposal 5B-2, we support it in order to let the gNB and UE have the same understanding on the UE COT information. For proposal 5B-3, We support it. |
| Moderator | Summary of view: Proposal 5B-1: - Supported by: Intel - only channelAccessPriority-r16 - Vivo, Nokia/NSB - No need: DCM, HW/HiSi Proposal 5B-2: - Supported by: vivo, Intel, FW, ZTE - No need: HW/HiSi, Apple, Nokia/NSB Proposal 5B-3:. - Supported by: vivo, Apple, HW/HiSi, ZTE, Nokia/NSB Proposal 5B-4:. - Supported by: vivo, Apple, HW/HiSi, ZTE, Nokia/NSB ================================= @Moderator recommendation: - Support Proposal 5B-3 and P5B-4. Suggest to Chair for approval in GTW/Email. - Support Proposal 5B-1 for channelAccessPriority-r16 after reviewing updated wording. - Continue disucsison on other IE in P5A-1 and P5B-2. Proponents to address the concerns raised. Proposal 5B-1 (updated): The following RRC parameters is NOT needed when UE is confiured to operate with semi-static channel access mode. - channelAccessPriority Proposal 5B-3: The following RRC parameters are NOT needed when cg-RetransmissionTimer is configured for CG operation with shared spectrum channel access. - pusch-RepTypeIndicator - startingFromRV0 Proposal 5B-4: The RRC parameter of phy-PriorityIndex should be applicable for CG operation in unlicensed band. |

## 2.5C CG-UCI multiplexing and high priority

Few companies indicated that it is necessary to harmonize the parameter of cg-UCI-Multiplexing for CG by taking into account Rel-16 and Rel-17 intra-UE prioritization/multiplexing mechanism. Below, a set of proposals are listed (with potential overlap between them) that are needed to be discussed to identify proper actions.

Proposal 5C-1:

Clarify further whether phy-PriorityIndex can be configured simultaneously with cg-retransmissionTimer.

- Supported by: Nokia/NSB

Proposal 5C-2:

When cg-RetransmissionTimer is configured and Rel-16 intra-UE prioritization/multiplexing mechanism is used,

- if there is resource overlapping between the PUCCH carrying HARQ-ACK and CG-PUSCH with different L1 priorities, regardless whether cg-UCI-Multiplexing is provided or not, UE transmits the channel (either PUCCH or CG-PUSCH) with high priority and does not transmit the channel with low priority;

- if there is resource overlapping between the PUCCH carrying HARQ-ACK and CG-PUSCH with the same L1 priority,

- if the UE is provided cg-UCI-Multiplexing, CG-UCI and HARQ-ACK are jointly encoded; otherwise, the UE does not transmit the PUSCH and multiplexes the HARQ-ACK information in a PUCCH transmission or in another PUSCH transmission.

- Supported by: vivo

Proposal 5C-3:

When cg-RetransmissionTimer is configured and Rel-17 intra-UE prioritization/multiplexing mechanism is used, that is multiplexing UCI in a PUSCH with different L1 priorities,

- if there is resource overlapping between the PUCCH carrying HARQ-ACK and CG-PUSCH with different L1 priorities, following options can be considered to re-interpret cg-UCI-Multiplexing:

- Option 1: If the UE is provided cg-UCI-Multiplexing, CG-UCI and HARQ-ACK are jointly encoded; otherwise, the UE transmits the channel (either PUCCH or CG-PUSCH) with high priority and does not transmit the channel with low priority.

- Option 2: Regardless whether the UE is provided cg-UCI-Multiplexing, CG-UCI and HARQ-ACK are separately encoded and multiplexed in CG-PUSCH.

- Supported by: vivo

Proposal 5C-4:

When the cg-RetransmissionTimer is enabled, if both HP and LP HARQ-ACK are to be multiplexed onto a CG-PUSCH that includes CG-UCI, CG-UCI is jointly encoded with HP HARQ-ACK with same beta offset.

- Supported by: Intel

## 2.5C.1	Discussion – 1st round

| Question: Please share your view on Proposals 5C-1, 5C-2, 5C-3, 5C-4. |  |
| --- | --- |
| Company | Comment |
| Sony | Proposal 5C-1: It is good to clarify this and we think that L1 priority and cg-RetransmissionTimer can be configured simultaneously. Proposal 5C-2: - Agreed with the 1st sub-bullet where we have different L1 priorities. - The 2nd and 3rd sub-bullet on same L1 priorities this should follow legacy behaviour Proposal 5C-3 and 5C-4: Option 2 of Proposal 5C-3 seems to be the same as Proposal 5C-4. Since we agreed to have separate encoding for LP & HP UCIs, then we would expect the CG-UCI and HARQ-ACK of different priorities to be separately encoded, i.e. Option 1 of Proposal 5C-3. |
| Intel | For Proposal 5C-1, our understanding is that it has been already agreed in the intra-UE AI that phy-PriorityIndex can be configured simultaneously with cg-retransmissionTimer. In fact, the following was agreed: Agreements: Support multiplexing for following scenarios in R17: - Multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). - Multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only) - Multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. - Multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. For the above multiplexing scenarios, - Support separate configurations of at least beta-offset values (FFS for alpha) for multiplexing with different priority combinations. - FFS for other separate configurations. - FFS: value range of beta-offset (e.g. <1). - FFS the conditions, if needed, for multiplexing, e.g. - FFS: Whether to support multiplexing in case a PUCCH/PUSCH overlaps with more than one PUCCH/PUSCH. - Timeline requirements. - FFS: details, if needed, of the multiplexing scheme, e.g. - How to minimize impact on the latency for high-priority HARQ-ACK. - How to multiplex the HARQ-ACK bits (e.g. multiplexing, bundling)? - How to encode the UCIs with different priorities (e.g. separate coding vs. joint coding). - How to guarantee the target code rate (e.g. payload control, multiplexing priority, LP HARQ-ACK compression/compaction). - Explicit indication for multiplexing. - Multiplexing rule and order (e.g. HP/LP multiplexing is after resolving collision within the same priority). How to handle multiplexing of UCI of different priorities and CG-UCI in a CG-PUSCH As for proposal 5C-2/5C-3, our initial view is that Rel.16 NR-U multiplexing rules could be used as a baseline. In case a PUCCH overlaps with a CG-PUSCH, regardless of whether a PUCCH or cg_PUSCH may have different priority, the NR-U multiplexing rules could be used, and dropping should be avoided. However, further discussion may be needed, and we are open to discuss further. As for proposal 5C-4, we support this proposal, as proponent company. |
| DOCOMO | We are open to further discuss these proposals. |
| Huawei, HiSilicon | We are OK wioth Proposal 5C-1. The behavior decribed in Proposal 5C-2 can be considered as a starting point. We are open to further discuss Proposals 5C-3 and 5C-4. |
| Apple | Just to confirm, is it the common understanding that R16 intra-UE prioritization/multiplexing does not apply on unlicensed spectrum in R16? So the proposals here are how to enable R16/R17 intra-UE prioritization/multiplexing for unlicensed spectrum? We are open to the discussion, but we need some more time to check the proposals. |
| vivo | If one or some of Proposal 5C-2 ~ 5C-4 can be agreed in this meeting, Proposal 5C-1 is not needed. We support Proposal 5C-2. Proposal 5C-2 is for the case that Rel-16 intra-UE prioritization/multiplexing mechanism is used, 1st bullet is to protect the channel with high priority when channels with different prioritiese are collided, it follows legacy Rel-16 URLLC behavior; the 2nd and 3rd sub-bullet are for the same L1 priorities, and it follows legacy Rel-16 NR-U behavior. Note the last bullet should be the subbullet of the 2nd bullet. Proposal 5C-2: When cg-RetransmissionTimer is configured and Rel-16 intra-UE prioritization/multiplexing mechanism is used, - if there is resource overlapping between the PUCCH carrying HARQ-ACK and CG-PUSCH with different L1 priorities, regardless whether cg-UCI-Multiplexing is provided or not, UE transmits the channel (either PUCCH or CG-PUSCH) with high priority and does not transmit the channel with low priority; - if there is resource overlapping between the PUCCH carrying HARQ-ACK and CG-PUSCH with the same L1 priority, - if the UE is provided cg-UCI-Multiplexing, CG-UCI and HARQ-ACK are jointly encoded; otherwise, the UE does not transmit the PUSCH and multiplexes the HARQ-ACK information in a PUCCH transmission or in another PUSCH transmission. We support Proposal 5C-3. Note that Proposal 5C-3 is for the case that there is resource overlapping between the PUCCH carrying HARQ-ACK and CG-PUSCH with different L1 priorities. Option 1 is same/similar as Proposal 5C-4 that the priority of CG-UCI is the same as the HARQ-ACK that to be multiplexed on the CG-PUSCH; while Option 2 is that the priority of CG-UCI depends on the priority of the CG-PUSCH; it has different priority of the HARQ-ACK that to be multiplexed on the CG-PUSCH. |
| Panasonic | We are fine with Proposal 5C-1. We are open to discuss Prposals 5C-2, 5C-3, and 5C-4 further. |
| Spreadtrum | Proposal 5C-1: support. Proposal 5C-2/3/4: we are open to further discuss. |
| LG | Regarding Proposal 5C-3 and 5C-4, it may be better to be treated/discussed under other agenda for intra-UE multiplexing from the perspective of overall multiplexing procedure, but anyhow, we would like to hear more companies’ views. |
| Nokia, NSB | We are in principle ok with supporting configuration of phy-PriorityIndexsimultaneously with cg-retransmissionTimer. However, given the little time we have left in the WI, we see this as a lower priority item to finalize, and should avoid any complicated optimizations. The discussions should also happen together with other AI. |
| Lenovo, Motorola Mobility | Proposal 5C-1: We support that phy-PriorityIndex can be configured simultaneously with cg-retransmissionTimer. Proposal 5C-2: Support the proposal Proposal 5C-3: We prefer option 2, with the following clarification: Option 2: CG-UCI L1 priority is same as CG PUSCH L1 priority. Regardless whether the UE is provided cg-UCI-Multiplexing, CG-UCI and HARQ-ACK with different L1 priorities are separately encoded and multiplexed in CG-PUSCH. If the UE is provided cg-UCI-Multiplexing, CG-UCI and HARQ-ACK with the same L1 priority are jointly encoded. Proposal 5C-4: We think CG-UCI L1 priority should be determined based on CG-PUSCH L1 priority. Further, CG-UCI should be jointly encoded with HARQ-ACK with the same L1 priority. |
| ZTE | We support proposal 5C-1. When cg-RetransmissionTimer is configured and intra-UE prioritization/multiplexing mechanism is used, - For LP PUCCH overlaps with HP CG PUSCH in the time domain, if cg-UCI-Multiplexing is configured, NR-U rule is reused, i.e., the HARQ-ACK and the UCI are multiplexed in the PUSCH. If cg-UCI-Multiplexing is not configured, Rel-16 URLLC rule is reused, i.e., only CG PUSCH is transmitted. - For HP PUCCH overlaps with LP CG PUSCH in the time domain, HP PUCCH is transmitted carrying COT information of CG-UCI additionally no matter whether cg-UCI-Multiplexing is configured. |
| ETRI | Proposal 5C-1: We support the proposal. Proposal 5C-2: One question is whether to limit the HARQ-ACK only. We think CSI can be still be discussed in Rel-16 mechanism. Proposal 5C-3: For Option 1, we think that CG-UCI and HARQ-ACK can encoded but it could be indicated dynamically. As DG PUSCH, when there are lots of HARQ-ACK bits, the dynamic indication to multiplex would be beneficial because effective code rate of TB can be maintained. For example, HP CG-UCI and LP HARQ-ACK are involved, the CG-UCI can dynamically indicate whether drop or multiplex HARQ-ACK. We think that this reduces the number of blind decodes at the gNB. Proposal 5C-4: In our understanding, the same priority CG-UCI and HARQ-ACK are jointly encoded by previous agreements. We think that the size of HP HARQ-ACK may not be ambiguous and the legacy behavior can be robust. |
| Samsung | Fine with Proposal 5C-1. Proposal 5C-2/3 are not clear. Is it only assuming two overlapping channels? What if there are multiple overlapping PUSCHs including both CG PUSCHs and DG PUSCHs? Fine with Proposal 5C-4. |
| Qualcomm | Further discuss is needed for these proposals. |
| Moderator | @Dear all: After reviewing your input, and more careful checking of the spec and contributions, it seems to me that we don need any of these proposals. I explain in the following and please review and comment, if there is inconsistency in the logic and hence, need for proposals to agree. - cg-UCI-Multiplexing is configured per CG. - Rel-16: phy-PriorityIndex is configured. Now, performing Intra-UE multiplexing procedure: - Multiplexing could only occur when we resolve overlapping between same priority. In this case, - If a PUCCH with HARQ-ACK overlaps with CG-PUSCH (both have the same priority): - If cg-UCI-Multiplexing is enabled for that CG-PUSCH, HARQ-ACK would be multiplexed in CG-PUSCH as descirbed in 38.213. - Otherwise, CG-PUSCH would be dropped as descirbed in 38.213. - Prioritization (cancelation) could only occur when we resolve overlapping between different priority. In this case, - If a PUCCH with HARQ-ACK and HP overlaps with CG-PUSCH with LP: - CG-PUSCH is dropped. - If a PUCCH with HARQ-ACK and LP overlaps with CG-PUSCH with HP: - PUCCH is dropped. - No matter if cg-UCI-Multiplexing is enabled or disabled. - Rel-17: phy-PriorityIndex is configured. Now, performing Intra-UE multiplexing procedure: - Multiplexing could occur when we resolve overlapping between different priority. - In this case if a PUCCH with LP HARQ-ACK overlaps with HP CG-PUSCH: - If cg-UCI-Multiplexing is enabled for that CG-PUSCH, HARQ-ACK would be multiplexed in CG-PUSCH as descirbed in 38.213. - Otherwise, CG-PUSCH would be dropped as descirbed in 38.213. - In this case if a PUCCH with HP HARQ-ACK overlaps with LP CG-PUSCH: - If cg-UCI-Multiplexing is enabled for that CG-PUSCH, HARQ-ACK would be multiplexed in CG-PUSCH as descirbed in 38.213. - Otherwise, CG-PUSCH would be dropped as descirbed in 38.213. Please note that the behaviour in 38.213 states that: When a UE would multiplex HARQ-ACK information in a PUSCH transmission that is configured by a ConfiguredGrantConfig, and includes CG-UCI [5, TS 38.212], the UE multiplexes the HARQ-ACK information in the PUSCH transmission if the UE is provided cg-UCI-Multiplexing; otherwise, the UE does not transmit the PUSCH and multiplexes the HARQ-ACK information in a PUCCH transmission or in another PUSCH transmission.. That means that in case of priortization/cancellation between LP and HP, cg-UCI-Multiplexing is irrelevant, since no multiplexing happens. In case of multiplexing between same priority or different priority, then the rule in 38.213 is applied, whether to multiplex or drop, no matter the priority. What happens, is based on NW configuration. But the UE behaviour is clear. The reason is that the NW configures CG. If CG is configured with HP, it is up to NW, to enable or disable cg-UCI-Multiplexing. Whehter it is good configuration or not, if CG is configured with HP and cg-UCI-Multiplexing is disabled, it is NW choice. NW could as well, configured CG with HP and enable cg-UCI-Multiplexing. With respect to Proposal 5C-4, as Sony explained, it is taken care under AI 8.3.4. @All: Moderator recommendation. Please share your view and comment on the following: - Do you agree or disagree? - If you agree and clarificaiton is needed, could we refine the wording and description of steps/cases above and capture it as conclusion? |

## 2.5D COT duration limit

Proposal 5D-1:

On the semi-static configuration of UE-initiated FFP in a given unlicensed channel, the UE should be provided with a parameter to limit its COT to an indicated duration. Configured UL resources for PUSCH/PUCCH/SRS can be masked/restored back in all u-FFP periods by providing/updating this parameter without reconfiguring all impacted UL resources.

- Supported by: HW/HiSi, LG

In the previous meeting, some companies argued that there is no need for this RRC parameter since existing mechanisms are sufficient to achieve that goal. In the following table, HW/HiSi summarize the issues with these existing mechanisms and explain their perspective that why they are not comparable to, or considered as substitute for, the this simple RRC parameter

| Existing Mechanism | Issues as compared to the proposed RRC parameter ‘COT duration limit’ |
| --- | --- |
| Proper gNB configuration | Not applicable to the illustrated case Would be applicable if gNB is configuring u-FFP and UL resources for the UEs of interlaced group 1 and the UEs of interlaced group 2 at the same time. |
| Reconfiguration of UE1 FFP periodicity/offset | Reconfiguring u-FFP1 periodicity/offset would render remaining configured UL resources not useful for the UE COT, leading to reconfiguring UL channels/signals for each URLLC UE operating with u-FFP1. e.g.., if the offset of u-FFP1 is shifted earlier, configured UL resources aligned with u-FFP1 boundary would no longer be used to initiate the UE COT. |
| Configuration of invalid symbols | InvalidSymbolPattern is only applicable to PUSCH and only when PUSCH Repetition Type B is used; cannot be used with configured PUCCH/SRS |
| Dynamic UL CI | Applies only to PUSCH and SRS; cannot be used with configured PUCCH Not reliable compared to RRC: Misdetection of GC DCI 2_4, e.g., by cell edge UEs, leads to blocking/collision between UEs using u-FFP1 and UEs using u-FFP2 Intricate mechanism to provide gNB with the flexibility to target UL resources in a dynamic manner. - With the target resources being semi-static and periodic in each u-FFP1, the dynamic overhead, monitoring of DCI 2_4 and complexity of procedures are not justified |
| Dynamic SFI | Not reliable compared to RRC: Misdetection of GC DCI 2_1, e.g., by cell edge UEs, leads to blocking/collision between UEs using u-FFP1 and UEs using u-FFP2 Could only dynamically override semi-static flexible symbols as DL in u-FFP1. UEs may indicate COT sharing in CG-UCI in every PUSCH for these resources whereas gNB would not transmit DL on them to allow for UL transmissions by UEs using u-FFP2 - With the target resources being semi-static and periodic in each u-FFP1, the dynamic overhead and monitoring of DCI 2_1 are not justified |

## 2.5D.1	Discussion – 1st round

| Question: Please share your view on Proposals 5D-1 |  |
| --- | --- |
| Company | Comment |
| Sony | We agreed that UL CI and Dynamic SFI have limitations in controlling UE’s COT. We think that for controlling of UE initiated COT, it is easier if the gNB could cancel the UE’s COT as described in Section 2.7. |
| Intel | We are open to further discuss this proposal. |
| Futurewei | We are OK with the proposal, however we think that should be extended to also include the maximum number of COTs that UE can initiate. |
| DOCOMO | We are open to further discuss this proposal. |
| Huawei, HiSilicon | We support the proposal as a proponent. The following is the related figure from our contribution for your convenience @Sony, please note that the issue discussed in Section 2.7 is about cancelling the UE COT initiation assumption which, if supported, can be used to cancel UL transmissions that would otherwise overlap with gNB’s idle period. However, this is not the case this proposal addresses, i.e., gNB controls the collisions/blocking between UEs on the same channel without interrupting the FBE operation of (group) UE1 and without the need to reconfigure and signal updated configurations for all of impacted configured UL signals and channels (for each UE in the group). Figure 2. gNB controls collisions/blocking between URLLC UEs (or group of interlaced UEs) configured with different FFP parameters without interruption and triggering reconfiguration of impacted UL for all existing URLLC UEs through providing a COT duration parameter. |
| Apple | We still do not see a strong need for the proposal. Also we prefer not to reopen the discussion that had already been concluded. |
| CATT | We do not see a strong need for the proposal. |
| vivo | We do not think Proposal 5D-1 is needed, since we agreed that the collision is under gNB’s control. Even if something is needed, there are already many exsiting mechansms as shown in the above table avalaible, especially the COT limitation by introducing a new RRC parameter can be realized by reconfiguring the FFP to avoid any unwanted collisions. |
| Spreadtrum | We do not see strong need for the proposal. If the intension is to avoid the collisions between two UEs. It can be solved by proper UE-initiated configurations, including period and offset. Shorter period can achieve similar effect as the proposal. |
| LG | We support the proposal, and are share the same motivation (TDM management between UEs and/or gNB) with Huawei’s explanation in above. |
| Nokia, NSB | We still see that the existing mechanisms are sufficient and see no need for a specific new mechanism. |
| Lenovo, Motorola Mobility | - We think the conclusion “potential collisions or blocking are controlled/mitigated by gNB” achieved before applies to this case also. - We are not convinced that already available mechanisms (e.g., proper configuration, DCI format 2_0 COT duration indicator, etc. ) to handle collision are not sufficient. We wonder how the proposed mehcanim to limit the COT duration is useful considering different UEs might have different FFP duration and offset (e.g., UE1 FFP=3ms, offset=0; UE2 FFP=2ms, offset=1) |
| ZTE | We do not see a strong need for the proposal. |
| Samsung | We don’t think optimization of the proposal is still needed, considering the minimum standard impact is expected for this WI. |
| Qualcomm | Further discussion is needed. |
| Moderator | Summary of views: - Support: HW/HiSi, FW, LG - FFS: Intel, DCM, QC - No need: Apple, CATT, vivo, Spreadtrum, Nokia/NSB, Len/MOT, ZTE, Samsung @Moderator recommendation: There is interest as well as support for this proposal. However, majority see no need for this enhancement. Therefore, Moderator is not driving the discussion. However, propoents are free to continue disucsison to promote the proposal. If proponets are interested to continue discussion, please let the moderator know to arrange a place holder in the summary document. |

## 2.5E Support for DCI 0_2 and 1_2 with dynamic channel access

At RAN1#105-e, it was agreed that whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT, is based on the content in the scheduling DCI. Furthermore, at RAN1#106e it was agreed that the size of the Rel-17 DCI 0_2 and 1_2 is 2 bits. Also, the interpretation of those bits was agreed.

Nokia explains that however, it is still open if the DCI field can be present in the case of dynamic channel access (LBE), and if so, what should the size of the bitfield be. There is clear motivation for enabling the use of DCI 0_2 and 1_2 in LBT case too, and hence, the channel access and CP extension field should be supported with those formats too. The size of the bitfield can be configurable in the same way as for DCI 0_1 and 1_1. For added flexibility and optimized PDCCH overhead, it makes sense that the configuration of the bitfield is independent from DCI 0_1 and 1_1, i.e. separate RRC parameters ul-AccessConfigListDCI-0-2 and ul-AccessConfigListDCI-1-2 are introduced for this purpose.

Proposal 5E-1:

Introduce new RRC parameters ul-AccessConfigListDCI-0-2 and ul-AccessConfigListDCI-1-2 to support indication of CP extension, LBT type, and CAPC with DCI 0_2 and 1_2 with dynamic channel access.

- Supported by: Nokia/NSB

## 2.5E.1	Discussion – 1st round

| Question: Please share your view on Proposals 5E-1 |  |
| --- | --- |
| Company | Comment |
| Intel | We support this proposal. |
| Futurewei | We are OK with the Proposal 5E-1 |
| DOCOMO | We think this is out of WID spope |
| Huawei, HiSilicon | We are OK with proposal if the majority views are still supportive. However, we note that it should be handled with low priority at least w.r.t other RRC related proposals due to its out-of-scope nature. |
| Apple | We are fine with the proposal. |
| CATT | We support proposal 5E-1. |
| vivo | For this topic, the channel access mode is semi-static. Not sure whether we need any agreement for dynamic channel access. |
| Panasonic | We are fine with the proposal. |
| Spreadtrum | We are fine with the proposal, and it can be with lower priority. |
| LG | We are open to discuss this issue. |
| Nokia, NSB | We support this as the proponents. |
| Lenovo, Motorola Mobility | We suggest discussing this proposal with low priority as it may not be within the WID scope. |
| ZTE | We support this proposal. |
| ETRI | We are OK with the proposal. |
| Moderator | Summary of view: - Support: Intel, FW, Apple, CATT, Pana, spreadtrum, Nokia/NSB, ETRI, ZTE - OK/disucss w low priority: HW/HiSi, LG, vivo, DCM ============================= @Moderator recommendation: This proposal was discussed intensively last meeting and was almost ready to go. We have been through the motivations, etc. To save the time and prioritizing other topics, is there any objection to support the following proposal? Proposal 5E-1: Introduce new RRC parameters ul-AccessConfigListDCI-0-2 and ul-AccessConfigListDCI-1-2 to support indication of CP extension, LBT type, and CAPC with DCI 0_2 and 1_2 with dynamic channel access. |

## 2.6	UE initiated COT for Wideband operation

It has been discussed last meeting that for Wideband operation whether the assumption on COT-initiator should be aligned across different RB sets or not. The source of issue that FFP parameters are agreed to be configured per cell when in Wideband, the cell bandwidth can be larger than 20 MHz. On the other hand, LBT bandwidth is 20 MHz. That means all the sensing and related actions are based on 20 MHz.

Summary of views:

- Intel, DCM, LG, vivo, MTK suggest aligned COT-initiator assumption across RB sets. From vivo’s perspective, when gNB indicates a COT initiator, or UE determines a COT initiator, it applies to the transmission over all the scheduled or configured frequency resources. When operating in a bandwidth with more than one RB set, the gNB or UE should transmit a transmission at a given time within only one COT (i.e. aligned COT-initiator assumption). Since the LBT outcome coud be different, the propoents propose a scheme for alignment. Other proponents may assume the possibility of a transmission with different COT-initarór assumptions but find it very complicated.

- Xiaomi and Len/MOT express that aligned COT-initiator assumption across RB sets is unnecessary. Len/MoT explains the motivation for alignment is to avoid UL to DL interference (e.g., impacting gNB LBT on some RB sets) if the COT initiator assumptions are different across the RB sets. They claim such interference can be avoided by gNB. Xiaomi explains that a device’s propose it to transmit the channel successfully, no matter it is associated with a gNB-initiated or UE-initiated COT and with unaligned COT-initiator assumption, it is just the matter that, in some LBT bandwidth, a CCA is needed before the transmission while in other LBT bandwidth, CCA can be omitted.

Moderator’s recommendation:

From the view expressed, it seems that it is important to establish a common understanding first, as it seems at least vivo and Xiaomi for example share different perspectives. Therefore, it is important to clairfy whether the following statement holds or not:

- When a transmission across multiple RB sets occurs, there can be time intervals during the transmission that the COT-initiator assumptions corresponding to different RB sets of the transmission are different.

  - If the statement above is false, then a solution for alignment is necessary.

  - If the statement above is true, the solution is not necessary, but it can be benefial.

## 2.6.1	Discussion – 1st round

Proposal conclusion 6-1:

Select one of the following:

- Alt-1: When a transmission across multiple RB sets occurs, there can be time intervals during the transmission that the COT-initiator assumptions corresponding to different RB sets of the transmission are different.

  - Supported by: Xiaomi

- Alt-2: When a transmission across multiple RB sets occurs, during the transmission time the COT-initiator assumptions corresponding to different RB sets of the transmission are the same.

  - Supported by: vivo

Proposal 6-2:

Select one of the following:

- Alt-1: No need to align COT-initiator assumption for multiple channels contained in a cell.

  - Supported by: Xiaomi, Len/MoT

- Alt-2: Align COT-initiator assumption for multiple channels contained in a cell.

  - Supported by: Intel, LG DCM, MTK, vivo

Note that Alt-1 in Proposal 6-2 is applicable only if Alt-1 in proposed conclusion 6-1 is adopted. Alt-2 in Proposal 6-2 is applicable for both Alt-1 and Alt-2 in proposed conclusion 6-1.

Proposal 6-3:

If, when operating on multiple carriers, the assumptions regarding the COT initiator are aligned across all carriers/ LBT BWs, a UE could assume to operate:

- as an initiating device over all RBs if for at least one LBT BW i) the UE assesses that it shall operate as initiating in that LBT BW or ii) the UE has received indication from the gNB that it shall operate as an initiating device; or

- as a responding device over all RBs, if for each LBT BW i) the UE assesses that it shall operate as a responding device or ii) the UE has received indication from from gNB that it shall operate as responding device.

Notice that before the UE can actually perform a transmission and decide whether to operate as initiating or responding device, it must assess whether the channel access requirements are met over all the RBs.

- Supported by: Intel, DCM, LG

Note that Proposal 6-3 is applicable only if Alt-2 in Proposal 6-2 is adopted.

Proposal 6-4:

Separate FFP configurations on different LBT bandwidths can be considered, and previous agreements for single FFP configuration on a cell can still be reused on per LBT bandwidth bases.

- Supported by: Xiaomi

Note that Proposal 6-4 is reasonable only if Alt-1 in Proposal 6-2 is adopted.

| Questions: - Q1: Companies are kindly requested to provide any update/correction on the discussion and their positions with respect to proposed conclusion 6-1 and Proposals 6-2, 6-3 and 6-4. - Q2: Please review Moderator recommendation and comment if you agree or not with this approach. - Q3: Please share any other comment if any. |  |
| --- | --- |
| Company | Comment |
| Intel | We support Alt-2 within proposal 6-2, and proposal 6-3. As for the need to align the assumptions regarding the COT initiator across all carriers/ LBT BWs, while in principle a UE may assume that it can operate differently in every LBT BW without violating the ETSI BRAN [7], this may have several drawbacks. The main issue is that either the UE and gNB would need to handle for each RB set not only the COT initiator assumptions but also keep track separately of the respective idle periods so that to obey with the regulatory requirements and avoid mutual interference across UEs, which would add a lot more complexity at both the UE and gNB. The additional issue is that this may induce additional interference and blocking among devices since synchronization among device, so that to leave empty gaps within the idle period of the initiating device, may become a non-trivial issue, since a device to transmit or continue transmission may need to perform LBT in different instances of time for different RB sets. |
| Futurewei |  |
| Huawei, HiSilicon | Alt 1 in Proposal 6-1 is more aligned with our current understanding. We agree with Intel that’ in principle a UE may assume that it can operate differently in every LBT BW without violating the ETSI BRAN’. However, we would like to hear other companies’ views as well regarding the drawbacks. |
| Apple | We may be missing something here, but before going into the detailed proposals, we wonder why this is particularly an issue for FBE but not LBE. |
| vivo | For Proposal conclusion 6-1, we support Alt.2, it is difficult to understand for a given UL transmission e.g. CG PUSCH occupying different RB sets, e.g. RB set #1, #2, #3 #4, how it works with Alt.1 that RB set #1 and #2 shares gNB’s COT; RB set #3, #4 uses UE its own COT. For Proposal 6-2, we support Alt-2. We would like to further clarify our view, the COT-initiator alignment for a device to initiate the COT only applies to the RB sets which are sensed as idle. For those which are sensed as busy, no such assumption can be made. E.g., if DL BWP is 80MHz and has 4 RB sets, after LBT, gNB senses that RB set#1 and RB set #2 are idle, then gNB and UE assumes that the COT initiator in RB set#1 and RB set#2 is gNB, i.e., the COT-initiator assumption is aligned. When UE shares the gNB-initiated COT, UE can only use RB set #1 and RB set#2. If UL BWP is 80MHz, for a UE with CG resources in RB set #1, #2, #3 #4, and aligns with the FFP-u boundary, if the channels in RB set RB set #1, #2, #3 #4 are all idle, and UE initiates a COT, then gNB and UE assumes that the COT initiator is UE in all the 4 RB sets. gNB can share the UE-initiated COT in all these 4 RB sets. For Proposal 6-3, we would like to further clarify our understanding: For example, a CG-PUSCH transmission uses RB set#1 and #2; UE transmits the CG-PUSCH by sharing gNB’s COT if the UE has determined that gNB has acquired the channel for both RB sets, RB set #1 and #2; Otherwise, if gNB has not aquire the channel on one RB set e.g. RB set#2, then UE can transmit the CG-PUSCH by intiating its own COT and when UE senses both RB set #1 and #2 are idle, the CG transmision can be performed by using UE’s COT, as hsown in Figure 1 below. Correct? Figure 1 |
| LG | Regarding Proposal 6-3, it doesn’t seem that Intel and LG have different consideration on this issue, so we may need more precise wording and expression to make complete proposal. In this context, we suggest following way to formulate the proposal. [Modified Proposal 6-3] If, when operating on multiple carriers/LBT BWs, the assumptions regarding the COT initiator are aligned across all carriers/LBT BWs, - a UE could assume to operate as an initiating device for a LBT BW i) if the UE didn’t assess and didn’t receive inication from the gNB that it shall operate as a responding device for any of LBT BWs, and ii) if the UE assesses or has received indication from the gNB that it shall operate as an initiating device for the LBT BW. - a UE could not assume to operate as an initiating device for any of LBT BWs i) if the UE assesses or has received inication from the gNB that it shall operate as a responding device for at least one LBT BW, and the UE could assume to operate as a responding device for a LBT BW i) if the UE assesses or has received indication from the gNB that it shall operate as a responding device for the LBT BW. |
| Nokia, NSB | The difference between proposed conclusion 6-1 and proposal 6-2 is not fully clear. Anyway, our view is that the UE should assume same COT initiator for transmissions spanning over multiple 20 MHz RB sets (i.e. Alt-2 in Proposal 6-2), as it is our understanding this is also the underlying assumption with dynamic channel access mode. We therefore support the intent of Proposal 6-3, though we are not sure an agreement (and corresponding specification text) is required. We are ok with proposal 6-4, but disagree with the note that it is only reasonable if Alt-1 in Proposal 6-2 is adopted. This is only in case the UE is transmitting on multiple RB sets. The UE could still be configured with resources on multiple RB sets (each RB set having its own FFP configuration) but transmit only on one RB set at a time. |
| Lenovo, Motorola Mobility | Would like to first clarify what is the main motivation of alignment (in order to choose between Alt-1 and Alt-2): if it is UL-DL interference blocking idle periods, isn’t it a case that the conclusion “potential collisions or blocking are controlled/mitigated by gNB” is applicable to? If it is complexity of tracking some RB sets associated with u-FFP and some with g-FFP, and hence tracking corresponding idle periods (one set of RB-sets for u-FFP, and one set of RB-sets for g-FFP), what is the source of complexity exactly considering in non-wideband case, one transmission can be associated with a g-COT and one transmission can be associated with a u-COT? |
| ETRI | Our view is aligned with Alt-1 in Proposal 6-1. Also, based on our current understanding, no other solution specific to the wideband operation is needed for URLLC operation in FBE. |
| Samsung | We have similar question as Apple. And don’t think need to discuss or optimize for this proposal |
| Qualcomm | We support Alt-2 for proposal 6-1, We support Alt-2 for proposal 6-2, We support proposal 6-3. |
| Moderator | Summary of views: Proposal conclusion 6-1: - Alt-1: - Supported by: Xiaomi, HW/HiSi, ETRI - Alt-2: - Supported by: vivo, QC Proposal 6-2: - Alt-1: - Supported by: Xiaomi, Len/MoT - Alt-2: - Supported by: Intel, LG DCM, MTK, vivo, Nokia/NSB, QC Proposal 6-3 : - Supported by: Intel, DCM, LG, Nokia/NSB, QC Proposal 6-4: - Supported by: Xiaomi =========================== @All: There has been questions (by Apple) why this is an issue for LBE, not FBE. Also, Nokia clarifies that Alt-2 in P6-2 is inline with underlying assumption for LBE (I hope that answers Len/Mot question). If companies can provide more clairficaitons, please do. ================================ @Moderator recommendation: It seems to Moderator, maybe we can consider Proposal 6-2 and 6-3 (with LG update) for further disucsison. I have updated 6-3 in addition to LG update to be more accurate. Please share your view. Proposal 6-3 (updated): If, In semi-static channel access mode, when operating on multiple carriers/LBT BWs, the assumptions regarding the COT initiator for a transmission are should be aligned across all carriers/LBT BWs at any transmission time. To align the assumptions, - a UE could assume to operate as an initiating device for a LBT BW i) if the UE didn’t assess and didn’t receive inication from the gNB that it shall operate as a responding device for any of LBT BWs, and ii) if the UE assesses or has received indication from the gNB that it shall operate as an initiating device for the LBT BW. - a UE could not assume to operate as an initiating device for any of LBT BWs i) if the UE assesses or has received inication from the gNB that it shall operate as a responding device for at least one LBT BW, and the UE could assume to operate as a responding device for a LBT BW i) if the UE assesses or has received indication from the gNB that it shall operate as a responding device for the LBT BW. |

## 2.7	Control of initiated COT

Below is a selection of set of proposals that impact the assumption of COT initiator or related procedures.

Proposal 7-1:

- In semi-static channel access mode, early termination or cancellation of a FFP is enabled by allowing the gNB to overwrite through DCI scheduling indication any prior decision regarding the initiator of the COT.

  - Supported by: Intel

- On Prposal 7-1, Moderator udnerstanding is that a later DCI can always override previous assumption. But if that is not a common understanding, it could be discussed and clairifed.Similarly to Proposal 7-2.

Proposal 7-2:

- Allow the gNB to cancel a UE initiated COT. A COT cancellation indicator can be introduced to dynamically indicate to a UE to cancel its initiated COT.

  - Supported by: Sony, LG (DCI 2_0)

- On Proposal 7-2, Sony propose to have a mechanism to cancel a UE-initiated COT. In Moderator’s view the principle is similar to previous DCI but overding is done with group common DCI. Xiaomi disucsses that the intention is to cancel an ongoing UL transmission and for that DCI 2_4 can do the job. However, it is not clear from Moderator point of view if that is the intention, or rather cancelling the assumption that a UE has initiated a COT. Changign the assumption wold impact other transmission that are not necessarily ongoing. Therefore, clarification is needed

Proposal 7-3:

- In semi-static channel access mode, when the gNB schedules by a DCI a UL transmission in a later u-FFP in the same g-FFP that carries the scheduling DCI and the transmission is not aligned with the later u-FFP boundary, the UE follows the indicated COT initiator as the following:

- If the UE validates the indicated COT initiator assumption and satisfies the applicable sensing conditions, the transmission occurs. Otherwise, the transmission is dropped.

- Supported by: HW/HiSi

Proposal 7-4:

Consider allowing the following UE behaviour for the scheduled UL not aligned with FFP-u boundary.

- The UE would drop the scheduled UL transmission in case when gNB indicates UE-initiated COT based TX for the UL, but the UE didn’t initiate COT for the FFP-u period.

  - Supported by: LG

Proposal 7-5:

Consider the determination/validation on the COT initiator for the scheduled UL transmission based on cross-CC scheduling.

- The validation of gNB-initiated COT (based on the detection of DL transmission) can be skipped for the scheduled UL by cross-CC (and same FFP-g) scheduling.

Supported by: LG

Proposal 7-6:

- Consider to define the FFP including or starting with essential DL/UL transmission occasions (such as SSB or CORESET#0) as default FFP-g.

- Supported by:LG

## 2.7.1	Discussion – 1st round

| Question: Q1: Please share your view on Proposals 7-1 to 7-6. Q2: Please provide clarifications where there is a misunderstanding on the intention of a proposal, or unclarity whether exist methods to achieve the goal of a proposal. Q3: Please suggest candid porposals that are critical for design and are not currently lifted up for discussions. |  |
| --- | --- |
| Company | Comment |
| Sony | Proposal 7-1 and 7-2 are the same but worded differently. The intention is allow the gNB to tell the COT initiating UE that its COT is cancelled. |
| Intel | As for proposal 7-1, we support this proposal. As for the FL’s assumption, that is correct. So this procedure would not come at any additional cost, since the current framework could be used. As for proposal 7-2, the text/intention should be further clarified. As the text is provided, we have same concerns as the moderator. As for proposal 7-3, we support the proposal. As for proposal 7-4, we support the proposal As for proposal 7-5, we are not OK with the proposal, since we believe this would lead to violate the ETSI BRAN: in fact if the UE does not check all the assumptions for each CC, the UE could be allowed to transmit even in CCs where the gNB’s would not have acquired the COT. As for proposal 7-6, we do not think this is necessary, and could be handled by the network. |
| Futurewei | We are Ok in principle with the Proposal 7-1 and 7-2, We support the Proposal 7-3 and 7-4. For 7-5 and 7-6 we have a similar understanding as Intel. In 7-5 the LBT is necessary for eac CC, for 7-6 it is not needed it. |
| DOCOMO | Proposal 7-1: Support Proposal 7-2: If the intention is same as Proposal 7-1 as clarified by Sony, it can be merged to Proposal 7-1 Propsoal 7-3: Support Propsoal 7-4: Support Propsoal 7-5: We share the view with Intel/Futurewei Propsoal 7-6: Clarification is necessary what the motivation is. |
| Huawei, HiSilicon | We support Proposal 7-3 and we understand that Proposal 7-4 is addressing the same issue. Proposal 7-3 is formulated following the language used in the agreements made last meeting though. The following is the related figure from our contribution for your convenience Figure 1. gNB schedules UL transmission in a later u-FFP in the same g-FFP that carries the scheduling DCI; the latest agreement does not apply. We do not think Proposal 7-6 is needed. |
| Apple | P7-1/7-2: We do not see a strong need/motivation. P7-3/7-4/7-5: We think these are covered by the agreements from last meeting already. P7-6: Do not see the need. |
| vivo | For Proposal 7-1 and 7-2, if it is already supported or natural consequnces of dynamic SFI/UL CI etc., then we do not need to agree them. Proposal 7-3 and 7-4 are similar and also the natural conseuqnce of the agreement on COT-initiator for DG? For proposal 7-5, not sure why it can be skipped, assume same COT-initiator across CCs? For proposal 7-6, clarification is needed on the motivation. |
| LG | Regarding Proposal 7-5, the situation is that DCI and the corresponding scheduled UL are on different CC/RB set but those are within same gNB FFP. Indicating gNB-intiated COT based TX for scheduled UL means that the gNB has already initiated even for the CC with the scheduled UL. In this case, we don’t see the reason to drop the scheduled UL even if DL is not detected by UE for the CC with the scheduled UL. |
| Nokia, NSB | Proposal 7-1: support, but probably no further agreement needed. To be discussed if UE-initiated transmission during idle period of an “active” gNB FFP (or the other way around) can represent a problem from regulatory/co-existence perspective. Proposal 7-2: not needed, UL cancellation mechanism already ion the specs. Proposal 7-3 and 7-4: agree on UE behavior but not clear whatr this additionally brings as compared to previous agreement(s). Proposal 7-5: agree. X-CC scheduling can itself imply that the gNB has or has not acquired a COT, depending on what it indicates to the UE. If the gNB indicates the UE as COT initiator, the UE shall still perform LBT. If gNB indicates itself as the COT initiator, the UE no longer needs to double check if the g-FFP exists. Proposal 7-6: this is not needed. There are already mechanisms agreed (e.g. use of SFI) to prevent UEs from initiating COT overlapping with gNB transmission of critical signals such as SSB or CORESET#0) |
| Lenovo, Motorola Mobility | 7-1/7-2: - it would be reasonable to first clarify the benefit of changing the COT initiator assumption for a transmission. In our view, changing the COT initiator assumption of a transmission could change - FFP idle periods the transmission could overlap with - COT initiator assumption of subsequent configured UL transmissions, which effectively could change FFP idle periods the subsequent configured UL transmissions could overlap with - We would like to understand what is meant by “gNB to overwrite through DCI scheduling indication any prior decision regarding the initiator of the COT”. Does it mean for the COT initiator assumption of a scheduled PUSCH transmission by a first scheduling DCI can be changed by sending a second scheduling DCI for the same PUSCH? Also, we are wondering if a timeline needs to be defined to determine the latest time an overriding DCI can be sent. - Besides, we are wondering if DCI 2_0 (COT duration indicator) can be used to limit a COT duration/COT cancellation 7-3/7-4: OK 7-5: suggest discussing afetr the outcome of wide-band discussion? 7-6: from LG contribution, seems the intention is to avoid “potential UL-to-DL interference due to COT initiation by UE”, wondering if the conclusion “potential collisions or blocking are controlled/mitigated by gNB” is applicable to this case? |
| ZTE | For Proposal 7-1 and 7-2, we share similar view with Moderator that overriding through a later DCI is within the current framework, so an explicit agreement may not be necessary. For Proposal 7-3 and 7-4, the conclusion of RAN1 #106e can cover these case. For Proposal 7-5, each CC needs LBT and we do not support this proposal. For Proposal 7-6, we don’t think it’s needed. |
| ETRI | Proposal 7-1/7-2: We do not see a strong need/motivation. Proposal 7-3/7-4: We have the same understanding but seems that they are already covered by the previous agreements. Proposal 7-5: In our understanding, the current ETSI regulation says that the channel access mechanism for FBE is applied independently per channel, and cross-channel LBT or COT sharing is not mentioned (which can be interpreted to be not allowed). Proposal 7-6: Clarification on motivation seems needed. |
| Samsung | For P7-1/2, we think that this is a kind of optimization issue, not essential issue to solve. Given this agenda is aiming at minimum standard impact, we are not supportive of these. For P7-3/4, motivation is not clear, and it is against the conclusion we made in last meeting about “validated” since validation is only valid if transmission would start at the FFP boundary in the following conclusion. - The association assumption is validated as follows: - “Initiating COT” assumption is validated if the transmission would start at the FFP boundary and would end before idle period of the FFP. - “Sharing COT” assumption is validated if the transmission would start after the FFP boundary and would end before idle period of the FFP and the CO corresponding to the FFP is initiated. For P7-5/6, motivation is not clear. It needs more discussion on whether it is critical or not. |
| Moderator | Summary of views: Proposal 7-1: - Supported by: Intel, Sony (merge 7-1&7-2), FW, DCM, Nokia/NSb (as clarifcation) - No need: Apple, vivo, Len/Mot, ZTE, ETRI, Samsung Proposal 7-2: - Supported by: Sony, LG (DCI 2_0), FW, DCM - No need: Apple, vivo, Nokia/NSB, ZTE, ETRI, Samsung Proposal 7-3: - Supported by: HW/HiSi, Intel, FW, DCM, Len/Mot - No need: Apple, vivo, Nokia/NSB?, ZTE, ETRI, Samsung Proposal 7-4: - Supported by: LG, Intel, FW, DCM, HW/HiSi?, Len/Mot - No need. Apple, vivo, Nokia/NSB, ZTE, ETRI, Samsung Proposal 7-5: Supported by: LG No need: Intel, FW, DCM, ZTE, Samsung Proposal 7-6: - Supported by:LG - No need: Intel, FW, DCM, HW/HiSi, Apple, vivo, Nokia/NSB, Len/Mot?, ZTE, Samsung @All: It seems most of the companies assume that Proposals 7-1 to 7-4 some how are convered by previous agreement. Proposal 7-5 and 7-6 lack support to be promoted. @Moderator recommendation: We can discuss to formualte proposed conclusion to cover P7-1/7-2 and P7-3/7-4. If that is agreeable, please help with suggestions. On Proposal 7-5, P7-6 moderator suggest to be down-priortized. However, propoents are free to drive the discusison. If that is the preference, please let moderator know to provide a place holder for disucssion. |

## 2.8	Other topics

Companies discussed other issues where few of them are listed below along with a recommendation from Moderator:

- Issue#1: Protect important DL transmissions and the corresponding channel access opportunities, especially transmission SS/PBCH block transmission (Sharp)

  - Moderator comment: Usually SS/PBCH transmissions are prirotized transmission as compared to any other transmission. It is not clear what protection is needed. Even if the UE access the channel for UL, based on already existing rules, it should prioritize SS/PBCH transmission.

- Issue#2: COT-initiator indication for multi-PUSCH scheduled by a single DCI (CATT)

  - Moderator comment: Moderator understanding is that the since the indicated COT-initiator is applicable to all PUSCH, every PUSCH follows the same behaviour that to be agreed in section 2.1.

- Issue#3: gNB may cancel a low priority UE’s transmission and release the corresponding UE initiated COT in order to support high priority URLLC transmission of another UE (NEC).

  - Moderator comment: It is not clear what new mechanism is needed to specify that is not already supported, e.g. DCI 2_4.

- Issue#4: Not dropping the beginning of a low priority transmission due to prioritization if it could initiate a UE COT (Len/MOT)

  - Moderator comment: This is beneficial and mantian channel access, but cant be done by implementation?

- Issue#5: Consider not to allow transmission of the configured UL in the idle period of FFP-g located within an FFP-u period even if the UE has initiated COT for the FFP-u, in order to avoid potential UE-to-gNB interference (LG).

  - Moderator comment: This issue has been discussed intensively previously and concluded that no special action is needed.

- Other issues:

  - Consideration with respect to timeline and processing time (MTK)

  - Disabling UE-initiated COT by RRC for P-CSI and/or SRS (Apple)

  - …

## 2.8.1	Discussion – 1st round/2nd round

| Questions: - Please share your view on the issues or recommendations above and provide suggestions on topics that are critical for design and should be prioritized for discussions, even if they are not included in this summary. |  |
| --- | --- |
| Company | Comment |
| Intel | Our proposal regarding the intra-symbol starting position is missing: Proposal 11: When a gNB configures CG-startingOffset-r16, a Rel.17 UE never applies the offset if the CG PUSCH aligns with a u-FFP, and the UE operates as initiating device regardless of whether the UE is configured to operate in partial or full bandwidth. However, the CG UE applies the offset in all other cases. The proposal above is meant to further clarify the following agreement: Agreements: - For semi-static channel access mode, - If sensing is needed, it is performed immediately before the configured/scheduled transmission opportunity. - For operation with semi-static channel access, the Rel-16 random starting offsets for UL configured grants with Full BW allocation when UE initiates a COT, is not supported.. This agreement seems to only constrain the intra-symbol starting position procedure when a UE is configured to operate in full BW, but not for partial BW, while it is imperative for a UE operating as initiating device that the initial transmission aligns with a valid u-FFP boundary (e.g., it starts from the first OFDM symbol of a valid u-FFP) so that CCA could be performed. Otherwise, blocking would occur. Furthermore, even for the case when a UE is configured to operate in full BW, the aforementioned agreement is not clear (and up to interpretation) on the exact conditions according to which the intra-symbol starting position procedure should be precluded or allowed. In this sense, given the above technical justification, for both full and partial BW operation the intra-symbol starting position should be only precluded when the UE operates as initiating device and the CG-PUSCH aligns with a u-FFP. |
| Panasonic | We also woud like to clarify the handling of starting offsets for CG PUSCH. In RAN#102e, it was agreed that for operation with semi-static channel access, the Rel.16 random starting offsets for UL CG with full bandwidth allocation when UE initiates a COT, is not supported, but there is still the remaining issue whether/how to support the Rel.16 fixed (configured) starting offsets for UL CG with resource allocation fewer than full bandwidth when UE initiates a COT. In our view to reuse the multiple starting time offset for CG would be useful for the prioritization among devices especially between gNB and UEsm but we are open to discuss further considering Intel’s proposal. |
| Nokia, NSB | We agree that the CG-startingoffset-r16 should not apply with partial BW allocations either. |
| Lenovo, Motorola Mobility | Issue#1: seems related to the outcome of section 2.3. Issue #2: we suggest concluding section 2.1 first and discussing this proposal later. Depending on the u-idle and g-idle durations, there could be cases that having different COT initiator assumptions for different PUSCHs can be useful Issue#3: gNB can handle that Issue#4: not clear what is meant by implementation in FL’s comment. Is it UE’s implementation? If UE’s implementation does not support this, then the UE may not initiate a COT (if cancels completely the low priority transmission at the beginning of U-FFP). Is it gNB’s imeplementation? Considering gNB assigns resources considering different services periodicity, etc, not sure how such problem can be addressed. Issue#5: agree with FL. Achieved conclusion: “potential UL-to-DL interference due to COT initiation by UE”. |
| Moderator | Please continue discussions. Moderator will provide recommendation in next update. |

# 3	Conclusion

## 3.1 List of agreements in RAN1#106bis-e

TBD

## 3.2	Recommendations for next RAN1 meeting

TBD

# 4	References

| R1-2108729 | Uplink enhancements for URLLC in unlicensed controlled environments | Huawei, HiSilicon | Decision |
| --- | --- | --- | --- |
| R1-2108788 | UE initiated COT for semi-static channel access | FUTUREWEI |  |
| R1-2108831 | Enhancements for IIoT/URLLC on Unlicensed Band | Ericsson | Decision |
| R1-2108842 | Discussion on unlicensed band URLLC/IIoT | ZTE |  |
| R1-2108907 | Discussion on enhancements for unlicensed band URLLCIIoT | Spreadtrum Communications | Decision |
| R1-2108968 | Enhancements for unlicensed band URLLC/IIoT | vivo | Decision |
| R1-2109095 | Enhancements for unlicensed band URLLC/IIoT | OPPO | Decision |
| R1-2109138 | UL enhancements for IIoT/URLLC in unlicensed controlled environment | Nokia, Nokia Shanghai Bell |  |
| R1-2109217 | Discussion on remaining issues on enhancements for unlicensed band URLLC/IIoT | CATT | Decision |
| R1-2109356 | Enhancements for unlicensed band URLLC/IIoT | NEC | Discussion |
| R1-2109407 | Enhancement for unlicensed band URLLC IIoT | Xiaomi | Decision |
| R1-2109453 | Enhancements for unlicensed band URLLC/IIoT | Panasonic Corporation | Discussion |
| R1-2109483 | Enhancements for unlicensed band URLLC/IIoT | Samsung | Decision |
| R1-2109576 | On the enhancements for unlicensed band URLLC/IIoT | MediaTek Inc. |  |
| R1-2109606 | Further Details for Enabling URLLC/IIOT in Unlicensed Band | Intel Corporation | Decision |
| R1-2109673 | Discussion on enhancements for unlicensed band URLLC | NTT DOCOMO, INC. | Decision |
| R1-2109784 | Considerations on unlicensed URLLC | Sony | Discussion |
| R1-2109810 | Enhancements for unlicensed band URLLC/IIoT | ETRI | Discussion |
| R1-2109894 | Enhancements for unlicensed band URLLC/IIoT | InterDigital, Inc. |  |
| R1-2109942 | Enhancements for unlicensed band URLLC/IIoT | Lenovo, Motorola Mobility | Decision |
| R1-2109972 | Discussion on unlicensed band URLLC/IIOT | LG Electronics | Decision |
| R1-2109994 | Enhancements for unlicensed band URLLC/IIoT | Sharp | Decision |
| R1-2110029 | URLLC uplink enhancements for unlicensed spectrum | Apple | Decision |
| R1-2110180 | Uplink enhancements for URLLC in unlicensed controlled environments | Qualcomm Incorporated | Decision |
| R1-2110323 | Discussion on enhancement for unlicensed URLLC/IIoT | WILUS Inc. | Discussion |
| R1-2110396 | Discussion on unlicensed band URLLC/IIoT | ZTE |  |

# 5	Appendix

## 5.1	List of agreements

### 5.1.1	Agreements in RAN1#102-e

Agreements:

- For semi-static channel access mode,

- If sensing is needed, it is performed immediately before the configured/scheduled transmission opportunity.

- For operation with semi-static channel access, the Rel-16 random starting offsets for UL configured grants with Full BW allocation when UE initiates a COT, is not supported.

Agreements:

- For semi-static channel access mode,

  - When gNB operates as an initiating device

    - The gNB is not allowed to transmit during the idle period of any FFP associated with the gNB in which the gNB initates a COT

  - When a UE operates as an initiating device

    - The UE is not allowed to transmit during the idle period of any FFP associated with the UE in which the UE initates a COT

  - When a UE shares a COT initiated by the gNB during an FFP associated with the gNB

    - The UE is not allowed to transmit during the idle period of that FFP in which the UE shares the COT initiated by the gNB

  - When the gNB shares a COT initiated by a UE during an FFP associated with the UE

    - The gNB is not allowed to transmit during the idle period of that the FFP in which the gNB shares the COT initiated by the UE

  - FFS whether/how to support additional restrictions to the idle period

Agreements:

- For semi-static channel access mode, support using the transmission of any scheduled/configured UL channel/signal to initiate a COT by a UE in RRC_CONNECTED mode

  - FFS the case when the UE is IDLE/INACTIVE mode

Agreements:

- A UE initiates a COT in an FFP associated with the UE, if the UE transmits a UL transmission burst starting at the beginning of the FFP and ending at any symbol before the FFP’s idle period after a successful CCA of 9us immediately before the UL transmission burst.

Update on 8/26

Agreements:

- At least for FBE, configuration of (cg-RetransmissionTimer) should not be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum.

Conclusion:

Further study and decide how to harmonize the CG features for Rel-16 URLLC and Rel-16 NR-U. Table 1 in R1-2005376 can be used as a starting point for the corresponding discussion and decision.

Agreements:

- Conditions on the channel access procedures with respect to sensing duration and transmission gap for UE-initiated COT with UE-to-gNB COT sharing is similar as those for gNB initiated COT and gNB-to-UE COT sharing in Rel-16 by exchanging UE and gNB roles.

Agreements:

- UE-to- gNB COT sharing in semi-static channel access mode is supported.

  - The gNB determines a COT in an FFP associated to a UE, that is initiated by the UE, if the gNB detects a UL transmission from the UE starting from the beginning of the FFP and ending before the idle period of the FFP.

    - FFS details

  - When the gNB determines a UE has initiated a COT in an FFP associated to the UE, the gNB can transmit within the FFP and before the idle period corresponding to the FFP.

    - FFS whether/how UE to gNB COT sharing when the gap is >16us

Update from 8/28 GTW

Agreements:

For semi-static channel access mode,

o    Start of FFP for UE-initiated COT can be different from the start of FFP for gNB-initiated COT.

o    FFS: FFP Periodicity for UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT.

Agreements:

- For semi-static channel access mode,

- FFP parameters for UE-initiated COT can be provided to the UE by at least dedicated RRC signaling.

  - FFS on to be provided by SIB-1

- FFS whether the UE FFP periodicity is explicitly configured, or implicitly determined based on other higher layer parameters

### 5.1.2	Agreements in RAN1#103-e

Agreements:

- In semi-static channel access mode, a single FFP (periodicity and offset) is associated to an initiating device (gNB or UE) at a given time which can be used for the purpose of channel occupancy. The FFP configuration that is used for initiating channel occupancy purposes, is such that it shall not be changed for at least 200ms

Conclusion:

- For operation on unlicensed channels and irrespective of the adopted LBT mechanism (LBE or FBE), all transmissions in DL and UL are controlled by gNB similarly to licensed channels, and potential collisions or blocking are controlled/mitigated by gNB.

Agreements:

  - UE-to-gNB COT sharing in semi-static channel access mode with a gap > 16us is supported

Conclusion:

If a device X at a given time is initiating a COT, the applicable FFP for the device X is the FFP associated with X.

If a device X at a given time is sharing a COT initiated by a device Y, the applicable FFP for the device X is the FFP associated with Y.

Note 1: One of the devices X and Y is a UE and the other is its serving gNB.

Note 2: Whether or not there is additional restriction on idle period is still FFS.

Agreements:

Down-select one of the following options (target RAN1#104-e):

- Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

- Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16.

- Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16

- Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE.

- Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions.

Agreements:

- The gNB configures a UE to initiate semi-static CO in an unlicensed channel(s) only if the gNB configures the UE also with the higher layer parameters of the gNB’s initiating semi-static CO in the same channel(s).

  - Note: UE initiated FBE configuration is configured per serving cell

Agreements:

In semi-static channel access mode, FFP Period for UE-initiated COT is separately provided from FFP period for gNB-initiated COT.

o    Note: Any value for the period, shall be at least 1ms and at most 10ms.

o    Note: Aim for low complexity operation to handle gNB and UE COT interactions

Agreements:

In semi-static channel access mode, a UE should be able to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT.

- UE determines the initiator of a COT based on at least one of the following alternatives:

  - Alt 1: Introduce additional bit field in the scheduling DCI

  - Alt 2: Based on ChannelAccess-CPext field in DCI

  - Alt. 3: Based on a predetermined rule(s)

  - Alt. 4: Based on RRC signalling

  - Alt. 5: Based on MAC CE

  - FFS other alternatives

- FFS on overriding possibility and/or the assumption

- Note: A scheduled UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

Agreements:

In semi-static channel access mode:

- When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, down-select one of the following:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

  - Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration.

- When a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE:

  - If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Otherwise, If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT.

- FFS on other conditions for determining the corresponding UE or gNB initiated COT

- Note: A configured UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

### 5.1.3	Agreements in RAN1#104-e

Agreement:

- PUSCH repetition Type B is supported for unlicensed band operation when using NR IIoT Rel-16 based CG

  - FFS whether/how to enhance

Agreement:

- In semi-static channel access mode, UE FFP periodicity is chosen from the following set of values in ms: {1, 2, 2.5, 4, 5,10}.

  - FFS on other values

Agreement:

- In semi-static channel access mode:

  - An FFP period for UE-initiated COT is configured as the same, integer multiple of, or inter-factor of the FFP period configured for gNB-initiated COT

  - FFP period for UE-initiated COT can be configured independently from FFP period of gNB-initiated COT, if the UE indicates the corresponding capability

  - FFP offset for UE-initiated COT is the starting point of first UE FFP relative to the radio frame X boundary.

    - The offset value range is 0 ≤ offset ＜FFP period of UE-initiated COT

      - FFS on X (e.g. X=0, or X= even index number)

Agreement:

In semi-static channel access mode when a UE can operate as initiating device,

- Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

  - Alt-b: Determination based on the rules applied for a configured UL transmission

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Agreement:

- In semi-static channel access mode, sharing a UE initiated COT through the gNB to other intra-cell UEs for UL transmissions, is not supported.

### 5.1.4	Agreements in RAN1#104bis-e

Agreements:

- Support explicit RRC configuration for the UE-FFP parameters including period and offset in RRC connected mode.

Agreements:

- For semi-static channel access mode, the offset value for configuration of a UE-FFP for a serving cell has a symbol level granularity.

The following agreements were made during the GTW on 16th:

Agreement:

- For semi-static channel access mode, in addition to the agreed set of period values for configuration of a UE-FFP for a serving cell:

  - Do not support any additional period value

Agreement:

- For semi-static channel access mode, the starting point of first UE FFP for a serving cell

  - is relative to the boundary of the radio frame of even index number (i.e. X=even indexed number in RAN1#104-e agreement).

Agreement:

- In semi-static channel access mode, the gNB can schedule by a DCI UL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI.

  - The UL transmission can occur only if the corresponding channel access requirements are met.

    - FFS on details.

Agreement:

- In semi-static channel access mode, the gNB can schedule by a DCI DL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI.

  - The DL transmission can occur only if the corresponding channel access requirements are met.

    - FFS on details.

Agreement:

- Select one of the following options (aiming for RAN1#105-e):

  - Option 1: Do not support PUSCH repetition Type Bwhen using based on NR-U Rel-16 based CG for unlicensed band operation.

  - Option 2: Support enhancements of PUSCH repetition Type B when using based on NR-U Rel-16based CG for unlicensed band operation. FFS whether/how to enhance

Agreements

- For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether PUSCH segmentation should take into account the idle period of an FFP.

  - FFS on details

Agreements

- For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted. FFS on details

Conclusion:

- In semi-static channel access mode, a UE as an initiating device, is allowed to transmit during the idle period of any FFP associated with the serving gNB if the UE transmission is based on UE initiated COT

  - Note: the gNB may disallow UL transmission during symbols of the idle period by configuring them either as semi-static DL symbols, or indicating them as DL with SFI.

Agreement:

- Option 2-b and option 3 are not considered further for the agreement in RAN1#103-e regarding CG harmonization

### 5.1.5	Agreements in RAN1#105-e

Agreement:

- Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE.

- Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- To determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

Agreement:

In semi-static channel access mode when a UE can operate as initiating device,

- To determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

### 5.1.6	Agreements in RAN1#106-e

Agreement

In semi-static channel access mode, the content in a scheduling DCI that indicates the assumption on the COT-initiator for the scheduled transmission is determined based on the channel access field in the DCI.

Agreement

In semi-static channel access mode,

- The inclusion of the channel access field in Rel-16 DCI 0_1 and 1_1 in Rel-17 DCI 0_2 and 1_2, respectively, is supported.

Agreement

In semi-static channel access mode, the size of channel access field in a scheduling DCI with format 0_0/1_0, 0_1/1_1, 0_2/1_2 is 2 bits.

Agreement

In semi-static channel access mode, the content of the channel access field in a DCI scheduling a UL transmission for a UE determines an index to a row in Table 1

TABLE 1

| Bit field mapped to index | Channel Access Type | The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211] | Initiator of a channel occupancy associated to UL transmission described in Clause x.x in TS 37.213 |
| --- | --- | --- | --- |
| 0 | No sensing as defined in Clause 4.3 in TS 37.213 | 0 | gNB |
| 1 | No sensing as defined in Clause 4.3 in TS 37.213 | 2 | gNB |
| 2 | 9us sensing within a 25us interval as defined in Clause 4.3 in TS 37.213 | 0 | gNB |
| 3 | 9us sensing as defined in Clause x.x in TS 37.213 | 0 | UE |

- Note: The last row in Table 1 is only applicable when the UE can operate as an initiating device as configured by gNB.

- Note 1: The intention of Clause x.x above is to describe the LBT procedure from a UE perspective when this operates as initiating device.

- Note 2: A UE operating as initiating device may transmit an UL transmission burst(s) within its u-FFP immediately after sensing the channel to be idle for at least a sensing slot duration  if the gap between the UL transmission burst(s) and any previous transmission burst is more than

Conclusion

Any UL or DL transmission that is expected to occur, should be associated to a Channel Occupancy (CO) with a corresponding FFP. When a transmission is associated to a CO with a corresponding FFP:

- The association of the transmission to a CO with corresponding FFP is based on either of the following assumption:

  - “Initiating COT”: This assumption implies that the transmission would initiate a CO corresponding the FFP.

  - “Sharing COT”: This assumption implies that the transmission would share a CO corresponding to the FFP.

- The association assumption is validated as follows:

  - “Initiating COT” assumption is validated if the transmission would start at the FFP boundary and would end before idle period of the FFP.

  - “Sharing COT” assumption is validated if the transmission would start after the FFP boundary and would end before idle period of the FFP and the CO corresponding to the FFP is initiated.

- A transmission based on a CO association assumption can occur if the CO association assumption is validated and if the following sensing conditions are met:

  - For CO association assumption as “Initiating COT”:

    - If a CCA is successful before the transmission.

  - For CO association assumption as “Sharing COT”

    - If the gap between the beginning of the transmission and the end of previous one sharing the same CO in that FFP is more than 16us and if a CCA is successful before the transmission.

    - IF the gap between the beginning of the transmission and the end of previous one sharing the same CO in that FFP is at most 16us

Agreement

In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include at least scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

- FFS whether/how the DL transmission burst can include transmission to any other UE in the cell than the COT initiating UE and/or broadcast transmission while ensuring that the COT initiated by the UE is not shared by any other UE in the cell for any UL transmission

Agreement

In semi-static channel access mode, when the gNB schedules by a DCI a UL transmission in a later g-FFP that is different from the g-FFP that carries the scheduling DCI:

- The UE follows the indicated COT initiator as the following:

  - If the UE validates the indicated COT initiator assumption and satisfies the applicable sensing conditions, the transmission occurs. Otherwise, the transmission is dropped.

Agreement

- When a UE operates as an initiating device, and the gNB shares a UE’s FFP for DL transmission, regardless of the gap between any UL and DL bursts, no restriction is imposed on the maximum duration of each of the DL bursts such that each can continue until the UE FFP idle period starts.

    - Note: The applicability of the EDT calculation based on the UE’s transmit power to the UE COT initiation in accordance to the UL-DL gap duration and/or the content of the DL burst is separately discussed

Conclusion

There is no consensus in RAN1 to support UE-initiated COT for semi-static channel occupancy in IDLE/INACTIVE mode.

Agreement

Do not support PUSCH repetition Type B based on NR-U Rel-16 CG for unlicensed band operation.

## 5.2	List of observations and proposals in contributions

### R1-2109606	Intel Corporation	Further Details for Enabling URLLC IIOT in Unlicensed Band

Proposal 1: When A UE operates as an initiating device, the gNB can transmit only unicast data and control to the UE that initiated the FFP that the gNB is sharing.

Proposal 2: When a UE operating as initiating device acquires its FFP, in any circumstances the ED threshold used to determine whether the channel is busy or idle is calculated solely based on the UE’s transmit power.

Proposal 3: When the cg-RetransmissionTimer-r16 is enabled and a UE operates as an initiating device, the COT sharing information field which is included within the CG-UCI indicates only the length of the shared resources and an offset, which indicates the exact starting symbol from when the gNB may be able to use those resources regardless of the slot boundary.

Proposal 4: When the cg-RetransmissionTimer-r16 is disabled and a CG UE operates as an initiating device, the same procedure established for DG UEs in Rel.16 is reused.

Proposal 5: When operating on multiple carriers, the assumptions regarding the COT initiator are aligned across all carriers/ LBT BWs.

Proposal 6: If, when operating on multiple carriers, the assumptions regarding the COT initiator are aligned across all carriers/ LBT BWs, a UE could assume to operate:

as an initiating device over all RBs if for at least one LBT BW i) the UE assesses that it shall operate as initiating in that LBT BW or ii) the UE has received indication from the gNB that it shall operate as an initiating device; or

as a responding device over all RBs, if for each LBT BW i) the UE assesses that it shall operate as a responding device or ii) the UE has received indication from from gNB that it shall operate as responding device.

Notice that before the UE can actually perform a transmission and decide whether to operate as initiating or responding device, it must assess whether the channel access requirements are met over all the RBs.

Proposal 7: When the cg-RetransmissionTimer is enabled, the CG-UCI is regarded as high priority when HARQ transmission is not present. When the cg-RetransmissionTimer is enabled, if cg-UCI-Multiplexing is configured the CG-UCI can be jointly multiplexed with LP HARQ-ACK, when HP HARQ-ACK is not present, otherwise if cg-UCI-Multiplexing is not configured, the entire PUSCH, including the CG-UCI, is dropped.

Proposal 8: When the cg-RetransmissionTimer is enabled, if both HP and LP HARQ-ACK are to be multiplexed onto a CG-PUSCH that includes CG-UCI, CG-UCI is jointly encoded with HP HARQ-ACK with same beta offset.

Proposal 9: If useInterlacePUCCH-PUSCH is configured, the cancellation indication carried in DCI format 2_4 is applied to an entire interlace, if at least one PRB of that interlace belongs to the indicated timeFrequencyRegion for that cancelation indication.

Proposal 10: In semi-static channel access mode, early termination or cancellation of a FFP is enabled by allowing the gNB to overwrite through DCI scheduling indication any prior decision regarding the initiator of the COT.

Proposal 11: When a gNB configures CG-startingOffset-r16, a Rel.17 UE never applies the offset if the CG PUSCH aligns with a u-FFP, and the UE operates as initiating device regardless of whether the UE is configured to operate in partial or full bandwidth. However, the CG UE applies the offset in all other cases.

Proposal 12: Independently on whether cg-RetransmissionTimer is enabled or disabled, multi-TB transmission should be supported to fully utilize the MCOT available.

Observation 1: When operating in unlicensed spectrum, the orphan symbol deriving from segmentation is highly detrimental for transmissions within either a UE or a gNB’s initiated COT.  Therefore, RAN1 should discuss how to prevent a UE from performing an additional LBT due to the occurrence of an orphan symbol.

Proposal 13: Do not support segmentation across the idle period. However, dropping is applied to any repetition which overlaps with an idle period where the UE is not allowed to transmit.

Proposal 14: When the cg-RetransmissionTimer is enabled, DCI 0_2 should be enhanced to carry the DFI information based on configuration.

### R1-2110180	Qualcomm Incorporated	Uplink enhancements for URLLC in unlicensed controlled environments

Proposal 1: Study the scheme of indication of gNB sharing UE-initiated COT for DL transmission to disable UE sharing the COT.

Proposal 2: Study ED thresholds selection when UE share its COT to gNB.

Observation 1: Either supporting UE initiated COT in IDLE/INACTIVE mode or allowing PRACH transmission in idle period can provide more chances for the UE to send PRACH.

Proposal 3: Study the following alternatives for PRACH transmission in idle mode:

Alt.1: Supporting UE initiated COT by PRACH transmission in idle mode;

Alt.2: Allowing PRACH transmission in idle period of an FFP.

Proposal 4: Study the following two alternatives for SSB to PRACH mapping:

Alt.1 Divide PRACH occasions into two groups and SSB is mapped to PRACH occasion per group;

Alt.2: Introduce two PRACH configurations and SSB is mapped to PRACH occasions per PRACH configuration.

Proposal 5: Study the following alternatives for MsgA transmission in idle mode:

Alt.1: Supporting UE initiated COT by MsgA transmission in idle mode;

Alt.2: Allowing MsgA transmission in idle period of an FFP.

Proposal 6: Study the following for RO-to-PO mapping:

Alt.1: Divide PUSCH occasions into two groups and PRACH occasion is mapped to PUSCH occasion per group;

Alt.2: Introduce two sets of PUSCH configurations and each PUSCH configuration is associated with one PRACH configuration.

Proposal 7: For LBE, configuration of (cg-RetransmissionTimer) should be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum.

Proposal 8: NR-U CG-PUSCH shall support type A PUSCH repetition introduced in Rel.16 URLLC by reinterpreting the # of repetitions in consecutive slots as the # of repetitions in consecutive transmission occasions.

Proposal 9: CP extension can be used to handle the non-transmission of orphan symbol for Tpye B PUSCH repetition

### R1-2108968	vivo	Enhancements for unlicensed band URLLC IIoT

Observation 1: There are different understandings regarding the CG UL transmission confined within the gNB FFP before the idle period of that FFP in both time-domain and frequency domain. Clarification within the group is necessary.

Observation 2: For CG transmission, Alt-a will cause misunderstanding between the gNB and the UE on the COT-initiator due to miss detection and other factors like DRX off etc.

Proposal 1: At least broadcast and group common signals/signaling can be transmitted in the UE-initiated COT.

Proposal 2: For gNB-to-UE COT sharing detection, the following options can be further considered:

Option 1: explicit gNB-to-UE COT sharing indication in DCI.

Option 2: DL signal detection from dedicated positions.

Proposal 3: The COT initiator information should be included in the CG UL transmission.

Proposal 4: Each wideband transmission should be transmitted within a single COT.

Proposal 5:

The following RRC parameters are NOT needed when cg-RetransmissionTimer is configured for CG operation with shared spectrum channel access.

pusch-RepTypeIndicator

startingFromRV0

The RRC parameter of phy-PriorityIndex should be applicable for CG operation in unlicensed band.

Proposal 6: It is necessary to harmonize the parameter of cg-UCI-Multiplexing for CG by taking into account Rel-16 and Rel-17 intra-UE prioritization/multiplexing mechanism.

Proposal 7: When cg-RetransmissionTimer is configured and Rel-16 intra-UE prioritization/multiplexing mechanism is used,

if there is resource overlapping between the PUCCH carrying HARQ-ACK and CG-PUSCH with different L1 priorities, regardless whether cg-UCI-Multiplexing is provided or not, UE transmits the channel (either PUCCH or CG-PUSCH) with high priority and does not transmit the channel with low priority;

if there is resource overlapping between the PUCCH carrying HARQ-ACK and CG-PUSCH with the same L1 priority,

if the UE is provided cg-UCI-Multiplexing, CG-UCI and HARQ-ACK are jointly encoded; otherwise, the UE does not transmit the PUSCH and multiplexes the HARQ-ACK information in a PUCCH transmission or in another PUSCH transmission.

Proposal 8: When cg-RetransmissionTimer is configured and Rel-17 intra-UE prioritization/multiplexing mechanism is used, that is multiplexing UCI in a PUSCH with different L1 priorities,

if there is resource overlapping between the PUCCH carrying HARQ-ACK and CG-PUSCH with different L1 priorities, following options can be considered to re-interpret cg-UCI-Multiplexing:

Option 1: If the UE is provided cg-UCI-Multiplexing, CG-UCI and HARQ-ACK are jointly encoded; otherwise, the UE transmits the channel (either PUCCH or CG-PUSCH) with high priority and does not transmit the channel with low priority.

Option 2: Regardless whether the UE is provided cg-UCI-Multiplexing, CG-UCI and HARQ-ACK are separately encoded and multiplexed in CG-PUSCH.

### R1-2109972	LG Electronics	Discussion on unlicensed band URLLC IIOT

Proposal #1: Consider to support dynamic indication of whether to allow UE-initiated COT for the next FFP based on the transmission of UE (group)-common DCI, at least for the control of potential congestion among multiple UEs in a same FFP.

Structure of the common DCI signaling (with indication of COT duration and SFI information) designed in Rel-16 NR-U can be reused.

Proposal #2: Consider to allow the following UE behaviour for the scheduled UL not aligned with FFP-u boundary.

The UE would drop the scheduled UL transmission in case when gNB indicates UE-initiated COT based TX for the UL, but the UE didn’t initiate COT for the FFP-u period.

Proposal #3: Consider the determination/validation on the COT initiator for the scheduled UL transmission based on cross-CC scheduling.

The validation of gNB-initiated COT (based on the detection of DL transmission) can be skipped for the scheduled UL by cross-CC (and same FFP-g) scheduling.

Proposal #4: Consider to define the FFP including or starting with essential DL/UL transmission occasions (such as SSB or CORESET#0) as default FFP-g.

Proposal #5: Consider to align the assumption of COT initiator for multiple RB sets within a same carrier as the following way.

The UE would assume gNB-initiated COT for all RB sets, if at least for one RB set (a) the UE detected DL transmission based on gNB-initiated COT or (b) the UE has received indication from the gNB that it is based on gNB-initiated COT.

The UE would assume UE-initiated COT for a RB set, if the UE didn’t detect DL transmission based on gNB-initiated COT for any of RB sets and if the UE decided to assume UE-initiated COT for the RB set.

Proposal #6: Consider to configure (limit) the maximum COT duration allowed by the UE within a FFP-u period for gNB control of UE multiplexing.

Proposal #7: Consider to allow the DL transmission based on sharing of a UE-initiated COT as the following way.

The DL transmission (based on sharing of a UE-initiated COT) only include the transmission to the COT-initiating UE in case where there is UL resource allocated for other UE than the COT-initiating UE after the DL reception.

The above DL transmission can include the transmission to any other UE in case where there is no UL resource allocated for any UE after the DL reception or there is only UL resource allocated for the COT-initiating UE after the DL reception.

Proposal #8: Consider the following aspects for the configuration of CG PUSCH.

A same CG type (e.g., Rel-16 NR-U CG type or Rel-16 URLLC CG type) is configured per cell.

How to select a CG PUSCH for the multiplexing of UCI (e.g. HARQ-ACK) needs to be further studied by considering multiple cells configured with different CG type and the UL skipping for NR-U CG due to the collision with HARQ-ACK PUCCH.

Proposal #9: Consider not to allow transmission of the configured UL in the idle period of FFP-g located within a FFP-u period even if the UE has initiated COT for the FFP-u, in order to avoid potential UE-to-gNB interference.

Proposal #10: Consider to support transmission of the orphan symbol created by PUSCH (repetition type B) segmentation, to avoid unnecessary LBT behaviour as well as undesirable PUSCH dropping (due to LBT failure).

### R1-2108729	Huawei, HiSilicon	Uplink enhancements for URLLC in unlicensed controlled environments

Proposal 1: It should be agreed in RAN1#106bis-e to extend the channel access procedures for consecutive UL transmissions specified in Rel-16 for the dynamic channel access mode to the semi-static channel access mode in Rel-17.

Proposal 2: In semi-static channel access mode, when the gNB schedules by a DCI a UL transmission in a later u-FFP in the same g-FFP that carries the scheduling DCI and the transmission is not aligned with the later u-FFP boundary, the UE follows the indicated COT initiator as the following:

If the UE validates the indicated COT initiator assumption and satisfies the applicable sensing conditions, the transmission occurs. Otherwise, the transmission is dropped.

Observation 1: UEs would not be aware of the FFP frame start points of each other on the same channel, avoiding mutual blocking/collisions among these UEs (or interlaced UE groups) through gNB’s semi-static configuration becomes quite intricate.

Configuring an FFP for a second UE or interlaced UE group would trigger reconfiguration of all impacted UL resources for the first operating URLLC UE/interlaced UE group

Observation 2: For gNB to control the collisions/blocking between UEs on the same channel, the existing mechanisms to cancel/prevent UL transmissions cannot be applied to all configured UL and/or incur increased dynamic overhead while targeting semi-static periodic resources.

Proposal 3: On the semi-static configuration of UE-initiated FFP in a given unlicensed channel, the UE should be provided with a parameter to limit its COT to an indicated duration. Configured UL resources for PUSCH/PUCCH/SRS can be masked/restored back in all u-FFP periods by providing/updating this parameter without reconfiguring all impacted UL resources.

Proposal 4: In semi-static channel access mode in Rel-17, restrict the content of the DL burst sharing UE initiated COT to only transmissions intended to the initiator UE.

Proposal 5: For semi-static channel access in unlicensed controlled environment, support gNB sharing of the CO initiated by the UE, without the UE adjusting the EDT, for transmissions including unicast user plane data to only the same UE.

Proposal 6: Support configuration of harq-ProcID-Offset2 for operation in unlicensed spectrum when the cg-RetransmissionTimer-r16 is not configured.

Proposal 7: If a nominal repetition overlaps with an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT or associated to UE’s FFP in case UE assumes UE-initiated COT, the nominal repetition is dropped (i.e. no segmentation around the idle period).

Proposal 8: Given the operation in unlicensed controlled environment, orphan symbol(s) should be dropped as in Rel-16 even if they are between two actual repetitions that are transmitted.

### R1-2108842	ZTE	Discussion on unlicensed band URLLC IIoT

Proposal 1: After introducing UE-initiated COT, one mechanism should be provided to let UE determine whether to share the COT based on explicit or implicit indication.

Proposal 2: ED threshold adjustment for semi-static channel access mode is not considered.

Proposal 3: For PUSCH repetition Type B on unlicensed spectrum, if a nominal repetition overlaps with an idle period associated to gNB’s FFP or UE’s FFP in case UE shares gNB-initiated COT, or associated to UE’s FFP or gNB’s FFP in case UE initiates COT, the nominal repetition within the idle period is segmented. All the symbols in the idle period should be considered as invalid symbols, which are not considered for an actual repetition as in Rel-16.

Proposal 4: For PUSCH repetition Type B enhancements on unlicensed spectrum, DMRS can be transmitted on the orphan symbol(s) if they are between two actual repetitions that are transmitted.

Proposal 5:  For the interaction with DL/UL directions for Type 1 CG PUSCH and Type 2 CG PUSCH without the first PUSCH (including all the repetitions), Rel-16 NR-U feature is used with modifying the repetition to actual repetition.

If dynamic SFI is not received and EnableConfiguredUL-r16 is not provided, the actual repetition is not transmitted if it conflicts with a semi-static flexible symbol.

If dynamic SFI is not received but EnableConfiguredUL-r16 is provided, the actual repetition can be transmitted.

Proposal 6: If PHY priority introduced in Rel-16 is supported in unlicensed band, how to handle the multiplexing and PHY prioritization of CG-UCI and HARQ-ACK codebooks should be considered.

Observation 1: In semi-static channel access mode, the absence of any other technology sharing the channel can be guaranteed on a long-term basis, and ED threshold calculation is not relevant with gNB’s or UE’s transmit power.

Observation 2: For PUSCH repetition Type B on unlicensed spectrum, in case UE shares gNB-initiated COT, if the symbols at the beginning of the COT are the invalid symbols, the PUSCH repetition can not be transmitted.

### R1-2109138	Nokia, Nokia Shanghai Bell	UL enhancements for IIoT URLLC in unlicensed controlled environment

Observation 1: Back-to-back PUSCH repetitions are already supported with NR-U as part of Type A repetitions.

Proposal 1: Non-back-to-back Type A repetitions are not supported in unlicensed band.

Proposal 2: 	For PUSCH repetition Type B on unlicensed spectrum, select Alt 2:

If a nominal repetition overlaps with an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT or associated to UE’s FFP in case UE assumes UE-initiated COT, the nominal repetition is dropped (i.e. no segmentation around the idle period).

Proposal 3: 	For PUSCH repetition Type B enhancements on unlicensed spectrum, orphan symbols are filled with a repetition of a previous or a following symbol.

Proposal 4: Clarify further whether phy-PriorityIndex can be configured simultaneously with cg-retransmissionTimer.

Proposal 5: In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, may include a scheduled DL transmission or a DCI intended for any UE, as long as it also includes a DL transmission or a DCI intended for the UE that initiated that FFP.

Observation 2: With semi-static channel occupancy, the mechanism used by a legacy (Rel-16) UE to detect and share a serving gNB COT needs further clarifications if UE-to-gNB COT sharing is supported in Rel-17.

Proposal 6: Clarify how to prevent Rel-16 UEs from erroneously assuming that a gNB transmission in a UE initiated COT implies a presence of a gNB initiated COT.

Support for DCI 0_2 and 1_2 with dynamic channel access

Proposal 7: Introduce new RRC parameters ul-AccessConfigListDCI-0-2 and ul-AccessConfigListDCI-1-2 to support indication of CP extension, LBT type, and CAPC with DCI 0_2 and 1_2 with dynamic channel access.

### R1-2109810	ETRI	Enhancements for unlicensed band URLLC IIoT

Proposal 1: The symbol offset for UE FFP configuration is determined based on the smallest SCS among the configured SCSs in the serving cell.

Proposal 2: Clarify whether the symbol offset of zero for UE FFP configuration is needed and whether the specification allows it.

Proposal 3: Clarify whether availability/unavailability condition(s) for UE’s COT initiation need to be captured in the specification.

Proposal 4: Revise the agreement in RAN1#106-e as follows:

Agreement

In semi-static channel access mode, when the gNB schedules by a DCI a UL transmission in a later g-FFP that is different from the g-FFP that carries the scheduling DCI:

The UE follows the indicated COT initiator as the following:

If the UE validates the indicated COT initiator assumption and satisfies the applicable sensing conditions, the transmission occurs. Otherwise, the transmission is dropped.

Note: The two g-FFPs may belong to the same channel, or may belong to different channels within the same carrier or across carriers.

Observation 1: For scheduled UL based on cross-FFP scheduling or for configured UL, UE should receive a DL signal other than a UL grant to be granted for PUSCH transmission within the same FFP.

Observation 2: The UL reliability performance of unlicensed URLLC can be severely degraded if UE’s processing time for DL detection to share a COT is not known to gNB.

Proposal 5: For gNB-to-UE COT sharing, define a UE processing time for detection of the DL signal granting UL authorization (and UL preparation).

Proposal 6: For UE-to-gNB COT sharing, consider defining processing time for gNB’s UL burst detection for UE power saving purpose.

Proposal 7: For FBE, a symbol overlapping with idle period of a FFP associated to PUSCH transmission is regarded as invalid symbol for PUSCH mapping type B.

Proposal 8: Do not support a special handling of orphan symbol(s) for PUSCH repetition type B for FBE.

Proposal 9: CG-UCI has an additional field which indicates whether HP UCI and LP UCI are multiplexed or not.

### R1-2109784	Sony	Considerations on unlicensed URLLC

Observation 1: It is beneficial for flexibility and latency purposes that the gNB is able to schedule another UE when transmitting under a UE initiated COT.

Observation 2: Using Dynamic SFI to cancel a UE COT has limited scope since only Flexible symbols can be changed, it prevents other UEs from initiating a COT and it reduces gNB scheduler flexibility.

Observation 3: The gNB can avoid PUSCH segmentation for a CG-PUSCH repetition by proper configuration of the TO’s.

Observation 4: An orphan symbol is likely caused by segmentation of CG-PUSCH around an Idle Period and the UE will have to perform LBT for transmission after the Idle Period.

Proposal 1: Allow the gNB to cancel a UE initiated COT.  A COT cancellation indicator can be introduced to dynamically indicate to a UE to cancel its initiated COT.

Proposal 2: If a nominal repetition overlaps with a set of symbols in an Idle Period associated to gNB’s FFP in case UE shares gNB-initiated COT for the nominal repetition or associated to UE’s FFP in case UE assumes UE-initiated COT for the nominal repetition, all the symbols in the Idle Period should be considered as invalid symbols which are not considered for an actual repetition as in Rel-16.

Segmentation before and/or after the Idle Period is applied when applicable.

The UE follows the COT initiating rule in determining the COT initiator

Proposal 3: Orphan symbols are dropped for PUSCH repetition Type B in unlicensed band operation.

### R1-2109942	Lenovo, Motorola Mobility	Enhancements for unlicensed band URLLC IIoT

Proposal 1: In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

Proposal 2: With medium priority, support

DL transmission burst being capable of including transmission to any other UE in the cell than the COT initiating UE and/or broadcast transmission.

group-common DCI at the beginning of a gNB-FFP can indicate whether the COT is a gNB-COT.

Proposal 3: UE should not drop from the beginning a first low-priority configured UL transmission initiating a UE-COT that overlaps with a later second high-priority configured UL transmission.

Proposal 4: UE should not drop a first low-priority configured UL transmission that overlaps with a later second high-priority configured UL transmission earlier than 16 us before the start of the second high-priority configured UL transmission.

Proposal 5: Discuss dynamic indication to change UE’s assumption on the associated COT initiator with low priority considering the required specification impact.

Proposal 6: For the case of UE-initiated COT with configured grant PUSCH transmission, the transmit power at the beginning of the acquired FFP can be higher than the transmit power associated with PUSCH transmissions of the configured grant (in transmission occasions other than those of the beginning of the acquired FFP).

Proposal 7: When a first transmission burst is followed by a high priority UL transmission burst on a CG resource, if the gap is more than 16µs between the two transmissions, the CP of the high priority UL transmission is extended to keep the effective gap under 16µs.

Observation 1: Dynamic indication to change UE’s assumption on the associated COT initiator for an UL transmission could allow transmission within the idle period of the former FFP as determined based on the scheduling DCI if the scheduled UL transmission overlaps with the idle period.

a timeline needs to be specified for receiving the dynamic indication e.g., w.r.t. the scheduling DCI/ UL transmission.

Observation 2: Enforcing the same COT initiator across the RB sets seems unnecessary considering potential collisions or blocking are controlled/mitigated by gNB.

### R1-2109894	InterDigital, Inc.	Enhancements for unlicensed band URLLC IIoT

Proposal 1: A UE sends an indication of the COT used for a configured transmission (gNB-initiated or UE-initiated).

Proposal 2: A UE is indicated the COT initiator associated to a DL transmission.

Proposal 3: PUSCH Type B repetition is enhanced such that segmentation considers LBT, idle period of an FFP and COT duration.

Proposal 4: A nominal PUSCH Type B repetition overlapping a COT boundary is segmented into two actual repetitions.

Proposal 5: Allow transmission on orphan symbols. FFS conditions when to transmit on orphan symbols and contents of orphan symbol transmission.

### R1-2109453	Panasonic Corporation	Enhancements for unlicensed band URLLC IIoT

Proposal 1: Multiple starting time offset for configured grant, which is configured as the amount of CP extension, can be reused to support UE-initiated COT.

Proposal 2: It should be clarified that whether the difference of CP extension is called as the change of FFP or not.

Proposal 3: If DG PUSCH is used for UE-initiated COT together with CG PUSCH, to support CP extension for multiple starting time offset as in CG PUSCH for DG PUSCH could be considered.

Proposal 4: If the difference of CP extension is called as the change of FFP, the start of FFP might be always CG PUSCH if DG PUSCH does not have CP extension. If DG PUSCH supports CP extension, the amount of CP extension for DG PUSCH should be same as that configured to CG PUSCH.

Proposal 5: For PUSCH repetition Type B on unlicensed spectrum, if a nominal repetition overlaps with a set of symbols in an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT for the nominal repetition or associated to UE’s FFP in case UE assumes UE-initiated COT for the nominal repetition, all the symbols in the idle period should be considered as invalid symbols which are not considered for an actual repetition as in Rel.16.

Proposal 6: For PUSCH repetition Type B on unlicensed spectrum, orphan symbol(s) are dropped as in Rel.16.

### R1-2108907	Spreadtrum Communications	Discussion on enhancements for unlicensed band URLLCIIoT

Proposal 1: Option 2: Orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted.

Proposal 2: Padding can be copy of the former end symbol or later starting symbol.

Proposal 3:  Alt 1: If a nominal repetition overlaps with a set of symbols in an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT for the nominal repetition or associated to UE’s FFP in case UE assumes UE-initiated COT for the nominal repetition, all the symbols in the idle period should be considered as invalid symbols which are not considered for an actual repetition as in Rel-16.

Segmentation before and/or after the idle period is applied when applicable.

### R1-2109673	NTT DOCOMO, INC.	Discussion on enhancements for unlicensed band URLLC

Proposal 1:

In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include at least scheduled DL transmission or a DCI intended for the UE that initiated the COT

The DL transmission burst can also include transmission to any other UE in the cell than the COT initiating UE and broadcast transmission

Proposal 2:

For semi-static channel access when a UE operating as an initiating device acquires its FFP, support gNB sharing of the CO initiated by the UE with a sensing ED threshold that is calculated based on the UE’s transmit power

Note: no additional restriction is necessary for the contents of the DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, as long as it include at least scheduled DL transmission or a DCI intended for the UE that initiated the COT

Proposal 3:

Align the assumption regarding the COT initiator device of a FFP across all multiple RB sets in a carrier/BWP under the unaligned FFP structure between UE and gNB

Following can be considered as starting point to guarantee/determine the COT initiator across all multiple RB sets:

a UE can assume to operate:

as an initiating device over all RB sets if for at least one RB set i) the UE assesses that it shall operate as initiating in that RB set or ii) the UE has received indication to the gNB that it shall operate as an initiating device; or

as a responding device over all RB sets, if for each RB set i) the UE assesses that it shall operate as a responding device or ii) the UE has received indication from the gNB that it shall operate as responding device.

Proposal 4:

For PUSCH repetition Type B enhancements on unlicensed spectrum, support Alt 1, i.e.,

If a nominal repetition overlaps with a set of symbols in an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT for the nominal repetition or associated to UE’s FFP in case UE assumes UE-initiated COT for the nominal repetition, all the symbols in the idle period should be considered as invalid symbols which are not considered for an actual repetition as in Rel-16.

Segmentation before and/or after the idle period is applied when applicable.

Proposal 5:

For PUSCH repetition Type B enhancements on unlicensed spectrum, orphan symbol is transmitted if it is between two actual repetitions that are transmitted for OFDM waveform.

The orphan symbol is generated as an actual repetition while UCI multiplexing is not applied

For DFT-s-OFDM waveform, Rel-16 rule (i.e., dropping the orphan symbol) is applied.

### R1-2110029	Apple	URLLC uplink enhancements for unlicensed spectrum

Proposal 1-1: UE-initiated COT is considered enabled once the FFP periodicity and offset are configured for a UE. Introduce a RRC parameter to disable UE-initiated COT for P-CSI and/or SRS.

FFS whether to introduce a RRC parameter to disable UE-initiated COT for each CG configuration, which overrides the per-UE configuration for this CG.

Proposal 1-2: When the gNB shares a UE’s COT for a DL transmission, to prevent another UE from mistakenly assuming this is gNB’s COT, adopt one of the following alternatives:

Alt 1: It is left to gNB implementation.

Alt 2: The DL transmission shall not include broadcast or UE-specific signaling/channels that can be detected by other UEs.

Proposal 2-1: For PUSCH repetition Type B enhancements on unlicensed spectrum, orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted.

Proposal 2-2: Adopt Alt 2: If a nominal repetition overlaps with an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT or associated to UE’s FFP in case UE assumes UE-initiated COT, the nominal repetition is dropped (i.e. no segmentation around the idle period).

### R1-2109576	MediaTek Inc.	On the enhancements for unlicensed band URLLC IIoT

Proposal 1: UE processing time needs to be considered in semi-static channel access mode for configured UL transmission.

Proposal 2: For a DL transmission based on sharing a UE intiated COT, other UEs can recognize that the gNB is sharing the UE COT through the gNB use of a different DMRS encoding.

Proposal 3: When operating on multiple LBT-BWs, FFP parameters and the assumptions regarding the COT initiator are aligned across all LBT-BWs

Proposal 4: In FBE mode, support enabling/disabling UE COT-initiating functionality dynamically.

Proposal 5: UE COT initiation enabling/disabling is determined based on the traffic priority.

Proposal 6: FFP parameters for UE-initiated COT could be provided by SIB-1.

Proposal 7: UE FFP periodicity determined from higher layer parameters but overridden by explicit dedicated signalling.

### R1-2108831	Ericsson	Enhancements for IIoT URLLC on Unlicensed Band

Proposal 1: In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include only scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

Proposal 2: If a nominal repetition overlaps with an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT or associated to UE’s FFP in case UE assumes UE-initiated COT, the nominal repetition is dropped (i.e. no segmentation around the idle period).

Proposal 3: For PUSCH repetition Type B on unlicensed spectrum, the orphan symbol(s) are dropped as in Rel-16.

### R1-2109217	CATT	Discussion on remaining issues on enhancements for unlicensed band URLLC IIoT

Proposal 1: In semi-static channel access mode when a UE can operate as initiating device, all the PUSCH transmissions scheduled by a single DCI is based on UE-initiated COT or sharing a gNB-initiated COT based on the content of the channel access field in the scheduling DCI.

Proposal 2: In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP cannot include transmission to any other UE in the cell than the COT initiating UE and/or broadcast transmission.

Proposal 3: For PUSCH repetition Type B for unlicensed band operation when cg-RetransmissionTimer-r16 is disabled, if one nominal repetition is divided into one or more actual repetitions due to invalid symbol(s), additional LBT window before actual repetition transmission is supported.

### R1-2109483	Samsung	Enhancements for unlicensed band URLLC IIoT

Proposal 1: For gNB-initiated COT detection, the following mechanism can be considered:

A UE assumes gNB has initiated a COT if the UE receives explicit indication in DCI 2_0.

A UE assumes gNB has initiated a COT if the UE detects DL transmission at the beginning of gNB FFP.

Proposal 2: For PUSCH repetition Type B over unlicensed band, to cope with FBE frame structure and LBT operation:

Support segmentation around idle period.

Support additional gaps to avoid LBT blocking from DL signals/channels.

No enhancement for orphan symbol.

### R1-2108788	FUTUREWEI	UE initiated COT for semi-static channel access

Proposal 1: For FBE mode of operation when gNB shares channel occupancy initiated by a UE, the gNB transmission shall contain transmission to the UE that initiated the channel occupancy and can include non-unicast and/or unicast transmissions where any unicast transmission that includes user plane data is only transmitted to the UE that initiated the channel occupancy. “

Proposal 2: Support Alt 2 i.e., no segmentation around the idle period.

Proposal 3: Support Option 1, i.e., orphan symbol(s) are dropped as in Rel-16.

### R1-2110323	WILUS Inc.	Discussion on enhancement for unlicensed URLLC IIoT

Proposal 1: We propose the following to resolve FFS issue on UE-to-gNB COT sharing.

In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include at least scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

The DL transmission burst can include transmission to any other UE in the cell than the COT initiating UE and/or broadcast transmission while ensuring that the COT initiated by the UE is not shared by any other UE in the cell for any UL transmission.

Proposal 2: It should be further discussed whether or not to possibly transmit configured-grant PUSCH with repetition at candidate SS/PBCH block positions for the same SS/PBCH block index after the detection of the SS/PBCH block index.

### R1-2109407	Xiaomi	Enhancement for unlicensed band URLLC IIoT

Proposal 1: Separate FFP configurations on different LBT bandwidths can be considered, and previous agreements for single FFP configuration on a cell can still be reused on per LBT bandwidth bases.

Proposal 2: No need to align COT-initiator assumption for multiple channels contained in a cell.

Observation 1: UL cancellation indication DCI 2-4 has been introduced to allow gNB to terminate an ongoing PUSCH(CG/DG)/SRS transmission.

Proposal 3: No further enhancement on controlling UE-initiated COT since UL cancellation indication DCI 2-4 can already apply.

### R1-2109356	NEC	Enhancements for unlicensed band URLLC IIoT

Proposal 1: Support EDT calculation based on the traffic priority to allow early transmission of high priority URLLC service.

Proposal 2: gNB may cancel a low priority UE’s transmission and release the corresponding UE initiated COT in order to support high priority URLLC transmission of another UE.

Proposal 3: Once a UE initiated COT is released by gNB, the UE may not initiate another COT for the same transmission/service until gNB reschedules its UL transmission.

### R1-2109994	Sharp	Enhancements for unlicensed band URLLC IIoT

Proposal 1: Mechanisms should be introduced to protect important DL transmissions and the corresponding channel access opportunities, especially transmission SS/PBCH block transmission.

Proposal 2: When the gNB shares a UE initiated COT, the DL transmission burst can include broadcast transmission while ensuring that the UE initiated COT is not shared by any other UE in the cell for any UL transmission.

### R1-2109095	OPPO	Enhancements for unlicensed band URLLC IIoT

Proposal 1:  cg-RetransmissionTimer can be configured for each configured grant independently.

Proposal 2: For the DCI content to determine gNB CO or UE CO, adopt Alt-1.

proposal 3: gNB can explicitly broadcast a signal to inform the UEs that the gNB initiated CO has been created.