3GPP TSG RAN WG1 #107-e	R1-2110910

e-Meeting, November 11th – 19th, 2021

Source: 	ZTE

Title: 	Discussion on mechanisms to Support Group Scheduling for RRC_CONNECTED UEs

Agenda item:	8.12.1

Document for:	Discussion/Decision

Introduction

During RAN#88-e meeting, the revised WI on NR MBS has been approved [1]. One of its objectives is to specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service for RRC_CONNECTED UEs.

In RAN1#106b-e meeting, some main issues about group scheduling mechanism for MBS transmission were further discussed, and some agreements and conclusion were made [2]. In this contribution, we focus on remaining details of mechanism to allow UEs in RRC_CONNECTED to receive Broadcast/Multicast service.

Common frequency resource

2.1	Impact on BWP-InactivityTimer

During RAN1#106-e and RAN1#106b-e meetings, potential impact on BWP-InactivityTimer by GC-PDCCH monitoring was discussed. The related options proposed by companies are listed below.

[High] Updated Proposal 1-5: If a UE is configured with a CFR in the active DL BWP, for timer-based active DL BWP switching to a default BWP, further study the following options: Option 1: UE also starts or restarts BWP-InactivityTimer when it successfully decodes a GC-PDCCH addressed to group-common RNTI (e.g., G-RNTI or G-CS-RNTI). Option 2: Introduce a new MBS-BWP-InactivityTimer for GC-PDCCH receptions. - Note: Other options are not precluded.

Currently, a UE will switch the active unicast DL BWP to the default DL BWP when BWP-InactivityTimer expires, and the BWP-InactivityTimer is reset upon reception of a DCI format with CRC scrambled by C-RNTI or CS-RNTI. Similarly, for GC-PDCCH with G-RNTI which is also applied to data scheduling, the impact on the BWP-InactivityTimer should also be considered.

For option 2 as described in [3], a new MBS-BWP-InactivityTimer can be introduced for GC-PDCCH monitoring. More specifically, if the MBS-BWP-InactivityTimer expires, UE will stop monitoring GC-PDCCH or switch to another search space set group for reducing the overhead for GC-PDCCH monitoring. From our point of views, the energy saving effect is limited. In a typical scenario, unicast PDCCH has a higher monitoring density compared with GC-PDCCH, so the energy saving effect of reducing or disabling GC-PDCCH blind detection only comes from reducing the number of blind detection within the slot, which brings limited energy saving effects.

Observation 1: the energy saving effect is limited by introducing an independent inactivity timer for GC-PDCCH reception.

As a more straightforward way, option 1 requires only a few standardization efforts, which is preferred in this later phase. Further, per current mechanism described in [4], SPS transmission for multicast should also be taken into account of BWP-InactivityTimer. So we have the following proposal.

Proposal 1: For timer-based BWP switching, UE starts or restarts BWP-InactivityTimer when it successfully decodes a GC-PDCCH addressed to group-common RNTI (e.g., G-RNTI or G-CS-RNTI) on/for the active BWP or when a MAC PDU for multicast is received in a configured downlink assignment.

2.2	CFR associated with initial BWP

During the discussion in RAN1#106-e meeting, the following agreement on CFR were made. Some remaining issues will be discussed below.

Agreement: Confirm the working assumption with the following update: Option 2B for CFR associated with UE active BWP other than initial DL BWP is supported at least for multicast of RRC-CONNECTED UEs. - FFS: CFR associated with initial BWP - FFS: CFR larger than initial BWP Note: The deleted FFSs can be discussed in another AI.

As also discussed on CFR for broadcast transmission in RRC_IDLE/INACTIVE states, if broadcast transmission can only use the initial DL BWP defined by CORESET#0, it would be too restrictive. As we know, all the common signals/channels have to be squeezed into this initial DL BWP, which makes it difficult to accommodate common PDCCH/PDSCH for broadcast services. Thus, it is agreed to configure a common frequency resource that is larger than CORESET#0 for broadcast services.

For MBS transmission in RRC_CONNECTED state, the same problem on transmission capacity also exists if CORESET#0 defined initial BWP is activated as the active BWP for a UE. Therefore, it is also reasonable to allow CFR for MBS to be larger than CORESET#0 defined initial BWP in such case.

Regarding the specific definition of CFR associated with initial BWP, the similar issue on CFR for broadcast transmission in RRC_IDLE/INACTIVE states has been discussed during the last several meetings. As noted in the above agreement, considering the consistency of solution design, CFR associated with initial BWP under RRC_CONNECTED state can follow the design of CFR under RRC_IDLE/INACTIVE states.

- Proposal 2: CFR can be configured larger than active downlink BWP when the active downlink BWP is the initial BWP defined by CORESET#0.

- Prioritize the corresponding discussion in RRC IDLE/INACTIVE state to strive for a consistent solution for all RRC states

2.3	Details of CFR configuration

Various parameters should be configured under the CFR for MBS transmission. Some of them may be the same for both unicast and MBS. It is reasonable to share the parameter configuration between unicast and MBS. That is, the parameters configured under the dedicated unicast BWP can be used for MBS transmission if the corresponding parameters are not configured under the CFR. For example, if the frequency range is not configured for the CFR, a UE can assume the MBS is transmitted within the frequency range of its dedicated unicast BWP.

Proposal 3: Regarding the CFR configuration, the parameters configured under the dedicated unicast BWP can be used for MBS transmission if the corresponding parameters are not configured under the CFR.

2.4	Multicast transmission in Scell

According to the current discussion, it is ambiguous on whether the multicast service can be transmitted over Scell or not, which requires some clarification. From our perspective, it is beneficial for improving the capacity for CA capable UE by supporting multicast transmission on the Scell. So we suggest to support multicast transmission over Scell, and CFR configured under Scell will be used for multicast reception.

Proposal 4: Group-common PDCCH/PDSCH for multicast can be received on SCell.

Detailed design of GC-PDCCH

3.1	GC-PDCCH for PTM retransmission

As agreed in the previous RAN1 meeting, both PTM scheme 1 and PTP can be used for retransmission for multicast if ACK/NACK based HARQ-ACK feedback is configured. While for NACK-only feedback, only PTM scheme 1 can be used as gNB doesn’t know which UE failed to receive the GC-PDSCH and fed back NACK.

Agreement: For RRC_CONNECTED UEs, if ACK/NACK based HARQ-ACK feedback is supported for PTM scheme 1, and if initial transmission for multicast is based on PTM transmission scheme 1, support retransmission(s) using PTP transmission. - The HARQ process ID and NDI indicated in DCI is used to associate the PTM scheme 1 and PTP transmitting the same TB.

If PTM scheme 1 is used for multicast retransmission scheduling, it is necessary to determine whether the GC-PDCCH for PTM retransmission shares the same monitoring configurations (such as, search space, CORESET, etc.) as the GC-PDCCH for PTM initial transmission. Naturally, there are two options:

- Option 1: The same monitoring configurations are shared by GC-PDCCH of PTM initial transmission and PTM retransmission;

- Option 2: Configuring separate monitoring configuration for GC-PDCCH of PTM retransmission independently.

For option 1, all the UEs in the multicast transmission group will be disturbed by this retransmission scheduling, including the UEs that have successfully received PTM initial transmission and fed back ACK since they have to decode the PDCCH and only to find out the PDCCH is scheduling retransmission. Some UE behaviors should be clarified. Such as,

- whether to further decode this PTM retransmission or not?

- whether/how to feed back to this PTM retransmission?

Option 2 is a simple way for avoiding such issues. The UE won’t monitor GC-PDCCH in MO for PTM retransmission anymore if it has already successfully received PTM initial transmission.

At this stage, from the perspective of accelerating the discussion process, option 2 is more appropriate, which can avoid some extra discussions.

Proposal 5: Monitoring configurations (e.g., CORESETs, Search Spaces, etc.) for GC-PDCCH of PTM retransmission can be configured separately from that for GC-PDCCH of PTM initial transmission.

3.2	Dynamic UEs grouping

Compared with LTE SC-PTM, one of the most significant features of NR MBS is beam sweeping. As the carrier frequency of NR can be much higher than LTE, NR has to introduce the beam sweeping for broadcast type information (such as, SSB, SIB, paging, etc.) to guarantee the same level of coverage as LTE. During discussion of beam sweeping transmission for RRC_IDLE/RRC_INACTIVE UEs, the following agreement was reached in RAN1#104-e meeting. Then, the association rule between PDCCH MOs and SSBs can be further studied.

Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, the UE may assume that group-common PDCCH/PDSCH is QCL’d with SSB. - It is up to UE implementation whether UE monitors monitoring occasions corresponding to all SSB indexes or monitoring occasions corresponding to a subset of all SSB indexes. - FFS: association rules between SSB indexes and UE monitoring occasions. - FFS: group-common PDCCH/PDSCH is QCl’d with TRS if configured.

For NR multicast transmission, UEs in the same beam direction will be divided into the same ‘multicast transmission group’, that is, a same CORESET is shared by them for GC-PDCCH monitoring, and to determine how to receive the GC-PDCCH according to the TCI state corresponding to the CORESET indicated by MAC-CE. A UE in the group should be configured with a new TCI state for GC-PDCCH monitoring if the beam direction of this UE changed.

Currently, for a UE specific PDCCH, the network can simply update the TCI state of the CORESET through a MAC-CE, so that the UE can use the new TCI state as the reference for receiving PDCCH in the original CORESET.

For multicast transmission scheduled by GC-PDCCH, the network cannot change the TCI state of this CORESET as other UEs in the same multicast transmission group still need to receive PDCCH according to the original TCI state of the original CORESET. Therefore, the only way is to configure a new CORESET for the GC-PDCCH monitoring of this UE. This requires an RRC-level reconfiguration, which will bring large delay and a great impact on the QoS of service reception.

As an example shown in Figure-1, the UE originally served under the beam2 and received the GC-PDCCH within the MO2 determined by the CORESET and searchspace configurations. After the UE moves into the service area of beam1, the network needs to configure a new CORESET with new TCI state and searchspace for the UE, to further schedule the UE to continue receiving GC-PDCCH within the MO1.

Figure-1: Grouping management of UEs receiving multicast service

Another way for changing UE group dynamically is defining beam sweeping for multicast transmission. More specifically,

- Configuring CORESET and search space for GC-PDCCH monitoring to a group of UEs receiving the multicast service

- Defining the association between GC-PDCCH MOs and SSBs/CSI-RSs, for example, GC-PDCCH MO1 is associated with RS for beam1 and GC-PDCCH MO2 is associated with RS for beam2, etc.

- Dynamic switching MOs from MO2 to MO1 for GC-PDCCH monitoring when the UE moves between different beams.

According to the above way, dynamic grouping of UEs for multicast reception can be achieved.

Proposal 6: For NR multicast, introduce beam sweeping via defining association between MOs of GC-PDCCH and SSBs or CSI-RSs.

3.3	DCI format

As agreed in RAN1#106-e and RAN1#106b-e meetings, at least two DCI formats are supported for GC-PDCCH of Rel-17 MBS. One is based on DCI format 1_0, another is based on DCI format 1_1. The remaining issue is detail fields design and DCI size alignment.

Agreement: The first DCI format for GC-PDCCH uses the same fields as DCI format 1_0 with CRC scrambled by C-RNTI with the following modifications: - At least ‘Identifier for DCI formats’ is not needed. - FFS: Whether the field should be ignored and reserved, or should be removed. - Agreement: The second DCI format for GC-PDCCH uses the same fields as DCI format 1_1 with the following modifications: - At least ‘Identifier for DCI formats’ and ‘SRS request’ are not needed. - FFS whether the fields should be ignored and reserved, or should be removed. - Note: At least the configurable fields in DCI format 1_1 remain configurable for the second DCI format Agreement: The ‘TPC command for scheduled PUCCH’ field is not needed for the first DCI format for multicast. - FFS: Whether the field should be reserved or should be removed. Agreement: The ‘TPC command for scheduled PUCCH’ field is not needed for the second DCI format for multicast. - FFS: Whether the field should be reserved or should be removed.

Reserved or removed

According to the e-mail discussion during RAN1#106b-e, different companies may have different understanding on the terminology of "reserved" and "removed". As shown in Table-1, our understanding is that Interpretation#1 refers to "reserved" and Interpretation#2 refers to "removed".

If yes, then the DCI field "TPC command for scheduled PUCCH" in Interpretation#1 is totally useless, why do we need to maintain a field for it?

Table-1 Understanding of reserved and removed

| Interpretation#1 (reserved) | DCI format 1_0 with CRC scrambled by G-RNTI: Field#1 …… TPC command for scheduled PUCCH 2bits Reserved Reserved bits N bits |
| --- | --- |
| Interpretation#2 (removed) | DCI format 1_0 with CRC scrambled by G-RNTI: Field#1 …… TPC command for scheduled PUCCH 2bits Reserved bits (N+2)bits |

Proposal 7: For DCI format of multicast, the unused fields should be removed from DCI format 1_0 for unicast.

Fields in the second DCI format

For the second DCI format for GC-PDCCH, the following fields in DCI format 1_1 should be reused.

-	Carrier indicator

-	Bandwidth part indicator

-	Frequency domain resource assignment

-	Time domain resource assignment

-	VRB-to-PRB mapping

-	PRB bundling size indicator

-	Rate matching indicator

-	Modulation and coding scheme

-	New data indicator

-	Redundancy version

-	HARQ process number

-	Downlink assignment index

-	PUCCH resource indicator

-	PDSCH-to-HARQ_feedback timing indicator

-	Antenna port(s)

-	Transmission configuration indication

-	DMRS sequence initialization

-	Priority indicator

Regarding ‘Carrier indicator’ in DCI format 1_1, it is beneficial for improving the capacity by supporting multicast transmission on the Scell as mentioned in section 2.4. And cross-carrier scheduling can also be considered as one way of multicast transmission in Scell. So ‘carrier indicator’ field in DCI format 1_1 can be reused in the second DCI format with CRC scrambled with G-RNTI.

About ‘Bandwidth part indicator’ in DCI format 1_1, it is also meaningful to switch the active BWP of a group of UEs and transmit the MBS service within the CFR associated with the new active BWP. For example, switch the active BWP of a group of UEs back to the initial BWP and receive the MBS service within the CFR associated with the initial BWP.

Some other fields, such as, Identifier for DCI formats (1 bit), scheduling information for the second transport block (8 bits), TPC command for scheduled PUCCH (2 bits), SRS request (2 bits), CBG transmission information (CBGTI) (0, 2, 4, 6 or 8 bits) and CBG flushing out information (CBGFI) (0 or 1 bit) are useless for multicast scheduling and can be removed.

Proposal 8: For the second DCI format for GC-PDCCH, fields (at least including, Identifier for DCI formats(1 bit), scheduling information for the second transport block(8 bits), TPC command for scheduled PUCCH(2 bits), SRS request(2 bits), CBG transmission information (CBGTI)(0, 2, 4, 6 or 8 bits) and CBG flushing out information (CBGFI)(0 or 1 bit) ) should be removed from DCI format 1_1 for unicast.

3.4	DCI size alignment

As confirmed in RAN1#105-e meeting, at least two DCI formats will be defined for GC-PDCCH, and the “3+1” DCI size budget defined in Rel-15 will be maintained for Rel-17 MBS. During RAN1#106-e and RAN1#106b-e, potential DCI size alignment mechanisms were discussed for both of the first DCI format and the second DCI format. For the first DCI format, it is agreed to align the size with DCI format 1_0 with CRC scrambled by C-RNTI monitored in CSS. However, no conclusion has been reached about the second DCI format.

Agreement: in RAN1#105-e Confirm the working assumption: Keep the “3+1” DCI size budget defined in Rel-15 for Rel-17 MBS. - FFS: Whether the G-RNTI is counted as “C-RNTI” or as “other RNTI” when considering the “3+1” DCI size budget rule for group-common PDCCH. Agreement: in RAN1#106-e For multicast of RRC-CONNECTED UEs, align the size of the first DCI format for GC-PDCCH with DCI format 1_0 with CRC scrambled by C-RNTI monitored in CSS.

For the second DCI format for multicast, as proposed during RAN1#106-e, the size can be configured by RRC signalling for RRC_CONNECTED UEs. And the network can ensure different UEs in the same MBS group have the same understanding on the configurable DCI fields of the second DCI format for multicast. Furthermore, it should be further studied for the rule of DCI size alignment when the size budget is exceeded.

If the size of the DCI format 1_1 for unicast has to be aligned with the second DCI format for multicast, the size of the second DCI format should be configured as a size greater than the size of DCI format 1_1 of any UE in the group. As discussed in section 3.3, several fields should be removed from DCI format 1_1 for unicast. There are above 20 bits gap between them. Then, the second DCI format should always be padded for the size alignment, which will greatly affect the detection performance of the second DCI format.

So it is better to be counted as “other RNTI”. Then, it is mainly an implementation issue of gNB for keeping number of different DCI sizes within the size budget. Lower standardization and performance impacts can be expected.

Proposal 9: Regarding DCI size alignment for the second DCI format of GC-PDCCH, it is counted as “other RNTI”, and gNB will ensure that the number of DCI sizes does not exceed budget.

3.5	Search space

During RAN1#105-e and RAN1#106-e meetings, the following agreement and conclusion on search space set type and monitoring priority for GC-PDCCH was achieved. The issue was further discussed in RAN1#106b-e meeting, companies focused on is it an editorial issue on whether the Type-x CSS is a Type-3 CSS, and which DCI format can also be transmitted in type-x CSS.

Agreement: For CSS of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, Alt 2 is supported: - Alt 2: support a Type-x CSS - The monitoring priority of Type-x CSS is determined based on the search space set indexes of the Type-x CSS set and USS sets, regardless of which DCI format of group-common PDCCH is configured in the Type-x CSS. - FFS: Whether the Type-x CSS is a Type-3 CSS Conclusion: The specification impact of having a new Type-x CSS for GC-PDCCH in RRC_CONNECTED state can be studied and discussed further. Initial proposal 2-1c: DCI format 1_0 with CRC scrambled by C-RNTI and the first DCI format for multicast can be configured in the same type-x CSS. - FFS whether the DCI format 1_0 with CRC scrambled by C-RNTI configured in type-x CSS can only be used for PTP retransmission for multicast or can also be used for unicast.

From our point of view, it is not only an editorial issue. There are some impact on defining type-x CSS as a type-3 CSS and requires agreement among different companies.

If the type-x CSS is defined as a type-3 CSS, some confusions may be caused by different monitoring priority rules of type-x CSS and current type-3 CSS. More specifically, when type3-CSS further serves as multicast scheduling search space, a same search space of type3-CSS can be configured for DCI format 1_0 with CRC scrambled by C-RNTI and the first DCI format with CRC scrambled by G-RNTI at the same time. For DCI format 1_0 with CRC scrambled by C-RNTI, it will always be monitored. While for the first DCI format with CRC scrambled by G-RNTI, it may be dropped as its monitoring priority depends on the order of search space index comparing with other USSs. That is to say, for the same search space of type3-CSS, there are two dropping rules that are conflicting with each other, and this will complicate the monitoring behavior of the UE.

Then, a possible UE behavior needs to be defined. For example, for the first DCI format with CRC scrambled by G-RNTI within type-3 CSS, it should always be monitored by the UE. For the second DCI format with CRC scrambled by G-RNTI within type-3 CSS, the UE determines monitoring priority according to search space index and further decides whether to monitor.

Proposal 10: If the type-x CSS is defined as a type-3 CSS, the following UE behavior on Type-3 CSS monitoring should be defined,

- For the first DCI format with CRC scrambled by G-RNTI within type-3 CSS, it should always be monitored by the UE.

- For the second DCI format with CRC scrambled by G-RNTI within type-3 CSS, the UE determines monitoring priority according to search space index and further decides whether to monitor.

- Regarding that whether DCI format 1_0 with CRC scrambled by C-RNTI and the first DCI format for multicast can be configured in the same type-x CSS, type-x CSS is defined for multicast service transmission. It has some features different from those of the current unicast transmission, such as PDCCH priority rules, scrambling sequence generation, etc. So it is not appropriate to adopt for all unicast services. The only special case that can be considered is PTP retransmission scheduling in type-x CSS.

- Proposal 11: The DCI format 1_0 with CRC scrambled by C-RNTI configured in type-x CSS can only be used for PTP retransmission for multicast.

Detailed design of GC-PDSCH

4.1	LBRM and TBS determination

The following agreement was reached during RAN1#106b-e about LBRM and TBS determination for PTP retransmission of multicast.

Agreement: Study the following options for the LBRM/TBS determination for PTP retransmission of multicast. - Option 1: based on the LBRM/TBS determination of the PTM initial transmission using same HPID and NDI. - Option 2: based on the LBRM/TBS determination of the legacy unicast PDSCH transmission.

From our perspective, it is impossible for UE to determine whether this is PTP retransmission of multicast or unicast PTP transmission especially if the PTM initial transmission is missed by the UE. Thus, Opiton1 is not workable from our perspective.

For option 2, companies argued that UE cannot apply HARQ combining of PTM and PTP retransmission for the same TB since the circular buffer is different. Considering that this is a key issue that affects the feasibility of PTP retransmission. Some compromised solutions should be considered at the last RAN1 meeting of R17.

For example, if the DCI format 1_0 with CRC scrambled by C-RNTI configured in type-x CSS can be used for PTP retransmission for multicast as proposed in section 3.5. A UE can identify that the DCI format 1_0 with CRC scrambled by C-RNTI transmitted in type-x CSS is a PTP retransmission of multicast TB, and then the LBRM and TBS size can be determined according to the multicast configuration parameters. For a PDSCH scheduled by the DCI format 1_0 with CRC scrambled by C-RNTI transmitted in a non-type-x CSS, the LBRM and TBS determination for it should use the legacy configuration in the dedicated BWP.

Then, from gNB’s perspective, if circular buffer can be kept the same between PTM initial transmission and PTP retransmission according to unicast configurations, gNB can use PTP retransmission in any configured search space. On the other hand, gNB can also select to use DCI format 1_0 in a type-x CSS for PTP retransmission scheduling. So that the LBRM/TBS can be determined according to multicast configurations, which is aligned with PTM initial transmission.

So we have the following proposal.

Proposal 12: For LBRM and TBS determination for PTP retransmission of multicast,

- In case that a PDSCH is scheduled by DCI format 1_0 with CRC scrambled by C-RNTI transmitted in type-x CSS, configurations for multicast should be used for LBRM and TBS determination.

- Otherwise if a PDSCH is scheduled by DCI format with CRC scrambled by C-RNTI transmitted in a search space other than type-x CSS, legacy configurations for unicast should be used for LBRM and TBS determination.

Note: This is based on proposal 11, i.e., assuming the DCI format 1_0 with CRC scrambled by C-RNTI configured in type-x CSS can only be used for PTP retransmission for multicast.

4.2	HARQ process management

During RAN1#104b-e meeting, the following agreement on association between initial transmission or re-transmission for multicast based on PTM transmission scheme 1 and PTP re-transmission were achieved. And in RAN1#105-e meeting, the issue on NDI collision issue was further proposed and the following agreement was made for further study whether/how to differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast.

Agreement: in RAN1#104b-e The same HARQ process ID and NDI are used for PTM scheme 1 (re)transmissions and PTP retransmissions of the same TB. Agreement: in RAN1#105-e For HARQ process management, further study whether/how to differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast.

The HPID is shared among unicast and multicast transmission. When a HPID is used for multicast scheduled by PTM PDCCH, it is difficult to guarantee that the NDI in PTM PDCCH is toggled relative to that in each UE’s latest PTP PDCCH for unicast.

If a UE misses the PTM PDCCH, a PTP PDCCH for scheduling the retransmission of this multicast TB may be detected by the UE. According to the current agreement above, the value of NDI in the PTP PDCCH is same as that in the PTM PDCCH. And it can be the same as the NDI in the UE’s latest PTP PDCCH corresponding with the same HPID. Then, the UE will erroneously soft-combine the received TB(a retransmission of multicast TB) scheduled by PTP PDCCH with the HARQ buffer if it was NACK’ed for the latest unicast transmission.

Two potential ways can be considered to avoid the above errors,

- Alt.1: Introducing an indicator in the PTP PDCCH to distinguish the PDSCH scheduled by the PTP PDCCH is a multicast TB or a unicast TB;

- Alt.2: Setting the value of the NDI in the PTP PDCCH for scheduling the retransmission of multicast TB to be toggled relative to the NDI in the UE’s latest PTP PDCCH for scheduling a unicast TB with the same HPID.

Alt.1 is a straightway way. The disadvantage is that the DCI size of PTP PDCCH will be increased and the reliability of all unicast PDCCHs will be reduced.

Table-1 gives an example of Alt.2, as the value of NDI in PTP PDCCH for scheduling the retransmission of multicast is always toggled compared with that in the latest PTP PDCCH for unicast scheduling, there will be no erroneously soft-combination anymore even if the PTM PDCCH is missed. On the other hand, if the PTM PDCCH is received, the NDIs of PTM and PTP retransmission will not be always same. Then, the UE can assume that a new TB is scheduled by the PTP PDCCH, through which erroneous soft-combination can be avoided.

Table-1: NDI values in different PDCCHs with same HPID

| UE | Latest PTP PDCCH for unicast scheduling | PTM PDCCH for multicast scheduling | PTP PDCCH for scheduling the retransmission of multicast |
| --- | --- | --- | --- |
| UE1 | NDI = 0 | NDI = 1 | NDI = 1 |
| UE2 | NDI = 1 | NDI = 1 | NDI = 0 |

According to the above analysis, Alt.2 can avoid erroneous combination without adding DCI size, which is preferred.

Proposal 13: Regarding how to differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast,

- The value of the NDI in the PTP PDCCH for scheduling the retransmission of multicast TB is toggled relative to the NDI in the UE’s latest PTP PDCCH for scheduling a unicast TB with the same HPID.

SPS-based MBS transmission

The following agreements on SPS-based MBS transmission were achieved during previous RAN1 e-meetings. Some remaining issues will be discussed below.

Agreement: The retransmission scheme for a given SPS group-common PDSCH can be either PTM scheme 1 or PTP. - FFS: Whether PTM scheme 1 retransmission and PTP retransmission can be used simultaneously for different UEs in the same MBS group Agreement: Confirm the working assumption: For activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state, - At least group-common PDCCH is supported - FFS: Whether and how to address the missed activation and deactivation - FFS: Whether UE-specific PDCCH is supported for activation/deactivation Agreement: For reliability of the group-common PDCCH activation of SPS group-common PDSCH, support at least one of the following alternatives. - Alt 1: retransmit the activation command via group-common PDCCH. - Alt 2: retransmit the activation command via UE-specific PDCCH. - Alt 3: retransmit the activation command via MAC-CE. - FFS other details. - Note: Down-selection can take into account the HARQ-ACK feedback scheme for SPS activation

UE-specific activation PDCCH

There may be two potential methods for SPS GC-PDSCH activation or deactivation, i.e. GC-PDCCH and UE-specific PDCCH. The activation and deactivation via GC-PDCCH has already been supported. The following analyzes the necessity of further supporting the activation and deactivation using UE-specific PDCCH.

Undoubtedly, activation or deactivation of a SPS transmission by a GC-PDCCH is applicable to indicate or change the scheduling parameters or stop the transmission for a group of UEs.

The question is that if a new UE joins in the MBS service group after the activation, whether a GC-activation PDCCH can also be used for the new joined UE. Some companies concerned that the GC-activation PDCCH without any parameter update will disturb irrelevant UEs. In practice, however, the UE anyway needs to monitor the GC-activation PDCCH according to the configuration of search space set and CORESET. This does not cause extra blind detection overhead for these UEs. So it does not seem to be a problem for using GC-PDCCH to reactivate the SPS transmission for the new joined UE.

For deactivation PDCCH, if a UE does not want to receive the SPS transmission for a MBS service, the UE will unsubscribe this MBS service. This is a question about whether a deactivation PDCCH is still required to be transmitted after the UE has already stopped to receive this SPS transmission.

Therefore, it is unnecessary to further support UE-specific PDCCH as activation/de-activation PDCCH under the prerequisite that GC-PDCCH has already be supported for activating/deactivating an SPS transmission.

Proposal 14: UE-specific PDCCH is not needed for activation/deactivation of an SPS GC-PDSCH given that group-common PDSCH has already been supported.

Retransmission scheme of the activation command

GC-PDCCH has supported for the initial transmission of the activation command. If NACK-only is supported and configured for SPS-based MBS transmission, only GC-PDCCH can be used for retransmitting the activation command. So it naturally takes the GC-PDCCH as a baseline mechanism for retransmitting the activation commands.

Some companies believe that higher reliability can be obtained by supporting UE-specific PDCCH for retransmission of the activation command. From our point of view, the GC-PDCCH can also achieve the same reliability as the UE-specific PDCCH by setting the transmission parameters (such as, beam, AL, etc.) of the GC-PDCCH according to the UE missed to detect the activation command. Then, the motivation is not so clear for further supporting UE-specific PDCCH for the initial transmission of the activation command.

Regarding retransmitting the activation command via MAC-CE, it will obviously bring about a larger indication delay in both information generation and processing. In addition, if the PDSCH carrying the MAC-CE is not decoded correctly by the UE and needs to be retransmitted, the time domain location of the retransmission PDSCH will be limited to avoid PDSCH re-coding.

As an example shown in Figure-2, PDSCH 2 carrying activation command is not decoded by the UE. According to the description of MAC-CE scheme[5], the slot that are aligned with those in the original group-based SPS activation PDCCH should be indicated in the MAC-CE. There may be two potential ways for slot indication, one is indicating the absolute time of the original activation PDCCH, and another is relative time (i.e., a slot offset) between PDSCH carrying MAC-CE and the original activation PDCCH or SPS PDSCH. The former mode introduces a large signaling overhead, which does not seem appropriate. If the latter mode is adopted, the retransmission PDSCH (i.e., PDSCH4) should be scheduled by considering the location of the original activation PDCCH or SPS PDSCH. That is, the same slot offset between the PDSCH carrying the activation command and the original activation PDCCH or SPS PDSCH should be maintained. In the example case of Figure-2, only slot 7 can be used for scheduling PDSCH4. This greatly affects scheduling flexibility.

Observation 2: Compared with retransmission of activation command via DCI, MAC-CE scheme introduces a larger indication delay and additional scheduling restriction.

Figure-2: Example case of retransmit the activation command via MAC-CE

Proposal 15: NR MBS supports to retransmit the activation command via GC-PDCCH.

Support on simultaneous operation with unicast reception

The following agreements were made in previous RAN1 e-meetings for multiplexing mechanism among unicast PDSCHs and GC-PDSCHs.

Agreements: - For RRC_CONNECTED UEs, at least support FDM between unicast PDSCH and group-common PDSCH in a slot based on UE capability. - FFS: TDM or SDM in a slot. Agreements: Further study the following cases for simultaneous reception of unicast PDSCH and group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs. - Case 1: support TDM between multiple TDMed unicast PDSCHs and one group-common PDSCH in a slot - Case 2: support TDM among multiple group-common PDSCHs in a slot - Case 3: support TDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot - Case 4: support FDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot - Case 5: support FDM among multiple group-common PDSCHs in a slot - FFS: maximum number of PDSCHs in a slot simultaneous received per UE Agreement: At least support the following cases for PDSCH reception for MBS in a slot based on UE capability for RRC_CONNECTED UEs - Case 1: support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - FFS: the value(s) of M - Case 2: support TDM among N (N>1) group-common PDSCHs in a slot per CC - FFS: the value(s) of N - Case 3: support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - FFS: the value(s) of K and L

From the perspective of UE capabilities, FDM between unicast PDSCH and group-common PDSCH in a slot is a UE capability. Similarly, FDM among two different group-common PDSCHs should be a same UE capability. So at least case 5 listed in the previous agreement should be supported. About the maximum number of FDMed group-common PDSCHs in a slot can be supported by a UE, at least two should be supported as an UE capability, which is same as one unicast PDSCH and one group-common PDSCH. Other value can be further discussed in UE feature.

Regarding the impact on Type-1 codebook construction, a per G-RNTI based sub-codebook can be defined. And more details can be found in our contribution [6].

Proposal 16: Support case 5 for PDSCH reception for MBS in a slot based on UE capability for RRC_CONNECTED UEs, i.e.,

- Case 5: support FDM among Q (Q>1) group-common PDSCHs in a slot per CC

  - FFS: the value(s) of Q

Conclusion

In this contribution, the discussion and analysis on mechanisms for group scheduling for RRC_CONNECTED UEs are presented with the following observations and proposals.

Common frequency resource

Observation 1: the energy saving effect is limited by introducing an independent inactivity timer for GC-PDCCH reception.

Proposal 1: For timer-based BWP switching, UE starts or restarts BWP-InactivityTimer when it successfully decodes a GC-PDCCH addressed to group-common RNTI (e.g., G-RNTI or G-CS-RNTI) on/for the active BWP or when a MAC PDU for multicast is received in a configured downlink assignment.

- Proposal 2: CFR can be configured larger than active downlink BWP when the active downlink BWP is the initial BWP defined by CORESET#0.

- Prioritize the corresponding discussion in RRC IDLE/INACTIVE state to strive for a consistent solution for all RRC states

Proposal 3: Regarding the CFR configuration, the parameters configured under the dedicated unicast BWP can be used for MBS transmission if the corresponding parameters are not configured under the CFR.

Proposal 4: Group-common PDCCH/PDSCH for multicast can be received on SCell.

Detailed design of GC-PDCCH

Proposal 5: Monitoring configurations (e.g., CORESETs, Search Spaces, etc.) for GC-PDCCH of PTM retransmission can be configured separately from that for GC-PDCCH of PTM initial transmission.

Proposal 6: For NR multicast, introduce beam sweeping via defining association between MOs of GC-PDCCH and SSBs or CSI-RSs.

Proposal 7: For DCI format of multicast, the unused fields should be removed from DCI format 1_0 for unicast.

Proposal 8: For the second DCI format for GC-PDCCH, fields (at least including, Identifier for DCI formats(1 bit), scheduling information for the second transport block(8 bits), TPC command for scheduled PUCCH(2 bits), SRS request(2 bits), CBG transmission information (CBGTI)(0, 2, 4, 6 or 8 bits) and CBG flushing out information (CBGFI)(0 or 1 bit) ) should be removed from DCI format 1_1 for unicast.

Proposal 9: Regarding DCI size alignment for the second DCI format of GC-PDCCH, it is counted as “other RNTI”, and gNB will ensure that the number of DCI sizes does not exceed budget.

Proposal 10: If the type-x CSS is defined as a type-3 CSS, the following UE behavior on Type-3 CSS monitoring should be defined,

- For the first DCI format with CRC scrambled by G-RNTI within type-3 CSS, it should always be monitored by the UE.

- For the second DCI format with CRC scrambled by G-RNTI within type-3 CSS, the UE determines monitoring priority according to search space index and further decides whether to monitor.

- Proposal 11: The DCI format 1_0 with CRC scrambled by C-RNTI configured in type-x CSS can only be used for PTP retransmission for multicast.

Detailed design of GC-PDSCH

Proposal 12: For LBRM and TBS determination for PTP retransmission of multicast,

- In case that a PDSCH is scheduled by DCI format 1_0 with CRC scrambled by C-RNTI transmitted in type-x CSS, configurations for multicast should be used for LBRM and TBS determination.

- Otherwise if a PDSCH is scheduled by DCI format with CRC scrambled by C-RNTI transmitted in a search space other than type-x CSS, legacy configurations for unicast should be used for LBRM and TBS determination.

Note: This is based on proposal 11, i.e., assuming the DCI format 1_0 with CRC scrambled by C-RNTI configured in type-x CSS can only be used for PTP retransmission for multicast.

Proposal 13: Regarding how to differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast,

- The value of the NDI in the PTP PDCCH for scheduling the retransmission of multicast TB is toggled relative to the NDI in the UE’s latest PTP PDCCH for scheduling a unicast TB with the same HPID.

SPS-based MBS transmission

Proposal 14: UE-specific PDCCH is not needed for activation/deactivation of an SPS GC-PDSCH given that group-common PDSCH has already been supported.

Observation 2: Compared with retransmission of activation command via DCI, MAC-CE scheme introduces a larger indication delay and additional scheduling restriction.

Proposal 15: NR MBS supports to retransmit the activation command via GC-PDCCH.

Support on simultaneous operation with unicast reception

Proposal 16: Support case 5 for PDSCH reception for MBS in a slot based on UE capability for RRC_CONNECTED UEs, i.e.,

- Case 5: support FDM among Q (Q>1) group-common PDSCHs in a slot per CC

  - FFS: the value(s) of Q

Reference

RP-201038, WID revision: NR Multicast and Broadcast Services, RAN#88e.

3GPP RAN1#106b-e, Chairman’s notes.

R1-2106912, Support of group scheduling for RRC_CONNECTED UEs, Samsung.

3GPP TS38.321, Medium Access Control (MAC) protocol specification, Release 16.

R1-2105914, Mechanisms to support MBS group scheduling for RRC_CONNECTED UEs, Ericsson.

R1-2110911, Discussion on mechanisms to Improve Reliability for RRC_CONNECTED UEs, ZTE.

3GPP RAN1#104b-e, Chairman’s notes.

3GPP RAN1#105-e, Chairman’s notes.