3GPP TSG-RAN WG1 Meeting #107-e	R1-2110985

# e-Meeting, November 11th - 19th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev |  | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UE behaviour in case of HARQ RTT restriction violation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | If PSFCH is configured in a resource pool, a UE should ensure a minimum time gap between any two selected resources of a TB transmission enabling HARQ feedback. However, for resource selection of a given sidelink grant with periodic resource reservation, it is possible that although resources in period #1 can meet the time gap restriction, the resources of a following period #n violate the time gap restriction. Thus, the reserved resources in period #n cannot be used for TB transmission with enabled HARQ feedback. In this case, the UE behaviour is not clear according to the current spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reuse the mode 1 behaviour in this case, i.e., if the HARQ RTT time gap restriction between two PSCCH/PSSCH resource is not satisfied, the UE is allowed to drop a next PSCCH/PSSCH retransmission with enabled SL HARQ feedback. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It remains unclear for the UE how to handle the case when resources in period #1 can meet the HARQ time gap restriction, while the resources of a following period #n violate the time gap restriction. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: Only impact the TX UE behaviour in mode 2. No impact to network side. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.6	UE PSSCH preparation procedure time

====Omitted====

For sidelink resource allocation mode 1, the UE does not expect that the first sidelink symbol in the sidelink allocation for a PSSCH for retransmission of a transport block and the associated PSCCH, including the DM-RS and the duplicated symbol as defined by the "Time resource assignment" field of the corresponding DCI for dynamic grant or for SL configured grant type 2, or by sl-TimeResourceCG-Type1 for configured grant type 1 starts earlier than at symbol L where L is defined as the next sidelink symbol with its CP starting Tprep+δ after the end of the last symbol of the PSFCH occasion corresponding to the most recent transmission of PSSCH for the same transport block, where Tprep is defined in Clause 16.5 of [6, TS 38.213] and δ=5∙10-4 s. Otherwise the UE may skip the retransmission of the PSSCH and the transmission of the corresponding PSCCH.

For sidelink resource allocation mode 2, if the first sidelink symbol in the sidelink allocation for a PSSCH for retransmission of a transport block and the associated PSCCH starts earlier than K symbols after the end of the last symbol of the PSFCH occasion corresponding to the most recent transmission of PSSCH for the same transport block, the UE may skip the retransmission of the PSSCH and the transmission of the corresponding PSCCH. K is a time duration required for PSFCH reception and processing plus SL retransmission preparation.

====Omitted====