8.173GPP TSG RAN WG1 Meeting #107-e	R1-2111594

e-Meeting, November 11th – 19th, 2021

Source: 	ZTE, Sanechips

Title: 	Discussion on NR Rel-17 IAB MT features

Agenda item:	8.16.10

Document for:	Discussion and Decision

Introduction

After the email discussion [106bis-e-R17-UE-features-eIAB-01], an updated table was agreed as baseline for further discussions during RAN1 #107-e, the details can be seen in [1] [2]. This contribution provides some analysis and our further views for Rel-17 IAB MT features.

Analysis for IAB MT features

According to the following agreements from previous meeting, only recommended beam is supported for the beam indication from the IMT-MT to its parent node.

Agreement The recommended beam indication from the IAB-MT to the parent node are provided via MAC-CE: - For DL Rx beam(s): using one or more of the following: - DL TCI state ID - FFS: UE/IAB-MT does not assume that DL Tx power adjustment (if provided) is applied to the SSB index (if supported) indicated as QCLed reference signal in DL TCI state ID. - [RS ID] - [R17 DL TCI, or joint DL/UL TCI ID] - For UL Tx beam(s): using one or more of the following: - [Spatial relation] - [RS ID] - [R17 UL TCI, or joint DL/UL TCI ID] - [SRI]

- So we propose to update FG31-3 to reflect the above agreement

Proposal 1: Adopt the following modification to 31-3 to reflect the agreement of RAN1#106bis-e:

| ... | .... |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31-3 | [Child IAB-MT beam recommendation indication] | Support [MT Beam Indication] transmission 1) IAB-MT DL beam 2) IAB-MT UL beam |  | yes | N/A | IAB-node cannot indicate recommended/[non-preferred] MT DL/UL beam to parent node | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |

Moreover, based on the following agreements in RAN1#106-e:

Agreement For Case 7 timing at a parent node, the IAB-MT Tx timing of the node is obtained via the legacy TA loop plus an offset from the parent node. - FFS range, granularity, and signaling details of the offset. Agreement An IAB-node is explicitly indicated by the parent node when Case 6 timing is performed at the IAB-node at least for specific time resources. - FFS: whether the indication should be associated with another dimensions, e.g. multiplexing cases - FFS whether an IAB-node is explicitly indicated by the parent node when Case 7 timing is performed at the IAB-node. Agreement An IAB-node is explicitly indicated by the parent node when Case 7 timing is performed at the parent node. - FFS for signalling details

A parent node may explicitly indicated the following information to an IAB-MT:

- When Case 6 timing is performed at the IAB-node.

- When Case 7 timing is performed at the parent node.

- IAB-MT Tx timing offset for Case 7 timing at the parent node.

- FFS whether when Case 7 timing is performed at the IAB-node is also indicated.

- So we propose to update FG31-4 and 31-5 to reflect the above agreements.

- Proposal 2: Adopt the following modification to 31-4 and 31-5 to reflect the agreements of RAN1#106-e:

| ... | .... |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31-4 | [Case 6 timing alignment] | 1.) Support [Case 6 transmission] 2.) Support [Case 6 timing at IAB-node indication] reception |  | Yes | N/A | Case 6 timing at the IAB-node is not supported. Switching across different timing cases (i.e., Case 1 at IAB-node, Case 6 at IAB-node, and/or Case 7 at the Parent) is not supported. When to perform Case 6 timing at the IAB-node cannot be controlled by the parent node. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31-5 | Case 7 timing [alignment] | 1.) Support [Case7 transmission] 2.) Support [Case 7 timing at parent-node indication] and [offset indication] reception |  | Yes | N/A | Parent-node cannot adopt both (and switch between) Case 1 and Case 7 timing. When to perform Case 7 timing at the parent IAB-node cannot be indicated to its child node. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |

Conclusion

In this contribution, proposals on NR Rel-17 IAB features are introduced.

Proposal 1: Adopt the following modification to 31-3 to reflect the agreement of RAN1#106bis-e:

| ... | .... |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31-3 | [Child IAB-MT beam recommendation indication] | Support [MT Beam Indication] transmission 1) IAB-MT DL beam 2) IAB-MT UL beam |  | yes | N/A | IAB-node cannot indicate recommended/[non-preferred] MT DL/UL beam to parent node | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |

- Proposal 2: Adopt the following modification to 31-4 and 31-5 to reflect the agreements of RAN1#106-e:

| ... | .... |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31-4 | [Case 6 timing alignment] | 1.) Support [Case 6 transmission] 2.) Support [Case 6 timing at IAB-node indication] reception |  | Yes | N/A | Case 6 timing at the IAB-node is not supported. Switching across different timing cases (i.e., Case 1 at IAB-node, Case 6 at IAB-node, and/or Case 7 at the Parent) is not supported. When to perform Case 6 timing at the IAB-node cannot be controlled by the parent node. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31-5 | Case 7 timing [alignment] | 1.) Support [Case7 transmission] 2.) Support [Case 7 timing at parent-node indication] and [offset indication] reception |  | Yes | N/A | Parent-node cannot adopt both (and switch between) Case 1 and Case 7 timing. When to perform Case 7 timing at the parent IAB-node cannot be indicated to its child node. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |

Reference

R1-2109916 Summary of UE features for IAB enhancements, Moderator (AT&T).

R1-2110587 Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e, Moderators (AT&T, NTT DOCOMO, INC.).