3GPP TSG RAN WG1 #107-e	                                         	      R1-2111843

e-Meeting, November 11th – 19th, 2021

Agenda Item:	5

Source:	Apple Inc.

Title:	Discussion on RAN4 LS R1-2108704 on beam information of PUCCH Scell in PUCCH SCell activation procedure

Document for:	Discussion/Decision

# View on the questions from RAN4 in R1-2108704

In RAN4 LS R1-21008707 [1] on the beam information of PUCCH SCell in PUCCH SCell activation procedure, RAN1 received three questions from RAN4. Below, we share our understanding of the questions raised by RAN4.

Q1: Whether UE can report CSI (e.g. L1-RSRP) of the target being-activated PUCCH SCell belonging to secondary PUCCH group by configuring CSI report setting (e.g. CSI-ReportConfig) on any active serving cells belonging to primary PUCCH group

When PUCCH group is discussed and defined in RAN1, our understanding is that it is to avoid cross PUCCH group operation for the L1 procedures related to PUCCH transmission, not limited to only the HARQ-ACK feedback. Unless specification explicitly allows cross PUCCH group operation, it is generally disallowed. Therefore, we don’t think RAN1 supports reporting CSI on PUCCH in one PUCCH group based on the measurement performed in the other PUCCH group.

Q2: Whether the above observation is correct, i.e. the identified four cases are not supported by the current RAN1 and RAN2 specification

We think  L3 measurement could be reported via PCell PUSCH without spec change, which can be used to support the identified four cases by RAN4

Q3: Whether the above identified cases can be supported by RAN1 and RAN2 spec updates within Rel-17 timeframe.

Since L3 measurement on PUSCH is already supported by the specification, we do not think any spec update is needed within Rel-17 timeframe.

# Reference

- R1-2108704. "LS on beam information of PUCCH SCell in PUCCH SCell activation procedure", 3GPP TSG RAN WG1 #106bis-e, e-Meeting, October 11th – 19th, 2021