3GPP TSG RAN WG1 #107-e	        		   R1-2112986

# e-Meeting, November 11th – 19th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of further enhancements on MIMO for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2021-12-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of MIMO enhancements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description for enhancements on link recovery, power control, reporting of UL control information, and reception of DL control information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for MIMO enhancements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6, 7, 7.1.1, 7.2.1, 7.3.1, 7.7.1, 8, 9, 9.1.2, 9.1.2.1, 9.1.2.2, 9.1.3, 9.1.3.1, 9.1.3.2, 9.2.1, 9.2.2, 9.2.3, 9.2.4, 9.2.5, 9.2.6, 10, 10.1, 11, 11.2, 11.2A, 12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.212, TS 38.214, TS 38.321, TS 38.331 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged text is omitted ***

# 6	Link recovery procedures

A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes and corresponding two sets q1,0 and q1,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList1 and candidateBeamRSList2, respectively, for radio link quality measurements on the BWP of the serving cell. The set q0,0 is associated with the set q1,0 and the set q0,1 is associated with the the set q1,1.

If the UE is not provided q0 by failureDetectionResourcesToAddModList for a BWP of the serving cell, the UE determines the set q0 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for respective CORESETs that the UE uses for monitoring PDCCH. If the UE is not provided q0,0 or  q0,1 for a BWP of the serving cell, the UE determines the set q0,0 or  q0,1 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for first and second CORESETs that the UE uses for monitoring PDCCH, where the UE is provided two coresetPoolIndex values 0 and 1 for the first and second CORESETs, or is not provided coresetPoolIndex value for the first CORESETs and is provided coresetPoolIndex value of 1 for the second CORESETs, respectively. If there are two RS indexes in a TCI state, the set q0 includes RS indexes configured with qcl-Type set to 'typeD' for the corresponding TCI states. If a CORESET that the UE uses for monitoring PDCCH includes two TCI states and the UE is provided sfnSchemePdcch set to 'sfnSchemeA' or 'sfnSchemeB', the set q0 includes RS indexes in the RS sets associated with the two TCI states. The UE expects the set q0 to include up to two RS indexes. The UE expects the set q0,0 or the set q0,1 to include up to a number of NBFD RS indexes indicated by capabilityparametername. If a number of active TCI states for PDCCH receptions in the first or second CORESETs is larger than NBFD, the UE determines the set q0,0 or q0,1 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets associated with the active TCI states for PDCCH receptions in the first or second CORESETs corresponding to search space sets according to an ascending order for monitoring periodicity. If more than one first or second CORESETs correspond to search space sets with same monitoring periodicity, the UE determines the order of the first or second CORESETs according to a descending order of a CORESET index. The UE expects single port RS in the set q0, or q0,0, or q0,1. The UE expects single-port or two-port CSI-RS with frequency density equal to 1 or 3 REs per RB in the set q1, or q1,0, or q1,1.

The thresholds Qout,LR and Qin,LR correspond to the default value of rlmInSyncOutOfSyncThreshold, as described in [10, TS 38.133] for Qout, and to the value provided by rsrp-ThresholdSSB or rsrp-ThresholdBFR, respectively.

The physical layer in the UE assesses the radio link quality according to the set q0, q0,0, or q0,1, of resource configurations against the threshold Qout,LR. For the set q0, the UE assesses the radio link quality only according to SS/PBCH blocks on the PCell or the PSCell or periodic CSI-RS resource configurations that are quasi co-located, as described in [6, TS 38.214], with the DM-RS of PDCCH receptions monitored by the UE. The UE applies the Qin,LR threshold to the L1-RSRP measurement obtained from a SS/PBCH block. The UE applies the Qin,LR threshold to the L1-RSRP measurement obtained for a CSI-RS resource after scaling a respective CSI-RS reception power with a value provided by powerControlOffsetSS.

In non-DRX mode operation, the physical layer in the UE provides an indication to higher layers when the radio link quality for all corresponding resource configurations in the set q0, or in the set q0,0 or q0,1, that the UE uses to assess the radio link quality is worse than the threshold Qout,LR. The physical layer informs the higher layers when the radio link quality is worse than the threshold Qout,LR with a periodicity determined by the maximum between the shortest periodicity among the SS/PBCH blocks on the PCell or the PSCell and/or the periodic CSI-RS configurations in the set q0, q0,0, or q0,1 that the UE uses to assess the radio link quality and 2 msec. In DRX mode operation, the physical layer provides an indication to higher layers when the radio link quality is worse than the threshold Qout,LR with a periodicity determined as described in [10, TS 38.133].

For the PCell or the PSCell, upon request from higher layers, the UE provides to higher layers the periodic CSI-RS configuration indexes and/or SS/PBCH block indexes from the set q1, or q1,0, or q1,1 and the corresponding L1-RSRP measurements that are larger than or equal to the Qin,LR threshold.

For the SCell, upon request from higher layers, the UE indicates to higher layers whether there is at least one periodic CSI-RS configuration index or SS/PBCH block index from the set q1, or q1,0, or q1,1 with corresponding L1-RSRP measurements that are larger than or equal to the Qin,LR threshold, and provides the periodic CSI-RS configuration indexes or SS/PBCH block indexes from the set q1, or q1,0, or q1,1 and the corresponding L1-RSRP measurements that are larger than or equal to the Qin,LR threshold, if any.

For the PCell or the PSCell, a UE can be provided a CORESET through a link to a search space set provided by recoverySearchSpaceId, as described in clause 10.1, for monitoring PDCCH in the CORESET. If the UE is provided recoverySearchSpaceId, the UE does not expect to be provided another search space set for monitoring PDCCH in the CORESET associated with the search space set provided by recoverySearchSpaceId.

For the PCell or the PSCell, the UE can be provided, by PRACH-ResourceDedicatedBFR, a configuration for PRACH transmission as described in clause 8.1. For PRACH transmission in slot n and according to antenna port quasi co-location parameters associated with periodic CSI-RS resource configuration or with SS/PBCH block associated with index qnew provided by higher layers [11, TS 38.321], the UE monitors PDCCH in a search space set provided by recoverySearchSpaceId for detection of a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI starting from slot n+4within a window configured by BeamFailureRecoveryConfig. For PDCCH monitoring in a search space set provided by recoverySearchSpaceId and for corresponding PDSCH reception, the UE assumes the same antenna port quasi-collocation parameters as the ones associated with index qnew until the UE receives by higher layers an activation for a TCI state or any of the parameters tci-StatesPDCCH-ToAddList and/or tci-StatesPDCCH-ToReleaseList. After the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI in the search space set provided by recoverySearchSpaceId, the UE continues to monitor PDCCH candidates in the search space set provided by recoverySearchSpaceId until the UE receives a MAC CE activation command for a TCI state or tci-StatesPDCCH-ToAddList and/or tci-StatesPDCCH-ToReleaseList.

For the PCell or the PSCell, after 28 symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId for which the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI and until the UE receives an activation command for PUCCH-SpatialRelationInfo [11, TS 38.321] or is provided PUCCH-SpatialRelationInfo for PUCCH resource(s), the UE transmits a PUCCH on a same cell as the PRACH transmission using

-	a same spatial filter as for the last PRACH transmission

-	a power determined as described in clause 7.2.1 with qu=0, qd=qnew, and l=0

For the PCell or the PSCell and for sets q0 and q1, after 28 symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId where a UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI, the UE assumes same antenna port quasi-collocation parameters as the ones associated with index qnew for PDCCH monitoring in a CORESET with index 0.

If a UE is provided TCI-State_r17 indicating a unified TCI state for the PCell or the PSCell [6, TS 38.214], after X symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId where the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI, the UE

-	if AdditionalPCIInfo is not provided, monitors PDCCH in all CORESETs, and receives PDSCH and aperiodic CSI-RS in a resource from a CSI-RS resource set with same indicated TCI state as for the PDCCH and PDSCH, using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any

-	transmits PUCCH, PUSCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUCCH and the PUSCH, using a same spatial domain filter as for the last PRACH transmission

For the remaining of this clause, if a PDCCH reception includes two PDCCH candidates from two linked search space sets based on searchSpaceLinking, as described in clause 10.1, the last symbol of the PDCCH reception is the last symbol of the PDCCH candidate that ends later. The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10, 11.1, and 11.1.1.

For the PCell or the PSCell, if BFR MAC CE [11, TS 38.321] is provided in Msg3 or MsgA of contention based random access procedure, and if a PUCCH resource is provided with PUCCH-SpatialRelationInfo, after 28 symbols from the last symbol of the PDCCH reception that determines the completion of the contention based random access procedure as described in clause 5.1.5 of [11, TS 38.321], the UE transmits the PUCCH on a same cell as the PRACH transmission using

-	a same spatial filter as for the last PRACH transmission

-	a power determined as described in clause 7.2.1 with qu=0, qd=qnew, and l=0, where qnew is the SS/PBCH block index selected for the last PRACH transmission.

If a UE is provided TCI-State_r17 indicating a unified TCI state for the PCell or the PSCell and the UE provides BFR MAC CE in Msg3 or MsgA of contention based random access procedure, after X symbols from the last symbol of the PDCCH reception that determines the completion of the contention based random access procedure as described in [11, TS 38.321], the UE

-	if AdditionalPCIInfo is not provided, monitors PDCCH in all CORESETs, and receives PDSCH and aperiodic CSI-RS resource in a CSI-RS resource set with same indicated TCI state as for the PDCCH and PDSCH using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any

-	transmits PUCCH, PUSCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUCCH and PUSCH, using a same spatial domain filter as for the last PRACH transmission

A UE can be provided, by schedulingRequestID-BFR-SCell, a configuration for PUCCH transmission with a link recovery request (LRR) as described in clause 9.2.4 for the UE to transmit PUCCH [11, TS 38.321]. If the PCell or the PSCell is associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, the UE can be provided by schedulingRequestIDForMTRPBFR a first configuration for PUCCH transmission with a LRR and, if the UE provides twoLRRcapability, a second configuration for PUCCH transmission with a LRR. If the UE is provided only the first configuration, the UE transmits a PUCCH with LRR for either set q0,0 or q0,1. If the UE is provided both the first and second configurations, the UE uses the first configuration to transmt a PUCCH with LRR associated with set q0,0 and the second configuration to transmit a PUCCH with LRR associated with set q0,1 [11, TS 38.321].

The UE can provide in a first PUSCH MAC CE index(es) for at least corresponding SCell(s) with radio link quality worse than Qout,LR, indication(s) of presence of qnew for corresponding SCell(s), and index(es) qnew for a periodic CSI-RS configuration or for a SS/PBCH block provided by higher layers, as described in [11, TS 38.321], if any, for corresponding SCell(s).

After 28 symbols from a last symbol of a PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value, the UE

-	monitors PDCCH in all CORESETs on the SCell(s) indicated by the MAC CE using the same antenna port quasi co-location parameters as the ones associated with the corresponding index(es) qnew, if any

-	transmits PUCCH on a PUCCH-SCell using a same spatial domain filter as the one corresponding to qnew, if any, for periodic CSI-RS or SS/PBCH block reception, as described in clause 9.2.2, and using a power determined as described in clause 7.2.1 with qu=0, qd=qnew, and l=0, if

-	the UE is provided PUCCH-SpatialRelationInfo for the PUCCH,

-	a PUCCH with the LRR was either not transmitted or was transmitted on the PCell or the PSCell, and

-	the PUCCH-SCell is included in the SCell(s) indicated by the MAC-CE

where the SCS configuration for the 28 symbols is the smallest of the SCS configurations of the active DL BWP for the PDCCH reception and of the active DL BWP(s) of the at least one SCell.

If a UE is provided TCI-State_r17 indicating a unified TCI state, after X symbols from a last symbol of a PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value, the UE

-	monitors PDCCH in all CORESETs, and receives PDSCH and aperiodic CSI-RS in a resource from a CSI-RS resource set using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any

-	transmits PUCCH, PUSCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUCCH and PUSCH, using a same spatial domain filter as the one corresponding to qnew, if any

For serving cells associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, the UE can provide in a second PUSCH MAC CE index(es) for cell(s) with q0,0 and/or q0,1 having radio link quality worse than Qout,LR, the index(es) of those q0,0 and/or q0,1, and indication(s) of presence of qnew and of index(es) qnew, if any, from corresponding sets q1,0 and/or q1,1 for the serving cells.

For serving cells associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, and having radio link quality worse than Qout,LR, after 28 symbols from a last symbol of a first PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for transmission of the second PUSCH and having a toggled NDI field value, the UE assumes antenna port quasi-collocation parameters

-	corresponding to qnew from q1,0, if any, for the first CORESETs,

-	corresponding to qnew from q1,1, if any, for the second CORESETs

where the SCS configuration for the 28 symbols is the smallest of the SCS configurations of the active DL BWP for the PDCCH reception and of the active DL BWP(s) of the serving cells.

# 7	Uplink Power control

Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions.

A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by SRS-PosResourceSet as described in clause 7.3.1. If the UE is provided a number of RS resources for pathloss estimation for PUSCH/PUCCH/SRS transmissions that is larger than 4, the UE maintains for pathloss estimation RS resources corresponding to RS resource indexes qd as described in clauses 7.1.1, 7.2.1, and 7.3.1. If an RS resource updated by MAC CE, as described in clauses 7.1.1, 7.2.1 and 7.3.1, is one from the RS resources the UE maintains for pathloss estimation for PUSCH/PUCCH/SRS transmissions, the UE applies the pathloss estimation based on the RS resources starting from the first slot that is after slot k+3∙Nslotsubframe,  μ where k is the slot where the UE would transmit a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE and μ  is the SCS configuration for the PUCCH or PUSCH, respectively.

A PUSCH/PUCCH/SRS/PRACH transmission occasion i is defined by a slot index ns,fμ within a frame with system frame number SFN, a first symbol S within the slot, and a number of consecutive symbols L. For a PUSCH transmission with repetition Type B, a PUSCH transmission occasion is a nominal repetition [6, TS 38.214].

In the remaining of this clause, if a UE is provided TCI-State_r17 and for an indicated TCI-State_r17 as described in [6, TS 38.214]

-	in clauses 7.1.1, 7.2.1, and 7.3.1, the RS index qd for obtaining the downlink pathloss estimate for PUSCH, PUCCH, and SRS transmission is provided by PL-RS associated with or included in the indicated TCI-StateID_r17

-	in clause 7.1.1, if p0-Alpha-CLID-PUSCH-Set is provided, the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l are provided by p0-Alpha-CLID-PUSCH-Set associated with the indicated TCI-StateID_r17

-	in clause 7.2.1, if p0-Alpha-CLID-PUCCHSet is provided, the values of PO_PUCCH,b,f,cqu and the PUCCH power control adjustment state l are provided by p0-Alpha-CLID-PUCCH-Set associated with the indicated TCI-StateID_r17

-	in clause 7.3.1, if p0-Alpha-CLID-SRS-Set is provided, the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0-Alpha-CLID-SRS-Set associated with the indicated TCI-StateID_r17

In the remaining of this clause, if a PDCCH reception by a UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1

-	a PDCCH monitoring occasion is the union of the PDCCH monitoring occasions for the two PDCCH candidates

-	the end of the PDCCH reception is the end of the PDCCH candidate that ends later

The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10, 11.1, and 11.1.1.

## *** Unchanged text is omitted ***

### 7.1.1	UE behaviour

If a UE transmits a PUSCH on active UL BWP b of carrier f of serving cell c using parameter set configuration with index j and PUSCH power control adjustment state with index l, the UE determines the PUSCH transmission power PPUSCH,b,f,c(i,j,qd,l) in PUSCH transmission occasion i as

[dBm]

where,

-	PCMAX,f,c(i)is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier f of serving cell c in PUSCH transmission occasion i.

-	PO_PUSCH,b,f,c(j) is a parameter composed of the sum of a component PO_NOMINAL,PUSCH,f,c(j) and a component PO_UE_PUSCH,b,f,c(j) where j∈0,1,…,J-1.

-	If a UE established dedicated RRC connection using a Type-1 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet or for a PUSCH (re)transmission corresponding to a RAR UL grant as described in clause 8.3,

j=0, PO_UE_PUSCH,b,f,c0=0, and PO_NOMINAL,PUSCH,f,c0=PO_PRE+∆PREAMBLE,Msg3,

where PO_PRE is provided by preambleReceivedTargetPower [11, TS 38.321] and ΔPREAMBLE_Msg3 is provided by msg3-DeltaPreamble, or ∆PREAMBLE,Msg3=0 dB if msg3-DeltaPreamble is not provided, for carrier f of serving cell c

-	If a UE established dedicated RRC connection using a Type-2 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet, or for a PUSCH transmission for Type-2 random access procedure as described in clause 8.1A,

j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH,

where PO_PRE is provided by msgA-preambleReceivedTargetPower, or by preambleReceivedTargetPower if msgA-preambleReceivedTargetPower is not provided and ΔMsgA_PUSCH is provided by msgA-DeltaPreamble, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgA-DeltaPreamble is not provided, for carrier f of serving cell c

-	For a PUSCH (re)transmission configured by ConfiguredGrantConfig, j=1, PO_NOMINAL,PUSCH,f,c1 is provided by p0-NominalWithoutGrant, or PO_NOMINAL,PUSCH,f,c1=PO_NOMINAL,PUSCH,f,c0 if p0-NominalWithoutGrant is not provided

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’, and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format that includes a SRS resource set indicator field, and for active UL BWP b of carrier f of serving cell

-	If the SRS resource set indicator value is 00, first PO_UE_PUSCH,b,f,c1 value is provided by the value of p0-PUSCH-Alpha in ConfiguredGrantConfig.

-	If the SRS resource set indicator value is 01, second PO_UE_PUSCH,b,f,c1 value is provided by the value of p0-PUSCH-Alpha2 in ConfiguredGrantConfig.

-	If the SRS resource set indicator value is 10 or 11, first and second PO_UE_PUSCH,b,f,c1 values are respectively provided by the values of p0-PUSCH-Alpha and by p0-PUSCH-Alpha2 in ConfiguredGrantConfig.

-	else, PO_NOMINAL,PUSCH,f,c1 is provided by p0 obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig that provides an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP b of carrier f of serving cell c.

-	, and  is provided by p0 obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig that provides an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP  of carrier  of serving cell

-	For j∈2,…,J-1=SJ, a PO_NOMINAL,PUSCH,f,cj value, applicable for all j∈SJ, is provided by p0-NominalWithGrant, or PO_NOMINAL,PUSCH,f,cj=PO_NOMINAL,PUSCH,f,c0 if p0-NominalWithGrant is not provided, for each carrier f of serving cell c and a set of PO_UE_PUSCH,b,f,cjvalues are provided by a set of p0 in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP b of carrier f of serving cell c

-	If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId and if a DCI format scheduling the PUSCH transmission includes an SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in the DCI format [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values and determines the value of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value. If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId

-	if the DCI format scheduling the PUSCH transmission includes two SRI fields and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ and if, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the two SRI fields and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines first and second values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values that are mapped to the values of the first and second SRI fields, respectively.

-	if the DCI format scheduling the PUSCH transmission includes two SRI fields and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between

-	a set of values for the first SRI field value and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the first value of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the first SRI field value, and

-	a set of values associated with the second SRI field value for a same number of layers as indicated by the first SRI field [5, TS 38.212], and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the second value of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212]

-	If the DCI format also includes an open-loop power control parameter set indication field and a value of the open-loop power control parameter set indication field is '1' and if the DCI format scheduling the PUSCH transmission includes an SRI field, the UE determines a value of PO_UE_PUSCH,b,f,cj from a first value in P0-PUSCH-Set with a p0-PUSCH-SetId value mapped to the SRI field value.

-	If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId

-	if a DCI format scheduling the PUSCH transmission includes two SRI fields and an open-loop power control parameter set indication field and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook'

-	if a value of the open-loop power control parameter set indication field is '0', the UE determines two values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values in SRI-PUSCH-PowerControl that are mapped to the two SRI values corresponding to each SRS resource set with usage set to ‘codebook’.

-	if a value of the open-loop power control parameter set indication field is '1', the UE determines two values of PO_UE_PUSCH,b,f,cj from first values in P0-PUSCH-Set in P0-PUSCH-SetList and P0-PUSCH-Set in P0-PUSCH-SetList2 with p0-PUSCH-SetId values mapped to the two SRI values corresponding to each SRS resource set with usage set to ‘codebook’, respectively.

-	if a DCI format scheduling the PUSCH transmission includes two SRI fields and an open-loop power control parameter set indication field and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook',

-	if a value of the open-loop power control parameter set indication field is '0', the UE determines two values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values in SRI-PUSCH-PowerControl that are mapped to the first SRI field value corresponding to the first SRS resource set with usage set to ‘nonCodebook’ and to a second value, that is associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, corresponding to the second SRS resource set with usage set to ‘nonCodebook’.

-	if a value of the open-loop power control parameter set indication field is '1', the UE determines two values of PO_UE_PUSCH,b,f,cj from first values in P0-PUSCH-Set in P0-PUSCH-SetList and P0-PUSCH-Set in P0-PUSCH-SetList2 with p0-PUSCH-SetId values mapped to the first SRI field value corresponding to the first SRS resource set with usage set to ‘nonCodebook, and a second value, that is associated with the second SRS field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, corresponding to the second SRS resource set with usage set to ‘nonCodebook’, respectively.

-	If the UE is not provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’ and if the PUSCH transmission, except for the PUSCH retransmission corresponding to a RAR UL grant, is scheduled by a DCI format that does not include an SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, j=2,

-	If P0-PUSCH-Set is provided to the UE and the DCI format includes an open-loop power control parameter set indication field, the UE determines a value of PO_UE_PUSCH,b,f,cj from

-	a first P0-PUSCH-AlphaSet in p0-AlphaSets if a value of the open-loop power control parameter set indication field is '0' or '00'

-	a first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value if a value of the open-loop power control parameter set indication field is '1' or '01'

-	a second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value if a value of the open-loop power control parameter set indication field is '10'

-	else, the UE determines PO_UE_PUSCH,b,f,cj from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’ and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field and includes an SRS resource set indicator field with value 10 or 11

-	If P0-PUSCH-Set is provided to the UE and the DCI format includes an open-loop power control parameter set indication field, the UE determines first and second values of PO_UE_PUSCH,b,f,cj as

-	first and second P0-PUSCH-AlphaSet in p0-AlphaSets if the open-loop power control parameter set indication value is '0' or '00'

-	first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList and first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList2, respectively, if the open-loop power control parameter set indication value is '1' or '01'

-	second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList and second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID in p0-PUSCH-SetList2, respectively, if the open-loop power control parameter set indication value is '10' or '11'

-	else, the UE determines first and second values PO_UE_PUSCH,b,f,cj from the value of the first and second P0-PUSCH-AlphaSet in p0-AlphaSets, respectively

-	For αb,f,cj

-	For j=0,

-	if PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH and msgA-Alpha is provided, αb,f,c(0) is the value of msgA-Alpha

-	elseif PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔPREAMBLE_Msg3 or msgA-Alpha is not provided, and msg3-Alpha is provided, αb,f,c(0) is the value of msg3-Alpha

-	else, αb,f,c0=1

-	For j=1,

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’, and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format that includes an SRS resource set indicator field, and for active UL BWP b of carrier f of serving cell

-	If the SRS resource set indicator value is 00, first αb,f,c(1) value is provided by p0-PUSCH-Alpha in ConfiguredGrantConfig.

-	If the SRS resource set indicator value is 01, first αb,f,c(1) value is provided by p0-PUSCH-Alpha2 in ConfiguredGrantConfig.

-	If the SRS resource set indicator value is 10 or 11, first and second αb,f,c(1) values are respectively provided by p0-PUSCH-Alpha and p0-PUSCH-Alpha2 in ConfiguredGrantConfig.

-	else αb,f,c(1) is provided by alpha obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig providing an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP b of carrier f of serving cell c

-	For j∈SJ, a set of αb,f,cj values are provided by a set of alpha in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP b of carrier f of serving cell c

-	If the UE is provided SRI-PUSCH-PowerControl and more than one values of p0-PUSCH-AlphaSetId in p0-AlphaSets

-	if a DCI format scheduling the PUSCH transmission includes two SRI fields and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for

-	the two SRI fields and a set of indexes provided by P0-PUSCH-AlphaSetId that map to P0-PUSCH-AlphaSet values, and determines first and second values of αb,f,cj from the P0-PUSCH-AlphaSetID values that are mapped to the values of the first and second SRI field values, respectively.

-	if a DCI format scheduling the PUSCH transmission includes two SRI fields and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for

-	the first SRI field and a set of indexes provided by P0-PUSCH-AlphaSetId that map to P0-PUSCH-AlphaSet values, and determines first value of αb,f,cj from the P0-PUSCH-AlphaSetID value that is mapped to the first SRI field value, and

-	the second value, associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the second value of αb,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the second SRI field value

-	if a DCI format scheduling the PUSCH transmission includes one SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in the DCI format [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values and determines the values of αb,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value

-	If the UE is not provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’ and if the PUSCH transmission except for the PUSCH retransmission corresponding to a RAR UL grant is scheduled by a DCI format that does not include an SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, j=2, and the UE determines αb,f,cj from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’ and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field and includes an SRS resource set indicator field with value 10 or 11, the UE determines αb,f,cj from first and second P0-PUSCH-AlphaSet in p0-AlphaSets

-	MRB,b,f,cPUSCH(i) is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c and μ is a SCS configuration defined in [4, TS 38.211]

-	PLb,f,c(qd) is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index qd for the active DL BWP, as described in clause 12, of carrier f of serving cell c

-	If the UE is not provided PUSCH-PathlossReferenceRS and enableDefaultBeamPL-ForSRS, or before the UE is provided dedicated higher layer parameters, the UE calculates PLb,f,c(qd) using a RS resource from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB

-	If the UE is configured with a number of RS resource indexes, up to the value of maxNrofPUSCH-PathlossReferenceRSs, and a respective set of RS configurations for the number of RS resource indexes by PUSCH-PathlossReferenceRS, the set of RS resource indexes can include one or both of a set of SS/PBCH block indexes, each provided by ssb-Index when a value of a corresponding pusch-PathlossReferenceRS-Id maps to a SS/PBCH block index, and a set of CSI-RS resource indexes, each provided by csi-RS-Index when a value of a corresponding pusch-PathlossReferenceRS-Id maps to a CSI-RS resource index. The UE identifies a RS resource index qd in the set of RS resource indexes to correspond either to a SS/PBCH block index or to a CSI-RS resource index as provided by pusch-PathlossReferenceRS-Id in PUSCH-PathlossReferenceRS

-	If the PUSCH transmission is scheduled by a RAR UL grant as described in clause 8.3, or for a PUSCH transmission for Type-2 random access procedure as described in clause 8.1A, the UE uses the same RS resource index qd as for a corresponding PRACH transmission

-	If the UE is provided SRI-PUSCH-PowerControl and more than one values of PUSCH-PathlossReferenceRS-Id, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field, or for first and second SRI fields if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook', or values for a first SRI field and values associated with a second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', in a DCI format scheduling the PUSCH transmission and a set of PUSCH-PathlossReferenceRS-Id values and determines the RS resource index qd, or respective first and second RS resource indexes qd, from the value of PUSCH-PathlossReferenceRS-Id that is mapped to the SRI field value, or from the values of PUSCH-PathlossReferenceRS-Id that are mapped to respective first and second SRI field values if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook', or from the values of PUSCH-PathlossReferenceRS-Id that are mapped to respective first SRI field value and a value associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook',

where the RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	If the PUSCH transmission is scheduled by DCI format 0_0, and

-	if two spatial settings from PUCCH-SpatialRelationInfo are activated for a PUCCH resource with a lowest index, the UE uses the same RS resource index qd as for a PUCCH transmission with a spatial setting from the two spatial settings with lowest index in the PUCCH resource with the lowest index

-	else, if the UE is provided a spatial setting by PUCCH-SpatialRelationInfo for a PUCCH resource with a lowest index for active UL BWP b of each carrier f and serving cell c, as described in clause 9.2.2, the UE uses the same RS resource index qd as for a PUCCH transmission in the PUCCH resource with the lowest index

-	If the PUSCH transmission is not scheduled by DCI format 0_0, and if the UE is provided enableDefaultBeamPL-ForSRS and is not provided PUSCH-PathlossReferenceRS and PUSCH-PathlossReferenceRS-r16, the UE uses the same RS resource index qd as for an SRS resource set with an SRS resource associated with the PUSCH transmission

-	If

-	the PUSCH transmission is scheduled by DCI format 0_0 and the UE is not provided a spatial setting for a PUCCH transmission, or

-	the PUSCH transmission is scheduled by DCI format 0_1 or DCI format 0_2 that does not include an SRI field, or

-	SRI-PUSCH-PowerControl is not provided to the UE,

the UE determines a RS resource index qd with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero where the RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	If

-	the PUSCH transmission is scheduled by DCI format 0_0 on serving cell c,

-	the UE is not provided PUCCH resources for the active UL BWP of serving cell c, and

-	the UE is provided enableDefaultBeamPL-ForPUSCH0-0

the UE determines a RS resource index qd providing a periodic RS resource configured with qcl-Type set to 'typeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the serving cell c. If the CORESET has two activated TCI states, as described in clause 10.1, the UE determines the RS resource index qd based on the first TCI state.

-	If

-	the PUSCH transmission is scheduled by DCI format 0_0 on serving cell c,

-	the UE is not provided a spatial setting for PUCCH resources on the active UL BWP of the primary cell [11, TS 38.321], and

-	the UE is provided enableDefaultBeamPL-ForPUSCH0-0

the UE determines a RS resource index qd providing a periodic RS resource configured with qcl-Type set to 'typeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the serving cell c. If the CORESET has two activated TCI states, as described in clause 10.1, the UE determines the RS resource index qd based on the first TCI state.

-	For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig, a RS resource index qd is provided by a value of pathlossReferenceIndex included in rrc-ConfiguredUplinkGrant where the RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking. If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’ and for configured grant Type 1 PUSCH, first and second RS resource indexes qd are provided by respective values of pathlossReferenceIndex and pathlossReferenceIndex2 in rrc-ConfiguredUplinkGrant.

-	For a PUSCH transmission configured by ConfiguredGrantConfig that does not include rrc-ConfiguredUplinkGrant, the UE determines a RS resource index qd from a value of PUSCH-PathlossReferenceRS-Id that is mapped to a SRI field value in a DCI format activating the PUSCH transmission.

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ and the DCI format activating the PUSCH transmission includes two SRI fields, the UE determines first and second RS resource indexes qd from respective first and second values of PUSCH-PathlossReferenceRS-Id that are mapped to the first and second SRI values corresponding to each SRS resource set with usage set to ‘codebook’, respectively.

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘nonCodebook’ and the DCI format activating the PUSCH transmission includes two SRI fields, the UE determines first and second RS resource indexes qd from respective first and second values of PUSCH-PathlossReferenceRS-Id that are mapped to the first SRI value corresponding to the first SRS resource set with usage set to ‘nonCodebook’, and the value, associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, corresponding to the second SRS resource set with usage set to ‘nonCodebook’.

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’ and the DCI format activating the PUSCH transmission does not include an SRI field, the UE determines first and second RS resource indexes qd with respective first and second PUSCH-PathlossReferenceRS-Id value being equal to zero and one.

-	If the DCI format activating the PUSCH transmission does not include an SRI field, the UE determines a RS resource index qd with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero

where the RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	If the UE is provided enablePL-RS-UpdateForPUSCH-SRS, a mapping between sri-PUSCH-PowerControlId and PUSCH-PathlossReferenceRS-Id values can be updated by a MAC CE as described in [11, TS 38.321]

-	For a PUSCH transmission scheduled by a DCI format that does not include an SRI field, or for a PUSCH transmission configured by ConfiguredGrantConfig and activated, as described in clause 10.2, by a DCI format that does not include an SRI field, the UE determines a RS resource index qd from the PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0. If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’, the UE determines first and second RS resource indexes qd from respective PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0 and sri-PUSCH-PowerControlId = 1.

-	If the UE is not provided enablePL-RS-UpdateForPUSCH-SRS

-	For a PUSCH transmission scheduled by a DCI format that does not include an SRI field, if the UE is provided two SRS resources in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’, the UE determines first and second RS resource indexes qd with respective first and second PUSCH-PathlossReferenceRS-Id values being equal to 0 and 1.

PLb,f,c(qd)= referenceSignalPower – higher layer filtered RSRP, where referenceSignalPower is provided by higher layers and RSRP is defined in [7, TS 38.215] for the reference serving cell and the higher layer filter configuration provided by QuantityConfig is defined in [12, TS 38.331] for the reference serving cell

If the UE is not configured periodic CSI-RS reception, referenceSignalPower is provided by ss-PBCH-BlockPower. If the UE is configured periodic CSI-RS reception, referenceSignalPower is provided either by ss-PBCH-BlockPower or by powerControlOffsetSS providing an offset of the CSI-RS transmission power relative to the SS/PBCH block transmission power [6, TS 38.214]. If powerControlOffsetSS is not provided to the UE, the UE assumes an offset of 0 dB.

-	∆TF,b,f,ci=10log102BPRE⋅Ks-1⋅βoffsetPUSCH for Ks=1.25 and ∆TF,b,f,ci=0 for Ks=0 where Ks is provided by deltaMCS for each UL BWP b of each carrier f and serving cell c. If the PUSCH transmission is over more than one layer [6, TS 38.214], ∆TF,b,f,ci=0. BPRE and βoffsetPUSCH, for active UL BWP b of each carrier f and each serving cell c, are computed as below

-	BPRE=r=0C-1KrNRE for PUSCH with UL-SCH data and BPRE=Qm⋅RβoffsetPUSCH for CSI transmission in a PUSCH without UL-SCH data, where

-	C is a number of transmitted code blocks, Kr is a size for code block r, and NRE is a number of resource elements determined as NRE=MRB,b,f,cPUSCH(i)⋅j=0Nsymb,b,f,cPUSCHi-1Nsc,dataRB(i,j), where Nsymb,b,f,cPUSCHi is a number of symbols for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c, Nsc,dataRB(i,j is a number of subcarriers excluding DM-RS subcarriers and phase-tracking RS samples [4, TS 38.211] in PUSCH symbol j and assuming no segmentation for a nominal repetition in case the PUSCH transmission is with repetition Type B, 0≤j<Nsymb,b,f,cPUSCHi, and C, Kr are defined in [5, TS 38.212]

-	βoffsetPUSCH=1 when the PUSCH includes UL-SCH data and βoffsetPUSCH=βoffsetCSI,1, as described in clause 9.3, when the PUSCH includes CSI and does not include UL-SCH data

-	Qm is the modulation order and R is the target code rate, as described in [6, TS 38.214], provided by the DCI format scheduling the PUSCH transmission that includes CSI and does not include UL-SCH data

-	For the PUSCH power control adjustment state fb,f,c(i,l) for active UL BWP b of carrier f of serving cell c in PUSCH transmission occasion i

-	δPUSCH,b,f,c(i,l) is a TPC command value included in a DCI format that schedules the PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c or jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, as described in clause 11.3

-	l∈0,1 if the UE is configured with twoPUSCH-PC-AdjustmentStates and l=0 if the UE is not configured with twoPUSCH-PC-AdjustmentStates or if the PUSCH transmission is scheduled by a RAR UL grant as described in clause 8.3

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’, and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format that includes a SRS resource set indicator field, and for active UL BWP b of carrier f of serving cell

-	If the SRS resource set indicator value is 00, l is equal to the value of powerControlLoopToUse in ConfiguredGrantConfig.

-	If the SRS resource set indicator value is 01, l is equal to the value of powerControlLoopToUse2 in ConfiguredGrantConfig.

-	If the SRS resource set indicator value is 10 or 11, a first l and a second l are respectively equal to powerControlLoopToUse and powerControlLoopToUse2 in ConfiguredGrantConfig.

-	else, for a PUSCH (re)transmission configured by ConfiguredGrantConfig, the value of l∈0,1 is provided to the UE by powerControlLoopToUse in ConfiguredGrantConfig.

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’, is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission includes two SRI fields, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the first and second SRI fields and the l values provided by sri-PUSCH-ClosedLoopIndex, and determines the l values mapped to the values of the first and second SRI fields corresponding to each SRS resource set with usage set to ‘codebook’, respectively

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘nonCodebook’, is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission includes two SRI fields,	the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for

-	the first SRI field value and the l values provided by sri-PUSCH-ClosedLoopIndex, and determines the l value that is mapped to the first SRI field value corresponding to the first SRS resource set with usage set to ‘nonCodebook’, and

-	the value, associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, and the l value(s) provided by sri-PUSCH-ClosedLoopIndex, and determines the l value that is mapped to the value corresponding to the second SRS resource set with usage set to ‘nonCodebook’

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’, is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission does not include an SRI field

-	If the UE is provided twoPUSCH-PC-AdjustmentStates

-	the UE determines l=0 for the PUSCH transmission corresponding to the first SRS resource set with usage set to ‘codebook’ or ‘nonCodebook’, and l=1 for the PUSCH transmission corresponding to the second SRS resource set with usage set to ‘codebook’ or ‘nonCodebook’

-	else

-	the UE determines l=0 for the PUSCH transmission

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’, and is provided twoPUSCH-PC-AdjustmentStates

-	If the DCI format includes two TPC command values and the PUSCH transmissions are associated with l=0 and l=1, the UE applies the first TPC command value for l=0 and applies the second TPC command value for l=1

-	If the DCI format includes two TPC command values and the PUSCH transmissions are associated with l=0, the UE applies the first TPC command value for l=0 and ignores the second TPC command value

-	If the DCI format includes two TPC command values and the PUSCH transmissions are associated with l=1, the UE applies the second TPC command value for l=1 and ignores the first TPC command value

-	If the DCI format includes one TPC command value, the UE applies the TPC command value for all l associated with the PUSCH transmission

-	If the UE is provided SRI-PUSCH-PowerControl, the UE obtains a mapping between a set of values for the SRI field in a DCI format scheduling the PUSCH transmission and the l value(s) provided by sri-PUSCH-ClosedLoopIndex and determines the l value that is mapped to the SRI field value

-	If the PUSCH transmission is scheduled by a DCI format that does not include an SRI field, or if an SRI-PUSCH-PowerControl is not provided to the UE, l=0

-	If the UE obtains one TPC command from a DCI format 2_2 with CRC scrambled by a TPC-PUSCH-RNTI, the l value is provided by the closed loop indicator field in DCI format 2_2

-	fb,f,ci,l=fb,f,ci-i0,l+m=0CDi-1δPUSCH,b,f,c(m,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i if the UE is not provided tpc-Accumulation, where

-	The δPUSCH,b,f,c values are given in Table 7.1.1-1

-	m=0CDi-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i0-1 symbols before PUSCH transmission occasion i-i0 and KPUSCH(i) symbols before PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i0>0 is the smallest integer for which KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 is earlier than KPUSCH(i) symbols before PUSCH transmission occasion i

-	If a PUSCH transmission is scheduled by a DCI format, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission

-	If a PUSCH transmission is configured by ConfiguredGrantConfig, KPUSCH(i) is a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c

-	If the UE has reached maximum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≥0, then fb,f,ci,l=fb,f,ci-i0,l

-	If UE has reached minimum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≤0, then fb,f,ci,l=fb,f,ci-i0,l

-	A UE resets accumulation of a PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c to fb,f,ck,l=0,  k=0,1,…,i

-	If a configuration for a corresponding PO_UE_PUSCH,b,f,cj value is provided by higher layers

-	If a configuration for a corresponding αb,f,cj value is provided by higher layers

where l is determined from the value of j as

-	If j>1 and the UE is provided higher SRI-PUSCH-PowerControl, l is the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to j

-	If j>1 and the UE is not provided SRI-PUSCH-PowerControl or j=0, l=0

-	If j=1, l is provided by the value of powerControlLoopToUse

-	fb,f,ci,l=δPUSCH,b,f,ci,l is the PUSCH power control adjustment state for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion  if the UE is provided tpc-Accumulation, where

-	δPUSCH,b,f,c absolute values are given in Table 7.1.1-1

If the UE transmits a PUSCH associated with the first RS resource index qd, the UE applies the first PO_UE_PUSCH,b,f,cj value, the first αb,f,cj value, and fb,f,ci,l for determining PPUSCH,b,f,c(i,j,qd,l). If the UE transmits a PUSCH associated with the second RS resource index qd, the UE applies the second PO_UE_PUSCH,b,f,cj value, the second αb,f,cj value, and fb,f,ci,l or fb,f,ci,0 if twoPUSCH-PC-AdjustmentStates is provided or not provided, respectively, for determining PPUSCH,b,f,c(i,j,qd,l).

-	If the UE receives a random access response message in response to a PRACH transmission or a MsgA transmission on active UL BWP b of carrier f of serving cell c as described in clause 8

-	fb,f,c0,l=∆Prampup,b,f,c+δmsg2,b,f,c, where l=0 and

-	δmsg2,b,f,c is a TPC command value indicated in a random access response grant of the random access response message corresponding to a PRACH transmission according to Type-1 random access procedure, or in a random access response grant of the random access response message corresponding to a MsgA transmission according to Type-2 random access procedure with RAR message(s) for fallbackRAR, on active UL BWP b of carrier f in the serving cell c, and

-	 and ∆Prampup_requested,b,f,c is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last random access preamble for carrier f in the serving cell c, MRB,b,f,cPUSCH(0) is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks for the first PUSCH transmission on active UL BWP b of carrier f of serving cell c, and ∆TF,b,f,c0 is the power adjustment of first PUSCH transmission on active UL BWP b of carrier f of serving cell c.

-	If the UE transmits the PUSCH in PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c as described in clause 8.1A, fb,f,c(0,l)=ΔPrampup,b,f,c, where

-	l=0, and

-	 and ΔPrampup_requested,b,f,c is provided by higher layers and corresponds to the total power ramp-up requested by higher layers, MRB,b,f,cPUSCH(i) is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks, and ΔTF,b,f,c(i) is the power adjustment of the PUSCH transmission in PUSCH transmission occasion i.

Table 7.1.1-1: Mapping of TPC Command Field in a DCI format scheduling a PUSCH transmission, or in DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, or in DCI format 2_3, to absolute and accumulated δPUSCH,b,f,c values or δSRS,b,f,c values

| TPC Command Field | Accumulated δPUSCH,b,f,c or δSRS,b,f,c [dB] | Absolute δPUSCH,b,f,c or δSRS,b,f,c [dB] |
| --- | --- | --- |
| 0 | -1 | -4 |
| 1 | 0 | -1 |
| 2 | 1 | 1 |
| 3 | 3 | 4 |

## 7.2	Physical uplink control channel

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG.

-	When the procedures are applied for MCG, the term 'serving cell' in this clause refers to serving cell belonging to the MCG.

-	When the procedures are applied for SCG, the term 'serving cell' in this clause refers to serving cell belonging to the SCG. The term 'primary cell' in this clause refers to the PSCell of the SCG.

If the UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group.

-	When the procedures are applied for the primary PUCCH group, the term 'serving cell' in this clause refers to serving cell belonging to the primary PUCCH group.

-	When the procedures are applied for the secondary PUCCH group, the term 'serving cell' in this clause refers to serving cell belonging to the secondary PUCCH group. The term 'primary cell' in this clause refers to the PUCCH-SCell of the secondary PUCCH group. If pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 is provided, pdsch-HARQ-ACK-Codebook is replaced by pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16.

### 7.2.1	UE behaviour

If a UE transmits a PUCCH on active UL BWP b of carrier f in the primary cell c using PUCCH power control adjustment state with index l, the UE determines the PUCCH transmission power PPUCCH,b,f,c(i,qu,qd,l) in PUCCH transmission occasion i as

[dBm]

where

-	PCMAX,f,c(i) is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier f of primary cell c in PUCCH transmission occasion i

-	PO_PUCCH,b,f,c(qu) is a parameter composed of the sum of a component PO_NOMINAL,PUCCH, provided by p0-nominal, or PO_NOMINAL,PUCCH=0 dBm if p0-nominal is not provided, for carrier f of primary cell c and, if provided, a component PO_UE_PUCCH(qu) provided by p0-PUCCH-Value in P0-PUCCH for active UL BWP b of carrier f of primary cell c, where 0≤qu<Qu. Qu is a size for a set of PO_UE_PUCCH values provided by maxNrofPUCCH-P0-PerSet. The set of PO_UE_PUCCH values is provided by p0-Set. If p0-Set is not provided to the UE, PO_PUCCH,b,f,cqu=0, 0≤qu<Qu

-	If the UE is provided PUCCH-SpatialRelationInfo, the UE obtains a mapping, by an index provided by p0-PUCCH-Id, between a set of pucch-SpatialRelationInfoId values and a set of p0-PUCCH-Value values. If the UE is provided more than one values for pucch-SpatialRelationInfoId and the UE receives an activation command [11, TS 38.321] indicating a value of pucch-SpatialRelationInfoId, the UE determines the p0-PUCCH-Value value through the link to a corresponding p0-PUCCH-Id index. The UE applies the activation command in the first slot that is after slot k+3⋅Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and μ is the SCS configuration for the PUCCH

-	If the UE is not provided PUCCH-SpatialRelationInfo, the UE obtains the p0-PUCCH-Value value from the P0-PUCCH with p0-PUCCH-Id value equal to the minimum p0-PUCCH-Id value in p0-Set

-	MRB,b,f,cPUCCH(i) is a bandwidth of the PUCCH resource assignment expressed in number of resource blocks for PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c and μ is a SCS configuration defined in [4, TS 38.211]

-	PLb,f,c(qd) is a downlink pathloss estimate in dB calculated by the UE using RS resource index qd as described in clause 7.1.1 for the active DL BWP b of carrier f of the primary cell c as described in clause 12

-	If the UE is not provided pathlossReferenceRSs or before the UE is provided dedicated higher layer parameters, the UE calculates PLb,f,c(qd) using a RS resource obtained from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB

-	If the UE is provided a number of RS resource indexes, the UE calculates PLb,f,c(qd) using RS resource with index qd, where 0≤qd<Qd. Qd is a size for a set of RS resources provided by maxNrofPUCCH-PathlossReferenceRSs. The set of RS resources is provided by pathlossReferenceRSs. The set of RS resources can include one or both of a set of SS/PBCH block indexes, each provided by ssb-Index in PUCCH-PathlossReferenceRS when a value of a corresponding pucch-PathlossReferenceRS-Id maps to a SS/PBCH block index, and a set of CSI-RS resource indexes, each provided by csi-RS-Index when a value of a corresponding pucch-PathlossReferenceRS-Id maps to a CSI-RS resource index. The UE identifies a RS resource in the set of RS resources to correspond either to a SS/PBCH block index or to a CSI-RS resource index as provided by pucch-PathlossReferenceRS-Id in PUCCH-PathlossReferenceRS

-	If the UE is provided pathlossReferenceRSs and PUCCH-SpatialRelationInfo, the UE obtains a mapping, by indexes provided by corresponding values of pucch-PathlossReferenceRS-Id, between a set of pucch-SpatialRelationInfoId values and a set of referenceSignal values provided by PUCCH-PathlossReferenceRS. If the UE is provided more than one values for pucch-SpatialRelationInfoId and the UE receives an activation command [11, TS 38.321] indicating a value of pucch-SpatialRelationInfoId, the UE determines the referenceSignal value in PUCCH-PathlossReferenceRS through the link to a corresponding pucch-PathlossReferenceRS-Id index. The UE applies the activation command in the first slot that is after slot k+3⋅Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and μ is the SCS configuration for the PUCCH

-	If PUCCH-SpatialRelationInfo includes servingCellId indicating a serving cell, the UE receives the RS for resource index qd on the active DL BWP of the serving cell

-	If the UE is provided pathlossReferenceRSs and is not provided PUCCH-SpatialRelationInfo, the UE obtains the referenceSignal value in PUCCH-PathlossReferenceRS from the pucch-PathlossReferenceRS-Id with index 0 in PUCCH-PathlossReferenceRS where the RS resource is either on the primary cell or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	If the UE

-	is not provided pathlossReferenceRSs, and

-	is not provided PUCCH-SpatialRelationInfo, and

-	is provided enableDefaultBeamPL-ForPUCCH, and

-	is not provided coresetPoolIndex value of 1 for any CORESET, or is provided coresetPoolIndex value of 1 for all CORESETs, in ControlResourceSet and no codepoint of a TCI field, if any, in a DCI format of any search space set maps to two TCI states [5, TS 38.212]

the UE determines a RS resource index qd providing a periodic RS resource configured with qcl-Type set to 'typeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the primary cell. If the CORESET has two activated TCI states, as described in clause 10.1, the UE determines the RS resource index qd based on the first activated TCI state. For a PUCCH transmission over multiple slots, a same qd applies to the PUCCH transmission in each of the multiple slots.

-	The parameter ΔF_PUCCHF is a value of deltaF-PUCCH-f0 for PUCCH format 0, deltaF-PUCCH-f1 for PUCCH format 1, deltaF-PUCCH-f2 for PUCCH format 2, deltaF-PUCCH-f3 for PUCCH format 3, and deltaF-PUCCH-f4 for PUCCH format 4, if provided; otherwise ΔF_PUCCHF=0.

-	∆TF,b,f,ci is a PUCCH transmission power adjustment component on active UL BWP b of carrier f of primary cell c

-	For a PUCCH transmission using PUCCH format 0 or PUCCH format 1, ∆TF,b,f,ci=10log10NrefPUCCHNsymbPUCCH(i)+∆UCIi where

-	NsymbPUCCH(i) is a number of PUCCH format 0 symbols or PUCCH format 1 symbols for the PUCCH transmission as described in clause 9.2.

-	NrefPUCCH=2 for PUCCH format 0

-	NrefPUCCH=Nsymbslot for PUCCH format 1

-	∆UCIi=0 for PUCCH format 0

-	∆UCIi=10log10OUCIi for PUCCH format 1, where OUCIi is a number of UCI bits in PUCCH transmission occasion i

-	For a PUCCH transmission using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 and for a number of UCI bits smaller than or equal to 11, ∆TF,b,f,ci=10log10K1⋅nHARQ-ACKi+OSRi+OCSIiNREi, where

-	K1=6

-	nHARQ-ACKi is a number of HARQ-ACK information bits that the UE determines as described in clause 9.1.2.1 or 16.5.1.1 for Type-1 HARQ-ACK codebook and as described in clause 9.1.3.1 or 9.1.3.3 or 16.5.2.1 for Type-2 HARQ-ACK codebook. nHARQ-ACKiis the same as OACKi as described in clause 9.1.4 for Type-3 HARQ-ACK codebook. If the UE is not provided any of pdsch-HARQ-ACK-Codebook, pdsch-HARQ-ACK-Codebook-r16, or pdsch-HARQ-ACK-OneShotFeedback, nHARQ-ACKi=1 if the UE includes a HARQ-ACK information bit in the PUCCH transmission; otherwise, nHARQ-ACKi=0

-	OSRi is a number of SR information bits that the UE determines as described in clause 9.2.5.1

-	OCSIi is a number of CSI information bits that the UE determines as described in clause 9.2.5.2

-	NREi is a number of resource elements determined as NRE(i)=MRB,b,f,cPUCCH(i)⋅Nsc,ctrlRB(i)⋅Nsymb-UCI,b,f,cPUCCH(i), where Nsc,ctrlRB(i) is a number of subcarriers per resource block excluding subcarriers used for DM-RS transmission, and ⋅Nsymb-UCI,b,f,cPUCCH(i) is a number of symbols excluding symbols used for DM-RS transmission, as defined in clause 9.2.5.2, for PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c

-	For a PUCCH transmission using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 and for a number of UCI bits larger than 11, ∆TF,b,f,ci=10log102BPRE⋅K2-1, where

-	K2=2.4

-	BPRE(i)=OACKi+OSRi+OCSIi+OCRCiNRE(i)

-	OACKi is a number of HARQ-ACK information bits that the UE determines as described in clause 9.1.2.1 or 16.5.1.1 for Type-1 HARQ-ACK codebook and as described in clause 9.1.3.1 or 9.1.3.3 or 16.5.2.1 for Type-2 HARQ-ACK codebook, or as described in clause 9.1.4 for Type-3 HARQ-ACK codebook. If the UE is not provided any of pdsch-HARQ-ACK-Codebook, pdsch-HARQ-ACK-Codebook-r16, or pdsch-HARQ-ACK-OneShotFeedback, OACK=1 if the UE includes a HARQ-ACK information bit in the PUCCH transmission; otherwise, OACK=0

-	OSRi is a number of SR information bits that the UE determines as described in clause 9.2.5.1

-	OCSIi is a number of CSI information bits that the UE determines as described in clause 9.2.5.2

-	OCRCi is a number of CRC bits that the UE determines as described in clause 9.2

-	NREi is a number of resource elements that the UE determines as NRE(i)=MRB,b,f,cPUCCH(i)⋅Nsc,ctrlRB(i)⋅Nsymb-UCI,b,f,cPUCCH(i), where Nsc,ctrlRB(i) is a number of subcarriers per resource block excluding subcarriers used for DM-RS transmission, and ⋅Nsymb-UCI,b,f,cPUCCH(i) is a number of symbols excluding symbols used for DM-RS transmission, as defined in clause 9.2.5.2, for PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c.

-	For the PUCCH power control adjustment state gb,f,ci,l for active UL BWP b of carrier f of primary cell c and PUCCH transmission occasion i

-	δPUCCH,b,f,c(i,l) is a TPC command value included in a DCI format associated with the PUCCH transmission for active UL BWP b of carrier f of the primary cell c that the UE detects for PUCCH transmission occasion i, or is jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUCCH-RNTI [5, TS 38.212], as described in clause 11.3

-	l∈0,1 if the UE is provided twoPUCCH-PC-AdjustmentStates and PUCCH-SpatialRelationInfo and l=0 if the UE is not provided twoPUCCH-PC-AdjustmentStates or PUCCH-SpatialRelationInfo

-	If the UE obtains a TPC command value from a DCI format associated with the PUCCH transmission and if the UE is provided PUCCH-SpatialRelationInfo, the UE obtains a mapping, by an index provided by p0-PUCCH-Id, between a set of pucch-SpatialRelationInfoId values and a set of values for closedLoopIndex that provide the l value(s). If the UE receives an activation command indicating a value of pucch-SpatialRelationInfoId, the UE determines the value closedLoopIndex that provides the value of l through the link to a corresponding p0-PUCCH-Id index

-	If the UE obtains a TPC command from a DCI format 2_2 with CRC scrambled by a TPC-PUCCH-RNTI, the l value is provided by the closed loop indicator field in DCI format 2_2

-	If the UE transmits the PUCCH with NPUCCHrepeat>1 repetitions, as described in clause 9.2.6, using a PUCCH resource that includes first and second spatial settings, or first and second sets of power control parameters, and the UE is provided twoPUCCH-PC-AdjustmentStates for the PUCCH resource

-	If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=0 and l=1, the UE applies the first TPC command value for l=0 and applies the second TPC command value for l=1

-	If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=0, the UE applies the first TPC command value for l=0 and ignores the second TPC command value

-	If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=1, the UE applies the second TPC command value for l=1 and ignores the first TPC command value

-	If the DCI format includes one TPC command value, the UE applies the TPC command value for all l associated with the PUCCH resource of the PUCCH transmission

-	gb,f,ci,l=gb,f,ci-i0,l+m=0CDi-1δPUCCH,b,f,c(m,l) is the current PUCCH power control adjustment state l for active UL BWP b of carrier f of primary cell c and PUCCH transmission occasion i, where

-	The δPUCCH,b,f,c values are given in Table 7.1.2-1

-	m=0CDi-1δPUCCH,b,f,c(m,l) is a sum of TPC command values in a set Ci of TPC command values with cardinality CCi that the UE receives between KPUCCHi-i0-1 symbols before PUCCH transmission occasion i-i0 and  symbols before PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c for PUCCH power control adjustment state, where i0>0 is the smallest integer for which KPUCCHi-i0 symbols before PUCCH transmission occasion i-i0 is earlier than KPUCCHi symbols before PUCCH transmission occasion i

-	If the PUCCH transmission is in response to a detection by the UE of a DCI format, KPUCCHi is a number of symbols for active UL BWP b of carrier f of primary cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUCCH transmission

-	If the PUCCH transmission is not in response to a detection by the UE of a DCI format, KPUCCHi is a number of KPUCCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of primary cell c

-	If the UE has reached maximum power for active UL BWP b of carrier f of primary cell c at PUCCH transmission occasion i-i0 and m=0CDi-1δPUCCH,b,f,c(m,l)≥0, then gb,f,ci,l=gb,f,ci-i0,l

-	If UE has reached minimum power for active UL BWP b of carrier f of primary cell c at PUCCH transmission occasion i-i0 and m=0CDi-1δPUCCH,b,f,c(m,l)≤0, then gb,f,ci,l=gb,f,ci-i0,l

-	If a configuration of a PO_PUCCH,b,f,cqu value for a corresponding PUCCH power control adjustment state l for active UL BWP b of carrier f of primary cell c is provided by higher layers,

-	gb,f,ck,l=0,  k=0,1,…,i

If the UE is provided PUCCH-SpatialRelationInfo, the UE determines the value of l from the value of qu based on a pucch-SpatialRelationInfoId value associated with the p0-PUCCH-Id value corresponding to qu and with the closedLoopIndex value corresponding to l; otherwise, l=0

-	Else,

-	gb,f,c0,l=∆Prampup,b,f,c+δb,f,c, where l=0, and δb,f,c is

-	the TPC command value indicated in a random access response grant corresponding to a PRACH transmission according to Type-1 random access procedure, or in a random access response grant corresponding to MsgA transmissions according to Type-2 random access procedure with RAR message(s) for fallbackRAR, or

-	the TPC command value indicated in a successRAR corresponding to MsgA transmissions for Type-2 random access procedure, or

-	the TPC command value in a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI that the UE detects in a first PDCCH reception in a search space set provided by recoverySearchSpaceId if the PUCCH transmission is a first PUCCH transmission after 28 symbols from a last symbol of the first PDCCH reception,

and, if the UE transmits PUCCH on active UL BWP b of carrier f of primary cell c,

∆Prampup,b,f,c=minmax0,PCMAX,f,c-PO_PUCCH,b,f,c+PLb,f,c(qd)+∆F_PUCCH+δb,f,c,∆Prampup_requested,b,f,c;

otherwise,

∆Prampup,b,f,c=minmax0,PCMAX,f,c-PO_PUCCH,b,f,c+PLb,f,c(qd),∆Prampup_requested,b,f,c where ∆Prampup_requested,b,f,c is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last preamble for active UL BWP b of carrier f of primary cell c, and ∆F_PUCCH corresponds to PUCCH format 0 or PUCCH format 1

Table 7.2.1-1: Mapping of TPC Command Field in a DCI format to accumulated δPUCCH,b,f,c values

| TPC Command Field | Accumulated δPUCCH,b,f,c [dB] |
| --- | --- |
| 0 | -1 |
| 1 | 0 |
| 2 | 1 |
| 3 | 3 |

## 7.3	Sounding reference signals

For SRS, a UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the configured antenna ports for SRS.

### 7.3.1	UE behaviour

If a UE transmits SRS based on a configuration by SRS-ResourceSet on active UL BWP b of carrier f of serving cell c using SRS power control adjustment state with index l, the UE determines the SRS transmission power PSRS,b,f,c(i,qs,l) in SRS transmission occasion i as

[dBm]

where,

-	PCMAX,f,c(i) is the UE configured maximum output power defined in [8, TS 38.101-1], [8-2, TS38.101-2] and [TS 38.101-3] for carrier f of serving cell c in SRS transmission occasion i

-	PO_SRS,b,f,c(qs) is provided by p0 for active UL BWP b of carrier f of serving cell c and SRS resource set qs provided by SRS-ResourceSet and SRS-ResourceSetId

-	MSRS,b,f,c(i) is a SRS bandwidth expressed in number of resource blocks for SRS transmission occasion i on active UL BWP b of carrier f of serving cell c and μ is a SCS configuration defined in [4, TS 38.211]

-	αSRS,b,f,c(qs) is provided by alpha for active UL BWP b of carrier f of serving cell c and SRS resource set qs

-	PLb,f,c(qd) is a downlink pathloss estimate in dB calculated by the UE using RS resource index qd as described in clause 7.1.1 for the active DL BWP of serving cell c and SRS resource set qs [6, TS 38.214]. The RS resource index qd is provided by pathlossReferenceRS associated with the SRS resource set qs and is either an ssb-Index providing a SS/PBCH block index or a csi-RS-Index providing a CSI-RS resource index. If the UE is provided enablePL-RS-UpdateForPUSCH-SRS, a MAC CE [11, TS 38.321] can provide by SRS-PathlossReferenceRS-Id a corresponding RS resource index qd for aperiodic or semi-persistent SRS resource set qs

-	If the UE is not provided pathlossReferenceRS or SRS-PathlossReferenceRS-Id, or before the UE is provided dedicated higher layer parameters, the UE calculates PLb,f,c(qd) using a RS resource obtained from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB

-	If the UE is provided pathlossReferenceLinking, the RS resource is on a serving cell indicated by a value of pathlossReferenceLinking

-	If the UE

-	is not provided pathlossReferenceRS or SRS-PathlossReferenceRS-Id,

-	is not provided spatialRelationInfo, and

-	is provided enableDefaultBeamPL-ForSRS, and

-	is not provided coresetPoolIndex value of 1 for any CORESET, or is provided coresetPoolIndex value of 1 for all CORESETs, in ControlResourceSet and no codepoint of a TCI field, if any, in a DCI format of any search space set maps to two TCI states [5, TS 38.212]

the UE determines a RS resource index qd providing a periodic RS resource configured with qcl-Type set to 'typeD' in

-	the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP, if CORESETs are provided in the active DL BWP of serving cell c. If the CORESET has two activated TCI states, as described in clause 10.1, the UE determines the RS resource index qd based on the first TCI state.

-	the active PDSCH TCI state with lowest ID [6, TS 38.214] in the active DL BWP, if CORESETs are not provided in the active DL BWP of serving cell c

-	For the SRS power control adjustment state for active UL BWP b of carrier f of serving cell c and SRS transmission occasion i

-	hb,f,ci,l=fb,f,ci,l, where fb,f,ci,l is the current PUSCH power control adjustment state as described in clause 7.1.1, if srs-PowerControlAdjustmentStates indicates a same power control adjustment state for SRS transmissions and PUSCH transmissions; or

-	 if the UE is not configured for PUSCH transmissions on active UL BWP b of carrier f of serving cell c, or if srs-PowerControlAdjustmentStates indicates separate power control adjustment states between SRS transmissions and PUSCH transmissions, and if tpc-Accumulation is not provided, where

-	The δSRS,b,f,c values are given in Table 7.1.1-1

-	δSRS,b,f,c(m) is jointly coded with other TPC commands in a PDCCH with DCI format 2_3, as described in clause 11.4

-	m=0CSi-1δSRS,b,f,c(m) is a sum of TPC command values in a set Si of TPC command values with cardinality CSi that the UE receives between KSRSi-i0-1 symbols before SRS transmission occasion i-i0 and KSRS(i) symbols before SRS transmission occasion i on active UL BWP b of carrier f of serving cell c for SRS power control adjustment state, where i0>0 is the smallest integer for which KSRS(i) symbols before SRS transmission occasion i-i0 is earlier than KSRS(i-i0) symbols before SRS transmission occasion i

-	if the SRS transmission is aperiodic, KSRS(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH triggering the SRS transmission and before a first symbol of the SRS transmission

-	if the SRS transmission is semi-persistent or periodic, KSRS(i) is a number of KSRS,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c

-	If the UE has reached maximum power for active UL BWP b of carrier f of serving cell c at SRS transmission occasion i-i0 and m=0CSi-1δSRS,b,f,c(m)≥0, then hb,f,ci=hb,f,ci-i0

-	If UE has reached minimum power for active UL BWP b of carrier f of serving cell c at SRS transmission occasion i-i0 and m=0CSi-1δSRS,b,f,c(m)≤0, then hb,f,ci=hb,f,ci-i0

-	If a configuration for a PO_SRS,b,f,c(qs) value or for a αSRS,b,f,c(qs)value for a corresponding SRS power control adjustment state l for active UL BWP b of carrier f of serving cell c is provided by higher layers

-	hb,f,ck=0, k=0,1,…,i

-	Else

-	hb,f,c(0)=∆Prampup,b,f,c+δmsg2,b,f,c

where

δmsg2,b,f,c is the TPC command value indicated in the random access response grant corresponding to the random access preamble that the UE transmitted on active UL BWP b of carrier f of the serving cell c, and

∆Prampup,b,f,c=minmax0,PCMAX,f,c-PO_SRS,b,f,cqs+10log102μ⋅MSRS,b,f,ci+αSRS,b,f,cqs⋅PLb,f,c(qd),∆Prampup_requested,b,f,c;

where ∆Prampuprequested,b,f,c is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last preamble for active UL BWP b of carrier f of serving cell c.

-	hb,f,c(i)=δSRS,b,f,c(i) if the UE is not configured for PUSCH transmissions on active UL BWP b of carrier f of serving cell c, or if srs-PowerControlAdjustmentStates indicates separate power control adjustment states between SRS transmissions and PUSCH transmissions, and tpc-Accumulation is provided, and the UE detects a DCI format 2_3 KSRS,min symbols before a first symbol of SRS transmission occasion i, where absolute values of δSRS,b,f,c  are provided in Table 7.1.1-1

-	if srs-PowerControlAdjustmentStates indicates a same power control adjustment state for SRS transmissions and PUSCH transmissions, the update of the power control adjustment state for SRS transmission occasion i occurs at the beginning of each SRS resource in the SRS resource set qs; otherwise, the update of the power control adjustment state SRS transmission occasion i occurs at the beginning of the first transmitted SRS resource in the SRS resource set qs.

If a UE transmits SRS based on a configuration by SRS-PosResourceSet on active UL BWP b of carrier f of serving cell c, the UE determines the SRS transmission power PSRS,b,f,ci,qs in SRS transmission occasion i as

[dBm]

where,

-	PO_SRS,b,f,cqs and αSRS,b,f,cqs are provided by p0-r16 and alpha-r16 respectively, for active UL BWP b of carrier f of serving cell c, and SRS resource set qs is indicated by SRS-PosResourceSetId from SRS-PosResourceSet, and

-	PLb,f,cqd is a downlink pathloss estimate in dB calculated by the UE, as described in clause 7.1.1 in case of an active DL BWP of a serving cell c, using RS resource indexed qd in a serving or non-serving cell for SRS resource set qs [6, TS 38.214]. A configuration for RS resource index qd associated with SRS resource set qs is provided by pathlossReferenceRS-Pos

-	if a ssb-IndexNcell is provided, referenceSignalPower is provided by ss-PBCH-BlockPower-r16

-	if a dl-PRS-ResourceId is provided, referenceSignalPower is provided by dl-PRS-ResourcePower

If the UE determines that the UE is not able to accurately measure PLb,f,cqd, or the UE is not provided with pathlossReferenceRS-Pos, the UE calculates PLb,f,cqd using a RS resource obtained from the SS/PBCH block of the serving cell that the UE uses to obtain MIB

The UE may indicate a capability for a number of pathloss estimates that the UE can simultaneously maintain for all SRS resource sets provided by SRS-PosResourceSet in addition to the up to four pathloss estimates that the UE maintains per serving cell for PUSCH/PUCCH transmissions and for SRS transmissions configured by SRS-Resource.

## *** Unchanged text is omitted ***

### 7.7.1	Type 1 PH report

If a UE determines that a Type 1 power headroom report for an activated serving cell is based on an actual PUSCH transmission then, for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c, the UE computes the Type 1 power headroom report as

[dB]

where PCMAX,f,c(i), PO_PUSCH,b,f,c(j), MRB,b,f,cPUSCH(i), αb,f,cj, PLb,f,c(qd), ∆TF,b,f,ci and fb,f,ci,l are defined in clause 7.1.1.

If a UE is configured with multiple cells for PUSCH transmissions, where a SCS configuration μ1 on active UL BWP b1 of carrier f1 of serving cell c1 is smaller than a SCS configuration μ2 on active UL BWP b2 of carrier f2 of serving cell c2, and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP b1 that overlaps with multiple slots on active UL BWP b2, the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the first slot of the multiple slots on active UL BWP b2 that fully overlaps with the slot on active UL BWP b1. If a UE is configured with multiple cells for PUSCH transmissions, where a same SCS configuration on active UL BWP b1 of carrier f1 of serving cell c1 and active UL BWP b2 of carrier f2 of serving cell c2, and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP b1, the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the slot on active UL BWP b2 that overlaps with the slot on active UL BWP b1.

If a UE is configured with multiple cells for PUSCH transmissions and provides a Type 1 power headroom report in a PUSCH transmission with PUSCH repetition Type B having a nominal repetition that spans multiple slots on active UL BWP b1 and overlaps with one or more slots on active UL BWP b2, the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the first slot of the one or more slots on active UL BWP b2 that overlaps with the multiple slots of the nominal repetition on active UL BWP b1.

For a UE configured with EN-DC/NE-DC and capable of dynamic power sharing, if E-UTRA Dual Connectivity PHR [14, TS 36.321] is triggered, the UE provides power headroom of the first PUSCH, if any, on the determined NR slot as described in clause 7.7.

If a UE is configured with multiple cells for PUSCH transmissions, the UE does not consider for computation of a Type 1 power headroom report in a first PUSCH transmission that includes an initial transmission of transport block on active UL BWP b1 of carrier f1 of serving cell c1, a second PUSCH transmission on active UL BWP b2 of carrier f2 of serving cell c2 that overlaps with the first PUSCH transmission if

-	the second PUSCH transmission is scheduled by a DCI format in a PDCCH received in a second PDCCH monitoring occasion, and

-	the second PDCCH monitoring occasion is after a first PDCCH monitoring occasion where the UE detects the earliest DCI format scheduling an initial transmission of a transport block after a power headroom report was triggered

or

-	the second PUSCH transmission is after the first uplink symbol of the first PUSCH transmission minus T'proc,2=Tproc,2 where Tproc,2 is determined according to [6, TS 38.214] assuming d2,1 =1, d2,2=0, and with µDL corresponding to the subcarrier spacing of the active downlink BWP of the scheduling cell for a configured grant if the first PUSCH transmission is on a configured grant after a power headroom report was triggered.

If the UE determines that a Type 1 power headroom report for an activated serving cell is based on a reference PUSCH transmission then, for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c, the UE computes the Type 1 power headroom report as

[dB]

where PCMAX,f,c(i) is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB. TC = 0 dB. MPR, A-MPR, P-MPR and TC are defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS 38.101-3]. The remaining parameters are defined in clause 7.1.1 where PO_PUSCH,b,f,c(j) and αb,f,cj are obtained using PO_NOMINAL,PUSCH,f,c0 and p0-PUSCH-AlphaSetId = 0, PLb,f,c(qd) is obtained using pusch-PathlossReferenceRS-Id = 0, and l=0.

If a UE is configured with two UL carriers for a serving cell and the UE determines a Type 1 power headroom report for the serving cell based on a reference PUSCH transmission, the UE computes a Type 1 power headroom report for the serving cell assuming a reference PUSCH transmission on the UL carrier provided by pusch-Config. If the UE is provided pusch-Config for both UL carriers, the UE computes a Type 1 power headroom report for the serving cell assuming a reference PUSCH transmission on the UL carrier provided by pucch-Config. If pucch-Config is not provided to the UE for any of the two UL carriers, the UE computes a Type 1 power headroom report for the serving cell assuming a reference PUSCH transmission on the non-supplementary UL carrier.

If a UE transmits a PUSCH associated with a RS resource index qd, as described in clause 7.1.1, on active UL BWP b of carrier f of serving cell c in slot n and provides a Type 1 power headroom report for an actual PUSCH repetition associated with the RS resource index qd, the Type 1 power headroom report is for the first PUSCH repetition associated with the RS resource index qd that overlaps with slot n.

If a UE transmits a PUSCH associated with a first RS resource index qd, as described in clause 7.1.1, on active UL BWP b of carrier f of serving cell c in slot n and is provided twoPHRMode, the UE provides a Type 1 power headroom report for PUSCH repetition associated with a second RS resource index qd, as described in clause 7.1.1, where

-	if the UE provides a Type 1 power headroom report for an actual PUSCH repetition associated with the first RS resource index qd,

-	if the UE transmits PUSCH repetitions associated with the second RS resource index qd in slot n, the UE provides a Type 1 power headroom report for a first actual PUSCH repetition associated with the second RS resource index qd that overlaps with slot n

-	otherwise, the UE provides a Type 1 power headroom report for a reference PUSCH transmission associated with the second RS resource index qd

-	otherwise, if the UE provides a Type 1 power headroom report for a reference PUSCH transmission associated with the first RS resource index qd, the UE provides a Type 1 power headroom report for a reference PUSCH transmission associated with the second RS resource index qd

## *** Unchanged text is omitted ***

## 8.1	Random access preamble

Physical random access procedure is triggered upon request of a PRACH transmission by higher layers or by a PDCCH order. A configuration by higher layers for a PRACH transmission includes the following:

-	A configuration for PRACH transmission [4, TS 38.211].

-	A preamble index, a preamble SCS, , a corresponding RA-RNTI, and a PRACH resource.

A PRACH is transmitted using the selected PRACH format with transmission power , as described in clause 7.4, on the indicated PRACH resource.

For Type-1 random access procedure, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion and a number R of contention based preambles per SS/PBCH block index per valid PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB.

For Type-2 random access procedure with common configuration of PRACH occasions with Type-1 random access procedure, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB and a number Q of contention based preambles per SS/PBCH block index per valid PRACH occasion by msgA-CB-PreamblesPerSSB-PerSharedRO. The PRACH transmission can be on a subset of PRACH occasions associated with a same SS/PBCH block index within an SSB-RO mapping cycle for a UE provided with a PRACH mask index by msgA-SSB-SharedRO-MaskIndex according to [11, TS 38.321].

For Type-2 random access procedure with separate configuration of PRACH occasions with Type-1 random access procedure, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion and a number R of contention based preambles per SS/PBCH block index per valid PRACH occasion by msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB when provided; otherwise, by ssb-perRACH-OccasionAndCB-PreamblesPerSSB.

For Type-1 random access procedure, or for Type-2 random access procedure with separate configuration of PRACH occasions from Type 1 random access procedure, if N<1, one SS/PBCH block index is mapped to 1N consecutive valid PRACH occasions and R contention based preambles with consecutive indexes associated with the SS/PBCH block index per valid PRACH occasion start from preamble index 0. If N≥1, R contention based preambles with consecutive indexes associated with SS/PBCH block index n, 0≤n≤N-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalN where Npreambletotal is provided by totalNumberOfRA-Preambles for Type-1 random access procedure, or by msgA-TotalNumberOfRA-Preambles for Type-2 random access procedure with separate configuration of PRACH occasions from a Type 1 random access procedure, and is an integer multiple of N.

For Type-2 random access procedure with common configuration of PRACH occasions with Type-1 random access procedure, if N<1, one SS/PBCH block index is mapped to 1N consecutive valid PRACH occasions and Q contention based preambles with consecutive indexes associated with the SS/PBCH block index per valid PRACH occasion start from preamble index R. If N≥1, Q contention based preambles with consecutive indexes associated with SS/PBCH block index n, 0≤n≤N-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalN+R, where Npreambletotal is provided by totalNumberOfRA-Preambles for Type-1 random access procedure.

For link recovery, a UE is provided N SS/PBCH block indexes associated with one PRACH occasion by ssb-perRACH-Occasion in BeamFailureRecoveryConfig. For a dedicated RACH configuration provided by RACH-ConfigDedicated, if cfra is provided, a UE is provided N SS/PBCH block indexes associated with one PRACH occasion by ssb-perRACH-Occasion in occasions. If N<1, one SS/PBCH block index is mapped to 1N consecutive valid PRACH occasions. If N≥1, all consecutive N SS/PBCH block indexes are associated with one PRACH occasion.

SS/PBCH block indexes provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon are mapped to valid PRACH occasions in the following order where the parameters are described in [4, TS 38.211].

-	First, in increasing order of preamble indexes within a single PRACH occasion

-	Second, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	Third, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

-	Fourth, in increasing order of indexes for PRACH slots

An association period, starting from frame 0, for mapping SS/PBCH block indexes to PRACH occasions is the smallest value in the set determined by the PRACH configuration period according Table 8.1-1 such that  SS/PBCH block indexes are mapped at least once to the PRACH occasions within the association period, where a UE obtains  from the value of ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon. If after an integer number of SS/PBCH block indexes to PRACH occasions mapping cycles within the association period there is a set of PRACH occasions or PRACH preambles that are not mapped to  SS/PBCH block indexes, no SS/PBCH block indexes are mapped to the set of PRACH occasions or PRACH preambles. An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec. PRACH occasions not associated with SS/PBCH block indexes after an integer number of association periods, if any, are not used for PRACH transmissions.

For a PRACH transmission triggered by a PDCCH order, the PRACH mask index field [5, TS 38.212], if the value of the random access preamble index field is not zero, indicates the PRACH occasion for the PRACH transmission where the PRACH occasions are associated with the SS/PBCH block index indicated by the SS/PBCH block index field of the PDCCH order. If the PDCCH reception for the PDCCH order includes two PDCCH candidates from two linked search space sets based on searchSpaceLinking, as described in clause 10.1, the last symbol of the PDCCH reception is the last symbol of the PDCCH candidate that ends later. The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10, 11.1, and 11.1.1.

For a PRACH transmission triggered by higher layers, if ssb-ResourceList is provided, the PRACH mask index is indicated by ra-ssb-OccasionMaskIndex which indicates the PRACH occasions for the PRACH transmission where the PRACH occasions are associated with the selected SS/PBCH block index.

The PRACH occasions are mapped consecutively per corresponding SS/PBCH block index. The indexing of the PRACH occasion indicated by the mask index value is reset per mapping cycle of consecutive PRACH occasions per SS/PBCH block index. The UE selects for a PRACH transmission the PRACH occasion indicated by PRACH mask index value for the indicated SS/PBCH block index in the first available mapping cycle.

For the indicated preamble index, the ordering of the PRACH occasions is

-	First, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	Second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

-	Third, in increasing order of indexes for PRACH slots

For a PRACH transmission triggered upon request by higher layers, a value of ra-OccasionList [12, TS 38.331], if csirs-ResourceList is provided, indicates a list of PRACH occasions for the PRACH transmission where the PRACH occasions are associated with the selected CSI-RS index indicated by csi-RS. The indexing of the PRACH occasions indicated by ra-OccasionList is reset per association pattern period.

Table 8.1-1: Mapping between PRACH configuration period and SS/PBCH block to PRACH occasion association period

| PRACH configuration period (msec) | Association period (number of PRACH configuration periods) |
| --- | --- |
| 10 | {1, 2, 4, 8, 16} |
| 20 | {1, 2, 4, 8} |
| 40 | {1, 2, 4} |
| 80 | {1, 2} |
| 160 | {1} |

For paired spectrum or supplementary uplink band all PRACH occasions are valid.

For unpaired spectrum,

-	if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last SS/PBCH block reception symbol, where Ngap is provided in Table 8.1-2 and, if channelAccessMode = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where the UE does not transmit [15, TS 37.213].

-	the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon , as described in clause 4.1

-	If a UE is provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if

-	it is within UL symbols, or

-	it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2, and if channelAccessMode = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where there shall not be any transmissions, as described in [15, TS 37.213]

-	the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon, as described in clause 4.1.

For preamble format B4 [4, TS 38.211], Ngap=0.

Table 8.1-2: Ngap values for different preamble SCS μ

| Preamble SCS | Ngap |
| --- | --- |
| 1.25 kHz or 5 kHz | 0 |
| 15 kHz or 30 kHz or 60 kHz or 120 kHz | 2 |

If a random access procedure is initiated by a PDCCH order, the UE, if requested by higher layers, transmits a PRACH in the selected PRACH occasion, as described in [11, TS 38.321], for which a time between the last symbol of the PDCCH order reception and the first symbol of the PRACH transmission is larger than or equal to NT,2+ ∆BWPSwitching+∆Delay+Tswitch msec, where

-	NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214] assuming μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH order and the SCS configuration of the corresponding PRACH transmission

-	∆BWPSwitching=0 if the active UL BWP does not change and ∆BWPSwitching is defined in [10, TS 38.133] otherwise

-	∆Delay=0.5 msec for FR1 and ∆Delay=0.25 msec for FR2

-	Tswitch is a switching gap duration as defined in [6, TS 38.214]

For a PRACH transmission using 1.25 kHz or 5 kHz SCS, the UE determines N2 assuming SCS configuration μ=0.

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration for the active UL BWP. For a PUSCH transmission with repetition Type B, this applies to each actual repetition for PUSCH transmission [6, TS 38.214].

## *** Unchanged text is omitted ***

# 9	UE procedure for reporting control information

If a UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG.

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG.

If a UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group

-	When the procedures are applied for the primary PUCCH group, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the primary PUCCH group respectively.

-	When the procedures are applied for secondary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including the PUCCH-SCell), serving cell, serving cells belonging to the secondary PUCCH group respectively. The term 'primary cell' in this clause refers to the PUCCH-SCell of the secondary PUCCH group. If pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 is provided, pdsch-HARQ-ACK-Codebook is replaced by pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16. If harq-ACK-SpatialBundlingPUCCH-secondaryPUCCHgroup is provided, harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUCCH-secondaryPUCCHgroup. If harq-ACK-SpatialBundlingPUSCH-secondaryPUCCHgroup is provided, harq-ACK-SpatialBundlingPUSCH is replaced by harq-ACK-SpatialBundlingPUSCH-secondaryPUCCHgroup.

If a UE is provided pdsch-HARQ-ACK-CodebookList-r16, pdsch-HARQ-ACK-Codebook is replaced by the relevant entry in pdsch-HARQ-ACK-CodebookList-r16.

In the remaining of this clause, when a PDCCH reception by a UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1

-	a PDCCH monitoring occasion is the union of the PDCCH monitoring occasions for the two PDCCH candidates

-	the start of the PDCCH reception is the start of the earlier PDCCH candidate

-	the end of the PDCCH reception is the end of the PDCCH candidate that ends later

The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10, 11.1, and 11.1.1.

In the remaining of this clause, a last DCI format is the DCI format that a UE detects in a last PDCCH monitoring occasion from the PDCCH monitoring occasions for which the UE would provide HARQ-ACK information in a PUCCH in a same slot.

If a UE

-	is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and

-	is provided coresetPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and

-	is provided ackNackFeedbackMode = separate

the UE shall separately apply the procedures described in clauses 9.1 and 9.2.3 for reporting HARQ-ACK information associated with the first CORESETs on active DL BWP of the serving cells and for reporting HARQ-ACK information associated with the second CORESETs on active DL BWP of the serving cells, and the UE does not expect to be provided with subslotLengthForPUCCH or to be indicated by pdsch-HARQ-ACK-CodebookList to generate two HARQ-ACK codebooks on active DL BWP of the serving cells. HARQ-ACK information reporting is associated with a CORESET through a reception of a PDCCH with a DCI format triggering the reporting of the HARQ-ACK information by the UE.

For NR-DC when both the MCG and the SCG operate either in FR1 or in FR2 and for a power headroom report transmitted on the MCG or the SCG, the UE computes PH assuming that the UE does not transmit PUSCH/PUCCH on any serving cell of the SCG or the MCG, respectively.

If a UE is configured for NR-DC operation, the UE does not expect to be configured with a PUCCH-SCell.

A PUSCH or a PUCCH transmission other than PUCCH transmissions with SL HARQ-ACK reports, including repetitions if any, can be of priority index 0 or of priority index 1. For a configured grant PUSCH transmission, a UE determines a priority index from phy-PriorityIndex, if provided. For a PUCCH transmission with HARQ-ACK information corresponding to a SPS PDSCH reception or a SPS PDSCH release, a UE determines a priority index from harq-CodebookID, if provided. For a PUCCH transmission with SR, a UE determines the corresponding priority as described in clause 9.2.4. For a PUSCH transmission with semi-persistent CSI report, a UE determines a priority index from a priority indicator field, if provided, in a DCI format that activates the semi-persistent CSI report. If a priority index is not provided to a UE for a PUSCH or a PUCCH transmission other than PUCCH transmissions with SL HARQ-ACK reports, the priority index is 0.

If a UE is provided one PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH in the PUCCH-Config, the PUCCH resource for any SR configuration with priority index 0 or any CSI report configuration in the PUCCH-Config is within the subslotLengthForPUCCH symbols in the PUCCH-Config

If a UE is provided two PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH in the first PUCCH-Config, the PUCCH resource for any SR configuration with priority index 0 or any CSI report configuration in any PUCCH-Config is within the subslotLengthForPUCCH symbols in the first PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH in the second PUCCH-Config, the PUCCH resource for any SR configuration with priority index 1 in any PUCCH-Config is within the subslotLengthForPUCCH symbols in the second PUCCH-Config

If in an active DL BWP a UE monitors PDCCH either for detection of DCI format 0_1 and DCI format 1_1 or for detection of DCI format 0_2 and DCI format 1_2, a priority index can be provided by a priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format 0_1 and DCI format 1_1 and for detection of DCI format 0_2 and DCI format 1_2, a DCI format 0_1 or a DCI format 0_2 can schedule a PUSCH transmission of any priority, a DCI format 1_1 or a DCI format 1_2 can schedule a PDSCH reception and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority, a DCI format 1_1 can indicate SCell dormancy and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority, and DCI format 1_1 or DCI format 1_2 can indicate a TCI state update and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority.

A DCI format indicating a SPS PDSCH release, or SCell dormancy without scheduling a PDSCH reception, or indicating a TCI state update without scheduling PDSCH reception, is referred to as a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception.

When a UE determines overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of larger and/or smaller priority index, the UE resolves the overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of each priority index as described in clause 9.2.5 and 9.2.6 before resolving the overlapping for PUCCH transmissions without SL HARQ-ACK or the overlapping for PUCCH transmissions and PUSCH transmissions.

When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission as described in clause 11.1, including repetitions if any, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in clauses 9.2.5 and 9.2.6. Then,

-	if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission

-	if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission

where

-	the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in clauses 9.2.5 and 9.2.6

-	any remaining PUCCH and/or PUSCH transmission after overlapping resolution is subjected to the limitations for UE transmission as described in clause 11.1

-	the UE expects that the transmission of the first PUCCH or the first PUSCH, respectively, would not start before Tproc,2 after a last symbol of the corresponding PDCCH reception

-	Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1= d1 [6, TS 38.214], based on μ and N2 as subsequently defined in this clause, and d1 is determined by a reported UE capability

If a UE is scheduled by a DCI format in a first PDCCH reception to transmit a first PUCCH or a first PUSCH of larger priority index that overlaps with a second PUCCH or a second PUSCH transmission of smaller priority index that, if any, is scheduled by a DCI format in a second PDCCH

-	Tproc,2 is based on a value of μ corresponding to the smallest SCS configuration of the first PDCCH, the second PDCCHs, the first PUCCH or the first PUSCH, and the second PUCCHs or the second PUSCHs

-	if the overlapping group includes the first PUCCH

-	if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell where the UE receives the first PDCCH and for all serving cells where the UE receives the PDSCHs corresponding to the second PUCCHs, and if processingType2Enabled of PUSCH-ServingCellConfig is set to enable for the serving cells with the second PUSCHs, N2 is 5 for μ=0, 5.5 for μ=1 and 11 for μ=2

-	else, N2 is 10 for μ=0, 12 for μ=1, 23 for μ=2, and 36 for μ=3;

-	if the overlapping group includes the first PUSCH

-	if processingType2Enabled of PUSCH-ServingCellConfig is set to enable for the serving cells with the first PUSCH and the second PUSCHs and if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for all serving cells where the UE receives the PDSCHs corresponding to the second PUCCHs, N2 is 5 for μ=0, 5.5 for μ=1 and 11 for μ=2

-	else, N2 is 10 for μ=0, 12 for μ=1, 23 for μ=2, and 36 for μ=3;

If a UE would transmit the following channels, including repetitions if any, that would overlap in time

-	a first PUCCH of larger priority index with SR and a second PUCCH or PUSCH of smaller priority index, or

-	a configured grant PUSCH of larger priority index and a PUCCH of smaller priority index, or

-	a first PUCCH of larger priority index with HARQ-ACK information only in response to PDSCH(s) reception without corresponding PDCCH(s) and a second PUCCH of smaller priority index with HARQ-ACK information only in response to PDSCH(s) reception without corresponding PDCCH(s), or a second PUCCH of smaller priority index with SR and/or CSI, or a configured grant PUSCH with smaller priority index, or a PUSCH of smaller priority index with SP-CSI report(s) without a corresponding PDCCH, or

-	a PUSCH of larger priority index with SP-CSI reports(s) without a corresponding PDCCH and a PUCCH of smaller priority index with SR, or CSI, or HARQ-ACK information only in response to PDSCH(s) reception without corresponding PDCCH(s), or

-	a configured grant PUSCH of larger priority index and a configured PUSCH of smaller priority index on a same serving cell

the UE is expected to cancel a repetition of the PUCCH/PUSCH transmissions of smaller priority index before the first symbol overlapping with the PUCCH/PUSCH transmission of larger priority index if the repetition of the PUCCH/PUSCH transmissions of smaller priority index overlaps in time with the PUCCH/PUSCH transmissions of larger priority index.

When a UE determines overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUSCH of smaller priority index, including repetitions if any, after resolving the overlapping PUCCH other than PUCCH transmissions with SL HARQ-ACK reports and/or PUSCH transmissions, if the PUSCH includes no UCI, the UE resolves the overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUSCH of smaller priority index as described in clauses 9.2.5 and 9.2.6.

When a UE determines overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUSCH of larger priority index only, including repetitions if any, after resolving the overlapping PUCCH other than PUCCH transmissions with SL HARQ-ACK reports and/or PUSCH transmissions, the UE does not transmit the PUCCH with SL HARQ-ACK reports

where

-	the UE expects that the transmission of the PUSCH would not start before Tproc,2+d1 after a last symbol of the corresponding PDCCH reception;

-	Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1=0 [6, TS 38.214], based on μ and N2 as subsequently defined in this clause, and d1 is determined by a reported UE capability.

The UE expects the PUCCH and PUSCH transmissions fulfill the conditions in clause 9 and clause 9.2.5 for UCI multiplexing replacing the reference time of "end of PDSCH" with "end of the last symbol of a last PSFCH reception occasion" as described in 16.5 and Tproc,1 with Tprep.

A UE does not expect that a PUCCH carrying SL HARQ-ACK reports overlaps with PUSCH with aperiodic or semi-persistent CSI reports.

A UE does not expect to be scheduled to transmit a PUCCH or a PUSCH with smaller priority index that would overlap in time with a PUCCH of larger priority index with HARQ-ACK information only in response to a PDSCH reception without a corresponding PDCCH. A UE does not expect to be scheduled to transmit a PUCCH of smaller priority index that would overlap in time with a PUSCH of larger priority index with SP-CSI report(s) without a corresponding PDCCH.

In the remaining of this clause, a UE multiplexes UCIs with same priority index in a PUCCH or a PUSCH before considering limitations for UE transmission as described in clause 11.1. A PUCCH or a PUSCH is assumed to have a same priority index as a priority index of UCIs a UE multiplexes in the PUCCH or the PUSCH.

In the remaining of this clause, the multiplexing or prioritization for overlapping channels are for overlapping channels with same priority index or for overlapping channels with a PUCCH carrying SL HARQ-ACK information.

In the remaining of this clause, if a UE is provided subslotLengthForPUCCH, a slot for an associated PUCCH resource of a PUCCH transmission with HARQ-ACK information includes a number of symbols indicated by subslotLengthForPUCCH, unless stated otherwise.

If a UE would transmit on a serving cell a PUSCH without UL-SCH that overlaps with a PUCCH transmission on a serving cell that includes positive SR information, the UE does not transmit the PUSCH.

If a UE would transmit CSI reports on overlapping physical channels, the UE applies the priority rules described in [6, TS 38.214] for the multiplexing of CSI reports.

If a UE has overlapping resources for PUCCH transmissions in a slot and at least one of the PUCCH transmissions is with repetitions over multiple slots, the UE first follows the procedures described in clause 9.2.6 for resolving the overlapping among the resources for the PUCCH transmissions.

If a UE

-	would multiplex UCI in a PUCCH transmission that overlaps with a PUSCH transmission, and

-	the PUSCH and PUCCH transmissions fulfill the conditions in clause 9.2.5 for UCI multiplexing,

the UE

-	multiplexes only HARQ-ACK information, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE multiplexes aperiodic or semi-persistent CSI reports in the PUSCH;

-	multiplexes only HARQ-ACK information and CSI reports, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE does not multiplex aperiodic or semi-persistent CSI reports in the PUSCH.

A UE does not expect to multiplex in a PUSCH transmission in one slot with SCS configuration μ1 UCI of same type that the UE would transmit in PUCCHs in different slots with SCS configuration μ2 if μ1<μ2.

A UE does not expect to multiplex in a PUSCH transmission or in a PUCCH transmission HARQ-ACK information that the UE would transmit in different PUCCHs.

A UE does not expect a PUCCH resource that results from multiplexing overlapped PUCCH resources, if applicable, to overlap with more than one PUSCHs if each of the more than one PUSCHs includes aperiodic CSI reports.

A UE does not expect to detect a DCI format scheduling a PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, and indicating a resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot if the UE previously detects a DCI format scheduling a PUSCH transmission in the slot and if the UE multiplexes HARQ-ACK information in the PUSCH transmission.

If a UE multiplexes aperiodic CSI in a PUSCH and the UE would multiplex UCI that includes HARQ-ACK information in a PUCCH that overlaps with the PUSCH and the timing conditions for overlapping PUCCHs and PUSCHs in clause 9.2.5 are fulfilled, the UE multiplexes only the HARQ-ACK information in the PUSCH and does not transmit the PUCCH.

If a UE transmits multiple PUSCHs in a slot on respective serving cells that include first PUSCHs that are scheduled by DCI formats and second PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH, and the UE would multiplex UCI in one of the multiple PUSCHs, and the multiple PUSCHs fulfil the conditions in clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in a PUSCH from the first PUSCHs.

If a UE transmits multiple PUSCHs in a slot on respective serving cells and the UE would multiplex UCI in one of the multiple PUSCHs and the UE does not multiplex aperiodic CSI in any of the multiple PUSCHs, the UE multiplexes the UCI in a PUSCH of the serving cell with the smallest ServCellIndex subject to the conditions in clause 9.2.5 for UCI multiplexing being fulfilled. If the UE transmits more than one PUSCHs in the slot on the serving cell with the smallest ServCellIndex that fulfil the conditions in clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in the earliest PUSCH that the UE transmits in the slot.

If a UE transmits a PUSCH over multiple slots and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots of the multiple slots, and the PUSCH transmission in the one or more slots fulfills the conditions in clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, the UE multiplexes the HARQ-ACK and/or CSI information in the PUSCH transmission in the one or more slots. The UE does not multiplex HARQ-ACK and/or CSI information in the PUSCH transmission in a slot from the multiple slots if the UE would not transmit a single-slot PUCCH with HARQ-ACK and/or CSI information in the slot in case the PUSCH transmission was absent.

If a UE transmits a PUSCH with repetition Type B and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots, the UE expects all actual repetitions of the PUSCH transmission [6, TS 38.214] that would overlap with the PUCCH transmission to fulfill the conditions in clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, and the UE multiplexes the HARQ-ACK and/or CSI information in the earliest actual PUSCH repetition of the PUSCH transmission that would overlap with the PUCCH transmission and includes more than one symbol. The UE does not expect that all actual repetitions that would overlap with the PUCCH transmission do not include more than one symbol.

If the PUSCH transmission over the multiple slots is scheduled by a DCI format that includes a DAI field, the value of the DAI field is applicable for multiplexing HARQ-ACK information in the PUSCH transmission in any slot from the multiple slots where the UE multiplexes HARQ-ACK information.

When a UE would multiplex HARQ-ACK information in a PUSCH transmission that is configured by a ConfiguredGrantConfig, and includes CG-UCI [5, TS 38.212], the UE multiplexes the HARQ-ACK information in the PUSCH transmission if the UE is provided cg-UCI-Multiplexing; otherwise, the UE does not transmit the PUSCH and multiplexes the HARQ-ACK information in a PUCCH transmission or in another PUSCH transmission.

## 9.1	HARQ-ACK codebook determination

If a UE is provided pdsch-HARQ-ACK-CodebookList, the UE can be indicated by pdsch-HARQ-ACK-CodebookList to generate one or two HARQ-ACK codebooks. If the UE is indicated to generate one HARQ-ACK codebook, the HARQ-ACK codebook is associated with a PUCCH of priority index 0. If a UE is provided pdsch-HARQ-ACK-CodebookList, the UE multiplexes in a same HARQ-ACK codebook only HARQ-ACK information associated with a same priority index. If the UE is indicated to generate two HARQ-ACK codebooks

-	a first HARQ-ACK codebook is associated with a PUCCH of priority index 0 and a second HARQ-ACK codebook is associated with a PUCCH of priority index 1

-	the UE is provided first and second for each of {PUCCH-Config, UCI-OnPUSCH, PDSCH-codeBlockGroupTransmission} by {PUCCH-ConfigurationList, UCI-OnPUSCH-ListDCI-0-1, PDSCH-CodeBlockGroupTransmissionList} or {PUCCH-ConfigurationList, UCI-OnPUSCH-ListDCI-0-2, PDSCH-CodeBlockGroupTransmissionList}, respectively, for use with the first and second HARQ-ACK codebooks, respectively

If a UE receives a PDSCH without receiving a corresponding PDCCH, or if the UE receives a PDCCH indicating a SPS PDSCH release, the UE generates one corresponding HARQ-ACK information bit. If the UE generates two HARQ-ACK codebooks, the UE is indicated by harq-CodebookID, per SPS PDSCH configuration, a HARQ-ACK codebook index for multiplexing the corresponding HARQ-ACK information bit.

If a UE is provided pdsch-HARQ-ACK-OneShotFeedback and the UE detects a DCI format in any PDCCH monitoring occasion that includes a One-shot HARQ-ACK request field with value 1

-	the UE includes the HARQ-ACK information in a Type-3 HARQ-ACK codebook, as described in clause 9.1.4

-	the UE does not expect that the PDSCH-to-HARQ_feedback timing indicator field of the DCI format provides an inapplicable value from dl-DataToUL-ACK-r16

In the remaining of this clause, reference is to one HARQ-ACK codebook and to DCI formats that schedule PDSCH reception, or have associated HARQ-ACK information without scheduling a PDSCH reception and are associated with the HARQ-ACK codebook.

If a UE is required to receive SPS PDSCHs in a slot according to Clause 5.1 of [6] and Clause 11.1 for SPS configurations that are indicated to be released by a DCI format, the UE is not expected to receive the DCI format in the slot if the end of the last symbol of the PDCCH reception is after the end of a last symbol of any of the SPS PDSCH receptions. For a SPS configuration subject to pdsch-AggregationFactor or pdsch-AggregationFactor-r16, the UE is not expected to receive the DCI format in a slot containing a SPS PDSCH transmission occasion other than the first transmission occasion required to be received by the UE for a TB.

If a UE is configured to receive SPS PDSCHs in a slot for SPS configurations that are indicated to be released by a DCI format, and if the UE receives the PDCCH providing the DCI format in the slot, and if HARQ-ACK information for the SPS PDSCH release and the SPS PDSCH receptions would be multiplexed in a same PUCCH, the UE does not expect to receive the SPS PDSCHs, does not generate HARQ-ACK information for the SPS PDSCH receptions, and generates a HARQ-ACK information bit for the SPS PDSCH release.

If a UE is configured to receive SPS PDSCH(s) in a slot for SPS configuration(s), the UE does not expect to receive a PDCCH providing a DCI format in the slot to indicate SPS PDSCH release of these SPS configuration(s), if HARQ-ACK information for the SPS PDSCH release and the SPS PDSCH reception(s) would map to different PUCCHs.

If a UE detects a DCI format 1_1 indicating

-	SCell dormancy without scheduling a PDSCH reception, as described in clause 10.3, and

-	is provided pdsch-HARQ-ACK-Codebook = dynamic or pdsch-HARQ-ACK-Codebook-r16

the UE generates a HARQ-ACK information bit as described in clause 9.1.3 for a DCI format 1_1 indicating SCell dormancy and the HARQ-ACK information bit value is ACK.

If a UE is not provided PDSCH-CodeBlockGroupTransmission, the UE generates one HARQ-ACK information bit per transport block.

For a HARQ-ACK information bit, a UE generates a positive acknowledgement (ACK) if the UE detects a DCI format that provides a SPS PDSCH release or correctly decodes a transport block, and generates a negative acknowledgement (NACK) if the UE does not correctly decode the transport block. A HARQ-ACK information bit value of 0 represents a NACK while a HARQ-ACK information bit value of 1 represents an ACK.

In the following, the CRC for a DCI format is scrambled with a C-RNTI, an MCS-C-RNTI, or a CS-RNTI.

## *** Unchanged text is omitted ***

### 9.1.2	Type-1 HARQ-ACK codebook determination

This clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static.

A UE does not expect to be configured with pdsch-HARQ-ACK-Codebook = semi-static for a codebook if a UE is provided subslotLength-ForPUCCH for the codebook.

A UE reports HARQ-ACK information for a corresponding PDSCH reception or SPS PDSCH release or TCI state update only in a HARQ-ACK codebook that the UE transmits in a slot indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format. The UE reports NACK value(s) for HARQ-ACK information bit(s) in a HARQ-ACK codebook that the UE transmits in a slot not indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format.

If a UE is not provided pdsch-HARQ-ACK-OneShotFeedback, the UE does not expect to receive a PDSCH scheduled by a DCI format that the UE detects in any PDCCH monitoring occasion and includes a PDSCH-to-HARQ_feedback timing indicator field providing an inapplicable value from dl-DataToUL-ACK-r16.

If the UE is provided pdsch-AggregationFactor-r16 in SPS-Config or pdsch-AggregationFactor in PDSCH-Config and no entry in pdsch-TimeDomainAllocationList and pdsch-TimeDomainAllocationListDCI-1-2 includes repetitionNumber in PDSCH-TimeDomainResourceAllocation-r16, NPDSCHrepeat,max is a maximum value of pdsch-AggregationFactor-r16 in SPS-Config or pdsch-AggregationFactor in PDSCH-Config; otherwise  NPDSCHrepeat,max=1. The UE reports HARQ-ACK information for a PDSCH reception

-	from DL slot nD-NPDSCHrepeat+1 to DL slot nD, if NPDSCHrepeat is provided by pdsch-AggregationFactor or pdsch-AggregationFactor-r16 [6, TS 38.214], or

-	from DL slot  nD-repetitionNumber+1 to DL slot nD, if the time domain resource assignment field in the DCI format scheduling the PDSCH reception indicates an entry containing repetitionNumber, or

-	in DL slot nD, otherwise

only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot n+k, where n is the last UL slot overlapping with DL slot nD and k is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format. If the UE reports HARQ-ACK information for the PDSCH reception in a slot other than slot n+k, the UE sets a value for each corresponding HARQ-ACK information bit to NACK.

If a UE reports HARQ-ACK information in a PUCCH only for

-	a SPS PDSCH release indicated by DCI format 1_0 with counter DAI field value of 1, or

-	a PDSCH reception scheduled by DCI format 1_0 with counter DAI field value of 1 on the PCell, or

-	SPS PDSCH reception(s)

within the MA,c occasions for candidate PDSCH receptions as determined in clause 9.1.2.1, the UE determines a HARQ-ACK codebook only for the SPS PDSCH release or only for the PDSCH reception or only for one SPS PDSCH reception according to corresponding MA,c occasion(s) on respective serving cell(s), where the value of counter DAI in DCI format 1_0 is according to Table 9.1.3-1 and HARQ-ACK information bits in response to more than one SPS PDSCH receptions that the UE is configured to receive are ordered according to the following pseudo-code; otherwise, the procedures in clause 9.1.2.1 and clause 9.1.2.2 for a HARQ-ACK codebook determination apply.

Set NcellsDL to the number of serving cells configured to the UE

Set NcSPS to the number of SPS PDSCH configuration configured to the UE for serving cell c

Set NcDL to the number of DL slots for SPS PDSCH reception on serving cell c with HARQ-ACK information multiplexed on the PUCCH

Set j=0 – HARQ-ACK information bit index

Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell

while c<NcellsDL

Set s=0 – SPS PDSCH configuration index: lower indexes correspond to lower RRC indexes of corresponding SPS configurations

while s<NcSPS

Set nD=0 – slot index

while nD<NcDL

if {

a UE is configured to receive SPS PDSCHs from slot nD-NPDSCHrepeat+1 to slot nD for SPS PDSCH configuration s on serving cell c, excluding SPS PDSCHs that are not required to be received in any slot among overlapping SPS PDSCHs, if any according to [6, TS 38.214], or based on a UE capability for a number of PDSCH receptions in a slot according to [6, TS 38.214], or due to overlapping with a set of symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated where NPDSCHrepeat is provided by pdsch-AggregationFactor-r16 in sps-Config or, if pdsch-AggregationFactor-r16 is not included in sps-Config, by pdsch-AggregationFactor in pdsch-config, and

HARQ-ACK information for the SPS PDSCH is associated with the PUCCH

}

ojACK = HARQ-ACK information bit for this SPS PDSCH reception

j=j+1;

end if

nD=nD+1;

end while

s=s+1;

end while

c=c+1;

end while

#### 9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a serving cell c, an active DL BWP, and an active UL BWP, as described in clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based:

a)	on a set of slot timing values K1 associated with the active UL BWP

a)	If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for either DCI format 1_1 or DCI format 1_2 on serving cell c, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8}

b)	If the UE is configured to monitor PDCCH for DCI format 1_1 and is not configured to monitor PDCCH for DCI format 1_2 for serving cell c, K1 is provided by dl-DataToUL-ACK

c)	If the UE is configured to monitor PDCCH for DCI format 1_2 and is not configured to monitor PDCCH for DCI format 1_1 for serving cell c, K1 is provided by dl-DataToUL-ACK-ForDCIFormat1_2

d)	If the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 for serving cell c, K1 is provided by the union of dl-DataToUL-ACK and dl-DataToUL-ACK-ForDCIFormat1_2

b)	on a set of row indexes R of a table that is associated with the active DL BWP and defining respective sets of slot offsets K0, start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by the union of row indexes of time domain resource allocation tables for DCI formats the UE is configured to monitor PDCCH for serving cell c

a)	if the UE is provided referenceOfSLIVDCI-1-2, for each row index with slot offset K0=0 and PDSCH mapping Type B in a set of row indexes of a table for DCI format 1_2 [6, TS 38.214], for any PDCCH monitoring occasion in any slot where the UE monitors PDCCH for DCI format 1_2 and with starting symbol S0>0, if S+S0+L≤14 for normal cyclic prefix and S+S0+L≤12  for extended cyclic prefix, add a new row index in the set of row indexes of the table by replacing the starting symbol S of the row index by S+S0

c)	on the ratio 2μDL-μUL between the downlink SCS configuration μDL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Downlink and BWP-Uplink for the active DL BWP and the active UL BWP, respectively

d)	if provided, on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated as described in clause 11.1

e)	if ca-SlotOffset is provided, on Nslot,offset,cDL and μoffset,DL,c provided by ca-SlotOffset for serving cell c, or on Nslot,offsetUL  and μoffset,UL provided by ca-SlotOffset for the primary cell, as described in [4, TS 38.211].

If a UE

-	is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and

-	is provided coresetPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and

-	is provided ackNackFeedbackMode = joint

where

-	a serving cell is placed in a first set S0 of NcellsDL,0 serving cells if the serving cell includes a first CORESET, and

-	a serving cell is placed in a second set S1 of NcellsDL,1 serving cells if the serving cell includes a second CORESET, and

-	serving cells are placed in a set according to an ascending order of a serving cell index

the UE generates a Type-1 HARQ-ACK codebook for the set S0 and the set S1 of serving cells separately by setting NcellsDL=NcellsDL,0 and NcellsDL=NcellsDL,1 in the following pseudo-code. The UE concatenates the HARQ-ACK codebook generated for the set S0 followed by the HARQ-ACK codebook generated for the set S1 to obtain a total number of  HARQ-ACK information bits.

For the set of slot timing values , the UE determines a set of  occasions for candidate PDSCH receptions or SPS PDSCH releases or TCI state update according to the following pseudo-code. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to a single SPS PDSCH release is same as for a corresponding SPS PDSCH reception. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to multiple SPS PDSCH releases by a single DCI format is same as for a corresponding SPS PDSCH reception with the lowest SPS configuration index among the multiple SPS PDSCH releases. If a UE provides HARQ-ACK information corresponding to detection of a DCI format that provides TCI state update without scheduling PDSCH reception, as described in [6, TS 38.214], a location in the Type-1 HARQ-ACK codebook for the HARQ-ACK information is same as when the DCI format schedules a PDSCH reception with CBGs or with transport blocks that are correctly decoded.

Set  - index of occasion for candidate PDSCH reception or SPS PDSCH release or TCI state update

Set

Set

Set  to the cardinality of set

Set k =0 – index of slot timing values , in descending order of the slot timing values, in set  for serving cell

If a UE is not provided ca-SlotOffset for any serving cell of PDSCH receptions and for the serving cell of corresponding PUCCH transmission with HARQ-ACK information

while

if

Set  – index of a DL slot within an UL slot

while

Set  to the set of rows

Set  to the cardinality of

Set  – index of row in set

if slot  starts at a same time as or after a slot for an active DL BWP change on serving cell  or an active UL BWP change on the PCell and slot  is before the slot for the active DL BWP change on serving cell  or the active UL BWP change on the PCell

nD=nD+1;

else

while

if the UE is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot nU-K1,k⋅2μDL-μUL+nD-NPDSCHrepeat,max+1 to slot nU-K1,k⋅2μDL-μUL+nD, at least one symbol of the PDSCH time resource derived by row  is configured as UL where  is the k-th slot timing value in set ,

;

else

;

end if

end while

if the UE does not indicate a capability to receive more than one unicast PDSCH per slot and ,

;

;

else

Set  to the cardinality of

Set  to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of

while

Set

while

if  for start OFDM symbol index  for row

; - index of occasion for candidate PDSCH reception, or SPS PDSCH release, or TCI state update associated with row

;

;

else

;

end if

end while

;

Set  to the smallest last OFDM symbol index among all rows of ;

end while

end if

nD=nD+1;

end if

end while

end if

;

end while

else

while

if  modnU-K1,k+(Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c)∙2μUL+1,max⁡(2μUL-μDL,1)=0

Set  – index of a DL slot within an UL slot

while

Set  to the set of rows

Set  to the cardinality of

Set  – index of row in set

if slot  starts at a same time as or after a slot for an active DL BWP change on serving cell  or an active UL BWP change on the PCell and slot  nU-K1,k∙2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD

is before the slot for the active DL BWP change on serving cell  or the active UL BWP change on the PCell

nD=nD+1;

else

while

if the UE is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot nU-K1,k∙2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD-NPDSCHrepeat,max+1 to slot nU-K1,k∙2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD, at least one symbol of the PDSCH time resource derived by row  is configured as UL where  is the k-th slot timing value in set ,

;

else

;

end if

end while

if the UE does not indicate a capability to receive more than one unicast PDSCH per slot and ,

;

;

else

Set  to the cardinality of

Set  to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of

while

Set

while

if  for start OFDM symbol index  for row

; - index of occasion for candidate PDSCH reception or SPS PDSCH release or TCI state update associated with row

;

;

else

;

end if

end while

;

Set  to the smallest last OFDM symbol index among all rows of ;

end while

end if

nD=nD+1;

end if

end while

end if

;

end while

end if

If the UE indicates a capability to receive more than one PDSCH per slot, for occasions of candidate PDSCH receptions corresponding to rows of R associated with a same value of , where , the UE does not expect to receive more than one PDSCH in a same DL slot associated with a same coresetPoolIndex value if provided, or if coresetPoolIndex is not provided.

If a UE receives a SPS PDSCH, or a SPS PDSCH release, or TCI state update, or a PDSCH that is scheduled by a DCI format that does not support CBG-based PDSCH receptions and if

-	the UE is configured with one serving cell, and

-	, and

-	PDSCH-CodeBlockGroupTransmission is provided to the UE

the UE generates HARQ-ACK information only for the transport block in the PDSCH, or only for the SPS PDSCH release, or only for the TCI state update.

If a UE receives a SPS PDSCH, or a SPS PDSCH release, or TCI state update, or a PDSCH that is scheduled by a DCI format that does not support CBG-based PDSCH receptions and if

-	the UE is configured with more than one serving cells, or

-	, and

-	PDSCH-CodeBlockGroupTransmission is provided to the UE

the UE repeats  times the HARQ-ACK information for the transport block in the PDSCH or for the SPS PDSCH release or for the TCI state update.

A UE does not expect to detect a DCI format switching a DL BWP within  symbols prior to a first symbol of a PUCCH transmission where the UE multiplexes HARQ-ACK information, where  is defined in clause 9.2.3.

If a UE is provided dl-DataToUL-ACK or dl-DataToUL-ACK-ForDCIFormat1_2, the UE does not expect to be indicated by DCI format 1_0 a slot timing value for transmission of HARQ-ACK information that does not belong to the intersection of the set of slot timing values {1, 2, 3, 4, 5, 6, 7, 8} and the set of slot timing values provided by K1 for the active DL BWP of a corresponding serving cell.

If maxNrofCodeWordsScheduledByDCI indicates reception of two transport blocks, when the UE receives a PDSCH with one transport block or a SPS PDSCH release or a TCI state update, the HARQ-ACK information is associated with the first transport block and the UE generates a NACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is not provided and generates HARQ-ACK information with value of ACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is provided.

A UE determines o0ACK, o1ACK,...,oOACK-1ACK HARQ-ACK information bits, for a total number of  HARQ-ACK information bits, of a HARQ-ACK codebook for transmission in a PUCCH according to the following pseudo-code. In the following pseudo-code, if the UE does not receive a transport block or a CBG, due to the UE not detecting a corresponding DCI format, the UE generates a NACK value for the transport block or the CBG. The cardinality of the set  defines a total number  of occasions for PDSCH reception or SPS PDSCH release or TCI state update for serving cell  corresponding to the HARQ-ACK information bits.

Set  – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cells including, when applicable, cells in the set S0 and the set S1

Set - HARQ-ACK information bit index

Set  to the number of serving cells configured by higher layers for the UE

while

Set  – index of occasion for candidate PDSCH reception or SPS PDSCH release or TCI state update

while

if harq-ACK-SpatialBundlingPUCCH is not provided, PDSCH-CodeBlockGroupTransmission is not provided, and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell ,

= HARQ-ACK information bit corresponding to a first transport block of this cell;

;

= HARQ-ACK information bit corresponding to a second transport block of this cell;

;

elseif harq-ACK-SpatialBundlingPUCCH is provided, and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell ,

= binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks of this cell - if the UE receives one transport block, the UE assumes ACK for the second transport block;

;

elseif PDSCH-CodeBlockGroupTransmission is provided, and  CBGs are indicated by maxCodeBlockGroupsPerTransportBlock for serving cell ,

Set - CBG index

while

= HARQ-ACK information bit corresponding to CBG  of the first transport block;

if the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell

= HARQ-ACK information bit corresponding to CBG  of the second transport block;

end if

;

end while

, where  is the value of maxNrofCodeWordsScheduledByDCI for the active DL BWP of serving cell ;

else

= HARQ-ACK information bit of serving cell ;

;

end if

;

end while

;

end while

If , the UE determines a number of HARQ-ACK information bits  for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as  where

-	 is the number of transport blocks the UE receives in PDSCH reception occasion  for serving cell  if harq-ACK-SpatialBundlingPUCCH and PDSCH-CodeBlockGroupTransmission are not provided, or the number of transport blocks the UE receives in PDSCH reception occasion  for serving cell  if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that does not support CBG-based PDSCH receptions, or the number of PDSCH receptions if harq-ACK-SpatialBundlingPUCCH is provided or SPS PDSCH release or TCI state update in PDSCH reception occasion  for serving cell  and the UE reports corresponding HARQ-ACK information in the PUCCH.

-	 is the number of CBGs the UE receives in a PDSCH reception occasion  for serving cell  if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that supports CBG-based PDSCH receptions and the UE reports corresponding HARQ-ACK information in the PUCCH.

#### 9.1.2.2	Type-1 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format that does not include a DAI field, then

-	if the UE has not received any PDSCH or SPS PDSCH release or TCI state update that the UE multiplexes corresponding HARQ-ACK information in the PUSCH, based on a value of a respective PDSCH-to-HARQ_feedback timing indicator field in a DCI format scheduling the PDSCH reception or the SPS PDSCH release or the TCI state update or on the value of dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in DCI format 1_1 or on the value of dl-DataToUL-ACK-ForDCI-Format1-2 if the PDSCH-to-HARQ_feedback timing indicator field is not present in DCI format 1_2, in any of the Mc occasions for candidate PDSCH receptions by a DCI format or SPS PDSCH on any serving cell c, as described in clause 9.1.2.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission;

-	else the UE generates the HARQ-ACK codebook as described in clause 9.1.2.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH, unless the UE receives only a SPS PDSCH release, or only SPS PDSCH reception, or only TCI state update, or only a PDSCH that is scheduled by DCI format 1_0 with a counter DAI field value of 1 on the PCell in the Mc occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the PDSCH reception or only for the TCI state update as described in clause 9.1.2.

A UE sets to NACK value in the HARQ-ACK codebook any HARQ-ACK information corresponding to PDSCH reception or SPS PDSCH release or TCI state update that the UE detects in a PDCCH monitoring occasion that starts after a PDCCH monitoring occasion where the UE detects a DCI format scheduling the PUSCH transmission.

A UE does not expect to detect a DCI format switching a DL BWP within N2 symbols prior to a first symbol of a PUSCH transmission where the UE multiplexes HARQ-ACK information, where N2 is defined in [6, TS 38.214].

If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by DCI format that includes a DAI field, the UE generates the HARQ-ACK codebook as described in clause 9.1.2.1 when a value of the DAI field is VT-DAIUL=1 except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH. The UE does not generate a HARQ-ACK codebook for multiplexing in the PUSCH transmission when VT-DAIUL=0 unless the UE receives only a SPS PDSCH release, or only SPS PDSCH(s), or only a TCI state update, or only a PDSCH that is scheduled by DCI format 1_0 with a counter DAI field value of 1 on the PCell in the Mc occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the TCI state update or only for the PDSCH reception as described in clause 9.1.2. VT-DAIUL=0 if the PUSCH is scheduled by a DCI format that includes a DAI field and the DAI field is set to '0'; otherwise, VT-DAIUL=1.

### 9.1.3	Type-2 HARQ-ACK codebook determination

This clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic or with pdsch-HARQ-ACK-Codebook-r16. Unless stated otherwise, a PDSCH-to-HARQ_feedback timing indicator field provides an applicable value.

A UE does not expect to multiplex in a Type-2 HARQ-ACK codebook HARQ-ACK information that is in response to a detection of a DCI format that does not include a counter DAI field.

If a UE receives a first DCI format that the UE detects in a first PDCCH monitoring occasion and includes a PDSCH-to-HARQ_feedback timing indicator field providing an inapplicable value from dl-DataToUL-ACK-r16,

-	if the UE detects a second DCI format, the UE multiplexes the corresponding HARQ-ACK information in a PUCCH or PUSCH transmission in a slot that is indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in the second DCI format, where

-	if the UE is not provided pdsch-HARQ-ACK-Codebook-r16, the UE detects the second DCI format in any PDCCH monitoring occasion after the first one, and where the slot indicated by the value of the PDSCH-to-HARQ_feedback timing indicator field in the second DCI format is no later than a slot for HARQ-ACK information in response to a SPS PDSCH reception, if any, received after the PDSCH scheduled by the first DCI format.

-	if the UE is provided pdsch-HARQ-ACK-Codebook-r16, the UE detects the second DCI format in any PDCCH monitoring occasion after the first one, and the second DCI format indicates a HARQ-ACK information report for a same PDSCH group index as indicated by the first DCI format as described in clause 9.1.3.3, and where the slot indicated by the value of the PDSCH-to-HARQ_feedback timing indicator field in the second DCI format is no later than a slot for HARQ-ACK information in response to a SPS PDSCH reception, if any, received after the PDSCH scheduled by the first DCI format.

-	if the UE is provided pdsch-HARQ-ACK-Codebook-r16, the UE receives the second DCI format later than the slot for HARQ-ACK information in response to a SPS PDSCH reception received after the PDSCH scheduled by the first DCI format, and the second DCI format indicates a HARQ-ACK information report for a same PDSCH group index as indicated by the first DCI format as described in clause 9.1.3.3.

-	if the UE is provided pdsch-HARQ-ACK-OneShotFeedback, the first DCI format does not indicate SPS PDSCH release or SCell dormancy or TCI state update, the UE detects the second DCI format in any PDCCH monitoring occasion after the first one, and the second DCI format includes a One-shot HARQ-ACK request field with value 1, the UE includes the HARQ-ACK information in a Type-3 HARQ-ACK codebook, as described in clause 9.1.4, and where the slot indicated by the value of the PDSCH-to-HARQ_feedback timing indicator field in the second DCI format is no later than a slot for HARQ-ACK information in response to a SPS PDSCH reception, if any, received after the PDSCH scheduled by the first DCI format.

-	if the UE is provided pdsch-HARQ-ACK-OneShotFeedback-r16, the first DCI format does not indicate SPS PDSCH release or SCell dormancy or TCI state update, and the UE receives the second DCI format later than the slot for HARQ-ACK information in response to a SPS PDSCH reception received after the PDSCH scheduled by the first DCI format, and the second DCI format includes a One-shot HARQ-ACK request field with value 1, the UE includes the HARQ-ACK information in a Type-3 HARQ-ACK codebook, as described in clause 9.1.4.

-	otherwise, the UE does not multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH transmission.

#### 9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

A UE determines monitoring occasions for PDCCH with DCI format scheduling PDSCH receptions or having associated HARQ-ACK information without scheduling PDSCH receptions on an active DL BWP of a serving cell c, as described in clause 10.1, and for which the UE transmits HARQ-ACK information in a same PUCCH in slot n based on

-	PDSCH-to-HARQ_feedback timing indicator field values for PUCCH transmission with HARQ-ACK information in slot n in response to PDSCH receptions or in response to DCI formats having associated HARQ-ACK information without scheduling PDSCH receptions

-	slot offsets K0 [6, TS 38.214] provided by time domain resource assignment field in a DCI format scheduling PDSCH receptions and by pdsch-AggregationFactor, or pdsch-AggregationFactor-r16, or repetitionNumber, when provided.

The set of PDCCH monitoring occasions for DCI formats scheduling PDSCH receptions or having associated HARQ-ACK information without scheduling PDSCH receptions is defined as the union of PDCCH monitoring occasions across active DL BWPs of configured serving cells. PDCCH monitoring occasions are indexed in an ascending order of their start times. The cardinality of the set of PDCCH monitoring occasions defines a total number M of PDCCH monitoring occasions.

A value of the counter downlink assignment indicator (DAI) field in DCI formats denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) or HARQ-ACK information associated with the DCI formats is present up to the current serving cell and current PDCCH monitoring occasion,

-	first, if the UE indicates by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion, in increasing order of the PDSCH reception starting time for the same {serving cell, PDCCH monitoring occasion} pair,

-	second in ascending order of serving cell index, and

-	third in ascending order of PDCCH monitoring occasion index m, where 0≤m<M.

If, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the value of the counter DAI is in the order of the first CORESETs and then the second CORESETs for a same serving cell index and a same PDCCH monitoring occasion index.

The value of the total DAI, when present [5, TS 38.212], in a DCI format denotes the total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) or HARQ-ACK information associated with DCI formats is present, up to the current PDCCH monitoring occasion m and is updated from PDCCH monitoring occasion to PDCCH monitoring occasion. If, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the total DAI value counts the {serving cell, PDCCH monitoring occasion}-pair(s) for both the first CORESETs and the second CORESETs.

Denote by NC-DAIDL the number of bits for the counter DAI and set TD=2NC-DAIDL. Denote by VC-DAI,c,mDL the value of the counter DAI in a DCI format scheduling PDSCH reception or having associated HARQ-ACK information without scheduling PDSCH reception on serving cell c in PDCCH monitoring occasion m according to Table 9.1.3-1 or Table 9.1.3-1A. Denote by VT-DAI,mDL the value of the total DAI in a DCI format in PDCCH monitoring occasion m according to Table 9.1.3-1. The UE assumes a same value of total DAI in all DCI formats that include a total DAI field in PDCCH monitoring occasion m. A UE does not expect to multiplex, in a same Type-2 HARQ-ACK codebook, HARQ-ACK information that is in response to detection of DCI formats with different number of bits for the counter DAI field.

If the UE transmits HARQ-ACK information in a PUCCH in slot n and for any PUCCH format, the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK  , for a total number of OACK HARQ-ACK information bits, according to the following pseudo-code:

Set m=0 – PDCCH, with DCI format scheduling PDSCH reception or having associated HARQ-ACK information without scheduling PDSCH reception, monitoring occasion index: lower index corresponds to earlier PDCCH monitoring occasion

Set j=0

Set Vtemp=0

Set Vtemp2=0

Set Vs=∅

Set NcellsDL to the number of serving cells configured by higher layers for the UE

-	if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs

-	if the UE indicates type2-HARQ-ACK-Codebook, a serving cell is counted NPDSCHMO times where NPDSCHMO is the number of PDSCH receptions that can be scheduled for the serving cell by DCI formats in PDCCH receptions at a same PDCCH monitoring occasion based on the reported value of type2-HARQ-ACK-Codebook

Set M to the number of PDCCH monitoring occasion(s)

while m<M

Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell

while c<NcellsDL

if PDCCH monitoring occasion m is before an active DL BWP change on serving cell c or an active UL BWP change on the PCell and an active DL BWP change is not triggered in PDCCH monitoring occasion m

c=c+1;

else

if there is a PDSCH on serving cell c associated with PDCCH in PDCCH monitoring occasion m, or there is a PDCCH providing a DCI format with associated HARQ-ACK information without scheduling a PDSCH reception on serving cell c

if VC-DAI,c,mDL≤Vtemp

j=j+1

end if

Vtemp=VC-DAI,c,mDL

if VT-DAI,mDL=∅

Vtemp,2=VC-DAI,c,mDL

else

Vtemp,2=VT-DAI,mDL

end if

if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of at least one serving cell,

o2⋅TD⋅j+2VC-DAI,c,mDL-1ACK = HARQ-ACK information bit corresponding to the first transport block of this cell

o2⋅TD⋅j+2VC-DAI,c,mDL-1+1ACK = HARQ-ACK information bit corresponding to the second transport block of this cell

Vs=Vs∪2⋅TD⋅j+2VC-DAI,c,mDL-1, 2⋅TD⋅j+2VC-DAI,c,mDL-1+1

elseif harq-ACK-SpatialBundlingPUCCH is provided to the UE and m is a monitoring occasion for PDCCH with a DCI format that supports PDSCH reception with two transport blocks and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks in at least one configured DL BWP of a serving cell,

oTD⋅j+VC-DAI,c,mDL-1ACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell

Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1

else

oTD⋅j+VC-DAI,c,mDL-1ACK = HARQ-ACK information bit of this cell

Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1

end if

end if

c=c+1

end if

end while

m=m+1

end while

Vtemp=j mod4TD×4TD+Vtemp

if UE does not set Vtemp2=VT-DAIUL and TD=2

Vtemp2=Vtemp

end if

j=j×TD4

if Vtemp2<Vtemp

j=j+1

end if

if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell,

OACK=2⋅4⋅j+Vtemp2

else

OACK=4⋅j+Vtemp2

end if

oiACK=NACK for any i∈0,1,⋯,OACK-1\Vs

If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for one activated SPS PDSCH reception in the PUCCH in slot n, the UE generates one HARQ-ACK information bit associated with the SPS PDSCH reception and appends it to the OACK HARQ-ACK information bits.

If a UE is configured to receive SPS PDSCH and the UE multiplexes HARQ-ACK information for multiple activated SPS PDSCH receptions in the PUCCH in slot n, the UE generates the HARQ-ACK information as described in clause 9.1.2 and appends it to the OACK HARQ-ACK information bits.

The UE generates HARQ-ACK information with ACK value in response to a detection of a DCI format that does not trigger a Type-3 HARQ-ACK codebook report and has associated HARQ-ACK information without scheduling a PDSCH reception.

For a PDCCH monitoring occasion with DCI format scheduling PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception in the active DL BWP of a serving cell, when a UE receives a PDSCH with one transport block or detects a DCI format having the associated HARQ-ACK information report and the value of maxNrofCodeWordsScheduledByDCI is 2, the HARQ-ACK information is associated with the first transport block and the UE generates a NACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is not provided and generates HARQ-ACK information with value of ACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is provided.

If a UE is not provided PDSCH-CodeBlockGroupTransmission for each of the NcellsDL serving cells, or for PDSCH receptions scheduled by a DCI format that does not support CBG-based PDSCH receptions, or for a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception, and if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as

nHARQ-ACK=nHARQ-ACK,TB=VDAI,mlastDL-c=0NcellsDL-1UDAI,cmodTDNTB,maxDL+c=0NcellsDL-1m=0M-1Nm,creceived+NSPS,c

where

-	if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling PDSCH reception having associated HARQ-ACK information without scheduling PDSCH reception, for any serving cell c that the UE detects within the M PDCCH monitoring occasions.

-	if NcellsDL>1

-	if the UE does not detect any DCI format that includes a total DAI field in a last PDCCH monitoring occasion within the M PDCCH monitoring occasions where the UE detects at least one DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception for any serving cell c, VDAI,mlastDL is the value of the counter DAI in a last DCI format the UE detects in the last PDCCH monitoring occasion

-	if the UE detects at least one DCI format that includes a total DAI field in a last PDCCH monitoring occasion within the M PDCCH monitoring occasions where the UE detects at least one DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception for any serving cell c, VDAI,mlastDL is the value of the total DAI in the at least one DCI format that includes a total DAI field

-	VDAI,mlastDL=0 if the UE does not detect any DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception for any serving cell c in any of the M PDCCH monitoring occasions.

-	UDAI,c is the total number of a DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception that the UE detects within the M PDCCH monitoring occasions for serving cell c. UDAI,c=0 if the UE does not detect any DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception for serving cell c in any of the M PDCCH monitoring occasions.

-	NTB,maxDL=2 if the value of maxNrofCodeWordsScheduledByDCI is 2 for any serving cell c and harq-ACK-SpatialBundlingPUCCH is not provided; otherwise, NTB,maxDL=1.

-	Nm,creceived is the number of transport blocks the UE receives in a PDSCH scheduled by a DCI format that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided, or the number of PDSCH scheduled by a DCI format that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided, or the number of DCI formats that the UE detects and have associated HARQ-ACK information without scheduling PDSCH receptions mcin PDCCH monitoring occasion m for serving cell c.

-	NSPS,c is the number of SPS PDSCH receptions by the UE on serving cell c for which the UE transmits corresponding HARQ-ACK information in the same PUCCH as for HARQ-ACK information corresponding to PDSCH receptions within the M PDCCH monitoring occasions.

If a UE

-	is provided PDSCH-CodeBlockGroupTransmission for NcellsDL,CBG serving cells; and

-	is not provided PDSCH-CodeBlockGroupTransmission, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,CBG=NcellsDL

the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications

-	NcellsDL is used for the determination of a first HARQ-ACK sub-codebook for

-	SPS PDSCH release,

-	SPS PDSCH reception,

-	DCI format 1_1 indicating SCell dormancy,

-	TCI state update, and

-	TB-based PDSCH receptions on the NcellsDL,CBG serving cells and on the NcellsDL,TB serving cells,

-	NcellsDL is replaced by NcellsDL,CBG for the determination of a second HARQ-ACK sub-codebook corresponding to the NcellsDL,CBG serving cells for CBG-based PDSCH receptions, and

-	if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted as two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs, and

-	instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,CBG serving cells, the UE generates NHARQ-ACK,maxCBG/TB,max HARQ-ACK information bits, where NHARQ-ACK,maxCBG/TB,max is the maximum value of NTB,cDL⋅NHARQ-ACK,cCBG/TB,max across all NcellsDL,CBG serving cells and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c. If for a serving cell c it is NTB,cDL⋅NHARQ-ACK,cCBG/TB,max<NHARQ-ACK,maxCBG/TB,max, the UE generates NACK for the last NHARQ-ACK,maxCBG/TB,max-NTB,cDL⋅NHARQ-ACK,cCBG/TB,max HARQ-ACK information bits for serving cell c

-	the pseudo-code operation when harq-ACK-SpatialBundlingPUCCH is provided is not applicable

-	The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook

-	The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook

If OACK+OSR+OCSI≤11, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,CBG for obtaining a PUCCH transmission power, as described in clause 7.2.1, with

nHARQ-ACK,CBG=VDAI,mlastDL-c=0NcellsDL,CBG-1UDAI,cCBGmodTDNHARQ-ACK,maxCBG/TB,max+c=0NcellsDL-1m=0M-1Nm,creceived,CBG

where

-	if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling CBG-based PDSCH reception for any serving cell c that the UE detects within the M PDCCH monitoring occasions

-	if NcellsDL>1,, VDAI,mlastDL is the value of the total DAI in the last DCI format scheduling CBG-based PDSCH reception for any serving cell c that the UE detects within the M PDCCH monitoring occasions

-	VDAI,mlastDL=0, if the UE does not detect any DCI format scheduling CBG-based PDSCH reception for any serving cell c in any of the M PDCCH monitoring occasions

-	UDAI,cCBG is the total number of DCI formats scheduling CBG-based PDSCH receptions that the UE detects within the M PDCCH monitoring occasions for serving cell c. UDAI,cCBG=0 if the UE does not detect any DCI format scheduling CBG-based PDSCH reception for serving cell c in any of the M PDCCH monitoring occasions

-	Nm,creceived,CBG is the number of CBGs the UE receives in a PDSCH scheduled by a DCI format that supports CBG-based PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c and the UE reports corresponding HARQ-ACK information in the PUCCH

Table 9.1.3-1: Value of counter DAI for NC-DAIDL=2 and of total DAI

| DAI MSB, LSB | VC-DAIDL or VT-DAIDL | Number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH indicating SPS PDSCH release or providing TCI state update or DCI format 1_1 indicating SCell dormancy is present, denoted as Y and Y≥1 |
| --- | --- | --- |
| 0,0 | 1 | Y-1modTD+1=1 |
| 0,1 | 2 | Y-1modTD+1=2 |
| 1,0 | 3 | Y-1modTD+1=3 |
| 1,1 | 4 | Y-1modTD+1=4 |

Table 9.1.3-1A: Value of counter DAI for NC-DAIDL=1

| DAI | VC-DAIDL | Number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH indicating SPS PDSCH release or providing TCI state update is present, denoted as Y and Y≥1 |
| --- | --- | --- |
| 0 | 1 | Y-1modTD+1=1 |
| 1 | 2 | Y-1modTD+1=2 |

#### 9.1.3.2	Type-2 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format that does not include a DAI field, then

-	if the UE has not received any PDCCH within the monitoring occasions for DCI formats scheduling PDSCH receptions, or providing a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception, on any serving cell c and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in clause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission;

-	else, the UE generates the HARQ-ACK codebook as described in clause 9.1.3.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH.

If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by a DCI format that includes a DAI field, the UE generates the HARQ-ACK codebook as described in clause 9.1.3.1, with the following modifications:

-	For the pseudo-code for the HARQ-ACK codebook generation in clause 9.1.3.1, after the completion of the c and m loops, the UE sets Vtemp2=VT-DAIUL where VT-DAIUL is the value of the DAI field according to Table 9.1.3-2

-	For the case of first and second HARQ-ACK sub-codebooks, the DCI format includes a first DAI field corresponding to the first HARQ-ACK sub-codebook and a second DAI field corresponding to the second HARQ-ACK sub-codebook

-	harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH.

If a UE is not provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format that includes a DAI field with value VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions for PDCCH with DCI format scheduling PDSCH receptions or having associated HARQ-ACK information without scheduling PDSCH receptions on any serving cell c and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in clause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission.

If a UE is provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format that includes a DAI field with first value VT-DAIUL=4 or with second value VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions for PDCCH with DCI format scheduling PDSCH reception or having associated HARQ-ACK information without scheduling PDSCH reception, on any serving cell c and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in clause 9.1.3.1, the UE does not multiplex HARQ-ACK information for the first sub-codebook or for the second sub-codebook, respectively, in the PUSCH transmission.

Table 9.1.3-2: Value of DAI

| DAI MSB, LSB | VT-DAIUL | Number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH indicating SPS PDSCH release or providing TCI state update or DCI format 1_1 indicating SCell dormancy is present, denoted as X and X≥1 |
| --- | --- | --- |
| 0,0 | 1 | X-1mod4+1=1 |
| 0,1 | 2 | X-1mod4+1=2 |
| 1,0 | 3 | X-1mod4+1=3 |
| 1,1 | 4 | X-1mod4+1=4 |

## *** Unchanged text is omitted ***

## 9.2	UCI reporting in physical uplink control channel

UCI types reported in a PUCCH include HARQ-ACK information, SR, LRR, and CSI. UCI bits include HARQ-ACK information bits, if any, SR information bits, if any, LRR information bit, if any, and CSI bits, if any. The HARQ-ACK information bits correspond to a HARQ-ACK codebook as described in clause 9.1. For the remaining of this clause, any reference to SR is applicable for SR and/or for LRR.

A UE may transmit one or two PUCCHs on a serving cell in different symbols within a slot. When the UE transmits two PUCCHs in a slot and the UE is not provided ackNackFeedbackMode = separate, at least one of the two PUCCHs uses PUCCH format 0 or PUCCH format 2.

If a UE is provided ackNackFeedbackMode = separate, the UE may transmit up to two PUCCHs with HARQ-ACK information in different symbols within a slot.

In clauses 9.2.3, 9.2.5.1 and 9.2.5.2, a UE assumes 11 CRC bits if a number of respective UCI bits is larger than or equal to 360; otherwise, the UE determines a number of CRC bits based on the number of respective UCI bits as described in [5, TS 38.212].

### 9.2.1	PUCCH Resource Sets

If a UE does not have dedicated PUCCH resource configuration, provided by PUCCH-ResourceSet in PUCCH-Config, a PUCCH resource set is provided by pucch-ResourceCommon through an index to a row of Table 9.2.1-1 for transmission of HARQ-ACK information on PUCCH in an initial UL BWP of NBWPsize PRBs.

The PUCCH resource set includes sixteen resources, each corresponding to a PUCCH format, a first symbol, a duration, a PRB offset RBBWPoffset, and a cyclic shift index set for a PUCCH transmission.

The UE transmits a PUCCH using frequency hopping if not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon; otherwise, the UE transmits a PUCCH without frequency hopping.

An orthogonal cover code with index 0 is used for a PUCCH resource with PUCCH format 1 in Table 9.2.1-1 except when index 3, 7, or 11 is indicated by pucch-ResourceCommon and useInterlacePUCCH-PUSCH in BWP-UplinkCommon is provided.

The UE transmits the PUCCH using the same spatial domain transmission filter as for a PUSCH transmission scheduled by a RAR UL grant as described in clause 8.3.

If a UE is not provided any of pdsch-HARQ-ACK-Codebook, pdsch-HARQ-ACK-Codebook-r16, or pdsch-HARQ-ACK-OneShotFeedback, the UE generates at most one HARQ-ACK information bit.

If the UE provides HARQ-ACK information in a PUCCH transmission in response to detecting a DCI format scheduling a PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, the UE determines a PUCCH resource with index rPUCCH, 0≤rPUCCH≤15, as rPUCCH=2⋅nCCE,0NCCE+2⋅∆PRI, where NCCE is a number of CCEs in a CORESET of a PDCCH reception with the DCI format, as described in clause 10.1, nCCE,0 is the index of a first CCE for the PDCCH reception, and ∆PRI is a value of the PUCCH resource indicator field in the DCI format. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1, the CORESET is associated with the search space set having the smaller index.

If rPUCCH8=0 and a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon

-	the UE determines the PRB index of the PUCCH transmission in the first hop as RBBWPoffset+rPUCCHNCS and the PRB index of the PUCCH transmission in the second hop as NBWPsize-1-RBBWPoffset-rPUCCHNCS, where NCS is the total number of initial cyclic shift indexes in the set of initial cyclic shift indexes

-	the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as rPUCCHmodNCS

If rPUCCH8=1 and a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon

-	the UE determines the PRB index of the PUCCH transmission in the first hop as NBWPsize-1-RBBWPoffset-rPUCCH-8NCS and the PRB index of the PUCCH transmission in the second hop as RBBWPoffset+rPUCCH-8NCS

-	the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as

If a UE is provided a PUCCH resource by pucch-ResourceCommon and is provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon

-	the UE determines for the PUCCH resource an interlace index m as m=m0+rPUCCHNCSmodM where M is a number of interlaces [4, TS 38.211] and m0=RBBWPoffset is an interlace index offset and RBBWPoffset is as given in Table 9.2.1-1

-	the UE determines an initial cyclic shift index in a set of initial cyclic shift indexes as rPUCCHmodNCS, where NCS is the total number of initial cyclic shifts indexes in the set of initial cyclic shift indexes in Table 9.2.1-1

-	if pucch-ResourceCommon indicates

-	index 0: the first symbol is 9 for a PUCCH resource with PUCCH format 0 if rPUCCH≥10

-	index 1 or 2: the first symbol is 9 for a PUCCH resource with PUCCH format 0 if rPUCCH=15

-	index 3, 7, or 11: an orthogonal cover code with index 1 is used for a PUCCH resource with PUCCH format 1 if rPUCCH≥10; otherwise, an orthogonal cover code with index 0 is used for a PUCCH resource with PUCCH format 1

-	the UE does not expect pucch-ResourceCommon to indicate index 15

Table 9.2.1-1: PUCCH resource sets before dedicated PUCCH resource configuration

| Index | PUCCH format | First symbol | Number of symbols | PRB offset | Set of initial CS indexes |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 12 | 2 | 0 | {0, 3} |
| 1 | 0 | 12 | 2 | 0 | {0, 4, 8} |
| 2 | 0 | 12 | 2 | 3 | {0, 4, 8} |
| 3 | 1 | 10 | 4 | 0 | {0, 6} |
| 4 | 1 | 10 | 4 | 0 | {0, 3, 6, 9} |
| 5 | 1 | 10 | 4 | 2 | {0, 3, 6, 9} |
| 6 | 1 | 10 | 4 | 4 | {0, 3, 6, 9} |
| 7 | 1 | 4 | 10 | 0 | {0, 6} |
| 8 | 1 | 4 | 10 | 0 | {0, 3, 6, 9} |
| 9 | 1 | 4 | 10 | 2 | {0, 3, 6, 9} |
| 10 | 1 | 4 | 10 | 4 | {0, 3, 6, 9} |
| 11 | 1 | 0 | 14 | 0 | {0, 6} |
| 12 | 1 | 0 | 14 | 0 | {0, 3, 6, 9} |
| 13 | 1 | 0 | 14 | 2 | {0, 3, 6, 9} |
| 14 | 1 | 0 | 14 | 4 | {0, 3, 6, 9} |
| 15 | 1 | 0 | 14 |  | {0, 3, 6, 9} |

If a UE has dedicated PUCCH resource configuration, the UE is provided by higher layers with one or more PUCCH resources.

A PUCCH resource includes the following parameters:

-	a PUCCH resource index provided by pucch-ResourceId

-	an index of the first PRB prior to frequency hopping or for no frequency hopping by startingPRB, if a UE is not provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	an index of the first PRB after frequency hopping by secondHopPRB, if a UE is not provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	an indication for intra-slot frequency hopping by intraSlotFrequencyHopping, if a UE is not provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	an index of a first interlace by interlace0, if a UE is provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	if provided, an index of a second interlace by interlace1, if a UE is provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	an index of an RB set by rb-SetIndex, if a UE is provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	a configuration for a PUCCH format provided by format

The UE expects that useInterlacePUCCH-PUSCH in BWP-UplinkCommon and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated are provided either in all UL BWPs or in none of the UL BWPs for a serving cell.

If a UE is provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated, the UE determines available RBs for PUCCH transmissions within the active UL BWP as the intersection of RBs corresponding to an interlace index provided by interlace0 and, if provided, interlace1, and RBs of an RB set provided by rb-SetIndex. The intersection results in Minterlace,0PUCCH RBs in the first interlace and the UE expects that Minterlace,0PUCCH is either 10 or 11. If interlace1 is provided, the intersection results in Minterlace,1PUCCH RBs in the second interlace and the UE expects that Minterlace,1PUCCH is either 10 or 11.

If the format indicates PUCCH-format0, the PUCCH format configured for a PUCCH resource is PUCCH format 0, where the PUCCH resource also includes an index for an initial cyclic shift provided by initialCyclicShift, a number of symbols for a PUCCH transmission provided by nrofSymbols, a first symbol for the PUCCH transmission provided by startingSymbolIndex.

If the format indicates PUCCH-format1, the PUCCH format configured for a PUCCH resource is PUCCH format 1, where the PUCCH resource also includes an index for an initial cyclic shift provided by initialCyclicShift, a number of symbols for a PUCCH transmission provided by nrofSymbols, a first symbol for the PUCCH transmission provided by startingSymbolIndex, and an index for an orthogonal cover code by timeDomainOCC.

If the format indicates PUCCH-format2 or PUCCH-format3, the PUCCH format configured for a PUCCH resource is PUCCH format 2 or PUCCH format 3, respectively, where the PUCCH resource also includes a number of PRBs provided by nrofPRBs, a number of symbols for a PUCCH transmission provided by nrofSymbols, and a first symbol for the PUCCH transmission provided by startingSymbolIndex. If a UE is provided by useInterlacePUCCH-PUSCH in BWP-UplinkDedicated, and the format indicates PUCCH-format2 or PUCCH-format3 and PUCCH-ResourceExt is provided, the PUCCH resource also includes an index of a second interlace by interlace1, if provided; otherwise, if interlace1 is not provided, the PUCCH resource also includes, if provided, an orthogonal cover code length by occ-Length and an orthogonal cover code index by occ-Index. If the format indicates PUCCH-format3 and PUCCH-ResourceExt is provided, the UE assumes that the MRBPUCCH,3 [4, TS38.211] PRBs with the lowest indexes within the first, and if configured, second interlace are used for PUCCH transmission.

If the format indicates PUCCH-format4, the PUCCH format configured for a PUCCH resource is PUCCH format 4, where the PUCCH resource also includes a number of symbols for a PUCCH transmission provided by nrofSymbols, an orthogonal cover code length by occ-Length, an orthogonal cover code index by occ-Index, and a first symbol for the PUCCH transmission provided by startingSymbolIndex.

If a UE is provided subslotLengthForPUCCH in a PUCCH-Config, the first symbol of a PUCCH resource provided by PUCCH-ResourceSet or SPS-PUCCH-AN-List in PUCCH-Config or by n1PUCCH-AN in SPS-Config for multiplexing HARQ-ACK in a PUCCH transmission is relative to the first symbol of the subslotLengthForPUCCH symbols [12, TS 38.331]. For the remaining cases, the first symbol of a PUCCH resource is relative to the first symbol of a slot with Nsymslot symbols [4, TS 38.211].

A UE can be configured up to four sets of PUCCH resources in a PUCCH-Config. A PUCCH resource set is provided by PUCCH-ResourceSet and is associated with a PUCCH resource set index provided by pucch-ResourceSetId, with a set of PUCCH resource indexes provided by resourceList that provides a set of pucch-ResourceId used in the PUCCH resource set, and with a maximum number of UCI information bits the UE can transmit using a PUCCH resource in the PUCCH resource set provided by maxPayloadSize. For the first PUCCH resource set, the maximum number of UCI information bits is 2. A maximum number of PUCCH resource indexes for a set of PUCCH resources is provided by maxNrofPUCCH-ResourcesPerSet. The maximum number of PUCCH resources in the first PUCCH resource set is 32 and the maximum number of PUCCH resources in the other PUCCH resource sets is 8.

If the UE transmits OUCI UCI information bits, that include HARQ-ACK information bits, the UE determines a PUCCH resource set to be

-	a first set of PUCCH resources with pucch-ResourceSetId = 0 if OUCI≤2 including 1 or 2 HARQ-ACK information bits and a positive or negative SR on one SR transmission occasion if transmission of HARQ-ACK information and SR occurs simultaneously, or

-	a second set of PUCCH resources with pucch-ResourceSetId = 1, if provided by higher layers, if 2<OUCI≤N2 where N2 is equal to maxPayloadSize if maxPayloadSize is provided for the PUCCH resource set with pucch-ResourceSetId = 1; otherwise N2 is equal to 1706, or

-	a third set of PUCCH resources with pucch-ResourceSetId = 2, if provided by higher layers, if N2<OUCI≤N3 where N3 is equal to maxPayloadSize if maxPayloadSize is provided for the PUCCH resource set with pucch-ResourceSetId = 2; otherwise N3 is equal to 1706, or

-	a fourth set of PUCCH resources with pucch-ResourceSetId = 3, if provided by higher layers, if N3<OUCI≤1706.

If the UE is provided SPS-PUCCH-AN-List and transmits OUCI UCI information bits that include only HARQ-ACK information bits in response to one or more SPS PDSCH receptions and SR, if any, the UE determines a PUCCH resource to be

-	a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the first entry in sps-PUCCH-AN-List if OUCI≤2 including 1 or 2 HARQ-ACK information bits and a positive or negative SR on one SR transmission occasion if transmission of HARQ-ACK information and SR occurs simultaneously, or

-	a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the second entry in sps-PUCCH-AN-List, if provided, if 2<OUCI≤N1,SPS where N1,SPS is either provided by maxPayloadSize obtained from the second entry in sps-PUCCH-AN-List or is otherwise equal to 1706, or

-	a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the third entry in sps-PUCCH-AN-List, if provided, if N1,SPS<OUCI≤N2,SPS where N2,SPS is either provided by maxPayloadSize obtained from the third entry in sps-PUCCH-AN-List or is otherwise equal to 1706, or

-	a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the fourth entry in sps-PUCCH-AN-List, if provided, if N2,SPS<OUCI≤N3,SPS where N3,SPS is equal to 1706.

### 9.2.2	PUCCH Formats for UCI transmission

If a UE is not transmitting PUSCH, and the UE is transmitting UCI, the UE transmits UCI in a PUCCH using

-	PUCCH format 0 if

-	the transmission is over 1 symbol or 2 symbols,

-	the number of HARQ-ACK information bits with positive or negative SR (HARQ-ACK/SR bits) is 1 or 2

-	PUCCH format 1 if

-	the transmission is over 4 or more symbols,

-	the number of HARQ-ACK/SR bits is 1 or 2

-	PUCCH format 2 if

-	the transmission is over 1 symbol or 2 symbols,

-	the number of UCI bits is more than 2

-	PUCCH format 3 if

-	the transmission is over 4 or more symbols,

-	the number of UCI bits is more than 2,

-	the PUCCH resource does not include an orthogonal cover code, or the UE is provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

-	PUCCH format 4 if

-	the transmission is over 4 or more symbols,

-	the number of UCI bits is more than 2,

-	the PUCCH resource includes an orthogonal cover code and the UE is not provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated

A spatial setting for a PUCCH transmission by a UE is provided by

-	an indicated TCI-State_r17, if provided, as described in [6, TS 38.214];

-	PUCCH-SpatialRelationInfo if the UE is configured with a single value for pucch-SpatialRelationInfoId;

-	as described in [11, TS 38.321], if the UE is provided multiple values for PUCCH-SpatialRelationInfo. The UE applies corresponding actions in [11, TS 38.321] and a corresponding setting for a spatial domain filter to transmit PUCCH in the first slot that is after slot k+3⋅Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information with ACK value corresponding to a PDSCH reception providing the PUCCH-SpatialRelationInfo, each slot consists of Nsymbslot symbols as defined in [4, TS 38.211], and μ is the SCS configuration for the PUCCH

-	If PUCCH-SpatialRelationInfo or the indicated TCI-State_r17 provides ssb-Index, the UE transmits the PUCCH using a same spatial domain filter as for a reception of a SS/PBCH block with index provided by ssb-Index for a same serving cell or, if servingCellId is provided, for a serving cell indicated by servingCellId

-	else if PUCCH-SpatialRelationInfo or the indicated TCI-State_r17 provides csi-RS-Index, the UE transmits the PUCCH using a same spatial domain filter as for a reception of a CSI-RS with resource index provided by csi-RS-Index for a same serving cell or, if servingCellId is provided, for a serving cell indicated by servingCellId

-	else PUCCH-SpatialRelationInfo or the indicated TCI-State_r17 provides srs, the UE transmits the PUCCH using a same spatial domain filter as for a transmission of a SRS with resource index provided by resource for a same serving cell and/or active UL BWP or, if servingCellId and/or uplinkBWP are provided, for a serving cell indicated by servingCellId and/or for an UL BWP indicated by uplinkBWP

If a UE

-	is not provided pathlossReferenceRSs in PUCCH-PowerControl,

-	is provided enableDefaultBeamPL-ForPUCCH, and

-	is not provided PUCCH-SpatialRelationInfo, and

-	is not provided coresetPoolIndex value of 1 for any CORESET, or is provided coresetPoolIndex value of 1 for all CORESETs, in ControlResourceSet and no codepoint of a TCI field, if any, in a DCI format of any search space set maps to two TCI states [5, TS 38.212]

a spatial setting for a PUCCH transmission from the UE is same as a spatial setting for PDCCH receptions by the UE in the CORESET with the lowest ID on the active DL BWP of the PCell. For a PUCCH transmission over multiple slots, a same spatial setting applies to the PUCCH transmission in each of the multiple slots.

A number of DMRS symbols for a PUCCH transmission using PUCCH format 3 or 4 is provided by additionalDMRS.

Use of /2-BPSK, instead of QPSK, for a PUCCH transmission using PUCCH format 3 or 4 is indicated by pi2BPSK.

### 9.2.3	UE procedure for reporting HARQ-ACK

A UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot per priority index, if the UE is not provided ackNackFeedbackMode = separate.

For DCI format 1_0, the PDSCH-to-HARQ_feedback timing indicator field values map to {1, 2, 3, 4, 5, 6, 7, 8}. For a DCI format, other than DCI format 1_0 or requesting Type-3 HARQ-ACK codebook report without scheduling a PDSCH reception as described in clause 9.1.4, the PDSCH-to-HARQ_feedback timing indicator field values, if present, map to values for a set of number of slots provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2, as defined in Table 9.2.3-1.

If the UE is provided subslotLengthForPUCCH, n is the last UL slot that overlaps with a PDSCH reception or with a PDCCH reception providing a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception; otherwise, n is the last UL slot that overlaps with the DL slot nD for the PDSCH reception or with the DL slot nD for the PDCCH reception.

For a SPS PDSCH reception ending in DL slot nD, the UE transmits the PUCCH in UL slot n+k where k is provided by the PDSCH-to-HARQ_feedback timing indicator field, if present, in a DCI format activating the SPS PDSCH reception.

If the UE detects a DCI format that does not include a PDSCH-to-HARQ_feedback timing indicator field and schedules a PDSCH reception or activates a SPS PDSCH reception ending in DL slot nD, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot n+k where k is provided by dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2.

If the UE detects a DCI format scheduling a PDSCH reception ending in DL slot nD or if the UE detects a DCI format for which the UE generates HARQ-ACK information through a PDCCH reception ending in DL slot nD,nD the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot n+k, where k is a number of slots and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2.

A PUCCH transmission with HARQ-ACK information is subject to the limitations for UE transmissions described in clause 11.1 and clause 11.1.1.

Table 9.2.3-1: Mapping of PDSCH-to-HARQ_feedback timing indicator field values to numbers of slots

| PDSCH-to-HARQ_feedback timing indicator |  |  | Number of slots |
| --- | --- | --- | --- |
| 1 bit | 2 bits | 3 bits |  |  |
| '0' | '00' | '000' | 1st value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2 |  |
| '1' | '01' | '001' | 2nd value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2 |  |
|  | '10' | '010' | 3rd value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2 |  |
|  | '11' | '011' | 4th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2 |  |
|  |  | '100' | 5th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2 |  |
|  |  | '101' | 6th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2 |  |
|  |  | '110' | 7th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2 |  |
|  |  | '111' | 8th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2 |  |

For a PUCCH transmission with HARQ-ACK information, a UE determines a PUCCH resource after determining a set of PUCCH resources for  HARQ-ACK information bits, as described in clause 9.2.1. The PUCCH resource determination is based on a PUCCH resource indicator field [5, TS 38.212], if present, in a last DCI format, among the DCI formats that have a value of a PDSCH-to-HARQ_feedback timing indicator field, if present, or a value of dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2, indicating a same slot for the PUCCH transmission, that the UE detects and for which the UE transmits corresponding HARQ-ACK information in the PUCCH where, for PUCCH resource determination, detected DCI formats are first indexed in an ascending order across serving cells indexes for a same PDCCH monitoring occasion and are then indexed in an ascending order across PDCCH monitoring occasion indexes. For indexing DCI formats within a serving cell for a same PDCCH monitoring occasion, if the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs on an active DL BWP of a serving cell, and with ackNackFeedbackMode = joint for the active UL BWP, detected DCI formats from PDCCH receptions in the first CORESETs are indexed prior to detected DCI formats from PDCCH receptions in the second CORESETs.

The PUCCH resource indicator field values map to values of a set of PUCCH resource indexes, as defined in Table 9.2.3-2 for a PUCCH resource indicator field of 3 bits, provided by resourceList for PUCCH resources from a set of PUCCH resources provided by PUCCH-ResourceSet with a maximum of eight PUCCH resources. If the PUCCH resource indicator field includes 1 bit or 2 bits, the values map to the first two values or the first four values, respectively, of Table 9.2.3-2. If the last DCI format does not include a PUCCH resource indicator field, the first value of Table 9.2.3-2 is used.

For the first set of PUCCH resources and when the size  of resourceList is larger than eight, when a UE provides HARQ-ACK information in a PUCCH transmission in response to detecting a last DCI format in a PDCCH reception, among DCI formats with a value of the PDSCH-to-HARQ_feedback timing indicator field, if present, or a value of dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2, indicating a same slot for the PUCCH transmission, the UE determines a PUCCH resource with index , , as

where  is a number of CCEs in CORESET  of the PDCCH reception for the DCI format as described in clause 10.1,  is the index of a first CCE for the PDCCH reception, and ∆PRI is a value of the PUCCH resource indicator field in the DCI format. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1, the CORESET is associated with the search space set having the smaller index. If the DCI format does not include a PUCCH resource indicator field, ∆PRI=0.

Table 9.2.3-2: Mapping of PUCCH resource indication field values to a PUCCH resource in a PUCCH resource set with maximum 8 PUCCH resources

| PUCCH resource indicator |  |  | PUCCH resource |
| --- | --- | --- | --- |
| 1 bit | 2 bits | 3 bits |  |  |
| '0' | '00' | '000' | 1st PUCCH resource provided by pucch-ResourceId obtained from the 1st value of resourceList |  |
| '1' | '01' | '001' | 2nd PUCCH resource provided by pucch-ResourceId obtained from the 2nd value of resourceList |  |
|  | '10' | '010' | 3rd PUCCH resource provided by pucch-ResourceId obtained from the 3rd value of resourceList |  |
|  | '11' | '011' | 4th PUCCH resource provided by pucch-ResourceId obtained from the 4th value of resourceList |  |
|  |  | '100' | 5th PUCCH resource provided by pucch-ResourceId obtained from the 5th value of resourceList |  |
|  |  | '101' | 6th PUCCH resource provided by pucch-ResourceId obtained from the 6th value of resourceList |  |
|  |  | '110' | 7th PUCCH resource provided by pucch-ResourceId obtained from the 7th value of resourceList |  |
|  |  | '111' | 8th PUCCH resource provided by pucch-ResourceId obtained from the 8th value of resourceList |  |

If a UE determines a first resource for a PUCCH transmission with HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH or detects a first DCI format indicating a first resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot and also detects at a later time a second DCI format indicating a second resource for a PUCCH transmission with corresponding HARQ-ACK information in the slot, the UE does not expect to multiplex HARQ-ACK information corresponding to the second DCI format in a PUCCH resource in the slot if the PDCCH reception that includes the second DCI format is not earlier than  from the beginning of a first symbol of the first resource for PUCCH transmission in the slot where, κ and Tc are defined in clause 4.1 of [4, TS 38.211] and  corresponds to the smallest SCS configuration among the SCS configurations of the PDCCHs providing the DCI formats and the SCS configuration of the PUCCH. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the second DCI format and for all serving cells with corresponding HARQ-ACK information multiplexed in the PUCCH transmission in the slot, for ,  for ,  for ; otherwise ,  for ,  for ,  for ,  for .

If a UE is not provided SPS-PUCCH-AN-List and transmits HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH, a PUCCH resource for corresponding PUCCH transmission with HARQ-ACK information is provided by n1PUCCH-AN.

If a UE transmits a PUCCH with HARQ-ACK information using PUCCH format 0, the UE determines values  and  for computing a value of cyclic shift  [4, TS 38.211] where  is provided by initialCyclicShift of PUCCH-format0 or, if initialCyclicShift is not provided, by the initial cyclic shift index as described in clause 9.2.1 and  is determined from the value of one HARQ-ACK information bit or from the values of two HARQ-ACK information bits as in Table 9.2.3-3 and Table 9.2.3-4, respectively.

Table 9.2.3-3: Mapping of values for one HARQ-ACK information bit to sequences for PUCCH format 0

| HARQ-ACK Value | 0 | 1 |
| --- | --- | --- |
| Sequence cyclic shift |  |  |

Table 9.2.3-4: Mapping of values for two HARQ-ACK information bits to sequences for PUCCH format 0

| HARQ-ACK Value | {0, 0} | {0, 1} | {1, 1} | {1, 0} |
| --- | --- | --- | --- | --- |
| Sequence cyclic shift |  |  |  |  |

If a UE transmits a PUCCH with HARQ-ACK information using PUCCH format 1, the UE is provided a value for  by initialCyclicShift of PUCCH-format1 or, if initialCyclicShift is not provided, by the initial cyclic shift index as described in clause 9.2.1.

If a UE transmits a PUCCH with  HARQ-ACK information bits and  bits using PUCCH format 2 or PUCCH format 3 in a PUCCH resource that includes  PRBs, the UE determines a number of PRBs  for the PUCCH transmission to be the minimum number of PRBs, that is smaller than or equal to a number of PRBs  provided respectively by nrofPRBs of PUCCH-format2 or nrofPRBs of PUCCH-format3 and start from the first PRB from the number of PRBs, that results to  and, if , , where , , , and  are defined in clause 9.2.5.2. For PUCCH format 3, if  is not equal  according to [4, TS 38.211],  is increased to the nearest allowed value of nrofPRBs for PUCCH-format3 [12, TS 38.331]. If , the UE transmits the PUCCH over  PRBs.

If a UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 in InterlaceAllocation and transmits a PUCCH with OACK HARQ-ACK information bits and OCRC bits using PUCCH format 2 or PUCCH format 3, the UE transmits the PUCCH over the first interlace if OACK+OCRC≤MInterlace,0PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r; otherwise, if the UE is provided a second interlace by interlace1 in PUCCH-format2 or PUCCH-format3, the UE transmits the PUCCH over the first and second interlaces.

## *** Unchanged text is omitted ***

### 9.2.4	UE procedure for reporting SR

A UE can be configured by SchedulingRequestResourceConfig a set of configurations for SR in a PUCCH transmission using either PUCCH format 0 or PUCCH format 1.

A UE can be configured by schedulingRequestID-BFR-SCell a configuration for LRR in a PUCCH transmission using either PUCCH format 0 or PUCCH format 1.

A UE can be configured by schedulingRequestIDForMTRPBFR a first configuration for LRR and, if the UE provides twoLRRcapability, a second configuration for LRR in a PUCCH transmission using either PUCCH format 0 or PUCCH format 1.

A UE can be configured by schedulingRequestID-LBT-SCell a configuration for consistent LBT failure recovery, as described in [11, TS 38.321], in a PUCCH transmission using either PUCCH format 0 or PUCCH format 1. The UE can be provided, by phy-PriorityIndex in SchedulingRequestResourceConfig, a priority index 0 or a priority index 1 for the SR. If the UE is not provided a priority index for SR, the priority index is 0.

The UE is also configured a periodicity  in symbols or slots and an offset  in slots by periodicityAndOffset for a PUCCH transmission conveying SR. If  is larger than one slot, the UE determines a SR transmission occasion in a PUCCH to be in a slot with number  [4, TS 38.211] in a frame with number  if .

If  is one slot, the UE expects that  and every slot is a SR transmission occasion in a PUCCH.

If  is smaller than one slot, the UE determines a SR transmission occasion in a PUCCH to start in a symbol with index  [4, TS 38.211] if  where  is the value of startingSymbolIndex.

If the UE determines that, for a SR transmission occasion in a PUCCH, the number of symbols available for the PUCCH transmission in a slot is smaller than the value provided by nrofSymbols, the UE does not transmit the PUCCH in the slot.

SR transmission occasions in a PUCCH are subject to the limitations for UE transmissions described in clause 11.1 and clause 11.1.1.

The UE transmits a PUCCH in the PUCCH resource for the corresponding SR configuration only when the UE transmits a positive SR. For a positive SR transmission using PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining  as described for HARQ-ACK information in clause 9.2.3 and by setting . For a positive SR transmission using PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting .

### 9.2.5	UE procedure for reporting multiple UCI types

This clause is applicable to the case that a UE has resources for PUCCH transmissions or for PUCCH and PUSCH transmissions that overlap in time and each PUCCH transmission is over a single slot without repetitions. Any case that a PUCCH transmission is with repetitions over multiple slots is described in clause 9.2.6. If a UE is configured with multiple PUCCH resources in a slot to transmit CSI reports

-	if the UE is not provided multi-CSI-PUCCH-ResourceList or if PUCCH resources for transmissions of CSI reports do not overlap in the slot, the UE determines a first resource corresponding to a CSI report with the highest priority [6, TS 38.214]

-	if the first resource includes PUCCH format 2, and if there are remaining resources in the slot that do not overlap with the first resource, the UE determines a CSI report with the highest priority, among the CSI reports with corresponding resources from the remaining resources, and a corresponding second resource as an additional resource for CSI reporting

-	if the first resource includes PUCCH format 3 or PUCCH format 4, and if there are remaining resources in the slot that include PUCCH format 2 and do not overlap with the first resource, the UE determines a CSI report with the highest priority, among the CSI reports with corresponding resources from the remaining resources, and a corresponding second resource as an additional resource for CSI reporting

-	if the UE is provided multi-CSI-PUCCH-ResourceList and if any of the multiple PUCCH resources overlap, the UE multiplexes all CSI reports in a resource from the resources provided by multi-CSI-PUCCH-ResourceList, as described in clause 9.2.5.2.

A UE multiplexes DL HARQ-ACK information, with or without SR, and CSI report(s) in a same PUCCH if the UE is provided simultaneousHARQ-ACK-CSI; otherwise, the UE drops the CSI report(s) and includes only DL HARQ-ACK information, with or without SR, in the PUCCH. If the UE would transmit multiple PUCCHs in a slot that include DL HARQ-ACK information and CSI report(s), the UE expects to be provided a same configuration for simultaneousHARQ-ACK-CSI each of PUCCH formats 2, 3, and 4.

If a UE would multiplex CSI reports that include Part 2 CSI reports in a PUCCH resource, the UE determines the PUCCH resource and a number of PRBs for the PUCCH resource or a number of Part 2 CSI reports assuming that each of the CSI reports indicates rank 1.

If a UE would transmit multiple overlapping PUCCHs in a slot or overlapping PUCCH(s) and PUSCH(s) in a slot and, when applicable as described in clauses 9.2.5.1 and 9.2.5.2, the UE is configured to multiplex different UCI types in one PUCCH, and at least one of the multiple overlapping PUCCHs or PUSCHs is in response to a DCI format detection by the UE, the UE multiplexes all corresponding UCI types if the following conditions are met. If one of the PUCCH transmissions or PUSCH transmissions is in response to a DCI format detection by the UE, the UE expects that the first symbol S0 of the earliest PUCCH or PUSCH, among a group overlapping PUCCHs and PUSCHs in the slot, satisfies the following timeline conditions

-	S0 is not before a symbol with CP starting after Tproc,1mux after a last symbol of any corresponding PDSCH, Tproc,1mux is given by maximum of Tproc,1mux,1,⋯,Tproc,1mux,i,⋯ where for the i-th PDSCH with corresponding HARQ-ACK transmission on a PUCCH which is in the group of overlapping PUCCHs and PUSCHs, Tproc,1mux,i=N1+d1,1+1⋅2048+144⋅κ⋅2-μ⋅TC, d1,1 is selected for the i-th PDSCH following [6, TS 38.214], N1 is selected based on the UE PDSCH processing capability of the i-th PDSCH and SCS configuration μ, where μ corresponds to the smallest SCS configuration among the SCS configurations used for the PDCCH scheduling the i-th PDSCH, the i-th PDSCH, the PUCCH with corresponding HARQ-ACK transmission for the i-th PDSCH, and all PUSCHs in the group of overlapping PUCCHs and PUSCHs.

-	S0 is not before a symbol with CP starting after Tproc,releasemux after a last symbol of a PDCCH reception providing a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception. Tproc,releasemux is given by maximum of Tproc,releasemux,1,⋯,Tproc,releasemux,i,⋯ where for the i-th PDCCH providing the DCI format with corresponding HARQ-ACK transmission on a PUCCH which is in the group of overlapping PUCCHs and PUSCHs, Tproc,releasemux,i=N+1⋅2048+144⋅κ⋅2-μ⋅TC, N as described in clause 10.2, where μ corresponds to the smallest SCS configuration among the SCS configurations used for the PDCCH, the PUCCH with corresponding HARQ-ACK information, and all PUSCHs in the group of overlapping PUCCHs and PUSCHs.

-	if there is no aperiodic CSI report multiplexed in a PUSCH in the group of overlapping PUCCHs and PUSCHs, S0 is not before a symbol with CP starting after Tproc,2mux after a last symbol of

-	any PDCCH with the DCI format scheduling an overlapping PUSCH, and

-	any PDCCH providing a DCI format with corresponding HARQ-ACK information in an overlapping PUCCH in the slot

If there is at least one PUSCH in the group of overlapping PUCCHs and PUSCHs, Tproc,2mux is given by maximum of Tproc,2mux,1,⋯,Tproc,2mux,i,⋯ where for the i-th PUSCH which is in the group of overlapping PUCCHs and PUSCHs, Tproc,2mux,i=maxN2+d2,1+1⋅2048+144⋅κ⋅2-μ⋅TC+Tswitch,d2,2, d2,1, d2,2 and Tswitch are selected for the i-th PUSCH following [6, TS 38.214], N2 is selected based on the UE PUSCH processing capability of the i-th PUSCH and SCS configuration μ, where μ corresponds to the smallest SCS configuration among the SCS configurations used for the PDCCH scheduling the i-th PUSCH, the PDCCHs scheduling the PDSCHs, or providing the DCI formats without scheduling PDSCHs, with corresponding HARQ-ACK information on a PUCCH which is in the group of overlapping PUCCHs/PUSCHs, and all PUSCHs in the group of overlapping PUCCHs and PUSCHs.

If there is no PUSCH in the group of overlapping PUCCHs and PUSCHs, Tproc,2mux is given by maximum of Tproc,2mux,1,⋯,Tproc,2mux,i,⋯ where for the i-th PDSCH, or the i-th PDCCH providing a DCI format without scheduling PDSCH, with corresponding HARQ-ACK information on a PUCCH which is in the group of overlapping PUCCHs, Tproc,2mux,i=N2+1⋅2048+144⋅κ⋅2-μ⋅TC, N2 is selected based on the UE PUSCH processing capability of the PUCCH serving cell if configured.  N2 is selected based on the UE PUSCH processing capability 1, if PUSCH processing capability is not configured for the PUCCH serving cell. μ is selected based on the smallest SCS configuration between the SCS configuration used for the PDCCH scheduling the i-th PDSCH, or providing the i-th DCI format without scheduling PDSCH with corresponding HARQ-ACK information on a PUCCH which is in the group of overlapping PUCCHs, and the SCS configuration for the PUCCH serving cell.

-	if there is an aperiodic CSI report multiplexed in a PUSCH in the group of overlapping PUCCHs and PUSCHs, S0 is not before a symbol with CP starting after Tproc,CSImux=maxZ+d⋅2048+144⋅κ⋅2-μ⋅TC+Tswitch,d2,2 after a last symbol of

-	any PDCCH with the DCI format scheduling an overlapping PUSCH, and

-	any PDCCH scheduling a PDSCH, or providing a DCI format, with corresponding HARQ-ACK information in an overlapping PUCCH in the slot

where μ corresponds to the smallest SCS configuration among the SCS configuration of the PDCCHs, the smallest SCS configuration for the group of the overlapping PUSCHs, and the smallest SCS configuration of CSI-RS associated with the DCI format scheduling the PUSCH with the multiplexed aperiodic CSI report, and d=2 for μ=0,1 , d=3 for μ=2, and d=4 for μ=3. Tswitch is defined in [6, TS 38.214] and it is applied only if Z1 of Table 5.4-1 in [6, TS 38.214] is applied to the determination of Z.

-	N1, N2, d1,1, d2,1, d2,2, and Z  are defined in [6, TS 38.214] and κ and TC are defined in [4, TS 38.211].

If a UE would transmit multiple overlapping PUCCHs in a slot or overlapping PUCCH(s) and PUSCH(s) in a slot, one of the PUCCHs includes HARQ-ACK information in response to an SPS PDSCH reception, and any PUSCH is not in response to a DCI format detection, the UE expects that the first symbol  of the earliest PUCCH or PUSCH satisfies the first of the previous timeline conditions with the exception that components associated to a SCS configuration for a PDCCH scheduling a PDSCH or a PUSCH are absent from the timeline conditions.

A UE does not expect a PUCCH or a PUSCH that is in response to a DCI format detection to overlap with any other PUCCH or PUSCH that does not satisfy the above timing conditions.

A UE that

-	is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and

-	is provided coresetPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and

-	is provided ackNackFeedbackMode = separate

does not expect a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the first CORESETs to overlap in time with a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the second CORESETs.

If there is one or more aperiodic CSI reports multiplexed on a PUSCH in the group of overlapping PUCCHs and PUSCHs and if symbol  is before symbol  that is a next uplink symbol with CP starting after  after the end of the last symbol of

-	the last symbol of aperiodic CSI-RS resource for channel measurements, and

-	the last symbol of aperiodic CSI-IM used for interference measurements, and

-	the last symbol of aperiodic NZP CSI-RS for interference measurements, when aperiodic CSI-RS is used for channel measurement for triggered CSI report

the UE is not required to update the CSI report for the triggered CSI report . is defined in [6, TS 38.214] and  corresponds to the smallest SCS configuration among the SCS configurations of the PDCCHs scheduling the PUSCHs, the smallest SCS configuration of aperiodic CSI-RSs associated with DCI formats provided by the PDCCHs triggering the aperiodic CSI reports, and the smallest SCS configuration of the overlapping PUCCHs and PUSCHs and  for ,  for  and  for .

If a UE would transmit multiple PUCCHs in a slot that include HARQ-ACK information, and/or SR, and/or CSI reports and any PUCCH with HARQ-ACK information in the slot satisfies the above timing conditions and does not overlap with any other PUCCH or PUSCH in the slot that does not satisfy the above timing conditions, the UE multiplexes the HARQ-ACK information, and/or SR, and/or  CSI reports and determines corresponding PUCCH(s) for transmission in the slot according to the following pseudo-code. If the multiple PUCCHs do not include HARQ-ACK information and do not overlap with any PUSCH transmission by the UE in response to a DCI format detection by the UE, the timing conditions do not apply.

If

-	a UE is not provided multi-CSI-PUCCH-ResourceList, and

-	a resource for a PUCCH transmission with HARQ-ACK information in response to SPS PDSCH reception and/or a resource for a PUCCH associated with a SR occasion overlap in time with two resources for respective PUCCH transmissions with two CSI reports, and

-	there is no resource for a PUCCH transmission with HARQ-ACK information in response to a DCI format detection that overlaps in time with any of the previous resources, and

-	the following pseudo code results to the UE attempting to determine a single PUCCH resource from the HARQ-ACK and/or the SR resource and the two PUCCH resources with CSI reports

the UE

-	multiplexes the HARQ-ACK information and/or the SR in the resource for the PUCCH transmission with the CSI report having the higher priority, and

-	does not transmit the PUCCH with the CSI report having the lower priority

Set  to the set of resources for transmission of corresponding PUCCHs in a single slot without repetitions where

-	a resource with earlier first symbol is placed before a resource with later first symbol

-	for two resources with same first symbol, the resource with longer duration is placed before the resource with shorter duration

-	for two resources with same first symbol and same duration, the placement is arbitrary

-	the above three steps for the set  are according to a subsequent pseudo-code for a function

-	a resource for negative SR transmission that does not overlap with a resource for HARQ-ACK or CSI transmission is excluded from set

-	if the UE is not provided simultaneousHARQ-ACK-CSI and resources for transmission of HARQ-ACK information include PUCCH format 0 or PUCCH format 2, resources that include PUCCH format 2, or PUCCH format 3, or PUCCH format 4 for transmission of CSI reports are excluded from the set  if they overlap with any resource from the resources for transmission of HARQ-ACK information

-	if the UE is not provided simultaneousHARQ-ACK-CSI and at least one of the resources for transmission of HARQ-ACK information includes PUCCH format 1, PUCCH format 3, or PUCCH format 4

-	resources that include PUCCH format 3 or PUCCH format 4 for transmission of CSI reports are excluded from the set

-	resources that include PUCCH format 2 for transmission of CSI reports are excluded from the set  if they overlap with any resource from the resources for transmission of HARQ-ACK information

Set  to the cardinality of

Set to be the first symbol of resource  in the slot

Set  to be the number of symbols of resource  in the slot

Set  - index of first resource in set

Set  - counter of overlapped resources

while

if  and resource  overlaps with resource

else

if

determine a single resource for multiplexing UCI associated with resources  as described in clauses 9.2.5.0, 9.2.5.1 and 9.2.5.2

set the index of the single resource to

% start from the beginning after reordering unmerged resources at next step

% function that re-orders resources in current set

Set  to the cardinality of

else

end if

end if

end while

The function  performs the following pseudo-code

{

while  % the next two while loops are to re-order the unmerged resources

while

if  OR

end if

end while

end while

}

For each PUCCH resource in the set  that satisfies the aforementioned timing conditions, when applicable,

-	the UE transmits a PUCCH using the PUCCH resource if the PUCCH resource does not overlap in time with a PUSCH transmission after multiplexing UCI following the procedures described in clauses 9.2.5.1 and 9.2.5.2

-	the UE multiplexes HARQ-ACK information and/or CSI reports in a PUSCH if the PUCCH resource overlaps in time with a PUSCH transmission, as described in clause 9.3, and does not transmit SR. In case the PUCCH resource overlaps in time with multiple PUSCH transmissions, the PUSCH for multiplexing HARQ-ACK information and/or CSI is selected as described in clause 9. If the PUSCH transmission by the UE is not in response to a DCI format detection and the UE multiplexes only CSI reports, the timing conditions are not applicable

-	the UE does not expect the resource to overlap with a second resource of a PUCCH transmission over multiple slots if the resource is obtained from a group of resources that do not overlap with the second resource.

clauses 9.2.5.0, 9.2.5.1 and 9.2.5.2 assume the following

-	resources for transmissions of UCI types, prior to multiplexing or dropping, overlap in a slot

-	multiplexing conditions of corresponding UCI types in a single PUCCH are satisfied, and

-	the UE does not transmit any PUSCH time-overlapping with PUCCH in the slot.

## *** Unchanged text is omitted ***

### 9.2.6	PUCCH repetition procedure

For PUCCH formats 1, 3, or 4, a UE can be configured a number of slots, NPUCCHrepeat, for repetitions of a PUCCH transmission by respective nrofSlots. If a UE is provided a PUCCH-config that includes subslotLengthForPUCCH, the UE does not expect the PUCCH-config to include nrofSlots.

For NPUCCHrepeat>1,

-	the UE repeats the PUCCH transmission with the UCI over NPUCCHrepeat slots

-	a PUCCH transmission in each of the NPUCCHrepeat slots has a same number of consecutive symbols, as provided by nrofSymbols in PUCCH-format1, nrofSymbols in PUCCH-format3, or nrofSymbols in PUCCH-format4

-	a PUCCH transmission in each of the NPUCCHrepeat slots has a same first symbol, as provided by startingSymbolIndex in PUCCH-format1, startingSymbolIndex in PUCCH-format3, or startingSymbolIndex in PUCCH-format4

-	the UE is configured by interslotFrequencyHopping whether or not to perform frequency hopping for PUCCH transmissions in different slots

-	if the UE is configured to perform frequency hopping for PUCCH transmissions across different slots

-	the UE performs frequency hopping per slot

-	the UE transmits the PUCCH starting from a first PRB, provided by startingPRB, in slots with even number and starting from the second PRB, provided by secondHopPRB, in slots with odd number. The slot indicated to the UE for the first PUCCH transmission has number 0 and each subsequent slot until the UE transmits the PUCCH in NPUCCHrepeat slots is counted regardless of whether or not the UE transmits the PUCCH in the slot

-	the UE does not expect to be configured to perform frequency hopping for a PUCCH transmission within a slot

-	If the UE is not configured to perform frequency hopping for PUCCH transmissions across different slots and if the UE is configured to perform frequency hopping for a PUCCH transmission within a slot, the frequency hopping pattern between the first PRB and the second PRB is same within each slot

If the UE determines that, for a PUCCH transmission in a slot, the number of symbols available for the PUCCH transmission is smaller than the value provided by nrofSymbols for the corresponding PUCCH format, the UE does not transmit the PUCCH in the slot.

A SS/PBCH block symbol is a symbol of an SS/PBCH block with candidate SS/PBCH block index corresponding to the SS/PBCH block index indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, as described in clause 4.1.

For unpaired spectrum, the UE determines the NPUCCHrepeat slots for a PUCCH transmission starting from a slot indicated to the UE as described in clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in clause 9.2.4 for SR reporting or in clause 5.2.1.4 of [6, TS 38.214] for CSI reporting and having

-	an UL symbol, as described in clause 11.1, or flexible symbol that is not SS/PBCH block symbol provided by startingSymbolIndex in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 as a first symbol, and

-	consecutive UL symbols, as described in clause 11.1, or flexible symbols that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4

For paired spectrum or supplementary uplink band, the UE determines the NPUCCHrepeat slots for a PUCCH transmission as the NPUCCHrepeat consecutive slots starting from a slot indicated to the UE as described in clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in clause 9.2.4 for SR reporting or in clause 5.2.1.4 of [6, TS 38.214] for CSI reporting.

If a UE would transmit a PUCCH over a first number NPUCCHrepeat>1 of slots and the UE would transmit a PUSCH with repetition Type A over a second number of slots, and the PUCCH transmission would overlap with the PUSCH transmission in one or more slots, and the conditions in clause 9.2.5 for multiplexing the UCI in the PUSCH are satisfied in the overlapping slots, the UE transmits the PUCCH and does not transmit the PUSCH in the overlapping slots.

If a UE would transmit a PUCCH over a first number NPUCCHrepeat>1 of slots and the UE would transmit a PUSCH with repetition Type B over a second number of slots, and the PUCCH transmission would overlap with actual PUSCH repetitions in one or more slots, and the conditions in clause 9.2.5 for multiplexing the UCI in the PUSCH are satisfied for the overlapping actual PUSCH repetitions, the UE transmits the PUCCH and does not transmit the overlapping actual PUSCH repetitions.

A UE does not multiplex different UCI types in a PUCCH transmission with repetitions over NPUCCHrepeat>1 slots. If a UE would transmit a first PUCCH over more than one slot and at least a second PUCCH over one or more slots, and the transmissions of the first PUCCH and the second PUCCH would overlap in a number of slots then, for each slot of the number of slots and with UCI type priority of HARQ-ACK > SR > CSI with higher priority > CSI with lower priority

-	the UE does not expect the first PUCCH and any of the second PUCCHs to start at a same slot and include a UCI type with same priority

-	if the first PUCCH and any of the second PUCCHs include a UCI type with same priority, the UE transmits the PUCCH starting at an earlier slot and does not transmit the PUCCH starting at a later slot

-	if the first PUCCH and any of the second PUCCHs do not include a UCI type with same priority, the UE transmits the PUCCH that includes the UCI type with higher priority and does not transmit the PUCCH that include the UCI type with lower priority

When a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes first and second spatial settings, or first and second sets of power control parameters, as described in [11, TS 38.321] and in clause 7.2.1, the UE

-	uses the first and second spatial settings, or the first and second sets of power control parameters, for first and second repetitions of the PUCCH transmission, respectively, when NPUCCHrepeat=2,

-	alternates between the first and second spatial settings, or between the first and second sets of power control parameters, respectively, per NPUCCHswitch repetitions of the PUCCH transmission, where NPUCCHswitch=1 if mappingPattern = ‘cyclicMapping’; else, NPUCCHswitch=2.

A UE does not expect a PUCCH that is in response to a DCI format detection to overlap with any other PUCCH that does not satisfy the corresponding timing conditions in clause 9.2.5.

If a UE would transmit a PUCCH over NPUCCHrepeat slots and the UE does not transmit the PUCCH in a slot from the NPUCCHrepeat slots due to overlapping with another PUCCH transmission in the slot, the UE counts the slot in the number of NPUCCHrepeat slots.

For DAPS operation, if a UE would transmit a PUCCH over NPUCCHrepeat slots on the source MCG and the UE does not transmit the PUCCH in a slot from the NPUCCHrepeat slots due to overlapping in time with UE transmission on the target MCG in the slot, the UE counts the slot in the number of NPUCCHrepeat slots.

## *** Unchanged text is omitted ***

# 10	UE procedure for receiving control information

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG except for PDCCH monitoring in Type0/0A/2-PDCCH CSS sets where the UE is not required to apply the procedures in this clause for the SCG

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG.

A UE monitors a set of PDCCH candidates in one or more CORESETs on the active DL BWP on each activated serving cell configured with PDCCH monitoring according to corresponding search space sets where monitoring implies receiving each PDCCH candidate and decoding according to the monitored DCI formats.

In the remaining of this clause, when a PDCCH reception by a UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1

-	a PDCCH monitoring occasion is the union of the PDCCH monitoring occasions for the two PDCCH candidates

-	the start of the PDCCH reception is the start of the earlier PDCCH candidate

-	the end of the PDCCH reception is the end of the PDCCH candidate that ends later

The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10, 11.1, and 11.1.1.

If a UE is provided monitoringCapabilityConfig for a serving cell, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs

-	per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or

-	per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability

If the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot.

A UE can indicate a capability to monitor PDCCH according to one or more of the combinations X,Y = (2, 2), (4, 3), and (7, 3) per SCS configuration of μ=0 and μ=1. A span is a number of consecutive symbols in a slot where the UE is configured to monitor PDCCH. Each PDCCH monitoring occasion is within one span. If a UE monitors PDCCH on a cell according to combination X,Y, the UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. A span starts at a first symbol where a PDCCH monitoring occasion starts and ends at a last symbol where a PDCCH monitoring occasion ends, where the number of symbols of the span is up to Y.

If a UE indicates a capability to monitor PDCCH according to multiple X,Y combinations and a configuration of search space sets to the UE for PDCCH monitoring on a cell results to a separation of every two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for one or more of the multiple combinations X,Y, the UE monitors PDCCH on the cell according to the combination X,Y, from the one or more combinations X,Y, that is associated with the largest maximum number of MPDCCHmax,X,Y,μ and CPDCCHmax,X,Y,μ defined in Table 10.1-2A and Table 10.1-3A. The UE expects to monitor PDCCH according to the same combination X,Y in every slot on the active DL BWP of a cell.

A UE capability for PDCCH monitoring per slot or per span on an active DL BWP of a serving cell is defined by a maximum number of PDCCH candidates and non-overlapped CCEs the UE can monitor per slot or per span, respectively, on the active DL BWP of the serving cell.

For monitoring of a PDCCH candidate by a UE, if the UE

-	has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and

-	does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1,

the UE is not required to monitor the PDCCH candidate.

For monitoring of a PDCCH candidate by a UE, if the UE

-	has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and

-	does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon,

the UE is not required to monitor the PDCCH candidate.

For monitoring of a PDCCH candidate by a UE, if the UE

-	has received ssb-PositionsInBurst in AdditionalPCIInfo for a serving cell, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in AdditionalPCIInfo with same physical cell identity as the one associated with a RS having same quasi-collocation properties as a CORESET for the PDCCH candidate,

the UE is not required to monitor the PDCCH candidate.

If a UE is not provided TCI-State_r17, the UE is not required to monitor PDCCH candidates for a Type0/0A/1-PDCCH CSS set when the active TCI state for a corresponding CORESET is not associated with physCellId in ServingCellConfigCommon.

If a UE monitors the PDCCH candidate for a Type0-PDCCH CSS set on the serving cell according to the procedure described in clause 13, the UE may assume that no SS/PBCH block is transmitted in REs used for monitoring the PDCCH candidate on the serving cell.

If at least one RE of a PDCCH candidate for a UE on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround, or of LTE-CRS-PatternList, the UE is not required to monitor the PDCCH candidate.

If a UE is provided availableRB-SetsPerCell, the UE is not required to monitor PDCCH candidates that overlap with any RB from RB sets that are indicated as unavailable for receptions by an available RB set indicator field in DCI format 2_0 as described in clause 11.1.1. If the UE does not obtain the available RB set indicator for a symbol, the UE monitors PDCCH candidates on all RB sets in the symbol.

If a UE can support

-	a first set of Ncells,0DL serving cells where the UE is either not provided coresetPoolIndex or is provided coresetPoolIndex with a single value for all CORESETs on all DL BWPs of each scheduling cell from the first set of serving cells, and

-	a second set of Ncells,1DL serving cells where the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with a value 0 for a first CORESET, and with a value 1 for a second CORESET on any DL BWP of each scheduling cell from the second set of serving cells

the UE determines, for the purpose of reporting pdcch-BlindDetectionCA, a number of serving cells as Ncells,0DL+R⋅Ncells,1DL where R is a value reported by the UE.

If a UE indicates in UE-NR-Capability a carrier aggregation capability larger than 4 serving cells and the UE is not provided monitoringCapabilityConfig for any downlink cell or if the UE is provided monitoringCapabilityConfig = r15monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE includes in UE-NR-Capability an indication for a maximum number of PDCCH candidates and for a maximum number of non-overlapped CCEs the UE can monitor per slot when the UE is configured for carrier aggregation operation over more than 4 cells. When a UE is not configured for NR-DC operation, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per slot that corresponds to Ncellscap downlink cells, where

-	Ncellscap is Ncells,0DL+R⋅Ncells,1DL if the UE does not provide pdcch-BlindDetectionCA where Ncells,0DL+Ncells,1DL is the number of configured downlink serving cells

-	otherwise, Ncellscap is the value of pdcch-BlindDetectionCA

When a UE is configured for NR-DC operation, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per slot that corresponds to Ncellscap=NcellsMCG downlink cells for the MCG where NcellsMCG is provided by pdcch-BlindDetection for the MCG and determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per slot that corresponds to Ncellscap=NcellsSCG downlink cells for the SCG where NcellsSCG is provided by pdcch-BlindDetection for the SCG. When the UE is configured for carrier aggregation operation over more than 4 cells, or for a cell group when the UE is configured for NR-DC operation, the UE does not expect to monitor per slot a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncellscap.

When a UE is configured for NR-DC operation with a total of NNR-DCDL,cells downlink cells on both the MCG and the SCG, the UE expects to be provided pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG with values that satisfy

-	pdcch-BlindDetection for the MCG + pdcch-BlindDetection for the SCG <= pdcch-BlindDetectionCA, if the UE reports pdcch-BlindDetectionCA, or

-	pdcch-BlindDetection for the MCG + pdcch-BlindDetection for the SCG <= NNR-DCDL,cells, if the UE does not report pdcch-BlindDetectionCA.

For NR-DC operation, the UE may indicate, through pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE, respective maximum values for pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG.

If the UE reports pdcch-BlindDetectionCA,

-	the value range of pdcch-BlindDetectionMCG-UE or of pdcch-BlindDetectionSCG-UE is [1, …, pdcch-BlindDetectionCA-1], and

-	pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= pdcch-BlindDetectionCA.

Otherwise, if NNR-DC,maxDL,cells is a maximum total number of downlink cells that the UE can be configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability,

-	the value range of pdcch-BlindDetectionMCG-UE or of pdcch-BlindDetectionSCG-UE is [1, 2, 3], and

-	pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= NNR-DC,maxDL,cells.

If a UE indicates in UE-NR-Capability a carrier aggregation capability larger than two downlink cells, the UE includes in UE-NR-Capability an indication for a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs that the UE can monitor per span when the UE is configured for carrier aggregation operation over more than two downlink cells with monitoringCapabilityConfig = r16monitoringcapability. When a UE is not configured for NR-DC operation and the UE is provided monitoringCapabilityConfig = r16monitoringcapability for all downlink cell where the UE monitors PDCCH, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per span that corresponds to Ncellscap-r16 downlink cells, where

-	Ncellscap-r16 is the number of configured downlink cells if the UE does not provide pdcch-MonitoringCA

-	otherwise, Ncellscap-r16 is the value of pdcch-MonitoringCA

When a UE is configured for NR-DC operation and the UE is provided monitoringCapabilityConfig = r16monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per span that corresponds to

-	Ncellscap-r16= Ncells, r16MCG downlink cells for the MCG where Ncells, r16MCG is provided by pdcch-BlindDetection2 for the MCG, and

-	Ncellscap-r16= Ncells, r16SCG downlink cells for the SCG where Ncells, r16SCG is provided by pdcch-BlindDetection2 for the SCG

When the UE is configured for carrier aggregation operation over more than 2 cells, or for a cell group when the UE is configured for NR-DC operation, the UE does not expect to monitor per span a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncellscap-r16.

When a UE is configured for NR-DC operation with a total of NNR-DCDL,cells downlink cells on both the MCG and the SCG and the UE is provided monitoringCapabilityConfig = r16monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE expects to be provided pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG with values that satisfy

-	pdcch-BlindDetection2 for the MCG + pdcch-BlindDetection2 for the SCG <= pdcch-MonitoringCA, if the UE reports pdcch-MonitoringCA, or

-	pdcch-BlindDetection2 for the MCG + pdcch-BlindDetection2 for the SCG <= NNR-DCDL,cells, if the UE does not report pdcch-MonitoringCA

When a UE is configured for NR-DC operation and the UE is provided monitoringCapabilityConfig = r16monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE may indicate, through pdcch-BlindDetectionMCG-UE-r16 and pdcch-BlindDetectionSCG-UE-r16, respective maximum values for pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG.

If the UE reports pdcch-MonitoringCA,

-	the value range of pdcch-BlindDetectionMCG-UE-r16 or of pdcch-BlindDetectionSCG-UE-r16 is [1, …, pdcch-MonitoringCA-1], and

-	pdcch-BlindDetectionMCG-UE-r16 + pdcch-BlindDetectionSCG-UE-r16 >= pdcch-MonitoringCA.

Otherwise, if NNR-DC,  max, r16DL,cells is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability

-	the value of pdcch-BlindDetectionMCG-UE-r16 or of pdcch-BlindDetectionSCG-UE-r16 is 1,

-	pdcch-BlindDetectionMCG-UE-r16 + pdcch-BlindDetectionSCG-UE-r16 >= NNR-DC,  max, r16DL,cells.

If a UE indicates in UE-NR-Capability a carrier aggregation capability larger than one downlink cell with monitoringCapabilityConfig = r15monitoringcapability or larger than one downlink cell with monitoringCapabilityConfig = r16monitoringcapability, the UE includes in UE-NR-Capability an indication for a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs the UE can monitor for downlink cells with monitoringCapabilityConfig = r15monitoringcapability or for downlink cells with monitoringCapabilityConfig = r16monitoringcapability when the UE is configured for carrier aggregation operation over more than two downlink cells with at least one downlink cell with monitoringCapabilityConfig = r15monitoringcapability and at least one downlink cell with monitoringCapabilityConfig = r16monitoringcapability. When a UE is not configured for NR-DC operation, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per slot or per span that corresponds to Ncells,r15cap-r16 downlink cells or to Ncells,r16cap-r16 downlink cells, respectively, where

-	Ncells,r15cap-r16 is the number of configured downlink cells if the UE does not provide pdcch-BlindDetectionCA1

-	otherwise,

-	if the UE reports only one combination of (pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2), Ncells,r15cap-r16 is the value of pdcch-BlindDetectionCA1

-	else, Ncells,r15cap-r16 is the value of pdcch-BlindDetectionCA1 from a combination of (pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2) that is provided by pdcch-BlindDetectionCA-CombIndicator

and

-	Ncells,r16cap-r16 is the number of configured downlink cells if the UE does not provide pdcch-BlindDetectionCA2

-	otherwise,

-	if the UE reports only one combination of (pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2), Ncells,r16cap-r16 is the value of pdcch-BlindDetectionCA2

-	else, Ncells,r16cap-r16 is the value of pdcch-BlindDetectionCA2 from a combination of (pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2) that is provided by pdcch-BlindDetectionCA-CombIndicator

When a UE is configured for NR-DC operation and is provided monitoringCapabilityConfig = r15monitoringcapability for at least one downlink cell and monitoringCapabilityConfig = r16monitoringcapability for at least one downlink cell where the UE monitors PDCCH, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs that corresponds to

-	Ncells,r15cap-r16= Ncells, r15MCG downlink cells for the MCG where Ncells, r15MCG is provided by pdcch-BlindDetection3 for the MCG,

-	Ncells,r15cap-r16= Ncells, r15SCG downlink cells for the SCG where Ncells, r15SCG is provided by pdcch-BlindDetection3 for the SCG, and

-	Ncells,r16cap-r16= Ncells, r16MCG downlink cells for the MCG where Ncells, r16MCG is provided by pdcch-BlindDetection2 for the MCG,

-	Ncells,r16cap-r16= Ncells, r16SCG downlink cells for the SCG where Ncells, r16SCG is provided by pdcch-BlindDetection2 for the SCG

When the UE is configured for carrier aggregation operation over more than two downlink cells with at least one downlink cell with monitoringCapabilityConfig = r15monitoringcapability and at least one downlink cell with monitoringCapabilityConfig = r16monitoringcapability, or for a cell group when the UE is configured for NR-DC operation, the UE does not expect to

-	monitor per slot a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncells,r15cap-r16, and

-	monitor per span a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncells,r16cap-r16

When a UE is configured for NR-DC operation with a total of NNR-DCDL,cells downlink cells on both the MCG and the SCG and the UE is provided monitoringCapabilityConfig = r15monitoringcapability for NNR-DC,r15DL,cells downlink cells and monitoringCapabilityConfig = r16monitoringcapability for NNR-DC,r16DL,cells downlink cells where the UE monitors PDCCH, the UE expects to be provided pdcch-BlindDetection3 and pdcch-BlindDetection2 for the MCG, and pdcch-BlindDetection3 and pdcch-BlindDetection2 for the SCG with values that satisfy

-	pdcch-BlindDetection3 for the MCG + pdcch-BlindDetection3 for the SCG <= pdcch-BlindDetectionCA1, if the UE reports pdcch-BlindDetectionCA1, or

-	pdcch-BlindDetection3 for the MCG + pdcch-BlindDetection3 for the SCG <= NNR-DC,r15DL,cells, if the UE does not report pdcch-BlindDetectionCA1

and

-	pdcch-BlindDetection2 for the MCG + pdcch-BlindDetection2 for the SCG <= pdcch-BlindDetectionCA2, if the UE reports pdcch-BlindDetectionCA2, or

-	pdcch-BlindDetection2 for the MCG + pdcch-BlindDetection2 for the SCG <= NNR-DC,r16DL,cells, if the UE does not report pdcch-BlindDetectionCA2

When a UE is configured for NR-DC operation and is provided monitoringCapabilityConfig = r15monitoringcapability for at least one downlink cell and monitoringCapabilityConfig = r16monitoringcapability for at least one downlink cell where the UE monitors PDCCH, the UE may indicate, through pdcch-BlindDetectionMCG-UE1 and pdcch-BlindDetectionSCG-UE1, respective maximum values for pdcch-BlindDetection3 for the MCG and pdcch-BlindDetection3 for the SCG, and through pdcch-BlindDetectionMCG-UE2 and pdcch-BlindDetectionSCG-UE2 respective maximum values for pdcch-BlindDetection2 for the MCG and pdcch-BlindDetection2 for the SCG.

If the UE reports pdcch-BlindDetectionCA1,

-	the value range of pdcch-BlindDetectionMCG-UE1 or of pdcch-BlindDetectionSCG-UE1 is [0, 1, …, pdcch-BlindDetectionCA1], and

-	pdcch-BlindDetectionMCG-UE1 + pdcch-BlindDetectionSCG-UE1 >= pdcch-BlindDetectionCA1.

Otherwise, if NNR-DC,  max, r15DL,cells is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig = r15monitoringcapability and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability

-	the value range of pdcch-BlindDetectionMCG-UE1 or of pdcch-BlindDetectionSCG-UE1 is [0, 1, 2],

-	pdcch-BlindDetectionMCG-UE1 + pdcch-BlindDetectionSCG-UE1 >= NNR-DC,  max, r15DL,cells.

If the UE reports pdcch-BlindDetectionCA2

-	the value range of pdcch-BlindDetectionMCG-UE2 or of pdcch-BlindDetectionSCG-UE2 is [0, 1, …, pdcch-BlindDetectionCA2], and

-	pdcch-BlindDetectionMCG-UE2 + pdcch-BlindDetectionSCG-UE2 >= pdcch-BlindDetectionCA2.

Otherwise, if NNR-DC,  max, r16DL,cells is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability

-	the value range of pdcch-BlindDetectionMCG-UE2 or of pdcch-BlindDetectionSCG-UE2 is [0, 1],

-	pdcch-BlindDetectionMCG-UE2 + pdcch-BlindDetectionSCG-UE2 >= NNR-DC,  max, r16DL,cells.

## 10.1	UE procedure for determining physical downlink control channel assignment

A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets

-	a Type0-PDCCH CSS set configured by pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell

-	a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a P-RNTI on the primary cell of the MCG

-	a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, or CI-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, CS-RNTI(s), or PS-RNTI and

-	a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, CS-RNTI(s), SL-RNTI, SL-CS-RNTI, or SL Semi-Persistent Scheduling V-RNTI.

For a DL BWP, if a UE is not provided searchSpaceSIB1 for Type0-PDCCH CSS set by PDCCH-ConfigCommon, the UE does not monitor PDCCH candidates for a Type0-PDCCH CSS set on the DL BWP. The Type0-PDCCH CSS set is defined by the CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level given in Table 10.1-1. If the active DL BWP and the initial DL BWP have same SCS and same CP length and the active DL BWP includes all RBs of the CORESET with index 0, or the active DL BWP is the initial DL BWP, the CORESET configured for Type0-PDCCH CSS set has CORESET index 0 and the Type0-PDCCH CSS set has search space set index 0.

For a DL BWP, if a UE is not provided searchSpaceOtherSystemInformation for Type0A-PDCCH CSS set, the UE does not monitor PDCCH for Type0A-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level for Type0A-PDCCH CSS set are given in Table 10.1-1.

For a DL BWP, if a UE is not provided ra-SearchSpace for Type1-PDCCH CSS set, the UE does not monitor PDCCH for Type1-PDCCH CSS set on the DL BWP. If the UE has not been provided a Type3-PDCCH CSS set or a USS set and the UE has received a C-RNTI and has been provided a Type1-PDCCH CSS set, the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI in the Type1-PDCCH CSS set.

If a UE is not provided pagingSearchSpace for Type2-PDCCH CSS set, the UE does not monitor PDCCH for Type2-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level for Type2-PDCCH CSS set are given in Table 10.1-1.

If a UE is provided a zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set as described in clause 13, and the UE is provided a C-RNTI, the UE monitors PDCCH candidates only at monitoring occasions associated with a SS/PBCH block, where the SS/PBCH block is determined by the most recent of

-	a MAC CE activation command indicating a TCI state of the active BWP that includes a CORESET with index 0, as described in [6, TS 38.214], where the TCI-state includes a CSI-RS which is quasi-co-located with the SS/PBCH block, or

-	a random access procedure that is not initiated by a PDCCH order that triggers a contention-free random access procedure

If a UE monitors PDCCH candidates for DCI formats with CRC scrambled by a C-RNTI and the UE is provided a non-zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set based on the search space set associated with the value of searchSpaceID.

The UE may assume that the DM-RS antenna port associated with PDCCH receptions in the CORESET configured by pdcch-ConfigSIB1 in MIB, the DM-RS antenna port associated with corresponding PDSCH receptions, and the corresponding SS/PBCH block are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable [6, TS 38.214], if the UE is not provided a TCI state indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in the CORESET. The value for the DM-RS scrambling sequence initialization is the cell ID. A SCS is provided by subCarrierSpacingCommon in MIB.

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not expect to monitor a PDCCH in a Type0/0A/2/3-PDCCH CSS set or in a USS set if a DM-RS for monitoring a PDCCH in a Type1-PDCCH CSS set is not configured with same qcl-Type set to 'typeD' properties [6, TS 38.214] with a DM-RS for monitoring the PDCCH in the Type0/0A/2/3-PDCCH CSS set or in the USS set, and if the PDCCH or an associated PDSCH overlaps in at least one symbol with a PDCCH the UE monitors in a Type1-PDCCH CSS set or with an associated PDSCH.

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, and

-	a C-RNTI, an MCS-C-RNTI, or a CS-RNTI

the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in the one or more search space sets in a slot where the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI, or P-RNTI.

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and

-	a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI

then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot.

Table 10.1-1: CCE aggregation levels and maximum number of PDCCH candidates per CCE aggregation level for CSS sets configured by searchSpaceSIB1

| CCE Aggregation Level | Number of Candidates |
| --- | --- |
| 4 | 4 |
| 8 | 2 |
| 16 | 1 |

For each DL BWP configured to a UE in a serving cell, the UE can be provided by higher layer signalling with

-	P≤3 CORESETs if coresetPoolIndex is not provided, or if a value of coresetPoolIndex is same for all CORESETs if coresetPoolIndex is provided

-	P≤5 CORESETs if coresetPoolIndex is not provided for a first CORESET, or is provided and has a value 0 for a first CORESET, and is provided and has a value 1 for a second CORESET

For each CORESET, the UE is provided the following by ControlResourceSet:

-	a CORESET index p, by controlResourceSetId  or by controlResourceSetId-v1610, where

-	0<p<12 if coresetPoolIndex is not provided, or if a value of coresetPoolIndex is same for all CORESETs if coresetPoolIndex is provided;

-	0<p<16 if coresetPoolIndex is not provided for a first CORESET, or is provided and has a value 0 for a first CORESET, and is provided and has a value 1 for a second CORESET;

-	a DM-RS scrambling sequence initialization value by pdcch-DMRS-ScramblingID;

-	a precoder granularity for a number of REGs in the frequency domain where the UE can assume use of a same DM-RS precoder by precoderGranularity;

-	a number of consecutive symbols provided by duration;

-	a set of resource blocks provided by frequencyDomainResources;

-	CCE-to-REG mapping parameters provided by cce-REG-MappingType;

-	an antenna port quasi co-location, from a set of antenna port quasi co-locations provided by TCI-State, indicating quasi co-location information of the DM-RS antenna port for PDCCH reception;

-	an indication for a presence or absence of a transmission configuration indication (TCI) field for a DCI format, other than DCI format 1_0, that schedules PDSCH receptions or has associated HARQ-ACK information without scheduling PDSCH and is provided by a PDCCH in CORESET p, by tci-PresentInDCI or tci-PresentDCI-1-2.

When precoderGranularity = allContiguousRBs, a UE does not expect

-	to be configured a set of resource blocks of a CORESET that includes more than four sub-sets of resource blocks that are not contiguous in frequency

-	any RE of a CORESET to overlap with any RE determined from lte-CRS-ToMatchAround, or from LTE-CRS-PatternList, or with any RE of a SS/PBCH block.

If a UE is provided two TCI states indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in a CORESET associated with a Type3-PDCCH CSS set, the UE may assume the quasi co-location information indicated in both of the two TCI states for the PDCCH reception in the CORESET.

For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap

-	if a CORESET is not associated with any search space set configured with freqMonitorLocations, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of  NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-Offset is not provided, or the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-Offset.

-	if a CORESET is associated with at least one search space set configured with freqMonitorLocations, the first NRBG,set0size  bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in each RB set k in the DL BWP bandwidth of  NRBBWP PRBs with starting common RB position RBs0+k,DLstart,μ  [6, TS 38.214], where the first common RB of the first group of 6 PRBs has common RB index RBs0+k,DLstart,μ+NRBoffset and k is indicated by freqMonitorLocations if provided for a search space set; otherwise, k=0. NRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP, and NRBoffset is provided by rb-Offset or NRBoffset=0 if rb-Offset is not provided. If a UE is provided RB sets in the DL BWP, the UE expects that the RBs of the CORESET are within the union of the PRBs in the RB sets of the DL BWP.

For a CORESET other than a CORESET with index 0,

-	if a UE has not been provided a configuration of TCI state(s) by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList for the CORESET, or has been provided initial configuration of more than one TCI states for the CORESET by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList but has not received a MAC CE activation command for one of the TCI states as described in [11, TS 38.321], the UE assumes that the DM-RS antenna port associated with PDCCH receptions is quasi co-located with the SS/PBCH block the UE identified during the initial access procedure;

-	if a UE has been provided a configuration of more than one TCI states by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList for the CORESET as part of Reconfiguration with sync procedure as described in [12, TS 38.331] but has not received a MAC CE activation command for one of the TCI states as described in [11, TS 38.321], the UE assumes that the DM-RS antenna port associated with PDCCH receptions is quasi co-located with the SS/PBCH block or the CSI-RS resource the UE identified during the random access procedure initiated by the Reconfiguration with sync procedure as described in [12, TS 38.331].

For a CORESET with index 0, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with

-	the one or more DL RS configured by a TCI state, where the TCI state is indicated by a MAC CE activation command for the CORESET, if any, or

-	a SS/PBCH block the UE identified during a most recent random access procedure not initiated by a PDCCH order that triggers a contention-free random access procedure, if no MAC CE activation command indicating a TCI state for the CORESET is received after the most recent random access procedure.

For a CORESET other than a CORESET with index 0, if a UE is provided a single TCI state for a CORESET, or if the UE receives a MAC CE activation command for one or two of the provided TCI states for a CORESET, the UE assumes that the DM-RS antenna port associated with PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by the TCI states. For a CORESET with index 0, the UE expects that a CSI-RS configured with qcl-Type set to 'typeD' in a TCI state indicated by a MAC CE activation command for the CORESET is provided by a SS/PBCH block

-	if the UE receives a MAC CE activation command for one of the TCI states, the UE applies the activation command in the first slot that is after slot k+3Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and μ is the SCS configuration for the PUCCH. The active BWP is defined as the active BWP in the slot when the activation command is applied.

If a UE is provided TCI-StateID_r17, a DM-RS antenna port for PDCCH receptions in a CORESET, other than a CORESET with index 0, associated only with USS sets and/or Type3-PDCCH CSS sets, and a DM-RS antenna port for PDSCH receptions scheduled by DCI formats provided by PDCCH receptions in the CORESET are quasi co-located with reference signals provided by the indicated TCI-State-r17 [6, TS 38.214].

If a UE is provided useIndicatedTCIState for a CORESET, other than a CORESET with index 0, associated only with CSS sets other than Type3-PDCCH CCS sets, and if useIndicatedTCIState is set as enabled, a DM-RS antenna port for PDCCH receptions in the CORESET and a DM-RS antenna port for PDSCH receptions scheduled by DCI formats provided by PDCCH receptions in the CORESET are quasi co-located with reference signals provided by the indicated TCI-state-r17.

If the UE is provided by simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2 up to two lists of cells for simultaneous TCI state activation, the UE applies the antenna port quasi co-location provided by one or two TCI-State, each with same activated tci-StateID value, to CORESETs with a same index in all configured DL BWPs of all configured cells in a list determined from a serving cell index, where one or two tci-StateID, the CORESET index, and the serving cell index are provided by a MAC CE command.

For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with S≤10  search space sets where, for each search space set from the S search space sets, the UE is provided the following by SearchSpace:

-	a search space set index s, 0<s<40 , by searchSpaceId

-	an association between the search space set s and a CORESET p by controlResourceSetId or by controlResourceSetId-v1610

-	a PDCCH monitoring periodicity of ks slots and a PDCCH monitoring offset of os slots, by monitoringSlotPeriodicityAndOffset

-	a PDCCH monitoring pattern within a slot, indicating first symbol(s) of the CORESET within a slot for PDCCH monitoring, by monitoringSymbolsWithinSlot

-	a duration of Ts<ks slots indicating a number of slots that the search space set s exists by duration

-	a number of PDCCH candidates Ms(L) per CCE aggregation level L by aggregationLevel1, aggregationLevel2, aggregationLevel4, aggregationLevel8, and aggregationLevel16, for CCE aggregation level 1, CCE aggregation level 2, CCE aggregation level 4, CCE aggregation level 8, and CCE aggregation level 16, respectively

-	an indication that search space set s is either a CSS set or a USS set by searchSpaceType

-	if search space set s is a CSS set

-	an indication by dci-Format0-0-AndFormat1-0 to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0

-	an indication by dci-Format2-0 to monitor one or two PDCCH candidates, or to monitor one PDCCH candidate per RB set if the UE is provided freqMonitorLocations for the search space set, for DCI format 2_0 and a corresponding CCE aggregation level

-	an indication by dci-Format2-1 to monitor PDCCH candidates for DCI format 2_1

-	an indication by dci-Format2-2 to monitor PDCCH candidates for DCI format 2_2

-	an indication by dci-Format2-3 to monitor PDCCH candidates for DCI format 2_3

-	an indication by dci-Format2-4 to monitor PDCCH candidates for DCI format 2_4

-	an indication by dci-Format2-6 to monitor PDCCH candidates for DCI format 2_6

-	a search space set index by searchSpaceLinking for another search space set that is linked to search space set s

-	if search space set s is a USS set, an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or an indication by dci-FormatsExt to monitor PDCCH candidates for DCI format 0_2 and DCI format 1_2, or for DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2, or an indication by dci-FormatsSL to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1

-	a bitmap by freqMonitorLocations, if provided, to indicate an index of one or more RB sets for the search space set s, where the MSB k in the bitmap corresponds to RB set k-1 in the DL BWP. For RB set k indicated in the bitmap, the first PRB of the frequency domain monitoring location confined within the RB set is given by RBs0+k,DLstart,μ+NRBoffset, where RBs0+k,DLstart,μ is the index of first common RB of the RB set k [6, TS 38.214], and NRBoffset is provided by rb-Offset or NRBoffset=0 if rb-Offset is not provided. For each RB set with a corresponding value of 1 in the bitmap, the frequency domain resource allocation pattern for the monitoring location is determined based on the first NRBG, set 0size bits in frequencyDomainResources provided by the associated CORESET configuration.

If the monitoringSymbolsWithinSlot indicates to a UE to monitor PDCCH in a subset of up to three consecutive symbols that are same in every slot where the UE monitors PDCCH for all search space sets, the UE does not expect to be configured with a PDCCH SCS other than 15 kHz if the subset includes at least one symbol after the third symbol.

A UE does not expect to be provided a first symbol and a number of consecutive symbols for a CORESET that results to a PDCCH candidate mapping to symbols of different slots.

A UE does not expect any two PDCCH monitoring occasions on an active DL BWP, for a same search space set or for different search space sets, in a same CORESET to be separated by a non-zero number of symbols that is smaller than the CORESET duration.

A UE determines a PDCCH monitoring occasion on an active DL BWP from the PDCCH monitoring periodicity, the PDCCH monitoring offset, and the PDCCH monitoring pattern within a slot. For search space set s, the UE determines that a PDCCH monitoring occasion(s) exists in a slot with number ns,fμ [4, TS 38.211] in a frame with number nf if (nf∙ Nslot frame, μ+ ns,f μ- os) modks=0nfNslotframe,μ+ns,fμ-op,smod kp,s=0. The UE monitors PDCCH candidates for search space set s for Ts consecutive slots, starting from slot ns,fμ, and does not monitor PDCCH candidates for search space set s for the next ks-Ts consecutive slots.

A USS at CCE aggregation level L∈1, 2, 4, 8, 16 is defined by a set of PDCCH candidates for CCE aggregation level L.

If a UE is configured with CrossCarrierSchedulingConfig for a serving cell the carrier indicator field value corresponds to the value indicated by CrossCarrierSchedulingConfig.

For an active DL BWP of a serving cell on which a UE monitors PDCCH candidates in a USS, if the UE is not configured with a carrier indicator field, the UE monitors the PDCCH candidates without carrier indicator field. For an active DL BWP of a serving cell on which a UE monitors PDCCH candidates in a USS, if a UE is configured with a carrier indicator field, the UE monitors the PDCCH candidates with carrier indicator field.

A UE does not expect to monitor PDCCH candidates on an active DL BWP of a secondary cell if the UE is configured to monitor PDCCH candidates with carrier indicator field corresponding to that secondary cell in another serving cell. For the active DL BWP of a serving cell on which the UE monitors PDCCH candidates, the UE monitors PDCCH candidates at least for the same serving cell.

For a search space set s associated with CORESET p, the CCE indexes for aggregation level L corresponding to PDCCH candidate ms,nCI(L)  ms,nCIof the search space set in slot ns,fμ for an active DL BWP of a serving cell corresponding to carrier indicator field value nCI are given by

L⋅Yp,ns,fμ+ms,nCI(L)ms,nCI⋅NCCE,pL⋅Ms,maxL+nCImodNCCE,pL+i

where

for any CSS, Yp,ns,fμ=0;

for a USS, Yp,ns,fμ=Ap⋅Yp,ns,fμ-1modD, Yp,-1=nRNTI≠0, Ap=39827 for pmod3=0, Ap=39829 for pmod3=1, Ap=39839 for pmod3=2, and D=65537;

i=0,⋯,L-1;

NCCE,p is the number of CCEs, numbered from 0 to NCCE,p-1, in CORESET p and, if any, per RB set;

nCI is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS, nCI=0;

ms,nCI(L)ms,nCI=0,⋯,Ms,nCIL-1, where Ms,nCIL is the number of PDCCH candidates the UE is configured to monitor for aggregation level L of a search space set s for a serving cell corresponding to nCI;

for any CSS, Ms,maxL=Ms,0L;

for a USS, Ms,maxL is the maximum of Ms,nCIL over all configured nCI values for a CCE aggregation level L of search space set s ;

the RNTI value used for nRNTI is the C-RNTI.

For search space sets si and sj that include searchSpaceLinking with value sj and si, respectively, a UE monitors, in monitoring occasions with same index according to each of search space sets si and sj in a slot, PDCCH candidates msi,nCI(L) and msj,nCI(L), with msi,nCI(L)=msj,nCI(L), for detection of a DCI format with same information. The UE expects ksi=ksj, osi=osj, Tsi=Tsj, Msi(L)=Msj(L), and a same number of non-overlapping PDCCH monitoring occasions per slot based on corresponding monitoringSymbolsWithinSlot, for search space sets si and sj. For CORESET pi associated with the search space set si and for CORESET pj associated with the search space set sj, the UE is provided tci-PresentInDCI or tci-PresentDCI-1-2 for either none or both of CORESETs pi and pj. For CORESET pi associated with the search space set si and for CORESET pj associated with the search space set sj, the UE is either not provided coresetPoolIndex value of 1 for any of the two CORESETs, or is provided coresetPoolIndex value of 1 for both CORESETs. The UE can indicate by countLinkedCandidates a capability for counting PDCCH candidates msi,nCI(L) and msj,nCI(L) either as 2 PDCCH candidates or as 3 PDCCH candidates.

For search space sets si, sj, sk, and sl, that include searchSpaceLinking with values sj, si, sl, and sk, respectively, a UE expects to simultaneously monitor PDCCH candidates msi,nCI,1(8)=msj,nCI,1(8), and msk,nCI,2(16)= msl,nCI,2(16) only if a first CCE of msi,nCI,1(8) or msj,nCI,1(8) has different index than a first CCE of msk,nCI,2(16) or msl,nCI,2(16) in a CORESET configured with cce-REG-MappingType = ‘nonInterleaved’ and with duration of one symbol.

If a UE

-	is provided monitoringCapabilityConfig = r16monitoringcapability for a downlink cell,

-	is provided, by searchSpaceLinking, for search space sets si and sj on the downlink cell respective values sj and si, and

-	indicates three-BDforSSsetLinking

the UE counts each PDCCH candidate for the one of the search space sets si and sj that the UE monitors PDCCH in the later span, as two PDCCH candidates.

A UE does not expect to be provided freqMonitorLocations for a search space set s in a serving cell if intraCellGuardBandsDL-List indicates that no intra-cell guard-bands are configured for the serving cell.

A UE that

-	is configured for operation with carrier aggregation, and

-	indicates support of search space sharing through searchSpaceSharingCA-UL or through searchSpaceSharingCA-DL, and

-	has a PDCCH candidate with CCE aggregation level L in CORESET p associated with search space set si for detection of a first DCI format, other than DCI format 0_0 or DCI format 1_0, having a first size and scheduling

-	PUSCH transmission or configured grant Type 2 PUSCH release on serving cell nCI,2, or

-	PDSCH reception or having associated HARQ-ACK information without scheduling PDSCH reception on serving cell nCI,2nCI,2

can receive a corresponding PDCCH through a PDCCH candidate with CCE aggregation level L in CORESET p associated with search space set sj for detection of a second DCI format having a second size and associated with scheduling on serving cell nCI,1 if the first size and the second size are same and if neither of search space sets si and sj includes searchSpaceLinking.

A UE expects to monitor PDCCH candidates for up to 4 sizes of DCI formats that include up to 3 sizes of DCI formats with CRC scrambled by C-RNTI per serving cell. The UE counts a number of sizes for DCI formats per serving cell based on a number of configured PDCCH candidates in respective search space sets for the corresponding active DL BWP.

A UE does not expect to detect, in a same PDCCH monitoring occasion, a DCI format with CRC scrambled by a SI-RNTI, RA-RNTI, MsgB-RNTI, TC-RNTI, P-RNTI, C-RNTI, CS-RNTI, or MCS-RNTI and a DCI format with CRC scrambled by a SL-RNTI or a SL-CS-RNTI for scheduling respective PDSCH reception and PSSCH transmission on a same serving cell.

A PDCCH candidate with index msj,nCI(L)msj,nCI for a search space set sj using a set of L CCEs in a CORESET p on the active DL BWP for serving cell nCI is not counted for monitoring if there is a PDCCH candidate with index msi,nCI(L)msi,nCI for a search space set si<sj, or if there is a PDCCH candidate with index nsj,nCI(L)nsj,nCI and nsj,nCI(L)nsj,nCI<msj,nCI(L)msj,nCI, in the CORESET p on the active DL BWP for serving cell nCI using a same set of L CCEs, the PDCCH candidates have identical scrambling, and the corresponding DCI formats for the PDCCH candidates have a same size; otherwise, the PDCCH candidate with index msj,nCI(L)msj,nCI is counted for monitoring.

For search space sets si and sj that include searchSpaceLinking with values sj and si, and for search space set sk that does not include searchSpaceLinking, when a UE

-	monitors PDCCH candidates msi,nCI,1(L)=msj,nCI,1(L) for detection of a first DCI format,

-	monitors PDCCH candidate msk,nCI,2(L) for detection of a second DCI format having a same size as the first DCI format,

-	the PDCCH candidate msi,nCI,1(L), or the PDCCH candidate msj,nCI,1(L), and the PDCCH candidate msk,nCI,2(L) have identical scrambling and use a same set of CCEs over same symbols in a slot in a CORESET p,

the PDCCH candidate msk,nCI,2(L) is not counted for monitoring and the UE assumes that a detected DCI format is the first DCI format. A UE may monitor PDCCH candidate msk,nCI,2(L) depending on a corresponding capability [16, TS 38.306].

For search space sets si and sj that include searchSpaceLinking with values sj and si, and for search space set sk that does not include searchSpaceLinking, when a UE

-	monitors PDCCH candidates msi,nCI,1(8)= msj,nCI,1(8) for detection of a first DCI format and monitors PDCCH candidate msk,nCI,2(16) for detection of a second DCI format, or monitors PDCCH candidates msi,nCI,1(16)=msj,nCI,1(16) for detection of the first DCI format and monitors PDCCH candidate mski,nCI,2(8) for detection of the second DCI format, and

-	one of the PDCCH candidates msi,nCI,1(8) and msj,nCI,1(8), and the PDCCH candidate msk,nCI,2(16), or one of the PDCCH candidates msi,nCI,1(16) and msj,nCI,1(16), and the PDCCH candidate msk,nCI,2(8), have a first CCE with same index and are simultaneously monitored in a CORESET p with cce-REG-MappingType = ‘nonInterleaved’ and duration of one symbol,

the UE assumes that a detected DCI format is the first DCI format.

For search space sets si, sj, sk, and sl that include searchSpaceLinking with values sj, si, sl, and sk, respectively, and for detection of DCI formats with same size, a UE expects different CCEs or different scrambling in a CORESET p for any of first PDCCH candidates msi,nCI,1(L) and msj,nCI,1(L), with msi,nCI,1(L)=msj,nCI,1(L), and any of second PDCCH candidates msk,nCI,2(L) and msl,nCI,2(L), with msk,nCI,2(L)=msl,nCI,2(L) that the UE would simultaneously monitor.

Table 10.1-2 provides the maximum number of monitored PDCCH candidates, MPDCCHmax,slot,μ, per slot for a UE in a DL BWP with SCS configuration μ for operation with a single serving cell.

Table 10.1-2: Maximum number MPDCCHmax,slot,μ of monitored PDCCH candidates per slot for a DL BWP with SCS configuration μ∈0, 1,2,3 for a single serving cell

| μ | Maximum number of monitored PDCCH candidates per slot and per serving cell MPDCCHmax,slot,μ |
| --- | --- |
| 0 | 44 |
| 1 | 36 |
| 2 | 22 |
| 3 | 20 |

Table 10.1-2A provides the maximum number of monitored PDCCH candidates, MPDCCHmax,X,Y,μ, per span for a UE in a DL BWP with SCS configuration μ for operation with a single serving cell.

Table 10.1-2A: Maximum number MPDCCHmax,X,Y,μ of monitored PDCCH candidates in a span for combination (X, Y) for a DL BWP with SCS configuration μ∈0, 1 for a single serving cell

|  | Maximum number MPDCCHmax,X,Y,μ of monitored PDCCH candidates per span for combination X,Y and per serving cell |  |  |
| --- | --- | --- | --- |
| μ | (2, 2) | (4, 3) | (7, 3) |
| 0 | 14 | 28 | 44 |
| 1 | 12 | 24 | 36 |

Table 10.1-3 provides the maximum number of non-overlapped CCEs, CPDCCHmax,slot,μ, for a DL BWP with SCS configuration μ that a UE is expected to monitor corresponding PDCCH candidates per slot for operation with a single serving cell.

CCEs for PDCCH candidates are non-overlapped if they correspond to

-	different CORESET indexes, or

-	different first symbols for the reception of the respective PDCCH candidates.

Table 10.1-3: Maximum number CPDCCHmax,slot,μ of non-overlapped CCEs per slot for a DL BWP with SCS configuration μ∈0, 1,2,3 for a single serving cell

| μ | Maximum number of non-overlapped CCEs per slot and per serving cell CPDCCHmax,slot,μ |
| --- | --- |
| 0 | 56 |
| 1 | 56 |
| 2 | 48 |
| 3 | 32 |

Table 10.1-3A provides the maximum number of non-overlapped CCEs, CPDCCHmax,X,Y,μ, for a DL BWP with SCS configuration μ that a UE is expected to monitor corresponding PDCCH candidates per span for operation with a single serving cell.

Table 10.1-3A: Maximum number CPDCCHmax,X,Y,μ of non-overlapped CCEs in a span for combination (X, Y) for a DL BWP with SCS configuration μ∈0, 1 for a single serving cell

|  | Maximum number CPDCCHmax,X,Y,μ of non-overlapped CCEs per span for combination X,Y and per serving cell |  |  |
| --- | --- | --- | --- |
| μ | (2, 2) | (4, 3) | (7, 3) |
| 0 | 18 | 36 | 56 |
| 1 | 18 | 36 | 56 |

If a UE

-	does not report pdcch-BlindDetectionCA or is not provided BDFactorR, γ=R

-	reports pdcch-BlindDetectionCA, the UE can be indicated by BDFactorR either γ=1 or γ=R

If a UE is configured with  Ncells,0DL,μ+Ncells,1DL,μ downlink cells for which the UE is not provided monitoringCapabilityConfig, or is provided monitoringCapabilityConfig = r15monitoringcapability but not provided CORESETPoolIndex, with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ where μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ≤Ncellscap, the UE is not required to monitor, on the active DL BWPs of the scheduling cells,

-	more than MPDCCHtotal,slot,μ=MPDCCHmax,slot,μ PDCCH candidates or more than CPDCCHtotal,slot,μ=CPDCCHmax,slot,μ non-overlapped CCEs per slot for each scheduled cell when the scheduling cell is from the Ncells,0DL,μ downlink cells, or

-	more than MPDCCHtotal,slot,μ=γ∙MPDCCHmax,slot,μ PDCCH candidates or more than CPDCCHtotal,slot,μ=γ∙CPDCCHmax,slot,μ non-overlapped CCEs per slot for each scheduled cell when the scheduling cell is from the Ncells,1DL,μ downlink cells

-	more than MPDCCHmax,slot,μ PDCCH candidates or more than CPDCCHmax,slot,μ non-overlapped CCEs per slot for CORESETs with same coresetPoolIndex value for each scheduled cell when the scheduling cell is from the Ncells,1DL,μ downlink cells

Ncellscap is replaced by Ncells,r15cap-r16, if a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability.

If a UE

-	is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells for which the UE is not provided monitoringCapabilityConfig, or is provided monitoringCapabilityConfig = r15monitoringcapability but not provided coresetPoolIndex,

-	with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cell(s) using SCS configuration μ, where μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ>Ncellscap, and

-	a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell,

the UE is not required to monitor more than MPDCCHtotal,slot,μ=Ncellscap⋅MPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j  PDCCH candidates or more than CPDCCHtotal,slot,μ=Ncellscap⋅CPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) from the Ncells,0DL,μ+Ncells,1DL,μ downlink cells. Ncellscap is replaced by Ncells,r15cap-r16 if a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability.

For each scheduled cell from the Ncells,0DL,μ downlink cells, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell more than minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot.

For each scheduled cell from the Ncells,1DL,μ downlink cells, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell

-	more than minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot

-	more than minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot for CORESETs with same coresetPoolIndex value

If a UE is configured with Ncells,r16DL,μ  downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, where μ=01Ncells,r16DL,μ≤Ncellscap-r16,  the UE is not required to monitor, on the active DL BWP of the scheduling cell, more than MPDCCHtotal,(X,Y),μ=MPDCCHmax,(X,Y),μ PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=CPDCCHmax,(X,Y),μ non-overlapped CCEs per span for each scheduled cell when the scheduling cell is from the Ncells,r16DL,(X,Y),μ downlink cells. If a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability, Ncellscap-r16 is replaced by Ncells,r16cap-r16.

If a UE is configured only with Ncells,r16DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs

-	per set of spans on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells within every X symbols, if the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells results to PDCCH monitoring according to the combination X,Y and any pair of spans in the set is within Y symbols, where first X symbols start at a first symbol with a PDCCH monitoring occasion and next X symbols start at a first symbol with a PDCCH monitoring occasion that is not included in the first X symbols

-	per set of spans across the active DL BWP(s) of all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells, with at most one span per scheduling cell for each set of spans, otherwise

where Ncells,r16DL,j is a number of configured cells with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration j. If a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability, Ncellscap-r16 is replaced by Ncells,r16cap-r16.

For each scheduled cell from the Ncells,r16DL,(X,Y),μ downlink cells using combination X,Y, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell, more than minMPDCCHmax,(X,Y),μ,MPDCCHtotal,(X,Y),μ PDCCH candidates or more than minCPDCCHmax,(X,Y),μ,CPDCCHtotal,(X,Y),μ non-overlapped CCEs per span.

A UE does not expect to be configured CSS sets that result to corresponding total, or per scheduled cell, numbers of monitored PDCCH candidates and non-overlapped CCEs per slot or per span that exceed the corresponding maximum numbers per slot or per span, respectively.

For same cell scheduling or for cross-carrier scheduling, a UE does not expect a number of PDCCH candidates, and a number of corresponding non-overlapped CCEs per slot or per span on a secondary cell to be larger than the corresponding numbers that the UE is capable of monitoring on the secondary cell per slot or per span, respectively. If a UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, except the first span of each slot, the UE does not expect a number of PDCCH candidates and a number of corresponding non-overlapped CCEs per span on the primary cell to be larger than the corresponding numbers that the UE is capable of monitoring on the primary cell per span.

For cross-carrier scheduling, the number of PDCCH candidates for monitoring and the number of non-overlapped CCEs per span or per slot are separately counted for each scheduled cell.

For all search space sets within a slot n or within a span in slot n, denote by Scss a set of CSS sets with cardinality of Icss and by Suss a set of USS sets with cardinality of Juss. The location of USS sets sj, 0≤j<Juss, in Suss is according to an ascending order of the search space set index.

Denote by MScss(i)(L), 0≤i<Icss, the number of counted PDCCH candidates for monitoring for CSS set Scss(i) and by MSuss(j)(L), 0≤j<Juss, the number of counted PDCCH candidates for monitoring for USS set Suss(j). If a UE indicates three-BDforSSsetLinking and is provided for search space set sj, by searchSpaceLinking, a search space set si with si<sj, set MScss(j)(L)=2∙MScss(j)(L) if si and sj are CSS sets or set MSuss(j)(L)=2∙MSuss(j)(L) if si and sj are USS sets.

For the CSS sets, a UE monitors MPDCCHcss=i=0Icss-1LMScss(i)(L) PDCCH candidates requiring a total of CPDCCHcss non-overlapping CCEs in a slot or in a span.

The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration μ in a slot if the UE is not provided monitoringCapabilityConfig for the primary cell or if the UE is provided monitoringCapabilityConfig = r15monitoringcapability for the primary cell, or in the first span of each slot if the UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, according to the following pseudocode. If for the USS sets for scheduling on the primary cell the UE is not provided coresetPoolIndex for first CORESETs, or is provided coresetPoolIndex with value 0 for first CORESETs, and is provided coresetPoolIndex with value 1 for second CORESETs, and if minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ>minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ or minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ>minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ, the following pseudocode applies only to USS sets associated with the first CORESETs. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring. In the following pseudocode, if the UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are replaced by MPDCCHmax,(X,Y),μ and CPDCCHmax,(X,Y),μ respectively, and MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ are replaced by MPDCCHtotal,(X,Y),μ and CPDCCHtotal,(X,Y),μ respectively.

Denote by VCCESuss(j) the set of non-overlapping CCEs for search space set Suss(j) and by CVCCESuss(j); the cardinality of VCCESuss(j) where a UE determines the non-overlapping CCEs for search space set Suss(j) considering the allocated PDCCH candidates for monitoring for the CSS sets and the allocated PDCCH candidates for monitoring for all search space sets Suss(k), 0≤k≤j.

Set MPDCCHuss=minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ-MPDCCHcss

Set CPDCCHuss=minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ-CPDCCHcss

Set j=0

while LMSuss(j)(L)≤MPDCCHuss AND CVCCESuss(j)≤CPDCCHuss

allocate LMSuss(j)(L) PDCCH candidates for monitoring to USS set Suss(j)

MPDCCHuss=MPDCCHuss-LMSuss(j)(L);

CPDCCHuss=CPDCCHuss-CVCCESuss(j);

j=j+1;

end while

If a UE

-	is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and

-	monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have been configured with same or different qcl-Type set to 'typeD' properties on active DL BWP(s) of one or more cells

the UE monitors PDCCHs only in a CORESET, and in any other CORESET from the multiple CORESETs that have been configured with qcl-Type set to same 'typeD' properties as the CORESET, on the active DL BWP of a cell from the one or more cells

-	the CORESET corresponds to the CSS set with the lowest index in the cell with the lowest index containing CSS, if any; otherwise, to the USS set with the lowest index in the cell with lowest index

-	the lowest USS set index is determined over all USS sets with at least one PDCCH candidate in overlapping PDCCH monitoring occasions

If a UE

-	is configured for single cell operation or for operation with carrier aggregation in a same frequency band,

-	monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have been configured with same or different qcl-Type set to 'typeD' properties on active DL BWP(s) of one or more cells, and

-	is provided two-QCLTypeDforPDCCHRepetition

the UE monitors PDCCHs only in a first CORESET with qcl-Type set to first 'typeD' properties and, if any, in a second CORESET with qcl-Type set to second 'typeD' properties that are different than the first 'typeD' properties, and in any other CORESET from the multiple CORESETs with corresponding qcl-Type set to the first 'typeD' properties and/or to the second 'typeD' properties

-	the first CORESET corresponds to the CSS set with the lowest index in the cell with the lowest index containing CSS sets, if any; otherwise, to the USS set with the lowest index in the cell with lowest index

-	excluding CSS sets and USS sets associated with CORESETs with qcl-Type set to first 'typeD' properties, the second CORESET corresponds to the CSS set with the lowest index in the cell with the lowest index containing CSS sets; if any; otherwise, to the USS set with the lowest index in the cell with lowest index, where the CSS set or the USS set includes searchSpaceLinking with a value indicating, respectively, any CSS set or any USS set associated with CORESETs with qcl-Type set to first 'typeD' properties

-	the lowest USS set index is determined over all USS sets with at least one PDCCH candidate in overlapping PDCCH monitoring occasions

If a UE

-	is configured for single cell operation or for operation with carrier aggregation in a same frequency band,

-	monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have been configured with same or different qcl-Type set to 'typeD' properties on active DL BWP(s) of one or more cells,

-	one or more CORESETs have two activated TCI states, and

-	reports twoTypeDcapabilityname

the UE monitors PDCCHs only in a CORESET with a first qcl-Type set to first 'typeD' properties and, if any, a second qcl-Type set to second 'typeD' properties that are different than the first 'typeD' properties, and in any other CORESET from the multiple CORESETs with corresponding qcl-Type set to the first 'typeD' properties or to the second 'typeD' properties

-	the CORESET corresponds to the CSS set with the lowest index in the cell with the lowest index containing CSS, if any; otherwise, to the USS set with the lowest index in the cell with lowest index

-	the lowest USS set index is determined over all USS sets with at least one PDCCH candidate in overlapping PDCCH monitoring occasions

For	 the purpose of determining the CORESET, a SS/PBCH block is considered to have different QCL 'typeD' properties than a CSI-RS.

For 	the purpose of determining the CORESET, a first CSI-RS associated with a SS/PBCH block in a first cell and a second CSI-RS in a second cell that is also associated with the SS/PBCH block are assumed to have same QCL 'typeD' properties.

The 	allocation of non-overlapping CCEs and of PDCCH candidates for PDCCH monitoring is according to all search space sets associated with the multiple CORESETs on the active DL BWP(s) of the one or more cells.

The 	number of active TCI states is determined from the multiple CORESETs.

If a UE

-	is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and

-	monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs where none of the CORESETs has TCI-states configured with qcl-Type set to 'typeD',

the UE is required to monitor PDCCH candidates in overlapping PDCCH monitoring occasions for search space sets associated with different CORESETs.

For a scheduled cell and at any time, a UE expects to have received at most 16 PDCCHs for DCI formats with CRC scrambled by C-RNTI, CS-RNTI, or MCS-C-RNTI scheduling 16 PDSCH receptions for which the UE has not received any corresponding PDSCH symbol and at most 16 PDCCHs for DCI formats with CRC scrambled by C-RNTI, CS-RNTI, or MCS-C-RNTI scheduling 16 PUSCH transmissions for which the UE has not transmitted any corresponding PUSCH symbol.

If a UE is not provided monitoringCapabilityConfig = r16monitoringcapability for any serving cell, and

-	is not configured for NR-DC operation and indicates through pdcch-BlindDetectionCA a capability to monitor PDCCH candidates for Ncellscap≥4 downlink cells and the UE is configured with NcellsDL>4 downlink cells or NcellsUL>4 uplink cells, or

-	is configured with NR-DC operation and for a cell group with NcellsDL downlink cells or NcellsUL uplink cells

the UE expects to have respectively received at most 16∙Ncellscap PDCCHs for

-	DCI formats with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-C-RNTI scheduling 16∙Ncellscap PDSCH receptions for which the UE has not received any corresponding PDSCH symbol over all NcellsDL downlink cells

-	DCI formats with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-C-RNTI scheduling 16∙Ncellscap PUSCH transmissions for which the UE has not transmitted any corresponding PUSCH symbol over all NcellsUL uplink cells

If a UE is provided monitoringCapabilityConfig = r16monitoringcapability for all serving cells, and

-	is not configured for NR-DC operation and indicates through pdcch-MonitoringCA a capability to monitor PDCCH candidates for Ncellscap-r16≥2 downlink cells and the UE is configured with NcellsDL>2 downlink cells or NcellsUL>2 uplink cells, or

-	is configured with NR-DC operation and for a cell group with NcellsDL downlink cells or NcellsUL uplink cells

the UE expects to have respectively received at most 16∙Ncellscap-r16 PDCCHs for

-	DCI formats with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-C-RNTI scheduling 16∙Ncellscap-r16 PDSCH receptions for which the UE has not received any corresponding PDSCH symbol over all NcellsDL downlink cells

-	DCI formats with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-C-RNTI scheduling 16∙Ncellscap-r16 PUSCH transmissions for which the UE has not transmitted any corresponding PUSCH symbol over all NcellsUL uplink cells.

If a UE is provided monitoringCapabilityConfig = r16monitoringcapability for at least one serving cell and is not provided monitoringCapabilityConfig = r16monitoringcapability for at least one serving cell, and

-	is not configured for NR-DC operation, and indicates a capability to monitor PDCCH candidates for Ncells,r15cap-r16≥1 downlink cells and Ncells,r16cap-r16≥1 downlink cells, and the UE is configured with  NcellsDL>1 downlink cell or NcellsUL>1 uplink cell, or

-	is configured with NR-DC operation and for a cell group with NcellsDL downlink cells or NcellsUL uplink cells

the UE expects to have respectively received

-	at most 16∙Ncells,r15cap-r16 PDCCHs for DCI formats with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-C-RNTI scheduling 16∙Ncells,r15cap-r16 PDSCH receptions for which the UE has not received any corresponding PDSCH symbol over all serving cells that are not provided monitoringCapabilityConfig = r16monitoringcapability

-	at most 16∙Ncells,r15cap-r16 PDCCHs for DCI formats with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-C-RNTI scheduling 16∙Ncells,r15cap-r16 PUSCH transmissions for which the UE has not transmitted any corresponding PUSCH symbol over all serving cells that are not provided monitoringCapabilityConfig = r16monitoringcapability

-	at most 16∙Ncells,r16cap-r16 PDCCHs for DCI formats with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-C-RNTI scheduling 16∙Ncells,r16cap-r16 PDSCH receptions for which the UE has not received any corresponding PDSCH symbol over all serving cells that are provided monitoringCapabilityConfig = r16monitoringcapability

-	at most 16∙Ncells,r16cap-r16 PDCCHs for DCI formats with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-C-RNTI scheduling 16∙Ncells,r16cap-r16 PUSCH transmissions for which the UE has not transmitted any corresponding PUSCH symbol over all serving cells that are provided monitoringCapabilityConfig = r16monitoringcapability

If a UE

-	is configured to monitor a first PDCCH candidate for a DCI format 0_0 and a DCI format 1_0 from a CSS set and a second PDCCH candidate for a DCI format 0_0 and a DCI format 1_0 from a USS set, where the CSS set and the USS set do not include searchSpaceLinking, in a CORESET with index zero on an active DL BWP, and

-	the DCI formats 0_0/1_0 associated with the first PDCCH candidate and the DCI formats 0_0/1_0 associated with the second PDCCH candidate have same size, and

-	the UE receives the first PDCCH candidate and the second PDCCH candidate over a same set of CCEs, and

-	the first PDCCH candidate and the second PDCCH candidate have identical scrambling, and

-	the DCI formats 0_0/1_0 for the first PDCCH candidate and the DCI formats 0_0/1_0 for the second PDCCH candidate have CRC scrambled by either C-RNTI, or MCS-C-RNTI, or CS-RNTI

the UE decodes only the DCI formats 0_0/1_0 associated with the first PDCCH candidate.

If a UE detects a DCI format with inconsistent information, the UE discards all the information in the DCI format.

A UE configured with a bandwidth part indicator in a DCI format determines, in case of an active DL BWP or of an active UL BWP change, that the information in the DCI format is applicable to the new active DL BWP or UL BWP, respectively, as described in clause 12.

For unpaired spectrum operation, if a UE is not configured for PUSCH/PUCCH transmission on serving cell c2, the UE does not expect to monitor PDCCH on serving cell c1 if the PDCCH overlaps in time with SRS transmission (including any interruption due to uplink or downlink RF retuning time [10, TS 38.133]) on serving cell c2 and if the UE is not capable of simultaneous reception and transmission on serving cell c1and serving cell c2.

If a UE is provided resourceBlocks and symbolsInResourceBlock in RateMatchPattern, or if the UE is additionally provided periodicityAndPattern in RateMatchPattern, the UE can determine a set of RBs in symbols of a slot that are not available for PDSCH reception as described in [6, TS 38.214]. If a PDCCH candidate in a slot is mapped to one or more REs that overlap with REs of any RB in the set of RBs in symbols of the slot, the UE does not expect to monitor the PDCCH candidate.

A UE does not expect to be configured with dci-FormatsSL and dci-FormatsExt in a same USS.

## *** Unchanged text is omitted ***

# 11	UE-group common signalling

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG.

In the remaining of this clause, unless stated otherwise, when a PDCCH reception by a UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1

-	a PDCCH monitoring occasion is the union of the PDCCH monitoring occasions for the two PDCCH candidates

-	the start of the PDCCH reception is the start of the earlier PDCCH candidate

-	the end of the PDCCH reception in the end of the PDCCH candidate that ends later

The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10, 11.1, and 11.1.1.

## *** Unchanged text is omitted ***

## 11.2	Interrupted transmission indication

If a UE is provided DownlinkPreemption, the UE is configured with an INT-RNTI provided by int-RNTI for monitoring PDCCH conveying DCI format 2_1 [5, TS 38.212]. The UE is additionally configured with

-	a set of serving cells by int-ConfigurationPerServingCell that includes a set of serving cell indexes provided by corresponding servingCellId and a corresponding set of locations for fields in DCI format 2_1 by positionInDCI

-	an information payload size for DCI format 2_1 by dci-PayloadSize

-	an indication granularity for time-frequency resources by timeFrequencySet

If a UE detects a DCI format 2_1 for a serving cell from the configured set of serving cells, the UE may assume that no transmission to the UE is present in PRBs and in symbols that are indicated by the DCI format 2_1, from a set of PRBs and a set of symbols of the last monitoring period. The indication by the DCI format 2_1 is not applicable to receptions of SS/PBCH blocks.

The set of PRBs is equal to the active DL BWP as defined in clause 12 and includes  PRBs.

If a UE detects a DCI format 2_1 in a PDCCH reception in a slot, the set of symbols is the last  symbols prior to the first symbol of the PDCCH reception in the slot where  is the PDCCH monitoring periodicity provided by the value of monitoringSlotPeriodicityAndOffset, as described in clause 10.1,  is the number of symbols per slot,  is the SCS configuration for a serving cell with mapping to a respective field in the DCI format 2_1,  is the SCS configuration of the DL BWP where the UE receives the PDCCH with the DCI format 2_1. If the UE is provided tdd-UL-DL-ConfigurationCommon, symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon are excluded from the last  symbols prior to the first symbol of the PDCCH reception in the slot. The resulting set of symbols includes a number of symbols that is denoted as .

The UE does not expect to be provided values of , , and  resulting to a value of  that is not an integer. The UE does not expect to be configured by monitoringSymbolsWithinSlot with more than one PDCCH monitoring occasion for DCI format 2_1 in a slot.

A UE is provided the indication granularity for the set of PRBs and for the set of symbols by timeFrequencySet.

If the value of timeFrequencySet is 'set0', 14 bits from MSB of a field in DCI format 2_1 have a one-to-one mapping with 14 groups of consecutive symbols from the set of symbols where each of the first  symbol groups includes  symbols, each of the last  symbol groups includes  symbols, a bit value of 0 indicates transmission to the UE in the corresponding symbol group and a bit value of 1 indicates no transmission to the UE in the corresponding symbol group.

If the value of timeFrequencySet is 'set1', 7 pairs of bits from MSB of a field in the DCI format 2_1 have a one-to-one mapping with 7 groups of consecutive symbols where each of the first  symbol groups includes  symbols, each of the last  symbol groups includes  symbols, a first bit in a pair of bits for a symbol group is applicable to the subset of first  PRBs from the set of  PRBs, a second bit in the pair of bits for the symbol group is applicable to the subset of last  PRBs from the set of  PRBs, a bit value of 0 indicates transmission to the UE in the corresponding symbol group and subset of PRBs, and a bit value of 1 indicates no transmission to the UE in the corresponding symbol group and subset of PRBs.

## 11.2A	Cancellation indication

If a UE is provided UplinkCancellation, the UE is provided, in one or more serving cells, search space sets for monitoring the first PDCCH candidate with a CCE aggregation level of LCI CCEs of each search space set for detection of a DCI format 2_4 [5, TS 38.212] with a CI-RNTI provided by ci-RNTI as described in clause 10.1. UplinkCancellation additionally provides to the UE

-	a set of serving cells, by ci-ConfigurationPerServingCell, that includes a set of serving cell indexes and a corresponding set of locations for fields in DCI format 2_4 by positionInDCI

-	a number of fields in DCI format 2_4, by positionInDCI-forSUL, for each serving cell for a SUL carrier, if the serving cell is configured with a SUL carrier

-	an information payload size for DCI format 2_4 by dci-PayloadSize-ForCI

-	an indication for time-frequency resources by timeFrequencyRegion

For a serving cell having an associated field in a DCI format 2_4, for the field denote by

-	NCI a number of bits provided by ci-PayloadSize

-	BCI a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion

-	TCI a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon, from a number of symbols that

-	is provided by timeDurationforCI in timeFrequencyRegion, if the PDCCH monitoring periodicity for the search space set with the DCI format 2_4 is one slot and there are more than one PDCCH monitoring occasions in a slot, or

-	is equal to the PDCCH monitoring periodicity, otherwise.

-	GCI a number of partitions for the TCI symbols provided by timeGranularityforCI in timeFrequencyRegion

GCI sets of bits from the MSB of the NCI bits have a one-to-one mapping with GCI groups of symbols where each of the first GCI-TCI+TCIGCI⋅GCI groups includes TCIGCI symbols and each of the remaining TCI-TCIGCI⋅GCI groups includes TCIGCI symbols. A UE determines a symbol duration with respect to a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

For a group of symbols, NBI=NCIGCI bits from MSB of each set of bits have a one-to-one mapping with NBI groups of PRBs where each of the first NBI-BCI+BCINBI⋅NBI groups includes BCINBI PRBs and each of the remaining BCI-BCINBI⋅NBI groups includes BCINBI PRBs. A UE determines a first PRB index as NRFRstart=Ocarrier+RBstart and a number of contiguous RBs as BCI=LRB  from frequencyRegionforCI that indicates an offset RBstart and a length LRB as RIV according to [6, TS 38.214], and from offsetToCarrier in FrequencyInfoUL-SIB or FrequencyInfoUL that indicates Ocarrier for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or an SRS transmission on the serving cell. If the PUSCH transmission or the SRS transmission is scheduled by a DCI format, the indication by the DCI format 2_4 is applicable to the PUSCH transmission or SRS transmission only if the last symbol of the PDCCH reception providing the DCI format is earlier than the first symbol of the PDCCH reception providing the DCI format 2_4.

For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after T'proc,2 from the end of a PDCCH reception where the UE detects the DCI format 2_4, where T'proc,2 is obtained from Tproc,2 for PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=doffset⋅2-μUL2-μ where doffset is provided by delta_Offset, μ being the smallest SCS configuration between the SCS configuration of the PDCCH and the smallest SCS configuration μUL provided in scs-SpecificCarrierList of FrequencyInfoUL or FrequencyInfoUL-SIB. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that is Tproc,2 assuming that d2,1=0 after a last symbol of the PDCCH reception where the UE detects the DCI format 2_4.

A UE that detects a DCI format 2_4 for a serving cell cancels a PUSCH transmission or an actual repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with repetition Type B, as determined in clauses 9 and 9.2.5 or in clause 6.1 of [6, TS 38.214], or an SRS transmission on the serving cell if, respectively,

-	the transmission is PUSCH with priority 0, if the UE is provided uplinkCancellationPriority,

-	a group of symbols, from the TCI symbols, has at least one bit value of '1' in the corresponding set of NBI bits in the DCI format 2_4 and includes a symbol of the (repetition of the) PUSCH transmission or of the SRS transmission, and

-	a group of PRBs, from the BCI PRBs, has a corresponding bit value of '1' in the set of bits corresponding to the group of symbols in the DCI format 2_4 and includes a PRB of the (repetition of the) PUSCH transmission or of the SRS transmission,

where

-	the cancellation of the (repetition of the) PUSCH transmission includes all symbols from the earliest symbol of the (repetition of the) PUSCH transmission that is in a group of symbols having corresponding bit values of '1' in the DCI format 2_4;

-	the cancellation of the SRS transmission includes only symbols that are in one or more groups of symbols having corresponding bit values of '1' in the DCI format 2_4.

If, based on an indication by a DCI format 2_4, a UE cancels a PUSCH transmission or an SRS transmission, the UE does not expect to be scheduled by a second DCI format to transmit a PUSCH or an SRS over symbols that include symbols of the cancelled PUSCH transmission or SRS transmission, where the last symbol of the PDCCH reception providing the second DCI format is later than the first symbol of the PDCCH reception providing the DCI format 2_4.

## *** Unchanged text is omitted ***

# 12	Bandwidth part operation

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG.

A UE configured for operation in bandwidth parts (BWPs) of a serving cell, is configured by higher layers for the serving cell a set of at most four bandwidth parts (BWPs) for receptions by the UE (DL BWP set) in a DL bandwidth by parameter BWP-Downlink or by parameter initialDownlinkBWP with a set of parameters configured by BWP-DownlinkCommon and BWP-DownlinkDedicated, and a set of at most four BWPs for transmissions by the UE (UL BWP set) in an UL bandwidth by parameter BWP-Uplink or by parameter initialUplinkBWP with a set of parameters configured by BWP-UplinkCommon and BWP-UplinkDedicated.

For operation with shared spectrum channel access, a UE expects that the BWP configured by the parameter initialUplinkBWP provided in UplinkConfigCommonSIB is mapped to only a single RB set.

If a UE is not provided initialDownlinkBWP, an initial DL BWP is defined by a location and number of contiguous PRBs, starting from a PRB with the lowest index and ending at a PRB with the highest index among PRBs of a CORESET for Type0-PDCCH CSS set, and a SCS and a cyclic prefix for PDCCH reception in the CORESET for Type0-PDCCH CSS set; otherwise, the initial DL BWP is provided by initialDownlinkBWP. For operation on the primary cell or on a secondary cell, a UE is provided an initial UL BWP by initialUplinkBWP. If the UE is configured with a supplementary UL carrier, the UE can be provided an initial UL BWP on the supplementary UL carrier by initialUplinkBWP.

If a UE has dedicated BWP configuration, the UE can be provided by firstActiveDownlinkBWP-Id a first active DL BWP for receptions and by firstActiveUplinkBWP-Id a first active UL BWP for transmissions on a carrier of the primary cell.

For each DL BWP or UL BWP in a set of DL BWPs or UL BWPs, respectively, the UE is provided the following parameters for the serving cell as defined in [4, TS 38.211] or [6, TS 38.214]:

-	a SCS by subcarrierSpacing

-	a cyclic prefix by cyclicPrefix

-	a common RB NBWPstart=Ocarrier+RBstart and a number of contiguous RBs NBWPsize=LRB provided by locationAndBandwidth that indicates an offset RBstart and a length LRB as RIV according to [6, TS 38.214], setting NBWPsize=275, and a value Ocarrier provided by offsetToCarrier for the subcarrierSpacing

-	an index in the set of DL BWPs or UL BWPs by respective BWP-Id

-	a set of BWP-common and a set of BWP-dedicated parameters by BWP-DownlinkCommon and BWP-DownlinkDedicated for the DL BWP, or BWP-UplinkCommon and BWP-UplinkDedicated for the UL BWP [12, TS 38.331]

For unpaired spectrum operation, a DL BWP from the set of configured DL BWPs with index provided by BWP-Id is linked with an UL BWP from the set of configured UL BWPs with index provided by BWP-Id when the DL BWP index and the UL BWP index are same. For unpaired spectrum operation, a UE does not expect to receive a configuration where the center frequency for a DL BWP is different than the center frequency for an UL BWP when the BWP-Id of the DL BWP is same as the BWP-Id of the UL BWP.

For each DL BWP in a set of DL BWPs of the PCell, a UE can be configured CORESETs for every type of CSS sets and for USS as described in clause 10.1. The UE does not expect to be configured without a CSS set on the PCell in the active DL BWP.

If a UE is provided controlResourceSetZero and searchSpaceZero in PDCCH-ConfigSIB1 or PDCCH-ConfigCommon, the UE determines a CORESET for a search space set from controlResourcesetZero as described in clause 13 and for Tables 13-1 through 13-10, and determines corresponding PDCCH monitoring occasions as described in clause 13 and for Tables 13-11 through 13-15. If the active DL BWP is not the initial DL BWP, the UE determines PDCCH monitoring occasions for the search space set only if the CORESET bandwidth is within the active DL BWP and the active DL BWP has same SCS configuration and same cyclic prefix as the initial DL BWP.

For each UL BWP in a set of UL BWPs of the PCell or of the PUCCH-SCell, the UE is configured resource sets for PUCCH transmissions as described in clause 9.2.1.

A UE receives PDCCH and PDSCH in a DL BWP according to a configured SCS and CP length for the DL BWP. A UE transmits PUCCH and PUSCH in an UL BWP according to a configured SCS and CP length for the UL BWP.

If a bandwidth part indicator field is configured in a DCI format, the bandwidth part indicator field value indicates the active DL BWP, from the configured DL BWP set, for DL receptions as described in [5, TS 38.212]. If a bandwidth part indicator field is configured in a DCI format, the bandwidth part indicator field value indicates the active UL BWP, from the configured UL BWP set, for UL transmissions as described in [5, TS 38.212]. If a bandwidth part indicator field is configured in a DCI format and indicates an UL BWP or a DL BWP different from the active UL BWP or DL BWP, respectively, the UE shall

-	for each information field in the DCI format

-	if the size of the information field is smaller than the one required for the DCI format interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, the UE prepends zeros to the information field until its size is the one required for the interpretation of the information field for the UL BWP or DL BWP prior to interpreting the DCI format information fields, respectively

-	if the size of the information field is larger than the one required for the DCI format interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, the UE uses a number of least significant bits of the DCI format equal to the one required for the UL BWP or DL BWP indicated by bandwidth part indicator prior to interpreting the DCI format information fields, respectively

-	set the active UL BWP or DL BWP to the UL BWP or DL BWP indicated by the bandwidth part indicator in the DCI format

If a bandwidth part indicator field is configured in a DCI format 0_1 and indicates an active UL BWP with different SCS configuration μ, or with different number NRB-set,ULBWP of RB sets, than a current active UL BWP, the UE determines an uplink frequency domain resource allocation Type 2 based on X' bits and Y' bits that are generated by independently truncating or padding the X MSBs and the Y LSBs [6, TS 38.214] of the frequency domain resource assignment field of DCI format 0_1, where truncation starts from the MSBs of the X bits or the Y bits, zero-padding prepends zeros to the X bits or the Y bits, and

-	if the indicated active UL BWP has SCS configuration μ=1 and the current active BWP has SCS configuration μ=0, the X MSBs are truncated to X'=X-1 bits, or

-	if the indicated active UL BWP has SCS configuration μ=0 and the current active BWP has SCS configuration μ=1, the X MSBs are zero-padded to X'=X+1 bits

-	otherwise, the X MSBs are unchanged

and

-	the Y LSBs are truncated or zero-padded to Y'=log2NRB-set,ULBWPNRB-set,ULBWP+12 bits where NRB-set,ULBWP is a number of RB sets configured for the indicated active UL BWP

A UE does not expect to detect a DCI format with a BWP indicator field that indicates an active DL BWP or an active UL BWP change with the corresponding time domain resource assignment field providing a slot offset value for a PDSCH reception or PUSCH transmission that is smaller than a delay required by the UE for an active DL BWP change or UL BWP change, respectively [10, TS 38.133].

If a UE detects a DCI format with a BWP indicator field that indicates an active DL BWP change for a cell, the UE is not required to receive or transmit in the cell during a time duration from the end of the third symbol of a slot where the UE receives the PDCCH that includes the DCI format in a scheduling cell until the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI format.

If a UE detects a DCI format with SCell dormancy indication that indicates an active DL BWP change for an Scell in slot n of primary cell, the UE is not required to receive or transmit in the SCell during a time duration specified in [10, TS 38.133].

If a UE detects a DCI format indicating an active UL BWP change for a cell, the UE is not required to receive or transmit in the cell during a time duration from the end of the third symbol of a slot where the UE receives the PDCCH that includes the DCI format in the scheduling cell until the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI format.

A UE does not expect to detect a DCI format indicating an active DL BWP change or an active UL BWP change for a scheduled cell within FR1 (or FR2) in a slot other than the first slot of a set of slots for the DL SCS of the scheduling cell that overlaps with a time duration where the UE is not required to receive or transmit, respectively, for an active BWP change in a different cell from the scheduled cell within FR1 (or FR2).

A UE expects to detect a DCI format with a BWP indicator field that indicates an active UL BWP change or an active DL BWP change only if a corresponding PDCCH is received within the first 3 symbols of a slot. If the UE detects the DCI format from two PDCCH receptions in search space sets si and sj that include searchSpaceLinking with value sj and si, respectively, as described in clause 10.1, the UE considers the PDCCH reception where the UE detects the DCI format to be the one from the two PDCCH receptions that ends later.

For a serving cell, a UE can be provided by defaultDownlinkBWP-Id a default DL BWP among the configured DL BWPs. If a UE is not provided a default DL BWP by defaultDownlinkBWP-Id, the default DL BWP is the initial DL BWP.

If a UE is provided by bwp-InactivityTimer a timer value for the serving cell [11, TS 38.321] and the timer is running, the UE decrements the timer at the end of a subframe for FR1 or at the end of a half subframe for FR2 if the restarting conditions in [11, TS 38.321] are not met during the interval of the subframe for FR1 or of the half subframe for FR2.

For a cell where a UE changes an active DL BWP due to a BWP inactivity timer expiration and for accommodating a delay in the active DL BWP change or the active UL BWP change required by the UE [10, TS 38.133], the UE is not required to receive or transmit in the cell during a time duration from the beginning of a subframe for FR1, or of half of a subframe for FR2, that is immediately after the BWP inactivity timer expires until the beginning of a slot where the UE can receive or transmit.

When a UE's BWP inactivity timer for a cell within FR1 (or FR2) expires within a time duration where the UE is not required to receive or transmit for an active UL/DL BWP change in the cell or in a different cell within FR1 (or FR2), the UE delays the active UL/DL BWP change triggered by the BWP inactivity timer expiration until a subframe for FR1 or half a subframe for FR2 that is immediately after the UE completes the active UL/DL BWP change in the cell or in the different cell within FR1 (or FR2).

If a UE is provided by firstActiveDownlinkBWP-Id a first active DL BWP and by firstActiveUplinkBWP-Id a first active UL BWP on a carrier of a secondary cell, the UE uses the indicated DL BWP and the indicated UL BWP as the respective first active DL BWP on the secondary cell and first active UL BWP on the carrier of the secondary cell.

A UE does not expect to monitor PDCCH when the UE performs RRM measurements [10, TS 38.133] over a bandwidth that is not within the active DL BWP for the UE.

## *** Unchanged text is omitted ***