3GPP TSG RAN WG1 #107bis-e		                                                                             R1-2200138

e-Meeting, January 17th – 25th, 2022

Source:	CATT

Title:	UE features for Rel-17 IIoT/URLLC

Agenda Item:	8.15.3

Document for:	Discussion and Decision

Introduction

In RAN1#107-e, updated Rel-17 UE feature lists in R1-2112902 [1] for NR is endorsed.

In this contribution, we discuss remaining issues of UE features for Rel-17 IIoT/URLLC.

Discussion

- FG 25-3b

| 25-3b | Inter-subslot frequency hopping for PUCCH repetitions | 1. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Formats 0, 1, 2, 3 and 4 for 7OS slot-based PUCCH configurations. 2. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS slot-based PUCCH configurations | TBD | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The prerequisite feature group for FG25-3b is TBD, which should be support of sub-slot based PUCCH repetition in our view. Accordingly, it is proposed that the prerequisite feature group for FG25-3b is 25-3.

Proposal 1: The prerequisite feature group for FG25-3b is 25-3.

- FG25-16

| 25-16 | HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] 4. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 5. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 6. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 7. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to separate capability for different UCI type | 11-3, 12-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

There is an FFS for FG25-16 on whether to separate capability for different UCI type. We do not think separate capability for different UCI type is needed. Therefore, it is proposed to delete the FFS.

Proposal 2: Separate capability for different UCI type for FG25-16 is not supported and the corresponding FFS is removed.

- FG25-18

| 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The following conclusions were made in RAN1#107-e. So the FG25-18 is proposed to be updated according to the conclusions.

Conclusion There is no consensus in RAN1 to support simultaneous PUCCH/PUSCH transmission of same priority over different cells in Rel-17. Conclusion There is no consensus in RAN1 to support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA in Rel-17.

Proposal 3: Adopt the following update to FG25-18.

| 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions of different priorities on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG25-19/25-19a

| 25-19 | RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS FFS whether to merge FG 25-19 and FG 25-19a | 2-51, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25-19a | RTT-based Propagation delay compensation based on DL PRS and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS FFS whether to merge FG 25-19 and FG 25-19a | 25-19, 13-1, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A | Note: FG 13-1 is now only reported to LMF. If UE reports the support of this FG, it needs to report FG 13-1 to gNB also. | Optional with capability signaling |

For FG25-19 and 25-19a, there is an FFS on whether to merge the two FGs. The difference is that RTT-based PDC is based on CSI-RS for FG25-19 and based on PRS for FG25-19a. We do not have a strong view but think it would be preferable if the two FGs can be merged.

Proposal 4: It is preferable that FG25-19 and FG25-19 are merged.

Conclusion

In this contribution, we discussed remaining issues of UE features for Rel-17 IIoT/URLLC with the following proposals.

Proposal 1: The prerequisite feature group for FG25-3b is 25-3.

Proposal 2: Separate capability for different UCI type for FG25-16 is not supported and the corresponding FFS is removed.

Proposal 3: Adopt the following update to FG25-18.

| 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions of different priorities on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4: It is preferable that FG25-19 and FG25-19 are merged.

References

- R1-2112902, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#107-e

- R1-2112135, Summary on UE features for enhanced IIoT and and URLLC, Moderator (NTT DOCOMO, INC.), RAN1#107-e