3GPP TSG RAN WG1 #107bis-e			R1-2200060

e-Meeting, January 17th – 25th, 2022

Agenda Item:	8.2.2

Source:	InterDigital, Inc.

Title:	Remaining issues for PDCCH monitoring enhancements

Document for:	Discussion and Decision

# Introduction

In RAN1#107-e [1], the following agreements on PDCCH monitoring enhancements were made:

Agreement - The following limits apply for multi-slot monitoring within the group of X slots - The maximum number of monitored PDCCH candidates per X=4 slots for a DL BWP with 480 kHz SCS configuration for a single serving cell is 20. - The maximum number of monitored PDCCH candidates per X=8 slots for a DL BWP with 960 kHz SCS configuration for a single serving cell is 20. - The maximum number of non-overlapped CCEs per X=4 slots for a DL BWP with 480 kHz SCS configuration for a single serving cell is 32. - The maximum number of non-overlapped CCEs per X=8 slots for a DL BWP with 960 kHz SCS configuration for a single serving cell is 32. - Note: Search spaces monitoring within the group of X slots will be additionally defined Agreement - For Group (1) SS: Type 1 CSS with dedicated RRC configuration and type 3 CSS, UE specific SS - A SS is monitored within Y consecutive slots within a slot group of X slots - The Y consecutive slots can be located anywhere within the slot group of X slots - Note: There is no requirement to align the Y consecutive slots across UEs or with slot n0 - The location of the Y consecutive slots within the slot group of X slots is maintained across different slot groups - BD attempts for all Group (1) SSs are restricted to fall within the same Y consecutive slots - For Group (2) SS: Type 1 CSS without dedicated RRC configuration and type 0, 0A, and 2 CSS - SS monitoring locations can be anywhere within a slot group of X slots, with the following exception - BD attempts for Type0-CSS for SSB/CORESET 0 multiplexing pattern 1, and additionally for Type0A/2-CSS if searchSpaceId = 0, occur in slots with index n0 and n0+X0, where n0 is as in Rel-15, X0=4 for 480 kHz SCS and X0=8 for 960 kHz SCS. - Supported combinations of (X,Y) - A UE capable of multi-slot monitoring mandatorily supports - For SCS 480 kHz: (X,Y) = (4,1) - For SCS 960 kHz: (X,Y) = (8,1) - A UE capable of multi-slot monitoring optionally supports - For SCS 480 kHz: (X,Y) = (4,2) - For SCS 960 kHz: (X,Y) = (8,4), (4,2), (4,1) - Working assumption: BD/CCE budget for (4,2), (4,1) is half that of X=8 - A UE capable of multi-slot monitoring mandatorily supports the following PDCCH monitoring within Y slots - For Y>1: FG3-1 (monitoring Group (1) SSs in the first 3 OFDM symbols of each of the Y slots) - For 960 kHz SCS For Y=1: FG3-5b with set1 = (7, 3) - [FL Note: The first number is the minimum gap in symbols between the start of two spans, the second number is the span duration in symbols (cf. TS 38.822)] - For 480 kHz SCS For Y=1: FG3-5b with set2 = (4, 3) and (7, 3) with a modification with maximum two monitoring spans in a slot - [FL Note: The first number is the minimum gap in symbols between the start of two spans, the second number is the span duration in symbols (cf. TS 38.822)] - The following supersedes FG3-5b and FG3-1 definition: - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of X slots per scheduled CC for FDD - Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of X slots per scheduled CC for TDD Working assumption The following values are adopted as minimum value of Pswitch for 120/480/960 kHz - Support only search space set group switching processing capability 1 with the following values μ; Minimum Pswitch value for UE processing capability 1 [symbols]. 3; 40. 5; 160. 6; 320. Agreement - SS set overbooking can be allowed with multi-slot PDCCH monitoring capability same as the current specification but applied per slot group, i.e., SS set overbooking is allowed for USS in PCell and PSCell, and UE expects no overbooking for CSS in PCell and PSCell and no overbooking in SCell. - The dropping rule for multi-slot PDCCH monitoring capability is the same as the current specification but evaluated per slot group, i.e., a UE drops UE specific search space set(s) in a slot group with higher index when SS sets are overbooked. Additional dropping rules are not precluded

In this contribution, we discuss potential issues and associated standards impacts on PDCCH monitoring for new SCSs in 52.6 – 71GHz.

# Discussions

In RAN1#107-e [1], minimum Pswitch value for search space set group switching were discussed with the following issues:

- Whether to support multiple capabilities (capability 1 and capability 2 as in Rel-16).

  - In Rel-16, UE cap report whether the UE supports search space set group switching Capability-2 via searchSpaceSwitchCapability2-r16.

- Value(s) for 120 kHz

  - In Rel-16, Pswitch values for capability 2 are based on SPS release timeline. Having said that, 25 symbols were suggested for μ=3 with capability 2.

- Whether to scale the value(s) of 120 kHz by 4 and 8 times for 480 and 960 kHz, respectively.

  - Most of UE capability values were determined based on scaled values by 4 or 8 times of 120 kHz value in NR 52-71.

- Whether to consider additional margin.

Based on the discussion, the following values with only one capability (i.e., UE processing capability 1) was agreed as a working assumption.

| μ | Minimum Pswitch value for UE processing capability 1 [symbols] |
| --- | --- |
| 3 | 40 |
| 5 | 160 |
| 6 | 320 |

As the values do not have any serious issues, it is preferred to agree the working assumption as it is.

Observation 1: No serious issue has been observed for the working assumption on the minimum Pswitch value.

Proposal 1: Confirm the working assumption on the minimum Pswitch value.

# Summary

In this contribution, we discussed the remaining issues for PDCCH monitoring enhancements of NR in 52.6 – 71 GHz. From the discussions, we made the following observation and proposal:

Observation 1: No serious issue has been observed for the working assumption on the minimum Pswitch value.

Proposal 1: Confirm the working assumption on the minimum Pswitch value.

# References

- “Final Report of 3GPP TSG RAN WG1 #107-e”, 3GPP RAN1 Meeting #107-e, e-Meeting, November 11th – 19th, 2021.