3GPP TSG RAN WG1 #107bis-e			        											             										 R1- 2200493

e-Meeting, January 17th – 25th, 2022

Agenda item:	8.15.1

Source:	LG Electronics

Title:	Discussion on Rel-17 UE feature for NR FeMIMO

Document for:	Discussion and Decision

Introduction

In RAN1#107-e, the following agreements were made.

Agreement: - The following are the components of the feature “joint TCI for intra-cell operation” - Joint DL/UL TCI update with their components: (configuration mechanism, [QCL rules,] applicable source and target signals, MAC-CE -based TCI] activation assignment) - Common cross multi-CC TCI update and activation (involving RRC common TCI state pool) - Association between TCI state and UL PC settings except for PL RS [for PUCCH, PUSCH, and SRS] - Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS - [MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] - [TCI states pool for configured reference BWP/CC shared by a set of BWP/CC] - [Maximum number of CCs configured with BFR] - [R17 mechanism reusing R15/16 signalling to indicate R17 TCI for individual DL channel/RS that cannot share the same unified TCI as UE-dedicated PDCCH/PDSCH] - [Configuration of both R17 TCI and R15/16 TCI and spatial relation] - The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band - [The maximum number of configured joint TCI states across all BWPs and all CCs in a band] - [The maximum number of configured joint TCI states per CC [in a band]] - [Support The maximum number of MAC-CE activated joint TCI states [per BWP per CC/across all BWPs and all CCs in a band]] - [Support any Whether a particular DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool)] - [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] - [The minimum time gap between the beam indication PDCCH and first slot where beam is applied] - This list gives no guidance as to how to group these components into FGs. They may be mapped to one or more FGs and one or more of these FGs may be agreed as basic. Specifically, different FGs can be grouped together through notes “a UE supporting this FG must also support FG 23-x-y”. This allows to create “basic” FGs while allowing, e.g., different types for different components/candidate values. - The purpose of this list is to find the individual components without having to worry about the signalling structure Agreement: Adopt the following changes highlighted in chromatic formatting 23. NR_FeMIMO; 23-1-3; MPE mitigation; 1. Support of enhanced [PHR] reporting for MPE mitigation (with which includes pairs of (P-MPR,SSBRI/CRI and [associated] P-MPR in PHR reporting) [2. Support of reporting up to Nmax Maximum number of reported P-MPR and SSBRI/CRI pairs] [3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation]; [2. Candidate value of {1,2,3,4}] [3. Candidate value FFS at least includes {64}]; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic formatting 23. NR_FeMIMO; 23-6-3; Simultaneous activation of two TCI states for PDCCH across multiple CCs [(HST/URLLC)]; Support of simultaneous activation of two TCI states [for CORESETs with the same CORESET ID] in all BWPs across a set of configured component carriers [by single MAC-CE]; 23-6-1 or 23-6-2; Per UE; Yes; Component 1 candidate values: {support, no support}; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic formatting 23. NR_FeMIMO; 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; - Maximum number of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Nmax=1 - FFS: Maximum number of NZP CSI-RS resources in one CSI-RS resource set per CMR group: Ks,max=[2,4] - CSI report mode selection FFS others (supported options; values for X, codebook types, number of ports of CMRs, CMR sharing among NCJT and sTRP measurement hypotheses for FR1, two CMR groups with Ks=K1+K2 CMRs in CSI-RS resource set, reporting of 2 PMIs, 2 RIs and 2 Lis for NCJT measurement hypothesis …); FFS; [Per band / per BC]; [Component 2 candidate value set: {2, 3, 4, 5, 6, 7, 8}] Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic formatting 23. NR_FeMIMO; 23-7-4; Support of Nmax=2 for Multi-TRP CSI; Support of maximum number of CMR pairs Nmax=2 configured in NZP-CSI-RS-ResourceSet for a given CSI report setting; 23-7-1; Per band; Component 1 candidate values: {support, no support}; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic formatting 23. NR_FeMIMO; 23-7-5; CMR sharing; [Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis] [FR2 only]; 23-7-1; Per band; FR2 only; Component 1 candidate values: {support, no support} Note: ‘NCJT’ and ‘singel-TRP’ are not used in RAN1 specifications and will be aligned with 38.214; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic formatting 23. NR_FeMIMO; 23-8-2; Triggering SRS only in DCI 0_1/0_2; Support of triggering SRS in DCI 0_1/0_2 without data and without CSI; 2-52; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic formatting 23. NR_FeMIMO; 23-8-3; SRS Antenna switching for >4Rx; 1. Support of SRS antenna switching xTyR with y>4 [2. Report whether the uplink TX antenna switching impact to downlink receiving in a band] [3. Report whether the UL Tx antenna is switched together with UL Tx in another band]; 2-55; Per BC; Component 1 candidate values: FFS; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic formatting 23. NR_FeMIMO; 23-8-4; Maximum 2 SP [and 1 periodic] SRS sets for antenna switching; Support of maximum [2] [SP] SRS resource sets [and maximum 1 periodic SRS resource set] for antenna switching; 2-53; [Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UEs that does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS] - The two SP-SRS resource sets are not activated at the same time; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic formatting 23. NR_FeMIMO; 23-8-5; Increased repetition for SRS; Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource; 10-11, 2-52; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic formatting 23. NR_FeMIMO; 23-8-6; Partial frequency sounding of SRS; Support of partial [(RB level)] frequency sounding for SRS; 23-8-6 2-52; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic formatting 23. NR_FeMIMO; 23-8-7; Start RB location hopping for partial frequency SRS; Support of start RB location hopping in partial [(RB level)] frequency SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent SRS]; [23-8-6]; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic formatting 23. NR_FeMIMO; 23-9-1; Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II); - [{Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to] support Port-selection FeType-II with Mv=1 and R=1 - Support rank 1,2 - Support parameter combinations with Mv=1 - FFS others Support number of PMI subbands with R=1; FFS 2-35; Per band and per BC; Component 1 candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {24 to 256}; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic formatting 23. NR_FeMIMO; 23-9-1a5; Codebook concurrency of Type I codebook and Rel-17 FeType II codebook Active CSI-RS resources and ports for mixed codebook types in any slot; - Supported mixed codebook types List of codebook combinations {codebook 1, codebook 2, codebook 3} - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination; [23-9-1]; FFS; [Component 1 candidate values: [Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS, NULL}, {Type II, FeType II PS },{eType II R=1, FeType II PS }}] {{FeType II PS M=1, NULL},{FeType II PS M=2 R=1, NULL}, {Type II, FeType II PS M=1}, {Type II, FeType II PS M=2 R=1} ,{eType II R=1, FeType II PS M=1},{eType II R=1, FeType II PS M=2 R=1}} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] Note 1：if a UE reports one or more codebook combinations in [23-9-5], then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations [Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-10 and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1, and 23-9-1]; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic formatting 23. NR_FeMIMO; 23-9-2; Support of Mv=2 for FeType-II; [1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with Mv=2] and R=1] [2. Support rank 1,2] 2. Support parameter combinations with Mv=2 [3. Support of DFT FD bases of size N]; [23-9-1]; [per band and per BC]; [Component 1 candidate values - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] [Component 4 candidate values: {N=2 only, N=2 and N=4}]; Optional with capability signalling. 23. NR_FeMIMO; 23-9-4; Support of R = 2 for FeType-II; 1. Support of R = 2 for FeType-II [2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support to support Port-selection FeType-II with M=2 and R=2]; 23-9-2; [per band and per BC]; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic formatting 23. NR_FeMIMO; 23-9-3; Support of rank 3, 4 for FeType-II; Support of rank 3, 4 for FeType-II FFS separate capabilities for M = 1 and M = 2; [23-9-1[, 23-9-2]; Per band; Optional with capability signalling.

In this contribution, we share our views on remaining issues on the FeMIMO UE feature.

Discussion

  - Multi-beam Operation

23-1-1

- Component 2: Remove the bracket for PUCCH/PUSCH/SRS

- Component 1 and 3: Support of DCI-based TCI indication with/without DL assignment were agreed as optional. Hence, MAC-CE based TCI indication is a basic component and UE signals whether to support DCI-based TCI indication with and/or without DL assignment additionally, with respective components according to the following agreement. One way can be to define two candidate values, i.e. DCI with DL assignment and DCI with and without DL assignment to capture the agreement correctly.

Agreement

For beam indication with Rel-17 unified TCI, support DCI format 1_1/1_2 without DL assignment:

- Support UE to report whether or not to support TCI update by DCI format 1_1/1_2.

    - For a UE supporting TCI update by DCI format 1_1/1_2, it must support TCI update by using DCI 1_1/1_2 with DL assignment, and support of the above feature for TCI update by DCI format 1_1/1_2 without DL assignment is UE optional

- Component 4/5: If we define component 6, the necessity of component 4/5 is unclear since TCI state pool/state is configured per BWP in a CC currently. If these are meant to report CC group that can be shared by common TCI or common TCI pool, it needs to be rephrased into such direction rather than reporting the total number of TCI states per band. We suggest to delete these for now.

- Component 7/8: UE can report whether to support joint TCI or separate TCI per CC basis so these components do not seem to be needed.

- Component 9: Support this component to capture the following agreement.

Agreement

On path-loss measurement for Rel.17 unified TCI framework, a PL-RS (configured for path-loss calculation) is either included in UL TCI state or (if applicable) joint TCI state or associated with UL TCI state or (if applicable) joint TCI state.

- Whether a UE supports “beam misalignment or not” (detailed definition FFS) between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is a UE capability

- Component 10: Having configurability of DL RSs for unified TCI is for NW flexibility and not related to UE capability. Suggest to delete this component.

- Component 11/12: Support these since supporting these UE capability report were part of RAN1 agreements.

Proposal 1: Adopt the following table for Rel-17 multi-beam 23-1-1.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-1 | Unified TCI for [intra- and inter-cell] beam management | [For both intra- and inter-cell beam management:] 1. Support of joint DL/UL TCI update [and separate DL/UL TCI update] with their components (configuration mechanism, QCL rules, applicable source and target signals, MAC-CE based TCI state indication) [and Common cross-CC TCI update and activation (involving RRC common TCI state pool)] 2. Support of association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS [ (PLRS and other, including handling of beam [alignment/misalignment] for PLRS)] 3. Supported mode of MAC-CE+DCI-based TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] 6. The maximum number of configured TCI states per CC in a band 9. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE 11. The minimum beam application time in Y symbols 12. The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states |  |  |  |  | [per band] |  |  |  | [4. candidate values {1, …, 32} 5. candidate values {64, 72, 80, 96, 128, 192, 256, 128*4*32} 6. candidate values {8, 16, 32, 64, 128, 256} 3. candidate values {DCI with DL assignment, DCI with and without DL assignment} 7. candidate values {1, 2, 3, 4, 5, 6, 7, 8} 8. candidate values {1, 2, 3, 4,5,6,7, 8} Note 1: for component 7 & 8, if two TCI states are in one codepoint, it should be counted as 2.] | Optional with capability signalling |

23-1-2

    - FG description: Suggest to clarify that this feature is of inter-cell ‘beam’ management/reporting.

    - Component 1: For UE feature description, it may be better to use the term ‘beam’ measurement/report to align with the description of the feature group.

    - Component 2: Suggest to use the ‘beam’ reporting on Kmax SSBRI-RSRP ‘pairs’, and [,SS-RSRP] is not needed in addition to SSBRI-RSRP. The description ‘in one report instance’ is preferred to be kept for clarity.  For candidate values, based on the below conclusion, the brackets on the candidate value should be removed.

Conclusion (106-e)

On Rel.17 L1-RSRP multi-beam measurement/reporting enhancements for inter-cell beam management and inter-cell mTRP, there is no consensus in supporting additional value(s) of KMAX other than 4

    - Component 3/4: Unclear to define two separate components for this. In addition, the number of PCIs should be RRC-configured as agreed below.

Agreement (106bis-e)

On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, NMAX (the maximum number of RRC-configured PCIs different from the serving cell for measurement/reporting) is up to UE capability with candidate values of at least 1 and X.

- Note: The upper bound for X as agreed in AI 8.1.2.2

- When NMAX is configured to be X, the UE is RRC-configured for L1-RSRP measurement with up to X PCIs different from the serving cell PCI

- Additional restriction may be added by RAN4

- FFS: UE measurement behaviour when SSBs associated with different PCIs overlap, including whether this is up to UE capability

Proposal 2: Adopt the following table for Rel-17 multi-beam 23-1-2.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-2 | Inter-cell beam measurement and reporting | 1. Support of beam measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of beam reporting of Kmax SSBRI-RSRP pairs, with at least one pair associated with a PCI different from serving cell PCI in one report instance 3. The number of RRC-configuredPCI(s) different from serving cell PCI for beam measurement [5. The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) different from serving cell PCI] [6. The max number of SSB resources configured to measure L1-RSRP across all PCI(s) different from serving cell PCI] [7. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement] | [2-24, 2-29] |  |  |  |  |  |  |  | {Support, Not support} 2. Candidate value of {1,2,3,4} [3. candidate values {1, 2, 4, 6}] [4. candidate values {1, 2, 4}] [5. candidate values: {2, 4, 8, FFS}] [6. candidate values: {4, 8, 16, 32, 64, FFS}] [7. candidate values {not support}] | Optional with capability signalling |

23-1-3

    - Component 1: Remove the brackets on PHR for clarity since it is based on existing PHR report with pairs of P-MPR and SSBRI/CRI.

Proposal 3: Adopt the following table for Rel-17 multi-beam 23-1-3.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-3 | MPE mitigation | 1. Support of enhanced PHR reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation |  |  |  |  |  |  |  |  | 2. Candidate value of {1,2,3,4} 3. Candidate value FFS | Optional with capability signalling |

23-1-4

    - In RAN1#107-e, it was agreed to support a UE reporting of a list of UE capability value sets where each value set could correspond to one or more UE panels.

Working Assumption (107-e)

Support the UE reporting a list of UE capability value sets

- Each UE capability value set comprises the max supported number of SRS ports

- For any two different value sets, at least one capability value needs to be different

  - FFS: If in addition also identical value sets are allowed.

- Whether the UE capability value set can be common across all BWPs/CCs in same band or BC can be discussed in UE feature session

Proposal 4: Adopt the following table for Rel-17 multi-beam 23-1-4.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-4 | MPUE support for UL | 1. Total number of UE capability value sets 2. Maximum supported number of SRS ports per resource per UE capability value set for codebook based UL |  |  |  |  |  |  |  |  | 1. candidate values: FFS 2. candidate values: [1,2,4] for each UE capability value set | Optional with capability signalling |

  - Multi-TRP PDCCH

23-2-1

    - Component 2: it is beneficial to report multiple BD numbers and configure one of them by RRC considering decoding performance and PDCCH overbooking.

    - Component 3: if 2 BD is configured by RRC, then individual candidate should not be monitored considering possibility of combining based BD implementation. On the other hand, if 3 BD is configured, individual candidate is monitored since UE conducts separate BD for each of linked candidate in case of 3 BD.

Proposal 5: Adopt the following table for Rel-17 M-TRP PDCCH.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-2-1 | [Multi-TRP] PDCCH repetition | 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) [with non-SFN scheme TDM and FDM] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one or more number(s) as required number of BDs for the two PDCCH candidates 3. Support to monitor the individual candidate when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET, if 3 is reported in component 2 and configured by RRC FFS: 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier |  |  |  |  |  |  |  |  | Component1: details FFS Component2: details FFS Component3: details FFS Component4: details FFS [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for [CORESET monitoring in] PDCCH [repetition] | Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA [when UE is configured with PDCCH repetition] | [23-2-1, 23-6-1, 23-6-2] |  |  |  |  |  |  | FR2 only |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-3 | PDCCH repetition for Type3 CSS | Support of PDCCH repetition for Type3 CSS |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

  - Multi-TRP PUSCH

23-3-1

    - Except for component 1 and 6, each component should be defined as a separated sub-FG so that UE is able to report whether to support each of them, separately.

    - Component 6: sequential mapping should be a component of this FG. On the other hand, cyclic mapping is agreed as optional so component 2 should be a separate FG.

    - Component 11: it should be removed since MTRP PUSCH repetition cannot be supported without supporting PUSCH operations.

Proposal 6: Adopt the following table for Rel-17 M-TRP PUSCH.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) 2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A 3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) 6. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A 7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission) 8. Support of A-CSI report on two PUSCH repetitions 9. Support of SP-CSI report on two PUSCH repetitions 10: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-1 | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) FFS: discuss details (to be discussed after type A is stable |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

  - Inter-cell Multi-TRP PDSCH

23-4

    - Component 3 and 4: X1 and X2 are for case 1 and 2, respectively, but not for FR 1 and FR 2. So, FR 1 and 2 in square brackets should be removed.

    - Component 2 and 5: these should be removed. Component 3 and 4 are enough.

Proposal 7: Adopt the following table for Rel-17 inter-cell M-TRP PDSCH.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info 3. Supported max number of RRC-configured PCIs different from serving cell PCI for case1 (X1)] [SSB time domain positions or periodicity of additional PCIs is not exactly the same as serving cell PCI) 4. Supported max number of RRC-configured PCIs different from serving cell PCI for case2 (X2)] [SSB time domain positions and periodicity are exactly the same among the additional PCIs and the same as serving cell PCI |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

  - Multi-TRP beam management

23-5-1

    - Component 1, 2 and 5: These are the basic feature of this FG, so component 2 and 5 can be removed and merged to component 1. We could add ‘from two CMR resource sets’ in component 1 for better clarity.

    - Component 6: this should be removed since there is no relevant agreement.

23-5-2

    - Component 3, 4 and 7: Component 7 is agreed in RAN1#104e, so this should be supported. Component 3 and 4 seems not needed since component 1 is sufficient according to the agreement.

Agreement (104e)

On BFD-RS of TRP-specific BFR

- BFD-RS resource number:

  - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability

  - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e

    - Alt1: max value is 2

    - Alt2: max value is a UE capability, including possible candidate value of 1

Agreement (106e)

The maximum number of BFD-RS resources per set is a UE capability, including a possible candidate value of 1 in Rel.17.

Proposal 8: Adopt the following table for Rel-17 M-TRP beam management.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-5-1 | Group based L1-RSRP reporting enhancements | 1. Support a single L1-RSRP reporting instance consisting of N beam groups and M=2 beams per [pair/group] from two CMR resource sets [in one report instance] 3. [Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs] 4. [Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs] |  |  |  |  |  |  |  |  | Candidate values: {1,2,3,4} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-5-2 | MTRP BFR enhancements | 1. Support of the maximum number of BFD-RS resources per set 2. [Support of Rel-17 M-TRP BFR based on two BFD-RS sets] 5. [Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs] 6. [Support up to two PUCCH-SR resources for MTRP BFRQ] 7. Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP 8. [Support of [single DCI/multi-DCI] based BFR enhancement] |  |  |  |  |  |  |  |  | [Candidate values: {1, 2,…}] | Optional with capability signalling |

  - Multi-TRP HST-SFN

23-6-4

    - Default beam is useful and all of components in FG 23-6-4 were agreed during RAN1 discussion. So, we support all components and ‘FFS’ should be removed.

Proposal 9: Adopt the following table for Rel-17 M-TRP HST-SFN.

| 23. NR_FeMIMO | 23-6-4 | Default beam setup [for HST] | 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI [when PDSCH is scheduled with offset equal or larger than the threshold] 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold 4. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 5. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 6. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - SRS

23-8-1

    - Support the version in the first round discussion on RAN1 #107-e, as below table.

23-8-7

    - Start RB location hopping was agreed to be supported for aperiodic SRS in RAN1 #107-e. So, square bracket should be removed and ‘for periodic/semi-persistent’ should also be removed.

Agreement

For aperiodic SRS, support same start RB location hopping approach as for P/SP SRS if there are multiple frequency hopping period in the slot

Proposal 10: Adopt the following table for Rel-17 SRS.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | Support of determining aperiodic SRS location based on available slot | 2-52 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency SRS transmission across different SRS frequency hopping periods | 23-8-6 |  |  |  |  |  |  |  |  | Optional with capability signalling |