3GPP TSG RAN WG1 #107bis	R1-2200625

e-Meeting, 17 – 25 January, 2022

Agenda item:		8.16.5

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for NR positioning enhancements

Document for:		Discussion and Decision

# Introduction

In RAN1#107-e the baseline for UE features has been further refined [1, 2]. In this contribution we present our views on the UE features for NR positioning enhancements.

# Discussion

Detailed comments on UE features are listed below. For reference, the baseline agreements from RAN1#107-e are available in the Annex.

- 27-1-1:

  - Rx TEG is not depending on the positioning technique (DL-TDOA, Multi-RTT), hence no need for separate row.

  - Per band

- 27-1-2:

  - Per band

- 27-1-2a:

  - Per band

  - Remove brackets and yellow highlight from ”If value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources.”

- 27-1-3:

  - Remove brackets and yellow highlight from ”If value=1 is indicated by the UE, the UE RxTx timing errors differences between two measurements are within a margin only if the UE reports an RxTx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements”

- 27-1-5 - Phase center offset compensation for TEGs (NEW):

  - Add new FG to cover the case of phase center offset compensation for TEGs. See details for proposed FG in the Annex. Given there is an existing RAN1 agreement to support PCO for TEGs, then in case a new FG is not agreeable, it has to be captured as a component of 27-1-1 and 27-1-2 instead.

- 27-2-1:

  - Per band

  - Add ”13-2 or 13-3” as pre-requisite, as it requires PRS reception for DL-AoD or DL-TDOA. Remove FG 13-8 as a prerequisite of this FG. FG 13-8 is about SRS for positioning so it is not related to DL PRS measurement reporting.

  - Remove the bracket in “Component 2 candidate values: [2,4,8,16,24] as it was agreed at RAN1 #107e.

  - Need to add a note in this FG such that the maximum number of first path PRS RSRP per TRP should be less than or equal to the maximum number of PRS RSRP (27-2-2) based on the agreement below from RAN1 #107e.

Agreement(RAN1 #107e)

For reporting of DL PRS RSRPP and PRS RSRP in UE-A DL-AOD

- The maximum number of DL PRS RSRPP M is a UE capability and its candidate values include {2,4,8,16,24}.

The capabilities for DL PRS RSRPP (M value) and DL PRS RSRP (N values) are such that M is less than or equal to N

- 27-2-2:

  - Add FG 13-5 as pre-requisite as this extends the Rel-16 parameter space.

  - Per band

- 27-3-1:

  - Add FG 13-1 as pre-requisite, remove 13-8.

  - Per band

- 27-3-2:

  - Confirm the FG, details to be decided later.

  - Add FG 13-1 as pre-requisite

  - Per UE capability, the processing itself (i.e. FG 27-3-3) can be a per band indication

- 27-3-3:

  - Confirm the FG.

  - Content of components 1 and 2 are swapped. Revise them as follows:

1. DL PRS buffering capability: Type 1 or Type 2

a) T: [{8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}] ms

a) Type 1 – sub-slot/symbol level buffering

b) Type 2 – slot level buffering

2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE.

a) Type 1 – sub-slot/symbol level buffering

a) T: [{8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}] ms

b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms

  - Per band indication

- 27-4-1:

  - Need to add corresponding Rel-16 FGs on measurement reports as pre-requisites, as this is an add-on.

  - No need for separate capability for soft and hard indication. OK to list as component in this capability though.

  - Given that supporting to LoS/NLoS indication in itself is not dependent on measurement type, there is no need to report it separately for measurement type either.

  - No need to signal candidate values.

- 27-5-1:

  - Add FG 13-1 as pre-requisite.

  - Confirm ”Need for location server to know if the feature is supported” in Notes column.

- 27-6:

  - Confirm the FG, but contents should be FFS, pending on input from RAN2.

- 27-7:

  - No need for such FG.

- 27-8:

  - Confirm FG definitions.

- 27-9:

  - This is mostly fine, but the definition of support values need to wait for outcome of RAN1 maintenance discussion.

- 27-10:

  - Confirm FG definitions.

- 27-11:

  - Contents are OK, though component 1 is already implied by component 2 in any case.

- 27-12:

  - Component 1 should be FFS for now, as it depends on how 27-4-1 is going to be defined.

- 27-14:

  - OK to confirm the FG

- 27-15:

  - Remove [for initial BWP] from title and add component for signaling support to {initial BWP/any BWP }.

- 27-17:

  - Remove brackets in FG title, i.e. keep the text. RAN1 agreed to define UE capability for DL PRS processing in RRC_INACTIVE state.

  - OK to confirm current FG definitions, and add a component on SRS transmission in RRC_INACTIVE:

    - 2. Support of positioning SRS transmission in RRC_INACTIVE state.

- 27-18a/b/c:

  - No need for separate FG, it can be considered as part of 27-17 if needed.

- 27-19:

  - It is OK to confirm the FG, but the definitions need more discussion, including the pontential need for a component on validity criteria.

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for NR positioning enhancements, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#107-e, Nov. 2021

- R1-2112902, ”Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e”, Moderators (AT&T, NTT DOCOMO, INC.), Nov. 2021

# Annex

(See next page)

New FG proposed:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27.NR_pos_enh | 27-1-5 | Phase center offset compensation for TEGs | Indicates if the UE supports PCO compensation as part of the TEG margin calculation. FFS: Separate capabilities for UE Rx TEG, UE Tx TEG, UE RxTx TEG. | [13-3,13-41,3-8] | No |  | PCO errors are not calibrated as part of TEGs | [Per UE] | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-1 | Support of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] | The maximum number of UE-RxTEG, which is supported and reported by UE for UE assisted DL TDOA and/or Multi-RTT positioning | 13-1, one or more of {13-3, 13-4} | No |  | UE-RxTEG reporting is not supported and no assumption can be made on the mitigation of UE Rx timing delays for the measurements | [per band or FS] | n/a | n/a | n/a | The candidate values are {[1,] 2,[ 3,] 4, 6, 8[, 12, 16, 24, 32]} Need for location server to know if the feature is supported FFS: Separate row for “Support of UE-RxTEG reporting for DL-TDOA”, and “Support of UE-RxTEG reporting for M-RTT” If UE supports this capability with the values > 1, and if the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement [If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports the same Rx-TEG-ID associated with both measurements, otherwise, no assumption can be made about the timing error differences between these measurements.] Note: The “per band” reporting on this capability does not imply, that the RxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTEG ID can span from 0, up to 31 | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-2 | Support of UE-TxTEGs for UL TDOA | The maximum number of UE-TxTEG, which is supported and reported by UE for UL TDOA | [13-4, ]13-8 | Yes |  | UE-TxTEGs for UL TDOA is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS” and “UE-TxTEGs for RTT is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS | FFS: per band or per FS | n/a | n/a | n/a | The candidate values are {[1, ]2, 4, 6, 8} [Need for location server to know if the feature is supported] Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA [if UL TDOA is supported by UE] [Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE] | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-2a | Support of UE-TxTEGs for Multi-RTT [and/or UL TDOA] positioning | The maximum number of UE-TxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4, 13-8 | No |  | UE-TxTEGs for Multi-RTT positioning is not supported | [per band per FS] | n/a | n/a | n/a | The candidate values are {[1, ] 2, 4, 6, 8} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if if the UE does not include TxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Tx timing delays for this SRS resource [If value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources.] [Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA] [Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE] | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-3 | Support of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning | [13-4 or 13-8] | No |  | Mitigation of UE RxTx timing delays is not supported | per band | n/a | n/a | n/a | The candidate values are {[1, ]2, 4, 6, 8, 12, 16, 24, 32[, 64, 128, 256]} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE RxTx timing delays for this measurement [If value=1 is indicated by the UE, the UE RxTx timing errors differences between two measurements are within a margin only if the UE reports an RxTx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements] Note: The “per band” reporting on this capability does not imply, that the RxTxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTxTEG ID can span from 0, up to [255] | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-4 | Support of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP | 27-1-1 | No |  | Up to 1 RxTEG is used to measure the same DL PRS resource of a TRP | per band | n/a | n/a | n/a | The candidate values are {2, 3, 4, 6, 8} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously | The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously |  | No |  |  | Per band | n/a | n/a | n/a | [The candidate values are {1,2, 4, 8}] | Optional with capability signaling |
| 27. NR_pos_enh | 27-2-1 | [UE-assisted] DL PRS RSRP of the first path for DL-AoD | 1.) Support of [measuring and reporting the] PRS RSRP of the first path for DL-AoD positioning method 2.) The maximum number of first path PRS RSRP per TRP | [13-2 or 13-3, 13-4, 13-5, 13-8] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Component 2 candidate values: [2,4,8,16,24] Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. | [13-5, 13-2] | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | FFS: Per UE or per band | n/a | Yes | n/a | The candidate values are {[12, ]16, 24[, 32, 64]} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-1 | M-sample measurements [of DL PRS measurement on single DL PRS period/occasion] | [The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set ] | [13-1, 13-4, 13-8] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1}] If the UE does not provide the capability, the UE [is assumed to] support M=4 only. Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-2 | DL PRS measurement outside MG [and in a PRS processing priority window] - processing types | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG [and in a PRS processing priority window] Candidate values: {Type 1A, Type 1B, Type 2}. Note: - Type 1A refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from all DL CCs (per UE) are affected. - Type 1B refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from certain DL CCs are affected. - Type 2 refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options [Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP] | [13-1] | FFS |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported FFS: Separate feature group for a UE to declare support of each of the Type-1A, Type-1B, Type-2” capabilities | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-3 | DL PRS Processing Capability outside MG - buffering capability | 1. DL PRS buffering capability: Type 1 or Type 2 a) T: [{8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}] ms b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-3-2 | No |  |  | Per band | n/a | n/a | n/a | FFS: Separate feature group for a UE to declare PRS processing capabilities of each of the Type-1A, Type-1B, Type-2” capabilities | Optional with capability signaling |
| 27. NR_pos_enh | 27-4-1 | LOS/NLOS Indicator for UE-assisted positioning | Support reporting LoS/NLoS indicator to LMF [for RSTD and UE Rx-Tx time difference measurements to LMF for DL and DL+UL positioning] FFS: whether to have separate capability component/FG for RSTD and UE Rx-Tx time difference measurements FFS: whether to have separate capability component for hard and soft indication |  | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate value are [0,1]] Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-5-1 | [UE-initiated] on-demand PRS | UE’s capability to support UE-initiated on-demand DL PRS [request signalling] | [13-1] | No |  |  | Per UE | n/a | n/a | n/a | FFS: Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-6 | DL PRS processing capabilities in RRC inactive state | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz |  |  |  |  |  |  |  |  |  |  |
| 27. NR_pos_enh | 27-7 | Maximum number of measurement instances which can be included in a single measurement report | Maximum number of measurement instances which can be included in a single measurement report |  |  |  |  |  |  |  |  |  |  |
| 27. NR_pos_enh | 27-8 | Support of PRS TEG association information for UE-based DL-TDOA | Support of reception of association between PRS and TRP Tx TEG for UE-based positioning |  | No |  | Positioning calculation assistance data containing association between PRS and TRP Tx TEG is not supported by UE | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. Agreement: Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs | Optional with capability signaling |
| 27. NR_pos_enh | 27-9 | Support of lower Rx beam sweeping factor | 1. Support of the lower Rx beam sweeping factor than 8 for FR2 2. Number of Rx beam sweeping factors: {1,2,3,4,5,6,7} |  | No |  | UE only supports 8 as the Rx beam sweeping factor defined by RAN4. | Per band (FR2 only) | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-10 | Support of UL MAC CE based MG activation request | 1. Support of using UL MAC CE to request measurement gap. |  | Yes |  | Using UL MAC CE to indicate PRS measurement to the gNB is not supported. | Per UE | No | No | No |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-11 | Support of DL MAC CE based MG activation | 1. Support of preconfiguration of MGs in RRC 2. Support of using DL MAC CE to activate the MG |  | Yes |  | Using DL MAC CE to activate the preconfigured MG is not supported | Per UE | No | No | No |  | Optional with capability signaling. |
| 27. NR_pos_enh | 27-12 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type: {softValue, hardValue, both} 2. LOS/NLOS indicator granularity {resourceSpecific, trpSpecific, both} |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-13 | Additional path reporting for UE-assisted DL-TDOA | [1. Support of TOA reporting for more than 2 additional paths.] 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-14 | Additional path reporting for Multi-RTT | 1. Support of TOA reporting for more than 2 additional paths 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-15 | Support of positioning SRS transmission in RRC_INACTIVE state [for initial BWP] | 1. Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. 2. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of periodic SRS Resources for positioning per BWP per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} 4. FFS: Applicability for initial BWP OLPC for SRS for positioning based on SSB from the last serving cell (the cell that releases UE from connection) is part of this FG. Note: no dedicated capability signaling is intended for this component |  | Yes |  |  | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-16 | OLPC for positioning SRS in RRC_INACTIVE state | Same as LPP OLPC-SRS-Pos-r16 RRC OLPC-SRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-17 | Support of [PRS measurement in RRC_INACTIVE] | Support of PRS measurement in RRC_INACTIVE Note: UE supporting this feature may support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference |  | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling. |
| 27. NR_pos_enh | 27-18a | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for DL-TDOA. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-18b | Support of PRS measurement in RRC_INACTIVE state for DL-AoD | Support of PRS measurement in RRC_INACTIVE state for DL-AoD Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for DL-AoD. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-18c | Support of PRS measurement in RRC_INACTIVE state for Multi-RTT | 1. Support of PRS measurement in RRC_INACTIVE state for Multi-RTT 2. Support of positioning SRS transmission in RRC_INACTIVE state. Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for Multi-RTT. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-19 | Spatial relation for positioning SRS in RRC_INACTIVE state | Same as LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |