3GPP TSG RAN WG1 #108-e	R1-22xxxxx

e-Meeting, Feb 21st – Mar 3rd, 2022

Title:	Draft Reply LS on the PL-RS configuration of PUCCH SCell to be activated

Response to:	R1-2200896/R4-2202602

Release:	Rel-17

Work Item:	NR_RRM_enh2-Core

Source:	Apple, [RAN1]

To:	RAN4, RAN2

Cc:

Contact Person:

Name:	Yushu Zhang

E-mail Address:	yushu_zhang@apple.com

# 1. Overall Description

RAN1 would like to thank RAN4 for the LS on the PL-RS configuration of PUCCH SCell to be activated. RAN1 would like to provide the answer to the following question.

RAN4 has made following working assumption and would like to kindly ask RAN1 to confirm/clarify:

Working assumption:

- RAN4 to agree that PL-RS assumptions defined in TS38.213 section 7.2.1 can be applied for the PUCCH of target being-activated SCell during the activation procedure. In FR2 if UE is not provided pathlossReferenceRSs but provided PUCCH-SpatialRelationInfo before receiving the PUCCH SCell activation command, as UE may not obtain MIB during activation procedure, UE shall use the associated DL-RS in PUCCH-SpatialRelationInfo as PL-RS.

[Question to RAN1]: Is the above working assumption in line with RAN1’s understanding?

  - If the answer to above question is NO, how could UE determine the PL-RS for PUCCH of target being-activated SCell during the activation procedure for the following scenarios respectively:

    - if the UE is not provided pathlossReferenceRSs and is provided PUCCH-SpatialRelationInfo before receiving the PUCCH SCell activation command

    - If the UE is not provided pathlossReferenceRSs, and is not provided PUCCH-SpatialRelationInfo before receiving the PUCCH SCell activation command

    - If the UE is provided pathlossReferenceRSs and PUCCH-SpatialRelationInfo before receiving the PUCCH SCell activation command

    - If the UE is provided pathlossReferenceRSs and is not provided PUCCH-SpatialRelationInfo before receiving the PUCCH SCell activation command

    - If any case is missing in above combinations for PL-RS determination for PUCCH of target being-activated SCell during the activation procedure, please RAN1 indicates and explains the UE behavior of PL-RS determination for the missing case(s).

Answer to RAN4: RAN1 has not identified any issue to use associated DL-RS in PUCCH-SpatialRelationInfo as PL-RS as the working assumption agreed in RAN4, assuming the scenario exists.

In addition, RAN1 would like to kindly ask RAN2 the following question:

Question to RAN2: is it possible that a UE is not provided pathlossReferenceRSs but provided PUCCH-SpatialRelationInfo from RRC signaling perspective?

# 3. Actions:

To: RAN4

RAN1 respectfully asks RAN4 to take the above information into account for future work.

To: RAN2

RAN1 respectfully asks RAN2 to clarify the corresponding question above if needed.

# 4. Date of Next RAN1 Meetings:

TSG RAN WG1   Meeting #109-e            	May. 16 – 27, 2021          	Online

TSG RAN WG1   Meeting #110            	   	Aug. 22 – 26, 2021          	Toulouse, France