3GPP TSG RAN WG1 Meeting #108-e	  R1-21xxxxx

e-Meeting, February 21 – March 3, 2022

Agenda Item:	7.2.5

Source:	Moderator (Huawei)

Title:	Summary of email discussion [108-e-R16-URLLC-05] Miscellaneous corrections on Rel-16 URLLC

Document for:	Discussion and Decision

# Introduction

The email discussion is to discuss the editorial spec changes for recommendation to the editor.

- [108-e-R16-URLLC-05]  Miscellaneous corrections on Rel-16 URLLC by February 25 – Chengyan (Huawei)

  - Issue#8 & Issue#9: Corrections on RRC parameter for Rel-16 URLLC

    - R1-2201821, R1-2201823

    - For recommendations to editors

  - Proposed conclusion #1: Dynamic BWP switching is only triggered by the DCI format(s) that are configured to be monitored in both the active BWP and the target BWP.

  - Proposed conclusion #2: SRS resource indication for Type 1 CG is determined by the SRS resource set configured by srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'nonCodebook',  regardless of whether DCI format 0_1 is configured to be monitored or not.

This document summarizes the above issue and provide some initial proposals for discussion. Companies are encouraged to provide the first round views by 02/22, 23:59pm UTC, then we can adjust the proposals for the next step discussions if necessary.

# R1-2201821: 38.213 corrections on RRC parameter for URLLC

ASUSTeK R1-2201821 Reason for change:; In the current 38.213, RRC parameter ‘dl-DataToUL-ACK-ForDCI-Format1-2‘ and ‘dl-DataToUL-ACKForDCIFormat1_2‘ are used, which are not aligned to current RRC specification.. Summary of change:; Change ‘dl-DataToUL-ACK-ForDCI-Format1-2‘ and ‘dl-DataToUL-ACKForDCIFormat1_2‘ to ‘dl-DataToUL-ACK-DCI-1-2‘.. Consequences if not approved:; RRC parameter is used incorrectly which causes confusion on determination of HARQ feedback timing.. #### 9.1.2.2 Type-1 HARQ-ACK codebook in physical uplink shared channel If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format that does not include a DAI field, then - if the UE has not received any PDSCH or SPS PDSCH release that the UE transmits corresponding HARQ-ACK information in the PUSCH, based on a value of a respective PDSCH-to-HARQ_feedback timing indicator field in a DCI format scheduling the PDSCH reception or the SPS PDSCH release or on the value of dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in DCI format 1_1 or on the value of dl-DataToUL-ACK-DCI-1-2 if the PDSCH-to-HARQ_feedback timing indicator field is not present in DCI format 1_2, in any of the Mc occasions for candidate PDSCH receptions by a DCI format or SPS PDSCH on any serving cell c, as described in clause 9.1.2.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission; <Unchanged parts omitted> ### 9.2.3 UE procedure for reporting HARQ-ACK A UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot per priority index, if the UE is not provided ackNackFeedbackMode = separate. For DCI format 1_0, the PDSCH-to-HARQ_feedback timing indicator field values map to {1, 2, 3, 4, 5, 6, 7, 8}. For a DCI format, other than DCI format 1_0 or requesting Type-3 HARQ-ACK codebook report without scheduling a PDSCH reception as described in clause 9.1.4, the PDSCH-to-HARQ_feedback timing indicator field values, if present, map to values for a set of number of slots provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2, as defined in Table 9.2.3-1. If the UE is provided subslotLengthForPUCCH, n is the last UL slot that overlaps with the PDSCH reception or with the PDCCH reception in case of SPS PDSCH release or in case of SCell dormancy indication or in case of the DCI format that requests Type-3 HARQ-ACK codebook report and does not schedule a PDSCH reception; otherwise, n is the last UL slot that overlaps with the DL slot nD for the PDSCH reception or with the DL slot nD for the PDCCH reception in case of SPS PDSCH release or in case of SCell dormancy indication or in case of the DCI format that requests Type-3 HARQ-ACK codebook report and does not schedule a PDSCH reception. For a SPS PDSCH reception ending in DL slot nD, the UE transmits the PUCCH in UL slot n+k where k is provided by the PDSCH-to-HARQ_feedback timing indicator field, if present, in a DCI format activating the SPS PDSCH reception. If the UE detects a DCI format that does not include a PDSCH-to-HARQ_feedback timing indicator field and schedules a PDSCH reception or activates a SPS PDSCH reception ending in DL slot nD, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot n+k where k is provided by dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2. If the UE detects a DCI format scheduling a PDSCH reception ending in DL slot nD or if the UE detects a DCI format indicating a SPS PDSCH release or indicating SCell dormancy through a PDCCH reception ending in DL slot nD, or if the UE detects a DCI format that requests Type-3 HARQ-ACK codebook report and does not schedule a PDSCH reception through a PDCCH reception ending in DL slot nD, as described in clause 9.1.4, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot n+k, where k is a number of slots and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2. A PUCCH transmission with HARQ-ACK information is subject to the limitations for UE transmissions described in clause 11.1 and clause 11.1.1. Table 9.2.3-1: Mapping of PDSCH-to-HARQ_feedback timing indicator field values to numbers of slots PDSCH-to-HARQ_feedback timing indicator; Number of slots. 1 bit; 2 bits; 3 bits. '0'; '00'; '000'; 1st value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2. '1'; '01'; '001'; 2nd value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2. '10'; '010'; 3rd value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2. '11'; '011'; 4th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2. '100'; 5th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2. '101'; 6th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2. '110'; 7th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2. '111'; 8th value provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2. For a PUCCH transmission with HARQ-ACK information, a UE determines a PUCCH resource after determining a set of PUCCH resources for HARQ-ACK information bits, as described in clause 9.2.1. The PUCCH resource determination is based on a PUCCH resource indicator field [5, TS 38.212], if present, in a last DCI format, among the DCI formats that have a value of a PDSCH-to-HARQ_feedback timing indicator field, if present, or a value of dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2, indicating a same slot for the PUCCH transmission, that the UE detects and for which the UE transmits corresponding HARQ-ACK information in the PUCCH where, for PUCCH resource determination, detected DCI formats are first indexed in an ascending order across serving cells indexes for a same PDCCH monitoring occasion and are then indexed in an ascending order across PDCCH monitoring occasion indexes. For indexing DCI formats within a serving cell for a same PDCCH monitoring occasion, if the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs on an active DL BWP of a serving cell, and with ackNackFeedbackMode = joint for the active UL BWP, detected DCI formats from PDCCH receptions in the first CORESETs are indexed prior to detected DCI formats from PDCCH receptions in the second CORESETs. The PUCCH resource indicator field values map to values of a set of PUCCH resource indexes, as defined in Table 9.2.3-2 for a PUCCH resource indicator field of 3 bits, provided by resourceList for PUCCH resources from a set of PUCCH resources provided by PUCCH-ResourceSet with a maximum of eight PUCCH resources. If the PUCCH resource indicator field includes 1 bit or 2 bits, the values map to the first two values or the first four values, respectively, of Table 9.2.3-2. If the last DCI format does not include a PUCCH resource indicator field, the first value of Table 9.2.3-2 is used. For the first set of PUCCH resources and when the size of resourceList is larger than eight, when a UE provides HARQ-ACK information in a PUCCH transmission in response to detecting a last DCI format in a PDCCH reception, among DCI formats with a value of the PDSCH-to-HARQ_feedback timing indicator field, if present, or a value of dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2, indicating a same slot for the PUCCH transmission, the UE determines a PUCCH resource with index , , as where is a number of CCEs in CORESET of the PDCCH reception for the DCI format as described in clause 10.1, is the index of a first CCE for the PDCCH reception, and ∆PRI is a value of the PUCCH resource indicator field in the DCI format. If the DCI format does not include a PUCCH resource indicator field, ∆PRI=0. <Unchanged parts omitted> #### 9.2.5.2 UE procedure for multiplexing HARQ-ACK/SR/CSI in a PUCCH For a transmission occasion of a single CSI report, a PUCCH resource is provided by pucch-CSI-ResourceList. For a transmission occasion of multiple CSI reports, corresponding PUCCH resources can be provided by multi-CSI-PUCCH-ResourceList. If a UE is provided first and second PUCCH-Config, multi-CSI-PUCCH-ResourceList is provided by the first PUCCH-Config, and PUCCH-ResourceId in pucch-CSI-ResourceList or multi-CSI-PUCCH-ResourceList indicates a corresponding PUCCH resource in PUCCH-Resource provided by the first PUCCH-Config. <Unchanged parts omitted> If a UE has HARQ-ACK, SR and wideband or sub-band CSI reports to transmit and the UE determines a PUCCH resource with PUCCH format 2, or the UE has HARQ-ACK, SR and wideband CSI reports [6, TS 38.214] to transmit and the UE determines a PUCCH resource with PUCCH format 3 or PUCCH format 4, where - the UE determines the PUCCH resource using the PUCCH resource indicator field [5, TS 38.212] in a last of a number of DCI formats with a value of a PDSCH-to-HARQ_feedback timing indicator field, if present, or a value of dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2, indicating a same slot for the PUCCH transmission, from a PUCCH resource set provided to the UE for HARQ-ACK transmission, and - the UE determines the PUCCH resource set as described in clause 9.2.1 and clause 9.2.3 for UCI bits and <Unchanged parts omitted>

Feature lead view: The issue is valid and the editorial spec changes on RRC parameter dl-DataToUL-ACK-ForDCI-Format1-2 and dl-DataToUL-ACKForDCIFormat1_2 in R1-2201821 look correct to me.

Proposal 2-1: The editorial spec changes in R1-2201821 are endorsed to improve clarity of RAN1 specifications (TS 38.213).

| Company | View |
| --- | --- |
| Samsung | Fine with the proposal. It should be included in alignment CR. |
| ZTE | Fine with the editorial change. |
| DOCOMO | Fine with the editorial change. |
| Intel | Support the proposal. |
| ASUSTeK | Support the proposal. |
| vivo | We are fine with the editorial change. |
| Nokia/NSB | Support |
| OPPO | Support |
| HW/HiSi | Support |
| Qualcomm | Fine with the editorial change. |

# R1-2201823: 38.214 corrections on RRC parameter for URLLC

ASUSTeK R1-2201823 Reason for change:; In the current 38.213, RRC parameter ‘maxRankForDCI-Format0-2‘, ‘pdsch-TimeDomainAllocationListForDCI-Format1-2‘, and ‘codebookSubsetForDCI-Format0-2‘ are used, which are not aligned to current RRC specification.. Summary of change:; Change ‘maxRankForDCI-Format0-2‘ to ‘maxRankDCI-0-2‘. Change ‘codebookSubsetForDCI-Format0-2‘ to ‘codebookSubsetDCI-0-2‘. Change ‘pdsch-TimeDomainAllocationListForDCI-Format1-2‘ to ‘pdsch-TimeDomainAllocationListDCI-1-2‘.. Consequences if not approved:; RRC parameters are used incorrectly which causes confusion on the determination of corresponding transmissions.. ##### 5.1.2.1.1 Determination of the resource allocation table to be used for PDSCH Table 5.1.2.1.1-1 and Table 5.1.2.1.1-1A define which PDSCH time domain resource allocation configuration to apply. Either a default PDSCH time domain allocation A, B or C according to tables 5.1.2.1.1-2, 5.1.2.1.1-3, 5.1.2.1.1-4 and 5.1.2.1.1-5 is applied, or the higher layer configured pdsch-TimeDomainAllocationList or pdsch-TimeDomainAllocationListDCI-1-2 is applied. For operation with shared spectrum channel access, as described in [16, TS 37.213], UE reinterprets S and L in row 9 of Table 5.1.2.1.1-2 as S=6 and L=7. < Unchanged parts are omitted > Table 5.1.2.1.1-1A: Applicable PDSCH time domain resource allocation for DCI format 1_2 PDSCH-ConfigCommon includes pdsch-TimeDomainAllocationList; PDSCH-Config includes pdsch-TimeDomainAllocationList; PDSCH-Config includes pdsch-TimeDomainAllocationListDCI-1-2; PDSCH time domain resource allocation to apply. No; No; No; Default A. Yes; No; No; pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon. No/Yes; Yes; No; pdsch-TimeDomainAllocationList provided in PDSCH-Config. No/Yes; No/Yes; Yes; pdsch-TimeDomainAllocationListDCI-1-2 provided in PDSCH-Config. < Unchanged parts are omitted > ## 6.1 UE procedure for transmitting the physical uplink shared channel PUSCH transmission(s) can be dynamically scheduled by an UL grant in a DCI, or the transmission can correspond to a configured grant Type 1 or Type 2. The configured grant Type 1 PUSCH transmission is semi-statically configured to operate upon the reception of higher layer parameter of configuredGrantConfig including rrc-ConfiguredUplinkGrant without the detection of an UL grant in a DCI. The configured grant Type 2 PUSCH transmission is semi-persistently scheduled by an UL grant in a valid activation DCI according to Clause 10.2 of [6, TS 38.213] after the reception of higher layer parameter configuredGrantConfig not including rrc-ConfiguredUplinkGrant. If configuredGrantConfigToAddModList is configured, more than one configured grant configuration of configured grant Type 1 and/or configured grant Type 2 may be active at the same time on an active BWP of a serving cell. For the PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant activated by DCI format 0_0 or 0_1, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubset, maxRank, scaling of UCI-OnPUSCH, which are provided by pusch-Config. For the PUSCH transmission corresponding to a Type 2 configured grant activated by DCI format 0_2, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubsetDCI-0-2, maxRankDCI-0-2, scaling of UCI-OnPUSCH, resourceAllocationType1GranularityDCI-0-2 provided by pusch-Config. If the UE is provided with transformPrecoder in configuredGrantConfig, the UE applies the higher layer parameter tp-pi2BPSK, if provided in pusch-Config, according to the procedure described in Clause 6.1.4 for the PUSCH transmission corresponding to a configured grant. < Unchanged parts are omitted > #### 6.1.1.1 Codebook based UL transmission For codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1, DCI format 0_2 or semi-statically configured to operate according to Clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_1, DCI format 0_2, or semi-statically configured to operate according to Clause 6.1.2.3, the UE determines its PUSCH transmission precoder based on SRI, TPMI and the transmission rank, where the SRI, TPMI and the transmission rank are given by DCI fields of SRS resource indicator and Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 or given by srs-ResourceIndicator and precodingAndNumberOfLayers according to clause 6.1.2.3. The SRS-ResourceSet(s) applicable for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 in SRS-config, respectively. Only one SRS resource set can be configured in srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to ‘codebook’, and only one SRS resource set can be configured in srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to ‘codebook’. The TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource. The transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to higher layer parameter nrofSRS-Ports in SRS-Config, as defined in Clause 6.3.1.5 of [4, TS 38.211]. When the UE is configured with the higher layer parameter txConfig set to ‘codebook’, the UE is configured with at least one SRS resource. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI. For codebook based transmission, the UE determines its codebook subsets based on TPMI and upon the reception of higher layer parameter codebookSubset in pusch-Config for PUSCH associated with DCI format 0_1 and codebookSubsetDCI-0-2 in pusch-Config for PUSCH associated with DCI format 0_2 which may be configured with ‘fullyAndPartialAndNonCoherent’, or ‘partialAndNonCoherent’, or ‘nonCoherent’ depending on the UE capability. When higher layer parameter ul-FullPowerTransmission is set to ‘fullpowerMode2’ and the higher layer parameter codebookSubset or the higher layer parameter codebookSubsetDCI-0-2 is set to ‘partialAndNonCoherent’, and when the SRS-resourceSet with usage set to “codebook” includes at least one SRS resource with 4 ports and one SRS resource with 2 ports, the codebookSubset associated with the 2-port SRS resource is ‘nonCoherent’. The maximum transmission rank may be configured by the higher layer parameter maxRank in pusch-Config for PUSCH scheduled with DCI format 0_1 and maxRankDCI-0-2 for PUSCH scheduled with DCI format 0_2. A UE reporting its UE capability of ‘partialAndNonCoherent’ transmission shall not expect to be configured by either codebookSubset or codebookSubsetDCI-0-2 with ‘fullyAndPartialAndNonCoherent’. A UE reporting its UE capability of ‘nonCoherent’ transmission shall not expect to be configured by either codebookSubset or codebookSubsetDCI-0-22 with ‘fullyAndPartialAndNonCoherent’ or with ‘partialAndNonCoherent’. A UE shall not expect to be configured with the higher layer parameter codebookSubset or the higher layer parameter codebookSubsetDCI-0-2 set to ‘partialAndNonCoherent’ when higher layer parameter nrofSRS-Ports in an SRS-ResourceSet with usage set to ‘codebook’ indicates that the maximum number of the configured SRS antenna ports in the SRS-ResourceSet is two. < Unchanged parts are omitted >

Feature lead view: The issue is valid and the editorial spec changes on RRC parameters in R1-2201823 look correct to me.

Proposal 3-1: The editorial spec changes in R1-2201823 are endorsed to improve clarity of RAN1 specifications (TS 38.214).

| Company | View |
| --- | --- |
| Samsung | Fine with the proposal. It should be included in alignment CR. |
| ZTE | Fine with the editorial change. |
| DOCOMO | Fine with the editorial change. |
| Intel | Support the proposal. |
| ASUSTeK | Support the proposal. |
| Vivo | We are fine with the editorial change. |
| Nokia/NSB | Support |
| OPPO | Support |
| HW/HiSi | Support |
| Qualcomm | Fine with the editorial change. |

# R1-2202439: Corrections on BWP switching using DCI format 0_2/1_2

Huawei R1-2202439 In Rel-16, the new DCI formats 0_2 and 1_2 are supported. DCI formats x_2 distinguish from x_1 in the way that they have configurable DCI field sizes to allow e.g. a compact size. The new DCI formats x_2 can also be used for BWP switching. It is now possible from the specification point of view to support BWP switching where DCI formats x_1 are monitored in one BWP and DCI formats x_2 are monitored in the other BWP since the search space set is configured per BWP. In fact, it could be even be desired for efficient support of URLLC services to only monitor DCI formats x_2 in a given BWP. As an example, assume that DCI formats x_2 are only configured to be monitored in the active BWP#1 and DCI formats x_1 are only configured to be monitored in the target BWP#2. When a DCI x_2 is received in the active BWP#1 and triggering a switch to BWP#2, the UE will switch from BWP#1 to BWP#2 and perform transmission or reception in BWP#2. Correspondingly, the related RRC parameters are needed in BWP#2 for UE-specific scheduling. According to our understanding, if a DCI format x_2 is not configured for monitoring in the target BWP, the RRC parameters for the DCI fields of DCI format x_2 are generally not configured in that BWP. Then UE may not obtain the required size of some DCI field for the target BWP. In such situation, the UE would not be able to interpret the DCI fields for the processing of the PDSCH/PUSCH in the target BWP according to the procedure described in TS 38.213, Section 12 and copied below: From 38.213, Section 12 [2]: If a bandwidth part indicator field is configured in a DCI format, the bandwidth part indicator field value indicates the active DL BWP, from the configured DL BWP set, for DL receptions as described in [5, TS 38.212]. If a bandwidth part indicator field is configured in a DCI format, the bandwidth part indicator field value indicates the active UL BWP, from the configured UL BWP set, for UL transmissions as described in [5, TS 38.212]. If a bandwidth part indicator field is configured in a DCI format and indicates an UL BWP or a DL BWP different from the active UL BWP or DL BWP, respectively, the UE shall - for each information field in the DCI format - if the size of the information field is smaller than the one required for the DCI format interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, the UE prepends zeros to the information field until its size is the one required for the interpretation of the information field for the UL BWP or DL BWP prior to interpreting the DCI format information fields, respectively - if the size of the information field is larger than the one required for the DCI format interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, the UE uses a number of least significant bits of the DCI format equal to the one required for the UL BWP or DL BWP indicated by bandwidth part indicator prior to interpreting the DCI format information fields, respectively - set the active UL BWP or DL BWP to the UL BWP or DL BWP indicated by the bandwidth part indicator in the DCI format. As a result, the UE will obtain a DCI with inconsistent information which is considered as a DCI false alarm. Therefore, the scheduled channels will not be processed and the intended BWP switch is not carried out. This behaviour is also described in 38.213 [2] and copied below: If a UE detects a DCI format with inconsistent information, the UE discards all the information in the DCI format.. Based on the above discussion, the UE behavior should be clarified for BWP switching. In our view, if different DCI formats are configured to be monitored in different BWPs, it seems that it should be regarded as an error case since the necessary RRC parameters are missing for the UE-specific transmission or reception in the target BWP. Otherwise, in order to ensure the UE-specific scheduling in the target BWP, the necessary RRC parameters should also be present in the target BWP, even if the triggering DCI format is not monitored in the target BWP. This could e.g. be achieved by gNB configuration or default values could be assumed. Alternatively, it can be restricted that BWP switching is only triggered by the DCI format(s) that are monitored in both BWPs. In other words, if a DCI format (i.e., DCI format x-1/x-2) is only configured in the active BWP but not in the target BWP, then it should not be used for triggering BWP switching. Looking at the pros and cons of the above two candidate solutions, it seems that limiting that BWP switching is only triggered by the DCI format(s) that are configured to be monitored in the current BWP and target BWP is better, thus we slightly prefer this solution. Proposal 1: To ensure a proper BWP switching, BWP switching is only triggered by the DCI format(s) that are configured to be monitored in both the active BWP and the target BWP.

Feature lead view: Based on outcome of the preparation phase, the follow conclusion is proposed for review, discussion, and revision if necessary.

Proposed Conclusion 4-1:

Dynamic BWP switching is only triggered by the DCI format(s) that are configured to be monitored in both the active BWP and the target BWP.

| Company | View |
| --- | --- |
| Feature lead | Note that at the end of the email discussion for preparation phase, Apple proposed the following revision as highlight in Red: If the UE is configured with both DCI format 0_1(resp, 1_1) and 0_2 (resp. 1_2), dynamic BWP switching is only triggered by the DCI format(s) that are configured to be monitored in both the active BWP and the target BWP. However, in my understanding the additional modification seems unnecessary, because this conclusion is applicable for all the cases and I don’t see any issue if we don’t add this kind of additional sentence. Even for DCI format 0_1/1_1, if it will be used for triggering BWP switching, it needs to be configured in both active BWP and the target BWP also. In addition, if we add this kind of description, it may give the impression that other cases doesn’t need to follow this rule? |
| Samsung | We do not agree to the proposed conclusion. There is no need for any conclusion. The argument is “Based on the above discussion, the UE behavior should be clarified for BWP switching” but the proposed conclusion “BWP switching is only triggered by the DCI format(s) that are configured to be monitored in both the active BWP and the target BWP” has nothing to do with UE behavior and is a restriction in NW operation. BWP switching can be very well supported for the case that DCI formats are different in each BWP – e.g. the BWP configurations can be identical except for the search space sets/DCI formats. A similar situation may even occur with SSSG as the resulting SS sets before and after SSSG can contain different DCI formats (for DCI 0_1/1_1 and DCI 0_2/_2). It is all a matter of a gNB providing proper configurations and there is no need to define UE behavior for each possible misconfiguration, especially when it is already clear. For this case, there is even the following captured in 38.213 “If a UE detects a DCI format with inconsistent information, the UE discards all the information in the DCI format.” |
| ZTE | The common sense is we don’t need any spec impact. Basically it means this issue is left to implementation. I am open to whether to have this conclusion or not. |
| DOCOMO | We are open for the conclusion. We understand the issue can be left to implementation and avoided by proper configurations by gNB. On the other hand, it may be beneficial if it is clarified in the spec. |
| Intel | No conclusion is necessary. We share the view from Samsung that this is mainly trying to address NW misconfiguration via imposing some restrictions. As noted by Samsung, in some cases, it may still be possible to schedule using DCI format A in original BWP such that the indicated resource allocation, etc. is still applicable in destination BWP that may not have DCI format A configured. |
| Vivo | In our understanding, the similar issue also exists in Rel-15, for example, DCI format 0_0/1_0 is only configured in BWP#0 while DCI format 0_1/1_1 is only configured in BWP#1. When UE receives DCI format 0_1 or 1_1 in BWP#1 to switch BWP from BWP#1 to BWP#0, UE would also not be able to interpret the DCI fields for the processing of the PDSCH/PUSCH in the target BWP. We are open for the conclusion. |
| Nokia/NSB | Agree with Samsung & Intel. |
| OPPO | We are open for the conclusion although we think it is unnecessary for the reasons mentioned above. |
| Hw/HiSi | We support the conclusion. The purpose is that we would like to ensure that the gNB and UE have the same understanding what RRC parameters to take into account when performing a BWP switch. According to the comment from Samsung a BWP switch can also be performed if the same DCI format is not monitored in both the active and the target BWP if their configurations are the same. However, during the preparation phase several companies expressed that the triggering DCI format should be monitored in both the active and target BWP, otherwise it is a misconfiguration. There seems to be some confusion and a conclusion would be needed to achieve clarity. It still seems that there are two possible UE behaviors when a UE has been provided with RRC parameters for a DCI format in the target BWP but the DCI format is not configured to be monitored. Shall the UE take these RRC parameters into account when performing the BWP switch or shall it not? As a side comment/question, we are wondering about the situation described by Samsung: when the UE has detected the BWP switch trigger in the active BWP and identified the target BWP, doesn’t it then need to take the RRC parameters from the target BWP into account? And if the RRC parameters for the triggering DCI are absent in the target BWP, can the UE then still know how to schedule PDSCH/PUSCH in the target BWP? |
| Qualcomm | We are open to make a conclusion to clarify the issue. One question we’d like to clarify: some companies think that the following spec “If a UE detects a DCI format with inconsistent information, the UE discards all the information in the DCI format” defines the UE behavior in the concerned scenarios. We would like to understand what does the “inconsistent information” refer to in the case of BWP switching from BWP#0 with DCI format 0_2/1_2 configured to BWP#1 with DCI format 0_2/1_2 not configured? |
| Apple | Drawing a conclusion is better than not. |

# R1-2201075: Discussion on SRS resource indication for Type 1 CG

Vivo R1-2201075 SRS resource set(s) with higher layer parameter usage in SRS-ResourceSet set to ‘codebook’ or ‘nonCodebook’ used for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 are separately configured by srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2. As stated in TS 38.212, SRS resource set configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2 is composed of the first NSRS, 0_2 SRS resources together with other configurations in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value ‘codeBook’ or ‘nonCodeBook’, respectively, except for the higher layer parameters ‘srs-ResourceSetId’ and ‘srs-ResourceIdList’. In our understanding, “if any” implies that it is possible to configure only srs-ResourceSetToAddModListDCI-0-2 without configuring srs-ResourceSetToAddModList, which is a valid use case for URLLC-only services. In current TS 38.214, SRS resource indication for Type 1 CG is determined by srs-ResourceSetToAddModList which is configured for DCI format 0_1. When there is no SRS resource set with usage set to ‘codebook’ or ‘nonCodebook’ configured by srs-ResourceSetToAddModList while SRS resource set configured by srs-ResourceSetToAddModListDCI-0-2, however, the UE behaviour of type 1 CG is not clear. So, we propose to allow SRS resource indication for type 1 CG to be determined by the SRS resource set configured by srs-ResourceSetToAddModListDCI-0-2 when srs-ResourceSetToAddModList is not configured with SRS resource set with usage set to ‘codebook’ or ‘nonCodebook’. SRS resource indication for Type 1 CG is determined by the SRS resource set configured by srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to ‘codebook’ or ‘nonCodebook’ when there is no SRS resource set with higher layer parameter usage in SRS-ResourceSet set to ‘codebook’ or ‘nonCodebook’ configured by srs-ResourceSetToAddModList.

Feature lead view: Based on outcome of the preparation phase, the follow conclusion is proposed for review, discussion, and revision if necessary.

Proposed Conclusion 5-1:

SRS resource indication for Type 1 CG is determined by the SRS resource set configured by srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to ‘codebook’ or ‘nonCodebook’,  regardless of whether DCI format 0_1 is configured to be monitored or not.

| Company | View |
| --- | --- |
| Samsung | Fine with the proposal. |
| ZTE | Fine with the proposal |
| DOCOMO | Fine with the proposal |
| Intel | Fine with the proposal. |
| ASUSTeK | Fine with the proposal. |
| Vivo | Fine with the proposal. |
| Nokia/NSB | Fine with the proposal. |
| OPPO | Fine with the proposal. |
| HW/HiSi | Fine with the proposal. |
| Qualcomm | Fine with the proposal. |
| Apple | Fine with the proposal. |

# References

- R1-2201821: 38.213 corrections on RRC parameter for URLLC (ASUSTeK)

- R1-2201823: 38.214 corrections on RRC parameter for URLLC (ASUSTeK)

- R1-2202439: Corrections on BWP switching using DCI format 0_2/1_2 (Huawei, HiSilicon)

- R1-2201075: Discussion on SRS resource indication for Type 1 CG (vivo)