3GPP TSG RAN WG1 #108-e						R1-220xxxx

e-Meeting, February 21th – March 3rd, 2022

Agenda item:	7.2.6

Source: 	Moderator (Samsung)

Title: 	Summary for Rel.16 NR eMIMO maintenance

Document for:	Discussion and Decision

- Introduction

The moderator summary of the maintenance-related issues raised in the submitted contributions for Rel.16 NR_eMIMO maintenance is given below. The listed maintenance issues are under the usual designations:

- LP: low-PAPR RS

- MB: Multi-beam operation

- MT: Multi-TRP

- MU: Type-II enhancement for MU-CSI

- UL: UL full power transmission

- O: Other

An initial assessment on each of the issues is given (but can be revised based on the outcome of the discussion during the preparation week). The assessment will be used as a basis to select four issues (per chairman instruction) for further discussion in the upcoming weeks.

- High priority (H): this includes high-priority item (essential, pending issues, broken spec components) and proposed editorial changes that either enhance the clarity of the specs or correct mistakes

- Non-essential (N): this includes all other purposes such as spec optimization and low priority issues

- Editorial (E): this includes editorial issues that will be handled as editorial CRs (to be communicated to the editors/chairs) and thereby not counted toward the four-thread quota

- Maintenance issues

The issues are summarized in the following table:

Table 1 Summary

| # | Issue (summary of CR proposal) | Companies | Initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| MT.1 | In 38.215 section 5.1.5, clarify that in the case of s-DCI based mTRP with scheme 3 or scheme 4, the indicated TCI states is the activated TCI states in the first slot with scheduled PDSCH (R1-2201631, R1-2201632) FL: The issue identified in the problem is valid. However, whether specification change is needed might need some discussion. The current spec with the words “single slot PDSCH” and “multi-slot PDSCH” seems cover this issue. | vivo | H |  |
| MU.1 | Correction on eType-II PMI indexing and reporting (R1-2201327) TS 38.214 section 5.2.3 For Type II CSI feedback, Part 1 contains RI (if reported), CQI, and an indication of the number of non-zero wideband amplitude coefficients per layer for the Type II CSI (see Clause 5.2.2.2.3). The fields of Part 1 – RI (if reported), CQI, and the indication of the number of non-zero wideband amplitude coefficients for each layer – are separately encoded. Part 2 contains the PMI and LI (if reported) of the Type II CSI. The elements of i1,4,l, i2,1,l (if reported) and i2,2,l (if reported) are reported in the increasing order of their indices, i=0,1,…, 2L-1, where the element of the lowest index is mapped to the most significant bits and the element of the highest index is mapped to the least significant bits. Part 1 and 2 are separately encoded ... - Group 0 includes indices i1,1 (if reported), i1,2 (if reported) and i1,8,l (l=1,…,υ). FL: Proposed CR is correct and editorial in nature | CATT | E |  |
| MU.2 | Correction on eType-II freq granularity (R1-2201993) Adding the following in TS 38.214 section 5.2.1.4 (based on Rel-17 agreement of new condition for wideband reporting, the following is proposed for Rel-16 eType-II) reportQuantity is set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI', codebookType is set to 'typeII -r16' or 'typeII-PortSelection-r16' with M_υ=1 and cqi-FormatIndicator is set to 'widebandCQI', or FL: Not essential and could cause some late implementation change. Since this was proposed in past meeting(s) and some conclusion is needed – either without or after email discussion) | Samsung | N |  |
| MU.3 | NZC partitioning in group 1 and 2 for eType-II (R1-2202121, R1-2202314/2315) Proposal to change coefficient partitioning for eType-II based on agreement for Rel-17 Type-II codebook Proposal: For Rel-16 Enhanced Type II and Enhanced Type II port-selection, clarify that UCI Group 1 includes the max(0,⌈K^NZ/2⌉-υ) highest priority elements of i_(2,4,l) and the max(0,⌈K^NZ/2⌉-υ) highest priority elements of i_(2,5,l) (l=1,…,υ). UCI Group 2 includes the min(K^NZ-v,⌊K^NZ/2⌋) lowest priority elements of i_(2,4,l) and the min(K^NZ-v,⌊K^NZ/2⌋) lowest priority elements of i_(2,5,l) (l=1,…,υ) Corrections are proposed in TS 38.212 (sec 6.3.2.1.2) and 214 (section 5.2.3): - Changing KNZ/2-υ to max0,KNZ2-υ - Changing KNZ/2 to minKNZ-ν,KNZ2 FL: The proposal seems technically correct although the identified problems could be corner cases (hence may be non-essential). This benefits from some discussion and needs some conclusion. | Qualcomm, Nokia/NSB | H |  |

- Discussion and proposal

From the inputs shared by participating companies during the preparation phase, the following observation can be made:

- The following issues can be handled as E (a part of editorial CR):

- The following issues can be designated as H (requiring discussion and additional agreements/conclusions):

# References

| 1 | R1-2201327 | Draft CR on Type II and enhanced Type II CSI feedback | CATT |
| --- | --- | --- | --- |
| 2 | R1-2201631 | Draft CR on indicated TCI states for single-DCI based MTRP schemes | vivo |
| 3 | R1-2201632 | Draft CR on indicated TCI states for single-DCI based MTRP schemes | vivo |
| 4 | R1-2201993 | Correction on frequency granularity of CSI based on Rel.16 Type II codebooks | Samsung |
| 5 | R1-2202121 | Discussion on coefficients partition in eT2 CSI | Qualcomm Incorporated |
| 6 | R1-2202314 | Correction in the number of NZC mapped in Group 1 and 2, 38.212 | Nokia, Nokia Shanghai Bell |
| 7 | R1-2202315 | Correction in the number of NZC mapped in Group 1 and 2, 38.214 | Nokia, Nokia Shanghai Bell |