3GPP TSG RAN WG1 #108-e			R1-2202607

e-Meeting, February 21th – March 3rd, 2022

Agenda item:	8.1.1

Source: 	Moderator (Samsung)

Title: 	Moderator Summary#2 for Maintenance on Rel-17 Multi-Beam: ROUND 1

Document for:	Discussion and Decision

## Introduction

In this summary, the term “item 1” refers to the first item in the Rel.17 NR FeMIMO WID, i.e. multi-beam enhancement:

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1: - Identify and specify features to facilitate more efficient (lower latency and overhead) DL/UL beam management for intra-cell and inter-cell scenarios to support higher UE speed and/or a larger number of configured TCI states: - Common beam for data and control transmission/reception for DL and UL, especially for intra-band CA - Unified TCI framework for DL and UL beam indication - Enhancement on signaling mechanisms for the above features to improve latency and efficiency with more usage of dynamic control signaling (as opposed to RRC) - For inter-cell beam management, a UE can transmit to or receive from only a single cell (i.e. serving cell does not change when beam selection is done). This includes L1-only measurement/reporting (i.e. no L3 impact) and beam indication associated with cell(s) with any Physical Cell ID(s) - The beam indication is based on Rel-17 unified TCI framework - The same beam measurement/reporting mechanism will be reused for inter-cell mTRP - This work shall only consider intra-DU and intra-frequency cases - Identify and specify features to facilitate UL beam selection for UEs equipped with multiple panels, considering UL coverage loss mitigation due to MPE, based on UL beam indication with the unified TCI framework for UL fast panel selection

This summary includes the following:

- Observation and proposal

- Summary of current companies’ positions on each of the aspects within the category

## Summary of companies’ inputs

### Issue 1 (Rel.17 unified TCI framework – note: for intra-cell beam management unless otherwise noted)

Table 1 Summary: issue 1

| # | Issue | Companies’ views |
| --- | --- | --- |
| 1.9 | On Rel-17 unified TCI framework, for P/SP-CSI-RS, the UE assumes that: - Alt2. Whether to apply the indicated Rel-17 TCI state is configured per CSI-RS resource by RRC – if not applied, use the legacy MAC-CE signalling mechanism - Alt3. The indicated Rel-17 TCI state is never applied, i.e. the legacy RRC/MAC-CE signalling mechanism is always used - Alt4. The indicated Rel-17 TCI state is applied when the UE is not configured with any TCI state for the P/SP CSI-RS FL Note: Open issue that needs to be resolved. From FL perspective, I agree with companies who stated that if there is no additional consensus on this issue, Alt3 is the default scheme/outcome. | Alt2: Fraunhofer IIS/HHI (2nd pref.), LG, Nokia/NSB, Samsung, CATT, Lenovo/MotM Alt3: MTK (add RRC), Qualcomm, OPPO, Xiaomi, ZTE, Spreadtrum, vivo, Futurewei, Huawei/HiSi Alt4: Apple, Ericsson, NTT Docomo, Fraunhofer IIS/HHI, TCL, CMCC, Intel |
| 1.12 | On Rel.17 unified TCI framework, for Rel-17 unified TCI, for DL channels/signals that share the same indicated Rel-17 TCI state as UE-dedicated reception on PDSCH/PDCCH (via Rel-17 MAC-CE/DCI TCI state update), the following option on source RSs and QCL-Types is also supported: - Option 3: CSI-RS for CSI is configured for QCL-TypeA and QCL-TypeD source RS FL Note: It was explained that the so-called “circular” issue is avoided in practice via NW implementation, i.e. NW will not configure the same CSI-RS for CSI both as source and target RSs. Need conclusion. | Support/fine: Sony, CMCC, Ericsson, Qualcomm, NTT Docomo, Fraunhofer IIS/HHI, Nokia/NSB, TCL, CMCC, CATT, ZTE, Spreadtrum, vivo, Futurewei, Intel, ZTE, Lenovo/MotM, Samsung Not support: Apple |
| 1.13 | For cross-carrier scheduling, support cross-carrier DCI-based TCI state indication FL Note: Spec impact of this proposal is unclear. Before this is fully clarified by the proponents, the discussion is suspended. So far only Qualcomm has a concrete proposal. | Support/fine: Qualcomm (extra latency for xCC beam indication), Samsung, Nokia/NSB, Xiaomi, CMCC, CATT, vivo, Futurewei, Huawei/HiSi Not support: Ericsson (no spec impact), Intel (already supported), MTK (support by default), NTT Docomo (supported by default), Fraunhofer IIS/HHI (supported by default), Lenovo/MotM (by default) |
| 1.14 | Proposed conclusion: On path-loss measurement for Rel.17 unified TCI framework, when both PL-RS and spatial relation RS in the UL or (if applicable) joint TCI state are not the same, whether and how to define the event(s) of “beam alignment” is left to RAN4. FL Note: Any additional event (bullet) doesn’t seem acceptable for a number of companies. Even the above, some still have concern. Need conclusion or leave to RAN4. | Support/fine (original FL proposal in ROUND 0): MTK, Samsung, Qualcomm, NTT Docomo, Nokia/NSB, TCL, CMCC, CATT, vivo, Futurewei, Lenovo/MotM Not support: Ericsson (leave to RAN4) Intel (leave to RAN4), ZTE, Huawei/HiSi |
| 1.15 | Support to report virtual PHR based on the power control parameters associated with indicated TCI state for PUSCH/PUCCH transmission. | Support/fine: Apple, ZTE Not support: Intel, Samsung, Qualcomm |

Table 2 Additional inputs: issue 1

| Company | Input |
| --- | --- |
| Mod V0 | - Check and update your view in Table 1 - Issue 1.13, if proponents cannot come up with a concrete proposal (feel free to discuss offline), I will suspend this issue from discussion. Most companies think that this is already supported. - Share more inputs here if needed |
| Qualcomm | For 1.9, support Alt3 For 1.12, support For 1.13, below is our proposal. Note that the UE capability is defined from end of DCI to the application time Proposal : If the TCI updating DCI has smaller SCS than the applied channel(s), the time gap between DCI and the application time should be no less than the corresponding UE capability plus an additional value to account for extra DCI decoding latency. - Value may reuse the additional beam switching timing delay d defined in 38.214 Table 5.2.1.5.1a-1. Agreement On Rel-17 DCI-based beam indication, regarding application time of the beam indication, the first slot that is at least X ms or Y symbols after the last symbol of the acknowledgment of the joint or separate DL/UL beam indication. Note: The gap between the last symbol of the beam indication DCI and that first slot shall satisfy the UE capability For 1.14, prefer to clarify in RAN1. RAN4 does not know the context and may send LS for RAN1 to clarify. Without any clarification, this capability may not work well For 1.15, not critical |

### Issue 2 (inter-cell beam management)

Table 3 Summary: issue 2

| # | Issue | Companies’ views |
| --- | --- | --- |
| 2.1 | For the already agreed NW-controlled inter-cell beam reporting, support reporting L1-RSRP for the subset of configured SSBs detected during the L3 measurement | Support/fine: Ericsson, vivo Not support: Qualcomm, Nokia/NSB (RAN4 issue), Samsung, OPPO, Xiaomi, CMCC, CATT, Spreadtrum, ZTE (UE implementation), Lenovo/MotM, MTK (supportive but RAN4) |
| 2.4 | MAC CE activates non-serving cell SSBs for measurement FL note: This was discussed several times before and needs conclusion | Support/fine: ZTE, Apple, Qualcomm, NTT Docomo, Xiaomi (for AP), CMCC, CATT, vivo, Futurewei Not support: MTK, Ericsson (already supported implicitly), Samsung (already supported implicitly), OPPO, LG , Intel, Spreadtrum, Lenovo/MOtM (implicit), Huawei/HiSi (implicit) |
| 2.5 | - For inter-cell cases, default beam mechanism should be determined separately. - For non-UE-dedicated DL channels/RSs, reuse legacy default beam mechanism defined in Rel-15/16 to obtain their QCL assumption respectively; - For UE-dedicated DL channels/RSs, follow the previous indicated TCI-state-r17; | Support/fine: vivo Not support: QC (always use indicated TCI) |
| 2.6 | For inter-cell case with one TCI pool configured within a set of CCs, when different PCIs are associated with the TCI states in different CCs, it should be allowed that the same TCI state ID can refer to different PCI on different CCs. | Support/fine: vivo Not support: QC (NW implementation) |
| 2.7 | PDCCH/PDSCH is rate matched around the SSBs configured for L1-RSRP measurement, besides SSBs associated with the same PCI as that of the activated/indicated TCI state of the PDCCH/PDSCH. | Support/fine: vivo, QC Not support: |

Table 4 Additional inputs: issue 2

| Company | Input |
| --- | --- |
| Mod V0 | - Check and update your view in Table 3 - Added 2.5/6/7 per vivo’s request at the end of ROUND 0 (please see vivo’s explanation below and share your view) - Share more inputs here if needed |
| vivo | Issue 2.5: default beam mechanism in inter-cell case DL channels/RSs are divided into two categories in Rel-17, non-UE-dedicated and UE-dedicated, where in inter-cell cases, non-UE-dedicated DL channels/RSs are defined as the DL channels/RSs associated with non-UE-dedicated CORESET and follow R15/16 TCI state deactivation/activation and indication mechanism, while the QCL assumption of DL UE-dedicated channels/RSs follows the indicated TCI-state-r17. Therefore, the default QCL assumption of DL channels/RSs should also be determined separately. - For UE-dedicated DL channels/RSs, they follow the previous indicated TCI-state-r17; - For non-UE-dedicated channels/RSs, reuse legacy default beam mechanism defined in Rel-15/16 to obtain their QCL assumption respectively. - Proposal: For inter-cell cases, default beam mechanism should be determined separately. - For non-UE-dedicated DL channels/RSs, reuse legacy default beam mechanism defined in Rel-15/16 to obtain their QCL assumption respectively; - For UE-dedicated DL channels/RSs, follow the previous indicated TCI-state-r17; Issue 2.6: beam indication across CCs For inter-cell case with one TCI pool configured within a set of CCs, when different PCIs are associated with the TCI states in different CCs, it should be allowed that the same TCI state ID can refer to different PCI on different CCs. - Proposal: For inter-cell case with one TCI pool configured within a set of CCs, when different PCIs are associated with the TCI states in different CCs, it should be allowed that the same TCI state ID can refer to different PCI on different CCs. Issue 2.7: rate match for PDSCH and PDCCH PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI, which means per PCI rate match, has been agreed in AI.8.1.2.2. From UE measurement perspective, when UE is configured to measure on SSBs while still receiving PDSCH on overlapped resources, there would be performance degradations and additional UE complexities to guarantee the corresponding performance. In legacy UE implementation, there is no such simultaneous L1-RSRP measurement and PDSCH reception on the same RE case. We think the same rule also apply to inter-cell measurement in Rel-17. Therefore, PDCCH/PDSCH should also be rate matched around the SSBs configured for L1-RSRP measurement, besides SSBs associated with the same PCI as that of the activated/indicated TCI state of the PDCCH/PDSCH. |
| Qualcomm | For 2.1, not support, it does not work as mentioned before For 2.4, support For 2.5, not support. Support indicated TCI as default beam always. Not clear how to define separate default beams for dedicated and non-dedicated signal For 2.6, the proposal can be achieved by NW implementation to our understanding For 2.7, fine to support |

### Issue 3 (signaling medium)

Table 5 Summary: issue 3

| # | Issue | Companies’ views |
| --- | --- | --- |
| 3.2 | Proposal 3.B: On Rel-17 MAC-CE-based and DCI-based beam indication, regarding application time of cross-carrier beam indication, in RAN1#108-e, further discuss and select one from the following alternatives for BAT configuration across CCs when common TCI state ID update is not configured/supported: - Alt1. The BAT is configured per-CC - Alt2. Use the same scheme as that with common TCI state ID update, i.e. a common BAT is determined by the smallest SCS among all the applied CC(s) in a band - Alt3. A BAT list is configured under the cell group config and applied for each CC in the CG. For CCs not configured with a common TCI state ID update, the BAT is determined by the SCS of the active BWP of the CC. FL Note: Discussed offline [1]. | Alt1: Huawei/HiSi, NTT Docomo, Xiaomi, Ericsson (no additional restriction), Samsung, CMCC, Intel (when common TCI state ID update is not configured/supported), MTK (also for non-CA case), NEC, CATT, OPPO, LG, CMCC, Nokia/NSB, TCL, IDC, Spreadtrum Alt2: Qualcomm, ZTE, Apple, Lenovo/MotM, ZTE, Lenovo/MotM Alt3: vivo, Qualcomm |
| 3.3 | On Rel-17 MAC-CE-based and DCI-based beam indication, regarding the CC list for common TCI state ID update and activation, the maximum number of CC lists can be configured is [X] FL Note: Please propose X | Value of X: |
| 3.5 | Proposal 3.D: For DCI format 1_1 and 1_2 with PDSCH assignment indicating TCI state, the acknowledgement to the TCI state update is the ACK of the PDSCH | Support/fine: OPPO, Qualcomm, NTT Docomo, NEC, Xiaomi, TCL, CMCC, Intel, ZTE, vivo, Futurewei, Lenovo/MotM, Spreadtrum, Qualcomm Not support: Huawei/HiSi (add “or NACK”) |
| 3.6 | The value range of RRC configured BAT (beamAppTime-r17) - {0, ..., 14} from NTT Docomo | Support/fine: NTT Docomo Other proposals: - {7, 14, 28, 42, 56, 70, 84, 98}): MTK, Ericsson, Samsung, Qualcomm, NTT Docomo, ZTE - In addition, add smaller values {1, 2, 4}: Samsung, NTT Docomo, vivo (fine with 0) - {24, 28, 42}: Apple |
| 3.7 | Proposal 3.E: For Rel-17 unified TCI framework, for the presence of TCI field in DCI format 1-1/1-2, in RAN1#108-e, down-selection from one of the following alternatives: - Alt1: Reuse tci-PresentInDCI to configure TCI field per CORESET - Alt2: Introduce a new RRC parameter to configure TCI field per BWP or per CC - Alt3: TCI field is always present in DCI format 1_1/1_2, UE ignores this bit field if one single TCI codepoint is activated FL Note: The proponents note that there is no RRC parameter like tci-PresentInDCI to make the TCI field configurable. Even if the majority view is based on tci-PresentInDCI, RAN1 still needs an agreement on this. From FL perspective, this comment is valid. | Alt1: Huawei/HiSi, Apple, Ericsson (for single activated TCI state), NTT Docomo, OPPO, LG, TCL, CMCC, CATT, ZTE, Spreadtrum, vivo, Lenovo/MotM Alt2: Alt3: MTK, Samsung, Qualcomm, Nokia/NSB, Intel |
| 3.9 | Regarding TCI indication by DCI without DL assignment, for type-1 HARQ-ACK codebook determination, virtual PDSCH is assumed in the same slot of the DCI by UE. | Support/fine: ZTE, Nokia/NSB, Lenovo/MotM (need discussion) Not support: Qualcomm, Apple, OPPO, TCL, CATT, Intel, vivo |
| 3.10 | For DCI formats 1_1 and 1_2 without DL assignment, the UCI carrying the HARQ feedback should be mapped to high priority HARQ codebook and PUCCH resources associated with priority index 1 when the UE is configured with two priority indexes. If UE is configured with single priority index, the UCI carrying the HARQ feedback for beam indication should be prioritized over other UCI. | Support/fine: Intel Not support: Ericsson (not essential), Qualcomm (no need), OPPO, ZTE, vivo |
| 3.11 | Proposal 3.B.1: On Rel-17 DCI-based beam indication, regarding application time of the beam indication for non-CA, the BAT is configured per-CC | Support/fine: MTK, Samsung, Intel, Huawei/HiSi Not support: Qualcomm (leave to RAN2) |

Table 6 Additional inputs: issue 3

| Company | Input |
| --- | --- |
| Mod V0 | - Check and update your view in Table 5 - 3.5: Proponents to check Huawei’s comment below on adding “or NACK” and comment if it is ok - 3.9: Opposing companies to check ZTE’s argument below. If nothing changes I will suspend this issue from discussion - 3.10: Opposing companies to check Intel’s argument below. If nothing changes I will suspend this issue from discussion - Share more inputs here if needed |
| Intel | Issue 3.10: If the ACK for beam indication is not mapped to priority 2, it may be dropped and then the gNB and UE will not maintain common understanding of the beam indication. This would mean that the gNB has to re-transmit the DCI which would be a waste of resources and possibly cause misalignment of beams due the increased latency of beam indication. Therefore, we think that the ACK for beam indication should always be prioritized. |
| ZTE | 3.9: If the virtual PDSCH is determined according to SLIV+K0 in the TDRA (which may be misled by the first highlighted part in the following agreement), and then based on the second highlighted part that ACK is reported in a PUCCH k slots after the end of the PDCCH reception where k is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format, for HARQ-ACK codebook determination, K1 for determining candidate PDSCH reception becomes ‘PDSCH-to-HARQ_feedback timing – K0’, which may be out of candidate list for semi-static HARQ-ACK codebook generation (i.e., dl-DataToUL-ACK, dl-DataToUL-ACK-ForDCIFormat1_2 for providing K1). More details can be found in R1-2101185. Agreement (RAN1#104be) For beam indication with Rel-17 unified TCI, support DCI format 1_1/1_2 without DL assignment: Use ACK/NACK mechanism analogous to that for SPS PDSCH release with both type-1 and type-2 HARQ-ACK codebook: Upon a successful reception of the beam indication DCI, the UE reports an ACK Note that upon a failed reception of the beam indication DCI, a NACK can be reported. For type-1 HARQ-ACK codebook, a location for the ACK information in the HARQ-ACK codebook is determined based on a virtual PDSCH indicated by the TDRA field in the beam indication DCI, based on the time domain allocation list configured for PDSCH For type-2 HARQ-ACK codebook, a location for the ACK information in the HARQ-ACK codebook is determined according to the same rule for SPS release The ACK is reported in a PUCCH k slots after the end of the PDCCH reception where k is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format, or provided dl-DataToUL-ACK or dl-DataToUL-ACK-ForDCI-Format1-2-r16 if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI … Figure 1 Application time of TCI state indication (i.e., Y symbols after ACK) for semi-static HARQ-ACK codebook, where virtual PDSCH is assumed in the same slot of the DCI by UE So we suggest the proposal to address the issue: Regarding TCI indication by DCI without DL assignment, for type-1 HARQ-ACK codebook determination, virtual PDSCH is assumed in the same slot of the DCI by UE. It should be noticed that the above is aligned with SPS-PDSCH-release. |
| Huawei, HiSilicon | Proposal 3.D: This proposal goes against the agreement in RAN1#103e that the ACK/NAK of the PDSCH scheduled by the DCI carrying the beam indication can be used as an ACK also for the DCI (copied below). Agreement On beam indication signaling medium to support joint or separate DL/UL beam indication in Rel.17 unified TCI framework: - Support L1-based beam indication using at least UE-specific (unicast) DCI to indicate joint or separate DL/UL beam indication from the active TCI states - The existing DCI formats 1_1 and 1_2 are reused for beam indication - Support a mechanism for UE to acknowledge successful decoding of beam indication - The ACK/NAK of the PDSCH scheduled by the DCI carrying the beam indication can be used as an ACK also for the DCI - FFS: Whether any additional specification support is needed. We suggest updating the proposal as: - Proposal 3.D: For DCI format 1_1 and 1_2 with PDSCH assignment indicating TCI state, the acknowledgement to the TCI state update is the ACK or NACK of the PDSCH |
| QC | For Proposal 3.B, support Alt2 or Alt3. Also, fine to leave to RAN2 For Proposal 3.3, two lists same as legacy For Proposal 3.D, support. NACK does not work For Proposal 3.E, support Alt3 For Proposal 3.9, we are fine to at least keep neutral after ZTE’s offline explanation For Proposal 3.10, not support For Proposal 3.B.1, leave to RAN2 for the best place |

### Issue 4 (MP-UE)

Table 7 Summary: issue 4

| # | Issue | Companies’ views |
| --- | --- | --- |
| 4.1 | Proposal 4.A: Confirm the following working assumption as an agreement with the following refinement (highlighted in red): Support the UE reporting a list of UE capability value sets - Each UE capability value set comprises the max supported number of SRS ports - For any two different value sets, at least one capability value needs to be different - FFS: In addition also identical value sets are allowed. - Whether the UE capability value set can be common across all BWPs/CCs in same band or BC can be discussed in UE feature session FL Note: Discussed offline [1] | Support/fine: MTK, Apple, Nokia/NSB, Fraunhofer IIS/HHI, NTT Docomo, NEC, LG, Qualcomm, OPPO, Xiaomi, LG, CMCC, Spreadtrum, vivo, CATT, Lenovo/MotM, TCL, Huawei/HiSi Not support: Ericsson, Samsung (not support FFS) Intel (do not support identical value sets), Qualcomm (not support identical values) |
| 4.2 | Proposal 4.B: On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, - From the perspective of UE capability, maximum number of supported UL Tx layers = min{maximum number of SRS ports for a reported set, maximum number of UL Tx layers reported by UE capability} FL Note: Discussed offline [1] | Support/fine: MTK, Apple, Nokia/NSB, NTT Docomo, NEC, LG (in principle), OPPO, Xiaomi, LG, CMCC, CATT, ZTE, Spreadtrum, Lenovo/MotM, Huawei/HiSi Not support: Ericsson (no need to discuss), Samsung, Intel, vivo (spec impact unclear), Qualcomm (no spec impact) |
| 4.3 | Proposal 4.C: On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, UE can report one index of UE capability value set for each reported CRI/SSBRI in one beam reporting. FL Note: Discussed offline [1] | Support/fine: MTK, Apple, Nokia/NSB, Ericsson, Qualcomm, Fraunhofer IIS/HHI, NTT Docomo, NEC, LG, Samsung, Xiaomi, CMCC, CATT, IDC, Intel, ZTE, Spreadtrum, vivo, TCL, Huawei/HiSi Not support: MotM/Lenovo |
| 4.4 | Proposal 4.D: On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, one value of the max supported number of SRS ports (e.g., 0) is reserved to indicate the DL-only panel FL Note: Discussed offline [1] | Support/fine: MTK, Ericsson, Qualcomm, NTT Docomo, LG, Xiaomi, CMCC, CATT, Lenovo/MotM Not support: Apple, Nokia/NSB, Fraunhofer IIS/HHI, NEC, Samsung, OPPO, Intel, Spreadtrum, vivo, ZTE |
| 4.5 | Proposal 4.E: On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, all types of time-domain behavior, i.e., periodic, semi-persistent, and aperiodic reporting, are supported for the enhanced beam report with index(es) of UE capability value set. FL Note: Discussed offline [1] | Support/fine: MTK, Nokia/NSB, Ericsson, Qualcomm (without sub-bullets), Fraunhofer IIS/HHI, NTT Docomo, NEC, LG, Samsung (without sub-bullets), OPPO (without sub-bullets), CMCC, CATT (without sub-bullets), IDC, Intel (without sub-bullets), ZTE (without sub-bullets), Spreadtrum, vivo (without sub-bullets), Lenovo/MotM (without sub-bullets), TCL, Huawei/HiSi Not support: Apple (On SP/AP report) |
| 4.6 | Proposal 4.F: On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, regarding acknowledgement mechanism of the reported correspondence from NW to UE, down-select the following alternatives: - Alt-1: Being based on TCI state activation/update mechanism where the activated TCI state includes reported RS (SSBRI or CSI-RS) [and is additionally associated with the index of UE capability value set]; - Alt-2: A dedicated SS can be configured to send the ACK, which is like PCell-BFR. - Alt-3: A scheme based on the BFR response in SCell BFR - Alt-4: acknowledgement mechanism is not supported. - Alt-5: use the indicated SRS resource set matching the reported SRS port # - Alt-6: No spec impact FL Note: Discussed offline [1]. If there is no consensus, Alt-4 becomes the default outcome. Need to conclude this meeting. | Alt1: MTK, NTT Docomo, Nokia/NSB, Samsung, ZTE Alt2: OPPO, CMCC, Intel Alt3: OPPO, CMCC, Intel Alt4: Apple, Ericsson, CATT, Spreadtrum, Huawei/HiSi Alt5: Qualcomm, Alt6: vivo |
| 4.7 | Proposal 4.G: On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, regarding how to update the number of SRS ports according to UE reporting, in RAN1#108-e, down-select the following alternatives: - Alt1: via UL BWP switching where each UL BWP has different number of SRS ports - FFS: BWP fallback mechanism which would let NW to control UE panel, i.e. switch to a specific UE panel or panel type when timer expires. - Alt2: via SRS resource set selection by DCI where each set has different number of ports - Note1: ‘SRS resource set indicator’ is already specified in DCI format 0_1/0_2 and it provides functionality to select one SRS resource set by the DCI between two SRS resource sets configured by RRC - Note2: TPMI/TRI mapping for varying number of SRS ports is already specified for fullpowerMode2. - Alt3: via TCI state update/activation mechanism with two options - Opt1: MAC-CE based TCI state update is targeted to a certain SRS resource set that the SRS resource set if the target set is different from the previous used set. - Opt2: UE can assume that the SRS resource set which can be triggered (aperiodic) or which is to be used (periodic or semi-persistent) is the one that is associated with the same capability set index as the UE provided in the report together with the reported RS and included in the activated TCI state. - FFS: Any other RRC parameters, e.g., the maximum number of UL layers, codebook subset, uplink full power mode, configuration of SRS for antenna switching and so on, may need to be updated simultaneously with the number of configured SRS ports. FL Note: Discussed offline [1] | Alt1: MTK (no spec impact), Nokia/NSB, vivo Alt2: Qualcomm, NTT Docomo, NEC, LG, Samsung, OPPO, Xiaomi, CMCC, IDC, ZTE, Lenovo/MotM, Spreadtrum, Huawei/HiSi Alt3: Not support: Apple, Ericsson |

Table 8 Additional inputs: issue 4

| Company | Input |
| --- | --- |
| Mod V0 | - Check and update your view in Table 7 - Share more inputs here if needed |
| QC | For Proposal 4.A, not support the identical values. Not clear the use case For Proposal 4.B, no spec impact is needed. It should be common sense For Proposal 4.C, support For Proposal 4.D, support For Proposal 4.E, support For Proposal 4.F, support Alt5. For Proposal 4.G, support Alt2 |

### Issue 5 (MPE)

Table 9 Summary: issue 5

| # | Issue | Companies’ views |
| --- | --- | --- |
| 5.1 | On Rel-17 enhancements to facilitate MPE mitigation, the SSB/CSI-RS resource set associated with P-MPR reporting should be also associated with L1-RSRP/SINR reporting | Support/fine: MTK, Samsung Not support: vivo, ZTE, Qualcomm, LG (unclear), Huawei/HiSi (unclear) |
| 5.2 | The beam-specific P-MPR should be triggered when the P-MPR for indicated UL/joint TCI met legacy condition defined in 38.321, i.e. P-MPR for the indicated TCI is above mpe-Threshold or P-MPR change for this TCI is above phr-Tx-PowerFactorChange | Support/fine: Apple Not support: vivo (change beam to panel), ZTE (already supported), Samsung, Qualcomm, LG (change beam to panel), Huawei/HiSi (RAN2/4) |
| 5.3 | For PHR report to facilitate MPE mitigation, reported PCMAX, PH and P-MPR parameters can be associated with the cell which the reported SSBRI/CRI is associated with | Support/fine: NEC, ZTE Not support: vivo, Samsung, Qualcomm, Huawei/HiSi |
| 5.2 | Limit the maximum number of P-MPR value larger than mpe-Threshold and without any available SSBRI/CRI to 1. | Support/fine: Xiaomi Not support: vivo, ZTE, Samsung, Qualcomm, Huawei/HiSi |

Table 10 Additional inputs: issue 5

| Company | Input |
| --- | --- |
| Mod V0 | - Check and update your view in Table 9 - Share more inputs here if needed |

# References