3GPP TSG RAN WG1 Meeting #108-e 	R1-2202917

e-Meeting, February 21th  – March 3rd, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | xxx | rev | x | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RAN1 IoT NTN additions for TS 36.300 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN |  |  |  |  |  | Date: |  |  | 2022-02-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of IoT NTN support for NB-IoT and eMTC |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN1 additions to TS 36.300 for IoT NTN support: - Impact on timing aspects - A description of the UE autonomous TA pre-compensation and frequency pre-compensation - A description of UL segmented transmission |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhancements on UL time and frequency synchronization for IoT NTN, IoT NTN timing relationships, and UL segmented transmission are not clarified in stage-2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.x.2 User Plane aspects |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged text is omitted ***

---------------------- TEXT PROPOSAL BEGIN ----------------------

### 16.x.2 User Plane aspects

- Impact on timing aspects:

To accommodate the long propagation delays, several IoT timings involving DL-UL timing interaction are enhanced by the support of two scheduling offsets:Koffset andKmac as illustrated in Figure below:

- Koffset is the RTT between the UE and the uplink time synchronization reference point (RP). It corresponds to the sum of the service link RTT and the common TA if indicated.

- Kmac is the RTT between the RP and the eNB.

DL and UL are frame aligned at the uplink time synchronization reference point with an offset given by NTA,offset

Figure 1 Timing relationship parameters

The timing relationships that need to be modified for NB-IoT using Koffset are summarized as follows:

- The transmission timing of NPDCCH scheduled NPUSCH format 1.

- The transmission timing of random access response (RAR) grant scheduled NPUSCH format 1.

- The transmission timing of HARQ-ACK on NPUSCH format 2.

- The transmission timing of NPDCCH ordered NB-IoT physical random access channel (NPRACH).

- The timing of the adjustment of uplink transmission timing upon reception of a corresponding timing advance command.

The timing relationships that need to be modified for eMTC using Koffset are summarized as follows:

- The transmission timing of MPDCCH scheduled PUSCH.

- The transmission timing of random access response (RAR) grant scheduled PUSCH.

- The timing of the first PUSCH transmission opportunity in UL SPS.

- The transmission timing of HARQ-ACK on physical uplink control channel (PUCCH).

- The transmission timing of MPDCCH ordered physical random access channel (PRACH).

- The timing of the adjustment of uplink transmission timing upon reception of a corresponding timing advance command.

- The transmission timing of aperiodic sounding reference signal (SRS).

- The CSI reference resource timing.

- The transmission timing of a preamble retransmission.

For initial access, the information of Koffset is carried in system information. Update of Koffset after initial access is supported. The UE-specific Koffset can be provided and updated by the network with MAC CE.

Kmac is a scheduling offset supported in NTN for MAC CE timing relationships enhancement. It is provided by the network if downlink and uplink frame timing are not aligned at eNB. And it is needed for UE action and assumption on downlink configuration indicated by a MAC-CE command in (N)PDSCH. The Kmac is also used in the pre-configured uplink resources, if the UE has initiated an (N)PUSCH transmission using pre-configured uplink resources ending in subframe n, the UE shall start or restart to monitor the N/MPDCCH from DL subframe n+4+Kmac.

- Timing pre-compensation at the UE:

To accommodate the long propagation delays experienced in IoT NTN on both service link and feeder link, the UE should be able to perform time pre-compensation for all its uplink transmissions; including (N)PRACH preamble transmissions and uplink transmissions during the RRC_CONNECTED state. To do such pre-compensation, the UE is assisted by its GNSS and by the network which periodically broadcasts assistance information including serving satellite ephemeris as well as higher layer Common-TA-related parameters, where the latter may be used to calculate the common RTD e.g. delay on the feeder link.

Release-17 specified the following formula for TA calculation that shall be applied by NTN UEs for (N)PRACH preamble transmission and in RRC_CONNECTED state:

TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUE×Ts

Where:

- NTA and NTA,offset were already specified in [TS 36.213] [TS 36.211] as part of the existing TA Control;

- NTA,adjcommon is network-controlled common TA, and may include any timing offset considered necessary by the network (e.g. feeder link delay). It is derived from the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation if configured, otherwise NTA,adjcommon=0;

-	NTA,adjUE is UE self-estimated TA to pre-compensate for the service link delay. It is computed by the UE based on satellite-ephemeris-related higher-layers parameters if configured, otherwise NTA,adjUE=0;

- Ts is the eMTC / NB-IoT basic time unit [TS 36.211].

Figure 3 Uplink/Downlink Radio Frame Timing at the UE [TS 36.211 Section 8.1]

- Frequency pre-compensation at the UE:

The UE shall be capable of using its acquired GNSS position and satellite ephemeris information (when provided by the network) to calculate frequency pre-compensation to counter shift the instantaneous Doppler shift experienced on the service link.

While the pre-compensation of the instantaneous Doppler shift experienced on the service link is to be performed by the UE, the management of Doppler shift experienced over the feeder link as well as any transponder frequency error whether it is introduced in Downlink or Uplink is left to the network implementation without any specification impacts in Release 17.

- UL segmented transmission

UL segmented transmission is supported for UL transmission with repetitions.  The UE shall apply UE pre-compensation per segment of UL transmission of PUSCH/PUCCH/PRACH for eMTC and NPUSCH/NPRACH for NB-IoT from one segment to the next segment. The configuration of UL transmission segment is indicated on SIB for initial access and can be re-configured by RRC signalling.

---------- TEXT PROPOSAL END ---------