3GPP TSG RAN WG1 #108-e			R1-220xxxx

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.15.1

Source: 	Moderator (Qualcomm Incorporated)

Title: 	Summary of 108-e-R17-LTE-5G-Bcast-01

Document for:	Discussion and Decision

- Introduction

In RAN1#108-e, the following contribution was submitted to propose updates to TR 36.976 and TS 36.300:

- R1-2201655	Updates to 36.976 and 36.300	Qualcomm Incorporated

The objective of this email discussion is to agree on text proposals for the above specifications.

- Issue#1: Update to 36.976

In x1655, the following TP is provided:

======================================Start TP1======================================

## 4.3	Enhancements targeting LTE terrestrial broadcast

In Release 14, the following key RAN enhancements were made to the specifications to enable LTE terrestrial broadcast:

-	MBMS-dedicated cell [3];

-	MBSFN subframes using f = 1.25 kHz [8], with a cyclic prefix duration of 200µs and a symbol duration of 1ms;

-	New information blocks on PBCH and PDSCH of CAS [3], [6]:

-	MIB-MBMS is transmitted with a 40ms periodicity and updated every 160 ms; and

-	SIB1-MBMS is transmitted with an 80ms periodicity and updated every 160 ms, containing information relevant for receiving MBMS service and, optionally, the scheduling of other system information blocks;

-	MBMSInterestIndication RRC signalling procedure (see clause 4.1).

NOTE:	For upper layer enhancements, see TS 23.246 [4] Annex D and E, TS 24.116 [7], TS 24.117 [12] and TS 26.346 [5] (ROM service aspects).

In Release 16, the following RAN enhancements were made to address the use cases described in clause 4.2:

-	MBSFN subframes using f = 0.37 kHz, with a cyclic prefix duration of 300µs and a symbol duration of 3ms, for the support of large ISD;

-	MBSFN subframes using f = 2.5 kHz, with a cyclic prefix duration of 100µs and a symbol duration of 0.5ms, for the support of high mobility;

-	The following enhancements on the CAS:

-	PDCCH enhancements:

-	CFI indication in MIB [6] to avoid the need to decode PCFICH; and

-	New aggregation level 16;

-	Repetition of PBCH to increase its robustness.

In Release 17, the following RAN enhancement was introduced to enable deployment of LTE-based 5G terrestrial broadcast in broadcast UHF spectrum, where the channelization is 6/7/8MHz (depending on the geography):

-	PMCH bandwidth of 30, 35 and 40 PRBs (corresponding to 6/7/8MHz), applicable for CAS bandwidth of 15 or 25 PRBs (corresponding to 3 and 5MHz).

======================================End TP1======================================

Table 1 Comments on TP1 (TR 36.976)

| Company | Comment |
| --- | --- |
| NTT DOCOMO | We support the proposed TP for TR36.976. |

- Issue #2: Update to 36.300

In x1655, the following TP is provided:

======================================Start TP2======================================

### 5.1.1b (new)	Basic transmission scheme based on OFDM for MBMS-dedicated cells

The downlink transmission scheme for MBMS-dedicated cells is as described in clause 5.1.1, with the differences that the PMCH bandwidth can be indicated to be larger than the carrier bandwidth. In particular, a PMCH bandwidth of 30, 35 and 40 PRBs (corresponding to 6/7/8MHz) can be indicated when the carrier bandwidth is 15 or 25 PRBs (corresponding to 3 and 5MHz).

======================================Start TP2======================================

Table 2 Comments on TP1 (TS 36.300)

| Company | Comment |
| --- | --- |
| NTT DOCOMO | We support the proposed TP for TS36.300. |

- Conclusions

<To be filled after discussion is completed>