3GPP TSG RAN WG1 Meeting #108-e																R1-22xxxxx

e-Meeting, February 21st – March 3rd, 2022

Source: 	Moderator (vivo)

Title:	Discussion summary #1 of [108-e-NR-52-71GHz-05]

Agenda item:	8.2.5

Document for:	Discussion and decision

# Introduction

In this contribution, we summarize issues regarding PDSCH/PUSCH enhancements for new SCSs on supporting NR from 52.6 GHz to 71 GHz for the following email discussion in RAN1 #108-e.

[108-e-NR-52-71GHz-05] Email discussion for maintenance on timeline related aspects adapted to each of the new numerologies 480kHz and 960kHz – Huaming (vivo)

- 1st check point: February 25

- Final check point: March 3

Note that the scope of agenda 8.2.5 including defining maximum bandwidth for new SCSs, time line related aspects adapted to each of the new numerologies 480kHz and 960kHz, reference signals, scheduling particularly w.r.t. multi-PDSCH/PUSCH with a single DCI, HARQ, etc. In this summary, only issues related to bandwidth for new SCSs, time line related aspects adapted to each of the new numerologies 480kHz and 960kHz and reference signals are summarized. Issues related to scheduling particularly w.r.t. multi-PDSCH/PUSCH with a single DCI, HARQ are not in the scope of this summary.

# PDSCH/PUSCH enhancements for new SCSs

In this section, we provide a summary of issues, observations and proposals related to PDSCH/PUSCH enhancements for new SCSs discussed in the submitted contributions.

As in WID, the related objectives for this summary of agenda 8.2.5 are the following.

- Physical layer aspects including [RAN1]:

  - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported.

Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz

  - Time line related aspects adapted to 480kHz and 960kHz, e.g., BWP and beam switching timing, HARQ timing, UE processing, preparation and computation timelines for PDSCH, PUSCH/SRS and CSI, respectively.

  - Evaluate, and if needed, specify the PTRS enhancement for 120kHz SCS, 480kHz SCS and/or 960kHz SCS, as well as DMRS enhancement for 480kHz SCS and/or 960kHz SCS.

## 2.1. Timeline

### Individual observations/proposals

The following are individual observations and proposals from the contributions.

| Sources | Observations/proposals |
| --- | --- |
| [12, Intel] | Proposal 1 - Agree on the following values for cg-minDFI-Delay - SCS 120 kHz: 7, m*14, - SCS 480 kHz: 7*4, m*14*4, - SCS 960 kHz: 7*8, m*14*8, where m = {1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32} |

### cg-minDFI-Delay

It is identified in [12, Intel] that there’s a configured parameter cg-minDFI-Delay in ConfiguredGrantConfig indicates the minimum duration (in unit of symbols) from the last symbol of a PUSCH or PUSCH repetition of the HARQ process and the first symbol of the CG-DFI PDCCH. According to TS 38.331, cg-minDFI-Delay is currently only defined for SCS 15/30/60kHz. The upper bound of cg-minDFI-Delay for SCS 15/30/60 kHz is 4ms.

TS 38.311, clause 6.3.2 cg-minDFI-Delay Indicates the minimum duration (in unit of symbols) from the ending symbol of the PUSCH to the starting symbol of the PDCCH containing the downlink feedback indication (DFI) carrying HARQ-ACK for this PUSCH. The HARQ-ACK received before this minimum duration is not considered as valid for this PUSCH (see TS 38.213 [13], clause 10.5). The following minimum duration values are supported, depending on the configured subcarrier spacing [symbols]: 15 kHz: 7, m*14, where m = {1, 2, 3, 4} 30 kHz: 7, m*14, where m = {1, 2, 3, 4, 5, 6, 7, 8} 60 kHz: 7, m*14, where m = {1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16}

In is argued in [12, Intel] to allow the same upper bound of 4ms for SCS 120 kHz, the maximum value of cg-minDFI-Delay could be extended to 32 and proposed to allow all integer values from 1 to 32 for SCS 120 kHz. Furthermore, [12, Intel] proposed to keep the minimum value of cg-minDFI-Delay as 7 for 120 kHz SCS. For SCS 480kHz or 960kHz, it proposed to scale the values for SCS 120 kHz by 4 or 8 respectively.

Moderator’s comment:

The extension of this timeline parameter for FR2-2 seems straightforward. The following proposal is formulated for discussion.

##### Proposal 1-1 (high priority)

Support the following values for cg-minDFI-Delay

SCS 120 kHz: 7, m*14,

SCS 480 kHz: 7*4, m*14*4,

SCS 960 kHz: 7*8, m*14*8,

where m = {1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32}

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| Intel | As original proponent, we support the proposal. |
| Huawei,HiSilicon | Support the proposal |
| ZTE, Sanechips | We are fine with Proposal 1-1. |
| Xiaomi | We are fine that the maximum values for 480/960khz are scaled up by 4/8. but we would also prefer to keep the same bit length as 60khz. But if it is majority view to support the proposal, we can also support it |
| DOCOMO | Support the proposal. |
| Ericsson | Support the proposal |
| Samsung | Fine with the proposal |
| CATT | Ok with the proposal |
| LG Electronics | We are fine with Proposal 1-1. |
| Nokia/NSB | Fine with the proposal. |
| Apple | Okay with proposal |
| vivo | We support the proposal. |
| Qualcomm | We are okay with the proposal |
| Moderator | Seems most companies are OK with this proposal as it is. Xiaomi indicated they can support if majority view. Will recommend for GTW/email approval. |

## 2.2. Issue(s) related to reference signal

### Individual observations/proposals

The following are individual observations/proposals from the contributions.

| Sources | Observations/proposals |
| --- | --- |
| [1, Huawei] | Proposal 1: Support DMRS bundling FG developed in CovEnh WI for the same TB to FR2-2 with 120 kHz SCS. |
| [3, InterDigital] | Proposal 4: The use of DMRS bundling FG developed in CovEnh WI for the same TB should not be prohibited in FR2-2 with 120 kHz SCS. |
| [4, vivo] | Proposal 1: For the case where the predefined requirements specified in Rel-17 CovEnh WI are met, i.e. the phase coherency and power consistency are guaranteed across repetitions of a PUSCH, DMRS bundling FG developed in Rel-17 CovEnh WI for same TB can be applied to FR2-2 with 120 kHz SCS. |

### DMRS bundling for same TB in FR2-2 with 120 kHz SCS

The following conclusion was reached in RAN1#107bis-e.

Conclusion

DMRS bundling across multiple PUSCHs scheduled by a single DCI is not supported for NR operation in FR2-2 in Rel-17.

- Note: applicability of DMRS bundling FG developed in CovEnh WI for same TB to FR2-2 with 120 kHz SCS can be further discussed.

Regarding the remaining issue whether DMRS bundling FG developed in CovEnh WI for same TB can be applied to FR2-2 with 120 kHz SCS, the following contributions expressed their views.

[1, Huawei] argued that coverage issue also exits in FR2-2 due to larger path loss than FR2-1 with the higher frequency range. Extending this feature to FR2-2 for 120kHz SCS is beneficial to enrich the usage scenario without additional standard effort.

[3, InterDigital] noted that TB repetition across multiple contiguous slots can be supported in FR2-2 by using the existing features. In such a situation, DMRS bundling FG developed in CovEnh WI could be used to improve the channels estimation performance. Therefore, the use of this FG developed in CovEnh WI should not be prohibited in FR2-2.

[4, vivo] observed that for PUSCH repetition with 120 kHz SCS, PUSCH DMRS bundling was introduced in Rel-17 CovEnh WI. This feature is not restricted to specific frequency band, and it is beneficial for PUSCH performance when the phase coherency and power consistency are guaranteed across repetitions of a PUSCH.

Moderator’s comment:

The performance benefits (coverage and channel estimation for PUSCH) of extending DMRS bundling for same TB in FR2-2 with 120 kHz SCS are identified. With no additional standard effort, it does make sense to allow this FG for FR2-2 with 120 kHz SCS. With that, the following proposal is formulated for consideration.

Proposal 2-1 (high priority)

DMRS bundling FG developed in CovEnh WI for the same TB is supported in FR2-2 with 120 kHz SCS.

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| Intel | Coverage enhancements has been excluded from the WID. Out preference is not agree at this stage. However, we would be ok to accept this only if there are no specification impact (other than allowing capability change to support FR2-2) associated with this conclusion. Ideally, we would like to have this as working assumption to make sure there is no other impact. If some specification efforts are identified, we think it should not be agreed at this stage. |
| Huawei, HiSilicon | Support the proposal. we think there is no additional standard impact if same mechanism is adopted as CovEnh. |
| ZTE, Sanechips | We agree with Intel’s idea of changing Proposal 2-1 to a working assumption. We can further agree on this issue if there is no specification impact found by other companies. |
| Moderator | Wording updated into Proposal 2-1a below to address comments. |

##### Proposal 2-1a (closed)

DMRS bundling FG developed in CovEnh WI for the same TB is supported in FR2-2 with 120 kHz SCS.

- Note: can be revisited if specification impact other than that for UE feature/capability signaling is identified

Companies are encouraged to provide comments.

| Company Name | Comments/Views |
| --- | --- |
| DOCOMO | Agree above as WA. |
| Ericsson | Since this is only about application of a feature group from another WI to FR2-2, shouldn't this be discussed under UE features? |
| Moderator | Response to Ericsson: Thanks for the comment. I’ve checked UE feature discussion under AI 8.16.8 (for CovEnh). I didn’t find any discussion on the applicability of FG 30-4 (DMRS bundling) for FR2-2. Nor is this issue being discussed under AI 8.16.2. If I missed anything, please let me know. I thought if we can make some progress (e.g., agreement/working assumption) here, then we may save some time in UE feature discussion. I’m fine to leave the whole thing to UE feature if majority companies prefer and/or there’s strong technical/procedure concern from companies. |
| Samsung | Agree the proposal as working assumption. |
| CATT | Prefer the following wording for the WF: DMRS bundling FG developed in CovEnh WI for the same TB is supported in FR2-2 with 120 kHz SCS if no further specification effort is needed. |
| Intel | We would be ok with Proposal 2-1a. To avoid ping-pong, if companies can agree in this agenda. It would be better to agree the working assumption in this agenda and use the working assumption agreement to reflect the changes in the UE feature discussion. In the UE feature discussion, the discussion is now mostly final tweaks to description, type of the FG, etc. |
| LG Electronics | We are fine with Proposal 2-1a unless additional specification impacts are needed. |
| ZTE, Sanechips | We agree with Proposal 2-1a as working assumption. |
| Apple | Fine with this a a WA. |
| vivo | Support the proposal as a working assumption |
| Qualcomm | We are okay with proposal |
| Huawei,HiSilicon | We are fine with the proposal |
| Moderator | Response to CATT: I think the note “can be revisited if specification impact other than that for UE feature/capability signaling is identified” serves the same purpose as putting “if no further specification effort is needed” into the main bullet. Given most companies are fine with this WF as it is, will recommend for GTW/email approval. |
| Ericsson | We actually do have a concern on this proposal. In principle, one could apply FG 30-4 (DMRS bundling) directly to FR2-2 operating with 120 kHz SCS without RAN1 specification impact. However the real question is if there is any benefit in doing so, and currently this is unknown due to that RAN4 has only studied FR1 and FR2-1. DMRS phase continuity tolerance requirements are being specified for FR1 and FR2-1 in RAN4; however, there has been no study, and no recommendation for the tolerance requirements for FR2-2. No company has even brought it up for discussion in RAN4 to our knowledge. Hence, we don't think it is proper for RAN1 to introduce a FG for DMRS bundling, or conclude that FG 30-4 should directly apply, when it requires further RAN4 specification work. For that reason, we do not support Proposal 2-1a. |
| Moderator | Summary of discussion: All companies other than Ericsson are fine with this proposal as a working assumption. Ericsson expressed concern on the lack of requirement study of extending DMRS bundling to FR2-2 in RAN4. Moderator’s understanding is that the purpose of working assumption is that companies in all WGs can work out details assuming such feature is supported. However, given the expressed concern cannot be addressed in RAN1, moderator’s suggestion is to close this discussion here in this meeting while interested companies can continue discussion on this matter in UE feature and/or RAN4. |

## 2.3. Other issue(s)

Companies are encouraged to provide comments below if any missing issue(s) related to bandwidth/timeline/reference signals.

| Company Name | Comments/Views |
| --- | --- |
| Ericsson | @Moderator: Is there any action needed in RAN1 based on the LS reply from RAN4 on wakeup and SCell dormancy indication in R1-2200897? |
| Moderator | Response to Ericsson: Thanks for the comment. Based on the LS reply from RAN4 in R1-2200897, my understanding is that there’s no action needed from RAN1. RAN4 stated that “X defined in TS 38.133 is the interruption length on other activation serving cells, which is irrelevant to X defined in 38.213 section 10.3”. In RAN1#107-e, we agreed that “From RAN1 perspective, for NR operation with 480 kHz and/or 960 kHz SCS, the value of minimum time gap for wake-up and Scell dormancy indication (DCI format 2_6) is scaled by 4 and 8 of the corresponding value of 120 kHz SCS for 480 kHz and 960 kHz SCS respectively.” Above agreement was already captured in 38.213 v17.0.0, clause 10.3, Table 10.3-1 Minimum time gap value X (copied below for reference). Table 10.3-1 Minimum time gap value X SCS (kHz); Minimum Time Gap X (slots). Value 1; Value 2. 15; 1; 3. 30; 1; 6. 60; 1; 12. 120; 2; 24. 480; 8; 96. 960; 16; 192. |

# Recommendation for GTW/email approval

##### Proposal 1-1

Support the following values for cg-minDFI-Delay

SCS 120 kHz: 7, m*14,

SCS 480 kHz: 7*4, m*14*4,

SCS 960 kHz: 7*8, m*14*8,

where m = {1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32}

# Conclusion

TBD

# Reference

- R1-2202490	Remaining issues of PDSCH/PUSCH enhancement for 52-71GHz spectrum	Huawei, HiSilicon, revision of R1-2200956

- R1-2200990	Remaining issues in PDSCH/PUSCH enhancements for Beyond 52.6GHz	FUTUREWEI

- R1-2201037	Remaining issues for PDSCH/PUSCH enhancements to supporting 52.6-71 GHz band in NR	InterDigital, Inc.

- R1-2201088	Remaining issues on PDSCH/PUSCH enhancements for NR operation from 52.6GHz to 71GHz	vivo

- R1-2201269	Discussion on remaining issue for PDSCH/PUSCH enhancements	OPPO

- R1-2201354	Remaining issues on PDSCH/PUSCH enhancements for up to 71GHz operation	CATT

- R1-2201392	Remaining issues on the data channel enhancements for 52.6 to 71GHz	ZTE, Sanechips

- R1-2201433	Discussion on PDSCH/PUSCH enhancements for NR 52.6-71 GHz	Panasonic Corporation

- R1-2201436	Remaining issues of multi-PDSCH scheduling via a single DCI	Fujitsu

- R1-2201473	Remaining issues on PDSCH/PUSCH enhancements for NR in FR2-2	NTT DOCOMO, INC.

- R1-2201665	PDSCH/PUSCH enhancements	Nokia, Nokia Shanghai Bell

- R1-2201691	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

- R1-2201739	PDSCH-PUSCH Enhancements	Ericsson

- R1-2201767	On remaining issues for PDSCH PUSCH Enhancements	Apple

- R1-2201900	Remaining issues on PDSCH enhancement for NR operation from 52.6GHz to 71GHz	NEC

- R1-2201915	Remaining issues on PDSCH and PUSCH enhancements for NR 52.6-71GHz	Xiaomi

- R1-2202007	Maintenance on PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz	Samsung

- R1-2202074	Remaining discussion on multi-PDSCH scheduling design for 52.6-71 GHz NR operation	MediaTek Inc.

- R1-2202132	PDSCH/PUSCH enhancements for NR in 52.6 to 71GHz band	Qualcomm Incorporated

- R1-2202283	Discussion on multi-PUSCH scheduling	ASUSTeK

- R1-2202338	PDSCH/PUSCH enhancements to support NR above 52.6 GHz	LG Electronics