3GPP TSG RAN WG1 Meeting #108-e	R1-2202526

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.4.1

Source:	Moderator (Huawei, HiSilicon)

Title:	Feature lead summary#1 on timing relationship enhancements

Document for:	Discussion

# Introduction

The Rel-17 work item on solutions for NR to support NTN was approved at RAN#86 and the first Rel-17 specifications were submitted to RAN#94e. RAN WG1 has entered the Rel-17 maintenance stage of this work item where one focus area are the timing relationship enhancements for NTN.

In this contribution, we summarize the related issues and proposals based on the contributions submitted to RAN1#108-e under agenda item 8.4.1 [4] – [18].

[108-e-R17-NR-NTN-01] Email discussion for maintenance on timing relationship enhancements – Jussi (Huawei)

- 1st check point: February 25

- Final check point: March 3

Since we have the first GTW on NR-NTN on Wednesday, during this round of the email discussion, please comment on the issues and text proposals before Feb, 23rd, Wednesday, UTC 16:59.

Particularly, the LS in issue#9 benefits from quick convergence regarding whether and what kind of response we would draft to RAN2. Early comments would be greatly appreciated.

# 1 [ACTIVE] Issue#1: K_offset update

## 1.1 On the update of cell specific K_offset during SIB modification period

During RAN WG1#107e there was a long discussion on whether there is a need to resolve ambiguity of which cell-specific K_offset value to use during the SIB modification period [1]. At that meeting 15 companies considered that there is no need for specification enhancement and two companies saw a need for further study. There are eight input contributions to this meeting that address this issue.

[CATT]

Proposal 2: There is no need of specification enhancement for the ambiguity of which cell-specific K_offset value to use during the SIB modification period.

[Spreadtrum]

Proposal 1: There is no ambiguity of which cell-specific K_offset value to use during the SIB modification period.

[Xiaomi]

Proposal 1: The ambiguity issue on the update of cell-specific K_offset can be handled by the network implementation.

[ZTE]

Proposal-1: The ambiguity of which cell-specific K_offset value to use during the SIB modification period can be resolved by gNB implementation without specification impact

[Baicells]

Proposal 1: Define the application time of the updated cell-specific K_offset to be the end of the modification period that contains the SIB for cell-specific K_offset update.

[NEC]

Proposal 1. There is no ambiguity of which cell-specific K_offset value to use during the SIB modification period. It could be handled by gNB implementation as legacy.

[CMCC]

Proposal 1: To address the ambiguity issue during the SIB modification period, it is preferred to leave to gNB implementation, e.g., scheduling restriction.

[Nokia]

Proposal 1: The application time of the updated cell-specific K_offset shall be the same for a UE acquiring the new SI via RRC or via SIB acquisition.

Proposal 2: The application time of the updated K_offset at cell level needs to pre-defined and different from the first SIB occasion in the modification period.

Proposal 3: RAN1 shall discuss the rules for the application time of cell-specific K_offset.

Proposal 4: As options for the application time of the recently acquired updated K_offset we propose one of the following alternatives with a slight preference for (B):

- The end of the first (or the n-th) SI-window for the SIB containing K_offset in the modification period

- The end of the first modification period after the update

- A specific SFN.

Proposal 7: RAN1 shall discuss UE behavior if UE fails to acquire updated K_offset within valid time.

Proposal 8: RAN1 to consider alternative procedures for restoring K_offset before declaring radio link failure.

We continue to have a large number of companies that do not consider it necessary to introduce any specification enhancement to address the potential ambiguity regarding which cell-specific K_offset value to use during the SIB modification period.

Initial proposal 1.1 (Moderator)

Moderator recommendation is not to address this in Rel-17 maintenance, but please provide your opinion below.

| Company | Comments |
| --- | --- |

## 1.2 On the update of UE specific K_offset with MAC CE

Two companies point out the potential ambiguity occurring when UE specific K_offset is updated with MAC CE command.

[CMCC]

Proposal 2: In order to address the ambiguity period issue for MAC CE updating UE specific K_offset, support if cell-specific K_offset is always used for the additional transmission timings related to fallback DCI format, use fallback DCI in the ambiguity period.

Proposal 3: Support always use the cell-specific K_offset for the additional transmission timings related to fallback DCI format.

[Nokia]

Proposal 5: K_offset component updated via MAC-CE to be valid from the start of the beggining of the slot m+3 Nslotsubframe,μ+1 , where n is the uplink slot where the MAC-CE command was received.

Proposal 6: UEs configured for using UE-specific K_offset should update the cell-specific component of the UE-specific K_offset using the same procedure and point of application that will be specified for the cell-specific K_offset

The problem is identified to exist in between the times when the gNB can be certain that the UE applies either the old or new UE specific K_offset as shown in the figure below.

Initial proposal 1.2 (Moderator)

The moderator would like to get companies thoughts on whether this is a topic we need to address at the maintenance phase or whether the impact can be mitigated by gNB implementation. In particular, what is the thinking regarding the use of cell-specific K_offset for the additional transmission timings related to fallback DCI format?

| Company | Comments |
| --- | --- |

# 2 [ACTIVE] Issue#2: MAC CE timing relationships

We have an agreement from several meetings ago that K_mac update is carried in system information. Proposals for additional mechanisms did not receive any support at RAN WG1#107e meeting, either. Two companies have proposals related to this issue of K_mac update.

[CATT]

Proposal 3: Support the update of K_mac with MAC CE.

[Spreadtrum]

Proposal 2: K_mac can be update based on SIB update mechanism

The moderator observes that CATT repeats the proposal from #107e meeting where it did not get support. Also the lack of input contributions on this topic would indicate that companies are indeed fine with only providing K_mac update via system information.

Initial proposal 2 (Moderator)

Moderator recommendation is not to address this issue in Rel-17 maintenance.

| Company | Comments |
| --- | --- |

# 3 [ACTIVE] Issue#3: K1 range extension

At RAN WG1#107e meeting no agreements were reached on K1 range extension enhancements beyond the already agreed range extension itself. Three companies contributed to this meeting and these proposals are in alignment what was already concluded last time about keeping the PDSCH-to-HARQ feedback timing indicator unchanged.

[Xiaomi]

Proposal 2: The bit-length of PDSCH-to-HARQ_feedback timing indicator field in the fallback DCI is kept unchanged.

[NEC]

Proposal 2. There is no need to extend the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI when the range of the K1 value is extended

[Apple]

Proposal 2: The K1 range extension in unpaired spectrum does not change the PDSCH-to-HARQ_feedback timing indicator field size in DCI.

For non-fallback DCI, only extend the value range of entries in the configured dl-DataToUL-ACK table.

For fallback DCI, introduce a scaling factor when determining K1 value.

Initial proposal 3 (Moderator)

Moderator recommendation is not to address this issue in Rel-17 maintenance.

| Company | Comments |
| --- | --- |

# 4 [ACTIVE] Issue#4: Configured grant type 1 timing relationship

At RAN1#108e, two companies provide proposals on this topic:

[Samsung]

Proposal 3: The timing relationship for Configured Grant Type 1 should be left to Network implementation.

[Baicells]

Proposal 3: In case of PUSCH transmission with configured grant and if K_offset is updated or will be updated,

- Option1: Release configured grant before the application of new K_offset.

- Option2: Continue the PUSCH transmission with configured grant based on the new K_offset but conflict zone should be avoided.

At RAN1#107e there was also this single proposal from Samsung on the topic and the moderator recommendation back then was not to discuss further the introduction of K_offset for type 1 configured grant. Baicells proposal apparently addresses the same issue with respect to how to continue configure grant assignment when K_offset is updated. Companies views are requested on whether it is sufficient to retain the conclusion from RAN1#107e and leave the timing relationship for network implementation or consider the options proposed by Baicells.

| Company | Comments |
| --- | --- |

# 5 [ACTIVE] Issue#5: UE reporting of information about the UE specific TA pre-compensation

At RAN1#107e the RAN2 LS indicating RAN2 agreements on UE specific TA reporting. RAN1 replied indicating that event-triggers can be supported and that these event-triggers can be based on TA values. One company continues discussion on the topic at this meeting.

[CATT]

Proposal 4: On UE-specific TA reporting, periodic reporting can be supported.

Proposal 5: Using RRC signaling to report TA can be supported.

Proposal 6: Reporting differential TA between current TA and previous TA is preferred.

RAN1 already concurred with the RAN2 agreements concluding to adopt event-triggered reporting. At the maintenance phase on Rel-17 specifications it likely is not feasible for RAN1 to add reporting mechanisms above and beyond what has been communicated between the two Working Groups.

Initial proposal 5 (Moderator)

Moderator recommendation is not to address periodic UE-specific TA reporting in Rel-17 maintenance.

| Company | Comments |
| --- | --- |

# 6 [ACTIVE] Issue#6: Beam-specific K_offset in initial access

Three companies provided proposals on K_offset in initial access, with particular focus on beam-specific K_offset.

[Samsung]

Proposal 1: Only Cell-specific K_offset in initial access is supported.

Proposal 2: More than one of above Koffset configurations can be supported, and using which one is dependent on gNB configuration.

[InterDigital]

Proposal-1: beam-specific K-offset indication is also supported optionally.

[CMCC]

Proposal 4: gNB has the flexibility of configuring cell-specific or beam specific value of K_offset.

- Beam specific SIB can be supported, i.e., different beam specific SIB may carry different beam specific values (e.g., K_offset).

It was already observed by the previous moderator at RAN1#107e that this issue has been discussed at many RAN1 meetings with several rounds of email discussion and debated at GTW sessions. The pros and cons of supporting beam specific K_offset configured in system information and used in initial access are clear to the group – same comments have been made by both sides over the meetings.

Initial proposal 6 (Moderator)

At this stage of Rel-17 where we already entered maintenance stage, the moderator recommendation is not to consider beam-specific K_offset for initial access.

| Company | Comments |
| --- | --- |

# 7 [ACTIVE] Issue #7: Beam application timing for PUCCH in BFR

Two companies provided proposals on beam failure recovery procedure which was discussed at RAN1#107-e meeting.

[InterDigital]

Proposal-2: specify UE assumption of the beam application timing for PUCCH based on any of interpretations adopted.

Proposal-3: K-offset enhancement is supported for beam application timing for PUCCH (i.e., Interpretation-1).

[Apple]

Proposal 3: RAN1 to introduce Koffset to enhance the timing relationship on the PUCCH transmission with new beam in the beam failure recovery procedure.

At the previous meeting we reached the following agreement:

Agreement

On beam failure recovery procedure, for PRACH transmission in uplink slot n, UE monitors the corresponding PDCCH starting from downlink slot “n + K_mac + 4” within a corresponding RAR window.

There was also a long discussion on two different interpretations on the delay between PDCCH reception and application of new PUCCH beam. The majority of companies considered interpretation 1 being the correct one

- Interpretation 1: “28 symbols” is the absolute time between the time UE receives PDCCH and the time UE applies new PUCCH beam

and no enhancement would be needed.

The moderator invites companies to give their views on whether this issue should be revisited now in Rel-17 maintenance phase or close the issue for beam application timing for PUCCH with an understanding that no enhancement is needed.

| Company | Comments |
| --- | --- |

# 8 [ACTIVE] Issue #8: Timing relationship of random access response

Huawei [3] points out a problem that emerges from the agreement reached on NR NTN to introduce an additional delay for the starts of ra-ResponseWindow and msgB-ResponseWindow, which is equal to sum of UE’s TA and K_mac.

Agreement: The starts of ra-ResponseWindow and msgB-ResponseWindow are delayed by an estimate of UE-gNB RTT. - The estimate of UE-gNB RTT is equal to the sum of UE’s TA and K_mac. Note 1: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by NTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×TcTTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc. The estimate of gNB-satellite RTT is equal to the sum of NTA,common×Tc and K_mac. How to treat NTA and NTA,offset can be further discussed. Note 2: According to the RAN1#104bis-e agreement: When UE is not provided by network with a K_mac value, UE assumes K_mac = 0. Note 3: The accuracy of the estimated UE-gNB RTT with respect to the true UE-gNB RTT can be further discussed. Note 4: Other options of determining the estimate of UE-gNB RTT can be further discussed.

.

The concern is that this agreement unintentionally changes the behaviour for a terrestrial UE. In particular, for a terrestrial UE, NTA, NTA, UE-specific, NTA,common and kmac can be set to zero. However, the value of NTA,offset may be non-zero and the value of NTA,offset is provided by n-TimingAdvanceOffset for the serving cell. The proponent identifies three options, one of which changes the behaviour for terrestrial UEs, one of which would require a modification of the NTN agreement, and one which both retains the legacy behaviour for terrestrial UEs and does not revert the NTN agreement. This approach is adopted in the TPs proposed in [2], where the additional delay is only applicable to NR NTN UEs.

Clause 8.2 of TS38.213

============================ Unchanged Text Omitted ===================================

In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in clause 10.1. If TTA>NTA,offsetTc, the window starts after an additional TTA+kmac msec where TTA is defined in [4, TS 38.211] and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided.The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

============================ Unchanged Text Omitted ===================================

Clause 8.2A of TS38.213

============================ Unchanged Text Omitted ===================================

In response to a transmission of a PRACH and a PUSCH, or to a transmission of only a PRACH if the PRACH preamble is mapped to a valid PUSCH occasion, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding MsgB-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the PUSCH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. If TTA>NTA,offsetTc, the window starts after an additional TTA+kmac msec where TTA is defined in [4, TS 38.211] and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by msgB-ResponseWindow.

============================ Unchanged Text Omitted ===================================

Feedback from companies on this approach to resolve the problem is requested below.

| Company | Comments |
| --- | --- |

# 9 [ACTIVE] Issue#9: RAN2 LS on NTN-SIB

RAN1 received an LS from RAN2 on NTN-specific SIB [2], which was assigned to be discussed in AI 8.4.1. The input contributions related to this topic can be found in [20], [21], [22], [23].

[CMCC]

Proposal 1: Update of assistance information in SIB will not trigger system information modification procedure.

- It is up to RAN2 to determine detailed solutions for updating the assistance information. (e.g., Changes of the assistance information should neither result in system information change notifications nor in a modification of valueTag in SIB1, just like “timeInfoUTC” field acts in SIB9.)

[MediaTek Inc]

Observation 5: It may be more flexible to have parameters like cell-specific Koffset, k_mac, Indication for network enabled/disabled TA report, Cell reference location, t-Service, Validity duration for UL sync information in NTN-specific SIB in case it is beneficial for the network to update these within the system information notification period.

[Panasonic R&D Center]

Proposal 2: Add to SIB-NTN a counter with at least 5 bits for the SFN-cycles which have elapsed since the first instance of the SIB-NTN in each validity period.

[Nokia]

Observation 7: RAN1 and RAN2 have different understandings of the applicability of the validity timer/validity duration.

Proposal 12: Inform RAN2 that the validity duration is only intended to be applicable for serving satellite ephemeris and common TA related parameters.

Observation 8: Is seems that RAN1 and RAN2 have different understandings of UE actions prior to the validity timer expiry.

Observation 9: There may be periods with uncertainty related to UE’s UL synchronization status if the UE is allowed to read serving satellite ephemeris and Common TA related parameters after the expiry of the validity timer.

Proposal 13: Inform RAN2 that under normal operation, a UE is expected to have read new and updated serving satellite ephemeris information prior to the expiry of the validity timer.

[Apple]

Proposal 1: For UEs updating the timing relationship enhancement parameters, down select one of the following two alternatives:

Alt 1: Update of cell-specific Koffset or Kmac in NTN-specific SIB affects the value tag and triggers SI modification procedure.

Alt 2: Introduce additional validity duration for timing relationship enhancement parameters in NTN-specific SIB.

The moderator would like to start discussion on the RAN2 LS in order to identify whether at RAN1 we can identify any problems with the RAN2 agreements.

RAN2 informed RAN1 of the following regarding the NTN-specific SIB:

And at least the following serving cell information will be broadcast by SIBx:

1) Ephemeris;

2) Common TA parameters;

3) Validity duration for UL sync information;

4) t-Service (the timing information on when the serving cell is going to stop serving the area);

5) Cell reference location;

6) Epoch time;

7) K_mac;

8) Cell-specific Koffset;

9) Indication for network enabled/disabled TA report.

RAN2 also agreed that the validity duration for UL sync information applies to the whole SIBx and UE acquires the updated SIBx when the timer expires (FFS if this applies only to RRC_CONNECTED mode or to RRC_IDLE UEs as well).

First we need to establish whether RAN1 is in agreement that all the above serving cell information is sent in the NTN-specific SIB. From input contributions it appears that some companies support this approach whereas it is also pointed out that some parameters do not need to be updated as frequently as e.g. ephemeris and common TA parameters.

Is the list of serving cell information (9 items) provided by RAN2 acceptable to RAN1 or is there a need to make adjustments, and if so, what changes are considered necessary?

| Company | Comments |
| --- | --- |

One input contribution points out that there would appear to be different understanding between RAN1 and RAN2 on the applicability of the validity timer and validity duration. This follows from the fact that most of the parameters do not need to be updated as frequently as the validity timer for serving satellite ephemeris and common TA related parameters.

What are companies views on this generalised statement by RAN2 that the validity duration applies to the whole NTN-specific SIB?

| Company | Comments |
| --- | --- |

# 10 Text proposals to specifications

## 10.1 TPs for TS38.213

#### 10.1.1 TP #1

Ericsson [11] points out that the RRC parameter for Kcell,offset is not called Koffset but CellSpecific_Koffset. They recommend adopting the approach used in TS38.214 where the Kcell,offset is referred to as CellSpecific_Koffset. There are five such occurrences in TS38.213 and the proposal is to replace each occasion as follows:

- Adopt the following TP for 38.213: “where Kcell,offset is provided by CellSpecific_Koffset”

The moderator believes this is a sensible correction recommends adopting the TP in the four occurrences within TS 38.213. Please provide your opinion below.

| Company | Comments |
| --- | --- |

#### 10.1.2 TP #2

OPPO has three text proposals addressing a clarification of the particular slot in question through stating that no timing advance is taken into account.

10.1	UE procedure for determining physical downlink control channel assignment

*** < Unchanged parts are omitted> ***

For a CORESET other than a CORESET with index 0, if a UE is provided a single TCI state for a CORESET, or if the UE receives a MAC CE activation command for one or two of the provided TCI states for a CORESET, the UE assumes that the DM-RS antenna port associated with PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by the TCI states. For a CORESET with index 0, the UE expects that a CSI-RS configured with qcl-Type set to 'typeD' in a TCI state indicated by a MAC CE activation command for the CORESET is provided by a SS/PBCH block

-	if the UE receives a MAC CE activation command for one of the TCI states, the UE applies the activation command in the first slot that is after slot k+3Nslotsubframe,μ+2μ∙kmac where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command without taking into account timing advance, μ is the SCS configuration for the PUCCH in the slot when the activation command is applied, and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided.

*** < Unchanged parts are omitted> ***

7	Uplink Power control

*** < Unchanged parts are omitted> ***

A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by SRS-PosResourceSet as described in clause 7.3.1. If the UE is provided a number of RS resources for pathloss estimation for PUSCH/PUCCH/SRS transmissions that is larger than 4, the UE maintains for pathloss estimation RS resources corresponding to RS resource indexes qd as described in clauses 7.1.1, 7.2.1, and 7.3.1. If an RS resource updated by MAC CE, as described in clauses 7.1.1, 7.2.1 and 7.3.1, is one from the RS resources the UE maintains for pathloss estimation for PUSCH/PUCCH/SRS transmissions, the UE applies the pathloss estimation based on the RS resources starting from the first slot that is after slot k+3∙Nslotsubframe,  μ+2μ∙kmac where k is the  slot where the UE would transmit a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE without taking into account timing advance, μ  is the SCS configuration for the PUCCH or PUSCH, respectively, that is determined in the slot when the MAC CE command is applied and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided.

*** < Unchanged parts are omitted> ***

6	Link recovery procedures

*** < Unchanged parts are omitted> ***

For the PCell or the PSCell, the UE can be provided, by PRACH-ResourceDedicatedBFR, a configuration for PRACH transmission as described in Clause 8.1. For PRACH transmission in slot  and according to antenna port quasi co-location parameters associated with periodic CSI-RS resource configuration or with SS/PBCH block associated with index  provided by higher layers [11, TS 38.321], the UE monitors PDCCH in a search space set provided by recoverySearchSpaceId for detection of a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI starting from slot n+4+2μ∙kmac without taking into account timing advance, where 𝜇 is the SCS configuration for the PRACH transmission and 𝑘mac is a number of slots provided by K-Mac [12, TS 38.331] or 𝑘mac =0 if K-Mac is not provided, within a window configured by BeamFailureRecoveryConfig. For PDCCH monitoring in a search space set provided by recoverySearchSpaceId and for corresponding PDSCH reception, the UE assumes the same antenna port quasi-collocation parameters as the ones associated with index  until the UE receives by higher layers an activation for a TCI state or any of the parameters tci-StatesPDCCH-ToAddList and/or tci-StatesPDCCH-ToReleaseList. After the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI in the search space set provided by recoverySearchSpaceId, the UE continues to monitor PDCCH candidates in the search space set provided by recoverySearchSpaceId until the UE receives a MAC CE activation command for a TCI state or tci-StatesPDCCH-ToAddList and/or tci-StatesPDCCH-ToReleaseList.

*** < Unchanged parts are omitted> ***

The moderator requests companies indicate whether this clarification is needed.

| Company | Comments |
| --- | --- |

#### 10.1.3 TP #3

OPPO proposes a change in how the slot where the UE applies the MAC command has 2μ∙kmac removed from the offset.

9	UE procedure for reporting control information

*** < Unchanged parts are omitted> ***

For the remaining of this clause, if a UE is provided Kcell,offset by Koffset in ServingCellConfigCommon or KUE,offset by a MAC CE command, reference to a slot n+k for a PUCCH transmission or PUSCH transmission corresponds to a slot n+k+2μ∙Koffset for the PUSCH or the PUCCH transmission, where μ is the SCS configuration for the PUCCH transmission or PUSCH transmission. If Koffset or if the MAC CE command is not provided, Kcell,offset=0 or KUE,offset=0, respectively. If the PUCCH transmission or the PUSCH transmission is scheduled by a DCI format with CRC scrambled by TC-RNTI, KUE,offset=0. If the UE is provided a KUE,offset value by a MAC CE command, the UE applies the MAC command in the first slot that is after slot k+3⋅Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the MAC CE command, μ is the SCS configuration for the PUCCH transmission that is determined in the slot when the MAC CE command is applied, and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided.

*** < Unchanged parts are omitted> ***

The moderator requests companies indicate whether this clarification is needed.

| Company | Comments |
| --- | --- |

#### 10.1.4 TP #4

LGE provides an TP to the same section as above in TP #3, but it is related to the definition of Koffset. Section 4.2 of TS38.213 defines that K_offset is determined based on the cell-specific K_offset – UE-specific K_offset.

9	UE procedure for reporting control information

*** Unchanged text is omitted ***

For the remaining of this clause, if a UE is provided Kcell,offset by Koffset in ServingCellConfigCommon or KUE,offset by a MAC CE command, reference to a slot n+k for a PUCCH transmission or PUSCH transmission corresponds to a slot n+k+2μ∙Koffset for the PUSCH or the PUCCH transmission, where μ is the SCS configuration for the PUCCH transmission or PUSCH transmission and Koffset=Kcell,offset-KUE,offset. If Koffset or if the MAC CE command is not provided, Kcell,offset=0 or KUE,offset=0, respectively. If the PUCCH transmission or the PUSCH transmission is scheduled by a DCI format with CRC scrambled by TC-RNTI, KUE,offset=0. If the UE is provided a KUE,offset value by a MAC CE command, the UE applies the MAC command in the first slot that is after slot k+3Nslotsubframe,μ+2μ∙kmac where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the MAC CE command, μ is the SCS configuration for the PUCCH transmission that is determined in the slot when the MAC CE command is applied, and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided.

The moderator thinks that this clarification makes sense and comments from companies are requested whether this TP is agreeable.

| Company | Comments |
| --- | --- |

## 10.2 TPs for TS38.214

#### 10.2.1 K_offset configuration in TS38.214

MediaTek points out that TS 38.213 specifies K_offset configuration and it is preferable to only refer to TS38.213 rather than repeat the K_offset configuration in TS38.214.

4.2	Transmission timing adjustments

**************************************************************************************

For a timing advance command received on uplink slot n and for a transmission other than a PUSCH scheduled by a RAR UL grant or a fallbackRAR UL grant as described in clause 8.2A or 8.3, or a PUCCH with HARQ-ACK information in response to a successRAR as described in clause 8.2A, the corresponding adjustment of the uplink transmission timing applies from the beginning of uplink slot n+k+1+2μ∙Koffset where k=Nslotsubframe,  μ∙NT,1+NT,2+NTA,max+0.5Tsf, NT,1 is a time duration in msec of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured, NT,2 is a time duration in msec of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214], NTA,max is the maximum timing advance value in msec that can be provided by a TA command field of 12 bits, Nslotsubframe,  μ is the number of slots per subframe, Tsf is the subframe duration of 1 msec, and Koffset=Kcell,offset-KUE,offset, where Kcell,offset is provided by Koffset in ServingCellConfigCommon and KUE,offset is provided by a MAC CE command; otherwise, if not respectively provided, Kcell,offset=0 or KUE,offset=0. N1 and N2 are determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and of all configured DL BWPs for the corresponding downlink carriers. For μ=0, the UE assumes N1,0=14 [6, TS 38.214]. Slot n and Nslotsubframe,  μ are determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG. NTA,max is determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and for all configured initial UL BWPs provided by initialUplinkBWP. The uplink slot n is the last slot among uplink slot(s) overlapping with the slot(s) of PDSCH reception assuming TTA=0, where the PDSCH provides the timing advance command and TTA is defined in [4, TS 38.211].

Ericsson has a text proposal in the same section 5.2.2.5 which only moves one section of text, but it is appended to a section proposed to be deleted by the MediaTek TP.

5.2.2.5	CSI reference resource definition (MediaTek)

The CSI reference resource for a serving cell is defined as follows:

-	In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates.

-	In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, if UE is configured with the higher layer parameter CellSpecific_Koffset, n-nCSI_ref, where Koffset is a parameter consigured by higher layer as specified in [TS 36.213 Section 4.2]and where μKoffsetis the subcarrier spacing configuration for Koffset,

Koffset

-	where  +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and and  are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and  are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211]

[Section 5.2.2.5, TS 38.214, v17.0.0] (Ericsson)

-	 In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, if UE is configured with the higher layer parameter CellSpecific_Koffset, n-nCSI_ref, μKoffsetKoffsetotherwise,

-	Koffset is provided with a value of ms for frequency range 1 and is equal to CellSpecific_Koffset - UESpecific_Koffset if UESpecific_Koffset is provided in MAC CE and CellSpecific_Koffset, otherwise. μKoffsetis the subcarrier spacing configuration for Koffset;

MediaTek proposes similar changes to sections 6.1.2.1 and 6.2.1.

6.1.2.1	Resource allocation in time domain (MediaTek)

/* No change to Omitted Text ---*/

in CSI-ReportConfig for the  triggered CSI Reporting Settings and  is the (m+1)th entry of .

-	The slot Ks where the UE shall transmit the PUSCH is determined by K2 as Ks =, if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, Ks=n⋅2μPUSCH2μPDCCH+K2+Koffset⋅2μPUSCH2μKoffset, where Koffset is a parameter consigured by higher layer as specified in [TS 36.213 Section 4.2], and where μKoffsetis the subcarrier spacing configuration for Koffset, n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH,  and  are the subcarrier spacing configurations for PUSCH and PDCCH, respectively,Koffset and the scheduling DCI is other than DCI format 0_0 with CRC scrambled by TC-RNTI.

/* No change to Omitted Text ---*/

6.2.1	UE sounding procedure (MediaTek)

/* No change to Omitted Text ---*/

If the UE receives the DCI triggering aperiodic SRS in slot n and except when SRS is configured with the higher layer parameter SRS-PosResource, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, Ks=n⋅2μSRS2μPDCCH+K2k+Koffset⋅2μSRS2μKoffset, where Koffset is a parameter consigured by higher layer as specified in [TS 36.213 Section 4.2], and where

-	k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

-	μKoffsetis the subcarrier spacing configuration for KoffsetKoffset.

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the  Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and when SRS is configured with the higher layer parameter SRS-PosResource, the UE transmits every aperiodic SRS resource in each of the triggered SRS resource set(s) in slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, Ks=n⋅2μSRS2μPDCCH+K2k+Koffset⋅2μSRS2μKoffset, where Koffset is a parameter consigured by higher layer as specified in [TS 36.213 Section 4.2], and where

-	k is configured via higher layer parameter slotOffset for each aperiodic SRS resource in each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

Koffset

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the  Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

/* No change to Omitted Text ---*/

[Section 6.2.1, TS 38.214, v17.0.0] (Ericsson)

-	If the UE receives the DCI triggering aperiodic SRS in slot n and at least one resource set is configured with parameter availableSlotOffset across all configured BWPs in a component carrier except when SRS is configured with the higher layer parameter SRS-PosResource, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in the (t + 1)-th available slot counting from slot if ca-SlotOffset is configured, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in the (t + 1)-th available slot counting from slot n⋅2μSRS2μPDCCH+k+Koffset⋅2μSRS2μKoffset, if UE is configured with the higher layer parameter CellSpecific_Koffset,  otherwise the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in the (t + 1)-th available slot counting from slot n⋅2μSRS2μPDCCH+k, where

-	k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command, respectively;

-	μKoffsetis the subcarrier spacing configuration for Koffset, and Koffset is provided with a value of ms for frequency range 1 and is equal to CellSpecific_Koffset - UESpecific_Koffset if UESpecific_Koffset is provided in MAC CE and CellSpecific_Koffset otherwise.

The moderator merges the text proposals from MediaTek and Ericsson to the three TPs below.

##### 10.2.1.1 TP #5

5.2.2.5	CSI reference resource definition

The CSI reference resource for a serving cell is defined as follows:

-	In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates.

-	In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, if UE is configured with the higher layer parameter CellSpecific_Koffset, n-nCSI_ref, where Koffset is a parameter configured by higher layer as specified in [TS 36.213 Section 4.2], and where μKoffsetis the subcarrier spacing configuration for Koffset,

Koffset

-	where  +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and and  are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and  are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211]

##### 10.2.1.2 TP #6

6.1.2.1	Resource allocation in time domain

<<< unchanged paragraphs omitted >>>

in CSI-ReportConfig for the  triggered CSI Reporting Settings and  is the (m+1)th entry of .

-	The slot Ks where the UE shall transmit the PUSCH is determined by K2 as Ks =, if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, Ks=n⋅2μPUSCH2μPDCCH+K2+Koffset⋅2μPUSCH2μKoffset, where Koffset is a parameter consigured by higher layer as specified in [TS 36.213 Section 4.2], and where μKoffsetis the subcarrier spacing configuration for Koffset, n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH,  and  are the subcarrier spacing configurations for PUSCH and PDCCH, respectively, Koffsetand the scheduling DCI is other than DCI format 0_0 with CRC scrambled by TC-RNTI.

##### 10.2.1.3 TP #7

6.2.1	UE sounding procedure

-	If the UE receives the DCI triggering aperiodic SRS in slot n and none of the resource sets is configured with parameter availableSlotOffset across all configured BWPs in a component carrier, and if the UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, except when SRS is configured with the higher layer parameter SRS-PosResource, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in slot , otherwise, the UE transmits aperiodic SRS in each of the triggered resource set(s) in slot Ks=n⋅2μSRS2μPDCCH+K2+Koffset⋅2μSRS2μKoffset, where Koffset is a parameter consigured by higher layer as specified in [TS 36.213 Section 4.2], and where

-	k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

-	μKoffsetis the subcarrier spacing configuration for KoffsetKoffset.

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the  Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and when SRS is configured with the higher layer parameter SRS-PosResource, the UE transmits every aperiodic SRS resource in each of the triggered SRS resource set(s) in slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, Ks=n⋅2μSRS2μPDCCH+K2+Koffset⋅2μSRS2μKoffset, where Koffset is a parameter consigured by higher layer as specified in [TS 36.213 Section 4.2], and where

-	k is configured via higher layer parameter slotOffset for each aperiodic SRS resource in each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

- μKoffsetis the subcarrier spacing configuration for Koffset.

Koffset

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the  Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

<<< unchanged paragraphs omitted >>>

Companies are requested to indicate whether the TPs #5, #6 and #7 are agreeable.

| Company | Comments |
| --- | --- |

#### 10.2.2 TP #8

Both Ericsson and MediaTek propose a correction in section 6.2.1 by replacing parameter K2 with k. Note that this change was present in some of the text proposals in section 10.2.3 but the intent is to address it separately here as an independent correction.

- Adopt the following TP for Section 6.2.1, 38.214: “Ks=n⋅2μSRS2μPDCCH+K2 k+Koffset⋅2μSRS2μKoffset, if UE is configured with the higher layer parameter CellSpecific_Koffset, Ks =, otherwise,”

This looks like a sensible fix and the moderator requests companies to indicate whether they agree with this correction.

| Company | Comments |
| --- | --- |

#### 10.2.3 K_mac timing offset

Three companies [3, 5, 14] have identified that an agreement from RAN1#105-e is not reflected in the first Rel-17 specifications.

Agreement: If a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac, where µ is the SCS configuration for the PUCCH. Note: Here K_mac is assumed to have the unit of the PUCCH slot. This can be revisited after the K_mac signaling design is finalized.

##### 10.2.3.1 Huawei, HiSilicon TPs

- TP#3 for Clause 5.1.4.2 of TS38.214 (Huawei, HiSilicon) ============================ Unchanged Text Omitted =================================== For a UE configured with a list of semi-persistent ZP-CSI-RS-ResourceSet(s) provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList: - when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, as described in clause 6.1.3.19 of [10, TS 38.321], for ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on the PDSCH RE mapping corresponding to the activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH. - when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, as described in clause 6.1.3.19 of [10, TS 38.321], for activated ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on cessation of the PDSCH RE mapping corresponding to the de-activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH. ============================ Unchanged Text Omitted ===================================

- TP#4 for Clause 5.1.5 of TS38.214 (Huawei, HiSilicon) ============================ Unchanged Text Omitted =================================== When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH. If tci-PresentInDCI is set to 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET scheduling the PDSCH, and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA', and when applicable, also with respect to qcl-Type set to 'typeD'. ============================ Unchanged Text Omitted ===================================

- TP#5 for Clause 5.2.1.5 of TS38.214 (Huawei, HiSilicon) ============================ Unchanged Text Omitted =================================== A trigger state is initiated using the CSI request field in DCI. - When all the bits of CSI request field in DCI are set to zero, no CSI is requested. - When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to trigger states to the codepoints of the CSI request field in DCI. is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH. ============================ Unchanged Text Omitted ===================================

##### 10.2.3.2 OPPO TPs

5.1.4.2 	PDSCH resource mapping with RE level granularity (OPPO)

*** < Unchanged parts are omitted> ***

For a UE configured with a list of semi-persistent ZP-CSI-RS-ResourceSet(s) provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList:

-	when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the activation command, as described in clause 6.1.3.19 of [10, TS 38.321], for ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on the PDSCH RE mapping corresponding to the activated ZP CSI-RS resource(s) shall be applied starting from the first   slot that is after slot n+3Nslotsubframe,µ+ K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided.

-	when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the activation command, as described in clause 6.1.3.19 of [10, TS 38.321], for activated ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on cessation of the PDSCH RE mapping corresponding to the de-activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+ K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided.

*** < Unchanged parts are omitted> ***

5.1.5 	Antenna ports quasi co-location (OPPO)

*** < Unchanged parts are omitted> ***

When the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ + K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided. If tci-PresentInDCI is set to 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET scheduling the PDSCH, and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA', and when applicable, also with respect to qcl-Type set to 'typeD'.

*** < Unchanged parts are omitted> ***

5.2.1.5.1 	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology (OPPO)

*** < Unchanged parts are omitted> ***

A trigger state is initiated using the CSI request field in DCI.

-	When all the bits of CSI request field in DCI are set to zero, no CSI is requested.

-	When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where  is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to  trigger states to the codepoints of the CSI request field in DCI.  is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ + K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided.

*** < Unchanged parts are omitted> ***

5.2.1.5.2	Semi-persistent CSI/Semi-persistent CSI-RS (OPPO)

*** < Unchanged parts are omitted> ***

For semi-persistent reporting on PUCCH, the PUCCH resource used for transmitting the CSI report are configured by reportConfigType. Semi-persistent reporting on PUCCH is activated by an activation command as described in clause 6.1.3.16 of [10, TS 38.321], which selects one of the semi-persistent Reporting Settings for use by the UE on the PUCCH. When the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the activation command, the indicated semi-persistent Reporting Setting should be applied starting from the first  slot that is after slot n+3Nslotsubframe,µ + K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided.

*** < Unchanged parts are omitted> ***

5.2.1.5.2	Semi-persistent CSI/Semi-persistent CSI-RS (OPPO)

*** < Unchanged parts are omitted> ***

For a UE configured with CSI resource setting(s) where the higher layer parameter resourceType set to 'semiPersistent'.

-	when a UE receives an activation command, as described in clause 6.1.3.12 of [10, TS 38.321], for CSI-RS resource set(s) for channel measurement and CSI-IM/NZP CSI-RS resource set(s) for interference measurement associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the selection command, the corresponding actions in [10, TS 38.321] and the UE assumptions (including QCL assumptions provided by a list of reference to TCI-State's, one per activated resource) on CSI-RS/CSI-IM transmission corresponding to the configured CSI-RS/CSI-IM resource configuration(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+ K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided. If a TCI-State referred to in the list is configured with a reference to an RS configured with qcl-Type set to 'typeD', that RS can be an SS/PBCH block, periodic or semi-persistent CSI-RS located in same or different CC/DL BWP.

-	when a UE receives a deactivation command, as described in clause 6.1.3.12 of [10, TS 38.321], for activated CSI-RS/CSI-IM resource set(s) associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the deactivation command, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of CSI-RS/CSI-IM transmission corresponding to the deactivated CSI-RS/CSI-IM resource set(s) shall apply starting from the first slot that is after slot n+3Nslotsubframe,µ + K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided.

*** < Unchanged parts are omitted> ***

5.2.4	CSI reporting using PUCCH (OPPO)

*** < Unchanged parts are omitted> ***

A UE shall perform semi-persistent CSI reporting on the PUCCH applied starting from the first slot that is after slot n+3Nslotsubframe,µ+ K_mac when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the activation command described in clause 6.1.3.16 of [10, TS 38.321] where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided. The activation command will contain one or more Reporting Settings where the associated CSI Resource Settings are configured. Semi-persistent CSI reporting on the PUCCH supports Type I CSI. Semi-persistent CSI reporting on the PUCCH format 2 supports Type I CSI with wideband frequency granularity. Semi-persistent CSI reporting on PUCCH formats 3 or 4 supports Type I CSI with wideband and sub-band frequency granularities and Type II CSI Part 1.

*** < Unchanged parts are omitted> ***

##### 10.2.3.3 Lenovo, Motorola Mobility TPs

5.1.4.2	PDSCH resource mapping with RE level granularity (Lenovo, Motorola Mobility)

<Unrelated parts omitted>

For a UE configured with a list of semi-persistent ZP-CSI-RS-ResourceSet(s) provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList:

-	when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, as described in clause 6.1.3.19 of [10, TS 38.321], for ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on the PDSCH RE mapping corresponding to the activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ∙kmac, where  is the SCS configuration for the PUCCH, .and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided

-	when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, as described in clause 6.1.3.19 of [10, TS 38.321], for activated ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on cessation of the PDSCH RE mapping corresponding to the de-activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ∙kmac, where  is the SCS configuration for the PUCCH, .and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided.

<Unrelated parts omitted>

5.1.5	Antenna ports quasi co-location (Lenovo, Motorola Mobility)

<Unrelated parts omitted>

When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ∙kmac, where  is the SCS configuration for the PUCCH, .and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided.. If tci-PresentInDCI is set to 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET scheduling the PDSCH, and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA', and when applicable, also with respect to qcl-Type set to 'typeD'.

<Unrelated parts omitted>

5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology (Lenovo, Motorola Mobility)

<Unrelated parts omitted>

A trigger state is initiated using the CSI request field in DCI.

-	When all the bits of CSI request field in DCI are set to zero, no CSI is requested.

-	When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where  is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to  trigger states to the codepoints of the CSI request field in DCI.  is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ +2μ∙kmac, where  is the SCS configuration for the PUCCH, .and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided .

<Unrelated parts omitted>

5.2.1.5.2	Semi-persistent CSI/Semi-persistent CSI-RS (Lenovo, Motorola Mobility)

For semi-persistent reporting on PUSCH, a set of trigger states are higher layer configured by CSI-SemiPersistentOnPUSCH-TriggerStateList, where the CSI request field in DCI scrambled with SP-CSI-RNTI activates one of the trigger states. A UE is not expected to receive a DCI scrambled with SP-CSI-RNTI activating one semi-persistent CSI report with the same CSI-ReportConfigId as in a semi-persistent CSI report which is activated by a previously received DCI scrambled with SP-CSI-RNTI.

For semi-persistent reporting on PUCCH, the PUCCH resource used for transmitting the CSI report are configured by reportConfigType. Semi-persistent reporting on PUCCH is activated by an activation command as described in clause 6.1.3.16 of [10, TS 38.321], which selects one of the semi-persistent Reporting Settings for use by the UE on the PUCCH. When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated semi-persistent Reporting Setting should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

For a UE configured with CSI resource setting(s) where the higher layer parameter resourceType set to 'semiPersistent'.

-	when a UE receives an activation command, as described in clause 6.1.3.12 of [10, TS 38.321], for CSI-RS resource set(s) for channel measurement and CSI-IM/NZP CSI-RS resource set(s) for interference measurement associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the selection command, the corresponding actions in [10, TS 38.321] and the UE assumptions (including QCL assumptions provided by a list of reference to TCI-State's, one per activated resource) on CSI-RS/CSI-IM transmission corresponding to the configured CSI-RS/CSI-IM resource configuration(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ +2μ∙kmac, where  is the SCS configuration for the PUCCH, .and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided. If a TCI-State referred to in the list is configured with a reference to an RS configured with qcl-Type set to 'typeD', that RS can be an SS/PBCH block, periodic or semi-persistent CSI-RS located in same or different CC/DL BWP.

-	when a UE receives a deactivation command, as described in clause 6.1.3.12 of [10, TS 38.321], for activated CSI-RS/CSI-IM resource set(s) associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of CSI-RS/CSI-IM transmission corresponding to the deactivated CSI-RS/CSI-IM resource set(s) shall apply starting from the first slot that is after slot n+3Nslotsubframe,µ +2μ∙kmac, where  is the SCS configuration for the PUCCH, .and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided .

##### 10.2.3.4 Discussion

OPPO proposes adding a clarification that “K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided”. Lenovo, Motorola Mobility make similar kind of addition. We can find this statement in section 8.2 of TS 36.213 so it is probably not necessary to repeat it in each respective section of TS 36.213.

8.2	Random access response - Type-1 random access procedure

In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in clause 10.1. The window starts after an additional TTA+kmac msec where TTA is defined in [4, TS 38.211] and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided.The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

The moderator would also draw attention to the fact that the agreement quoted above had this note:

Note: Here K_mac is assumed to have the unit of the PUCCH slot. This can be revisited after the K_mac signaling design is finalized.

Thus, the moderator is of the opinion that it is not necessary to clarify that Kmac is a number of slots in the equations.

This would leave the clarifications of “uplink” we can find in the OPPO text proposals to address.

As a first proposal, the moderator provides five TPs (#9, #10, #11, #12 and #13) below where only the Kmac summand is retained according to the discussion above. Companies are invited to indicate whether

- they support the simplified text of only reflecting the addition of Kmac, and

- whether there is a need to reflect the clarification of uplink slot from OPPO

| Company | Comments |
| --- | --- |

##### 10.2.3.5 TP #9

5.1.4.2 PDSCH resource mapping with RE level granularity <<< unchanged paragraphs omitted >>> For a UE configured with a list of semi-persistent ZP-CSI-RS-ResourceSet(s) provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList: - when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, as described in clause 6.1.3.19 of [10, TS 38.321], for ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on the PDSCH RE mapping corresponding to the activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH. - when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, as described in clause 6.1.3.19 of [10, TS 38.321], for activated ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on cessation of the PDSCH RE mapping corresponding to the de-activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH. <<< unchanged paragraphs omitted >>>

##### 10.2.3.6 TP #10

5.1.5	Antenna ports quasi co-location

<<< unchanged paragraphs omitted >>>

When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH. If tci-PresentInDCI is set to 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET scheduling the PDSCH, and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA', and when applicable, also with respect to qcl-Type set to 'typeD'.

<<< unchanged paragraphs omitted >>>

##### 10.2.3.7 TP #11

5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology

<<< unchanged paragraphs omitted >>>

A trigger state is initiated using the CSI request field in DCI.

-	When all the bits of CSI request field in DCI are set to zero, no CSI is requested.

-	When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where  is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to  trigger states to the codepoints of the CSI request field in DCI.  is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH.

<<< unchanged paragraphs omitted >>>

##### 10.2.3.8 TP #12

5.2.1.5.2	Semi-persistent CSI/Semi-persistent CSI-RS

For semi-persistent reporting on PUSCH, a set of trigger states are higher layer configured by CSI-SemiPersistentOnPUSCH-TriggerStateList, where the CSI request field in DCI scrambled with SP-CSI-RNTI activates one of the trigger states. A UE is not expected to receive a DCI scrambled with SP-CSI-RNTI activating one semi-persistent CSI report with the same CSI-ReportConfigId as in a semi-persistent CSI report which is activated by a previously received DCI scrambled with SP-CSI-RNTI.

For semi-persistent reporting on PUCCH, the PUCCH resource used for transmitting the CSI report are configured by reportConfigType. Semi-persistent reporting on PUCCH is activated by an activation command as described in clause 6.1.3.16 of [10, TS 38.321], which selects one of the semi-persistent Reporting Settings for use by the UE on the PUCCH. When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated semi-persistent Reporting Setting should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH.

For a UE configured with CSI resource setting(s) where the higher layer parameter resourceType set to 'semiPersistent'.

-	when a UE receives an activation command, as described in clause 6.1.3.12 of [10, TS 38.321], for CSI-RS resource set(s) for channel measurement and CSI-IM/NZP CSI-RS resource set(s) for interference measurement associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the selection command, the corresponding actions in [10, TS 38.321] and the UE assumptions (including QCL assumptions provided by a list of reference to TCI-State's, one per activated resource) on CSI-RS/CSI-IM transmission corresponding to the configured CSI-RS/CSI-IM resource configuration(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH. If a TCI-State referred to in the list is configured with a reference to an RS configured with qcl-Type set to 'typeD', that RS can be an SS/PBCH block, periodic or semi-persistent CSI-RS located in same or different CC/DL BWP.

-	when a UE receives a deactivation command, as described in clause 6.1.3.12 of [10, TS 38.321], for activated CSI-RS/CSI-IM resource set(s) associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of CSI-RS/CSI-IM transmission corresponding to the deactivated CSI-RS/CSI-IM resource set(s) shall apply starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH.

<<< unchanged paragraphs omitted >>>

##### 10.2.3.9 TP #13

5.2.4	CSI reporting using PUCCH

A UE is semi-statically configured by higher layers to perform periodic CSI Reporting on the PUCCH. A UE can be configured by higher layers for multiple periodic CSI Reports corresponding to multiple higher layer configured CSI Reporting Settings, where the associated CSI Resource Settings are higher layer configured. Periodic CSI reporting on PUCCH formats 2, 3, 4 supports Type I CSI with wideband granularity.

A UE shall perform semi-persistent CSI reporting on the PUCCH applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command described in clause 6.1.3.16 of [10, TS 38.321] where  is the SCS configuration for the PUCCH. The activation command will contain one or more Reporting Settings where the associated CSI Resource Settings are configured. Semi-persistent CSI reporting on the PUCCH supports Type I CSI. Semi-persistent CSI reporting on the PUCCH format 2 supports Type I CSI with wideband frequency granularity. Semi-persistent CSI reporting on PUCCH formats 3 or 4 supports Type I CSI with wideband and sub-band frequency granularities and Type II CSI Part 1.

#### 10.2.4 TP #14

OPPO has two further text proposals for TS 38.214 where it is again proposed to add clarification that uplink slots are being addressed in the specification text.

As a first proposal, companies are invited to indicate whether

- there is a need to reflect the clarification of uplink slot from OPPO

| Company | Comments |
| --- | --- |

6.2.1	UE sounding procedure (OPPO)

*** < Unchanged parts are omitted> ***

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource is set to 'semi-persistent':

-	when a UE receives an activation command, as described in clause 6.1.3.17 or 6.1.3.36 of [10, TS 38.321], for an SRS resource, and when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the activation command, the corresponding actions in [10, TS 38.321] and the UE assumptions on SRS transmission corresponding to the configured SRS resource set shall be applied starting from the first uplink slot that is after uplink slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. The activation command also contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the activated SRS resource set.

*** < Unchanged parts are omitted> ***

-	when a UE receives a deactivation command [10, TS 38.321] for an activated SRS resource set, and when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the deactivation command, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of SRS transmission corresponding to the deactivated SRS resource set shall apply starting from the first uplink slot that is after uplink slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

*** < Unchanged parts are omitted> ***

6.2.1	UE sounding procedure (OPPO)

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource is set to 'aperiodic':

-	the UE receives a configuration of SRS resource sets,

*** < Unchanged parts are omitted> ***

-	when a UE receives an spatial relation update command, as described in clause 6.1.3.26 of [10, TS 38.321], for an SRS resource configured with the higher layer parameter SRS-Resource, and when the HARQ-ACK corresponding to the PDSCH carrying the update command is transmitted in uplink slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions on updating spatial relation for the SRS resource shall be applied for SRS transmission starting from the first uplink slot that is after uplink slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. The update command contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the updated SRS resource set. Each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the update command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the update command if present, same serving cell and bandwidth part as the SRS resource set otherwise. When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', the UE shall not expect to be configured with different spatial relations for SRS resources in the same SRS resource set.

*** < Unchanged parts are omitted> ***

# References

R1-2112716, Feature lead summary#5 on timing relationship enhancements, Moderator (Ericsson)

R1-2200875. LS on NTN-specific SIB, RAN2

R1-2200937, Maintenance on timing relationship enhancements for NTN, Huawei, HiSilicon

R1-2201215, Timing relationship enhancements for NR-NTN, MediaTek Inc.

R1-2201271, Discussion on remaining issue for timing relationship enhancement, OPPO

R1-2201358, Remaining issues on timing relationship enhancements for NTN, CATT

R1-2201546, Discussion on timing relationship enhancements for NTN, Spreadtrum Communications

R1-2201645, Maintenance aspects of time relations for Rel-17 NR over NTN, Nokia, Nokia Shanghai Bell

R1-2201744, Remaining issues on timing relationship enhancement for NTN, InterDigital, Inc.

R1-2201771, Remaining Issues of Timing Relationship Enhancements for NR NTN, Apple

R1-2201804, On timing relationship maintenance issues for NR NTN, Ericsson Hungary Ltd

R1-2201852, Remaining issues on timing relationship enhancements for NTN, CMCC

R1-2201921, Remaining issues on the timing relationship for NTN, Xiaomi

R1-2201969, Discussion on NTN timing relationship, Lenovo, Motorola Mobility

R1-2202011, Maintenance issues on Timing relationship enhancements for NTN, Samsung

R1-2202206, Remaining issues of timing relationship for NR-NTN, ZTE

R1-2202241, Remaining issues on timing relationship enhancement for NTN, Baicells

R1-2202285, Remaining issues on timing relationship enhancements in NTN, LG Electronics

R1-2202360, Remaining issues on timing relationship enhancements for NTN, NEC

R1-2201646, Maintenance aspects of time and frequency synchronization for Rel-17 NR over NTN, Nokia

R1-2201853, Remaining issues on enhancements on UL time and frequency synchronization for NTN, CMCC

R1-2201216, Enhancements on UL Time and Frequency Synchronisation for NR-NTN, MediaTek Inc

R1-2201387, Enhancements on UL time and frequency synchronization, Panasonic R&D Center