3GPP TSG RAN WG1 Meeting #108-e	R1-2202606

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.4.1

Source:	Moderator (Huawei, HiSilicon)

Title:	Feature lead summary#1 on timing relationship enhancements

Document for:	Discussion

Introduction

The Rel-17 work item on solutions for NR to support NTN was approved at RAN#86 and the first Rel-17 specifications were submitted to RAN#94e. RAN WG1 has entered the Rel-17 maintenance stage of this work item where one focus area are the timing relationship enhancements for NTN.

In this contribution, we summarize the related issues and proposals based on the contributions submitted to RAN1#108-e under agenda item 8.4.1 [4] – [18].

[108-e-R17-NR-NTN-01] Email discussion for maintenance on timing relationship enhancements – Jussi (Huawei)

- 1st check point: February 25

- Final check point: March 3

Since we have the first GTW on NR-NTN on Wednesday, during this round of the email discussion, please comment on the issues and text proposals before Feb, 23rd, Wednesday, UTC 16:59.

Particularly, the LS in issue#9 benefits from quick convergence regarding whether and what kind of response we would draft to RAN2. Early comments would be greatly appreciated.

1 [ACTIVE] Issue#1: K_offset update

1.1 On the update of cell specific K_offset during SIB modification period

1.1.1 Background

During RAN WG1#107e there was a long discussion on whether there is a need to resolve ambiguity of which cell-specific K_offset value to use during the SIB modification period [1]. At that meeting 15 companies considered that there is no need for specification enhancement and two companies saw a need for further study. There are eight input contributions to this meeting that address this issue.

[CATT]

Proposal 2: There is no need of specification enhancement for the ambiguity of which cell-specific K_offset value to use during the SIB modification period.

[Spreadtrum]

Proposal 1: There is no ambiguity of which cell-specific K_offset value to use during the SIB modification period.

[Xiaomi]

Proposal 1: The ambiguity issue on the update of cell-specific K_offset can be handled by the network implementation.

[ZTE]

Proposal-1: The ambiguity of which cell-specific K_offset value to use during the SIB modification period can be resolved by gNB implementation without specification impact

[Baicells]

Proposal 1: Define the application time of the updated cell-specific K_offset to be the end of the modification period that contains the SIB for cell-specific K_offset update.

[NEC]

Proposal 1. There is no ambiguity of which cell-specific K_offset value to use during the SIB modification period. It could be handled by gNB implementation as legacy.

[CMCC]

Proposal 1: To address the ambiguity issue during the SIB modification period, it is preferred to leave to gNB implementation, e.g., scheduling restriction.

[Nokia]

Proposal 1: The application time of the updated cell-specific K_offset shall be the same for a UE acquiring the new SI via RRC or via SIB acquisition.

Proposal 2: The application time of the updated K_offset at cell level needs to pre-defined and different from the first SIB occasion in the modification period.

Proposal 3: RAN1 shall discuss the rules for the application time of cell-specific K_offset.

Proposal 4: As options for the application time of the recently acquired updated K_offset we propose one of the following alternatives with a slight preference for (B):

- The end of the first (or the n-th) SI-window for the SIB containing K_offset in the modification period

- The end of the first modification period after the update

- A specific SFN.

Proposal 7: RAN1 shall discuss UE behavior if UE fails to acquire updated K_offset within valid time.

Proposal 8: RAN1 to consider alternative procedures for restoring K_offset before declaring radio link failure.

We continue to have a large number of companies that do not consider it necessary to introduce any specification enhancement to address the potential ambiguity regarding which cell-specific K_offset value to use during the SIB modification period.

1.1.2 Company views

Initial proposal 1.1 (Moderator)

Moderator recommendation is not to address this in Rel-17 maintenance, but please provide your opinion below.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | We believe this topic is related to maintenance rather than optimization of current agreements. Mediatek and Spreadtrum argue that there is no ambiguity. But in their documents, they both provide different understanding of the point of application. Which is exactly what we want to avoid. Other companies understand that there is such ambiguity. CATT, Xiaomi, ZTE and CMCC seem to argue in favor of scheduling restrictions. This would cause the network to waste considerable amount of resources, as no user could be allocated in UL for the whole duration of the ambiguity period. In NTN coverage may be scarce in time, and the PHY efficiency is already lower. Leaving this for NW implementation may cause more harm than just deciding the point of application. Baicells propose a point of application in their document. We think we should discuss this approach. This is not the same as legacy updates. Any mismatch before would not be so damaging for PHY allocation resources. This time, a misunderstanding between gNB and UE will cause the UE to use the wrong UL transmit timing and hence create unwanted interference. |
| Apple | Fine with the proposal. It can be left to network implementation. |
| Lenovo | We think this can be up to implementation. |
| Huawei, HiSilicon | Agree with FL proposal with the understanding that the update of cell-specific K_offset can be very infrequent and there is no need to optmize a case. At minimum, one can always configure the most conservative K_offset values for the cell without making update in SIB. As long as the UE gets connected, UE-specific K_offset can be configured. Therefore, the system works fine. |
| ZTE | Support. Network implementation is enough to handle the ambiguity issue and there is no obvious impact on throughput since update of cell specific K_offset is low frequency event. |
| Thales | We support Moderator recommendation |
| NEC | Agree with Moderator’s proposal. It is not necessary to introduce any specification enhancement to address the potential ambiguity. It could be left to NW implementation. |
| Sony | Support moderator’s recommendation. |
| Panasonic | We support the moderator’s proposal. Update of the Koffset can be avoided with appropriate cell specific Koffset setting. |
| Intel | Agree with the moderator |
| Samsung | OK with the proposal 1.1. |
| InterDigital | Agree with the moderator |
| OPPO | We agree with the moderator. It can be left to the network implementation . |
| CATT | Agree with the moderator |
| Xiaomi | We are fine to leave it to UE implementation. |
| CMCC | Agree with Moderator’s proposal. The ambiguity issue could leave to gNB implementation. |
| QC | Network implementation can solve the problem but the cost may be high. Further discussion is needed. |
| NTT DOCOMO | We are fine with the proposal. |
| LG Electronics | This issue could leave to gNB implementation. |
| Baicells | There will be cost in scheduling if no enhancement is done. But we will be fine to agree with the moderator’s proposal. |
| MediaTek | Support, it is not necessary to introduce any specification enhancement. |
| Ericsson | OK with the proposal |
| LM | Agree with Moderator proposal. (Agree with Huawei comment) |

1.1.3 Summary of 1st round discussions

The status with respect to whether to resolve the ambiguity of which cell-specific K_offset value to use during the SIB modification period remains largely unchanged from RAN1#107-e meeting.

| Views | Proponent(s) |
| --- | --- |
| No need of specification enhancement | [20] sources: [Apple, Lenovo/MM, Huawei/HiSilicon, ZTE, Thales, NEC, Sony, Panasonic, Intel, Samsung, InterDigital, OPPO, CATT, Xiaomi, CMCC, NTT DOCOMO, LGE, MediaTek, Ericsson, Lockheed Martin] |
| Need further study | [2] sources: [Nokia/NSB, Qualcomm] |
| Ready to compromise | [1] source: Baicells |

It is again difficult to see how to move forward at this meeting. Nokia addresses the same concerns we have seen at the previous meeting but have not been able to convince other companies. Please let the moderator know if there is any progress offline on this topic.

1.2 On the update of UE specific K_offset with MAC CE

1.2.1 Background

Two companies point out the potential ambiguity occurring when UE specific K_offset is updated with MAC CE command.

[CMCC]

Proposal 2: In order to address the ambiguity period issue for MAC CE updating UE specific K_offset, support if cell-specific K_offset is always used for the additional transmission timings related to fallback DCI format, use fallback DCI in the ambiguity period.

Proposal 3: Support always use the cell-specific K_offset for the additional transmission timings related to fallback DCI format.

[Nokia]

Proposal 5: K_offset component updated via MAC-CE to be valid from the start of the beggining of the slot m+3 Nslotsubframe,μ+1 , where n is the uplink slot where the MAC-CE command was received.

Proposal 6: UEs configured for using UE-specific K_offset should update the cell-specific component of the UE-specific K_offset using the same procedure and point of application that will be specified for the cell-specific K_offset

The problem is identified to exist in between the times when the gNB cannot be certain that the UE applies either the old or new UE specific K_offset as shown in the figure below.

1.2.2 Company views

Initial proposal 1.2 (Moderator)

The moderator would like to get companies thoughts on whether this is a topic we need to address at the maintenance phase or whether the impact can be mitigated by gNB implementation. In particular, what is the thinking regarding the use of cell-specific K_offset for the additional transmission timings related to fallback DCI format?

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Fallback DCI formats should be using the cell specific value for K_offset. But we need to clarify Nokia‘s position: we are not claiming ambiguity or a problem. We just want to have captured in specificiation from what slot this new value will be valid. The UE will receive a MAC-CE command and will apply it. For all MAC-CE commands that alter DL or UL behavior, similar actions are described in the specs. This is even more drastic in terms of interference than a Timing Advance command, and even for timing advance command the specifications have decided where it should be accounted as valid. |
| Apple | We do not support to always use cell-specific K_offset for additional transmission timings related to fallback DCI format. |
| Lenovo | We don’t think there is ambiguilty for the activation of MAC CE. We don’t think it is necessary to use cell-specific K-offset for fallback DCI. |
| Huawei, HiSilicon | We tend to agree that the issue is valid and also agree that the simplest solution is to always use cell-specific K_offset for fallback DCI formats. |
| ZTE | We do not think there is ambiguity issue. The MAC CE should be activated after the HARQ ACK. Therefore, the old UE specific K_offset should be adopted during Period 2 and there is no ambiguity. |
| NEC | We do not think there an issue. Similar to the ‘issue’ we discussed in Section 1.1, this could be left to gNB implementation. |
| Panasonic | The same principle as MAC CE action timing of other MAC CE command should be applied. Because MAC CE for UE specific update of Koffset is UL related command, the timing should be slot n + 3ms (i.e. slot m) in the above figure. The related discussion is 10.1.3 in this document in our understanding. |
| Intel | In our understanding there is no ambiguity if the UE applies the new Koffset only for UL tX corresponding to the UL grants after the MAC CE activation time. |
| Samsung | We don’t think there is an ambiguity. It is up to gNB implementation. |
| InterDigital | We don’t think there is ambiguity issue and similar view with ZTE. |
| OPPO | This topic is similar to the section 1.1, it can be left to the network implementation. In particular, the issue which K_offset should be used for the additional transmission timings related to fallback DCI format has an default option in summary at RAN#1 106bis-e that “UE-specific K_offset if provided (otherwise, the cell-specific K_offset)” is used for the additional transmission timings related to fallback DCI formats. So there is no need to discuss the additional transmission timings related to fallback DCI format. |
| CATT | Using cell specific K-offset for fallback DCI would be one safe choice. |
| Xiaomi | We don’t see the ambiguity issue here. gNB and UE have common understanding on the timing of MAC CE take effects. |
| CMCC | As shown in the figure above, our understanding is that in Period 2 (based on our analysis in R1-2201852, the duration of Period 2 is about RTT), gNB can NOT assure whether new or old UE specific K_offset is applied at UE side, which depends on whether the PDSCH carrying MAC CE is successfully decoded or not. We agree the ambiguity issue in Period 2 may be resolved up to gNB implementation, but there was a performance cost due to the time duration of Period 2 cannot be used for scheduling. In terrestrial network, the performance cost seems negligible, since the duration of Period 2 is very short (e.g., less than 1 slot). However, in NTN, the duration of Period 2 is very large (e.g., for GEO, duration of Period 2 ≈ RTT = 541 ms), thus the performance loss is significant. In line with MAC CE signalling, update of UE specific K_offset seems not like a low frequency event. Thus, based on the above discussion, the ambiguity period issue for MAC CE updating UE specific K_offset needs to be resolved. And support the use of cell-specific K_offset for the additional transmission timings related to fallback DCI format, which seems to be the simplest solution. |
| QC | There is an ambiguity period on the network side. However, the problem of the ambiguity on the network side can be avoided by network implementation. |
| NTT DOCOMO | We think it can be handled by gNB. |
| LG Electronics | This issue could leave to gNB implementation. |
| Baicells | gNB and UE should have common understanding on the MAC CE’s application timing. For the Network, the application time should be after it after it receives the HARQ ACK. Based on this assumption, there will be no ambiguity period. |
| MediaTek | The potential issue of ambiguity period can be handled by gNB. |
| Ericsson | We have the same view as CMCC. |
| LM | Agree that an ambiguity period exists. The issue can be addressed by network implementation, but at a performance cost. To prevent UL collisions, the scheduler would need to reserve double the UL resource, as at the time of scheduling, it would not know if the MAC CE UE-specific K_offset update had been successfully received. |

1.2.3 Summary of 1st round of discussions

Company views are summarized in the table below.

| Option | For | Against |
| --- | --- | --- |
| Use the cell-specific K_offset for the additional transmission timings related to fallback DCI format. | [5] sources: Nokia/NSB, Huawei/HiSilicon, CATT, CMCC, Ericsson | [10] sources: Apple, Lenovo, ZTE, NEC, Panasonic, Intel, Samsung, InterDigital, OPPO, Xiaomi |
| There is an ambiguity issue to be resolved | [6] sources: Nokia/NSB, Huawei/HiSilicon, CATT, CMCC, Ericsson, Lockheed Martin | [15] sources: Apple, Lenovo, ZTE, NEC, Panasonic, Intel, Samsung, InterDigital, OPPO, Xiaomi, Qualcomm, NTT DOCOMO, LGE, Baicells, MediaTek |

The majority of respondents do not see an issue to be resolved, and it is difficult to see what kind of compromise could be proposed.

2 [CLOSED] Issue#2: MAC CE timing relationships

2.1 Background

We have an agreement from several meetings ago that K_mac update is carried in system information. Proposals for additional mechanisms did not receive any support at RAN WG1#107e meeting, either. Two companies have proposals related to this issue of K_mac update.

[CATT]

Proposal 3: Support the update of K_mac with MAC CE.

[Spreadtrum]

Proposal 2: K_mac can be update based on SIB update mechanism

The moderator observes that CATT repeats the proposal from #107e meeting where it did not get support. Also the lack of input contributions on this topic would indicate that companies are indeed fine with only providing K_mac update via system information.

2.2 Company views

Initial proposal 2 (Moderator)

Moderator recommendation is not to address this issue in Rel-17 maintenance.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | We agree with moderator. |
| Apple | We are fine with the proposal. |
| Lenovo | Agree with moderator. |
| Huawei, HiSilicon | Support the FL reccomendation |
| ZTE | Support |
| Thales | Support Moderator recommendation |
| NEC | We support this proposal. |
| Sony | Support moderator’s recommendation. |
| Panasonic | We support the moderator’s proposal. |
| Intel | Agree |
| Samsung | OK. |
| InterDigital | Support |
| OPPO | We agree with the moderator. |
| CATT | We can agree the majority views. |
| Xiaomi | Support |
| CMCC | Agree with moderator. |
| QC | Agree |
| NTT DOCOMO | Support |
| LG Electronics | Agree with FL. |
| Baicells | Agree |
| MediaTek | Support |
| Ericsson | Agree |
| Lockheed Martin | Agree |

  - Summary of 1st round of discussions

There is unanimous support to the moderator recommendation, so the issue is moved to email discussion to reach a conclusion before Friday 25th February checkpoint.

Based on the support for moderator recommendation, the following proposal is made:

Proposal 2a (Moderator)

Update of K_mac with MAC CE is not supported in Rel-17 NTN

3 [CLOSED] Issue#3: K1 range extension

3.1 Background

At RAN WG1#107e meeting no agreements were reached on K1 range extension enhancements beyond the already agreed range extension itself. Three companies contributed to this meeting and these proposals are in alignment what was already concluded last time about keeping the PDSCH-to-HARQ feedback timing indicator unchanged.

[Xiaomi]

Proposal 2: The bit-length of PDSCH-to-HARQ_feedback timing indicator field in the fallback DCI is kept unchanged.

[NEC]

Proposal 2. There is no need to extend the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI when the range of the K1 value is extended

[Apple]

Proposal 2: The K1 range extension in unpaired spectrum does not change the PDSCH-to-HARQ_feedback timing indicator field size in DCI.

For non-fallback DCI, only extend the value range of entries in the configured dl-DataToUL-ACK table.

For fallback DCI, introduce a scaling factor when determining K1 value.

3.2 Company views

Initial proposal 3 (Moderator)

Moderator recommendation is not to address this issue in Rel-17 maintenance.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | We agree with moderator. |
| Lenovo | Agree with moderator. |
| Huawei, HiSilicon | Support the FL reccomendation |
| ZTE | OK with the proposal. |
| Thales | Support Moderator recommendation |
| NEC | Support Moderator’s proposal. |
| Sony | Support moderator’s reccomendation. |
| Panasonic | We support the moderator’s proposal. |
| Intel | OK |
| Samsung | OK. |
| InterDigital | Support |
| OPPO | We agree with the moderator. |
| CATT | OK |
| CMCC | Agree with moderator. |
| QC | Agree |
| NTT DOCOMO | Support |
| LG Electronics | OK with FL. |
| Baicells | Agree |
| MediaTek | Support |
| Ericsson | OK |
| Lockheed Martin | Agree |

3.3 Summary of 1st round of discussions

There is unanimous support to the moderator recommendation, so the issue is moved to email discussion to reach a conclusion before Friday 25th February checkpoint.

Based on the support for moderator recommendation, the following proposal is made:

Proposal 3a (Moderator)

The size of the PDSCH-to-HARQ_feedback timing indicator field in DCI is not extended when the range of the K1 value is extended in Rel-17 NTN

4 [ACTIVE] Issue#4: Configured grant type 1 timing relationship

4.1 Background

At RAN1#108e, two companies provide proposals on this topic:

[Samsung]

Proposal 3: The timing relationship for Configured Grant Type 1 should be left to Network implementation.

[Baicells]

Proposal 3: In case of PUSCH transmission with configured grant and if K_offset is updated or will be updated,

- Option1: Release configured grant before the application of new K_offset.

- Option2: Continue the PUSCH transmission with configured grant based on the new K_offset but conflict zone should be avoided.

4.2 Company views

At RAN1#107e there was also this single proposal from Samsung on the topic and the moderator recommendation back then was not to discuss further the introduction of K_offset for type 1 configured grant. Baicells proposal apparently addresses the same issue with respect to how to continue configure grant assignment when K_offset is updated. Companies views are requested on whether it is sufficient to retain the conclusion from RAN1#107e and leave the timing relationship for network implementation or consider the options proposed by Baicells.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | We support Option 2 as proposed by Baicells. |
| Apple | We could retain the conclusion from RAN1 #107e and leave it for network implementation. |
| Lenovo | When there is update of K-offset during the configured grant type 1 PUSCH, the new K-offset can be used of gNB and UE have common understanding on when to use the new K-offset. For K-offset updated by MAC CE, we think common understanding is possible. For K-offset updated by system inforamtion, we think we may need to first conclude on issue#1. Anyway, K-offset update via system information within configured grant type 1 PUSCH can be avoided by scheduling restriction/implementation. |
| Huawei, HiSilicon | Support the FL reccomendation |
| ZTE | We think there is no need to introduce K_offset for type 1 configured grant. |
| Thales | Support Moderator recommendation |
| NEC | We think this could be left to NW implementation. |
| Panasonic | Koffset is not used to determine the transmission timing of configured grant type 1 in our understanding. Therefore, update of Koffset would not impact on the configured grant PUSCH timing. |
| Samsung | We think this is up to NW implementation. |
| OPPO | We agree with ZTE. |
| CATT | No need to have additional solution, it is up to gNB implementation. |
| Xiaomi | Support |
| CMCC | We share the same view with ZTE and Panasonic. |
| QC | There is no agreement to apply Koffset for CG type-1. |
| LG Electronics | Agree with Samsung that it can be upto NW implementation. |
| Baicells | We will be OK with Moderator recommendation if majority support it. |
| MediaTek | Support FL recommendation not to discuss further the introduction of K_offset for type 1 configured grant. We can leave it for network implementation without introducing K_offset. |
| Ericsson | There is no need to apply Koffset for configured grant type 1. |
| Lockheed Martin | We agree this can be left to NW implementation |

4.3 Summary of 1st round of discussions

The company views regarding configured grant type 1 timing relationship are heavily leaning towards leaving this to implementation.

Five companies think that there is no need to apply K_offset for type 1 configured grant, and 10 companies prefer to leave this to network implementation. In addition, the proponent is ready to agree with the moderator recommendation if majority supports it.

4.4 2nd round of discussion

Considering the large majority of views expressed above, the moderator would like to close this issue and no longer pursue it in Rel-17 maintenance. Comment only if you have a major concern with this conclusion.

| Company | Comments |
| --- | --- |
| Lenovo | Support |

5 [CLOSED] Issue#5: UE reporting of information about the UE specific TA pre-compensation

5.1 Background

At RAN1#107e the RAN2 LS indicating RAN2 agreements on UE specific TA reporting. RAN1 replied indicating that event-triggers can be supported and that these event-triggers can be based on TA values. One company continues discussion on the topic at this meeting.

[CATT]

Proposal 4: On UE-specific TA reporting, periodic reporting can be supported.

Proposal 5: Using RRC signaling to report TA can be supported.

Proposal 6: Reporting differential TA between current TA and previous TA is preferred.

RAN1 already concurred with the RAN2 agreements concluding to adopt event-triggered reporting. At the maintenance phase on Rel-17 specifications it likely is not feasible for RAN1 to add reporting mechanisms above and beyond what has been communicated between the two Working Groups.

5.2 Company views

Initial proposal 5 (Moderator)

Moderator recommendation is not to address periodic UE-specific TA reporting in Rel-17 maintenance.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Agree with moderator. |
| Lenovo | Agree with moderator. |
| Huawei, HiSilicon | Support the FL reccomendation |
| ZTE | OK with the proposal. |
| Thales | Support Moderator recommendation |
| NEC | Support Moderator’s propose. |
| Sony | Support moderator’s recommendation. |
| Panasonic | Support the moderator’s proposal. |
| Samsung | OK. |
| InterDigital | Support |
| OPPO | We agree with the moderator. |
| CATT | We can agree the majority‘s view. |
| CMCC | Agree with moderator. |
| LG Electronics | Agree with FL. |
| Baicells | Agree with the proposal. |
| MediaTek | Support |
| Ericsson | Agree |
| Lockheed Martin | Agree |

5.3 Summary of 1st round of discussions

There is unanimous support to the moderator recommendation, so the issue is moved to email discussion to reach a conclusion before Friday 25th February checkpoint.

Based on the support for moderator recommendation, the following proposal is made:

Proposal 5a (Moderator)

K_offset is not introduced for type 1 configured grant in Rel-17 NTN

6 [CLOSED] Issue#6: Beam-specific K_offset in initial access

6.1 Background

Three companies provided proposals on K_offset in initial access, with particular focus on beam-specific K_offset.

[Samsung]

Proposal 1: Only Cell-specific K_offset in initial access is supported.

Proposal 2: More than one of above Koffset configurations can be supported, and using which one is dependent on gNB configuration.

[InterDigital]

Proposal-1: beam-specific K-offset indication is also supported optionally.

[CMCC]

Proposal 4: gNB has the flexibility of configuring cell-specific or beam specific value of K_offset.

- Beam specific SIB can be supported, i.e., different beam specific SIB may carry different beam specific values (e.g., K_offset).

It was already observed by the previous moderator at RAN1#107e that this issue has been discussed at many RAN1 meetings with several rounds of email discussion and debated at GTW sessions. The pros and cons of supporting beam specific K_offset configured in system information and used in initial access are clear to the group – same comments have been made by both sides over the meetings.

6.2 Company views

Initial proposal 6 (Moderator)

At this stage of Rel-17 where we already entered maintenance stage, the moderator recommendation is not to consider beam-specific K_offset for initial access.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Agree with moderator |
| Apple | Agree with moderator |
| Lenovo | Agree. |
| Huawei, HiSilicon | Support the FL reccomendation |
| ZTE | Agree |
| Thales | Agree with Moderator |
| NEC | Support Moderator’s propose. |
| Panasonic | Support the moderator’s proposal. |
| Samsung | OK. |
| InterDigital | We will be ok with the moderator’s proposal to spend time for more essential issues |
| OPPO | We agree with the moderator. |
| CATT | OK |
| NTT DOCOMO | Support |
| LG Electronics | OK |
| Baicells | OK |
| MediaTek | Support |
| Ericsson | Agree |
| Lockheed Martin | Agree |

6.3 Summary of 1st round of discussions

There is unanimous support to the moderator recommendation, so the issue is moved to email discussion to reach a conclusion before Friday 25th February checkpoint.

Based on the support for moderator recommendation, the following proposal is made:

Proposal 6a (Moderator)

Beam-specific K_offset is not supported for initial access in Rel-17 NTN

7 [ACTIVE] Issue #7: Beam application timing for PUCCH in BFR

7.1 Background

Two companies provided proposals on beam failure recovery procedure which was discussed at RAN1#107-e meeting.

[InterDigital]

Proposal-2: specify UE assumption of the beam application timing for PUCCH based on any of interpretations adopted.

Proposal-3: K-offset enhancement is supported for beam application timing for PUCCH (i.e., Interpretation-1).

[Apple]

Proposal 3: RAN1 to introduce Koffset to enhance the timing relationship on the PUCCH transmission with new beam in the beam failure recovery procedure.

At the previous meeting we reached the following agreement:

Agreement

On beam failure recovery procedure, for PRACH transmission in uplink slot n, UE monitors the corresponding PDCCH starting from downlink slot “n + K_mac + 4” within a corresponding RAR window.

There was also a long discussion on two different interpretations on the delay between PDCCH reception and application of new PUCCH beam. The majority of companies considered interpretation 1 being the correct one

- Interpretation 1: “28 symbols” is the absolute time between the time UE receives PDCCH and the time UE applies new PUCCH beam

and no enhancement would be needed.

7.2 Company views

The moderator invites companies to give their views on whether this issue should be revisited now in Rel-17 maintenance phase or close the issue for beam application timing for PUCCH with an understanding that no enhancement is needed.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | We agree with moderator |
| Apple | We think this issue needs to be revisited in Rel-17 maintenance phase. Although we agree with Interpretation 1, we still think the enhancement is needed for gNB to know the beam application timing. |
| Lenovo | We think that with interpreation 1 we can conclude that no enhancement is needed. |
| Huawei, HiSilicon | We support Interpretation 1, i.e. the 28 symbols delay is an absolute time between PDCCH reception and application of new PUCCH beam at the UE. The application timing for new PUCCH beam when the UE receives an activation command for PUCCH-SpatialRelationInfo has been defined in specification already. We are not sure there is a need for change. [TS38.213, section 9.2.2] The UE applies corresponding actions in [11, TS 38.321] and a corresponding setting for a spatial domain filter to transmit PUCCH in the first slot that is after slot where is the slot where the UE would transmit a PUCCH with HARQ-ACK information with ACK value corresponding to a PDSCH reception providing the PUCCH-SpatialRelationInfo, each slot consists of Nsymbslot symbols as defined in [4, TS 38.211], and is the SCS configuration for the PUCCH |
| ZTE | Agree that no enhancement is needed. |
| NEC | Our understanding is Interpretation 1. We agree with the moderator, no enhancement is needed. |
| Panasonic | We agree with interpretation 1 and no enhancement would not be needed. |
| Intel | Agree with Interpretation 1 |
| Samsung | OK for Interpretation 1. |
| InterDigital | We think no matter which interpretation is agreed, it should be clarified in the specification (or at least capture the interpretation 1 as conclusion). Although we prefer interpretation 2, we will be ok with interpretation 1 if majority support it. |
| OPPO | We agree with the moderator. |
| CATT | Agree with Interpretation 1 |
| Xiaomi | No enhancement is needed |
| CMCC | We agree with interpretation 1. Further enhancement is not needed. |
| LG Electronics | Agree with FL that no enhancement is needed. |
| Baicells | Agree with Interpretation 1 |
| MediaTek | Agree with FL that no enhancement is needed. Interpretation 1 is the correct one |
| Ericsson | Ok to close the issue. |
| Lockheed Martin | Agree with Interpretation 1 |

7.3 Summary of 1st round of discussions

The company views are shown in the table below.

| Option | For | Against |
| --- | --- | --- |
| Support Interpretation I | [13] sources: Nokia/NSB, Lenovo. Huawei/HiSilicon, NEC, Panasonic, Intel, Samsung, OPPO, CATT, CMCC, Baicells, MediaTek, Lockheed Martin | [1] source: InterDigital (but ok to compromise) |
| Koffset enhancement is needed for beam application timing for PUCCH | [2] sources: Apple, InterDigital | [17] sources: Nokia/NSB, Lenovo. Huawei/HiSilicon, ZTE, NEC, Panasonic, Intel, Samsung, OPPO, CATT, Xiaomi, CMCC, LGE, Baicells, MediaTek, Ericsson, Lockheed Martin |

There is a very clear majority supporting the understanding that no Koffset enhancement is needed for beam application timing for PUCCH. It is also evident that Interpretation I is widely agreed to be the correct one. InterDigital does request to at least capture this latter state as a conclusion.

7.4 2nd round of discussions

Based on the overwhelming feedback recommending not to consider any enhancement and also the agreement on Interpretation I, the moderator makes the following proposal.

Capture as a conclusion Interpretation I:

Regarding the delay between PDCCH reception and application of new PUCCH beam, “28 symbols” is the absolute time between the time UE receives PDCCH and the time UE applies new PUCCH beam

Company views are requested on capturing this conclusion

| Company | Comments |
| --- | --- |
| ZTE | Agree. |
| Lenovo | Support. |

8 [ACTIVE] Issue #8: Timing relationship of random access response

8.1 Background

Huawei [3] points out a problem that emerges from the agreement reached on NR NTN to introduce an additional delay for the starts of ra-ResponseWindow and msgB-ResponseWindow, which is equal to sum of UE’s TA and K_mac.

Agreement: The starts of ra-ResponseWindow and msgB-ResponseWindow are delayed by an estimate of UE-gNB RTT. - The estimate of UE-gNB RTT is equal to the sum of UE’s TA and K_mac. Note 1: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by NTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×TcTTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc. The estimate of gNB-satellite RTT is equal to the sum of NTA,common×Tc and K_mac. How to treat NTA and NTA,offset can be further discussed. Note 2: According to the RAN1#104bis-e agreement: When UE is not provided by network with a K_mac value, UE assumes K_mac = 0. Note 3: The accuracy of the estimated UE-gNB RTT with respect to the true UE-gNB RTT can be further discussed. Note 4: Other options of determining the estimate of UE-gNB RTT can be further discussed.

.

The concern is that this agreement unintentionally changes the behaviour for a terrestrial UE. In particular, for a terrestrial UE, NTA, NTA, UE-specific, NTA,common and kmac can be set to zero. However, the value of NTA,offset may be non-zero and the value of NTA,offset is provided by n-TimingAdvanceOffset for the serving cell. The proponent identifies three options, one of which changes the behaviour for terrestrial UEs, one of which would require a modification of the NTN agreement, and one which both retains the legacy behaviour for terrestrial UEs and does not revert the NTN agreement. This approach is adopted in the TPs proposed in [2], where the additional delay is only applicable to NR NTN UEs.

Clause 8.2 of TS38.213

============================ Unchanged Text Omitted ===================================

In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in clause 10.1. If TTA>NTA,offsetTc, the window starts after an additional TTA+kmac msec where TTA is defined in [4, TS 38.211] and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided.The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

============================ Unchanged Text Omitted ===================================

Clause 8.2A of TS38.213

============================ Unchanged Text Omitted ===================================

In response to a transmission of a PRACH and a PUSCH, or to a transmission of only a PRACH if the PRACH preamble is mapped to a valid PUSCH occasion, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding MsgB-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the PUSCH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. If TTA>NTA,offsetTc, the window starts after an additional TTA+kmac msec where TTA is defined in [4, TS 38.211] and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by msgB-ResponseWindow.

============================ Unchanged Text Omitted ===================================

8.2 Company views

Feedback from companies on this approach to resolve the problem is requested below.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | For different reasons and based on flexibility of future applications, we prefer to say: “If, NTA, UE-specific or , NTA,common is different of zero, the window starts…” |
| Apple | Nokia’s proposed text looks good to us. |
| Lenovo | We think N_TA,offset is a small value and the impact to legacy UE is rather small. We prefer no enhancment is introduced. |
| Huawei, HiSilicon | We believe the issue exists for terrestial UEs with the current specification. We don’t see fundamental difference between proposed TP from Nokia and Huawei. |
| ZTE | The additional delay of RAR window for TN introduced by N_TA,offset is very small and will not affect the performance. Hence, we prefer to keep the unified and simple description in current specification without change. |
| NEC | Fine with the intention. |
| Panasonic | We are fine with Nokia’s suggestion. |
| Samsung | OK with Nokia’s suggestion. |
| OPPO | In our view, the NTA,offset is too small to impact the TN users. So the current spec is unnecessary to be enhanced. |
| CATT | Fine with Nokia’s suggestion. |
| CMCC | We share the same view with Lenovo and ZTE. No enhancment is preferred. |
| QC | Fine with Nokia’s version |
| LG Electronics | We also share the view with ZTE. |
| Baicells | Fine with Nokia’s version |
| MediaTek | Fine with Nokia’s version |
| Ericsson | We are fine with Nokia's version except that the autonomous TA components should be called NTA,adjUE and NTA,adjcommon to align with 38.211. |
| Lockheed Martin | Either one of Hauwei’s or Nokia’s proposals are acceptable |

8.3 Summary of 1st round of discussions

There seems to be a general agreement that there is a problem to be solved. This will be raised at the 23rd February GTW.

8.4 2nd round of discussions

There was no agreement on the proposed approach of addressing sections 8.2 and 8.2A of TS38.213 at the 23rd February GTW. We continue this discussion directly over email.

9 [ACTIVE] Issue#9: RAN2 LS on NTN-SIB

9.1 Background

RAN1 received an LS from RAN2 on NTN-specific SIB [2], which was assigned to be discussed in AI 8.4.1. The input contributions related to this topic can be found in [20], [21], [22], [23].

[CMCC]

Proposal 1: Update of assistance information in SIB will not trigger system information modification procedure.

- It is up to RAN2 to determine detailed solutions for updating the assistance information. (e.g., Changes of the assistance information should neither result in system information change notifications nor in a modification of valueTag in SIB1, just like “timeInfoUTC” field acts in SIB9.)

[MediaTek Inc]

Observation 5: It may be more flexible to have parameters like cell-specific Koffset, k_mac, Indication for network enabled/disabled TA report, Cell reference location, t-Service, Validity duration for UL sync information in NTN-specific SIB in case it is beneficial for the network to update these within the system information notification period.

[Panasonic R&D Center]

Proposal 2: Add to SIB-NTN a counter with at least 5 bits for the SFN-cycles which have elapsed since the first instance of the SIB-NTN in each validity period.

[Nokia]

Observation 7: RAN1 and RAN2 have different understandings of the applicability of the validity timer/validity duration.

Proposal 12: Inform RAN2 that the validity duration is only intended to be applicable for serving satellite ephemeris and common TA related parameters.

Observation 8: Is seems that RAN1 and RAN2 have different understandings of UE actions prior to the validity timer expiry.

Observation 9: There may be periods with uncertainty related to UE’s UL synchronization status if the UE is allowed to read serving satellite ephemeris and Common TA related parameters after the expiry of the validity timer.

Proposal 13: Inform RAN2 that under normal operation, a UE is expected to have read new and updated serving satellite ephemeris information prior to the expiry of the validity timer.

[Apple]

Proposal 1: For UEs updating the timing relationship enhancement parameters, down select one of the following two alternatives:

Alt 1: Update of cell-specific Koffset or Kmac in NTN-specific SIB affects the value tag and triggers SI modification procedure.

Alt 2: Introduce additional validity duration for timing relationship enhancement parameters in NTN-specific SIB.

The moderator would like to start discussion on the RAN2 LS in order to identify whether at RAN1 we can identify any problems with the RAN2 agreements.

RAN2 informed RAN1 of the following regarding the NTN-specific SIB:

And at least the following serving cell information will be broadcast by SIBx:

1) Ephemeris;

2) Common TA parameters;

3) Validity duration for UL sync information;

4) t-Service (the timing information on when the serving cell is going to stop serving the area);

5) Cell reference location;

6) Epoch time;

7) K_mac;

8) Cell-specific Koffset;

9) Indication for network enabled/disabled TA report.

RAN2 also agreed that the validity duration for UL sync information applies to the whole SIBx and UE acquires the updated SIBx when the timer expires (FFS if this applies only to RRC_CONNECTED mode or to RRC_IDLE UEs as well).

First we need to establish whether RAN1 is in agreement that all the above serving cell information is sent in the NTN-specific SIB. From input contributions it appears that some companies support this approach whereas it is also pointed out that some parameters do not need to be updated as frequently as e.g. ephemeris and common TA parameters.

9.2 Company views

Is the list of serving cell information (9 items) provided by RAN2 acceptable to RAN1 or is there a need to make adjustments, and if so, what changes are considered necessary?

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | The list is acceptable, and we do not see any need for adjustments. |
| Apple | We are fine with the list of parameters for serving cell. Besides that, polarization signaling should also be contained in SIB. |
| Lenovo | We think the list is fine. |
| Huawei, HiSilicon | We are fine with the existing list of parameters from RAN2. On the update of K_offset and K_mac, as we repled to Issue #1, even through there is a possiblity to update K_offset and K_mac as other system information, we think the most robust operation mode is that to keep them unchanged. |
| ZTE | The polarization information should also be included in SIBx since RAN1 has made following agreements in RAN1#105e and RAN1#106e: Agreement: For explicit indication of polarization information for DL by the network, support indication in SIB - FFS: Signaling details for indication in SIB Agreement: - Polarization information for UL may be indicated in SIB by the network - UE assumes a same polarization for UL and DL, when the UL polarization information is absent. - FFS: Signaling details for indication in SIB Agreement: When polarization signalling is present in SIB - SIB indicates DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - FFS: whether polarization signalling is per SSB |
| Thales | ntnPolarizationDL and ntnPolarizationUL shall be added to the list |
| NEC | The list of serving cell information is acceptable for us. |
| Panasonic | In addition to the listed parameters, polarization information should be contained. |
| Samsung | OK for the list. |
| InterDigital | The list looks ok |
| OPPO | OK for the list. |
| CATT | OK |
| Xiaomi | We are fine with the list |
| CMCC | In line with the following agreements achieved in RAN1#107e: Agreement - When explicitly provided through SIB, Epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information. - Otherwise, when indicated in SIB (other than SIB1), epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is implicitly known as the end of the SI window during which the SI message is transmitted. - When provided through dedicated signaling, epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number. Explicitly indication of 6) Epoch time seems not necessary. Without it, epoch time of assistance information can be implicitly known as the end of the SI window during which the SI message is transmitted. Thus, at least broadcasting of 6) Epoch time should be optional. |
| NTT DOCOMO | We think polarization information is missing, on the other hand, it seems to be captured in the running CR to 38.331. Therefore the list looks fine. |
| LG Electronics | Agree with ZTE that polarization info is missing in the LS. On the other hand, R2-2201895 running CR for 38.331 captures this information in the NTN-Config-r17 IE. So, some clarification may helpful. |
| Baicells | Polarization info should be included. |
| Ericsson | Polarization information should be added. |
| Lockheed Martin | Polarization information should be added. |

One input contribution points out that there would appear to be different understanding between RAN1 and RAN2 on the applicability of the validity timer and validity duration. This follows from the fact that most of the parameters do not need to be updated as frequently as the validity timer for serving satellite ephemeris and common TA related parameters.

What are companies views on this generalised statement by RAN2 that the validity duration applies to the whole NTN-specific SIB?

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | We disagree with this view. This would cause unnecessary waste of power in NTN users. If the expiration is for the whole SIB, it means that after the validity timer expires (e.g. 10 s), all the SIB is considered not valid. It means all UEs would need to reacquire the SIB, including IDLE mode UEs (first paragraph of Section 5.2.2.2.1, on TS 38.331: “and whenever the UE does not have a valid version of a stored SIB“. The validity of ephemeris and common TA are only required at the UE side for the occasion of UL transmissions. So, it seems that this would cause unnecessary trigger for re-acaquistion of SIB for most UEs (idle and inactive). The validity timer should only apply for the fully dynamic elements of the NTN specific SIB. That is, for the Serving satellite ephemeris and the Common TA related parameters. |
| Apple | We do not agree with this statement. The timing relationship enhancement parameters (i.e., Koffset, Kmac) do not have the same validity duration as the uplink synchronization parameters (i.e., common TA, ephemeris). Hence, it is not suitable to assume the validity duration of uplink synchronization parameters is extended to the whole NTN-specific SIB. |
| Lenovo | We share similar view as Nokia that the validity timer is only applicable to satellite ephmeris and common TA related parameters. |
| Huawei, HiSilicon | We believe RAN2 had similar discussions and the reason why they think the validation duration is applicable for the whole SIBx is that the update of K_offset and K_mac are less frequent than satellite ephemeris and common TA parameters. It should also be noted that RAN1 also had some discussion on the need of define seperate validity durations for common TA and satellite ephemeris but in the end, it was agreed that a single value is adopted. |
| Thales | We do not agree with this statement. the validity duration does not apply to the whole NTN-specific SIB. In LS reply, RAN1 may highlight that at least the set of parameters hereafter is essential for initial access and uplink transmission: - Ephemeris; - Common TA parameters; - Validity duration for UL sync information - Epoch time - K_mac; - Cell-specific Koffset; Thus, these parameters should be always transmitted and thereby, for NTN-specific SIB si-BroadcastStatus shall be always set to broadcasting. Further valueTag field should be excluded when determining changes in system information, i.e. changes of Ephemeris and Common TA parameters should neither result in system information change notifications nor in a modification of valueTag in SIB1. |
| NEC | We do not agree with RAN2’s understanding. The validity duration shall be applied to the agreed parameters, i.e. ephemeris and common TA parameters. Applying the validate duration to the whole SIB will result in unnecessary SIB acquisition. |
| Samsung | Agree with Nokia |
| OPPO | From our point, the validity duration applies to the whole NTN-specific SIB is a simple way. We agree with it. |
| CATT | Different parameters have different valid duration, so it is not reasonable to have single valid duration. |
| Xiaomi | Our understanding is that UE in IDLE or inactive UE is only required to have a valid SIB for required SIB, as stated in current spec 38.331: “The UE in RRC_IDLE and RRC_INACTIVE shall ensure having a valid version of (at least) the MIB, SIB1 through SIB4, SIB5 (if the UE supports E-UTRA), SIB11 (if the UE is configured for idle/inactive measurements), SIB12 (if UE is capable of NR sidelink communication and is configured by upper layers to receive or transmit NR sidelink communication), and SIB13, SIB14 (if UE is capable of V2X sidelink communication and is configured by upper layers to receive or transmit V2X sidelink communication).” It is still not decided how to capture the NTN-specific SIB in RAN2. Thus we can have a response to RAN2 that we don’t expect the UE is required to re-acquire the SIB after the validity timer expires for IDLE/Inactive UEs. It is up to RAN2 to decide how to update the spec. |
| CMCC | From RAN1 perspective, the validity duration for UL sync information is determined by the outdating time of Ephemeris and Common TA parameters. Nevertheless, from RAN2 perspective, the validity duration for UL sync information applies to the whole SIBx, and UE acquires the updated SIBx when the timer expires. In order to avoid parameters outdating before UE acquires the updated SIBx, the validity duration should be determined by the shortest outdating time of all parameters broadcast by SIBx. |
| QC | Agree with Nokia. In addition, change of ephemeris and common TA does not trigger SIB modification while others will. |
| LG Electronics | Agree with Nokia. |
| Baicells | We have similar view as Nokia, NEC and many others. The validity duration shall be applied to the agreed parameters, i.e. ephemeris and common TA parameters. |
| OPPO2 | After double check, we do agree with QC's view that among these nine serving cell information in SIBx, except for ephemeris and common TA, these information should not be acquired based on validity timer duration but rather based on triggering of system information modification. This is because that the periodically reading these information may cause serious problem of ambiguity between different UEs. |
| MediaTek | The validity timer is only applicable to satellite ephemeris and common TA related parameters. For other parameters, these can be in the same NTN-specific SIB as per RAN2 agreement. Whether there is a need to indicate validity for the other parameters in NTN-specific SIB can be for further discussion in RAN2. We have same understanding as Qualcomm that change of ephemeris and common TA does not trigger SIB modification while others will. |
| Ericsson | It might be an OK solution, but further evaluation is needed. In RRC_IDLE state it might lead to unnecessary reading of NTN-SIB. Since the validity duration is defined relative to the Epoch time, it should be clarified if the validity of all parameters in the NTN-SIB are coupled to the Epoch time. This may have implications on the discussion about potential ambiguity during update of cell-specific Koffset. |
| Lockheed Martin | Disagree with RAN2 that validity timer applies to all fields of new SIB. As Nokia indicates, validity timer applies only to ephemeris and common TA parameters. |

9.3 Summary of 1st round of discussions

The list of serving cell information items in RAN2 LS is generally acceptable to companies that commented. Two companies propose adding ntnPolarizationDL and ntnPolarizationUL to the list.

There was wide disagreement with RAN2 understanding of the applicability of the validity duration for UL sync information. RAN1 companies’ understanding is that the validity duration only applies to ephemeris and common TA parameters. This is raised for further discussion at 23rd February GTW before deciding the next steps.

9.4 2nd round of discussions

We did not converge at the GTW beyond the bullets a) and b) with minor modifications. The discussion on the LS will continue on the email reflector, and a draft liaison and a separate discussion document will be available in a separate folder under 8.4.1 in the draft inbox:

Directory Listing /ftp/tsg_ran/WG1_RL1/TSGR1_108-e/Inbox/drafts/8.4.1/RAN2 LS reply for NTN-Specific SIB (3gpp.org)

10 Text proposals to specifications

10.1 TPs for TS38.213

10.1.1 [CLOSED] TP #1

Ericsson [11] points out that the RRC parameter for Kcell,offset is not called Koffset but CellSpecific_Koffset. They recommend adopting the approach used in TS38.214 where the Kcell,offset is referred to as CellSpecific_Koffset. There are five such occurrences in TS38.213 and the proposal is to replace each occasion as follows:

- Adopt the following TP for 38.213: “where Kcell,offset is provided by CellSpecific_Koffset”

10.1.1.1 Company views

The moderator believes this is a sensible correction recommends adopting the TP in the four occurrences within TS 38.213. Please provide your opinion below.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | We agree with moderator. |
| Apple | Fine with the change. |
| Lenovo | Agree. |
| Huawei, HiSilicon | Support the FL reccomendation |
| ZTE | Support |
| Thales | We agree with moderator |
| NEC | Agree. |
| Panasonic | Agree. |
| Samsung | OK |
| InterDigital | Agree |
| OPPO | OK |
| CATT | OK |
| Xiaomi | Support |
| CMCC | Agree |
| QC | Agree |
| LG Electronics | Agree |
| MediaTek | Agree |
| Ericsson | Agree |
| Lockheed Martin | Agree |

10.1.1.2 Summary of 1st round of discussion

There is good support for this text proposal. The moderator will send it out over email for final endorsement and the stable TP can also be found in section 11 of this FL summary.

10.1.2 [CLOSED] TP #2

OPPO has three text proposals addressing a clarification of the particular slot in question through stating that no timing advance is taken into account.

10.1	UE procedure for determining physical downlink control channel assignment

*** < Unchanged parts are omitted> ***

For a CORESET other than a CORESET with index 0, if a UE is provided a single TCI state for a CORESET, or if the UE receives a MAC CE activation command for one or two of the provided TCI states for a CORESET, the UE assumes that the DM-RS antenna port associated with PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by the TCI states. For a CORESET with index 0, the UE expects that a CSI-RS configured with qcl-Type set to 'typeD' in a TCI state indicated by a MAC CE activation command for the CORESET is provided by a SS/PBCH block

-	if the UE receives a MAC CE activation command for one of the TCI states, the UE applies the activation command in the first slot that is after slot k+3Nslotsubframe,μ+2μ∙kmac where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command without taking into account timing advance, μ is the SCS configuration for the PUCCH in the slot when the activation command is applied, and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided.

*** < Unchanged parts are omitted> ***

7	Uplink Power control

*** < Unchanged parts are omitted> ***

A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by SRS-PosResourceSet as described in clause 7.3.1. If the UE is provided a number of RS resources for pathloss estimation for PUSCH/PUCCH/SRS transmissions that is larger than 4, the UE maintains for pathloss estimation RS resources corresponding to RS resource indexes qd as described in clauses 7.1.1, 7.2.1, and 7.3.1. If an RS resource updated by MAC CE, as described in clauses 7.1.1, 7.2.1 and 7.3.1, is one from the RS resources the UE maintains for pathloss estimation for PUSCH/PUCCH/SRS transmissions, the UE applies the pathloss estimation based on the RS resources starting from the first slot that is after slot k+3∙Nslotsubframe,  μ+2μ∙kmac where k is the  slot where the UE would transmit a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE without taking into account timing advance, μ  is the SCS configuration for the PUCCH or PUSCH, respectively, that is determined in the slot when the MAC CE command is applied and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided.

*** < Unchanged parts are omitted> ***

6	Link recovery procedures

*** < Unchanged parts are omitted> ***

For the PCell or the PSCell, the UE can be provided, by PRACH-ResourceDedicatedBFR, a configuration for PRACH transmission as described in Clause 8.1. For PRACH transmission in slot  and according to antenna port quasi co-location parameters associated with periodic CSI-RS resource configuration or with SS/PBCH block associated with index  provided by higher layers [11, TS 38.321], the UE monitors PDCCH in a search space set provided by recoverySearchSpaceId for detection of a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI starting from slot n+4+2μ∙kmac without taking into account timing advance, where 𝜇 is the SCS configuration for the PRACH transmission and 𝑘mac is a number of slots provided by K-Mac [12, TS 38.331] or 𝑘mac =0 if K-Mac is not provided, within a window configured by BeamFailureRecoveryConfig. For PDCCH monitoring in a search space set provided by recoverySearchSpaceId and for corresponding PDSCH reception, the UE assumes the same antenna port quasi-collocation parameters as the ones associated with index  until the UE receives by higher layers an activation for a TCI state or any of the parameters tci-StatesPDCCH-ToAddList and/or tci-StatesPDCCH-ToReleaseList. After the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI in the search space set provided by recoverySearchSpaceId, the UE continues to monitor PDCCH candidates in the search space set provided by recoverySearchSpaceId until the UE receives a MAC CE activation command for a TCI state or tci-StatesPDCCH-ToAddList and/or tci-StatesPDCCH-ToReleaseList.

*** < Unchanged parts are omitted> ***

10.1.2.1 Company views

The moderator requests companies indicate whether this clarification is needed.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | The current specification text does not imply or indicate that timing advance is taken into account, so we do not see a big benefit from putting this clarification. Suggest to not introduce this clarification. |
| Apple | We do not see the necessity of this modification. |
| Lenovo | We don’t think this correction is necessary. |
| Huawei, HiSilicon | We don‘t see the need to adding the clarification. |
| ZTE | No need to revise. |
| Thales | We also propose to not introduce this clarification. |
| NEC | There is no need to clarify this in spec. |
| Panasonic | We don’t see the need of this change. |
| Samsung | We don’t think it is needed. |
| InterDigital | Not needed |
| OPPO | From our perspective, the slot in‘... where k is the slot where the UE would transmit a PUCCH with HARQ-ACK…’refers the uplink slot, and the slot in ‘…after slot k+3Nslotsubframe,μ+2μ∙kmac…’ refers the downlink slot. In order to avoid the misinterpretation we think it is necessary to add the clarification ‘without taking into account timing advance’. At the same time, correct our TP in Chapter 6 ‘Link recovery procedures ‘. There is no need to add the clarification. |
| CATT | NO need. |
| CMCC | We think no need to the clarification. |
| QC | No need |
| LG Electronics | Not needed. |
| MediaTek | No need to modify current spec. |
| Ericsson | We don't think the clarification is needed. |

10.1.2.2 Summary of 1st round of discussion

There is little support to adopt this text proposal and the moderator does not see a reason to try endorsing it.

10.1.3 [ACTIVE] TP #3

OPPO proposes a change in how the slot where the UE applies the MAC command has 2μ∙kmac removed from the offset.

9	UE procedure for reporting control information

*** < Unchanged parts are omitted> ***

For the remaining of this clause, if a UE is provided Kcell,offset by Koffset in ServingCellConfigCommon or KUE,offset by a MAC CE command, reference to a slot n+k for a PUCCH transmission or PUSCH transmission corresponds to a slot n+k+2μ∙Koffset for the PUSCH or the PUCCH transmission, where μ is the SCS configuration for the PUCCH transmission or PUSCH transmission. If Koffset or if the MAC CE command is not provided, Kcell,offset=0 or KUE,offset=0, respectively. If the PUCCH transmission or the PUSCH transmission is scheduled by a DCI format with CRC scrambled by TC-RNTI, KUE,offset=0. If the UE is provided a KUE,offset value by a MAC CE command, the UE applies the MAC command in the first slot that is after slot k+3⋅Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the MAC CE command, μ is the SCS configuration for the PUCCH transmission that is determined in the slot when the MAC CE command is applied, and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided.

*** < Unchanged parts are omitted> ***

10.1.3.1 Company views

The moderator requests companies indicate whether this clarification is needed.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | We do not see a need for this clarification. Current text is sufficient. |
| Apple | We are fine with the modification. |
| Lenovo | We don’t think this correction is necessary. |
| Huawei, HiSilicon | We don’t understand why the change is needed. |
| ZTE | Fine with the modification. Moreover, if the modification is adopted, the following description about kmac should also be removed, i.e.,: ... where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the MAC CE command, μ is the SCS configuration for the PUCCH transmission that is determined in the slot when the MAC CE command is applied, and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided. |
| Thales | This is not needed |
| NEC | There is no need to clarify this in spec. |
| Panasonic | We agree with OPPO’s view. 2μ∙kmac is not needed because K_offset is used for uplink transmission. (Note that RAN1 agreement was to apply k_mac for downlink related configuration. ) In the TP, description on k_mac in the last sentence should also be removed. |
| Samsung | We don’t think it is needed. |
| InterDigital | No need this clarfication |
| OPPO | Following the agreement in RAN#1 105-e as below, the case UE is provided a KUE,offset value by a MAC CE command does not involve downlink slot, so we think the UE applies the MAC command has 2μ∙kmac removed from the offset. And as mentioned by ZTE, the description about kmac should also be removed. RAN#1 105-e: Agreement: If a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac, where µ is the SCS configuration for the PUCCH. Note: Here K_mac is assumed to have the unit of the PUCCH slot. This can be revisited after the K_mac signaling design is finalized. |
| CATT | For UL, this clarification is needed. |
| Xiaomi | More clarification is needed. Our understanding is that the Koffset update is an UL configuration, in this sense, the Kmac may be removed based on the agreement we have. However, there might be some collision issue happens. |
| QC | Kmac does not apply to Koffset update MAC-CE. |
| LG Electronics | Not needed. |
| MediaTek | No need to modify current spec. |

10.1.3.2 Summary of 1st round of discussion

The company views are shown in the table below.

|  | Yes | No |
| --- | --- | --- |
| Clarification is needed | [7] sources: Apple, ZTE, Panasonic, OPPO, CATT, Xiaomi, Qualcomm | [9] sources: Nokia, Lenovo, Huawei/HiSilicon, Thales, NEC, Samsung, InterDigital, LGE, MediaTek |

The views are fairly evenly split. The moderator proposes to have another go and adopt the deletion by ZTE which would be applicable when 2μ∙kmac is not present in the spec section.

10.1.3.3 2nd round of discussions

9	UE procedure for reporting control information

*** < Unchanged parts are omitted> ***

For the remaining of this clause, if a UE is provided Kcell,offset by Koffset in ServingCellConfigCommon or KUE,offset by a MAC CE command, reference to a slot n+k for a PUCCH transmission or PUSCH transmission corresponds to a slot n+k+2μ∙Koffset for the PUSCH or the PUCCH transmission, where μ is the SCS configuration for the PUCCH transmission or PUSCH transmission. If Koffset or if the MAC CE command is not provided, Kcell,offset=0 or KUE,offset=0, respectively. If the PUCCH transmission or the PUSCH transmission is scheduled by a DCI format with CRC scrambled by TC-RNTI, KUE,offset=0. If the UE is provided a KUE,offset value by a MAC CE command, the UE applies the MAC command in the first slot that is after slot k+3Nslotsubframe,μ+2μ∙kmac where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the MAC CE command, μ is the SCS configuration for the PUCCH transmission that is determined in the slot when the MAC CE command is appliedkmacμ=0kmac=0.

Considering the comments made regarding the agreement made in RAN1#105-e (quoted in OPPO’s comment) and the comments that the UE would not apply Koffset to update MAC-CE, would the revised TP #3A be acceptable?

The moderator requests companies to indicate whether after reviewing the supporting companies comments this revised TP would be agreeable.

| Company | Comments |
| --- | --- |
| ZTE | Kmac is agreed to be introduced for DL MAC CE configuration. However, MAC CE command here is for UL configuration so that Kmac may not be needed. |
| Lenovo | Support. We understand that updated K-offset by MCE CE is for uplink transmission, so K-mac is not necessary. |

10.1.4 [CLOSED] TP #4

LGE provides an TP to the same section as above in TP #3, but it is related to the definition of Koffset. Section 4.2 of TS38.213 defines that K_offset is determined based on the cell-specific K_offset – UE-specific K_offset.

9	UE procedure for reporting control information

*** Unchanged text is omitted ***

For the remaining of this clause, if a UE is provided Kcell,offset by Koffset in ServingCellConfigCommon or KUE,offset by a MAC CE command, reference to a slot n+k for a PUCCH transmission or PUSCH transmission corresponds to a slot n+k+2μ∙Koffset for the PUSCH or the PUCCH transmission, where μ is the SCS configuration for the PUCCH transmission or PUSCH transmission and Koffset=Kcell,offset-KUE,offset. If Koffset or if the MAC CE command is not provided, Kcell,offset=0 or KUE,offset=0, respectively. If the PUCCH transmission or the PUSCH transmission is scheduled by a DCI format with CRC scrambled by TC-RNTI, KUE,offset=0. If the UE is provided a KUE,offset value by a MAC CE command, the UE applies the MAC command in the first slot that is after slot k+3Nslotsubframe,μ+2μ∙kmac where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the MAC CE command, μ is the SCS configuration for the PUCCH transmission that is determined in the slot when the MAC CE command is applied, and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided.

10.1.4.1 Company views

The moderator thinks that this clarification makes sense and comments from companies are requested whether this TP is agreeable.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | We agree with moderator and support the TP. |
| Apple | Fine with the modification. |
| Lenovo | Agree. |
| Huawei, HiSilicon | We are fine with the proposed TP. |
| ZTE | Fine with the revision. |
| Thales | We support the TP |
| NEC | Ok. |
| Panasonic | Agree. |
| Intel | OK |
| Samsung | OK |
| OPPO | OK |
| CATT | Actually we think current CR is unclear. For MAC CE, it is used to indicate one differential K-offset, not one UE specific K-offset. So we propose: Change the KUE,offset as the KUE-differential,offset. |
| Xiaomi | OK |
| CMCC | Fine with the TP |
| LG Electronics | Agree |
| MediaTek | Agree |
| Ericsson | OK |
| Locklheed Martin | Agree |

10.1.4.2 Summary of 1st round of discussion

There is good support for this text proposal apart from one comment recommending UE-differential offset. The moderator will send it out over email for final endorsement and the stable TP can also be found in section 11 of this FL summary.

10.2 TPs for TS38.214

10.2.1 [CLOSED] K_offset configuration in TS38.214

MediaTek points out that TS 38.213 specifies K_offset configuration and it is preferable to only refer to TS38.213 rather than repeat the K_offset configuration in TS38.214.

4.2	Transmission timing adjustments

**************************************************************************************

For a timing advance command received on uplink slot n and for a transmission other than a PUSCH scheduled by a RAR UL grant or a fallbackRAR UL grant as described in clause 8.2A or 8.3, or a PUCCH with HARQ-ACK information in response to a successRAR as described in clause 8.2A, the corresponding adjustment of the uplink transmission timing applies from the beginning of uplink slot n+k+1+2μ∙Koffset where k=Nslotsubframe,  μ∙NT,1+NT,2+NTA,max+0.5Tsf, NT,1 is a time duration in msec of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured, NT,2 is a time duration in msec of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214], NTA,max is the maximum timing advance value in msec that can be provided by a TA command field of 12 bits, Nslotsubframe,  μ is the number of slots per subframe, Tsf is the subframe duration of 1 msec, and Koffset=Kcell,offset-KUE,offset, where Kcell,offset is provided by Koffset in ServingCellConfigCommon and KUE,offset is provided by a MAC CE command; otherwise, if not respectively provided, Kcell,offset=0 or KUE,offset=0. N1 and N2 are determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and of all configured DL BWPs for the corresponding downlink carriers. For μ=0, the UE assumes N1,0=14 [6, TS 38.214]. Slot n and Nslotsubframe,  μ are determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG. NTA,max is determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and for all configured initial UL BWPs provided by initialUplinkBWP. The uplink slot n is the last slot among uplink slot(s) overlapping with the slot(s) of PDSCH reception assuming TTA=0, where the PDSCH provides the timing advance command and TTA is defined in [4, TS 38.211].

Ericsson has a text proposal in the same section 5.2.2.5 which only moves one section of text, but it is appended to a section proposed to be deleted by the MediaTek TP.

5.2.2.5	CSI reference resource definition (MediaTek)

The CSI reference resource for a serving cell is defined as follows:

-	In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates.

-	In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, if UE is configured with the higher layer parameter CellSpecific_Koffset, n-nCSI_ref, where Koffset is a parameter consigured by higher layer as specified in [TS 36.213 Section 4.2]and where μKoffsetis the subcarrier spacing configuration for Koffset,

Koffset

-	where  +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and and  are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and  are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211]

[Section 5.2.2.5, TS 38.214, v17.0.0] (Ericsson)

-	 In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, if UE is configured with the higher layer parameter CellSpecific_Koffset, n-nCSI_ref, μKoffsetKoffsetotherwise,

-	Koffset is provided with a value of ms for frequency range 1 and is equal to CellSpecific_Koffset - UESpecific_Koffset if UESpecific_Koffset is provided in MAC CE and CellSpecific_Koffset, otherwise. μKoffsetis the subcarrier spacing configuration for Koffset;

MediaTek proposes similar changes to sections 6.1.2.1 and 6.2.1.

6.1.2.1	Resource allocation in time domain (MediaTek)

/* No change to Omitted Text ---*/

in CSI-ReportConfig for the  triggered CSI Reporting Settings and  is the (m+1)th entry of .

-	The slot Ks where the UE shall transmit the PUSCH is determined by K2 as Ks =, if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, Ks=n⋅2μPUSCH2μPDCCH+K2+Koffset⋅2μPUSCH2μKoffset, where Koffset is a parameter consigured by higher layer as specified in [TS 36.213 Section 4.2], and where μKoffsetis the subcarrier spacing configuration for Koffset, n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH,  and  are the subcarrier spacing configurations for PUSCH and PDCCH, respectively,Koffset and the scheduling DCI is other than DCI format 0_0 with CRC scrambled by TC-RNTI.

/* No change to Omitted Text ---*/

6.2.1	UE sounding procedure (MediaTek)

/* No change to Omitted Text ---*/

If the UE receives the DCI triggering aperiodic SRS in slot n and except when SRS is configured with the higher layer parameter SRS-PosResource, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, Ks=n⋅2μSRS2μPDCCH+K2k+Koffset⋅2μSRS2μKoffset, where Koffset is a parameter consigured by higher layer as specified in [TS 36.213 Section 4.2], and where

-	k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

-	μKoffsetis the subcarrier spacing configuration for KoffsetKoffset.

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the  Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and when SRS is configured with the higher layer parameter SRS-PosResource, the UE transmits every aperiodic SRS resource in each of the triggered SRS resource set(s) in slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, Ks=n⋅2μSRS2μPDCCH+K2k+Koffset⋅2μSRS2μKoffset, where Koffset is a parameter consigured by higher layer as specified in [TS 36.213 Section 4.2], and where

-	k is configured via higher layer parameter slotOffset for each aperiodic SRS resource in each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

Koffset

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the  Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

/* No change to Omitted Text ---*/

[Section 6.2.1, TS 38.214, v17.0.0] (Ericsson)

-	If the UE receives the DCI triggering aperiodic SRS in slot n and at least one resource set is configured with parameter availableSlotOffset across all configured BWPs in a component carrier except when SRS is configured with the higher layer parameter SRS-PosResource, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in the (t + 1)-th available slot counting from slot if ca-SlotOffset is configured, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in the (t + 1)-th available slot counting from slot n⋅2μSRS2μPDCCH+k+Koffset⋅2μSRS2μKoffset, if UE is configured with the higher layer parameter CellSpecific_Koffset,  otherwise the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in the (t + 1)-th available slot counting from slot n⋅2μSRS2μPDCCH+k, where

-	k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command, respectively;

-	μKoffsetis the subcarrier spacing configuration for Koffset, and Koffset is provided with a value of ms for frequency range 1 and is equal to CellSpecific_Koffset - UESpecific_Koffset if UESpecific_Koffset is provided in MAC CE and CellSpecific_Koffset otherwise.

The moderator merges the text proposals from MediaTek and Ericsson to the three TPs below.

10.2.1.1 TP #5

5.2.2.5	CSI reference resource definition

The CSI reference resource for a serving cell is defined as follows:

-	In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates.

-	In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, if UE is configured with the higher layer parameter CellSpecific_Koffset, n-nCSI_ref, where Koffset is a parameter configured by higher layer as specified in [TS 36.213 Section 4.2], and where μKoffsetis the subcarrier spacing configuration for Koffset,

Koffset

-	where  +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and and  are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and  are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211]

10.2.1.2 TP #6

6.1.2.1	Resource allocation in time domain

<<< unchanged paragraphs omitted >>>

in CSI-ReportConfig for the  triggered CSI Reporting Settings and  is the (m+1)th entry of .

-	The slot Ks where the UE shall transmit the PUSCH is determined by K2 as Ks =, if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, Ks=n⋅2μPUSCH2μPDCCH+K2+Koffset⋅2μPUSCH2μKoffset, where Koffset is a parameter consigured by higher layer as specified in [TS 36.213 Section 4.2], and where μKoffsetis the subcarrier spacing configuration for Koffset, n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH,  and  are the subcarrier spacing configurations for PUSCH and PDCCH, respectively, Koffsetand the scheduling DCI is other than DCI format 0_0 with CRC scrambled by TC-RNTI.

10.2.1.3 TP #7

6.2.1	UE sounding procedure

-	If the UE receives the DCI triggering aperiodic SRS in slot n and none of the resource sets is configured with parameter availableSlotOffset across all configured BWPs in a component carrier, and if the UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, except when SRS is configured with the higher layer parameter SRS-PosResource, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in slot , otherwise, the UE transmits aperiodic SRS in each of the triggered resource set(s) in slot Ks=n⋅2μSRS2μPDCCH+K2+Koffset⋅2μSRS2μKoffset, where Koffset is a parameter consigured by higher layer as specified in [TS 36.213 Section 4.2], and where

-	k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

-	μKoffsetis the subcarrier spacing configuration for KoffsetKoffset.

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the  Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and when SRS is configured with the higher layer parameter SRS-PosResource, the UE transmits every aperiodic SRS resource in each of the triggered SRS resource set(s) in slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, Ks=n⋅2μSRS2μPDCCH+K2+Koffset⋅2μSRS2μKoffset, where Koffset is a parameter consigured by higher layer as specified in [TS 36.213 Section 4.2], and where

-	k is configured via higher layer parameter slotOffset for each aperiodic SRS resource in each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

- μKoffsetis the subcarrier spacing configuration for Koffset.

Koffset

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the  Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

<<< unchanged paragraphs omitted >>>

10.2.1.4 Company views

Companies are requested to indicate whether the TPs #5, #6 and #7 are agreeable.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | We agree with these TPs (#5, #6 and #7). |
| Apple | We are fine with the TPs. |
| Huawei, HiSilicon | Except the typo consigured in TP#7, we support the FL reccomendated TP. |
| ZTE | The referred TS should be 38.213 instead of 36.213. We are fine with these TPs except that we prefer Ericsson’s proposal, i.e., keep the explanation of Koffset instead of referring to TS 38.213. |
| Thales | We support the TPs |
| NEC | We are OK with the TPs. |
| Panasonic | We agree with the TPs. |
| Intel | OK |
| Samsung | OK |
| OPPO | OK |
| Xiaomi | Fine |
| QC | OK |
| LG Electronics | OK |
| MediaTek | We are overall fine with TPs (#5, #6 and #7). On TP#5, it seems there is a typo. The term “n-nCSI_ref,” should be deleted as shown below Modified TP #5: 5.2.2.5 CSI reference resource definition The CSI reference resource for a serving cell is defined as follows: - In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates. - In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, if UE is configured with the higher layer parameter CellSpecific_Koffset, n-nCSI_ref, where Koffset is a parameter configured by higher layer as specified in [TS 36.213 Section 4.2], and where μKoffsetis the subcarrier spacing configuration for Koffset, - where +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and and are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211] the subcarrier spacing configuration for Koffset, |
| Ericsson | OK in principle. We propose further corrections below, highlighted in yellow: TP#5: In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, where Koffsetis a parameter configured by higher layer as specified in [TS 38.213 clause 4.2], and where μKoffsetis the subcarrier spacing configuration for Koffset, TP#6: The slot Ks where the UE shall transmit the PUSCH is determined by K2 as Ks =, if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, Ks=n⋅2μPUSCH2μPDCCH+K2+Koffset⋅2μPUSCH2μKoffset, otherwise, where Koffsetis a parameter configured by higher layer as specified in [TS 38.213 clause 4.2], and where μKoffsetis the subcarrier spacing configuration for Koffset, n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH, and are the subcarrier spacing configurations for PUSCH and PDCCH, respectively, Koffsetand the scheduling DCI is other than DCI format 0_0 with CRC scrambled by TC-RNTI. TP#7: If the UE receives the DCI triggering aperiodic SRS in slot n and none of the resource sets is configured with parameter availableSlotOffset across all configured BWPs in a component carrier, and if the UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, except when SRS is configured with the higher layer parameter SRS-PosResource, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in slot , otherwise, the UE transmits aperiodic SRS in each of the triggered resource set(s) in slot Ks=n⋅2μSRS2μPDCCH+K2+Koffset⋅2μSRS2μKoffset, where Koffsetis a parameter configured by higher layer as specified in [TS 38.213 clause 4.2], and where |

10.2.1.5 Summary of 1st round of discussion

One company objects to simplification of the text through referring to TS 38.213. The moderator observes that a spec reader always has to look at a number of specifications in combination, and reading a definition from TS 38.213 whilst studying TS38.214 should not be cumbersome.

The typo correction from MediaTek is adopted in the stable TP #5A sent for email endorsement. The typos pointed out by Huawei and ZTE are also adopted in the revised proposals. Ericsson in their comment proposed some editorials which were already partly captured by earlier comments, and the other Ericsson text was also adopted in revised TPs #5A, #6A and #7A.

Since the comments mainly focused on the editorials mentioned above, they appear stable enough for further discussion to take place over the email reflector. The moderator will send them out over email for potential endorsement and the TPs can also be found in section 11 of this FL summary.

10.2.2 [CLOSED] TP #8

Both Ericsson and MediaTek propose a correction in section 6.2.1 by replacing parameter K2 with k. Note that this change was present in some of the text proposals in section 10.2.3 but the intent is to address it separately here as an independent correction.

- Adopt the following TP for Section 6.2.1, 38.214: “Ks=n⋅2μSRS2μPDCCH+K2 k+Koffset⋅2μSRS2μKoffset, if UE is configured with the higher layer parameter CellSpecific_Koffset, Ks =, otherwise,”

10.2.2.1 Company views

This looks like a sensible fix and the moderator requests companies to indicate whether they agree with this correction.

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | We agree with moderator. |
| Lenovo | Agree |
| Huawei, HiSilicon | We are fine with the proposed TP. |
| ZTE | Support |
| Thales | We agree with this correction |
| NEC | Support. |
| Panasonic | Agree |
| Intel | OK |
| Samsung | OK |
| OPPO | OK |
| CATT | OK |
| CMCC | Agree |
| LG Electronics | OK |
| MediaTek | Agree |
| Ericsson | OK |
| Lockheed Martin | Agree |

10.2.2.2 Summary of 1st round of discussion

There is good support for this text proposal. The moderator will send it out over email for final endorsement and the TP can also be found in section 11 of this FL summary.

10.2.3 [ACTIVE] K_mac timing offset

Three companies [3, 5, 14] have identified that an agreement from RAN1#105-e is not reflected in the first Rel-17 specifications.

Agreement: If a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac, where µ is the SCS configuration for the PUCCH. Note: Here K_mac is assumed to have the unit of the PUCCH slot. This can be revisited after the K_mac signaling design is finalized.

10.2.3.1 Huawei, HiSilicon TPs

- TP#3 for Clause 5.1.4.2 of TS38.214 (Huawei, HiSilicon) ============================ Unchanged Text Omitted =================================== For a UE configured with a list of semi-persistent ZP-CSI-RS-ResourceSet(s) provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList: - when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, as described in clause 6.1.3.19 of [10, TS 38.321], for ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on the PDSCH RE mapping corresponding to the activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH. - when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, as described in clause 6.1.3.19 of [10, TS 38.321], for activated ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on cessation of the PDSCH RE mapping corresponding to the de-activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH. ============================ Unchanged Text Omitted ===================================

- TP#4 for Clause 5.1.5 of TS38.214 (Huawei, HiSilicon) ============================ Unchanged Text Omitted =================================== When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH. If tci-PresentInDCI is set to 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET scheduling the PDSCH, and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA', and when applicable, also with respect to qcl-Type set to 'typeD'. ============================ Unchanged Text Omitted ===================================

- TP#5 for Clause 5.2.1.5 of TS38.214 (Huawei, HiSilicon) ============================ Unchanged Text Omitted =================================== A trigger state is initiated using the CSI request field in DCI. - When all the bits of CSI request field in DCI are set to zero, no CSI is requested. - When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to trigger states to the codepoints of the CSI request field in DCI. is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH. ============================ Unchanged Text Omitted ===================================

10.2.3.2 OPPO TPs

5.1.4.2 	PDSCH resource mapping with RE level granularity (OPPO)

*** < Unchanged parts are omitted> ***

For a UE configured with a list of semi-persistent ZP-CSI-RS-ResourceSet(s) provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList:

-	when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the activation command, as described in clause 6.1.3.19 of [10, TS 38.321], for ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on the PDSCH RE mapping corresponding to the activated ZP CSI-RS resource(s) shall be applied starting from the first   slot that is after slot n+3Nslotsubframe,µ+ K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided.

-	when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the activation command, as described in clause 6.1.3.19 of [10, TS 38.321], for activated ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on cessation of the PDSCH RE mapping corresponding to the de-activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+ K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided.

*** < Unchanged parts are omitted> ***

5.1.5 	Antenna ports quasi co-location (OPPO)

*** < Unchanged parts are omitted> ***

When the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ + K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided. If tci-PresentInDCI is set to 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET scheduling the PDSCH, and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA', and when applicable, also with respect to qcl-Type set to 'typeD'.

*** < Unchanged parts are omitted> ***

5.2.1.5.1 	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology (OPPO)

*** < Unchanged parts are omitted> ***

A trigger state is initiated using the CSI request field in DCI.

-	When all the bits of CSI request field in DCI are set to zero, no CSI is requested.

-	When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where  is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to  trigger states to the codepoints of the CSI request field in DCI.  is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ + K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided.

*** < Unchanged parts are omitted> ***

5.2.1.5.2	Semi-persistent CSI/Semi-persistent CSI-RS (OPPO)

*** < Unchanged parts are omitted> ***

For semi-persistent reporting on PUCCH, the PUCCH resource used for transmitting the CSI report are configured by reportConfigType. Semi-persistent reporting on PUCCH is activated by an activation command as described in clause 6.1.3.16 of [10, TS 38.321], which selects one of the semi-persistent Reporting Settings for use by the UE on the PUCCH. When the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the activation command, the indicated semi-persistent Reporting Setting should be applied starting from the first  slot that is after slot n+3Nslotsubframe,µ + K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided.

*** < Unchanged parts are omitted> ***

5.2.1.5.2	Semi-persistent CSI/Semi-persistent CSI-RS (OPPO)

*** < Unchanged parts are omitted> ***

For a UE configured with CSI resource setting(s) where the higher layer parameter resourceType set to 'semiPersistent'.

-	when a UE receives an activation command, as described in clause 6.1.3.12 of [10, TS 38.321], for CSI-RS resource set(s) for channel measurement and CSI-IM/NZP CSI-RS resource set(s) for interference measurement associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the selection command, the corresponding actions in [10, TS 38.321] and the UE assumptions (including QCL assumptions provided by a list of reference to TCI-State's, one per activated resource) on CSI-RS/CSI-IM transmission corresponding to the configured CSI-RS/CSI-IM resource configuration(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+ K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided. If a TCI-State referred to in the list is configured with a reference to an RS configured with qcl-Type set to 'typeD', that RS can be an SS/PBCH block, periodic or semi-persistent CSI-RS located in same or different CC/DL BWP.

-	when a UE receives a deactivation command, as described in clause 6.1.3.12 of [10, TS 38.321], for activated CSI-RS/CSI-IM resource set(s) associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the deactivation command, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of CSI-RS/CSI-IM transmission corresponding to the deactivated CSI-RS/CSI-IM resource set(s) shall apply starting from the first slot that is after slot n+3Nslotsubframe,µ + K_mac where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided.

*** < Unchanged parts are omitted> ***

5.2.4	CSI reporting using PUCCH (OPPO)

*** < Unchanged parts are omitted> ***

A UE shall perform semi-persistent CSI reporting on the PUCCH applied starting from the first slot that is after slot n+3Nslotsubframe,µ+ K_mac when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the activation command described in clause 6.1.3.16 of [10, TS 38.321] where  is the SCS configuration for the PUCCH and K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided. The activation command will contain one or more Reporting Settings where the associated CSI Resource Settings are configured. Semi-persistent CSI reporting on the PUCCH supports Type I CSI. Semi-persistent CSI reporting on the PUCCH format 2 supports Type I CSI with wideband frequency granularity. Semi-persistent CSI reporting on PUCCH formats 3 or 4 supports Type I CSI with wideband and sub-band frequency granularities and Type II CSI Part 1.

*** < Unchanged parts are omitted> ***

10.2.3.3 Lenovo, Motorola Mobility TPs

5.1.4.2	PDSCH resource mapping with RE level granularity (Lenovo, Motorola Mobility)

<Unrelated parts omitted>

For a UE configured with a list of semi-persistent ZP-CSI-RS-ResourceSet(s) provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList:

-	when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, as described in clause 6.1.3.19 of [10, TS 38.321], for ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on the PDSCH RE mapping corresponding to the activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ∙kmac, where  is the SCS configuration for the PUCCH, .and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided

-	when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, as described in clause 6.1.3.19 of [10, TS 38.321], for activated ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on cessation of the PDSCH RE mapping corresponding to the de-activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ∙kmac, where  is the SCS configuration for the PUCCH, .and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided.

<Unrelated parts omitted>

5.1.5	Antenna ports quasi co-location (Lenovo, Motorola Mobility)

<Unrelated parts omitted>

When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ∙kmac, where  is the SCS configuration for the PUCCH, .and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided.. If tci-PresentInDCI is set to 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET scheduling the PDSCH, and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA', and when applicable, also with respect to qcl-Type set to 'typeD'.

<Unrelated parts omitted>

5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology (Lenovo, Motorola Mobility)

<Unrelated parts omitted>

A trigger state is initiated using the CSI request field in DCI.

-	When all the bits of CSI request field in DCI are set to zero, no CSI is requested.

-	When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where  is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to  trigger states to the codepoints of the CSI request field in DCI.  is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ +2μ∙kmac, where  is the SCS configuration for the PUCCH, .and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided .

<Unrelated parts omitted>

5.2.1.5.2	Semi-persistent CSI/Semi-persistent CSI-RS (Lenovo, Motorola Mobility)

For semi-persistent reporting on PUSCH, a set of trigger states are higher layer configured by CSI-SemiPersistentOnPUSCH-TriggerStateList, where the CSI request field in DCI scrambled with SP-CSI-RNTI activates one of the trigger states. A UE is not expected to receive a DCI scrambled with SP-CSI-RNTI activating one semi-persistent CSI report with the same CSI-ReportConfigId as in a semi-persistent CSI report which is activated by a previously received DCI scrambled with SP-CSI-RNTI.

For semi-persistent reporting on PUCCH, the PUCCH resource used for transmitting the CSI report are configured by reportConfigType. Semi-persistent reporting on PUCCH is activated by an activation command as described in clause 6.1.3.16 of [10, TS 38.321], which selects one of the semi-persistent Reporting Settings for use by the UE on the PUCCH. When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated semi-persistent Reporting Setting should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

For a UE configured with CSI resource setting(s) where the higher layer parameter resourceType set to 'semiPersistent'.

-	when a UE receives an activation command, as described in clause 6.1.3.12 of [10, TS 38.321], for CSI-RS resource set(s) for channel measurement and CSI-IM/NZP CSI-RS resource set(s) for interference measurement associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the selection command, the corresponding actions in [10, TS 38.321] and the UE assumptions (including QCL assumptions provided by a list of reference to TCI-State's, one per activated resource) on CSI-RS/CSI-IM transmission corresponding to the configured CSI-RS/CSI-IM resource configuration(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ +2μ∙kmac, where  is the SCS configuration for the PUCCH, .and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided. If a TCI-State referred to in the list is configured with a reference to an RS configured with qcl-Type set to 'typeD', that RS can be an SS/PBCH block, periodic or semi-persistent CSI-RS located in same or different CC/DL BWP.

-	when a UE receives a deactivation command, as described in clause 6.1.3.12 of [10, TS 38.321], for activated CSI-RS/CSI-IM resource set(s) associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of CSI-RS/CSI-IM transmission corresponding to the deactivated CSI-RS/CSI-IM resource set(s) shall apply starting from the first slot that is after slot n+3Nslotsubframe,µ +2μ∙kmac, where  is the SCS configuration for the PUCCH, .and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided .

10.2.3.4 Discussion

OPPO proposes adding a clarification that “K_mac is a number of slots provided by K-Mac[12, TS 38.331] or K_mac=0 if K_mac is not provided”. Lenovo, Motorola Mobility make similar kind of addition. We can find this statement in section 8.2 of TS 36.213 so it is probably not necessary to repeat it in each respective section of TS 36.213.

8.2	Random access response - Type-1 random access procedure

In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in clause 10.1. The window starts after an additional TTA+kmac msec where TTA is defined in [4, TS 38.211] and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided.The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

The moderator would also draw attention to the fact that the agreement quoted above had this note:

Note: Here K_mac is assumed to have the unit of the PUCCH slot. This can be revisited after the K_mac signaling design is finalized.

Thus, the moderator is of the opinion that it is not necessary to clarify that Kmac is a number of slots in the equations.

This would leave the clarifications of “uplink” we can find in the OPPO text proposals to address.

10.2.3.5 TP #9

5.1.4.2 PDSCH resource mapping with RE level granularity <<< unchanged paragraphs omitted >>> For a UE configured with a list of semi-persistent ZP-CSI-RS-ResourceSet(s) provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList: - when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, as described in clause 6.1.3.19 of [10, TS 38.321], for ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on the PDSCH RE mapping corresponding to the activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH. - when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, as described in clause 6.1.3.19 of [10, TS 38.321], for activated ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on cessation of the PDSCH RE mapping corresponding to the de-activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH. <<< unchanged paragraphs omitted >>>

10.2.3.6 TP #10

5.1.5	Antenna ports quasi co-location

<<< unchanged paragraphs omitted >>>

When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH. If tci-PresentInDCI is set to 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET scheduling the PDSCH, and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA', and when applicable, also with respect to qcl-Type set to 'typeD'.

<<< unchanged paragraphs omitted >>>

10.2.3.7 TP #11

5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology

<<< unchanged paragraphs omitted >>>

A trigger state is initiated using the CSI request field in DCI.

-	When all the bits of CSI request field in DCI are set to zero, no CSI is requested.

-	When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where  is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to  trigger states to the codepoints of the CSI request field in DCI.  is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH.

<<< unchanged paragraphs omitted >>>

10.2.3.8 TP #12

5.2.1.5.2	Semi-persistent CSI/Semi-persistent CSI-RS

For semi-persistent reporting on PUSCH, a set of trigger states are higher layer configured by CSI-SemiPersistentOnPUSCH-TriggerStateList, where the CSI request field in DCI scrambled with SP-CSI-RNTI activates one of the trigger states. A UE is not expected to receive a DCI scrambled with SP-CSI-RNTI activating one semi-persistent CSI report with the same CSI-ReportConfigId as in a semi-persistent CSI report which is activated by a previously received DCI scrambled with SP-CSI-RNTI.

For semi-persistent reporting on PUCCH, the PUCCH resource used for transmitting the CSI report are configured by reportConfigType. Semi-persistent reporting on PUCCH is activated by an activation command as described in clause 6.1.3.16 of [10, TS 38.321], which selects one of the semi-persistent Reporting Settings for use by the UE on the PUCCH. When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated semi-persistent Reporting Setting should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH.

For a UE configured with CSI resource setting(s) where the higher layer parameter resourceType set to 'semiPersistent'.

-	when a UE receives an activation command, as described in clause 6.1.3.12 of [10, TS 38.321], for CSI-RS resource set(s) for channel measurement and CSI-IM/NZP CSI-RS resource set(s) for interference measurement associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the selection command, the corresponding actions in [10, TS 38.321] and the UE assumptions (including QCL assumptions provided by a list of reference to TCI-State's, one per activated resource) on CSI-RS/CSI-IM transmission corresponding to the configured CSI-RS/CSI-IM resource configuration(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH. If a TCI-State referred to in the list is configured with a reference to an RS configured with qcl-Type set to 'typeD', that RS can be an SS/PBCH block, periodic or semi-persistent CSI-RS located in same or different CC/DL BWP.

-	when a UE receives a deactivation command, as described in clause 6.1.3.12 of [10, TS 38.321], for activated CSI-RS/CSI-IM resource set(s) associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of CSI-RS/CSI-IM transmission corresponding to the deactivated CSI-RS/CSI-IM resource set(s) shall apply starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac where  is the SCS configuration for the PUCCH.

<<< unchanged paragraphs omitted >>>

10.2.3.9 TP #13

5.2.4	CSI reporting using PUCCH

A UE is semi-statically configured by higher layers to perform periodic CSI Reporting on the PUCCH. A UE can be configured by higher layers for multiple periodic CSI Reports corresponding to multiple higher layer configured CSI Reporting Settings, where the associated CSI Resource Settings are higher layer configured. Periodic CSI reporting on PUCCH formats 2, 3, 4 supports Type I CSI with wideband granularity.

A UE shall perform semi-persistent CSI reporting on the PUCCH applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command described in clause 6.1.3.16 of [10, TS 38.321] where  is the SCS configuration for the PUCCH. The activation command will contain one or more Reporting Settings where the associated CSI Resource Settings are configured. Semi-persistent CSI reporting on the PUCCH supports Type I CSI. Semi-persistent CSI reporting on the PUCCH format 2 supports Type I CSI with wideband frequency granularity. Semi-persistent CSI reporting on PUCCH formats 3 or 4 supports Type I CSI with wideband and sub-band frequency granularities and Type II CSI Part 1.

10.2.3.10 Company views

As a first proposal, the moderator provides five TPs (#9, #10, #11, #12 and #13) above where only the Kmac summand is retained according to the discussion above. Companies are invited to indicate whether

- they support the simplified text of only reflecting the addition of Kmac, and

- whether there is a need to reflect the clarification of uplink slot from OPPO

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | - For consistence, as we have done for K_offset, we think we should add the scaling factor 2μPUSCH2μKmac to the K_mac factor. The logic applied to K_offset seems the same to be applied over here, with K_mac unit also being slots over a reference unit. We believe the “uplink“ clarification may be useful to avoid misinterpretations. |
| Lenovo | - We prefer 2^u to be kept as K_mac anyway needs to be in unit of slot. - We don’t think uplink is necessary. For TP#12, we think the first K-mac is not necessary, as it is a uplink MAC CE. For TP#13, we also think K_mac is not necessary. The reson is that K_mac is only necessary for DL MAC CE rather than UL MAC CE. |
| Huawei, HiSilicon | Support the FL reccomendation on the first bullet and we don’t think there is a need for clarification on uplink |
| ZTE | We support to add kmac in the specification. But we do not think simplification on the text is needed. The 38.214 editor has added the definition of K_offset for each places where K_offset is used. Hence, it is OK to also add the definition of kmac for each places. Moreover, the TPs are for 38.214 instead of 38.213 and the definition of K_mac in 38.213 cannot be directly used in 38.214. |
| Thales | We support the simplified text of only reflecting the addition of Kmac. Reflecting the clarification of uplink slot as proposed by OPPO could be useful. |
| Panasonic | 1) we prefer to add 2^u to be consistent with Koffset. 2) “uplink” would not be necessary because what is "uplink slot” may cause another confusion. |
| OPPO | 1) This related text on how to reflecting the addition of Kmac can be left to the spec-editor. 2) In our view, it is necessary to introduce the ‘uplink‘ clarification to avoid the misinterpretations. |
| QC | we prefer to add 2^u to be consistent with Koffset. |
| MediaTek | - 2μ∙kmac is better. It is unnecessary to add extra clarification of uplink slot. |
| Ericsson | - We prefer to add 2^µ - No need to clarify that the slot is uplink |

10.2.3.11 Summary of 1st round discussion

A number of companies prefer to have a similar scaling factor to Kmac as is used to Koffset:  2μPUSCH2μKmac. Doing this may gain the support of all companies to the TPs and this is reflected in the new, revised text proposals.

There is also a comment that TPs #12 and #13 do not need the addition of Kmac since they are only related to uplink timing. The moderator would agree with this observation.

One company objects to simplification of the text through referring to TS 38.213. The moderator observes that a spec reader always has to look at a number of specifications in combination, and reading a definition from TS 38.213 whilst studying TS38.214 should not be cumbersome.

Revised text proposals can be found below (#9A, #10A, #11A, #12A) for further discussion.

10.2.3.12 TP #9A

5.1.4.2 PDSCH resource mapping with RE level granularity <<< unchanged paragraphs omitted >>> For a UE configured with a list of semi-persistent ZP-CSI-RS-ResourceSet(s) provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList: - when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, as described in clause 6.1.3.19 of [10, TS 38.321], for ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on the PDSCH RE mapping corresponding to the activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ∙kmac, where  is the SCS configuration for the PUCCH. - when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, as described in clause 6.1.3.19 of [10, TS 38.321], for activated ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on cessation of the PDSCH RE mapping corresponding to the de-activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ∙kmac where  is the SCS configuration for the PUCCH. <<< unchanged paragraphs omitted >>>

10.2.3.13 TP #10A

5.1.5	Antenna ports quasi co-location

<<< unchanged paragraphs omitted >>>

When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ∙kmac where  is the SCS configuration for the PUCCH. If tci-PresentInDCI is set to 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET scheduling the PDSCH, and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA', and when applicable, also with respect to qcl-Type set to 'typeD'.

<<< unchanged paragraphs omitted >>>

10.2.3.14 TP #11A

5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology

<<< unchanged paragraphs omitted >>>

A trigger state is initiated using the CSI request field in DCI.

-	When all the bits of CSI request field in DCI are set to zero, no CSI is requested.

-	When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where  is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to  trigger states to the codepoints of the CSI request field in DCI.  is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ∙kmac where  is the SCS configuration for the PUCCH.

<<< unchanged paragraphs omitted >>>

10.2.3.15 TP #12A

5.2.1.5.2	Semi-persistent CSI/Semi-persistent CSI-RS

For semi-persistent reporting on PUSCH, a set of trigger states are higher layer configured by CSI-SemiPersistentOnPUSCH-TriggerStateList, where the CSI request field in DCI scrambled with SP-CSI-RNTI activates one of the trigger states. A UE is not expected to receive a DCI scrambled with SP-CSI-RNTI activating one semi-persistent CSI report with the same CSI-ReportConfigId as in a semi-persistent CSI report which is activated by a previously received DCI scrambled with SP-CSI-RNTI.

For semi-persistent reporting on PUCCH, the PUCCH resource used for transmitting the CSI report are configured by reportConfigType. Semi-persistent reporting on PUCCH is activated by an activation command as described in clause 6.1.3.16 of [10, TS 38.321], which selects one of the semi-persistent Reporting Settings for use by the UE on the PUCCH. When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated semi-persistent Reporting Setting should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

For a UE configured with CSI resource setting(s) where the higher layer parameter resourceType set to 'semiPersistent'.

-	when a UE receives an activation command, as described in clause 6.1.3.12 of [10, TS 38.321], for CSI-RS resource set(s) for channel measurement and CSI-IM/NZP CSI-RS resource set(s) for interference measurement associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the selection command, the corresponding actions in [10, TS 38.321] and the UE assumptions (including QCL assumptions provided by a list of reference to TCI-State's, one per activated resource) on CSI-RS/CSI-IM transmission corresponding to the configured CSI-RS/CSI-IM resource configuration(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ∙kmac where  is the SCS configuration for the PUCCH. If a TCI-State referred to in the list is configured with a reference to an RS configured with qcl-Type set to 'typeD', that RS can be an SS/PBCH block, periodic or semi-persistent CSI-RS located in same or different CC/DL BWP.

-	when a UE receives a deactivation command, as described in clause 6.1.3.12 of [10, TS 38.321], for activated CSI-RS/CSI-IM resource set(s) associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of CSI-RS/CSI-IM transmission corresponding to the deactivated CSI-RS/CSI-IM resource set(s) shall apply starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ∙kmac where  is the SCS configuration for the PUCCH.

<<< unchanged paragraphs omitted >>>

10.2.3.16 Second round of discussion: company views

The moderator adopted the recommendation to add 2^u to be consistent with Koffset. Also the two comments from Lenovo were adopted, i.e. remove the first Kmac instance in #12A and not take #13 forward at all.

The moderator invites further company comments on these revised TPs (#9A, #10A, #11A, #12A).

| Company | Comments |
| --- | --- |
| ZTE | OK with the TPs. |
| Lenovo | Support. |

10.2.4 TP #14

OPPO has two further text proposals for TS 38.214 where it is again proposed to add clarification that uplink slots are being addressed in the specification text.

6.2.1	UE sounding procedure (OPPO)

*** < Unchanged parts are omitted> ***

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource is set to 'semi-persistent':

-	when a UE receives an activation command, as described in clause 6.1.3.17 or 6.1.3.36 of [10, TS 38.321], for an SRS resource, and when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the activation command, the corresponding actions in [10, TS 38.321] and the UE assumptions on SRS transmission corresponding to the configured SRS resource set shall be applied starting from the first uplink slot that is after uplink slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. The activation command also contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the activated SRS resource set.

*** < Unchanged parts are omitted> ***

-	when a UE receives a deactivation command [10, TS 38.321] for an activated SRS resource set, and when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to the PDSCH carrying the deactivation command, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of SRS transmission corresponding to the deactivated SRS resource set shall apply starting from the first uplink slot that is after uplink slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

*** < Unchanged parts are omitted> ***

6.2.1	UE sounding procedure (OPPO)

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource is set to 'aperiodic':

-	the UE receives a configuration of SRS resource sets,

*** < Unchanged parts are omitted> ***

-	when a UE receives an spatial relation update command, as described in clause 6.1.3.26 of [10, TS 38.321], for an SRS resource configured with the higher layer parameter SRS-Resource, and when the HARQ-ACK corresponding to the PDSCH carrying the update command is transmitted in uplink slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions on updating spatial relation for the SRS resource shall be applied for SRS transmission starting from the first uplink slot that is after uplink slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. The update command contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the updated SRS resource set. Each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the update command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the update command if present, same serving cell and bandwidth part as the SRS resource set otherwise. When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', the UE shall not expect to be configured with different spatial relations for SRS resources in the same SRS resource set.

*** < Unchanged parts are omitted> ***

10.2.4.1 Company views

As a first proposal, companies are invited to indicate whether

- there is a need to reflect the clarification of uplink slot from OPPO

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | We believe the clarification may help avoiding ambiguity. |
| Apple | We do not think this modification is needed. It by default refers uplink slot. |
| Lenovo | We don’t think uplink is necessary. |
| Huawei, HiSilicon | We don’t see the need to add uplink since it is a slot where UE would transmit a PUCCH with HARQ-ACK information. |
| ZTE | Slot n is implicitly defined as a UL slot since it is for HARQ-ACK transmission corresponding to PDSCH. Therefore, there is no need for further clarification. |
| Thales | Reflecting the clarification of uplink slot as proposed by OPPO could be useful |
| NEC | There is no need to reflect the clarification. |
| Panasonic | We don’t see the need of this modification. |
| Samsung | We don’t think it is needed. |
| OPPO | From the RAN1 agreement in 105e as below, it clearly differentiates the slot following uplink timing vs slot following downlink timing. It uses 'uplink slot' (highlighted in yellow ) to indicate uplink timing and uses 'slot' (highlighted in blue) to indicate downlink timing. Our TP restrictly complies with the RAN1 agreement. RAN#1 105-e: Agreement: If a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac, where µ is the SCS configuration for the PUCCH. Note: Here K_mac is assumed to have the unit of the PUCCH slot. This can be revisited after the K_mac signaling design is finalized. |
| QC | No need |
| LG Electronics | Not needed. |
| MediaTek | Not needed |
| Ericsson | No need to clarify this. |
| Lockheed Martin | Not needed |

10.2.4.2 Summary of 1st round of discussion

There is little support for the clarification, and a number of replies point out that it is already clear that the specification text refers to uplink. The moderator conclusion is that this TP is not needed to be endorsed,

11 Stable Text Proposals

11.1 TP #1

TP #1 for endorsement:

------------------------------------------------------------------------------------------------------------------------------------------------

Adopt the following TP for 38.213: “where Kcell,offset is provided by Koffset in ServingCellConfigCommon CellSpecific_Koffset”

------------------------------------------------------------------------------------------------------------------------------------------------

11.2 TP #4

TP #4 for endorsement:

---------------------------------------------------------------------------------------------------------------------------------------------------

9	UE procedure for reporting control information

*** Unchanged text is omitted ***

For the remaining of this clause, if a UE is provided Kcell,offset by Koffset in ServingCellConfigCommon or KUE,offset by a MAC CE command, reference to a slot n+k for a PUCCH transmission or PUSCH transmission corresponds to a slot n+k+2μ∙Koffset for the PUSCH or the PUCCH transmission, where μ is the SCS configuration for the PUCCH transmission or PUSCH transmission and Koffset=Kcell,offset-KUE,offset. If Koffset or if the MAC CE command is not provided, Kcell,offset=0 or KUE,offset=0, respectively. If the PUCCH transmission or the PUSCH transmission is scheduled by a DCI format with CRC scrambled by TC-RNTI, KUE,offset=0. If the UE is provided a KUE,offset value by a MAC CE command, the UE applies the MAC command in the first slot that is after slot k+3Nslotsubframe,μ+2μ∙kmac where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the MAC CE command, μ is the SCS configuration for the PUCCH transmission that is determined in the slot when the MAC CE command is applied, and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided.

------------------------------------------------------------------------------------------------------------------------------------------------

11.3 TP #5A

TP #5A for endorsement:

-----------------------------------------------------------------------------------------------------------------------------------------------

5.2.2.5	CSI reference resource definition

The CSI reference resource for a serving cell is defined as follows:

-	In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates.

-	In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, if UE is configured with the higher layer parameter CellSpecific_Koffset, n-nCSI_ref, , where Koffset is a parameter configured by higher layer as specified in [TS 38.213 clause 4.2], and where μKoffsetis the subcarrier spacing configuration for Koffset, otherwise,

-	Koffset is provided with a value of ms for frequency range 1 and is equal to CellSpecific_Koffset - UESpecific_Koffset if UESpecific_Koffset is provided in MAC CE and CellSpecific_Koffset, otherwise;

-	where  +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and and  are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and  are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211]

-----------------------------------------------------------------------------------------------------------------------------------------------

11.4 TP #6A

TP #6A for endorsement:

-----------------------------------------------------------------------------------------------------------------------------------------------

6.1.2.1	Resource allocation in time domain

<<< unchanged paragraphs omitted >>>

in CSI-ReportConfig for the  triggered CSI Reporting Settings and  is the (m+1)th entry of .

-	The slot Ks where the UE shall transmit the PUSCH is determined by K2 as Ks =, if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, Ks=n⋅2μPUSCH2μPDCCH+K2+Koffset⋅2μPUSCH2μKoffset, otherwise, where Koffset is a parameter configured by higher layer as specified in [TS38.213 clause 4.2]if the UE is configured with the higher layer parameter CellSpecific_Koffset, Ks =, otherwise, and where μKoffsetis the subcarrier spacing configuration for Koffset, n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH,  and  are the subcarrier spacing configurations for PUSCH and PDCCH, respectively, Koffset is provided with a value of ms for frequency range 1 and is equal to CellSpecific_Koffset - UESpecific_Koffset if UESpecific_Koffset is provided in MAC CE and the scheduling DCI is other than DCI format 0_0 with CRC scrambled by TC-RNTI, and CellSpecific_Koffset otherwise.

-----------------------------------------------------------------------------------------------------------------------------------------------

11.5 TP #7A

TP #7A for endorsement:

-----------------------------------------------------------------------------------------------------------------------------------------------

6.2.1	UE sounding procedure

-	If the UE receives the DCI triggering aperiodic SRS in slot n and none of the resource sets is configured with parameter availableSlotOffset across all configured BWPs in a component carrier, and if the UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, except when SRS is configured with the higher layer parameter SRS-PosResource, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in slot , otherwise, the UE transmits aperiodic SRS in each of the triggered resource set(s) in slot Ks=n⋅2μSRS2μPDCCH+K2+Koffset⋅2μSRS2μKoffset, where Koffset is a parameter configured by higher layer as specified in [TS38.213 clause 4.2]if the UE is configured with the higher layer parameter CellSpecific_Koffset, Ks =, otherwise, and where

-	k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

-	μKoffsetis the subcarrier spacing configuration for Koffset, and Koffset is provided with a value of ms for frequency range 1 and is equal to CellSpecific_Koffset -UESpecific_Koffset if UESpecific_Koffset is provided in MAC CE and CellSpecific_Koffset otherwise.

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the  Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and when SRS is configured with the higher layer parameter SRS-PosResource, the UE transmits every aperiodic SRS resource in each of the triggered SRS resource set(s) in slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, Ks=n⋅2μSRS2μPDCCH+K2+Koffset⋅2μSRS2μKoffset, where Koffset is a parameter configured by higher layer as specified in [TS 38.213 clause 4.2]if UE is configured with the higher layer parameter CellSpecific_Koffset, Ks =, otherwise, and where

-	k is configured via higher layer parameter slotOffset for each aperiodic SRS resource in each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

-	μKoffsetis the subcarrier spacing configuration for Koffset.

-	Koffset is provided with a value of ms for frequency range 1 and is equal to CellSpecific_Koffset -UESpecific_Koffset if UESpecific_Koffset is provided in MAC CE and CellSpecific_Koffset otherwise.

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the  Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-----------------------------------------------------------------------------------------------------------------------------------------------

11.6 TP #8

---------------------------------------------------------------------------------------------------------------------------------------

TP #8 for endorsement:

Adopt the following TP for Section 6.2.1, 38.214: “Ks=n⋅2μSRS2μPDCCH+K2 k+Koffset⋅2μSRS2μKoffset, if UE is configured with the higher layer parameter CellSpecific_Koffset, Ks =, otherwise,”

---------------------------------------------------------------------------------------------------------------------------------------

12 Proposals for discussion at GTW sessions

12.1 GTW on Feb 23rd, 2022

Issue #8: Timing relationship of random access response

Conclusion 8

The additional delay introduced for the starts of ra-ResponseWindow and msgB-ResponseWindow should only apply to NTN. Adopt the following change in a TP to TS38.213 sections 8.2 and 8.2A:

“If NTA,adjUE or NTA,adjcommon is different from zero, the window starts after an additional TTA+kmac msec …”

Issue #9: RAN2 LS on NTN-SIB

Proposal 9.1

Send a reply LS to RAN2, indicating

  - RAN1 agrees with the list of nine parameters of serving cell information

  - RAN1 proposes two additional parameters (ntnPolarizationDL and ntnPolarizationUL) to be added into the parameters of serving cell information

  - clarify RAN1 understanding regarding the validity duration for UL sync information, and whether this would impact the SIBx design

Regarding bullet c):

RAN1 companies have different views on validity duration for UL sync information. One interpretation is that the validity duration only applies to ephemeris and common TA parameters, another interpretation for validity duration applies it to the whole NTN-specific SIB indicated in the RAN2 LS. There are also views that a change in ephemeris and common TA parameters would not trigger SIB modification.

For further discussion at GTW:

Limiting the validity duration to only ephemeris and common TA parameters may mean that a single NTN-specific SIB may not be sufficient for carrying all the parameters.

- validity timer applies to all parameters: one SIBx is enough

- validity timer applies only to ephemeris and common TA parameters: two SIBx needed?

References

R1-2112716, Feature lead summary#5 on timing relationship enhancements, Moderator (Ericsson)

R1-2200875. LS on NTN-specific SIB, RAN2

R1-2200937, Maintenance on timing relationship enhancements for NTN, Huawei, HiSilicon

R1-2201215, Timing relationship enhancements for NR-NTN, MediaTek Inc.

R1-2201271, Discussion on remaining issue for timing relationship enhancement, OPPO

R1-2201358, Remaining issues on timing relationship enhancements for NTN, CATT

R1-2201546, Discussion on timing relationship enhancements for NTN, Spreadtrum Communications

R1-2201645, Maintenance aspects of time relations for Rel-17 NR over NTN, Nokia, Nokia Shanghai Bell

R1-2201744, Remaining issues on timing relationship enhancement for NTN, InterDigital, Inc.

R1-2201771, Remaining Issues of Timing Relationship Enhancements for NR NTN, Apple

R1-2201804, On timing relationship maintenance issues for NR NTN, Ericsson Hungary Ltd

R1-2201852, Remaining issues on timing relationship enhancements for NTN, CMCC

R1-2201921, Remaining issues on the timing relationship for NTN, Xiaomi

R1-2201969, Discussion on NTN timing relationship, Lenovo, Motorola Mobility

R1-2202011, Maintenance issues on Timing relationship enhancements for NTN, Samsung

R1-2202206, Remaining issues of timing relationship for NR-NTN, ZTE

R1-2202241, Remaining issues on timing relationship enhancement for NTN, Baicells

R1-2202285, Remaining issues on timing relationship enhancements in NTN, LG Electronics

R1-2202360, Remaining issues on timing relationship enhancements for NTN, NEC

R1-2201646, Maintenance aspects of time and frequency synchronization for Rel-17 NR over NTN, Nokia

R1-2201853, Remaining issues on enhancements on UL time and frequency synchronization for NTN, CMCC

R1-2201216, Enhancements on UL Time and Frequency Synchronisation for NR-NTN, MediaTek Inc

R1-2201387, Enhancements on UL time and frequency synchronization, Panasonic R&D Center