3GPP TSG RAN WG1 Meeting #108-e	R1-22xxxxx

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.4.1

Source:	Moderator (Huawei, HiSilicon)

Title:	Responding to RAN2 LS on NTN-specific SIB

Document for:	Discussion

# Introduction

RAN1 received the LS on NTN-specific SIB (R1-2200875) from RAN2, and it was addressed during the 1st round of discussions in AI 8.4.1 FL summary. The comments indicated that RAN1 would indeed send a reply to RAN2, but the specifics regarding concerns on the validity duration applicability were not resolved during the GTW on 23rd February.

There is an agreement to at least reply with the following two bullets:

- RAN1 has no concern with the list of nine parameters of serving cell information

- RAN1 proposes two additional parameters (ntnPolarizationDL and ntnPolarizationUL) to be added into the parameters of serving cell information, including the relevant RAN1 agreements

The purpose of this document is to continue discussion and try to converge on any further text we can add to the reply LS to RAN2.

# Discussion

As observed at the GTW, RAN1 companies have different views on validity duration for UL sync information. One interpretation is that the validity duration only applies to ephemeris and common TA parameters, another interpretation for validity duration applies it to the whole NTN-specific SIB indicated in the RAN2 LS.

RAN2 agreement on system information update

1. Update of ephemeris and common TA information does not affect the value tag and does not trigger SI modification procedure.
2. The ntnUlSyncValidityDuration applies to the whole SIBX. UE acquires the updated SIBX when the timer expires. FFS whether to also include it in the LS to RAN1. FFS if this applies only to Connected mode or to idle mode UE as well

The RAN2 agreement makes it clear that the update of ephemeris and common TA information does not trigger SI modification procedure so this is not a concern.

From this agreement it would appear that there is no ambiguity between UEs with respect to applying the parameters other than ephemeris and common TA, because they would be taken into use at the start of a modification period.

Question 1:

In the current SI modification procedure an updated SI message is broadcasted in the modification period following the one where SI change indication is transmitted. Is the understanding of companies that the new SI information (other than ephemeris and common TA) only applies from the start of the modification period and not the instant that the UE reads it from the NTN-specific SIB.

| Company | Comments |
| --- | --- |

Question 2:

If the validity duration applies only to ephemeris and common TA parameters, do you think that the single SIBx structure considered by RAN2 is sufficient? That is , the UE reads not only ephemeris and common TA parameters but also all the other parameters each time.

| Company | Comments |
| --- | --- |