3GPP TSG RAN WG1 #108-e			R1-22xxxxx

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:			8.8.1.3

Source:			Moderator (China Telecom)

Title:			[108-e-R17-CovEnh-06] Summary of email discussion for incoming LS on Stage 2 description for Coverage Enhancements

Document for:		Discussion

- Introduction

RAN2 has sent an LS [1] to RAN1. RAN2 asks RAN1 to check the stage 2 CR in [2] and then inform RAN2 whether the stage 2 CR can be endorsed from RAN1’s perspective or not, and provide a RAN1-endorsed revision of the CR if necessary. The stage 2 CR in [2] is as follows.

18 Support for NR coverage enhancements To improve NR uplink coverage for both FR1 and FR2 as well as TDD and FDD, the following enhancements on PUSCH, PUCCH and MSG3 PUSCH are supported: - For PUSCH repetition Type A, the maximum number of repetitions is increased up to 32, applicable to both PUSCH transmission with and without dynamic grant. In addition, counting based on available slots is supported, the increased maximum number of repetitions for counting based on available slots and counting based on physical slots are both 32. - TB processing over multi-slot is supported for PUSCH transmission with and without dynamic grant. For a single transmission of TB processing over multi-slot PUSCH, the TB size is based on all the allocated REs across the multiple slots, and the number of slots is counted based on the available slots for UL transmission. In addition, repetition of TB processing over multi-slot PUSCH is also supported. - DMRS bundling is supported for PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, for PUSCH repetition Type A with configured grant, for PUSCH repetition Type B, for TB processing over multi-slot PUSCH and for PUCCH repetitions of PUCCH format 1, 3, 4. - Dynamic PUCCH repetition factor indication configured per PUCCH resource is introduced, applicable to all PUCCH formats. - PUSCH repetition Type A for MSG3 transmission is supported on both NUL and SUL, applicable to 4-step CBRA. If configured, the UE requests MSG3 repetition via separate PRACH resource when the RSRP of DL path-loss reference is lower than a configured threshold. Editor’s Note: The support for repetition of CFRA PUSCH is FFS, depending on whether the work assumption made in RAN1#107-e meeting that support repetition for CFRA PUSCH is confirmed in RAN1 or not.

This contribution is a summary of the following email discussion.

[108-e-R17-CovEnh-06] Email discussion for incoming LS on Stage 2 description for Coverage Enhancements (R1-2200879) by February 25 – Jianchi (China Telecom)

- Email discussion (1st round)

- Structure of the stage 2 CR

[8] has following proposals on the structure of the CR.

- the first bullet (PUSCH repetition type A) and the forth bullet (dynamic PUCCH repetition factor indication) are not necessary for stage 2 specification;

- the second bullet (TB processing over multiple slots PUSCH) can be added into Clause 5.3.1 of TS 38.300 as a new UL transmission scheme;

- the third bullet (DMRS bundling) can be added into Clause 5.3.1 of TS 38.300;

- the fifth bullet (PUSCH repetition Type A for MSG3 transmission) can be added into Clause 9.2.6 of TS 38.300 as enhanced PRACH procedure.

FL comments: From FL understanding, the structure of the CR depends on RAN2. RAN2 has already endorsed the stage 2 CR. RAN2 is asking RAN1 whether the functionalities of Rel-17 coverage enhancements are correctly captured from technical point of view. Therefore FL suggests not to revise the structure of current stage 2 CR from RAN2.

Do you agree with FL’s view?

| Companies | Comments |
| --- | --- |

- PUSCH repetition Type A

For PUSCH repetition Type A, [4][6][7] propose some revisions.

- For PUSCH repetition Type A, the maximum number of repetitions is increased to 32, applicable to both PUSCH transmission with dynamic grant scheduled by DCI format 0_1 and 0_2, and configured grant. In addition, counting based on available slots is supported for PUSCH repetition Type A with dynamic grant and configured grant. The maximum number of repetitions for counting based on available slots and counting based on physical slots are both 32.

Companies are encouraged to provide comments on the above revisions.

| Companies | Comments |
| --- | --- |

- TBoMS

For TBoMS, [4][6] propose some revisions.

- TB processing over multi-slot is supported for both PUSCH transmission with dynamic grant scheduled by DCI format 0_1 and 0_2, and configured grant. For a single transmission of TB processing over multi-slot PUSCH, the TB size is determined based on all the allocated REs across the multiple slots, the number of slots is counted based on the available slots for UL transmission, and same symbol allocation is applied across the multiple slots. In addition, repetition of TB processing over multi-slot PUSCH is supported.

Companies are encouraged to provide comments on the above revisions.

| Companies | Comments |
| --- | --- |

- DMRS bundling

For DMRS bundling, [4] proposes some revisions.

- DMRS bundling and inter-slot frequency hopping with inter-slot bundling are supported for PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, for PUSCH repetition Type A with configured grant, for PUSCH repetition Type B, for TB processing over multi-slot PUSCH and for PUCCH repetitions of PUCCH format 1, 3, 4.

Companies are encouraged to provide comments on the above revisions.

| Companies | Comments |
| --- | --- |

- Dynamic PUCCH repetition factor indication

For dynamic PUCCH repetition factor indication, [7] proposes some revisions.

- Dynamic PUCCH repetition factor indication configured per PUCCH resource a PUCCH with associated scheduling DCI is introduced, applicable to all PUCCH formats.

Companies are encouraged to provide comments on the above revisions.

| Companies | Comments |
| --- | --- |

- Msg3 repetition

For Msg3 repetition, [4][5][6] propose some revisions.

- PUSCH repetition Type A for MSG3 initial transmission and retransmission is supported on both NUL and SUL, applicable to 4-step CBRA. If supported, the UE requests MSG3 repetition via separate PRACH resource when the RSRP of DL path-loss reference is lower than a configured threshold.

Companies are encouraged to provide comments on the above revisions.

| Companies | Comments |
| --- | --- |

- Editor’s Note

For Editor’s Note, [7] proposes some revisions.

Editor’s Note: The support for repetition of CFRA PUSCH is a working assumption made in RAN1#107-e meeting.

Companies are encouraged to provide comments on the above revisions.

| Companies | Comments |
| --- | --- |

- Reference

- 3GPP R2-2201784, LS on Stage 2 description for Coverage Enhancements, RAN2, China Telecom, RAN2#116bis-e, Jan 17th - 25th, 2022.

- 3GPP R2-2201963, Running 38300 CR for NR coverage enhancements, China Telecom, RAN2#116bis-e, Jan 17th - 25th, 2022.

- 3GPP R1-2201157, [Draft] Reply LS on Stage 2 description for Coverage Enhancements, ZTE, February 21st – March 3rd, 2022.

- 3GPP R1-2201675, Discussion on Stage 2 description for Coverage Enhancements, Intel, February 21st – March 3rd, 2022.

- 3GPP R1-2201843, Discussion on RAN2 LS on Stage 2 description for Coverage Enhancements, CMCC, February 21st – March 3rd, 2022.

- 3GPP R1-2201928, Discussion on RAN2 LS on Stage 2 description for Coverage Enhancements, Xiaomi, February 21st – March 3rd, 2022.

- 3GPP R1-2202415, Draft Reply LS on Stage 2 Description for Coverage Enhancements, Ericsson, February 21st – March 3rd, 2022.

- 3GPP R1-2202463, Discussion on LS on Stage 2 description for Coverage Enhancements, Huawei, HiSilicon, February 21st – March 3rd, 2022.