3GPP TSG RAN WG1 Meeting #108-e	R1-22xxxxx

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.9.1

Source:	Moderator (Huawei)

Title:	Feature lead summary #1 on 108-e-LTE-Rel17-NB-IoT-eMTC-01

Document for:	Discussion and Decision

# Introduction

The WID for Rel-17 enhancements for NB-IoT and LTE-MTC [1] includes an objective to support 16-QAM for unicast in UL and DL in NB-IoT.

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4]

  - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4]

This documents provides the proposals and summary of discussions of the corresponding email discussion according to the inputs [2-11].

[108-e-LTE-Rel17-NB-IoT-eMTC-01] Email discussion on support of 16-QAM for unicast in UL and DL for NB-IoT – Yubo (Huawei)

- 1st check point: November 15

- Final check point: November 19

# Discussion

## Uplink power control

### Issue 1: uplink power control

The companies’ proposals are listed in the table below

| Sourcing | Proposals |
| --- | --- |
| [2] | Proposal 2：The new power control term can be applied to NPUSCH with QPSK when configured with 16QAM. |
| [3] | Observation 1: For legacy QPSK, there is no clear requirements to introduce the new term ∆TF,c for uplink power control in NB-IoT. Observation 2: For NPUSCH with QPSK and TBS 1-6, uplink power reduction caused by ∆TF,c will lead to a performance loss if ∆TF,c is applied to QPSK. Proposal 1: An offset can be applied on ∆TF,c to reduce the power difference between QPSK and 16QAM. - The offset could be indicated by higher layers. |
| [4] | Proposal 2: The new uplink power control term ∆TF,ci is also applied to QPSK when UE is configured with 16-QAM. |
| [5] | Proposal 2: The new term ΔTF also applies to QPSK, when configured with 16-QAM. |
| [6] | Proposal 2: The new term ∆TF,c introduced for power control of NPUSCH applies to QPSK and 16QAM when configured with 16QAM. |
| [7] | Proposal 2: The new term ∆TF,c should apply to both 16QAM and QPSK, no offset needed. |
| [8] | Observation 7 A new term (ΔTF) for 16-QAM in UL was introduced as to account for the larger number of bits per RE that this higher order modulation scheme introduces. Observation 8 Due to the introduction of ΔTF, it was pointed out the possibility of introducing a way to prevent a large power difference between QPSK and 16-QAM. Observation 9 Two proposals remained considered to alleviate the power difference between QPSK and 16-QAM: 1) “Introducing ΔTF for QPSK” and 2) “Introducing an Offset acting on ΔTF for 16-QAM”. Observation 10 “Introducing ΔTF for QPSK” has as a side effect QPSK resulting in an UL power control behavior that will be different with and without 16-QAM configured. Observation 11 Due that it was not possible to reach a consensus towards 1) or 2), at some point one company commented that “in terms of open loop such jump up to 6.5dB is very common, perhaps we could let it go”. Observation 12 In our view, the WID’s objective was about introducing 16-QAM for NB-IoT and therefore we should not create side effects (i.e., different behaviors) from making modifications touching upon legacy modulation schemes. Observation 13 Based on observation 12, any solution intended to alleviate the power difference between QPSK and 16-QAM should be based on a solution acting on 16-QAM elements (i.e., offset acting on ΔTF), otherwise is preferred to deal with a power difference between QPSK and 16-QAM. Proposal 3 If the power difference between QPSK and 16-QAM is to be alleviated, it should be based on a solution acting on 16-QAM elements (i.e., offset acting on ΔTF), otherwise is preferred to live with such a power difference between modulation schemes. |

The following has been agreed in last meeting:

Agreement

The following working assumption is confirmed.

For the new term ∆TF,c introduced for power control of NPUSCH,

- Reuse the LTE definition simplified for NB-IoT: ∆TF,ci=10log102BPRE∙Ks-1 for Ks=1.25 and ∆TF,ci=0 for Ks=0, where Ks is given by higher layer parameter deltaMCS-Enabled, and BPRE=KNRE where K is the code block size.

- FFS: whether the new term applies to QPSK when configured with 16QAM, if it does not, whether an additional term is introduced to avoid jump between QPSK and 16QAM

On the FFS part, based on the comments, it will be down-selected from the following options:

- Option 1: The term ∆TF,ci can also be applied to NPUSCH with QPSK, when 16-QAM is configured.

- Option 2: An offset to ∆TF,ci is configured from a set of {[1dB], [2dB], [4dB], [6dB]}, when 16-QAM is configured.

For information, the ∆TF,ci calculated are summarized in the following table:

| Modulation | ITBS |  | IRU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| QPSK | 13 | TBS | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |
|  |  | ∆TF,ci | 4.546765 | 5.238875 | 5.374201 | 5.708471 | 5.481782 | 5.686359 | 5.542038 | 5.562083 |
| 16QAM | 14 | TBS | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |

And the power control values for both options are listed as below, assuming 5RUs, and that the default P0 is 0dB, and the power of 16QAM NPUSCH for option 1 and option 2 is the same.

| ITBS | Modulation | TBS | ∆TF,c : option 1 | P0 setting for option 1 | ∆TF,c : option 2 | Offset for option 2 | P0 setting for option 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | QPSK | 176 | -6.27282 | 0 | 0 | 0 | 6.5 |
| 3 | QPSK | 256 | -4.42746 | 0 | 0 | 0 | 6.5 |
| 4 | QPSK | 328 | -3.15198 | 0 | 0 | 0 | 6.5 |
| 5 | QPSK | 424 | -1.76735 | 0 | 0 | 0 | 6.5 |
| 6 | QPSK | 504 | -0.7883 | 0 | 0 | 0 | 6.5 |
| 7 | QPSK | 584 | 0.083221 | 0 | 0 | 0 | 6.5 |
| 8 | QPSK | 680 | 1.026534 | 0 | 0 | 0 | 6.5 |
| 9 | QPSK | 776 | 1.887086 | 0 | 0 | 0 | 6.5 |
| 10 | QPSK | 872 | 2.685284 | 0 | 0 | 0 | 6.5 |
| 11 | QPSK | 1000 | 3.676093 | 0 | 0 | 0 | 6.5 |
| 12 | QPSK | 1128 | 4.603156 | 0 | 0 | 0 | 6.5 |
| 13 | QPSK | 1256 | 5.481782 | 0 | 0 | 0 | 6.5 |
| 14 | 16QAM | 1416 | 6.528084 | 0 | 6.528084 | -6.5 | 6.5 |
| 15 | 16QAM | 1544 | 7.332797 | 0 | 7.332797 | -6.5 | 6.5 |
| 16 | 16QAM | 1608 | 7.726365 | 0 | 7.726365 | -6.5 | 6.5 |
| 17 | 16QAM | 1800 | 8.878457 | 0 | 8.878457 | -6.5 | 6.5 |
| 18 | 16QAM | 1992 | 9.996363 | 0 | 9.996363 | -6.5 | 6.5 |
| 19 | 16QAM | 2152 | 10.90802 | 0 | 10.90802 | -6.5 | 6.5 |
| 20 | 16QAM | 2344 | 11.98355 | 0 | 11.98355 | -6.5 | 6.5 |
| 21 | 16QAM | 2536 | 13.04336 | 0 | 13.04336 | -6.5 | 6.5 |

The company positions for the two options are as following:

- Option 1: The term ∆TF,ci can also be applied to NPUSCH with QPSK, when 16-QAM is configured.

  - Huawei, HiSilicon, Nokia, NSB, Qualcomm, MediaTek,

- Option 2: An offset to ∆TF,ci is configured from a set of {[1dB], [2dB], [4dB], [6dB]}, when 16-QAM is configured.

  - ZTE, Sanechips, Ericsson

As this issue has discussed for several meetings without consensus, please input your comments of following:

- technical concerns that the option not preferred could not work

- any update to the option not preferred so that it’s acceptable to you.

| Companies | Comments |
| --- | --- |
| Ericsson | The technical concern we have with Option 1 is that we should not only see the issue from the perspective of UEs supporting 16-QAM, but also from the perspective of the co-existence of UEs supporting 16-QAM and UEs not supporting this feature. For those UEs configured with 16-QAM the QPSK transmissions will account for an extra parameter, which won’t be the case for the QPSK transmissions of UEs not supporting 16-QAM. Thus, if the power difference between QPSK and 16-QAM is to be alleviated, it should be based on a solution acting on 16-QAM elements (e.g., an offset acting on ΔTF as per Option 2), otherwise is preferred to live with such a power difference between modulation schemes. |
| Qualcomm | Regarding Ericsson’s concern, if it is desirable to keep the same power control between QPSK and 16-QAM UEs, the base station can disable this feature by setting Ks=0. In any case, the power difference between QPSK and 16-QAM cannot be too large. |
| Lenovo, MotoM | Support Option 1. For NPUSCH uplink power control, the parameter P0 is UE-specific configured. So, if we want to address the UE uplink power fairness concern for UE with/wo 16QAM, different UEs can have different P0, which can be done by eNB implementation. For Option 2, what is the metric for eNB to determine the offset ∆TF,ci is configured as [1dB], [2dB], [4dB] or [6dB]? |
| MTK | Ok with Option1 |
| Ericsson v006 | To Lenovo: On the question about the “metric”, isn’t so that the same question applies to your proposal of using “P0”. If I’m not wrong in both cases it would be up to the eNodeB to determine the offset/P0. |

## Channel quality reporting

### Issue 2: Configuration and switching of CQI table

The companies’ proposals are listed in the table below

| Sourcing | Proposals |
| --- | --- |
| [2] | Proposal 1: The use of legacy table or the new CQI table is indicated by UE in MAC CE, if 16QAM is configured. |
| [3] | Proposal 2: When DL 16QAM is configured, UE indicates the use of legacy or new CQI table via MAC CE. |
| [4] | Proposal 1: On the use of legacy measurement reporting table, our preferences are – - 1st preference: The eNB can configure, via higher-layer signalling, the CQI table to be used by the UE when configured with 16-QAM. - 2nd preference: If 16-QAM in DL is configured, then the UE should use the 16-QAM CQI table, otherwise the UE will use the legacy table. |
| [5] | Proposal 4: The UE uses the 16-QAM CQI table if it is configured with 16-QAM, otherwise it uses the QPSK table. |
| [6] | Proposal 1: When 16QAM is configured, the new CQI table is used. UE determines the legacy or new CQI table based on Rmax, or eNB indicates the use of legacy or new CQI table via MAC CE or RRC configuration. |
| [7] | Proposal 1: switching of CQI table should down selected from following two options. - Option 1: UE indicates the use of legacy or new CQI table via MAC CE. - Option 2: eNB indicates the use of legacy or new CQI table via MAC CE. |
| [8] | - Observation 1 Towards the end of RAN1# 107-e, the possibility of switching between the Rel-17 CQI table and the legacy CQI table was discussed. - Observation 2 There were several proposals for performing the CQI Table switching such as using MAC CE, Rmax, RRC configuration, and “if 16QAM in DL is configured in msg4, then the UE should use the 16QAM CQI table, otherwise the UE will use the legacy table”. Observation 3 During RAN1# 107-e, it was mentioned that the designed Rel-17 CQI table should be sufficient as to do not require a switching to the legacy table since UEs should be configured with a small Rmax value, and the radio conditions of such UEs cannot change so drastically as to require reports tied to a very large number of repetitions. - Observation 4 16-QAM was designed to be used with 1 repetition, if due to a change in the radio conditions were necessary to switch to QPSK, it seems that the Rel-17 CQI Table covers a reasonable margin of NPDCCH repetitions (up to 32 repetitions). - Observation 5 We need to consider that going beyond the number of repetitions (> 32 repetitions) covered by the Rel-17 CQI table, may even result in an RLF for a UE configured with a small Rmax value. - Observation 6 Thus, for a scenario requiring a large number of repetitions (i.e., > 32) a CQI table switching mechanism may result to be irrelevant, since in those scenarios a larger Rmax would need to be configured. - Proposal 1 Based on the Rel-17 CQI table design which covers up to 32 repetitions for QPSK, introducing a table switching mechanism is no longer necessary. |

On configuration and switching of the CQI table, the company positions are summarized as below:

  - Option 1: UE indicates the use of legacy or new CQI table via MAC CE.

    - Huawei, HiSilicon, ZTE, Sanechips, MediaTek

  - Option 2: eNB indicates the use of legacy or new CQI table via MAC CE.

    - MediaTek, Lenovo, Moto

  - Option 3: eNB configures the use of legacy or new CQI table via RRC configuration

    - Nokia, NSB (1st), Lenovo, Moto

  - Option 4: if Rmax<=16, the new CQI table is used, otherwise, the legacy CQI table is used.

    - Lenovo, Moto

  - Option 5: the 16-QAM CQI table is used if DL 16-QAM is configured, otherwise the legacy CQI table is used

    - Nokia, NSB (2nd), Qualcomm, Ericsson

As the views are still very diverse, it is proposed to down-select from the two options with support of most number of companies, i.e., option 1 and option 5.

Proposal 1: When 16QAM is configured, the new CQI table is used. On use of the legacy CQI table, it’s down-selected from following options:

  - Option 1: UE indicates the use of legacy or new CQI table via MAC CE.

  - Option 5: the 16-QAM CQI table is used if DL 16-QAM is configured, otherwise the legacy CQI table is used

Please input your preference regarding the two options.

| Companies | Comments |
| --- | --- |
| Ericsson | Given the final design of the Rel-17 CQI table, we think that there is no need of introducing any CQI table switching mechanism. Hence, Option 5 is our preference which in our understanding does not need any specification statement since it is an inherent fact for the feature. |
| Qualcomm | Option 5 – it is very unclear why any of the other options are needed. |
| Lenovo, MotoM | Consider the status, we are OK to support option 5, no optimization is needed. For option 1, Is the additional 1-bit table selection indication always along with 4bit CQI reporting in MAC CE? Why not directly keep all entries of legacy table and extend the 16QAM CQI entries, report 5 bit at the beginning? Option 1 is not straightforward way (4+1 instead of 5). |
| MTK | Actually, 16QAM configured UE might encounter channel condition in which NPDCCH repetition is more than 32, option5 can’t accommodate this . Considering the fact Option1 extends reporting bits to 5 bits, option5 with 5 bits CQI table including all legacy and 16QAM extended entries might be the optimal solution. |

## Text proposals

### EPRE for 16-QAM

In section 1 of [5], it is proposed to replace the description of constant power between symbols by equations, with the following text proposal

### TP1 (TS 36.213) ### 16.2.2 Downlink power allocation […] If a UE is configured with higher layer parameters npdsch-16QAM-Config and nrs-PowerRatio, # - if higher layer parameter operationModeInfo indicates '10' or '11', - the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs (not applicable to NPDSCH REs with zero EPRE) is given by the parameter nrs-PowerRatio in symbols without NRS, and - the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs in symbols with NRS is given by 1s×(6ρ-1), where ρ is given by the parameter nrs-PowerRatio, and ρ=4 for a cell with two NRS antenna ports and ρ=5 for a cell with one NRS antenna port - otherwise, - the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs (not applicable to NPDSCH REs with zero EPRE) is given by the parameter nrs-PowerRatio in symbols without NRS and CRS, and - the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs (not applicable to NPDSCH REs with zero EPRE) is given by the parameter nrs-PowerRatioWithCRS in symbols with CRS, and - the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs in symbols with NRS is given by 1s×(6ρ-1), where ρ is given by the parameter nrs-PowerRatio, and ρ=4 for a cell with two NRS antenna ports and ρ=5 for a cell with one NRS antenna port .

Please input your comments regarding the above text proposal:

| Companies | Comments |
| --- | --- |
| Ericsson | Math-wise the proposed TP seems to be correct, the only thing that needs to be amended is the following variable “ρs=4 for a cell with two NRS antenna ports and ρs=5 for a cell with one NRS antenna port”. |
| Qualcomm | Agree with the typo correction by Ericsson, it should be s. |
| Lenovo, MotoM | We are also OK to use the equation to illustrate the power ratio. However, there is no definition of s in the current spec, so we can directly use the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs in symbols with NRS is given by 15(6ρ-1) for a cell with one NRS antenna port and 14(6ρ-1) for a cell with two NRS antenna ports, where ρ is given by the parameter nrs-PowerRatio. |

### Configuration for PUR

In section 2 of [5], it is proposed that the configuration/behavior of 16-QAM for downlink is as following:

- If 16 QAM is enabled in PUR, and the DCI is mapped to the search space by PUR-RNTI, and MCS=’1111’, or

- If 16QAM is enabled in UE-specific RRC, and the DCI is mapped to the search space by C-RNTI and MCS = ‘1111’, then

  - Use 16-QAM as the modulation order.

And it is proposed to endorse the following text proposal:

#### TP 3(TS 36.213): #### 16.4.1.5 Modulation order and transport block size determination To determine the modulation order in the NPDSCH, the UE shall - if the UE is configured with higher layer parameter npdsch-16QAM-Config and the DCI is mapped onto the UE specific search space and the 4-bit "modulation and coding scheme" field () in the DCI is set to ‘1111’, or if the UE is configured with higher layer parameter pur-DL-16QAM-Config and the DCI is mapped onto the UE specific search space given by PUR-RNTI and the 4-bit "modulation and coding scheme" field () in the DCI is set to ‘1111’, - use modulation order, = 4 - otherwise - use modulation order, = 2.

In section 2.2 of [8], the same issue is discussed, and the following text proposal is proposed:

------------------------------------------------------- Text Start ----------------------------------------------------------- #### 16.4.1.5 Modulation order and transport block size determination To determine the modulation order in the NPDSCH, the UE shall - if the UE is configured with higher layer parameter npdsch-16QAM-Config and the DCI is mapped onto the UE specific search space given by C-RNTI or if the UE is configured with higher layer parameter pur-DL-16QAM-Config and the DCI is mapped onto the UE specific search space given by PUR-RNTI, and the 4-bit "modulation and coding scheme" field () in the DCI is set to ‘1111’, - use modulation order, = 4 - otherwise - use modulation order, = 2. ------------------------------------------------------- Text End -----------------------------------------------------------

Please input your comments regarding the above two text proposals:

| Companies | Comments |
| --- | --- |
| Ericsson | Either of the two TPs above are ok. Perhaps we slightly prefer the one at the bottom due that it is a bit shorter, but if the one on the top were adopted it seems that we would need to add “given by C-RNTI” before the track changes start. |
| Qualcomm | Either TP works. |
| Lenovo, MotoM | If the concern of section 2.4 is addressed, that is, common understanding of the configuration of DL 16QAM in connected mode and idle mode has been achieved, we are OK to add the DL 16QAM in PUR in modulation determination. For the first TP, there is duplicated condition 5 in the IF condition if 1 and 2 and 5 or if 3 4 and 5 For the second TP, there may be some misunderstanding of condition 5 in the logic if 1 and 2 or if 3 and 4, and 5 How about the following combination of the above two TP? To determine the modulation order in the NPDSCH, the UE shall - if the UE is configured with higher layer parameter npdsch-16QAM-Config and the DCI is mapped onto the UE specific search space given by C-RNTI, or the UE is configured with higher layer parameter pur-DL-16QAM-Config and the DCI is mapped onto the UE specific search space given by PUR-RNTI, - if the 4-bit "modulation and coding scheme" field () in the DCI is set to ‘1111’, - use modulation order, = 4 - otherwise - use modulation order, = 2 - otherwise - use modulation order, = 2. |
| Ericsson v006 | Lenovo’s proposal seems in principle to be fine as well and a bit clearer. |

### Support of 16-QAM in TB processing of NPUSCH

In section 3.2.1, it is proposed to capture the missed part of 16-QAM in TB processing of NPUSCH, with the following text proposal:

### --------------------------------------------------------- Text Start --------------------------------------------------------- ### 6.3.2 Uplink shared channel Figure 6.3.2-1 shows the processing structure for the UL-SCH transport channel. Data arrives to the coding unit in the form of a maximum of one transport block over a number of resource units per UL cell. The number of resource units is scheduled according to [3]. The following coding steps can be identified: - CRC attachment - Channel coding - Rate matching Figure 6.3.2-1: Transport block processing for UL-SCH The CRC attachment, channel coding, and rate matching are performed according to clauses 5.2.2.1, 5.2.2.3, and 5.2.2.4, respectively, with the following differences: - In clause 5.1.4.1.2 in the calculation of , Qm is 1 for π/2-BPSK, 2 for π/4-QPSK and 4 for 16QAM, and rvidx = 0 or 2. In addition, after rate matching interleaving is applied per resource unit according to clauses 5.2.2.7 and 5.2.2.8 without any control information in order to apply a time-first rather than frequency-first mapping, where the input sequence to 5.2.2.7 is the portion of e for a resource unit instead of f, and where is the number of SC-FDMA symbols for NPUSCH in a UL resource unit as given in clause 10.1.2.3 of [2]. ------------------------------------------------------- Text End -----------------------------------------------------------

Please input your comments regarding the above text proposal:

| Companies | Comments |
| --- | --- |
| Ericsson | OK with the TP. It seems needed, since now 16QAM is supported. |
| Qualcomm | It is unclear why this change is needed. This clause defines the value of Qm for those modulation schemes that are only supported in NB-IoT (and therefore are not present in the following text in 5.1.4.1.2): Set where Qm is equal to 2 for QPSK, 4 for 16QAM, 6 for 64QAM and 8 for 256QAM, and where 16-QAM is already there, so there is no need to list it in “with the following differences”. Actually, the mentioning of pi/2 BPSK could be removed since it is supported in eMTC as well (and referred in 5.1.4.1.2 explicitly) – the text above is from latest Rel-13, where pi/2 BPSK was not supported for eMTC. |
| Lenovo, MotoM | It is better to keep the original text (although 2 for π/4-QPSK is redundant from Rel.13) |
| Ericsson v006 | Thanks to Qualcomm for the comment, in that case we need to make the specification consistent and under the same argument given that “π/2-BPSK” is already in 5.1.4.1.2 then it should be removed since it does not fall anymore into category “with the following differences” either. |

### Uplink power control for PUR NPUSCH with 16QAM

In section 3.2.2.1 of [8], it is proposed to add the uplink power control for NPUSCH with 16-QAM, with following text proposal:

### ------------------------------------------------------- Text Start ---------------------------------------------------------- ##### 16.2.1.1.1 UE behaviour The setting of the UE Transmit power for a Narrowband Physical Uplink Shared Channel (NPUSCH) transmission is defined as follows. For FDD, if the UE is capable of enhanced random access power control [12], and it is configured by higher layers, and for TDD, enhanced random access power control shall be applied for a UE which started the random access procedure in the first or second configured NPRACH repetition level. ### ------------------------------------------------------- Text Omitted ------------------------------------------------------- - If NPUSCH (re)transmissions with 16QAM or NPUSCH (re)transmission corresponding to preconfigured uplink resource with 16QAM, - ΔTF,ci=10log102BPRE⋅Ks-1 for and ∆TF,c(i)=0 for where is given by the parameter deltaMCS-Enabled provided by higher layers for serving cell , and - BPRE=K/NRE where K is the code block size and NRE is the number of resource elements determined as NRE=(NsymbUL-1)NslotsULNscRUNRU where NsymbUL, NslotsUL, NscRU are defined in [3], and NRU is defined in section 16.5.1.1 - otherwise ∆TF,c(i)=0. ------------------------------------------------------- Text End -----------------------------------------------------------

Please input your comments regarding the above text proposal:

| Companies | Comments |
| --- | --- |
| Ericsson | Ok with the TP. The new term in the UE’s transmit power control equation is also applicable for PUR, nonetheless since the statement “NPUSCH (re)transmissions with 16QAM” does not encompass PUR, then an explicit statement about PUR needs to be added. |
| Lenovo, MotoM | The CR is not necessary. “If NPUSCH (re)transmissions with 16QAM” includes NPUSCH (re)transmission with PUR |
| Ericsson v006 | I do not think “NPUSCH (re)transmissions with 16QAM” includes “NPUSCH (re)transmission corresponding to preconfigured uplink resource with 16QAM”. All over the place in the technical specifications we have been distinguishing ordinary “NPUSCH (re)transmissions” from “NPUSCH (re)transmission corresponding to preconfigured uplink resource” through such a differentiated wording. |

### The indices of MCS for PUR NPUSCH

In section 3.2.2.2 of [8], it is proposed to clarify how the indices of MCS for PUR NPUSCH is provided, with the following text proposal:

### ------------------------------------------------------- Text Start ---------------------------------------------------------- #### 16.5.1.2 Modulation order, redundancy version and transport block size determination To determine the modulation order, redundancy version and transport block size for the NPUSCH, the UE shall first - read the "modulation and coding scheme" field () in the DCI or configured by higher layers for NPUSCH transmission using preconfigured uplink resource, and - read the "redundancy version" field () in the DCI or initiate with rvDCI=0 for NPUSCH transmission using preconfigured uplink resource, and - read the "resource assignment" field () in the DCI or configured by higher layers for NPUSCH transmission using preconfigured uplink resource, and - compute the total number of allocated subcarriers (), number of resource units (), and repetition number () according to Clause 16.5.1.1. ### ------------------------------------------------------- Text Omitted ------------------------------------------------------- The UE shall use (,) and Table 16.5.1.2-2 to determine the TBS to use for the NPUSCH. is given in Table 16.5.1.2-1 if , or ITBS=IMCS'+14 if NPUSCH with 16QAM except for NPUSCH transmission using preconfigured uplink resource in which case the corresponding indices are provided in PUR-Config-NB, otherwise. IMCS' is the value of the "modulation and coding scheme for 16QAM" in the DCI. ------------------------------------------------------- Text End -----------------------------------------------------------

Please input your comments regarding the above text proposal:

| Companies | Comments |
| --- | --- |
| Ericsson | Ok with the TP, since it is not captured from where the information is obtained in the case of NPUSCH transmission using preconfigured uplink resources. |
| Lenovo, MotoM | OK with the TP in general. Can we directly use ITBS instead of the corresponding indices. or ITBS=IMCS'+14 if NPUSCH with 16QAM except for NPUSCH transmission using preconfigured uplink resource in which case ITBS is given by npusch-MCS-r17 in PUR-Config-NB, |

## Others

There are also following proposals:

| Sourcing | Proposals |
| --- | --- |
| [6] | Proposal 3: DL 16QAM in PUR is configured only in condition that DL 16QAM in connected mode is configured |

Please input your comments regarding the above proposal, or any other critical issues you think should be discussed:

| Companies | Comments |
| --- | --- |
| Ericsson | In our view there is no need to tie idle-mode to connected-mode for the 16-QAM feature. PUR has its own toolbox to perform adjustments (e.g., ways of determining if the UE requires a PUR re-configuration) and therefore there is no need of conditioning the usage of 16-QAM for PUR based on a configuration for connected-mode, since the PUR feature should maintain its autonomy. |
| Qualcomm | This restriction is unnecessary. |
| Lenovo, MotoM | We are willing to accept the configurations separately. However, eNB schedules MCS for DL 16QAM in connected mode based on the new CQI table, while UE in idle mode (e.g., PUR) don’t support the CQI reporting. We are wondering how does the eNB configure a suitable MCS (e.g., 16QAM) for UE in PUR when DL16QAM in connected mode is disabled while the DL 16QAM in PUR is enabled? Based on RSRP/RSRQ? Based on statical TB BLER? |

# Summary

# References

- RP-211340, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon, RAN#92e, E-meeting, June 2021.

- R1-2200976	Support of 16QAM for unicast in UL and DL in NB-IoT	Huawei, HiSilicon

- R1-2201135	Discussion on remaining issues for NB-IoT 16QAM	ZTE, Sanechips

- R1-2201407	Support of 16-QAM for unicast in UL and DL for NB-IoT	Nokia, Nokia Shanghai Bell

- R1-2201650	Support of 16-QAM for NB-IoT	Qualcomm Incorporated

- R1-2201968	Support 16QAM for NBIoT	Lenovo, Motorola Mobility

- R1-2202076	Remaining issue for support 16QAM in NB-IOT R17	MediaTek Inc.

- R1-2202277	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

- R1-2202280	Clarification on the support of 16-QAM for NB-IoT in TS 36.212	Ericsson

- R1-2202281	Clarification on the support of 16-QAM for NB-IoT in TS 36.213	Ericsson

- R1-2202477	Further considerations on Rel-17 NB-IoT and eMTC enhancements	Huawei, HiSilicon