3GPP TSG RAN WG1 Meeting #108-e	R1-22xxxxx

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.9.1

Source:	Moderator (Huawei)

Title:	Text proposals for NB-IoT 16QAM

Document for:	Discussion and Decision

Introduction

Based on the following email discussion, the text proposals in section 2 are proposed to be endorsed.

[108-e-LTE-Rel17-NB-IoT-eMTC-01] Email discussion on support of 16-QAM for unicast in UL and DL for NB-IoT – Yubo (Huawei)

- 1st check point: November 15

- Final check point: November 19

Text proposals

Text proposals to TS 36.213

EPRE for 16-QAM

It is proposed to replace the description of constant power between symbols by equations, with the following text proposal

===========================TP starts============================== ### 16.2.2 Downlink power allocation <unchanged parts omitted> If a UE is configured with higher layer parameters npdsch-16QAM-Config and nrs-PowerRatio, - - the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs in symbols with NRS is given by 15×(6ρ-1) for a cell with one NRS antenna port and 14×(6ρ-1) for a cell with two NRS antenna ports, where ρ is given by the parameter nrs-PowerRatio. - if higher layer parameter operationModeInfo indicates '10' or '11', - the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs (not applicable to NPDSCH REs with zero EPRE) is given by the parameter nrs-PowerRatio in symbols without NRS - otherwise, - the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs (not applicable to NPDSCH REs with zero EPRE) is given by the parameter nrs-PowerRatio in symbols without NRS and CRS, and - the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs (not applicable to NPDSCH REs with zero EPRE) is given by the parameter nrs-PowerRatioWithCRS in symbols with CRS. ===========================TP ends==============================

Configuration for PUR

It is proposed to capture the use of 16QAM for NPUSCH with the following text proposal:

===========================TP starts============================== #### 16.4.1.5 Modulation order and transport block size determination To determine the modulation order in the NPDSCH, the UE shall - if the UE is configured with higher layer parameter npdsch-16QAM-Config and the DCI is mapped onto the UE specific search space given by C-RNTI, or the UE is configured with higher layer parameter pur-DL-16QAM-Config and the DCI is mapped onto the UE specific search space given by PUR-RNTI, - If the 4-bit "modulation and coding scheme" field () in the DCI is set to ‘1111’, - use modulation order, = 4 - otherwise - use modulation order, = 2. - otherwise - use modulation order, = 2. ===========================TP ends==============================

The indices of MCS for PUR NPUSCH

It is proposed to clarify how the indices of TBS for PUR NPUSCH is provided with the following text proposal:

===========================TP starts============================== #### 16.5.1.2 Modulation order, redundancy version and transport block size determination <unchanged part is omitted> The UE shall use (,) and Table 16.5.1.2-2 to determine the TBS to use for the NPUSCH. is given in Table 16.5.1.2-1 if , or ITBS=IMCS'+14 if NPUSCH with 16QAM except for NPUSCH transmission using preconfigured uplink resource in which case ITBS is given by higher layers in PUR-Config-NB, otherwise. IMCS' is the value of the "modulation and coding scheme for 16QAM" in the DCI. ===========================TP ends==============================