3GPP TSG-RAN WG1 Meeting #108-e	R1-2xxxxxx

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.9.2

Source:	Moderator (Ericsson)

Title:	Moderator Summary [108-e-R17-NB-IoT-eMTC-02]

Document for:	Discussion and Decision

1	Introduction

As part of the Work Item (WI) on “Additional enhancements for NB-IoT and LTE-MTC” [1] the following enhancement for LTE-MTC was specified in Rel-17:

- Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL, for HD-FDD Cat M1 UEs. [LTE-MTC] [RAN1]

This document summarizes remaining issues on the introduction of 14 HARQ processes in DL for HD-FDD Cat M1 UEs according with [2-5].

2	Moderator summary on 14 HARQ processes in DL in LTE-MTC

2.1 	Usability of the “Repetition number” field

During RAN1# 107-e, the following agreement on the “Repetition number” field was reached [2]:

Agreement In Rel-17, for the 14 HARQ processes feature the “Repetition number” field is: · Opt-3: 2-bits as in legacy Note: Further optimization for using Repetition number” field is not pursued

Towards the end of RAN1# 107-e, the following potential conclusion was drafted as to be resolved during the maintenance phase [3]:

Potential Conclusion: - Alt-A: In Rel-17 for the 14 HARQ processes feature, the usage of the “Repetition Number” field follows the legacy behavior where if the “HARQ-ACK bundling flag” field is set to 1 the UE shall assume that the PDSCH repetitions equal 1. - Note 1: There is no impact on TS 36.212. - Note 2: TS 36.213 to capture that the above legacy behavior also applies for the 14 HARQ processes feature. - Alt-B: In Rel-17 for the 14 HARQ processes feature, the usage of the “Repetition Number” field is left up to the eNodeB to handle.

Table 1 summarizes the observations and proposals as in [2-6]:

Table 1: HARQ-ACK delay sets for Alt-2e according with [2-6]

| Company | Compendium of views on the usability of the “Repetition number” field [2], [4-5]. |
| --- | --- |
| Huawei, Hi-silicon | Proposal 1: For the 14 HARQ processes feature, the usage of the “Repetition Number” field is left up to the eNodeB to handle. |
| Nokia, Nokia Shanghai Bell | Conclusion: In Rel-17 for the 14 HARQ processes feature, the usage of the “Repetition Number” field follows the legacy behavior where if the “HARQ-ACK bundling flag” field is set to 1 the UE shall assume that the PDSCH repetitions equal 1. • Note 1: There is no impact on TS 36.212. • Note 2: TS 36.213 to capture that the above legacy behavior also applies for the 14 HARQ processes feature. |
| Ericsson | Proposal 1 RAN1 needs to ponder the importance of the extra “mixed” scenarios that would be possible to address with Alt-B, versus the benefits that Alt-A brings in terms of certainty to all entities (e.g., importance for the IODT phase). |

Companies are encouraged to consider the pros and cons on Alt-A and Alt-B as to provide their views towards selecting one of the two alternatives.

| Company | Please state your views/arguments on which option you prefer: Alt-A or Alt-B | Comments |
| --- | --- | --- |
| Qualcomm | Alt-A | There is unclear need to modify the legacy behavior. |
| Lenovo, MotoM | Alt-A | We slightly prefer to follow the legacy behavior that if the “HARQ-ACK bundling flag” field is set to 1 the UE shall assume that the PDSCH repetitions equal 1, and “Repetition Number” field is pending as legacy. |

2.2 	TP on TS 36.211: Editorial on a HL parameter name

The TP on TS 36.211 as presented in [3], basically aims at performing an editorial change to write “ce” instead of “CE” in the higher layer parameter name “ce-PDSCH-14HARQ-Config”.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~ Start of text proposal to 36.211 ~~~~~~~~~~~~~~~~~~~~~~~~~~~ -----------------------------< Start of the 1st Change >------------------------------ - 5.4.3 Mapping to physical resources < Unchanged parts are omitted > For BL/CE UEs, PUCCH is transmitted with repetitions. - The BL/CE UE is not expected to transmit with NrepPUCCH>1 when ce-PDSCH-14HARQ-Config is configured. < Unchanged parts are omitted > ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~End of text proposal to 36.211 ~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Companies are encouraged to provide their views.

| Company | OK with the Editorial TP on TS 36.211? | Comments |
| --- | --- | --- |
| Qualcomm |  | We would suggest to wait till RAN2 has finalized the ASN.1 before making editorial changes. |
| Lenovo, MotoM | OK |  |

2.3 	TP on TS 36.213: More specific description on the PDSCH scheduling delay value

The TP on TS 36.213 as presented in [3], intends to add a more specific description on clause 7.1.11 for the cited PDSCH scheduling delay value.

~~~~~~~~~~~~~~~~~~~~~~~~~~~ Start of text proposal to 36.213 ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~ -----------------------------< Start of the 1st Change >------------------------------ - 7.1.11 PDSCH subframe assignment for BL/CE UE A BL/CE UE shall upon detection of a MPDCCH with DCI format 6-1A/6-1B/6-2 intended for the UE, decode the corresponding PDSCH in subframe(s) n+ki with i = 0, 1, …, NTBN-1 according to the MPDCCH, where < Unchanged parts are omitted > - otherwise, - subframe(s) ni = n+ki with i=0,1,…, NTBN-1 are NTBN consecutive BL/CE DL subframe(s), where , and subframe n+x is the jth BL/CE DL subframe after subframe n, and j is given by the value of the PDSCH scheduling delay options as defined in [4] if the UE is configured with CEModeA and 'PDSCH scheduling delay and HARQ-ACK delay for 14 HARQ' field is present in the corresponding DCI, j=2 otherwise. < Unchanged parts are omitted > ~~~~~~~~~~~~~~~~~~~~~~~~~~~~End of text proposal to 36.213 ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Companies are encouraged to provide their views.

| Company | OK with the TP on TS 36.213? | Comments |
| --- | --- | --- |
| Lenovo, MotoM | OK |  |

5	References

RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, RAN #88e, Electronic Meeting, June 29th – July 3rd, 2020.

R1-2200977, “Support of 14-HARQ processes in DL for HD-FDD MTC UEs,” Huawei, Hisilicon, RAN1# 108-e, e-Meeting, February 21st – March 3rd, 2022.

R1-2201894, “Remaining issues for introduction of 14-HARQ processes in DL for eMTC,” ZTE, Sanechips, RAN1# 108-e, e-Meeting, February 21st – March 3rd, 2022.

R1-2202278, “Support of 14 HARQ processes in DL in LTE-MTC,” Ericsson, RAN1# 108-e, e-Meeting, February 21st – March 3rd, 2022.

R1-2202369, “Support of 14-HARQ processes in DL for eMTC,” Nokia, Nokia Shanghai Bell, RAN1# 108-e, e-Meeting, February 21st – March 3rd, 2022.