3GPP TSG-RAN WG1 Meeting #109-e	R1-220xxxx

# Electronic Meeting, May 9th – 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Draft CR] Corrections on SRS carrier switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, [ ],[ ] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | [TBD] |  |  |  |  |  | Date: |  |  | 2022-05-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrections on SRS carrier switching for NR |  |  |  |  |  |  |  |  |
| Summary of change: |  | In RAN1#108-e, following agreement on prioiritzation rules for SRS carrier switching for intra-band and inter-band CA was reached and working assumption on descrition of new UE capability was agreed Agreement For SRS carrier switching in Rel-17, - Introduce prioritization rules for carriers that are in the same band as the source CC for intra-band CA - Introduce prioritization rules for carriers that are in a different band as the source CC for inter-band CA - Introduce a new capability Working Assumption A new UE capability is defined as below, - For each “source-target” pair (as indicated by srs-SwitchingTimesListNR), the UE can indicate which other bands in the band combination are affected by the SRS switch. If this new indication is missing, the UE defaults to Rel-15 behavior. - If the UE indicates the new list of bands, the dropping rules / timelines apply to the bands indicated by the list (requires update in RAN1 specs). Note: the new UE capability has no impact on the legacy capability txSwitchImpactToRx and txSwitchWithAnotherBand When half duplex TDD CA and SRS carrier switching are supported simultaneously by UE, following agreement was made in RAN1#108-e to clarfiy the UE behavior: Agreement For Rel-17, when the UE supports half duplex TDD CA and SRS carrier switching simultaneously, the UE first applies SRS prioritization rules, next applies collision directional rules. In RAN1#109-e, corresponding text proposals are agreed for TS 38.214 clause 6.2.1.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Prioritization for SRS carrier switching for intra-band, inter-band CA is undefined and UE behavior is ambiguous when the UE supports half duplex TDD CA and SRS carrier switching. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.306.. CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | New |  |  |  |  |  |  |  |  |

6.2.1.3	UE sounding procedure between component carriers

For a carrier of a serving cell c1 with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, denote c2 as the corresponding carrier of a serving cell whose UL transmissions are temporarily suspended as signalled by higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier. Define the set Sc2={c2, s1(c2)…sN-1(c2)} as the set of carriers of serving cells that each carrier meets one of the following conditions:

-               si(c2) is in the same band and same TAG as c2,

-               si(c2) is a carrier of inter-band CA with c2 and si(c2) is indicated through the capability signalling ImpactedBands-SRS-CS-v17 to be affected by the SRS switch from c2 to c1.

Where 1≤i≤N-1.

A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c2.

For an SRS transmission starting in symbol Nc1 of carrier c1 and a conflicting transmission in any carrier c2 within the set Sc2 starting in symbol NSc2,  the UE shall apply the prioritization / dropping rules in the remainder of this clause taking into account:

-	DCI(s) for which the time interval between the last symbol of PDCCH and Nc1 is at least N2 symbols and an additional time duration TSRSCS,  and the time interval between the last symbol of PDCCH and NSc2 is at least   N2 symbols; and

-	semi-persistent CSI reports or SRS considered active at least N2 symbols and an additional time duration TSRSCS before Nc1, and considered active at least N2 symbols before NSc2.

where TSRSCS=max⁡{switchingTimeUL,switchingTimeDL}, and the time interval unit of OFDM symbol is counted based on the smaller subcarrier spacing across any carrier within the set Sc2,  c1, c2 and their corresponding scheduling cells.

The following prioritization rules shall be applied in case of collision between a transmission of SRS over carrier c1  and transmission of a physical signal/channel over a carrier of a serving cell in set Sc2

- the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c2  and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol.

- the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c2 and PUSCH transmission carrying aperiodic CSI on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol.

- the UE shall drop PUCCH/PUSCH transmission carrying periodic/semi-persistent CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on a carrier of a serving cell in set Sc2   configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c2 happen to overlap in the same symbol.

- the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR on a carrier of a serving cell in set Sc2 whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c2 happen to overlap in the same symbol.

For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA', and given by SRS-CarrierSwitching, without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells in the indicated set of serving cells configured by higher layers, where the UE in each serving cell transmits the configured one or two SRS resource set(s) from srs-ResourceSetToAddModList with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'.

For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB' without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells with aperiodic SRS triggered in the DCI, and the UE in each serving cell transmits the configured one or two SRS resource set(s) from srs-ResourceSetToAddModList with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'.

If the UE is not configured for PUSCH/PUCCH transmission on carrier c1 with slot formats comprised of DL and UL symbols, and if the UE is not capable of simultaneous reception and transmission on carrier c1 and serving cell c2, the UE is not expected to be configured or indicated with SRS resource(s) such that SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) would collide with the REs corresponding to the SS/PBCH blocks configured for the UE or the slots belonging to a control resource set indicated by MIB or SIB1 on serving cell c2.

For n-th (n ≥ 1) aperiodic SRS transmission on a cell c, upon detection of a positive SRS request on a grant, the UE shall commence this SRS transmission on the configured symbol and slot provided

-	it is no earlier than the summation of

-	the maximum time duration between the two durations spanned by N OFDM symbols of the numerology of cell c and the cell carrying the grant respectively, and

-	the UL or DL RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR,

-	it does not collide with any previous SRS transmissions, or interruption due to UL or DL RF retuning time.

otherwise, n-th SRS transmission is dropped, where N is the reported capability as the minimum time interval in unit of symbols, between the DCI triggering and aperiodic SRS transmission.

In case of inter-band carrier aggregation, a UE can simultaneously transmit SRS and PUCCH/PUSCH across component carriers in different bands subject to the UE's capability.

In case of inter-band carrier aggregation, a UE can simultaneously transmit PRACH and SRS across component carriers in different bands subject to UE's capability.

If the UE is not configured for PUSCH/PUCCH transmission for at least one serving cell configured with slot formats comprised of DL and UL symbols, and if the UE is not capable of simultaneous reception and transmission on serving cell c1 and serving cell s(c2), and if a UE

-	is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = ‘enabled’ for a set of serving cell(s) among the configured multiple serving cells including serving cell c1 and s(c2), and

-	indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and

-	is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells,

the UE shall apply first the prioritization/dropping rules described above for sounding procedure between component carriers and then apply the procedures for directional collision handling in clause 11.1 of [6, TS 38.213].