3GPP TSG-RAN WG1 Meeting #109-e	R1- 22xxxxx

e-Meeting, May 9th – 20th, 2022

Agenda Item:	7.1

Source:	Moderator (Ericsson)

Title:	Summary of email discussion [109-e-NR-CRs-08]: Correction for parallel transmission of SRS and PUSCH/PUCCH

Document for:	Discussion, Decision

# Introduction

This document summarizes the discussion during RAN1#109-e meeting for the following email discussion tasked by Chair:

[109-e-NR-CRs-08] Correction for parallel transmission of SRS and PUSCH/PUCCH by May 13 – Sorour (Ericsson)

- Relevant tdoc: R1-2204555

## Problem description in R1-2204555

The following problem was described in [3]:

Based on the agreement RAN1#93 shown below, the following description in TS38.214, clause 6.2.1 with corresponding capability in TS38.306 was implemented:

Agreements(RAN1#93):

- The UE is not expected to be configured to transmit on the same OFDM symbol with an SRS resource and a PUCCH/PUSCH across different CCs in intra-band CA

  - Note: no spec change is needed.

- Parallel SRS and PUCCH/PUSCH transmissions across CCs is supported in inter-band CA.

  - Note: if case parallel SRS and PUCCH/PUSCH is supported, the SRS resource does not belong to a set which is for antenna switching, if the SRS resource set for antenna switching has more than one SRS resource (T < R)

  - Supporting of this feature is subject to UE capability which is a separate capability

Agreements(RAN1#93):

Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs is supported in inter-band CA

### TS 38.214 V15.16.0 [1] ### 6.2.1 UE sounding procedure .... In case of intra-band carrier aggregation or in inter-band CA band combination if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by UE, the UE is not expected to be configured with SRS from a carrier and PUSCH/UL DM-RS/UL PT-RS/PUCCH formats from a different carrier in the same symbol. In case of intra-band carrier aggregation or in inter-band CA band combination if simultaneous SRS and PRACH transmissions are not supported by UE, the UE shall not transmit simultaneously SRS resource(s) from a carrier and PRACH from a different carrier. .....

### TS 38.306 V15.16.0 [2] ### parallelTxSRS-PUCCH-PUSCH ### Indicates whether the UE supports parallel transmission of SRS and PUCCH/ PUSCH across CCs in an inter-band CA band combination

The usage of “configured” in the specification text (as well as the agreement) is ambiguous for the following reasons:

Firstly, it is not clear whether the word “configured” is intended for SRS “configuration” or for TDRA//UL DM-RS/UL PT-RS/PUCCH “configurations”. Secondly, the description implies that no entries in the TDRA table or PUCCH resource set configurations would be allowed to overlap with an SRS, even if the gNB would avoid indicating such an entry in a slot with SRS for UEs. Such a consequence clearly imposes severe and meaningless restrictions given that the PUSCH TDRA resources and PUCCH resources are configured, while the collision with SRS can be avoided by scheduling. The intention of the agreements seems to be avoiding collision between the “actual transmissions” rather than any configurations.

Moreover, the source of ambiguity is the formulation of the corresponding agreement for “intra-band CA” for simultaneous transmission of SRS and PUSCH/PUSCH as shown above. This ambiguity is incorrectly reflected for the specification of the “inter-band CA” case while there is no mention of “configuration” in the supporting agreement for this case.  In fact, the agreements for support of simultaneous transmission of SRS and PUSCH/PUCCH and simultaneous transmission of SRS and PRACH in the absence of corresponding capabilities, are formulated similarly as shown above but their corresponding specifications are described differently, and without any ambiguity in case of the latter.

Summary of problem description:

- The usage of “configured” in the specification text (as well as the agreement) is ambiguous.

  - For intra-band CA and intre-band CA when UE is not capable of simultanoues transmission of SRS and PUSCH/PUCCH, it is not clear if configuration of TDRA table or PUCCH resources that overlap with SRS are permitted, even no collison in actuall transmissions would be planned.

- It seems the intetion of the agreement, as well as the specifcation was “the actual transmission”, and not the corresponding “configuration” for the actual transmission.

## Proposed solution in R1-2204555

To solve the problem described in previous section, [3] proposes the following TP for TS 38.214 V15.16.0.

6.2.1 UE sounding procedure <unchanged text omitted> In case of intra-band carrier aggregation or in inter-band CA band combination if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by UE, the UE is not expected to transmit SRS from a carrier and PUSCH/UL DM-RS/UL PT-RS/PUCCH from a different carrier in the same symbol. In case of intra-band carrier aggregation or in inter-band CA band combination if simultaneous SRS and PRACH transmissions are not supported by UE, the UE shall not transmit simultaneously SRS resource(s) from a carrier and PRACH from a different carrier. <unchanged text omitted>

# Discussion

## Frist discussion round

Please share your view regarding the following questions:

Question 1:

- Do you agree that the intention of the agreement and the corresponding specifcation text that disucssed in section 1.1 was “the actual transmissions” of SRS/PUSCH/PUCCH, and not the “configurations” corresponding to the actual transmissions of SRS/PUSCH/PUCCH?

  - If the answer is No, what is your explanation towards the issues raised in section 1.1?

  - If the answer is Yes, please continue with Question 2.

| Please share your view regarding Question 1 above. |  |
| --- | --- |
| Company | Comment |

Question 2 (Only if the answer to Question 1 is Yes):

- Which of the following alternatives is preferred? Please indicate if you prefer more than one alternative.

    - Alt-1:  Endorse a Rel-15 TP to align the specifcation description with the intention

      - Use TP in R1-2204555 as baseline and discuss further updates if needed.

        - Please provide any suggestion to improve the TP, if needed.

    - Alt-2:  Endorse a Rel-16 TP to align specifcation description with the intention

      - Use TP in R1-2204555 as baseline and discuss further updates if needed.

      - Add in the cover page a text describing that “It is common understanding that the implementations based on the previous release of the specification is expected to be according to the CR”.

        - Please provide any suggestion to improve the TP and/or additional suggested text for the cover page, if needed.

    - Alt-3: Do not endorse any TP. But endorse a conclusion for Rel-15/16 to capture the intention as the following:

      - Proposed Conclusion: For intra-band CA and intre-band CA when UE is not capable of simultanoues transmission of SRS and PUSCH/PUCCH, the UE is not expected to transmit simultanouesly SRS from a carrier and PUSCH/UL DM-RS/UL PT-RS/PUCCH from a different carrier in the same symbol. There is no restriction on the corresponding configurations of these UL transmissions with respect to collision in time-domain.

        - Please provide any suggestion to improve the description of the proposed conclusion, if needed.

    - Alt- 4: Do nothing.

      - In this case, please explain how the NW can be assured that there is no UE implementation behaving differently than the proposed TP in R1-2204555.

| Please share your view regarding Question 2 above given that the answer is Yes to Question 1. |  |
| --- | --- |
| Company | Comment |

# Conclusion

TBD

# References

3GPP TS 38.214	NR; Physical layer procedures for data, V15.16.0.

3GPP TS 38.306	NR; User Equipment (UE) radio access capabilities, V15.16.0

R1-2204555		Correction for parallel transmission of SRS and PUSCH/PUCCH; Ericsson