3GPP TSG-RAN WG1 Meeting #109-e	R1-220xxxx

# Electronic Meeting, May 9th – 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Simultaneous transmission of SRS and other channels for intra-band non-contiguous carrier aggregation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Qualcomm) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Rel-15 NR has an artificial limitation in not allowing simultaneous transmission of SRS and other channels in intra-band non-contiguous CA. During R1#109, this issue was discussed under email thread [109-e-NR-CRs-12], summarized in R1-21XXXXX, where the following was agreed (TPs and FGs omitted for brevity): Proposal: Introduce feature groups X-1 and X-2 as described below. · Introduce a new Rel-17 RRC parameter (UE-specific) to enable the UE behavior under X-2. · Endorse TP2v2 as described below NOTE: For parallel SRS + PUSCH/PUCCH (X-1), it is RAN1’s understanding that: · The gNBs who do not support parallel Tx (i.e. who do not understand the new capability) will not indicate/configure/schedule simultaneous transmission, and therefore there are no backward compatibility issues. · Rel-17 UE who reported capability of parallel Tx conducts parallel Tx if the gNB indicates/configures/schedules simultaneous transmission. Therefore - gNBs who do not support Rel-17 parallel Tx do not indicate/configure/schedule simultaneous transmission. - gNBs who support Rel-17 parallel Tx can indicate/configure/schedule simultaneous transmission accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For SRS + PUSCH/PUCCH, the UE can perform simultaneous transmission in intra-band non-contiguous CA if its capability indicates so. For PRACH + SRS, the UE can perform simultaneous transmission in intra-band non-contiguous CA if its capability indicates so and the UE is enabled by RRC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE canot perform simultaneous transmission of SRS and other channels in intra-band non-contiguous CA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.306, 38.331 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2.1	UE sounding procedure

<Unchanged parts are omitted>

In case of intra-band contiguous carrier aggregation, or in inter-band or intra-band non-contiguous CA band combination if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by UE, the UE is not expected to be configured with SRS from a carrier and PUSCH/UL DM-RS/UL PT-RS/PUCCH formats from a different carrier in the same symbol.

In case of intra-band contiguous carrier aggregation, or in inter-band CA band combination if simultaneous SRS and PRACH transmissions are not supported by UE, or in case of intra-band non-contiguous CA band combination if the UE is not configured with higher layer parameter intraBandNC-PRACH-simulTx-r17, the UE shall not transmit simultaneously SRS resource(s) from a carrier and PRACH from a different carrier.

<Unchanged parts are omitted>