3GPP TSG RAN WG1 #109-e	R1-22xxxxx

e-Meeting, May 9th – 20th, 2022

Agenda item:		7.1

Source:	Moderator (Qualcomm)

Title:	[109-e-NR-CRs-12] On parallel transmission of SRS and other channels

Document for:		Discussion

# Introduction

In [1], the issue of simultaneous transmission of SRS and other channels for intra-band non-contiguous CA was brought up.

In particular, the following two capabilities are mentioned:

| parallelTxSRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of SRS and PUCCH/ PUSCH across CCs in an inter-band CA band combination. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| parallelTxPRACH-SRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of PRACH and SRS/PUCCH/PUSCH across CCs in an inter-band CA band combination. | BC | No | N/A | N/A |

And the following paragraph in TS 38.214:

In case of intra-band carrier aggregation or in inter-band CA band combination if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by UE, the UE is not expected to be configured with SRS from a carrier and PUSCH/UL DM-RS/UL PT-RS/PUCCH formats from a different carrier in the same symbol.

In case of intra-band carrier aggregation or in inter-band CA band combination if simultaneous SRS and PRACH transmissions are not supported by UE, the UE shall not transmit simultaneously SRS resource(s) from a carrier and PRACH from a different carrier.

The issue discussed in [1] is whether intra-band non-contiguous should be treated as intra-band or inter-band. In [1], it is argued that intra-band non-contiguous CA may be similar to inter-band, since the signaling for a BC is by repeating the same band more than once, and also the UE may physically use multiple power amplifiers for the different carriers (unlike the intra-band contiguous, which is assumed to use a single power amplifier).

TPs for clarifying that the capability is applicable to intra-band non-contiguous CA are provided in [1] and presented in the appendix for completeness.

In line with the points above, the Feature Lead proposes the following two points for discussion:

# Email discussion

Q1: Do you agree that a UE operating in intra-band non-contiguous CA should be able to indicate the capability for simultaneous SRS and PUCCH/PUSCH, or simultaneous PRACH and SRS/PUCCH/PUSCH across CCs?

NOTE: This feature is optional, i.e., if a UE does not support this capability for a given band combination, it can skip including it.

| Company | Comment |
| --- | --- |

Q2: If the answer to Q1 is “yes”, do you agree to clarify the above in TS 38.214 and TS 38.306? (and from which release)

NOTE: This would require an LS to RAN2 for the changes in TS 38.306

| Company | Comment |
| --- | --- |

# References

[1] R1-2204973	On parallel transmission of SRS and other channels	Qualcomm Incorporated

# Appendix – TPs from [1]

=====================================TP1 38.306 ======================================

| parallelTxSRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of SRS and PUCCH/ PUSCH across CCs in an inter-band or intra-band non-contiguous CA band combination. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| parallelTxPRACH-SRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of PRACH and SRS/PUCCH/PUSCH across CCs in an inter-band or intra-band non-contiguous CA band combination. | BC | No | N/A | N/A |

====================================================================================

=====================================TP2 38.214 ======================================

In case of intra-band contiguous carrier aggregation, or in inter-band or intra-band non-contiguous CA band combination if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by UE, the UE is not expected to be configured with SRS from a carrier and PUSCH/UL DM-RS/UL PT-RS/PUCCH formats from a different carrier in the same symbol.

In case of intra-band contiguous carrier aggregation, or in inter-band or intra-band non-contiguous CA band combination if simultaneous SRS and PRACH transmissions are not supported by UE, the UE shall not transmit simultaneously SRS resource(s) from a carrier and PRACH from a different carrier.

====================================================================================