3GPP TSG-RAN WG1 Meeting #109-e	R1-220xxxx

e-Meeting, May 9– 20, 2022

Agenda Item:	7.2.4

Source:	Moderator (Huawei)

Title:	Summary of [109-e-R16-V2X-05] Alignment CR on TS 38.214

Document for:	Discussion and decision

# Introduction

This contribution provides discussions related to the following email thread:

[109-e-R16-V2X-05] Alignment CR on TS 38.214 by May 11 – Mixiang (Huawei)

- Discuss R1-2204901

# Discussion

## Issue

R1-2204901 pointed out TS 38.214 Clause 8 uses incorrect subscript notation for determination on slots in a resource pool, where NS_SSB and NSSSB should be replaced with NS-SSB. The following TP is proposed.

# 8 Physical sidelink shared channel related procedures A UE can be configured by higher layers with one or more sidelink resource pools. A sidelink resource pool can be for transmission of PSSCH, as described in Clause 8.1, or for reception of PSSCH, as described in Clause 8.3 and can be associated with either sidelink resource allocation mode 1 or sidelink resource allocation mode 2. In the frequency domain, a sidelink resource pool consists of sl-NumSubchannel contiguous sub-channels. A sub-channel consists of sl-SubchannelSize contiguous PRBs, where sl-NumSubchannel and sl-SubchannelSize are higher layer parameters. The set of slots that may belong to a sidelink resource pool is denoted by (t0SL,t1SL,⋯,tTmax-1SL) where - 0≤tiSL<10240×2μ, 0≤i<Tmax, - the slot index is relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell or DFN 0, - the set includes all the slots except the following slots, - NS-_SSB slots in which S-SS/PSBCH block (S-SSB) is configured, - NnonSL slots in each of which at least one of Y-th, (Y+1)-th, …, (Y+X-1)-th OFDM symbols are not semi-statically configured as UL as per the higher layer parameter tdd-UL-DL-ConfigurationCommon of the serving cell if provided or sl-TDD-Configuration if provided or sl-TDD-Config of the received PSBCH if provided, where Y and X are set by the higher layer parameters sl-StartSymbol and sl-LengthSymbols, respectively. - The reserved slots which are determined by the following steps. 1) the remaining slots excluding NS-_SSB slots and NnonSL slots from the set of all the slots are denoted by (l0,l1,⋯,l(10240×2μ-NS-SSBSSSB-NnonSL-1)) arranged in increasing order of slot index. 2) a slot lr (0≤r<10240×2μ-NS-SSBSSSB-NnonSL) belongs to the reserved slots if r=m∙(10240×2μ-NS-SSBSSSB-NnonSL)Nreserved, here m=0,1,⋯,Nreserved-1 and Nreserved=10240×2μ-NS-SSBSSSB-NnonSL mod Lbitmap where Lbitmap denotes the length of bitmap configured by higher layers. - The slots in the set are arranged in increasing order of slot index. The UE determines the set of slots assigned to a sidelink resource pool as follows: - a bitmap b0,b1,…,bLbitmap-1 associated with the resource pool is used where Lbitmap the length of the bitmap is configured by higher layers. - a slot tkSL (0≤k<10240×2μ-NS-SSBSSSB-NnonSL-Nreserved) belongs to the set if bk'=1 where k'=k mod Lbitmap. - The slots in the set are re-indexed such that the subscripts i of the remaining slots t'iSL are successive {0, 1, …, T'max-1} where T'max is the number of the slots remaining in the set. <Unchanged parts omitted>

## Question

Question: Do you agree with the TP in section 2.1?

| Company | Do you agree? | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Agree |  |
| ZTE,Sanechips | Agree |  |
| OPPO | Agree |  |
| vivo | Agree |  |
| Qualcomm | Agree |  |
| Samsung | Agree |  |
| NTT DOCOMO | Agree |  |
| CATT, GOHIGH | Agree |  |
| Sharp | Agree |  |
| Intel | Agree |  |
| Ericsson | Agree |  |
| Nokia, Nokia Shanghai Bell | Agree |  |
| NEC | Agree |  |
| LGE | Agree |  |

## Summary of 1st round discussion

According to the feedback in Section 2.2, it’s good to see that all companies that provided comments agree with the TP in R1-2204901 and there is no need to update the TP.

Therefore, the following proposal is given:

Proposal: The editorial specification changes in R1-2204901 is provided to improve clarity of TS 38.214 specification.

# Conclusions

# References

- R1-2204901	Correction for slots determination in a resource pool	Huawei, HiSilicon