3GPP TSG RAN WG1 #109-e                            				R1-22xxxx

e-Meeting, May 9th – 20th, 2022

Title:	FL Summary of updates to RRC parameters for Rel-17 positioning enhancements

Source:	Moderator (CATT)

Agenda item:	8.5

Document for:	Discussion and Decision

1. Introduction

This document provides a summary of the following email discussion for AI 8.5:

[109-e-R17-ePos-06] Email discussion on updates to RRC parameters from new agreements, until May 17 – Ren Da (CATT)

The RRC parameters for Rel-17 positioning enhancements from the last meeting in R1-2202541 [1] is used as the base for the udaptes.

2. Issue #1

Agreement

The agreement from RAN1#108e on LMF initiated request of on-demand PRS is amended as follow:

- Note: no RAN1 specification impact is expected

- send the agreement as part of the reply LS to RAN3

Agreement (RAN#109e) - From RAN1 perspective, for LMF-initiated request of on-demand DL PRS, the following group of on-demand DL PRS parameters is defined and signaled - per resource set per positioning frequency layer per FR - DL PRS Periodicity - DL PRS Resource Bandwidth - DL PRS Resource Repetition Factor - Number of DL PRS Resource Symbols per DL PRS Resource - DL-PRS CombSizeN - Two options for indication of DL PRS QCL-Info, either - Option 1: per resource set per positioning frequency layer per FR - LMF recommends a list of QCL sources - Option 2: per resource set per positioning frequency layer per FR - LMF requests to provide the QCL information in the assistance data in NRPPa - per FR - Number of DL PRS frequency layers - either per resource set per positioning frequency layer or per UETRP - Start/end time of DL PRS transmission - either per resource, or per resource set, or per UETRP - ON/OFF indicator (for LMF initiated request only)

## (Round 1) Proposed Changes # 1

- Row 157,  Column P, updated with above agreement, i.e., change UE to TRP

- Rows 162, 163,168, Colum M, changes are made based on above agreement, i.e., change “per UE” to per “per TRP”

| Company | comments |
| --- | --- |

3. Issue #2

Agreement

- Support up to 32 measurement instances in a single measurement report.

- Inform RAN2/RAN3 on RAN1’s decision

## (Round 1) Proposed Changes # 2

- Row 179,  add a new row for the new RRC parameter of maximum measurement instances in a single measurement report

| Company | comments |
| --- | --- |

4. Issue #3

Agreement

Support the LMF to request the Rx beam sweeping factor.

Agreement

The request from LMF on the Rx beam sweeping factor is a single bit per positioning method, which can take two values.

- Value 1: Equal to the UE’s reported Rx beam sweeping factor in the corresponding capability for the band UE supports the feature, and equal to 8 for the FR2 bands that UE does not support the feature.

- Value 2: Equal to 8 (default assumption) for FR2 bands.

- The bit value should be set to the same across DL-TDOA, DL-AoD, and Multi-RTT for hybrid positioning.

## (Round 1) Proposed Changes # 3

- Row 180,  add a new row for the new parameter for the LMF to request UE to use the Rx beam sweeping factor

| Company | comments |
| --- | --- |

6. Summary

TBD

10. References

- R1-2202541 Consolidated higher layers parameter list for Rel-17 NR	Moderator (Ericsson)

- Chairman’s notes, RAN1#109e