3GPP TSG RAN WG1 Meeting #109-e	R1-22xxxxx

e-Meeting, May 9 - 20, 2022

Agenda Item:	8.9

Source:	Moderator (Huawei)

Title:	Text proposal on NB-IoT 16QAM

Document for:	Discussion and Decision

Text proposal on use of DwPTS for NPDSCH with 16QAM

Reason for change: In RAN1#108e meeting, the following was agreed, which has not been captured in spec.

Agreement:

- FDD/TDD differentiation is needed for FGs 1-1/1-2

  - DwPTS in special subframe configuration 9 for normal cyclic prefix is not used for NPDSCH transmission with 16QAM, when 16QAM is configured.

Summary of change: DwPTS in special subframe configuration 9 for normal cyclic prefix is not used for NPDSCH transmission with 16QAM.

Consequences if not approved: DwPTS in special subframe configuration 9 for normal cyclic prefix may be used for NPDSCH transmission with 16QAM, which exceeds the maximum coding rate for NPDSCH.

======================Start of Text proposal to TS 36.211============================

- 10.0.1.2	Frame structure type 2

Frame structure type 2 is applicable to TDD operation only.

The following restrictions apply:

-	Uplink-downlink configuration 0 and 6 are not supported.

-	UpPTS is not used for NPUSCH or NPRACH.

-	DwPTS and UpPTS in special subframe configuration 10 is not used for transmissions.

-	On an NB-IoT carrier for which higher-layer parameter operationModeInfo indicates inband-SamePCI or inband-DifferentPCI, or higher-layer parameter inbandCarrierInfo is present, or on an NB-IoT carrier for SystemInformationBlockType1-NB for which sib1-carrierInfo indicates non-anchor and the value of the higher layer parameter sib-GuardbandInfo is set to sib-GuardbandInbandSamePCI or sib-GuardbandinbandDiffPCI, DwPTS in special subframe configuration 0 and 5 for normal cyclic prefix is not used for NPDCCH and NPDSCH transmission, in addition when npdsch-16QAM-Config-r17 is configured DwPTS in special subframe configuration 9 for normal cyclic prefix is not used for NPDSCH transmission with 16QAM.

-	Higher-layer parameter symbolBitmap does not apply to special subframes.

======================Start of Text proposal to TS 36.211============================

Text proposal on the RRC configuration for NPDSCH 16QAM regarding power allocation in PUR procedure

Reason for change: The RRC parameter of PUR procedure is missed in power allocation for NPDSCH with 16QAM.

Summary of change: The RRC parameter of PUR procedure is added in power allocation for NPDSCH with 16QAM.

Consequences if not approved: The NPDSCH EPRE to NRS EPRE cannot be determined for NPDSCH with 16QAM in PUR procedure.

======================Start of Text proposal to TS 36.213============================

- 16.2.2	Downlink power allocation

<unchanged parts are omitted>

If a UE is configured with the higher layer parameter nrs-PowerRatio in npdsch-16QAM-Config or pur-DL-16QAM-Config,

-	the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs in symbols with NRS is given by 15×(6ρ-1) for a cell with one NRS antenna port and 14×(6ρ-1) for a cell with two NRS antenna ports, where ρ is given by the parameter nrs-PowerRatio.

-	if higher layer parameter operationModeInfo indicates '10' or '11',

-	the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs (not applicable to NPDSCH REs with zero EPRE) is given by the parameter nrs-PowerRatio in symbols without NRS

-	otherwise,

-	the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs (not applicable to NPDSCH REs with zero EPRE) is given by the parameter nrs-PowerRatio in symbols without NRS and CRS, and

-	the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs (not applicable to NPDSCH REs with zero EPRE) is given by the parameter nrs-PowerRatioWithCRS in symbols with CRS.

======================End of Text proposal to TS 36.213============================