3GPP TSG RAN WG1 #109-e	        		   R1-22xxxxx

# e-Meeting, May 9th – 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on UE power savings enhancements in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-05-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Clarify the number of PDCCH candidates per CCE aggregation level and the CCE aggregation levels for Type2A-PDCCH CSS set when searchSpaceZero is used in clause 10.1. - New agreement for a UE to stop skipping PDCCH monitoring after a PUCCH transmission with positive SR in clause 10.4. - Clarify the number of frames for detection of DCI format 2_7 in clause 10.4A. - Number of paging occasions depends on whether or not number of subgroups per paging occasion is provided in clause 10.4A which is not according to RAN2#117-e and RAN2#118-e agreements. - Clarify reference point of TRS availability indication in clause 10.4B. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the number of PDCCH candidates per CCE aggregation level and the CCE aggregation levels for Type2A-PDCCH CSS set when searchSpaceZero is used in clause 10.1. - Capture that a UE stops skipping PDCCH monitoring after a PUCCH transmission with positive SR in clause 10.4. - Adjustment of the text in clause 10.4A. - Remove dependence of umber of paging occasions on whether or not number of subgroups per paging occasion is provided in clause 10.4A. - Update text in clause 10.4B to capture that the frame providing the DCI format with the TRS availability indication is within the DRX cycle. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for for UE power savings enhancements in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1, 10.4, 10.4A, 10.4B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS 38.212 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged text is omitted ***

## 10.1	UE procedure for determining physical downlink control channel assignment

If a UE is not provided pagingSearchSpace for Type2-PDCCH CSS set, the UE does not monitor PDCCH for Type2-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level for Type2-PDCCH CSS set are given in Table 10.1-1.

If a UE is not provided pei-SearchSpace for Type2A-PDCCH CSS set, the UE does not monitor PDCCH for Type2A-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the maximum number of PDCCH candidates per CCE aggregation level for Type2A-PDCCH CSS set are given in Table 10.1-1. If the UE is provided pei-SearchSpace with zero value for the Type2A-PDCCH CSS set index, and for the SS/PBCH block and CORESET multiplexing patterns 2 and 3, the UE determines PDCCH monitoring occasions as described in clause 13 and the CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level for Type2A-PDCCH CSS set are given in Table 10.1-1.

*** Unchanged text is omitted ***

## 10.4	Search space set group switching and skipping of PDCCH monitoring

*** Unchanged text is omitted ***

When the PDCCH monitoring adaptation field indicates to a UE to start PDCCH monitoring according to search space sets with a first group index and stop PDCCH monitoring according to search space sets with a second group index, the UE applies the indication

-	at the beginning of a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field when μ∈0, 1, 2, 3,

-	at the beginning of a first slot, of a slot group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field when μ∈5, 6

When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. If the UE transmits a PUCCH providing a positive SR after the UE detects a DCI format providing the PDCCH monitoring adaptation field, the UE resumes PDCCH monitoring starting at the beginning of a first slot that is after a last symbol of the PUCCH transmission.

If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring an active DL BWP of on a serving cell and the timer is running, the UE

-	resets the timer after a slot of the active DL BWP of the serving cell if the UE detects a DCI format in a PDCCH reception in the slot for with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI

-	otherwise, decrements the timer value by one after a slot of the active DL BWP of the serving cell

*** Unchanged text is omitted ***

## 10.4A	PDCCH monitoring for early indication of paging

A UE can be provided the following for detection of a DCI format 2_7 in RRC_IDLE state or in RRC_INACTIVE state [12, TS 38.331]

-	a search space set, by pei-SearchSpace, to monitor PDCCH for detection of DCI format 2_7 according to a Type2A-PDCCH CSS set as described in clause 10.1

-	a number of frames, by pei-FrameOffset, from the start of a frame to the start of a first paging frame of paging frames associated with a number of PDCCH monitoring occasions for DCI format 2_7 [17, TS 38.304]

-	a number of symbols, by firstPDCCH-MonitoringOccasionOfPEI-O, from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7

-	a size, by payloadSizeDCI-2-7

-	a number of subgroups per paging occasion, NSGPO, by subgroupsNumPerPO

-	a number of paging occasions associated with the number of PDCCH monitoring occasions for DCI format 2_7, NPOPEI, by po-NumPerPEI

A paging indication field of DCI format 2_7 includes NPOPEI segments of K bits, where K=NSGPONSGPO>0K=1NSGPO. For a subgroup index iSG, 0≤iSG<K, a UE determines a value for the iPO⋅K+iSG bit in the paging indication field, where iPO=UE_IDmodN⋅NS+iSmodNPOPEI is a paging occasion index, and UE_ID, N, NS, iSG, and iS are defined in [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion.

If NPOPEI<NS, the number of symbols from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 that is associated with paging occasion index iPO is the iSNPOPEI+1-th value from the NSNPOPEI values provided by firstPDCCH-MonitoringOccasionOfPEI-O.

*** Unchanged text is omitted ***

## 10.4B	Indication of TRS resources

A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7, if pei-SearchSpace is provided, and a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2.

A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame within the DRX cycle that includes the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap indicates no change to a current assumption for the availability or unavailability of associated TRS resource sets.

A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames.

## *** Unchanged text is omitted ***