Proposal 1.A: The work scope of Type-II codebook refinement for CJT mTRP includes refinement of the following codebooks:

  - Rel-16 eType-II regular codebook

  - Rel-17 FeType-II port selection (PS) codebook

FFS: Whether to prioritize/down-select from the two

Proposal 1.B: The work scope of Type-II codebook refinement for CJT mTRP includes the support of NTRP={1, 2, 3, 4} cooperating TRPs

  - FFS: Signaling of NTRP, e.g. higher-layer (RRC) vs. dynamic

  - FFS: Determination of NTRP, e.g. NW-configured vs UE-selected

  - FFS: Whether to prioritize or only support NTRP={1, 2}

Proposal 1.C: The work scope of Type-II codebook refinement for CJT mTRP includes the following NZP CSI-RS (CMR) setups in Resource Setting associated with Rel-18 Type-II codebook for CJT

- Opt1: 1 NZP CSI-RS resource, max # ports = 32

  - FFS: whether/how to associate TCI states and CSI-RS ports

- Opt2: K>1 NZP CSI-RS resources with the same number of ports (representing K TRPs)

  - FFS: The maximum number of ports per resource, and the total number of ports across all resources

FFS: Whether to prioritize/down-select from the two options

Proposal 1.D: The work scope of Type-II codebook refinement for CJT mTRP includes down-selecting at least one or merging from the following codebook structures:

- Alt1A. Per-TRP/TRP group (port-group or resource) SD/FD basis selection + relative co-phasing/amplitude (including WB and/or SB). Example formulation (N = number of TRPs or TRP groups):

(a1p1)×W1,1W2,1Wf,1H⋮(aNpN)×W1,NW2,NWf,NH

  - ar = co-amplitude and

  - pr = co-phase

  - Including special case of ar=pr=1 (no co-scaling)

- Alt1B. Per-TRP/TRP group (port-group or resource) joint SD-FD basis selection + relative co-phasing/amplitude (including WB and/or SB). Example formulation (N = number of TRPs or TRP groups):

(a1p1)×WSF,1W2,1⋮(aNpN)×WSF,NW2,N

  - ar = co-amplitude and

  - pr = co-phase

  - Including special case of ar=pr=1 (no co-scaling)

- Alt2. Per-TRP/TRP group (port-group or resource) SD basis selection and joint (across N TRPs) FD basis selection. Example formulation (N = number of TRPs or TRP groups):

W1,100⋱00000000W1,NW2WfH

Proposal 2.A: The work scope of Type-II codebook refinement for high/medium velocities includes refinement of the following codebooks, based on a common design framework:

  - Rel-16 eType-II regular codebook

  - Rel-17 FeType-II port selection (PS) codebook

FFS: Whether to prioritize/down-select from the two

Proposal 2.B: The work scope of Type-II codebook refinement for high/medium velocities includes down selection from the following codebook structures (for discussion purposes):

- Alt1. Time-domain basis,

  - Alt1A: Time-domain basis commonly selected for all SD/FD bases, e.g. Wf*⨂W1W2WtH

  - Alt1B: Time-domain basis independently selected for different SD/FD bases

- Alt2. Doppler-domain basis

  - Alt2A: Doppler-domain basis commonly selected for all SD/FD bases, e.g. W1W2(Wf⨂Wd)H

  - Alt2B: Doppler-domain basis independently selected for different SD/FD bases

  - Note that Wd may be the identity as a special case

- Alt3. Reuse Rel-16/17 (F)eType-II codebook with multiple W2 and a single W1 and Wf report.

Proposal 2.C: The work scope of Type-II codebook refinement for high/medium velocities includes down selection from the following Doppler-/time-domain basis waveforms for codebook design:

- Alt1. Orthogonal DFT (with or without rotation factor)

- Alt2. Oversampled DFT

- Alt3. Other waveforms, e.g. DCT, Slepian

- Alt4. Identity (i.e. no Doppler-/time-domain compression)

Proposal 2.D: The work scope of Type-II codebook refinement for high/medium velocities includes the following CSI measurement and calculation aspects:

- Potential refinement on Resource setting configuration on CSI-RS (for CSI and/or tracking) for measuring a burst of CSI-RS, including the applicable time-domain behaviors

- Whether/how UE-side or gNB-side prediction is assumed for CQI/PMI/RI calculation

- Potential enhancements on CQI definition and calculation procedure in relation to the PMI of Rel-18 Type-II codebook for high/medium velocities

- Potential enhancement on definition of CSI reference resource

Proposal 3.A: The work scope of TRS-based TDCP reporting focuses on the following use cases for evaluation purposes:

- Targeting medium and high UE speed, e.g. 10-120km/h as well as HST speed

- Aiding gNB to determine

  - CSI reporting configuration and CSI-RS resource configuration parameters,

  - Precoding scheme, using one of the CSI feedback based precoding schemes or an UL-SRS reciprocity based precoding scheme

- Aiding gNB-side CSI prediction

Proposal 3.B: The work scope of TRS-based TDCP reporting includes down selection from the following TDCP reporting formats:

- Alt1. Stand-alone reporting (no inter-dependence with other CSI/UCI parameters)

  - Note: This doesn’t preclude multiplexing with other UCI parameters (e.g. CSI, ACK, SR, …) on PUCCH/PUSCH, if applicable

- Alt2. Inter-dependent and reported with other CSI parameter(s)

Proposal 3.C: The work scope of TRS-based TDCP reporting includes down selection from the following TDCP parameters:

- Alt1. Doppler shift

- Alt2. Doppler spread

- Alt3. Cross-correlation in time

- Alt4A. Relative Doppler shift of a number of peaks in CIR

- Alt4B. Relative Doppler shifts of different TRSs

- Alt5: CSI-RS resource and/or CSI reporting setting configuration assistance