3GPP TSG RAN WG1 Meeting #109-e	R1-2203141

e-Meeting, May 9 – 20, 2022

Agenda Item:	9.2.2.2

Source:	Huawei, HiSilicon

Title:	Discussion on AI/ML for CSI feedback enhancement

Document for:	Discussion and Decision

Introduction

In the RAN#94 e-meeting, a new SI to study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface had been approved [1], where the objectives are given in the Appendix. During the study item, three use cases should be investigated to evaluate achievable gains and complexity of AI/ML based techniques. The potential specification impact required to enable AI/ML techniques for the air-interface should also be identified. Furthermore, sub use cases for each of the three use cases should be finalized by RAN#98.

This contribution mainly discusses the potential specification impact and the consideration of sub use cases for CSI feedback enhancement.

Sub use cases

As described in the SID, three targets for CSI feedback are listed as examples, including overhead reduction, improved accuracy and prediction. To achieve these targets, the sub use cases for CSI feedback may include CSI compression and CSI prediction, where CSI compression may further include CSI compression in spatial and frequency domain and CSI compression involving time domain.

For CSI compression, we mainly discuss the two sub use cases subject to two-sided AI/ML models where both gNB and UE are involved in the inference. For CSI prediction, it can be based on a single-sided AI/ML model where the inference is performed at either UE or gNB.

Sub use case 1 - CSI compression in spatial and frequency domain

The legacy Rel-15 Type II codebook compresses the eigenvector of the channel in spatial domain, while the legacy Rel-16 Type II codebook compresses the eigenvector of the channel in spatial and frequency domain. Both of the Rel-15 and Rel-16 Type II codebooks use the linear combination of some DFT bases to represent the spatial-frequency matrix of the eigenvector. As a non-linear method, AI/ML-based CSI compression can learn the channel characteristics in spatial and frequency domain, and can achieve around 10% throughput gain or around 60% overhead reduction compared to the legacy Rel-16 Type II codebook as shown in our companion contribution [2].

Sub use case 2 - CSI compression involving time domain

Besides spatial and frequency domain, there may be some correlations between channels over slots, which can be exploited to conduct further compression. AI/ML-based CSI compression is a potential way to utilize the correlations of channels in time domain and can achieve an additional throughput gain on top of CSI compression in spatial and frequency domain as shown in our companion contribution [2].

Sub use case 3 - CSI prediction

Considering the timeline of legacy CSI feedback, the slots for CSI measurement, CSI feedback and DL transmission utilizing the CSI feedback for precoding are usually different. The CSI information used for DL transmission may be outdated, which may lead to inaccurate DL precoding as a consequence, especially for high mobility scenarios. AI/ML-based CSI prediction is a way to compensate the time difference between CSI measurement and CSI used for DL transmission. The theories behind CSI prediction and CSI compression in time domain are similar, both of them utilize the channel correlations in time domain. But different from the purpose of CSI compression in time domain which is to reduce the feedback overhead, CSI prediction is applied to predict a future channel with high accuracy.

Proposal 1: For AI/ML-based CSI feedback enhancement, the following three sub use cases can be studied:

- CSI compression in spatial and frequency domain

- CSI compression involving time domain

- CSI prediction

Potential specification impact

Since normative work is not the main task of the study item, this section will only briefly discuss some potential aspects that may be required to be considered for AI/ML-based CSI feedback enhancement, rather than the detailed specification design.

Potential specification impact for CSI compression

    - AI/ML model matching between UE and gNB for two-sided model

CSI compression is a typical case that may require joint AI/ML inference operation between UE and gNB. The auto-encoder based AI/ML model is a generally adopted example of two-sided model for AI/ML based CSI feedback enhancement, including the CSI compression part at UE and a corresponding CSI decompression part at gNB, as we discussed in our companion contribution for evaluations [2]. As shown in Figure 1, the CSI encoder located at the UE and the CSI decoder located at the gNB should be matched as a pair to ensure that the CSI decoder can understand the output of the CSI encoder (i.e., compressed CSI feedback) and recovers the original CSI accurately.

Figure 1  Auto-encoder based CSI feedback

In our companion contribution for the AI/ML framework [3], we analyze 3 levels of collaboration between UE and gNB for further study, as shown in Figure 2. To achieve matched AI/ML models via training, there may be two options based on level 1 (AI/ML operation without model exchange) and level 2 (AI/ML operation with model exchange), respectively.

Figure 2  UE-gNB collaboration levels

Option 1 – Jointly train the CSI auto-encoder at one side and deliver the CSI encoder and/or decoder to the other side via air interface.

Option 1 needs an AI/ML model exchange over the air interface and belongs therefore to collaboration level 2. As we discussed in our companion contribution for the AI/ML framework [3], collaboration 2 faces the issues of AI/ML model delivery overhead and AI/ML model representation format (MRF) across platforms.

Option 2- Separate training at gNB and UE with gNB-UE interaction during training.

The gNB and UE can separately adopt proprietary models for their own part of the two-sided model, and perform separate training, so that there is no need to exchange the model from air interface, i.e., this option can work under collaboration level 1 as analyzed in [3]. This option can avoid the above issues of Option 1. To guarantee the trained gNB part and the trained UE part are still matched, some interaction can be taken into account. For example, UE and gNB may utilize a common auto-encoder as a reference to perform training on their own proprietary CSI encoder and CSI decoder, respectively. Alternatively, UE and gNB may share some intermediate training information as training labels so that the other side can have the knowledge on the direction of training.

Observation 1: AI/ML-based CSI feedback enhancement requires joint AI/ML inference between gNB and UE, which relies on CSI model matching between UE and gNB.

Observation 2: For two-sided AI/ML models, interaction between gNB and UE may be helpful to guarantee that gNB and UE can separately train their models to achieve matched models without AI/ML model exchange over the air interface.

Observation 3: Supporting separate training for a two-sided model between UE and gNB may have potential specification impact.

    - Training procedure for CSI feedback

For the situation where the AI/ML model training is performed at gNB side, it requires collecting training inputs and ground-truth labels. For CSI compression, both the training inputs and ground-truth labels can be channel responses or eigenvectors of the original channel. As discussed above, separate training for a two-sided model may require that both the UE and gNB perform model training. Therefore, it may require the UE to report the channel responses or eigenvectors of the original channel with a high resolution so that the gNB can obtain ground-truth channel information for training. In addition, exchange of some intermediate results between the AI/ML models at the UE and gNB may be needed to facilitate separate model training under collaboration level 1 as previously mentioned.

Observation 4: Training procedure for CSI feedback may have potential specification impact.

    - AI/ML-based CSI codebook for CSI compression

From Rel-15 to Rel-17, several precoding matrix indicator (PMI) codebooks have been specified, such as Type I codebook, Type II codebook and enhanced Type II codebook. For AI/ML-based CSI feedback, an AI/ML-based codebook for CSI compression may also need to be specified. Different from the legacy codebook which has physical meanings and is specified with a generation procedure, the AI/ML-based codebook is the output of the AI/ML model, so how to generate the codebook belongs to the implementation and does not need to be specified. The potential specification impact for an AI/ML-based CSI feedback codebook may include the following aspects.

- The dimension of the input and output of CSI compression, such as antenna configurations, sub band configurations and feedback bit configurations.

- Pre-processing and post-processing of the input and output of CSI compression.

Observation 5: Supporting AI/ML-based CSI codebook for CSI compression may have potential specification impact.

    - AI/ML model monitoring for CSI feedback

As discussed in our companion contribution for the AI/ML framework [3], AI/ML model monitoring plays an important role in life cycle management to identify a potential mismatch between the AI/ML model and the environment to avoid AI/ML model inference failure(s). For AI/ML-based CSI feedback, AI/ML model monitoring can be performed at either the UE side or gNB side. But in order to monitor and control the overall performance of a cell, the gNB should take responsibility for making decision on enabling/disabling the AI/ML model.

As discussed in [3], the monitoring metric can be intermediate results or the final KPI. For AI/ML-based CSI compression, an example of the intermediate result is the generalized cosine similarity (GCS) between original and recovered CSI, and an example of the final KPI is the throughput. If the GCS is monitored at the UE side, it may need to report the monitoring results to the gNB. If GCS is monitored at the gNB side, it may ask the UE to feedback the original CSI without compression which is similar to the training procedure. In order to assess the throughput, co-existence of the AI/ML mode and the legacy non-AI/ML mode is needed to compare the throughputs between the AI/ML mode and the legacy mode.

Observation 6: Supporting AI/ML model monitoring for CSI feedback may have potential specification impact.

    - Scalability of AI/ML model for CSI compression

Once an AI/ML model has been trained, the input dimension and output dimension of the AI/ML model is fixed. For CSI feedback, the dimensions of input and output depend on the number of CSI-RS antenna ports, sub bands and the number of feedback bits, etc. However, if one CSI compression model only can be used for one fixed set of configurations, the UE and gNB would need to store plenty of CSI compression models applicable for various configurations. Therefore, it is beneficial to consider scalability of the AI/ML model for CSI compression so that it can adapt to various configurations. The potential specification impact to enable such CSI compression model can be studied.

Observation 7: Supporting scalability of the AI/ML model for CSI feedback may have potential specification impact.

Based on the above discussions and observations, we have the following proposal.

Proposal 2: Study whether potential specification impact is needed for following aspects:

- Separate training for a two-sided model between UE and gNB

- Training procedure for CSI feedback

- AI/ML-based CSI codebook for CSI compression

- AI/ML model monitoring for CSI feedback

- Scalability of AI/ML model for CSI compression

Potential specification impact for CSI prediction

CSI prediction can be performed by a single-sided AI/ML model at the UE side or gNB side. For UE-oriented CSI prediction, the UE and gNB should align their understanding of the predicted CSI. For gNB-oriented CSI prediction, the UE may need to report the ground-truth labels, e.g., channel responses or eigenvectors of channel with high accuracy, to facilitate model training and model monitoring.

Observation 8: Training/monitoring procedure for CSI prediction may have potential specification impact.

Conclusions

In this contribution, we discussed the potential specification impact and the consideration of sub use cases for CSI feedback enhancement. Based on the discussions, we have the following observations and proposals.

Observation 1: AI/ML-based CSI feedback enhancement requires joint AI/ML inference between gNB and UE, which relies on CSI model matching between UE and gNB.

Observation 2: For two-sided AI/ML models, interaction between gNB and UE may be helpful to guarantee that gNB and UE can separately train their models to achieve matched models without AI/ML model exchange over the air interface.

Observation 3: Supporting separate training for a two-sided model between UE and gNB may have potential specification impact.

Observation 4: Training procedure for CSI feedback may have potential specification impact.

Observation 5: Supporting AI/ML-based CSI codebook for CSI compression may have potential specification impact.

Observation 6: Supporting AI/ML model monitoring for CSI feedback may have potential specification impact.

Observation 7: Supporting scalability of the AI/ML model for CSI feedback may have potential specification impact.

Observation 8: Training/monitoring procedure for CSI prediction may have potential specification impact.

Proposal 1: For AI/ML-based CSI feedback enhancement, the following three sub use cases can be studied:

- CSI compression in spatial and frequency domain

- CSI compression involving time domain

- CSI prediction

Proposal 2: Study whether potential specification impact is needed for following aspects:

- Separate training for a two-sided model between UE and gNB

- Training procedure for CSI feedback

- AI/ML-based CSI codebook for CSI compression

- AI/ML model monitoring for CSI feedback

- Scalability of AI/ML model for CSI compression

- References

RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#94e, December 2021.

R1-2203140, “Evaluations on AI/ML for CSI feedback enhancement”, RAN1#109e, Huawei, HiSilicon, May 2022.

R1-2203139, “Discussion on general aspects of AI/ML framework”, RAN1#109e, Huawei, HiSilicon, May 2022.

- Appendix - Objectives in WID

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.