3GPP TSG RAN WG1 #109-e		 R1-2203748

e-Meeting, May 9th – 20th, 2022

Source:	Panasonic

Title: 	Inter-UE coordination for Mode 2 enhancements

Agenda Item:	8.11.2

Item for:	Discussion, Decision

Introduction

Remaining open issues for Scheme 1 were listed as following [1].

- Finalization of UE-B’s behavior when it receives both preferred resource set and non-preferred resource set from the same UE-A or different UE-As

- Finalization of relationship between start/end slots of resource selection window used for sidelink transmission carrying inter-UE coordination information and start/end slots of resource selection window for determining the set of resources

This document provides our view on remaining issues.

Discussion

Reception of preferred resource set and non-preferred resource set

We summarize the condition of inter-UE coordain in Table 1.

Table 1 The condition of inter-UE coordain

|  | Explicit request | Other than explicit request |  |
| --- | --- | --- | --- |
| Cast Type | Unicast | Unicast | Groupcast/Broadcast |
| Container | Both SCI format 2-C and MAC CE or only MAC CE. | Both SCI format 2-C and MAC CE or only MAC CE. | MAC CE |
| Resource set type | Preferred /non-preferred Alt 1 or Alt 2 is (pre-)configured. Alt 1) UE-A’s implementation Alt 2) Indicated by UE-B’s request | Preferred /non-preferred UE-A’s implementation | Non-preferred |
| Trigger condition | The request reception is determined by UE-A’s implementation. | Alt 1 or Alt 2 is (pre-)configured. Alt1) It is up to UE-A implementation Alt2) It can be triggered only when UE-A has data to be transmitted together with the inter-UE coordination information to UE-B |  |

Followings are agreement in RAN1#108-e.

Agreement - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A - It is up to UE-B implementation to use one or multiple of them in its resource (re)selection - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection Agreement - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the different UE-As, - UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set. - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the different UE-As. - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e (except for the processing timeline) - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As, - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

UE-B receives both preferred resource set and non-preferred resource set from same UE-A

Only non-preferred resource is supported in groupcast/broadcast. The source ID of unicast/groupcast/broadcast can be different for same UE-A. Therefore UE-B is not able to identify the combination between preferred resource of unicast and non-preferred resource of groupcast/broadcast even if these are sent from the same UE-A. These are handled as from different UEs from UE-B perspective. The discussion would be only the combination between unicast preferred resource and unicast non-preferred resource.

When UE-B receives both preferred resource set and non-preferred resource set from same UE-A in unicast, there are following 3 cases.

1) UE-B transmits multiple explicit requests to UE-A in different times. UE-A transmits two inter-UE coordination to UE-B in different times. One is preferred resource set, and the other is non-preferred resource set. This behavior is allowed by UE-A’s implementation when UE-B doesn’t indicate the resource set type and UE-B requests both preferred resource set and non-preferred resource set.

2) UE-A transmits two inter-UE coordination to UE-B in different times with other than explicit requests. One is preferred resource set, and the other is non-preferred resource set. This behavior is allowed by UE-A’s implementation.

3) One is explicit request, and the other is other than explicit request.

In these 3 cases, both preferred resource set and non-preferred resource set are useful information for UE-B in unicast. Our view is there should be no difference between explicit request based inter-UE coordination and other than explicit request based inter-UE coordination from same UE-A. Therefore, we propose that UE-B uses both the received preferred resource set and non-preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A. In email discussion [108-e-R17-Sidelink-02], "UE-B does not expect to receive more than one IUC transmission within a latency bound (up to RAN2) from the same UE-A for its resource selection for the same TB transmission" was proposed. We see if the behavior of UE-B receives both preferred resource set and non-preferred resource set from same UE-A is not clarified, this approach is better than just UE implementation matter.

Proposal 1: UE-B uses both the received preferred resource set and non-preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A.

UE-B receives both preferred resource set and non-preferred resource set from different UE-A

When UE-B receives preferred resource set from UE-A(1), non-preferred resource set from UE-A(2), there are following 3 cases.

1) UE-B transmits explicit requests to UE-A(1) and UE-A(2) in different times. UE-A1 transmits preferred resource set to UE-B and UE-A2 transmits non-preferred resource set to UE-B. This behavior is allowed by UE-A’s implementation when UE-B doesn’t indicate resource set type and UE-B’s decision when UE-B request different resource set type for UE-A(1) and UE-A(2).

2) UE-A(1) transmits preferred resources set to  UE-B in unicast and UE-A(2) transmits non-preferred resource set to UE-B in unicast/groupcast/broadcast. This behavior is allowed by UE-A’s implementation.

3) One is explicit request in unicast, and the other is other than explicit request in unicast/groupcast/broadcast.

The preferred resource is only useful for destination UE-A(1) in unicast, The non-preferred resource is useful for both UE-A(2) and UE-A(1) when this information is based on resource overlapping (Condition 2-A-1) in unicast/groupcast/broadcast. If non-preferred resource is based on half duplex issue (Condition 2-A-2) in unicast, this information is not useful for resource allocation of UE-A(1). However, UE-B cannot distinguish condition 2-A-1 and 2-A-2 in unicast. For safety side, UE-B doesn’t use the resource in non-preferred resources set from UE-A(2) to UE-A(2) and UE-A(1).

Therefore, we propose UE-B uses both the received preferred resource set and non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. UE-B uses the received non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the non-preferred resource set.

Proposal 2: UE-B uses both the received preferred resource set and non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. UE-B uses the received non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the non-preferred resource set.

Resource selection window

Followings are agreement in RAN1#108-e.

Agreement - Notations: - (n+T_1) – Start slot of resource selection window for determining the set of resources - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_1) is provided by UE-B’s request as per the existing agreement - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_1) is determined by UE-A’s implementation as per the existing agreement - (n+T_2) – End slot of resource selection window for determining the set of resources - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_2) is provided by UE-B’s request as per the existing agreement - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_2) is determined by UE-A’s implementation as per the existing agreement - (n’+T’_1) – Start slot of resource selection window used for sidelink transmission carrying inter-UE coordination information - (n’+T’_2) – End slot of resource selection window used for sidelink transmission carrying inter-UE coordination information - n' is the slot where UE procedure of determining TX resources of sidelink transmission carrying inter-UE coordination information is triggered - For inter-UE coordination information triggered by UE-B’s explicit request - Alt 1-1: - X1 ≤ (n’+T’_1) - (n’+T’_2) ≤ X2 - For inter-UE coordination information triggered by a condition other than explicit request reception, - Alt 2-2: - (n’+T’_2) < X3 - FFS: Values for X1, X2, X3 Agreement For sensing window for determining the set of resources in Scheme 1, - Notations: - For inter-UE coordination information triggered by UE-B’s explicit request, the values of (n+T_1) and (n+T_2) are provided by the request as per the existing agreement. - For inter-UE coordination information triggered by a condition other than explicit request reception, the values of (n+T_1) and (n+T_2) are determined by UE-A’s implementation as per the existing agreement. - T’’_1 is up to UE-A’s implementation under 0 <= T’’_1 <= Tproc,1 - (n’+T’_1) – Start slot of resource selection window used for sidelink transmission carrying inter-UE coordination information - n' is the slot where UE procedure of determining TX resources of inter-UE coordination information is triggered - Alt 1: - No further change is supported. Note that the sensing window for determining the set of resources is already derived based on the location (n+T_1) and (n+T_2) used for determining the set of resources in TS38.214 section 8.1.4, i.e., sensing window is defined by the range of slots [(n+T_1) – T_0 – T’’_1, (n+T_1) – T_proc,0 – T’’_1].

For inter-UE coordination information triggered by UE-B’s explicit request, X1 and X2 are FFS in the agreement.

・X1 ≤ (n’+T’_1)

・(n’+T’_2) ≤ X2

Two alternatives are considered as shown in Figure 1. In alt 1, UE-B transmits the request after the sensing window for determining the set of resources. In this case, UE-A’s processing time to decode the request (roughly T_proc,0) and to prepare the inter-UE coordination (roughly T_proc,1). In alt1, X1 is n+T_1 and X2 is n+T_2. In alt 2, UE-B can transmit the request to UE-A enough earlier. The resource selection window for determining the sidelink transmission carrying inter-UE coordination information should be located before the T_proc,0+ T_proc,1. For X1, at least is later than (n+T_1) – T_0 – T’’_1 (start of sensing window). When half of sensing window is guaranteed for inter-UE coordination information, X1 is (n+T_1) – (T_0 +Tproc,0)/2 – T’’ _1. X2 is (n+T_1)- (Tproc,0+Tproc,1). Inter-UE coordination should be transmitted before the resource selection window for determining the set of resources. Then we propose alt 2.

Figure 1 Inter-UE coordination information triggered by UE-B’s explicit request.

Proposal 3: For inter-UE coordination information triggered by UE-B’s explicit request,

・(n+T_1) – (T_0+Tproc,0)/2 – T’’_1 ≤ (n’+T’_1)

・(n’+T’_2) ≤ (n+T_1)- (Tproc,0+Tproc,1).

For inter-UE coordination information triggered by a condition other than explicit request reception, X3 is FFS in the agreement.

・(n’+T’_2) < X3

For this case, as same as Alt 2 for explicit request, inter-UE coordination should be transmitted before the resource selection window for determining the set of resources. Then we propose X3 is (n+T_1)- (Tproc,0+Tproc,1).

Proposal 4: For inter-UE coordination information triggered by a condition other than explicit request reception,

・(n’+T’_2) < (n+T_1)- (Tproc,0+Tproc,1).

Conclusion

This document provided our view on enhancement of mode 2 for reliability and latency enhancements. Based on the discussions, we have following proposals.

Proposal 1: UE-B uses both the received preferred resource set and non-preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A.

Proposal 2: UE-B uses both the received preferred resource set and non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. UE-B uses the received non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the non-preferred resource set.

Proposal 3: For inter-UE coordination information triggered by UE-B’s explicit request,

・(n+T_1) – (T_0+ Tproc,0)/2 – T’’_1 ≤ (n’+T’_1)

・(n’+T’_2) ≤ (n+T_1)- (Tproc,0+Tproc,1).

Proposal 4: For inter-UE coordination information triggered by a condition other than explicit request reception,

・(n’+T’_2) < (n+T_1)- (Tproc,0+Tproc,1).

Reference

[1] RP-220945, “Status report of WI NR sidelink enhancement”, RAN1

[2] R1-2202666, “Feature lead summary #7 for AI 8.11.2.2 Feasibility and benefits for mode 2 enhancements”, Moderator (LG Electronics)