3GPP TSG- RAN WG1 Meeting #109-e                                             R1- 2204252

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.1.2

Source:	Apple Inc.

Title:	On Evaluation of Sidelink positioning

Document for:	Discussion/Decision

# Introduction

A SID defining objectives for a Study on expanded and improved NR positioning was agreed in RAN #94 [1], with the following justification for sidelink positioning:

- There is a need for 3GPP to study and develop sidelink positioning solutions that can support the use cases, scenarios and requirements identified for V2X, public safety, commercial and IIOT use cases.

and the following objective:

- Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1].

In this contribution, we address the evaluation methodology for the different SL positioning use cases.

# Discussion

## General Issues for All Use Cases

These issues need to be addressed for all the different use cases under discussion:

Evaluation Metrics: The metrics used should be the same as defined in 38.857 (Study on NR Positioning Enhancements Rel-17) [2] i.e.  the horizontal accuracy, the vertical accuracy and the latency where:

- Horizontal accuracy is the difference between the calculated horizontal position and the actual horizontal position of a UE.

- Vertical accuracy is the difference between the calculated vertical position and the actual vertical position of a UE.

- Latency can be higher layer latency which includes processing delay of the various nodes and signaling between the SL nodes or can be physical layer latency will depend on the specific positioning method used. The PHY layer latency will depend on the SL-positioning techniques that are defined in the study.

Coverage Scenarios: For each of the use cases, the evaluation methodology from Rel-17 should be updated to model in-coverage, partial coverage and out-of-coverage scenarios.

UE deployment and positioning set selection: The evaluation methodology should be updated to incorporate the deployment of multiple sidelink UEs e.g. shadow fading modeling between multiple SL UEs. Also, in the case of the different coverage scenarios, a methodology on the selection of the target UE and the candidate UEs for positioning needs to be decided on.

Proposal 1:

- Evaluation Metrics: Horizontal accuracy, vertical accuracy and PHY/end-to-end latency

- Coverage Scenarios: Evaluation model for in-coverage, partial coverage and out-of-coverage scenarios is needed

- UE deployment and positioning set selection:: SL-UE deployment and candidate SL-UEs/gNBs  and target SL-UE selection for the positioning evaluation are needed.

## Use Case Specific Issues

## In the following, we discuss the baseline evaluation methodology for the commercial, IIoT, public safety and V2X use cases.

### Use case 1: Commercial

For the commercial use case, the following proposal should be used as a baseline for sidelink positioning evaluation:

Proposal 2:  Commercial Use Case:

- Methodology: Re-use methodology in 38.855 (Study on NR positioning support Rel-16) and 38.857 (Study on NR Positioning Enhancements Rel-17)[2]

- Scenarios: Indoor Office, UMi street canyon, UMa (ISD 500m) with focus on FR1

- Maximum BW: 5MHz, 50MHz for 2GHz, 100MHz for 4GHz

### Use case 2: IIoT Use Case

For the IIoT use case, the following proposal should be used as a baseline for sidelink positioning evaluation:

Proposal 3: IIoT Use Case:

- Methodology: Re-use methodology in 38.855 (Study on NR positioning support Rel-16) [3] and 38.857 (Study on NR Positioning Enhancements Rel-17) [2].

- Scenarios:, IIoT (InF-SH and InF-DH)  with focus on FR1

- Maximum BW: 5MHz, 50MHz for 2GHz, 100MHz for 4GHz

### Use case 3: V2X Use Case

For the V2X use case, the following proposal should be used as a baseline for sidelink positioning evaluation:

Proposal 4: V2X Use Case:

- Methodology: Re-use methodology in 38.885 (Study on NR Vehicle-to-Everything (V2X) Rel-16) [4]

- Scenarios: Highway and Urban

- Maximum BW: 20 MHz for 6 GHz and 100 MHz for 30 GHz

### Use case 4: Public Safety Use case

For the Public safety use case, the following proposal should be used as a baseline for sidelink positioning evaluation:

Proposal 5: Public Safety: for both indoor and outdoor scenarios.

- Methodology: Use methodology in 38.855[3]

- Scenarios: Re-use Indoor Office, UMi street canyon and UMa (ISD 500m) from 38.855[3]

- NOTE: based on requirements [5], evaluation can be incorporated into Commercial use case.

# Conclusion

In this contribution, we address SL positioning scenarios and requirements including specific target performance requirements with the following proposals:

Proposal 1:

- Evaluation Metrics: Horizontal accuracy, vertical accuracy and PHY/end-to-end latency

- Coverage Scenarios: Evaluation model for in-coverage, partial coverage and out-of-coverage scenarios is needed

- UE deployment and positioning set selection:: SL-UE deployment and candidate SL-UEs/gNBs  and target SL-UE selection for the positioning evaluation are needed.

Proposal 2:  Commercial Use Case:

- Methodology: Re-use methodology in 38.855 (Study on NR positioning support Rel-16) and 38.857 (Study on NR Positioning Enhancements Rel-17)[2]

- Scenarios: Indoor Office, UMi street canyon, UMa (ISD 500m) with focus on FR1

- Maximum BW: 5MHz, 50MHz for 2GHz, 100MHz for 4GHz

Proposal 3: IIoT Use Case:

- Methodology: Re-use methodology in 38.855 (Study on NR positioning support Rel-16) [3] and 38.857 (Study on NR Positioning Enhancements Rel-17) [2].

- Scenarios:, IIoT (InF-SH and InF-DH)  with focus on FR1

- Maximum BW: 5MHz, 50MHz for 2GHz, 100MHz for 4GHz

Proposal 4: V2X Use Case:

- Methodology: Re-use methodology in 38.885 (Study on NR Vehicle-to-Everything (V2X) Rel-16) [4]

- Scenarios: Highway and Urban

- Maximum BW: 20 MHz for 6 GHz and 100 MHz for 30 GHz

Proposal 5: Public Safety: for both indoor and outdoor scenarios.

- Methodology: Use methodology in 38.855[3]

- Scenarios: Re-use Indoor Office, UMi street canyon and UMa (ISD 500m) from 38.855[3]

- NOTE: based on requirements [5], evaluation can be incorporated into Commercial use case.

# Reference

- RP-213588, Revised SID on Study on expanded and improved NR positioning

- 38.857, Study on NR Positioning Enhancements; (Release 17)

- 38.855, Study on NR positioning support; (Release 16)

- 38.885, Study on NR Vehicle-to-Everything (V2X), (Release 16)

- R1-2204251, Discussion on SL scenarios and requirements, Apple, RAN1 #109-e