3GPP TSG-RAN WG1 Meeting #110 	R1-2208183

Toulouse, France, August 22 – 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0320 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on slot offsets of CSI-RS resource pairs for MTRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Moderator] Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-08-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The brackets for configuring slot offsets of the two resources in a CSI resource pair for Multi-TRP CSI measurement shall be removed in order to capture following agreement and avoid ambiguous understanding of CSI-RS resource configuration at time domain: For CSI measurement associated with a CSI-ReportingConfig for NCJT, support two CMRs within the same CMR pair configured for NCJT measurement hypothesis to be restricted within X continuous slot(s) without DL/UL switch between two CMRs - X=1, 2 - whereas X=1 implying the same slot and X=2 implying two adjacent slots - FFS other restrictions for FR2 - FFS whether UE capability is needed for X=2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the bracket for slot offsets of two CSI-RS resources within a CSI-RS resource pair. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is ambiguous what kind of time domain configuration are allowed for CSI-RS resources of a resource pair for Multi-TRP CSI measurement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.2.2.3	Reference signal (CSI-RS)

##### 5.2.2.3.1	NZP CSI-RS

< Unchanged parts are omitted >

All CSI-RS resources within one set are configured with same density and same nrofPorts, except for the NZP CSI-RS resources used for interference measurement.

The UE expects that all the CSI-RS resources of a resource set are configured with the same starting RB and number of RBs and the same cdm-type.

For a CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, the slot offsets of the two resources in a Resource Pair are configured within X∈{1,2}  X∈{1,[2]}slots, without DL/UL switching in between the two resources, where X=1 implies that the two resources are configured in the same slot, and X=2 implies that the two resources are configured within two adjacent slots.

< Unchanged parts are omitted >