3GPP TSG-RAN WG1 Meeting #110-bis-e	R1-2208477

e-Meeting, October 10 – 19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to channel access field in RAR UL grant in FR2-2 in TS38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN1#110 it was agreed that for operation with shared spectrum access in FR2-2, the RAR UL grant includes a 2-bit ChannelAccess-CPext field for explicit indication of one of Type1/2/3 channel access procedures. However, the current specifications in TS 38.213 do not capture the agreement. Similar to the ChannelAccess-CPext field in fallback DCI formats, the 2 bits shall be present only if the channelAccessMode2-r17 is provided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the description of the ChannelAccess-CPext field of the RAR UL grant, for Type-1 and Type-2 random access procedures, capture that for operation in FR2-2 and if channelAccessMode2-r17 is provided, the channelAccess-CPext field size is 2 bits; 0 bit otherwise. The corresponding row of Table 8.2-1 is modified accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications for UL scheduled by the RAR UL grant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2, 8.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged text omitted ***

## 8.2	Random access response - Type-1 random access procedure

*** Unchanged text omitted ***

The ChannelAccess-CPext field indicates a channel access type and CP extension for operation with shared spectrum channel access [15, TS 37.213] as defined in Table 7.3.1.1.1-4 in TS 38.212 or Table 7.3.1.1.1-4A in TS 38.212 if channelAccessMode-r16 = "semiStatic" is provided. For operation in a cell in frequency range 2-2, the ChannelAccess-CPext field indicates a channel access type as defined in Table 7.3.1.1.1-4B in TS 38.212 if channelAccessMode2-r17 is provided.

Table 8.2-1: Random Access Response Grant Content field size

| RAR grant field | Number of bits |
| --- | --- |
| Frequency hopping flag | 1 |
| PUSCH frequency resource allocation | 14, for operation without shared spectrum channel access 12, for operation with shared spectrum channel access |
| PUSCH time resource allocation | 4 |
| MCS | 4 |
| TPC command for PUSCH | 3 |
| CSI request | 1 |
| ChannelAccess-CPext | 0, for operation without shared spectrum channel access, or for operation in FR2-2 when channelAccessMode2-r17 is not provided. 2, for operation with shared spectrum channel access in FR-1, or in FR2-2 if channelAccessMode2-r17 is provided. |

*** Unchanged text omitted ***

## 8.2A	Random access response - Type-2 random access procedure

*** Unchanged text omitted ***

-	for operation with shared spectrum channel access, a channel access type and CP extension [15, TS 37.213] for a PUCCH transmission is indicated by a ChannelAccess-CPext field in the successRAR as defined in Table 7.3.1.1.1-4 in TS 38.212 or Table 7.3.1.1.1-4A in TS 38.212 if channelAccessMode-r16 = "semiStatic" is provided. For operation in a cell in frequency range 2-2, the ChannelAccess-CPext field indicates a channel access type as defined in Table 7.3.1.1.1-4B in TS 38.212 if channelAccessMode2-r17 is provided.

-	the PUCCH transmission is with a same spatial domain transmission filter and in a same active UL BWP as a last PUSCH transmission

*** Unchanged text omitted ***